| L.A.M. FRE 2243 | FIRST SPIRE | Ref : LAM.PJT.SPI.SPT.200007 Ind 6 Author : K.Dohlen and D.Pouliquen | Page : 1 / 13 Date : 12 June 2001 |
| --- | --- | --- | --- |
| SPIRE Mirrors Specification |  |  |  |

Distribution List :

| RAL | Colin Cunningham | X | Ken J. King | X |
| --- | --- | --- | --- | --- |
|  | Bruce Swinyard | X | Tony Richards | X |
| CNES | Martine Joubert |  | Yvan Blanc |  |
| CEA-Sap | Jean-Louis Auguères |  |  |  |
| DESPA | Guy Michel |  |  |  |
| GSFC | Don.E. Jennings |  |  |  |
| LAM | Armand Artinian |  | Jean-Paul Baluteau | X |
|  | Claude Colin |  | Pascal Dargent | X |
|  | Kjetil Dohlen | X | Didier Ferrand |  |
|  | Michel Jevaud |  | Patrick Levacher |  |
|  | Alain Origné | X | Dominique Pouliquen | X |

_________________________________________________________________________________

LABORATOIRE d'ASTROPHYSIQUE de MARSEILLE-  CNRS    BP 8   13376  MARSEILLE Cedex 12 FRANCE

Tél : (33) +4 91 05 59 00   -   Fax : (33) +4 91 05 69 59

Update

| Date | Indice | Remarks |
| --- | --- | --- |
| 5 June 2000 | 1 | Creation of the document |
| 12 Oct 2000 | 2 | Updated mirror dimensions. Spec P5 added. |
| 28 Nov 2000 | 3 | Corrected illustration of CM3 (p. 9) |
| 18 Dec 2000 | 4 | Remodified CM3 dimension. Spec P6 added (RoC) |
| 21 Feb 2001 | 5 | Fig. p5 replaced by CM3. Sec. 3.3: no. of sets. Added notes in table 1. Corrected baseline version numbers |
| 12 June 2001 | 6 | Clarification of dimensions and decenters in Table 1. |

| Host system | Windows 95 |
| --- | --- |
| Word Processor | Microsoft Word 97 |
| File | /tmp/in/input.doc |

Table of contents

1.	Scope of the document	4

2.	Documents	4

2.1.	Applicable documents	4

2.2.	Glossary	4

3.	The mirrors	5

3.1.	Instrument description	5

3.2.	Mirrors description	6

3.3.	Mission profile	6

3.4.	Product tree	7

4.	Requirements	7

4.1.	Functional requirements	7

4.1.1.	Performance requirements	7

4.1.2.	Technical requirements	7

4.1.3.	Mirror parameters	8

4.2.	Operational requirements	11

4.2.1.	Relialibility	11

4.2.2.	Lifetime	11

4.2.3.	Operating modes	11

4.2.4.	Telemetry	11

4.2.5.	Telecommands	11

4.3.	Interface requirements	12

4.4.	Design and manufacture requirements	12

4.4.1.	Design requirements	12

4.4.2.	Design rules	12

4.4.3.	Manufacture requirements	12

4.5.	Logistic requirements	12

4.6.	Environment requirements	13

4.6.1.	Natural environement	13

4.6.2.	Operating environment	13

4.7.	Verification requirements	13

# Scope of the document

This specification defines the requirements applied to the performances, the design and the qualification of the SPIRE mirrors. It is applicable to the PFM and the FS.

SPIRE (Spectral and Photometric Imaging REceiver) is one of the three instruments which will equip the Far Infrared Space Telescope (FIRST), an ESA mission planned to be launched in 2007. It will provide astronomical images in the 200-670 µm band.

# Documents

## Applicable documents

|  | Title | Author | Reference | Date |
| --- | --- | --- | --- | --- |
| AD1 | Instrument Requirements Document | B.M.Swinyard | SPIRE-RAL-PRJ-000034 Issue 0.21 | 30 nov 1999 |
| AD2 | Structure/Optics Interface |  | SPIRE 1.1/1.2 |  |
| AD3 | SMEC / Optics Interface |  | SPIRE 1.2/1.5.2 |  |

## Glossary

| AD | Applicable Document |  |  |
| --- | --- | --- | --- |
| BSM | Beam Steering Mirror | LAM | Laboratoire d'Astrophysique de Marseille |
| BSMm | BSM cryogenic mechanism |  |  |
| CEA | Commissariat à l'Energie Atomique | MGSE | Mechanical Ground Support Equipment |
| CDR | Critical Design Review | MM | Mechanical Model |
| CNES | Centre National des Etudes Spatiales | MSSL | Mullard Space Science Laboratory |
| CoG | Center of Gravity | NA | Not Applicable |
| CQM | Cryogenic Qualification Model | OGSE | Optical Ground Support Equipment |
| DDR | Detailed Design Review | PDR | Preliminary Design Review |
|  |  | PFM | Prototype Flight Model |
| DM | Development Model | RAL | Rutherford Appleton Laboratory |
|  |  | RD | Reference Document |
|  |  | SMEC | Spectrometer mirror MEChanism |
|  |  | SMECm | SMEC cryogenic mechanism |
|  |  | SMECp | SMEC cold preamplifier |
|  |  | SPIRE | Spectral and Photometric Imaging REceiver |
| FIRST | Far InfraRed Submillimeter Telescope | TBC | To Be Confirmed |
| FPU | Focal Plane Unit | TBD | To Be Defined |
| FS | Flight Spare model | TBU | To Be Updated |
| FTS | Fourier Transform Spectrometer | TBW | To Be Written |
| GSFC | Goddard Space and Flight Center | WE | Warm Electronics |

# The mirrors

## Instrument description

Figure 1 shows a schematic overview of the SPIRE instrument. It consists of two channels, Photometer (upper) and Spectrometer (lower), located one on either side of a central panel. Mirrors CM3, CM4 and CM5 are common. Mirrors PM6 and SM6 are field mirrors picking off a part of the beam for each channel.

Photometer channel.

Spectrometer channel.

Figure 1. Schematic layout and nomenclature for the SPIRE instrument.

## Mirrors description

The basic design of the mirrors is the same as the one used for ISO-LWS.

The mirrors are made in Aluminium 6061.

They all have a standard interface with the structure, i.e. an M8 screw and a pin.

Each mirror is machined in a single block of aluminium (diamond cutting). The screw part of the attachment exerts pressure only on the shoulder part of the mirror, avoiding deformation of the optical surface.

The mount of each mirror is located on the optical bench by means of a pin.

This pin ensures that in case of dismounting of the mirror it will be mounted again in the same position.

During integration of the mirrors in the SPIRE structure, the mirrors are mounted on brackets which are provided by MSSL.

The figure below gives a drawing of the CM3 mirror, which is the more complex.

Figure 2.

The interface between the corner cube mirrors and the SMEC mechanism is TBD.

## Mission profile

Here are the successive phases of the subsystem life from the end of manufacturing to the end of life.

These are for information only. Discrepancies with actual AIV and operation plan are allowed.

Durations are TBC.

| Operation | Where | Duration | Note |
| --- | --- | --- | --- |
| Bakeout | LAM | 1 week |  |
| Control | LAM | 4 weeks |  |
| Warm Vibrations | LAM | 1 week | On the CM5 mirror only |
| Thermal cycles | LAM | 4 weeks |  |
| Control | LAM | 4 weeks |  |
| Transport | From LAM to RAL | 2 days |  |
| Integration in the SPIRE Structure | RAL | TBD |  |
| Bakeout | RAL | 1 week |  |
| Warm vibrations | RAL | 1 week |  |
| Transport | From RAL to ? | TBD |  |
| Cold vibrations | ? | 3+ weeks |  |
| Transport | From ? to RAL | TBD |  |
| Thermal cycles | RAL | TBD |  |
| Calibrations | RAL | TBD |  |
| Transport | From RAL to ESA | TBD |  |
| Satellite tests | ESA | TBD |  |
| Storage | ESA | 2 years (TBC) |  |
| Launch | Kourou | TBD |  |
| Beginning of operation | Orbit | TBD |  |
| Operations | Orbit | 4.25 years |  |
| End of operations | Orbit | TBD |  |

## Product tree

Each mirror is made of three parts : the mirror, a dowell pin and a nut.

The corner cubes are not yet defined.

Two complete sets of mirrors are required.

# Requirements

## Functional requirements

### Performance requirements

The overall instrument performance requirements are listed in [AD1] and have been translated into the following requirements for the mirrors.

| - # | - Parameter | - Value | - IRD | - Note |
| --- | --- | --- | --- | --- |
| - P1 | - Infrared relflectivity | - >0.99 |  |  |
| - P2 | - Infrared emissivity | - <0.01 |  |  |
| - P3 | - Visible reflectivity | - >0.8 (TBC) |  | - For vis. alignment purpose |
| - P4 | - Surface roughness | - < 10nm RMS |  | - For vis. alignment purpose |
| - P5 | - Surface shape | - < 1 µm RMS (TBC) |  | - Corresponds to 2 µm on the reflected wave, i.e., /100 at 200µm |
| - P6 | - Radius of curvature | - R/R < 10-3 |  | - For non-flat surfaces |
| - P7 |  |  |  |  |
| - P8 |  |  |  |  |
| - P9 |  |  |  |  |
| - P10 |  |  |  |  |

### Technical requirements

| # | Parameter | Value | IRD | Note |
| --- | --- | --- | --- | --- |
| Tm1 | Axis definition | TBD |  |  |
| Tm2 | Alignment tolerances | 0.5arcmin, 0.05mm |  |  |
| Tm3 | Dimensions |  |  | See table 1 below |
| Tm4 | Center of gravity | TBD |  |  |
| Tm5 | Mass | 2.5 kg including 20% margin |  | See table 1 below |
| Tm6 | Inertia | TBD |  |  |

### Mirror parameters

All the data given in Table 1 are generated from the optical design model. Current baseline designs are BOLPHT155 (photometer) and BOLSP501F (spectrometer). The dimensions given are valid at operating conditions (4K). A scaling factor of 1.00415 (TBC) allows calculating room temperature values. No factor must be applied to the conic constant, CC, which is dimensionless.

The mirrors are defined with respect to a co-ordinate system local to each mirror whose z-axis is perpendicular to the optical surface at the summit of the surface, not necessarily coinciding with the centre of the mirror.

Positive radius of curvature signifies concave mirror. For toric mirrors, Ry is the radius of curvature in the y-z plane and Rx is the radius of curvature in the x-z plane.

Dimensions of the mirrors are given for the projection of the mirror aperture onto the local x-y plane. The differences with respect to actual mirror dimensions are particularly important for CM3. The following dimensions are given:

Rectangular mirrors (rect):	a and b are sides along x and y axes, respectively

Elliptical mirrors (ellipt):		a and b are diameters along x and y axes, respectively

Circular mirrors (circle):		dia is diameter

Decentering data are projected onto the local x-y plane. One identifies three points in this plane, see Figure 3:

The surface summit, origin of the local co-ordinate system (0,0)

The projection of the spigot intersection point between mirror surface and spigot axis (xs,ys)

The centre of the projected mirror aperture (c,d)

Mirror thickness at the spigot is 15 mm (TBC) at room temp (TBC) for CM3, CM5, PM7, PM9. All the other mirrors have centre thickness 7 mm (TBC) at room temp (TBC).

Calculated masses are TBC and exclude margin.

CM4 and CC faces are not part of the main mirror manufacturing contract.

Figure 3. Illustration of aperture dimensions and decentering.

Table 1 : Mirror parameters

| Subass'y | Mirror | Type | R or Ry (mm) | CC or Rx (mm) | Shape and size a x b or dia | Spigot intersection xs, ys | Aperture centre c, d | Nb. of parts per model | Mass (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fore optics | CM3 | Off-axis asphere | 365.963 | -0.5095 | Rect 139x62 | -19.5, 145 See fig. 4. | -19.5, 145 | 1 | 0.256 |
|  | CM4 (BSM) | Flat |  |  | Ellipt. 30x32 |  |  | 1 | 0.021 |
|  | CM5 | Toric | 294.638 | 278.418 | Rect 161x85 | 19.5, -1.5 | 19.5, -1.5 | 1 | 0.360 |
| Photometer | PM6 (Pick-off) | Toric | -307.49 | -359.42 | Rect 46x27 |  |  | 1 | 0.027 |
|  | PM7 | Sphere | 330.70 |  | Rect 118x101 |  | 0, -1.0 | 1 | 0.300 |
|  | PM8 | Sphere | -286.651 |  | Circle Ø 60 |  |  | 1 | 0.056 |
|  | PM9 | Sphere | 350.851 |  | Circle Ø 112 |  |  | 1 | 0.223 |
|  | PM10 (Fold mirror) | Flat |  |  | Rect 78x40 |  | 2.5, 0 | 1 | 0.065 |
|  | PM11 (Fold mirror) | Flat |  |  | Rect 56x53 |  | 0, -2.75 | 1 | 0.060 |
| Spectrometer | SM6 (Pick-off) | Toric | 523.79 | 269.92 | Ellipt 18x24 |  | 0, 1.0 | 1 | NB a corriger |
|  | SM7 (Fold mirror) | Flat |  |  | Rect 40x57 |  | 0, 4.0 | 1 | 0.044 |
|  | SM8A, B (Relay in) | Toric | 230.34 | 202.00 | Circle Ø 60 |  |  | 2 | 0.112 |
|  | SM9A, B (Collimator) | Sphere | 259.50 |  | See fig. 5. Ø50 |  |  | 2 | 0.074 |
|  | CC face1 | Flat |  |  | See fig. 6 40x60 |  |  | 2 | 0.044 |
|  | CC face2 | Flat |  |  | See fig. 6 28x68 |  |  | 2 | 0.070 |
|  | CC face3 | Flat |  |  | See fig. 6 28x68 |  |  | 2 | 0.070 |
|  | SM10A, B (Camera) | Sphere | 260.00 |  | See fig. 5 Ø60 |  |  | 2 | 0.112 |
|  | SM11A, B (Relay out) | Toric | 196.99 | 169.84 | Circle Ø 74 |  | 0, 1 | 2 | 0.194 |
|  | SM12A, B (Fold mirrors) | Flat |  |  | Ellipt 42x32 |  | -1, 0 | 2 | 0.050 |
| Total |  |  |  |  |  |  |  | 27 | 2.167 |

Figure 4. Mirror CM3, off-axis part of asphere. Values of a, b, c and d are given in Table 1. The symmetric substrate radius is calculated by: e=a2+c2+b2+d2. Its current value is 202 mm.

Figure 5. Mirrors SM9 and SM10. "Chopped-off" circular apertures.

| (a) | (b) |
| --- | --- |

Figure 6. Corner cube faces. (a): face 1, (b): faces 2 and 3.

## Operational requirements

### Relialibility

### Lifetime

| # | Parameter | Value | IRD | Note |
| --- | --- | --- | --- | --- |
| OL1 | Ground Storage lifetime | 2 years |  | A guess |
| OL2 | Ground Integrated lifetime | 4 years |  | About |
| OL3 | Ground operational lifetime | 1.5 years |  | 6 months for subsystem acceptance 6 months for SPIRE acceptance 6 months for FIRST acceptance Under 1g conditions |
| OL4 | On orbit operational lifetime | 4.25 years |  |  |

### Operating modes

Non applicable

### Telemetry

Non Applicable

### Telecommands

Non Applicable

## Interface requirements

The interfaces are defined in the relevant applicable documents.

| Part | Interface | With | Document |
| --- | --- | --- | --- |
| SMECm | Mechanical | SPIRE Structure | SPIRE 1.1/1.5.2 |
|  | Thermal | SPIRE Structure | SPIRE 1.1.1/1.5.2 |
|  | Optics | SPIRE Optics | SPIRE 1.2 / 1.5.2 |
| SMECp | Mechanical | SPIRE Structure | SPIRE 1.1/1.5.2 |
|  | Thermal | SPIRE Structure | SPIRE 1.1.1/1.5.2 |
| SMECe | Mechanical | DRCU | SPIRE 1.5.2 / 2.2 |
|  | Thermal | DRCU | SPIRE 1.5.2 / 2.2 |
|  | Electronic | DRCU | SPIRE 1.5.2 / 2.2 |
|  | Electronic | MCU | TBW |
| MCU | Mechanical | DRCU | SPIRE 1.5.2 / 2.2 |
|  | Thermal | DRCU | SPIRE 1.5.2 / 2.2 |
|  | Electronics | DRCU | SPIRE 1.5.2 / 2.2 |

## Design and manufacture requirements

### Design requirements

TBD

### Design rules

TBD

### Manufacture requirements

These are requirements on accessibility, dismountability, testability and manufacturing processes.

- TBD fluids to be forbidden during manufacture to avoid pollution.

- TO BE COMPLETED

## Logistic requirements

The subsystem wil be transported to and from RAL.

The containers will have to guarantee that:

- no shocks are greater than those defined for the launch.

- no pollution sneaks to the mirrors

- TO BE COMPLETED

## Environment requirements

These requirements describe the environement in which the mirrors will live.

### Natural environement

This is the description of the natural environment around the mirrors.

| # | Parameter | Value | Note |
| --- | --- | --- | --- |
| EN1 | Vacuum | Less than 10-4 Pa | During tests, launch and in operation |
| EN2 | Operating temperature | during system qualif and on orbit = 4K during subsystem qualification = 300K and 20K |  |
| EN3 | Storage and handling temperature | -20 to +30 °C | Overall, on ground |
|  | Humidity | Less than 45% | In clean room |
|  | Cleanliness | Class TBD | In clean room |
| EN4 | Radiations | Less than 3.5 kRAD | On orbit |

### Operating environment

This is the description of the environment imposed by the location of the subsystem in SPIRE and in FIRST.

| # | Parameter | Value | IRD | Note |
| --- | --- | --- | --- | --- |
| ON1 | Vibrations | TBD |  | At 4K |
| ON2 | Shocks | TBD |  | At 4K |
| ON3 | Microvibrations | TBD |  | NA |
| ON4 | Acoustic | NA |  | Launched under vacuum |

## Verification requirements

TO BE COMPLETED