# Test Control --- TOPE requirements matrix at May 4, 2001

## Test procedure handling

| Requ.-ID | Delivery | Description | Status | Comment |
| --- | --- | --- | --- | --- |
| TPE-TP-01 | SUB | It shall be possible to request any instrument specific command and to provide all its required parameters as defined in the MIB. |  |  |
| TPE-TP-02 | SUB | It shall be possible to request any command for external test equipment and environment control and to provide all its required parameters as defined in the MIB. |  |  |
| TPE-TP-03 | ILT | It shall be possible to use the actual value of a TM parameter within a test procedure. |  |  |
| TPE-TP-04 | ILT | It shall be possible to synchronize the whole EGSE set-up with a reference time via a special TM packet. |  |  |
| TPE-TP-05 | ILT | Test procedures may generate event packets. |  |  |
| TPE-TP-06 | ILT | It shall be possible to request to schedule any observation from FCSS. |  |  |
| TPE-TP-07 | ILT | A requested observation will return a command sequence, which shall be uplinked automatically. |  |  |
| TPE-TP-08 | ILT | It shall be possible within a few seconds to update the time stamps of the received command sequence with the actual time (considering the delta times as well). |  |  |
| TPE-TP-09 | SUB/ILT | It shall be possible to time-tag commands or observation requests with absolute times. |  |  |
| TPE-TP-10 | SUB | It shall be possible to generate and edit test procedures from within Test Control. |  |  |
| TPE-TP-11 | SUB | It shall be possible to store a TBD number (~10<n<~100) of test procedures within the TPE. |  |  |
| TPE-TP-12 | SUB | It shall be possible to select and execute test procedures from TPE and provide all user input eventually needed by these procedures. |  | User input via dialog windows and command line, but no formal test procedure parameters |
| TPE-TP-13 | SUB | It shall be possible to abort, stop and resume any running test procedure by manual interaction. | ? | Only in debug mode Not during modal dialogs |
| TPE-TP-14 | SUB | In case of errors the execution of the running test procedure shall be suspended. | No | What kind of errors? |
| TPE-TP-15 | ILT | It shall be possible to execute several test procedures in the background to allow implementation of autonomy procedures. |  |  |
| TPE-TP-16 | ILT | Background procedures may stop or abort ongoing test procedures. |  |  |
| TPE-TP-17 | ILT | Background procedures may generate warning messages to the operator. |  |  |
| TPE-TP-18 | SUB | A log of the test procedure execution shall be produced. |  |  |
| TPE-TP-19 | ILT | Each background test procedure shall have its own log. |  |  |
| TPE-TP-21 | SUB | The actual commands generated by the test procedure are logged, incl. the time at which they were sent. |  |  |
| TPE-TP-22 | SUB | It shall be possible to log comments entered by the operator. |  |  |
| TPE-TP-23 | SUB | It shall be possible to track an ongoing test procedure via a display indicating the position within the procedure and showing at least the recent past and near future steps of the procedure. Back and forward scrolling shall be possible. |  | Only in debug mode |
| TPE-TP-24 | SUB | At any moment the message window for the feedback from autonomy functions and test procedures shall be displayed. | ? | No background procedures, but up to three concurrently running test procedure plus another one in debug mode. |
| TPE-TP-25 | SUB | On user request it shall be possible to display the logging information originating from test procedures. |  | It is displayed all the time. |
| TPE-TP-26 | SUB | There shall be a separate window from where test procedures, autonomy procedures or command timelines/schedules can be selected, started, aborted, stopped and resumed. |  | Up to three test procedure environments can be started to start/edit/control test procedures. In addition there is a single debugging environment as well. |
| TPE-TP-27 | ILT | Both, the spacecraft reference time (= simulated test time local time) at 1 sec resolution and the actual local time are displayed to the Test Control user. |  |  |
| TPE-TP-28 | SUB | There shall be a separate window in which editing of autonomy and test procedures and logs are possible. Export and import activities from/to Test Control shall be carried out here as well. |  |  |
| TPE-TP-29 | SUB | There shall be a separate window in which export and import activities from/to Test Control are carried out. | Not yet |  |

## SCOS 2000 interface

| Requ.-ID | Delivery | Description | Status | Comment |
| --- | --- | --- | --- | --- |
| TPE-SC-01 | SUB | It shall be possible to issue command requests to the SCOS 2000 command dispatcher within test procedures. |  |  |
| TPE-SC-02 | ILT | It shall be possible to read actual TM parameter values from SCOS 2000. |  |  |
| TPE-SC-03 | ILT | It shall be possible to issue an event packet. |  |  |

## FCSS interface

| Requ.-ID | Delivery | Description | Status | Comment |
| --- | --- | --- | --- | --- |
| TPE-CS-01 | ILT | It shall be possible to request to schedule an observation. |  |  |
| TPE-CS-02 | ILT | It shall be possible to retrieve the resulting command sequence. |  |  |
| TPE-CS-03 | ILT | It shall be possible to report back the actual start time of the observation. |  |  |
| TPE-CS-04 | ILT | Test procedure logs shall be exported to the FCSS. |  |  |
| TPE-CS-05 | ILT | It shall be possible to import/export test procedures and associated files (e.g. calibration files) from/to FCSS. |  |  |

## MIB interface

| Requ.-ID | Delivery | Description | Status | Comment |
| --- | --- | --- | --- | --- |
| TPE-MB-01 | ILT | All instrument related commands can be imported from the MIB. |  |  |
| TPE-MB-02 | ILT | Test equipment and environment control commands can be imported from the MIB. |  |  |
| TPE-MB-03 | ILT | It shall be easy to specify commands defined in the MIB during test procedure editing. |  |  |
| TPE-MB-04 | ILT | It shall be easy to specify TM parameters defined in the MIB during test procedure editing. |  |  |
| TPE-MB-05 | ILT(V2) | Syntax and parameter range of instrument commands shall be checked as early as possible. |  |  |