# Introduction

This note takes the Naming Convention Specification (H-P-1-ASPI-SP-0141) for the Herschel/Planck satellites’ database and applies it to the SPIRE instrument to provide a recommended naming convention for the SPIRE Mission Implementation Base (MIB).

The SPIRE MIB contains Monitor and Command packets, Monitor and Command parameters plus the corresponding calibration curves and displays.

The following naming convention shall be used:

| Item | scos-name | Name | Convention |
| --- | --- | --- | --- |
| Command parameter | cpc_pname | SPinnppp | 5130 |
| Monitor parameter | pcf_name | SMinnppp | 5130 |
| Command packet | ccf_cname | SCinnppp | 4580 |
| Monitor packet | pid_spid | tttnnnnppp | 4380 |
| Command num calibration | cca_numbr | tttnnnrrr | 5370 |
| Command txt calibration | paf_numbr | tttnnnrrr | 5370 |
| Monitor num calibration | caf_numbr | tttnnnrrr | 5370 |
| Monitor txt calibration | txf_numbr | tttnnnrrr | 5370 |
| Polynomial calibration | mcf_numbr | tttnnnrrr | 5370 |
| Numerical display | and | SAinnppp | 6100 |
| Graphical display | grd | SGinnppp | 6110 |
| Scrolling display | srd | SLinnppp | 6130 |

Where:

- i is the System Element ID (see section 2.1)

- nn, nnn and nnnn are running numbers starting at zero and incrementing

- ttt is the System Element number (see section 2.1)

- rrr is the Real Element number (see section 2.3)

- ppp is the Position Identifier  (see section 2.2)

# Identifiers

Identifiers are defined in the Naming Convention Specification with SPIRE allocated particular numbers and characters for the various fields. This section defines how these are used in the SPIRE naming convention.

## System Elements

Each distinct type of subsystem in the instrument (called a System Element Type) is assigned a unique id and number:

| System Element Type | ID | Number |
| --- | --- | --- |
| OBS | B | 480 |
| DPU | P | 490 |
| DCU | D | 500 |
| MCU | M | 510 |
| SCU | S | 520 |

## Position Identifiers

Each model of the instrument (including a ‘theoretical’ model) is composed of a set of System Elements. This may include more than one System Element of any given type. Each distinct System Element for each instrument model is assigned a unique number. This unique number is also used as the Position Identifier.

|  | Model |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System Element | Theoretical | AVM | QM | FM | FS |
| OBS | 480 | 481 | 483 | 485 | 487 |
| DPU | 490 | 4911 4922 | 4931 4942 | 4951 4962 | 4971 4982 |
| DCU | 500 | 501 | 503 | 505 | 507 |
| MCU | 510 | 511 | 513 | 515 | 517 |
| SCU | 520 | 521 | 523 | 525 | 527 |

1. Valid for prime element

2. Valid for redundant element

## Real Element Numbers

These numbers identify individual configurations of each System Element (there may be more than one configuration if, for example, electronic boards have to be exchanged during testing). The table below gives the Real Element Number for the first configuration of the System Element. They should be incremented by 1 for each new configuration.

|  | Model |  |  |  |
| --- | --- | --- | --- | --- |
| System Element | AVM | QM | FM | FS |
| OBS | 100 | 300 | 500 | 700 |
| DPU | 1001 2002 | 3001 4002 | 5001 6002 | 7001 8002 |
| DCU | 100 | 300 | 500 | 700 |
| MCU | 100 | 300 | 500 | 700 |
| SCU | 100 | 300 | 500 | 700 |

1. Valid for prime element

2. Valid for redundant element