SIR-2

Near Infrared Spectrometer

EAICD

CHANDRAYAAN-1

SIR-2 to Planetary Archive Interface Control Document

C1-SIR-ICD-3004

30.10.09

Prepared by the SIR-2 Team

Distribution List

| Recipient | Organization |
| --- | --- |
| Dr. U. Mall | Max-Planck-Institut für Sonnensystemforschung D-37191 Katlenburg-Lindau, Germany |
| Dr. D. Annadurai | INDIAN SPACE RESEARCH ORGANISATION Bangalore – 560017 IN India |
| Dr. Patrick Martin | ESA ESTEC, SCI-AM Postbus 299 2200 AG Noordwijk NL Netherlands |
| Dr. Detlef Koschny | ESA ESTEC, SCI-AM Keplerlaan 1 2200 AG Noordwijk NL Netherlands |
| Dr. Bjoern Grieger | ESAC, SCI-OS P.O. Box 78 E-28691 Villaneuva de la Canada Madrid Spain |
| Dr. Andreas Nathues | Max-Planck-Institut für Sonnensystemforschung D-37191 Katlenburg-Lindau, Germany |
| Arne Dannenberg | Max-Planck-Institut für Sonnensystemforschung D-37191 Katlenburg-Lindau, Germany |
| B Gopala Krishna | Space Applications Centre (ISRO) Ahmedabad – 380 015 IN India |

Change Log

| Date | Sections Changed | Reasons for Change |
| --- | --- | --- |

Table of Contents

1 Introduction	8

1.1 Purpose and Scope	8

1.2 Contents	8

1.3  Intended Readership	8

1.4 Applicable Documents	8

1.5 Reference Documents	9

1.6 Acronyms and Abbreviations	10

1.7 Contact Names and Addresses	12

2 Description of the instrument	14

2.1 Scientific Objectives	16

3 Overview of Process and product generation	24

3.1 Pre-Flight Data Products	24

3.2 Instrument Calibrations	24

3.3 In-Flight Data Products	24

3.4 Software	24

3.5   Documentation	24

3.6 Derived and other Data Products	24

4 Archive Format and Content	25

4.1  Format and Conventions	25

4.1.1 Deliveries and Archive Volume Format	25

4.1.2   Data Set ID Formation	25

4.1.3 Data Directory Naming Convention	25

4.1.4 Filenaming Convention 	26

5 Standards Used in Data Product Generation	27

5.1 PDS Standards	27

5.2 Time Standards	27

5.2.1 START_TIME and STOP_TIME Formation	27

5.3 Reference Systems	27

5.4 Other Applicable Standards	28

6  Data Validation	28

7 PDS content for Long Term Archive	29

7.1 Volume Set	29

7.2 Data Set	29

7.3 Directories	29

7.4 Root Directory	29

7.5 Calibration Directory	30

7.6 Catalog Directory	30

7.7  Index Directory	31

7.8  Browse Directory and Browse Files	31

7.9 Geometry Directory	31

7.10 Software Directory	32

7.11 Gazetteer Directory	32

7.12 Label Directory	32

7.13 Document Directory	32

7.14 Extras Directory	32

7.15 Data Directory	32

8 Detailed Interface Specification	33

8.1 Data Product Design	33

8.1.1 L1b Data Label and Data Structure Design	33

8.1.2 SIR-2 Instrument Science Data: Data Product Design 	33

8.1.2.1 PDS Version ID	33

8.1.2.2 File Characteristics Data	34

8.1.2.3 Pointer to Data Objects	34

8.1.2.4 Identification Data Elements	35

8.1.2.5 Data and Time Related Information	36

8.1.2.5.1 Instrument Related Parameters	37

8.1.2.5.2 Data Objects Definition	37

8.1.2.5.3 SIR SC Table	38

8.1.2.5.4 Exposure Time	39

8.1.2.5.5 Real Exposure Time	40

8.1.2.5.6 Spectrum	40

Data Quality ID	41

8.1.2.5.7 Longitude	41

8.1.2.5.8 Latitude	42

8.1.2.5.9 Incidence Angle	42

8.1.2.5.10  Phase Angle	43

8.1.2.5.11  Emission Angle	43

8.1.2.5.12  Local Hour Angle	44

8.1.2.5.13  Sub Spacecraft Longitude	44

8.1.2.5.14  Sub Spacecraft Latitude	45

8.1.2.5.15  Spacecraft Altitude	45

8.1.2.5.16  Sittime	46

8.1.2.5.17  MMODE	47

8.1.2.5.18  AVERAGE	48

8.1.2.5.19  ADCDEL	48

8.1.2.5.20  READDIV	49

8.1.2.5.21  NSPEC	49

8.1.2.5.22  PXPER	50

8.1.2.5.23  NSMPL	50

8.1.2.5.24  CCSDS Counter	51

8.1.2.5.25  HK Clock	51

8.1.2.5.26  HK Clock OBT Synchronization	52

8.1.2.5.27  OBT Synchronization	52

8.1.2.5.28  TEC	53

8.1.2.5.29  TEC Volt	53

8.1.2.5.30  TEC CURR	54

8.1.2.5.31  CMON5A	54

8.1.2.5.32  CMON1D	55

8.1.2.5.33 CMON3D	55

8.1.2.5.34  CMON5D	56

8.1.2.5.35  VMON5A	56

8.1.2.5.36  V14 Volt	57

8.1.2.5.37  VMON3D	57

8.1.2.5.38  ISU PDA TEMP	58

8.1.2.5.39  ISU PCB TEMP	58

8.1.2.5.40  PSU PCB TEMP	59

8.1.2.5.41  COOL TEMP	59

8.1.2.5.42  QUA TEMP	60

8.1.2.5.43   VMON5D	60

8.1.2.5.44  VMON1D	61

8.1.2.5.45  ERRORID	61

8.1.3 SIR-2 Instrument Housekeeping Data: Data Product Design	62

8.1.3.1 PDS Version ID 	62

8.1.3.2 File Characteristics Data Elements	62

8.1.3.3 Pointer to Data Objects	62

8.1.3.4 Identification Data Elements	63

8.1.3.5 Data and Time Related Information	64

8.1.3.6 Instrument Related Parameters	65

8.1.3.7 Data Objects Definition	65

8.1.3.7.1 SIR2 HK Table	66

8.1.3.7.2 UTC Time	66

8.1.3.7.3 SCET Time	67

8.1.3.7.4 HK Clock	67

8.1.3.7.5 HK Clock OBT Synchronization	68

8.1.3.7.6 OBT Synchronization	68

8.1.3.7.7 TEC	69

8.1.3.7.8 TEC Volt	69

8.1.3.7.9 TEC CURR	70

8.1.3.7.10  CMON5A	70

8.1.3.7.11  CMON1D	71

8.1.3.7.12  CMON3D	71

8.1.3.7.13  CMON5D	72

8.1.3.7.14  VMON5A	72

8.1.3.7.15  V14 Volt	73

8.1.3.7.16  VMON3D	73

8.1.3.7.17  ISU PDA TEMP	74

8.1.3.7.18  ISU PCB TEMP	74

8.1.3.7.19  PSU PCB TEMP	75

8.1.3.7.20  COOL TEMP	75

8.1.3.7.21  QUA TEMP	76

8.1.3.7.22  VMON5D	76

8.1.3.7.23  VMON1D	77

8.1.3.7.24  CCSDS Counter	77

8.1.3.7.25  ERRORID	78

8.2 Sample Labels	79

8.2.1 PDS version	79

8.2.2 File Characteristics and data elements	79

8.2.3 Identification Data Elements	79

8.2.4 Descriptive related parameters                                      	80

8.2.5 Data objects definition	82

9 Appendix: Available Software to read PDS files	98

10 Appendix: Directory Listing of Data Set CH1ORB-SIR2-2-NPO-V1.0	98

# Introduction

## Purpose and Scope

The purpose of this EAICD (Experimenter to (Science) Archive Interface Control Document) is to provide users of the SIR-2 instrument with detailed description of the product and a description of how it was generated, including data sources. Secondly, it is the official interface between the SIR-2 instrument team and the archiving authority.

## Contents

This document describes the data flow of the SIR-2 instrument on CHANDRAYAAN-1 from the s/c until the insertion into the Chandrayaan-1 Science Data Archive (CSDA). It includes information on how data were processed, formatted, labeled and uniquely identified. The document discusses general naming schemes for data volumes, data sets, data and label files. Standards used to generate the product are explained. Software that may be used to access the product is explained further on.

The design of the data set structure and the data product is given. Examples of these are given in the appendix.

## Intended Readership

The staff of the archiving authority (Chandrayaan-1 Science Data Archive, CSDA) and any potential user of the SIR-2 data.

## Applicable Documents

|  | Name | Reference |
| --- | --- | --- |
| AD1 | Planetary Data System Preparation Workbook Part 1 | JPL-D-7669/3.1, 01 Feb 1995 |
| AD2 | Planetary Data System Standards Reference | JPL-D-7669/3.5, 30 Oct 2002 |
| AD3 | CHANDRAYAAN-1 Archive Plan | CH1-SAC-PL-001, Issue 1, Rev.a, 31 Dec 2007 |
| AD4 | CHANDRAYAAN-1 Archive Conventions | CH1-SAC-PL-002, Issue 1, Rev. a, 31 Dec 2007 |

## Reference Documents

|  | Name | Reference |
| --- | --- | --- |
| RD1 | Navigation and Ancillary Information Facility (NAIF), | http://pds-naif.jpl.nasa.gov |
| RD2 | SIR-2 PFM User Manual | C1-SIR-MA-3001 2.3, 26 Sep 2007 |
| RD3 |  |  |
| RD4 |  |  |

## Acronyms and Abbreviations

| A/D | Analog Digital Converter |
| --- | --- |
| ADC | Analog Digital Converter |
| AGND | Analog Ground |
| Anti-SS | Anti Sun Side |
| APID | Application Process Identifier |
| ASS | Anti Sun Side |
| BC | Bus Controller |
| BDH | Base-band Data Handling |
| BMU | Bus Management Unit |
| C-1 | Chandrayaan-1 |
| CAD | Computer Aided Design |
| CCSDS | Consultative Committee for Space Data Systems |
| CDR | Critical Design Review |
| CMD | Command |
| CMDAD | Command Address |
| CMOS | Complementary Metal Oxide Semiconductor |
| CoG | Center of Gravity |
| CPU | Central Processor Unit |
| CS | Checksum |
| CSDA | Chandrayaan-1 Science Data Archive |
| CSR | Clementine Spectral Reflectance |
| DC | Direct Current |
| DC/DC | DC to DC Converter |
| DGND | Digital Ground |
| DH | Data Handling |
| EAICD | Experimenter to (Science) Archive Interface Control Document |
| E-Box | Electronic Box |
| EEM | Electrical Engineering Model |
| EEPROM | Electrical Erasable PROM |
| EGSE | Electrical Ground Support Equipment |
| EMC | Electromagnetic Compatibility |
| EMI | Electromagnetic Interference |
| ESA | European Space Agency |
| ESD | Electro Static Discharge |
| ETO | Earth Transfer Orbit |
| FoV | Field of View |
| FPGA | Field Programmable Gate Array |
| FS | flight spare |
| FS | Flight Spare |
| GTO | Geostationary Transfer Orbit |
| H/W | Hardware |
| HK | Housekeeping |
| HKDP | Housekeeping Data Package |
| HySI | Hyper Spectral Imager |
| I/F | Interface |
| I/O | Input / Output |
| ICD | Interface Control Document |
| ICU | Instrument Control Unit |
| IEH | Instrument External Harness |
| IHDM | Instrument Health Data Message |
| ILH | Instrument Local Harness |
| InGaAs | Indium Gallium Arsenide |
| INP | Input Power Supply |
| IP-core | Intellectual Property Core |
| IR | Infrared |
| ISO | International Standard Organization |
| ISR | interrupt service routine |
| ISRO | Indian Space Research Organization |
| ISSDC | Indian Space Science Data Centre |
| ISU | Instrument Sensor Unit |
| LOI | Lunar Insertion Maneuver |
| LTT | Lunar Transfer Orbit |
| LVDS | Low Voltage Differential Signal |
| M3 | Moon Mineralogy Mapper |
| M4 | metric screw thread 4mm |
| MD | Mass Dummy |
| MDDP | Memory Dump Data Package |
| MICD | Mechanical Interface Control Document |
| MIL-Std | Military Standard |
| MLI | Multi Layer Insulation |
| MoI | Moment of Inertia |
| MOSFET | Metal Oxide Semiconductor Field Effect Transistor |
| MPS | Max-Planck-Institute for Solar System Research |
| MUX | Multiplexer |
| NA | Not applicable |
| NIR | Near Infrared |
| O-Box | Optics Box |
| OBT | Onboard Time |
| OGSE | Optical Ground Support Equipment |
| OPAMPS | Operation Amplifiers |
| OSIRIS | Optical, Spectroscopic, and Infrared Remote Imaging System |
| PCB | Printed Circuit Board |
| PDA | Photo Diode Array |
| PDS | Planetary Data System |
| PFM | Proto Flight Model |
| PID | Proportional plus integral plus derivative controller |
| POC | Payload Operations Centre |
| PREP | Preparation Mode of SIR-2 |
| PROM | Programmable Read Only Memory |
| PSU | Power Supply Unit |
| RAM | Random Access Memory |
| ROM | Read Only Memory |
| RT | Remote Terminal |
| RTN | RETURN |
| RX | Receiver |
| S/C | Spacecraft |
| S/N | Signal to Noise Ratio |
| SCDP | Science Data Package |
| SGRP | Spacecraft Ground Reference Point |
| SH | Sensor-Head |
| SID | Sub Identifier |
| SIR | SMART-1 Infrared Spectrometer |
| SIR-2 | Spectrometer Infrared - 2 |
| SIR2CMD | SIR-2 Command |
| SIS | Spacecraft Simulator |
| SMART-1 | Small Missions for Advanced Research in Technology |
| SNR | Signal to Noise Ratio |
| SRES | Soft Reset |
| SSDR | Solid State Data Recorder |
| STDBY | Standby Mode of SIR-2 |
| STM | Structure Model |
| SUB-ID | Sub Identifier |
| TBC | To be confirmed |
| TBD | To be defined |
| TEC | Thermo Electrical Cooler |
| TM | Telemetry |
| TMC | Terrain Mapping Camera |
| TMM | Thermal Mathematical Model |
| TV | Thermal Vacuum |
| TX | Transmitter |
| TX/RXA | Transmitter/Receiver A |
| TX/RXB | Transmitter/Receiver B |
| UoB | University of Bergen |
| UV | Ultraviolet |
| UVVIS | Ultraviolet and Visible light |
| Vdd | Power Supply Voltage |
| VIS | Visible light |
| Vref | Reference Voltage |
| WD | digital 16-Bit Word |
| WDGEN | Watchdog enable |

## Contact Names and Addresses

U. Mall (PI), Max-Planck-Institut für Sonnensystemforschung, D-37191 Katlenburg-Lindau, Germany, Tel: [49]5556-979-152  Fax: [49]5556-9796152 E-mail: mall@linmpi.mpg.de

A. Nathues (CoPI), Max-Planck-Institut für Sonnensystemforschung, D-37191 Katlenburg-Lindau, Germany, Tel: [49]5556-979433, Fax: [49]5556-979-139, E-mail: nathues@linmpi.mpae.gwdg.de

A. Dannenberg (SIR-2 Operations-Manager), Max-Planck-Institut für Sonnensystemforschung, D-37191 Katlenburg-Lindau, Germany, Tel: [49]5556-979-359, Fax: [49]-5556-979-240, E-mail: dannenberg@linmpi.mpae.gwdg.de

# Description of the instrument

SIR-2 is a highly compact, monolithic, grating, near-infrared point spectrometer, covering the wavelength range between 0.9 and 2.4 µm, with a spectral resolution of ∆ Pixel = 6 nm. Key parameters of SIR-2 are listed in Table 1.

Table 1 Characteristics of SIR-2.

| Type of Instrument | Grating NIR Point Spectrometer |
| --- | --- |
| Wavelength Range | 0.9 – 2.4 m |
| Spectral Resolution | Pixel = 6 nm |
| S/N | >100 |
| Dynamic (ADC) | 16 bit |
| Angular Resolution / field of view | 2.2 mrad |
| f-ratio of front-end optics | 2.5 |
| Aperture (main mirror diameter) | 72 mm |
| Focal length | 180 mm |
| Exposure times | Selectable between 0.1 and 1000 ms |
| Power consumption | Max. 11W |
| Total mass of instrument | 3 kg |
| Dimension of O-Box | 199 x 161 x 93 mm |
| Dimension of Sensor-Head/Radiator Unit | 228 x 320 x 249 mm |
| Dimension E-Box | 146 x 125 x 58mm |

SIR-2 collects the Sun’s light reflected by the Moon with the help of the optical box (O-Box) which houses a main and secondary mirror. This light then enters an optical fiber, which transmits the light to the Sensor-Head/Radiator Unit. Inside the Sensor-Head the light passes filter, slit and the quartz body and reaches the dispersing grating. The dispersed light then passes the quartz body again to reach a second order filter, which is glued onto the window. After passing this window the light is detected by the photosensitive pixels. The electrons released by the individual pixels are collected and a sequential read out is performed after the integration time has ended. The adjacent voltage will be measured for each pixel and the obtained values are converted to digits. The obtained values (counts) are finally embedded in the TM packages in order to send them to the spacecraft’s mass memory.

Figure 1: Functional principal of SIR-2

The near-infrared spectrometer SIR-2 consists of three individual units:

- O-Box (front-end optics), located on an extension of the Anti-SS panel

- Sensor-Head/Radiator Unit (spectrometer), located on the Anti-SS panel

- E-Box (digital electronics and power converter), located inside the S/C cube on the Anti-SS panel

Figure 2: SIR-2 units: O-Box (right), Sensor-Head/Radiator Unit (left) and E-Box.

These units are linked by:

An optical fiber connecting O-Box and Sensor-Head/Radiator Unit

An electrical harness (ILH) connecting Sensor-Head/Radiator Unit and E-Box

SIR-2 uses a linear InGaAs photodiode array which is sensitive between 0.9 and 2.55 m. It has 256 pixels with a pixel pitch of 50 m. Three voltages are required to operate the detector (Vdd = 5V, Vref = 1.26V and INP = 4.5V). The detector can be clocked with frequencies up to 4 MHz. For achieving sufficient signal-to-noise ratios a constant detector operating temperature of -50°C during noon-midnight orbit and else -60C is anticipated. A two-stage cooling system consisting of the detector’s thermoelectric cooler (TEC) and a passive radiation system is implemented. The detector is located inside the Sensor-Head. The support mechanics (Cooling Console) of the detector is thermally decoupled from the Sensor-Head housing and connected via the Cooling-Band with the Sensor-Head Radiator which radiates energy into space to ensure suitable temperatures. The stability of the detector temperature will be obtained by a control algorithm for the detector’s thermoelectric cooler.

For further details the reader is referred to [RD2].

## Scientific Objectives

The Moon is unique in terms of its origin, size and density. It is, with the exception of the Earth, the only planetary body from which we have samples from known locations. Laboratory studies of lunar samples brought back by the APOLLO and LUNA missions, as well as remote sensing of the Moon, particularly by the CLEMENTINE and LUNAR PROSPECTOR missions, have provided a considerable amount of data which gave us insight into the processes responsible for its formation and subsequent chemical and geological evolution. A synthesis of these results shows that most likely the impact of a giant, differentiated planetesimal on the proto-Earth, followed by the accumulation of the impactor’s crustal material and the impact generated terrestrial ejecta, thrown in a low circumterrestrial orbit, led to the formation of the Moon.

As the Earth’s next body, the Moon allows a view into the early history of the solar system. Traces of the past are wiped out on Earth, since the Earth is a geologically very active planet whose surface is subject to continuous change through plate tectonics and a multitude of other erosive processes. In direct contrast to the Earth is the Moon, which is structured in a much simpler way. Processes of erosions are restricted to impacts (from dust to asteroid sizes). In the planetary context the Moon provides an excellent record of primordial crust formation, and complete records of the cratering history and the formation and evolution of its secondary crust.

It has been recognized early that the ancient impact environments on the Moon must also have impacted the Earth and that an accurate understanding of the early history of the Moon is also essential for a realistic understanding of the impact environment on the Earth which is so important in finding the constraints of both time and place in which life originated.

Despite all our efforts on a wide front of research, major questions still await to be resolved. Among those are: What is the cause of the global asymmetry of the Moon, which on the far side has a thicker crust and lacks the Mare structures which are characteristic for the near side? Is it related to convection and density inversion dynamics, early giant impacts, asymmetric crystallization of the Magma Ocean, or Earth-Moon tidal effects? How does a magma ocean work? What was the early thermal evolution of the Moon? What is the vertical and lateral structure of the lunar crust and how did it develop? What is the composition and structure of the lunar mantle? What role did early volcanism play? What were the timing and the effects of major basin-forming events on the lunar stratigraphy? What is the nature of the South Pole-Aitken basin, does it penetrate into the mantle, and how did it affect early crustal evolution? Why is the Moon different from other planets and how do planets work in terms of surface processes, heat transfer, and geologic evolution? Are the Apollo geophysical measurements representative of the Moon, or are they only valid for the small regions around the Apollo landing sites? How does the formation and evolution of the Moon and its well preserved record of impact modification and solar activity relate to the Earth’s ancient history?

The lunar surface, not affected by an atmosphere, water or life, presents a unique record of solar system activities in the direct vicinity of the Earth. To decipher this information one has to know and understand the building blocks of the lunar surface. Lunar rocks, sampled by six Apollo and three Luna missions, together with meteorites from the Moon, are our only direct information about the Moon’s minerals, chemical composition and age. Lunar surface material is usually classified into four different groups: i) basaltic volcanic rocks, ii) pristine rocks from the lunar highlands, iii) complex polymict breccias and iv) lunar soil. While the majority of these rocks were originally igneous (formed by cooling of molten material), most consist of only a few minerals: pyroxenes, olivines, plagioclase feldspar and ilmenite. Quite surprisingly, our present knowledge of the surface chemistry and mineralogy is far from complete.

The Apollo Program was entirely focused on preparations for a manned landing and thus attention was mainly devoted to potential landing sites, in the equatorial regions of the near side. It is fair to say that the present distribution of nine sampling sites is heavily biased: none are on the far side, or at high latitudes, while all but two are in or close to mare. The differences between lunar maria and highlands indicate large-scale chemical differentiation of the original materials that accreted to form the Moon. While this fact is commonly accepted the details of this differentiation are still not known. Early recognition of the fact that the highlands are composed mostly of the Ca-rich plagioclase, a relatively light mineral, led to the suggestion that this mineral represents crystal floating at the top of a very deep magma ocean (Wood et al., 1970). While this idea has survived in several modified forms it is not clear whether or not the concept of a lunar crust evolving from extensive magmatism that is cumulatively concentrated plagioclase in the upper part of the crust (Walker, 1983) is also feasible. More than any other question about the Moon, our understanding of the evolution of the lunar crust is critically dependent on the completeness of available compositional data and is most vulnerable to the temptations of extrapolating from limited information. At this point global coverage of surface composition data can yield the decisive answer to understand the formation and evolution of the lunar crust and the processes that modify it.

The determination of the chemical composition of a planet’s crust and mantle is among the foremost important goals of planetary research. UV, VIS, and NIR (e.g., Hapke, 1993), as well as X-Ray (e.g., Yin et al., 1993) and Gamma-Ray (e.g., Evans et al., 1993) spectroscopy, provide remote sensing methods to characterize the mineralogical and elemental composition of planetary surfaces. For more than a century (reviewed in Minnaert, 1961) it has been known that information on the physical characteristics of a planetary crust is contained in the manner in which light interacts with a surface. The photometric parameters of the surface provide information on the optical properties of constituent particles and the small-scale texture, as well as on the degree of large-scale roughness. Analysis of the absorption bands reveals mineralogy and composition.

Spectral features of most rock-forming minerals in the visible and near-infrared range originate from the locations of transition elements in the crystal structure of the minerals. Spectral profiles of sunlight reflected from planetary surfaces, when correlated with measured optical spectra of rock-forming minerals, may be used to detect the presence of transition metal ions and constituent minerals, as well as to determine model mineralogy of regoliths on terrestrial planets (Burns, 1993).

Among the various spectral approaches, near-infrared spectroscopy has proved to be a powerful tool in the study of the surfaces of the planets and of the small bodies in the solar system. Diagnostic absorption bands of various minerals and ices, that are expected to be found on the surfaces of planetary bodies, are located in the near-IR range, between 0.7 and 3.0 μm. As an example Figure 3 shows the near-IR laboratory reflectance spectra of pure minerals that were identified on the lunar surface (Singer, 1981).

Figure 3 Reflectance spectra of pure minerals (Singer, 1981).

Most of the associated mineral absorption bands are very broad when compared with stellar bands. Their depths can be measured with a low resolution spectrometer or even with a spectro-photometer that uses filters targeted at particular absorption bands. However, some of the mineral subtypes have weaker and narrower spectral features which can be studied only by moderate resolution spectroscopy. Good examples are the feldspar feature at ~1.3 μm and a set of sharp bands typical for clay minerals and carbonates between 1.3 μm and 3 μm (Gaffey et al., 1993).

shows the spectral changes due to variations of the olivine-to-orthopyroxene ratio in a sample (Singer, 1981). The shown figures clarify the importance of the NIR spectral region to analyze the mineral composition of solid surfaces. The depth of the absorption features depends mainly on the mineral composition and grain size. Moreover, in the surfaces of the solar system bodies the minerals are mostly present in mixtures. This results in the deformation of the shape and a shift of the minimum of the absorption features. The detailed quantitative study of the surface composition usually requires spectral measurements with moderate spectral resolution.

Figure 4 Reflectance spectra of weight percentage mixtures of olivine and orthopyroxene (Singer, 1981).

Ices of different volatiles are often present

on the surface of the planets and small

bodies. The NIR spectral range also contains

the spectral features of the ices and frosts.

shows the laboratory spectra of the various ices which are most frequently met in the solar system (Gaffey et al., 1993). The possibility of finding deposits of hydrogen at the poles of the Moon in the form of frozen water ice has been discussed since about 1960 (e.g., Watson et al., 1961; Vasavada et al., 1999). The initial analysis of the Clementine bistatic radar experiment seemed to indicate a positive detection of rather pure water ice near the south pole of the Moon (Nozette et al., 1996). However, a more detailed analysis of the same data showed that the initial conclusion was ambiguous (Simpson and Tyler, 1999).

Figure 5 Reflectance spectra of typical frosts (Gaffey et al., 1993)

The neutron spectrometer measurements made by the Lunar Prospector mission have shown that surface materials at the lunar poles contain increased contents of hydrogen (Feldman et al., 2000). In the permanently shadowed craters near the south pole, the abundance of hydrogen is estimated to be 1700  900 ppm by weight, which corresponds to 1.5  0.8 wt.% of water ice. At the north pole the content of water ice may be even larger (>10 wt.%) (Feldman et al., 2001). These results, however, need clarification by future studies. The footprint of the Lunar Prospector neutron spectrometer measurements is ~60 km in diameter and it is therefore difficult to correlate specific geological features with the identified locations of hydrogen-bearing deposits. While Lunar Prospector could not answer the question as to whether the hydrogen-bearing material is located on the surface or close to it, SIR-2, with its ability to directly observe the surface, has all the potential to contribute to this key question in lunar science. If this question can be solved with the new SIR-2 data, one may eventually understand the depositional history of this material. For example, McConnochie et al. (2002) suggested that if these deposits are buried and lack any surface expression, this may imply that the hydrogen has been delivered by large cometary impact events in the past or by a mechanism no longer operating. On the other hand, the existence of both surficial frost and buried deposits would suggest a steady supply of water and a continuous burial through regolith gardening. The effect of impact gardening on deposition of ice is considered by Crider and Vondrak (2003).

Despite Apollo’s and Luna’s collection of samples, the role of remote sensing of the lunar surface is gradually increasing. High-quality, near-IR spectra of the Moon were recorded using the 2.2-m telescope at Mauna Kea Observatory, Hawaii (Pieters, 1986). These spectra cover the range between 0.65 μm and 2.55 μm with a resolution of λ/Δλ ~70 and were recorded for selected regions with a size of ~5 km. Figure 6 presents the examples of the ground-based spectra (Smerkar and Pieters, 1985). These spectra show the diversity of the spectral properties, even within particular geological units. Note the variation of the shapes of the 1- and 2-μm absorption band. The weak band at ~1.3 μm belonging to feldspar is clearly seen in the bottom spectrum in the right figure. These moderate resolution spectra allow one to analyze composition of main geological units on the Moon. However, the analysis is limited by a coarse spatial resolution.

Figure 6 Representative reflectance spectra of the highland craters (left) and central peaks (right) after continuum removal (Smrekar and Pieters, 1985).

Significant steps forward in the lunar studies were made by the Clementine mission (Nozette et al., 1994). The spacecraft studied the Moon from the lunar polar orbit and provided a global mapping of the surface at spatial resolution of ~200 m in 11 narrow-band filters in the 0.4 - 2.6 μm range. The filter band-passes are shown in Figure 7 along with typical spectra of the lunar samples. The positions of the near-IR filters of the Clementine spectral imager were selected so as to observe the 1.0-μm and 2.0-μm absorption bands.

Figure 7 Positions of the filters of the Clementine cameras superimposed on the reflectance spectra of lunar samples (Nozette et al., 1994).

Figure 8 Clementine reflectance data for Aristarchus central peak and laboratory reflectance spectra (McEven et al., 1994).

The main results of the Clementine mission were:

- Global mapping of the Moon’s surface in several wavelength bands which are diagnostic for pyroxenes and olivine;

- Identification of geological units and their correlation with topography;

- Study of the composition variations between the units;

- Study of the structure and composition of the crater ejecta.

The Clementine low resolution reflectance data show spectral features typical for olivine and feldspar (Figure 7, McEven et al., 1994). However, their comparison to the ground-based spectra (Figure 6) shows that their poor spectral resolution does not allow them to describe the existing mixtures and subtypes of minerals observed on the surface of the Moon.

# Overview of Process and product generation

## Pre-Flight Data Products

No pre-flight data products are available

## Instrument Calibrations

The calibration of the SIR-2 FS(P)FM instrument was performed at MPS in April/May 2008 (see C1-SIR-TP-3011).

## In-Flight Data Products

The data products are spectra.  Each spectrum consists of 256 integers representing the measured intensities within small wavelength ranges. MPS will provide decoded raw data (level 0). This includes SIR-2 HK and telemetry information.

## Software

Standard procedures for IDL or MATLAB are sufficient to analyze the data. For reading PDS product see chapter 8.

## Documentation

The document directory will contain the SIR-2 User Manual and ICD. A document in PDF format is provided. It is not planned to convert the reports into ASCII format.

## Derived and other Data Products

No other data products are foreseen.

# Archive Format and Content

## Format and Conventions

### Deliveries and Archive Volume Format

### Data Set ID Formation

The dataset ID formation is referred to AD4 and will contain (raw, edited) data of the lunar mapping phase.

DATA_SET_ID=”<INSTRUMENT_HOST_ID>-<TARGET_ID>-<INSTRUMENT_ID>-<data_processing_level_number>-<mission_phase_abbreviation>-<description>-<version>”

<INSTRUMENT_HOST_ID>                                                                                   CH1ORB

<TARGET_ID>                                                                                                                      L

<INSTRUMENT_ID>                                                                                                      SIR-2

<data_processing_level_number>                        2 (for edited data) or 3 (for calibrated data)

<mission_phase_abbreviation>                                         NPO (for Normal Phase Operation)

<description>                                                    free character string containing only A-Z, 0-9 (optional)

<version>                                                                                                                          Vx.x

An example would be:

"CH1ORB-L-SIR2-2-NPO-V1.0"

### Data Directory Naming Convention

At the root level, the PDS required naming scheme will be followed: DATA DOCUMENT INDEX CALIB GEOMETRY CATALOG.

/DATA/ORBIT_<orbit number>_TO_<orbit number>/<data filename>

An example would be: "DATA/ORBIT_00200_TO_00299/"

### Filenaming Convention

| Identification | Description |
| --- | --- |
| CH1SIR2_ | Instrument, fixed to “CH1SIR2”, followed by underscore, “_” |
| CH1SIR2_N | <mission phase> denotes the mission phase: N = “Normal Phase Operations” (NPO) T = “Terminal Phase Operations” (TPO) |
| CH1SIR2_NE | <data type> denotes the data type: E = “Experiment Data Record” (EDR) |
| CH1SIR2_NE2_ | 2 for level 2 data |
| CH1SIR2_NE2_SC_ | SC for Science Data, HK for Housekeeping Data |
| CH1SIR2_NE2_SC_R02357 | <R00000> where R is mandatory and stands for “revolution”, 00000 is the consecutive orbit number. The number will be padded by leading zeros if required, e.g. ‘R00329’. |
| CH1SIR2_NE2_SC_R02357.LBL CH1SIR2_NE2_SC_R02357.FIT | LBL stands for PDS label files and FIT stands for FITS files |

# Standards Used in Data Product Generation

## PDS Standards

All keywords in the header follow the PDS Standards Reference Version 3.8 dated February 27, 2009.

## Time Standards

All time information in the data follows the SPICE time standards. Please, see [RD1] for details.

Within the data products themselves, the time standard used is ET (Ephemeris Time), which is a double precision number of seconds.  The starting point for this time is the J2000 epoch. This epoch is Greenwich noon on January 1, 2000 Barycentric Dynamical Time. This ephemeris time is calculated from the Spacecraft Onboard Time using the appropriate SPICE routines and the time correlation packages which are provided by ISRO as a SPICE Clock Kernel. Outside of the products themselves, there are a few instances in the SIR-2 data sets where time flags are provided.  The main time values are provided in the data product labels, which provide a start and stop time for the measurement, and a corresponding clock count from the spacecraft.  The standards used to define these values are described in 5.2.1.

### START_TIME and STOP_TIME Formation

The PDS formation rule for dates and time in UTC is:

YYYY-MM-DDThh:mm:ss.fff

YYYY           YYYY     	year (0000-9999)

MM               MM       	month (01-12)

DD                DD     		day of month (01-31)

T                   T       		date/time separator

hh                  hh      		hour (00-23)

mm                mm       	minute (00-59)

ss                   		second (00-59)

fff     	            	            fractions of second (000-999) (restricted to 3 digits)

This standard is followed for all START_TIME and STOP_TIME values in the products included in the SIR data sets.

## Reference Systems

The reference systems used for orbit, attitude, and target body follow the SPICE standards and are defined in the different SPICE kernels. Please, see [RD1] for details.

All latitudes and longitudes are given in degrees, assuming a spherical shape with a radius of 1737.4 km.

## Other Applicable Standards

No other standards are used

# Data Validation

Each of the individual orbit will be visualized by a SIR-2 team member and marked as ‘inspected’ before delivery.

The following checks will be performed on individual data product labels:

- the values of the keywords START_TIME and SPACECRAFT_CLOCK_START_COUNT will be compared against each other;

- the values of the keywords STOP_TIME and SPACECRAFT_CLOCK_STOP_TIME will be compared against each other;

- the value of the keyword TARGET_NAME will be compared with the target planned in the planning database;

The following checks will be performed on all spectral data:

- the values of the keyword CCSDS_COUNTER are in a continuous increasing order without any gaps (completeness of data).

- the spectral data showing typical values.

# PDS content for Long Term Archive

## Volume Set

The VOLDESC.CAT file in the root directory of each dataset must contain volume information as requested by the PDS standard. The information given in this object will be:

| Keyword | Value |
| --- | --- |
| VOLUME_NAME | “”CHANDRAYAAN-1 VOLUME 1 RAW SIR2 DATA |
| VOLUME_SERIES_NAME | “MISSION TO THE MOON” |
| VOLUME_SET_ID | “DE_MPG_MPS_CH1SIR2_1000” |
| VOLUMES | 2 |
| VOLUME_ID | “CH1SIR2_1001” |
| VOLUME_VERSION_ID | “VERSION 1” |
| PUBLICATION_DATA | 2009-11-04 |
| DATA_SET_ID | CH1ORB-L-SIR2-2-NPO-V1.0 |
| VOLUME_FORMAT | “ISO-9660” |
| MEDIUM_TYPE | “ELECTRONIC” |
| DESCRIPTION | “This volume contains one data set from the SIR-2 instrument flown on the Chandrayaan-1 spacecraft. It contains data from the normal phase operations.” |

## Data Set

For each mission phase and for each processing level, one data set is created.

## Directories

The files residing in the root directory of the data set are described in section 7.4.  The files residing in the subdirectories of the root directory are described in respective sections 7.5-7.15.

## Root Directory

The files listed below are contained in the root directory.. They are produced by the SIR-2 PI team with support from ESA/ISRO.

AAREADME.TXT

This file describes the volume (= data set) as a whole.  It gives an overview of the contents and organization of the data set, general instructions for its use and contact information.

VOLDESC.CAT

This file gives a high level description of the contents of the volume (= data set).

ERRATA.TXT

A cumulative listing of all known errors, omissions, and areas of non-conformance with PDS standards on this and all previous volumes (=data sets).

## Calibration Directory

The calibration directory contains the calibration tables (.TAB files) and associated labels (.LBL files) necessary to process the raw data (Level-0) to calibrated science units (Level-1).  These files are produced by the SIR-2 PI team with support from ESA/ISRO. Initially released data sets may not contain a CALIBRATION directory.

CALINFO.TXT

A description of the contents of the CALIB directory and the usage of the calibration tables in the directory.

## Catalog Directory

The files in the CATALOG directory provide a top-level understanding of the SIR-2 experiment and its data products.  ISRO provides the mission and spacecraft files.  The SIR-2 team provides information for the instrument and data set files, and the SIR-2 PI team with support from  ESA  formats  these  files  into PDS standard form. All catalog files are ASCII text files with <CR><LF> line  termination  and  line lengths are not longer than 78 characters.

CATINFO.TXT

A description of the contents of the CATALOG directory

MISSION.CAT

PDS mission catalog information about the Chandrayaan-1 mission will be provided by ISRO.

INSTHOST.CAT

PDS instrument host  catalog  information  about  the  Chandrayaan-1  spacecraft  and  the mounting relationship of the instruments within the spacecraft will be provided by  ISRO.

INST.CAT

PDS instrument catalog description of the SIR-2 instrument

DATASET.CAT

PDS data set catalog description of the respective processing level data set

REF.CAT

PDS reference catalog information about published references mentioned in the above catalog objects or their components.

SOFTWARE.CAT

PDS software catalog description  of  the  SIR-2  software included in the SOFTWARE directory

TARGET.CAT

PDS target catalog information about the observation target, i.e. star or Earth, will be provided by ISRO.

PERSON.CAT

PDS personnel catalog description of SIR-2 team members  and other persons involved with the generation of data products

## Index Directory

The following files are contained in the INDEX directory and are produced by the SIR-2 PI team with support from ESA/ISRO.  The INDEX.TAB and INDEX.LBL files are generated using the PSA PVV tool.

INDXINFO.TXT

A description of the contents of the INDEX directory

INDEX.TAB

A table listing the SIR-2 data products for the data set

INDEX.LBL

A PDS detached label that describes INDEX.TAB

GEO_MOON.TAB

A table of the geometry and position descriptions for the data products in the data set

GEO_MOON.LBL

A PDS detached label that describes GEOINDEX.TAB

## Browse Directory and Browse Files

SIR-2 data sets do not have browse products, thus a BROWSE directory does not exist.

## Geometry Directory

The geometry index table in the INDEX directory (cf. section 7.7) contains all the geometry data for the SIR-2 data sets.  The SPICE kernels used to compute the geometry data are provided as separate data set. Therefore there is no GEOMETRY directory.

## Software Directory

No software is provided, hence there is no SOFTWARE directory.

## Gazetteer Directory

There are no plans to include a GAZETTEER directory for SIR-2 data set deliveries.

## Label Directory

There are no referenced files to be included, thus there is no LABEL directory.

## Document Directory

The document directory contains this EAICD document in MS Word.

DOCINFO.TXT

A description of the contents of the DOCUMENT directory. See this file for a list of additional documents provided.

## Extras Directory

There is no EXTRAS directory in SIR-2 data sets.

## Data Directory

The DATA directory contains the actual data products for SIR-2.  There are two types of data products, science data files and housekeeping data files.  The former are provided as FITS files with accompanying detached PDS label files.  The latter are provided as ASCII files with accompanying detached PDS labels. The naming convention for these files is described in section 4.1.4.   Refer to section 8 for data product descriptions, data file contents and structures, and sample labels for each data set.

# Detailed Interface Specification

## Data Product Design

### L1b Data Label and Data Structure Design

This section describes the detailed specification of each of the raw/edited (L1b) products to be supplied to the PSA.

The SIR-2datasets will contain 2 different kind of data products:

SIR-2 Instrument Science Data

SIR-2 Instrument Housekeeping Data

Each of these data products is defined in one of the following subsections.

### SIR-2 Instrument Science Data: Data Product Design

The SIR-2 science structure is made up of records of length 2880. This is the length of a minimal FITS header, and each FITS header has to be a multiple of it. We have to pad out to a multiple of 2880 bytes at the end of each science data record.

#### PDS Version ID

| Keyword | C1-Value | Type | Description |
| --- | --- | --- | --- |
| PDS_VERSION_ID | PDS3 | ID |  |

#### File Characteristics Data

| /*** File Characteristics Data Elements ***/ |  |  | Comment |
| --- | --- | --- | --- |
| FILE_NAME |  | CHAR | See section 4.1.4 |
| RECORD_TYPE | FIXED_LENGTH | ID | binary records with fixed lengths |
| RECORD_BYTES | 2880 | INT | Output of data processor |
| FILE_RECORDS |  | INT | Number of records in the detached file |
| INTERCHANGE_FORMAT | BINARY | ID |  |
| RELEASE_ID | 0001 | INT |  |
| REVISION_ID | 0000 | INT |  |

#### Pointer to Data Objects

| /*** Pointer to Data Objects ***/ |  |  | Comment |
| --- | --- | --- | --- |
| ^SIR2_SC_HEADER | (“FN”) | CHAR | e.g. (“CH1SIR2_NE2_SC_R01971.FITS”,1<BYTES>) |
| ^SIR2_SC_TABLE | (“FN”, 14401 <BYTES>) | CHAR | e.g. (“CH1SIR2_NE2_SC_R01971.FITS”, 14401 <BYTES>)) |

#### Identification Data Elements

| /*** Identification Data Elements |  |  | Comment |
| --- | --- | --- | --- |
| DATA_SET_ID |  | ID | e.g. “CH1ORB-L-SIR2-2-DATA-V1.0” |
| DATA_SET_NAME |  | CHAR | e.g. “CHANDRAYAAN-1-ORBITER LUNAR SIR2 2 NPO V1.0” |
| PRODUCT_ID |  | CHAR | e.g. “CH1SIR2_NE2_SC_R01971” |
| PRODUCT_CREATION_TIME |  | TIME | In PDS time standard YYYY-MM-DDTHH:MM:SS |
| PRODUCT_TYPE | EDR | ID | SIR-2 Experiment Data Recored, Science Data. |
| PROCESSING_LEVEL_ID | 2 | INT | CODMAC level, 2 for raw/edited data and 3 for calibrated data |
| PROCESSING_LEVEL_DESC | “EXPERIMENT DATA RECORD” | CHAR |  |
| PRODUCER_ID | “SIR2 TEAM” | ID |  |
| PRODUCER_FULL_NAME | “DR. URS MALL”” | CHAR |  |
| PRODUCER_INSTITUTION_NAME | “MAX PLANCK INSTITUTE FOR SOLAR SYSTEM RESEARCH” |  |  |
| DATA_QUALITY_ID |  | INT |  |
| DATA_QUALITY_DESC | “THE DATA QUALITY ID FOR ALL SPECTRA IN THIS PRODUCT IS CALCULATED AS THE AVERAGE OF THE INDIVIDUAL QUALITY ID FOR EACH SPECTRUM ROUNDED TO AN INTEGER VALUE. (SEE THE DATA_QUALITY_ID CLUMN OBJECT ABOVE) | INT |  |
| MISSION_ID | CH1 | ID |  |
| MISSION_NAME | “CHANDRAYAAN-1” | CHAR |  |
| INSTRUMENT_HOST ID | CH1ORB | ID |  |
| INSTRUMENT_HOST_NAME | “CHANDRAYAAN-1-ORBITER” | CHAR |  |
| MISSION_PHASE_NAME | “NORMAL PHASE OPERATIONS” | ID | “NORMAL PHASE OPERATIONS” |
| TARGET_NAME | MOON | ID |  |
| TARGET_TYPE | SATELLITE | ID | SATELLITE, |

#### Data and Time Related Information

| /*** Date and Time Related Information ***/ |  | COMMENT |  |
| --- | --- | --- | --- |
| START_TIME |  | TIME | In PDS time standard |
| STOP_TIME |  | TIME | In PDS time standard |
| SPACECRAFT_CLOCK_START_COUNT |  | CHAR | e.g. “8/0034534.3321”. The value before the “/” is the partition. And the value after the “/” is the time elapsed after the start of the partition |
| SPACECRAFT_CLOCK_STOP_COUNT |  | CHAR |  |
| ORBIT_NUMBER |  | INT | An integer number starting from 1. |

##### Instrument Related Parameters

| /*** Instrument Related Parameters ***/ |  | COMMENT |  |
| --- | --- | --- | --- |
| INSTRUMENT_ID | SIR2 | ID |  |
| INSTRUMENT_NAME | “INFRARED SPECTROMETER 2” | CHAR |  |
| INSTRUMENT_TYPE | “INFRARED SPECTROMETER” | CHAR |  |
| INSTRUMENT_MODE_ID | OPERATING | ID |  |
| INSTRUMENT_MODE_ID_DESC | “MEASUREMENT MODE” | CHAR | Instrument is in measurement mode |
| NOTE | “THIS DATA PRODUCT HAS BEEN GENERATED BY THE GDP SOFTWARE” | CHAR | Followed by a list of the “SPACE PACKET FILE AND CONFIGURATION FILES USED” and “SPICE KERNELS USED” |

##### Data Objects Definition

| /* DATA OBJECTS DEFINITION Description */ |  |  |  |
| --- | --- | --- | --- |
| OBJECT | SIR2_SC_HEADER | ID |  |
| BYTES | 14400 | INT | FITs extention header length |
| HEADER_TYPE | FITS | ID |  |
| INTERCHANGE_FORMAT | BINARY | ID |  |
| RECORDS | 5 | INT | The size of a SIR2 record is defined by the length of a FITS header |
| DESCRIPTION | “SIR2 FITS HEADER” | CHAR |  |
| END_OBJECT | SIR2_SC_HEADER |  |  |

##### SIR SC Table

| OBJECT | SIR_SC_TABLE | ID |  |
| --- | --- | --- | --- |
| NAME | “SIR2 SPECTRAL OBSERVATIONS” | CHAR |  |
| INTERCHANGE_FORMAT | BINARY | ID |  |
| COLUMNS | 45 | INT |  |
| ROWS | 12423 | INT | Depends on the number of spectra observed |
| ROW_BYTES | 712 | INT |  |
| DESCRIPTION | “SIR-2 TIME-TAGGED SPECTRAL OBSERVATIONS IN ORIGINAL DIGITAL NUMBER” | CHAR |  |
| OBJECT | COLUMN | ID |  |
| NAME | “UTC_TIME” | CHAR |  |
| COLUMN_NUMBER | 1 | INT |  |
| BYTES | 23 | INT |  |
| DATA_TYPE | TIME | INT |  |
| START_BYTE | 1 | INT |  |
| DESCRIPTION | “START TIME OF MEASUREMENT (UTC).” | CHAR |  |
| UNIT | ”UT” | FORMATION |  |
| VALID_MAXIMUM | ”N/A” | FORMATION |  |
| VALID_MINIMUM | ”N/A” | FORMATION |  |
| END_OBJECT | COLUMN | ID |  |

##### Exposure Time

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “EXPOSURE_TIME” | CHAR |  |
| COLUMN_NUMBER | 3 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | INT |  |
| START_BYTE | 41 | INT |  |
| DESCRIPTION | “DETECTOR EXPOSURE TIME SPECIFIED IN THIS PARAMETER IS GIVEN IN UNITS OF 8 CLOCKS PERIODS. THIS VALUE IS THEN DEPENDING ON THE SENSOR READOUT CLOCK DIVISOR PARAMETER (READDIV), SEE COLUMN OBJECT NAMED READDIV. THE REAL EXPOSURE TIME CAN BE CALCULATED WITH THIS FORMULA: EXPOSURE TIME*8/(20MHZ/READDIV) WHERE READDIV IS THE SENSOR READOUT REQUENCY DIVISOR.” | CHAR |  |
| UNIT | “N/A” | FORMATION |  |
| VALID_MAXIMUM | 65535 | INT |  |
| VALID_MINIMUM | 1 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### Real Exposure Time

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “REAL_EXPOSURE_TIME” | CHAR |  |
| COLUMN_NUMBER | 4 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | REAL |  |
| START_BYTE | 43 | INT |  |
| DESCRIPTION | “DETECTOR EXPOSURE TIME IN MILLISECONDS” | CHAR |  |
| VALID_MAXIMUM | “N/A” | REAL |  |
| VALID_MINIMUM | “N/A” | REAL |  |
| UNIT | “N/A” | CHAR |  |
| END_OBJECT | COLUMN | ID |  |

##### Spectrum

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SPECTRUM” | CHAR |  |
| COLUMN_NUMBER | 5 | INT |  |
| BYTES | 512 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 47 | INT |  |
| ITEMS | 256 | INT |  |
| ITEMS_BYTES | 2 | INT |  |
| DESCRIPTION | “DETECTOR 256 PIXEL VALUES PHOTO DIODE ARRAY MEASUREMENTS IN ORIGINAL DIGITAL NUMBERS” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 65535 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### Data Quality ID

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “DATA_QUALITY_ID” | CHAR |  |
| COLUMN_NUMBER | 6 | INT |  |
| BYTES | 1 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 559 | INT |  |
| DESCRIPTION | “THE DATA QUALITY IS EVALUATED ACCORDING TO THE NUMBER OF SATURATED PIXELS OUT OF THE 256 PIXELS. -4 IS HIGHEST QUALITY, NO SATURATED PIXELS. -3 IS HIGH QUALITY, 14 OR LESS SATURATED. -2 IS MEDIUM QUALITY, 58 OR LESS SATURATED. -1 IS LOW QUALITY, 132 OR LESS SATURATED. -0 IS LOWEST QUALITY, MORE THAN 132 SATURATED. - -1 IS NON EXISTENT.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 4 | INT |  |
| VALID_MINIMUM | -1 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### Longitude

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “LONGITUDE” | CHAR |  |
| COLUMN_NUMBER | 7 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 560 | INT |  |
| DESCRIPTION | “MOON LONGITUDE AT TARGET POINT” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 180.0 | REAL |  |
| VALID_MINIMUM | -180.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Latitude

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “LATITUDE” | CHAR |  |
| COLUMN_NUMBER | 8 | INT |  |
| BYTES | 3 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 564 | INT |  |
| DESCRIPTION | “MOON LATITUDE AT TARGET POINT” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 90.0 | REAL |  |
| VALID_MINIMUM | -90.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Incidence Angle

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “INCIDENCE_ANGLE” | CHAR |  |
| COLUMN_NUMBER | 9 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 568 | INT |  |
| DESCRIPTION | “THE ANGLE BETWEEN THE TARGET POINT NORMAL VECTOR AND THE TARGET POINT – SUN VECTOR” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 180.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Phase Angle

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “PHASE_ANGLE” | CHAR |  |
| COLUMN_NUMBER | 10 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 572 | INT |  |
| DESCRIPTION | “THE ANGLE BETWEEN THE TARGET POINT – SPACECRAFT VECTOR AND THE TARGET POINT – SUN VECTOR” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 180.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Emission Angle

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “EMISSION_ANGLE” | CHAR |  |
| COLUMN_NUMBER | 11 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 576 | INT |  |
| DESCRIPTION | “THE ANGLE BETWEEN THE TARGET POINT NORMAL VECTOR AND THE TARGET POINT – SPACECRAFT VECTOR” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 180.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Local Hour Angle

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “LOCAL_HOUR_ANGLE” | CHAR |  |
| COLUMN_NUMBER | 12 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 580 | INT |  |
| DESCRIPTION | “THE ANGLE BETWEEN THE LOCAL SURFACE NORMAL PROJECTED ONTO THE ECLIPTIC PLANE AND THE DIRECTION VECTOR TO THE SUN. IT IS GIVEN FOR THE VIEWPOINT ON THE LUNAR SURFACE WITH ABERRATION CORRECTION. THE VALUE IS ALWAYS 0 DEG AT NOON, 90 DEG AT SUNSET, AND 270 DEG AT SUNRISE. THE LOCAL HOUR ANGLE DOES NOT ADVANCE WITH A CONSTANT RATE. "” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 360.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Sub Spacecraft Longitude

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SUB_SPACECRAFT_LONGITUDE” | CHAR |  |
| COLUMN_NUMBER | 13 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 584 | INT |  |
| DESCRIPTION | “SUB SPACECRAFT POINT LONGITUDE” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 180.0 | REAL |  |
| VALID_MINIMUM | -180.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Sub Spacecraft Latitude

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SUB_SPACECRAFT_LATITUDE” | CHAR |  |
| COLUMN_NUMBER | 14 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 588 | INT |  |
| DESCRIPTION | “SUB SPACECRAFT POINT LATITUDE” | CHAR |  |
| UNIT | “DEGREES” | CHAR |  |
| VALID_MAXIMUM | 90.0 | REAL |  |
| VALID_MINIMUM | -90.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Spacecraft Altitude

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SPACECRAFT_ALTITUDE” | CHAR |  |
| COLUMN_NUMBER | 15 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 592 | INT |  |
| DESCRIPTION | “SAPCECRAFT ALTITUDE” | CHAR |  |
| UNIT | “Km” | CHAR |  |
| VALID_MAXIMUM | “N/A” | REAL |  |
| VALID_MINIMUM | “N/A” | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### Sittime

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SITTIME” | CHAR |  |
| COLUMN_NUMBER | 16 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 596 | INT |  |
| DESCRIPTION | “SAMPLE INTERVAL TIME. TIME PERIOD BETWEEN TWO SPECTRA TAKEN IN UNITS OF 10 MILISECONDS.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 65535 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### MMODE

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “MMODE” | CHAR |  |
| COLUMN_NUMBER | 17 | INT |  |
| BYTES | 1 | INT |  |
| DATA_TYPE | MSB_UNSIGNED_INTEGER | ID |  |
| START_BYTE | 598 | INT |  |
| DESCRIPTION | “MEASUREMENT MODE. THERE ARE TWO POSSIBLE MODES: NORMAL MODE AND AVERAGE MODE. IN NORMAL MODE, SINGLE SPECTRA ARE SENT WITH NO PROCESSING. IN AVERAGE MODE, A CONFIGURABLE NUMBER OF SPECTRA ARE AVERAGED (ARITHMETIC MEAN) BEFORE THEY ARE SENT. 0 MEANS NORMAL MODE AND 1 MEANS AVERAGE MODE. THE NUMBER OF SPECTRA TO AVERAGE IS SPECIFIED IN THE SPECTRA TO AVERAGE PARAMATER (AVERAG), SEE THE COLUMN OBJECT NAMED AVERAG.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 1 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### AVERAGE

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “AVERAGE” | CHAR |  |
| COLUMN_NUMBER | 18 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 599 | INT |  |
| DESCRIPTION | “NUMBER OF SPECTRA TO AVERAGE IN AVERAGE MODE. THIS VALUE IS IGNORED IN NORMAL MODE. THE AVERAGING METHOD USED IS THE ARITHMETIC MEAN. THE NUMBER IS A POWER OF TWO.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 32768 | INT |  |
| VALID_MINIMUM | 2 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### ADCDEL

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ADCDEL” | CHAR |  |
| COLUMN_NUMBER | 19 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 601 | INT |  |
| DESCRIPTION | “ADC TRIGGER DELAY. THE NUMBER OF SENSOR CLOCK PERIODS TO WAIT BETWEEN THE ANALOG PIXEL VALUE BEING AVAILABLE, AND IT BEING READ.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 65535 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### READDIV

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “READDIV” | CHAR |  |
| COLUMN_NUMBER | 20 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 603 | INT |  |
| DESCRIPTION | “SENSOR READOUT FREQUENCY DIVISOR.DEFINES WHICH NUMBER THE SYSTEM CLOCK (20MHz) HAS BEEN DIVIDED BY TO GET THE SENSOR READOUT CLOCK.READOUT FREQUENCY (MHZ) = 20MHZ/READDIV.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 65535 | INT |  |
| VALID_MINIMUM | 13 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### NSPEC

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “NSPEC” | CHAR |  |
| COLUMN_NUMBER | 21 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 605 | INT |  |
| DESCRIPTION | “WHEN IN MEASUREMENT MORE THIS PARAMETER SPECIFIES THE NUMBER OF SPECTRA TO OBTAIN BEFORE RETURNING TO PREPARATION MODE. IF SET TO ZERO, THE INSTRUMENT WILL STAY IN MEASUREMENT MODE INDEFINITELY.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### PXPER

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “PXPER” | CHAR |  |
| COLUMN_NUMBER | 22 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 607 | INT |  |
| DESCRIPTION | “PIXEL PERIOD TIME. THE TIME ELAPSED AFTER A PIXEL HAS BEEN SHIFTED OUT UNTIL THE NEXT PIXEL BEING SHIFTED OUT. THE PIXEL PERIOD IS SPECIFIED IN NUMBER OF SENSOR READOUT CLOCK PERIODS (SEE THE COLUMN OBJECT NAMED READDIV).” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### NSMPL

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “NSMPL” | CHAR |  |
| COLUMN_NUMBER | 23 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 609 | INT |  |
| DESCRIPTION | “NUMBER OF SAMPLES TAKEN PER PIXEL. IF MORE THAN ONE, ALL SAMPLES WILL BE AVERAGED (ARITHMETIC MEAN) BY THE INSTRUMENT BEFORE BEING SENT.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### CCSDS Counter

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CCSDS_COUNTER” | CHAR |  |
| COLUMN_NUMBER | 24 | INT |  |
| BYTES | 2 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 611 | INT |  |
| DESCRIPTION | “PACKET SEQUENCE COUNT IN THE PACKET PRIMARY HEADER (CCSDS STANDARD). THE PACKET SEQUENCE COUNT FIELD PROVIDES THE SEQUENTIAL BINARY COUNT OF EACH SPACE PACKET GENERATED BY THE USER APPLICATION IDENTIFIED BY THEAPID(APPLICATIONIDENTIFIER).” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 16383 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| OFFSET | 32768 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### HK Clock

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “HK_CLOCK” | CHAR |  |
| COLUMN_NUMBER | 25 | INT |  |
| BYTES | 13 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 613 | INT |  |
| DESCRIPTION | “HK CLOCK AT TIME OF MEASUREMENT STARTS. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### HK Clock OBT Synchronization

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “HK_CLOCK_OBT_SYNCRONIZATION” | CHAR |  |
| COLUMN_NUMBER | 26 | INT |  |
| BYTES | 13 | INT |  |
| DATA_TYPE | MSB_INTEGER | ID |  |
| START_BYTE | 626 | INT |  |
| DESCRIPTION | “HK CLOCK AT TIME OF OBT SYNCHRONIZATION. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | “N/A” | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### OBT Synchronization

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “OBT_SYNCRONIZATION” | CHAR |  |
| COLUMN_NUMBER | 27 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 639 | INT |  |
| DESCRIPTION | “OBT AT TIME OF SYNCHRONIZATION IN MS.” | CHAR |  |
| UNIT | “ms” | CHAR |  |
| VALID_MAXIMUM | “N/A” | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC” | CHAR |  |
| COLUMN_NUMBER | 28 | INT |  |
| BYTES | 1 | INT |  |
| DATA_TYPE | MSB_UNSIGNED_INTEGER | ID |  |
| START_BYTE | 647 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER SET VALUE. WHEN THE TEC IS CONTROLLED MANUALLY THIS VALUE GIVES A TEC CURRENT LINEAR VALUE.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | “N/A” | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC Volt

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC_VOLT” | CHAR |  |
| COLUMN_NUMBER | 29 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 648 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 1.5 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC CURR

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC_CURR” | CHAR |  |
| COLUMN_NUMBER | 30 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 652 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER CURRENT.” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 5000.0 | INT |  |
| VALID_MINIMUM | 0.0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON5A

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON5A” | CHAR |  |
| COLUMN_NUMBER | 31 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 656 | INT |  |
| DESCRIPTION | “VCC CURRENT.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 120.0 | REAL |  |
| VALID_MINIMUM | 30.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON1D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON1D” | CHAR |  |
| COLUMN_NUMBER | 32 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 660 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 1.5V.” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 250.0 | REAL |  |
| VALID_MINIMUM | 170.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON3D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON3D” | CHAR |  |
| COLUMN_NUMBER | 33 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 664 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 3.3V.” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 450.0 | REAL |  |
| VALID_MINIMUM | 10.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON5D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON5D” | CHAR |  |
| COLUMN_NUMBER | 34 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 668 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 5.0V.” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 10.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON5A

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON5A” | CHAR |  |
| COLUMN_NUMBER | 35 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 672 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT VCC VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 5.3 | REAL |  |
| VALID_MINIMUM | 5.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### V14 Volt

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “V14_VOLT” | CHAR |  |
| COLUMN_NUMBER | 36 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 676 | INT |  |
| DESCRIPTION | “14 V VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 15.0 | REAL |  |
| VALID_MINIMUM | 13.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON3D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON3D” | CHAR |  |
| COLUMN_NUMBER | 37 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 680 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 3.3V VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 3.4 | REAL |  |
| VALID_MINIMUM | 3.2 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### ISU PDA TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ISU_PDA_TEMP” | CHAR |  |
| COLUMN_NUMBER | 38 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 684 | INT |  |
| DESCRIPTION | “PHOTO DIODE ARRAY TEMPERATURE (ON-CHIP THERMISTOR).” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | REAL |  |
| VALID_MINIMUM | -70.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### ISU PCB TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ISU_PCB_TEMP” | CHAR |  |
| COLUMN_NUMBER | 39 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 688 | INT |  |
| DESCRIPTION | “INSTRUMENT SENSOR UNIT PCB TEMPERATURE.” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 50.0 | REAL |  |
| VALID_MINIMUM | -50.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### PSU PCB TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “PSU_PCB_TEMP” | CHAR |  |
| COLUMN_NUMBER | 40 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 692 | INT |  |
| DESCRIPTION | “POWER SUPPLY UINT PCB TEMPERATURE.” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 55.0 | REAL |  |
| VALID_MINIMUM | 10.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### COOL TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “COOL_TEMP” | CHAR |  |
| COLUMN_NUMBER | 41 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 696 | INT |  |
| DESCRIPTION | “COOLING CONSOLE TEMPERATURE.” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | INT |  |
| VALID_MINIMUM | -70.0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### QUA TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “QUA_TEMP” | CHAR |  |
| COLUMN_NUMBER | 42 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 700 | INT |  |
| DESCRIPTION | “QUARTZ-BODY TEMPERATURE.” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | REAL |  |
| VALID_MINIMUM | -70.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON5D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON5D” | CHAR |  |
| COLUMN_NUMBER | 43 | INT |  |
| BYTES | 4 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 704 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 5.0V VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 5.4 | REAL |  |
| VALID_MINIMUM | 4.9 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON1D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON1D” | CHAR |  |
| COLUMN_NUMBER | 44 | REAL |  |
| BYTES | 4 | REAL |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 708 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 1.5V VOLTAGE.” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 1.55 | REAL |  |
| VALID_MINIMUM | 1.4 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### ERRORID

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ERRORID” | CHAR |  |
| COLUMN_NUMBER | 45 | INT |  |
| BYTES | 1 | INT |  |
| DATA_TYPE | MSB_UNSIGNED_INTEGER | ID |  |
| START_BYTE | 712 | INT |  |
| DESCRIPTION | “ERROR IDENTIFIER FROM THE LAST IHDM (INSTRUMENT HEALTH DATA MESSAGE) THAT WAS SENT. SEE SIR2 ICD FOR MORE DETAILED INFORMATION.” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 11 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |
| END |  |  |  |

### SIR-2 Instrument Housekeeping Data: Data Product Design

#### PDS Version ID

| Keyword | C1-Value | Type | Description |
| --- | --- | --- | --- |
| PDS_VERSION_ID | PDS3 | ID |  |

#### File Characteristics Data Elements

| /*** File Characteristics Data Elements ***/ |  |  | Comment |
| --- | --- | --- | --- |
| FILE_NAME |  | CHAR | See section 4.1.4 |
| RECORD_TYPE | FIXED_LENGTH | ID | binary records with fixed lengths |
| RECORD_BYTES | 2880 | INT | Output of data processor |
| FILE_RECORDS |  | INT | Number of records in the detached file |
| INTERCHANGE_FORMAT | ASCII | ID |  |
| RELEASE_ID | 0001 | INT |  |
| REVISION_ID | 0000 | INT |  |

#### Pointer to Data Objects

| /*** Pointer to Data Objects ***/ |  |  | Comment |
| --- | --- | --- | --- |
| ^SIR2_HK_HEADER | (“FN”) | CHAR | e.g. (“CH1SIR2_NE2_SC_R01971.FIT”,1<BYTES>) |
| ^SIR2_HK_TABLE | (“FN”, 11521 <BYTES>) | CHAR | e.g. (“CH1SIR2_NE2_SC_R01971.FIT”, 11521 <BYTES>)) |

#### Identification Data Elements

| /*** Identification Data Elements |  |  | Comment |
| --- | --- | --- | --- |
| DATA_SET_ID |  | ID | e.g. “CH1ORB-L-SIR2-2-NPO-V1.0” |
| DATA_SET_NAME |  | CHAR | e.g. “CHANDRAYAAN-1-ORBITER LUNAR SIR2 2 NPO V1.0” |
| PRODUCT_ID |  | CHAR | e.g. “CH1SIR2_NE2_HK_R00122” |
| PRODUCT_CREATION_TIME |  | TIME | In PDS time standard YYYY-MM-DD”T”HH:MM:SS |
| PRODUCT_TYPE | EDR | ID | SIR-2 Experiment Data Recored |
| PROCESSING_LEVEL_ID | 2 | INT | CODMAC level, 2 for raw/edited data and 3 for calibrated data |
| PROCESSING_LEVEL_DESC | “EXPERIMENT DATA RECORD” | CHAR |  |
| PRODUCER_ID | “SIR2 TEAM” | ID |  |
| PRODUCER_FULL_NAME | “DR. URS MALL” | CHAR |  |
| PRODUCER_INSTITUTION_NAME | “MAX PLANCK INSTITUTE FOR SOLAR SYSTEM RESEARCH” |  |  |
| DATA_QUALITY_ID | “N/A” | INT | THE DATA QUALITY ID IS DEFINED FOR THE SPECTRAL DATA ONLY. |
| DATA_QUALITY_DESC | “N/A” | INT | FOR THE HK DATA A DATA QUALITY ID IS NOT APPLICABLE |
| MISSION_ID | CH1 | ID |  |
| MISSION_NAME | “CHANDRAYAAN-1” | ID |  |
| INSTRUMENT_HOST ID | CH1ORB | ID |  |
| INSTRUMENT_HOST_NAME | “CHANDRAYAAN-1-ORBITER” | ID |  |
| MISSION_PHASE_NAME | “NORMAL PHASE OPERATIONS” | ID | ““NORMAL PHASE OPERATIONS” |
| TARGET_NAME | MOON | ID |  |
| TARGET_TYPE | SATELLITE | ID | SATELLITE |

#### Data and Time Related Information

| /*** Date and Time Related Information ***/ |  | COMMENT |  |
| --- | --- | --- | --- |
| START_TIME |  | TIME | In PDS time standard |
| STOP_TIME |  | TIME | In PDS time standard |
| SPACECRAFT_CLOCK_START_COUNT |  | CHAR | e.g. “8/0034534.3321”.In the form PARTITION / TIME |
| SPACECRAFT_CLOCK_STOP_COUNT |  | CHAR |  |
| ORBIT_NUMBER | 1971 | INT | Integer number starting from 1. |

#### Instrument Related Parameters

| /*** Instrument Related Parameters ***/ |  | COMMENT |  |
| --- | --- | --- | --- |
| INSTRUMENT_ID | SIR2 | ID |  |
| INSTRUMENT_NAME | “INFRARED SPECTROMETER 2” | CHAR |  |
| INSTRUMENT_TYPE | “INFRARED SPECTROMETER” | CHAR |  |
| INSTRUMENT_MODE_ID | OPERATING | ID |  |
| INSTRUMENT_MODE_ID_DESC | “OPERATING” | CHAR | SIR-2 is in MEASUREMENT MODE an collecting data. |
| NOTE | “THIS DATA PRODUCT HAS BEEN GENERATED BY THE GDP SOFTWARE” | CHAR | Followed by a list of the “SPACE PACKET FILE AND CONFIGURATION FILES USED” and “SPICE KERNELS USED” |

#### Data Objects Definition

| /* DATA OBJECTS DEFINITION Description */ |  |  |  |
| --- | --- | --- | --- |
| OBJECT | SIR2_HK_HEADER | ID |  |
| BYTES | 11520 | INT | FITs header length |
| HEADER_TYPE | FITS | ID |  |
| INTERCHANGE_FORMAT | BINARY | ID |  |
| RECORDS | 4 | INT | The size of a SIR2 record is defined by the length of a FITS header |
| DESCRIPTION | “SIR2 FITS HEADER” | CHAR |  |
| END_OBJECT | SIR2_HK_HEADER |  |  |

##### SIR2 HK Table

| OBJECT | SIR2_HK_TABLE | ID |  |
| --- | --- | --- | --- |
| NAME | “SIR2 HOUSEKKEPING DATA” | CHAR |  |
| INTERCHANGE_FORMAT | ASCII | ID |  |
| COLUMNS | 24 | INT |  |
| ROWS | 3776 | INT |  |
| ROW_BYTES | 245 | INT |  |
| DESCRIPTION | “THE FILE CONTAINS A TABLE WITH THE HOUSEKKEPING OF SIR2 SPECTRAL OBSERVATIONS” | CHAR |  |
| END_OBJECT | SIR2_HK_TABLE |  |  |

##### UTC Time

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “UTC_TIME” | CHAR |  |
| COLUMN_NUMBER | 1 | INT |  |
| BYTES | 23 | INT |  |
| DATA_TYPE | TIME | ID |  |
| START_BYTE | 1 | INT |  |
| DESCRIPTION | “START TIME OF MEASUREMENT (UTC).” | CHAR |  |
| FORMAT | “A23” |  |  |
| UNIT | ”UT” | FORMATION |  |
| VALID_MAXIMUM | ”N/A” | FORMATION |  |
| VALID_MINIMUM | ”N/A” | FORMATION |  |
| END_OBJECT | COLUMN | ID |  |

##### SCET Time

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “SCET_TIME” | CHAR |  |
| COLUMN_NUMBER | 2 | INT |  |
| BYTES | 17 | INT |  |
| DATA_TYPE | CHARACTER | ID |  |
| START_BYTE | 25 | INT |  |
| DESCRIPTION | “START TIME OF MEASUREMENT (SCET).” | CHAR |  |
| FORMAT | “A17” | CHAR |  |
| UNIT | ”N/A” | FORMATION |  |
| VALID_MAXIMUM | ”N/A” | FORMATION |  |
| VALID_MINIMUM | ”N/A” | FORMATION |  |
| END_OBJECT | COLUMN | ID |  |

##### HK Clock

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “HK_CLOCK” | CHAR |  |
| COLUMN_NUMBER | 3 | INT |  |
| BYTES | 13 | INT |  |
| DATA_TYPE | ASCII_INTEGER | ID |  |
| START_BYTE | 43 | INT |  |
| DESCRIPTION | “HK CLOCK AT TIME OF MEASUREMENT STARTS. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS.” | CHAR |  |
| FORMAT | “A13” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### HK Clock OBT Synchronization

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “HK_CLOCK_OBT_SYNCROIZATION” | CHAR |  |
| COLUMN_NUMBER | 4 | INT |  |
| BYTES | 13 | INT |  |
| DATA_TYPE | ASCII_INTEGER | ID |  |
| START_BYTE | 57 | INT |  |
| DESCRIPTION | “HK CLOCK AT TIME OF OBT SYNCHRONIZATION. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS.” | CHAR |  |
| FORMAT | “A13” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | “N/A” | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### OBT Synchronization

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “OBT_SYNCRONIZATION” | CHAR |  |
| COLUMN_NUMBER | 5 | INT |  |
| BYTES | 15 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 71 | INT |  |
| DESCRIPTION | “OBT AT TIME OF SYNCHRONIZATION IN MS.” | CHAR |  |
| FORMAT | “F15.5” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | “N/A” | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC” | CHAR |  |
| COLUMN_NUMBER | 6 | INT |  |
| BYTES | 3 | INT |  |
| DATA_TYPE | ASCII_INTEGER | ID |  |
| START_BYTE | 87 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER SET VALUE. WHEN THE TEC IS CONTROLLED MANUALLY THIS VALUE GIVES A TEC CURRENT LINEAR VALUE.” | CHAR |  |
| FORMAT | “I3” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | “N/A” | INT |  |
| VALID_MINIMUM | “N/A” | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC Volt

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC_VOLT” | CHAR |  |
| COLUMN_NUMBER | 7 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 91 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER VOLTAGE.” | CHAR |  |
| FORMAT | “F8,2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 1.5 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### TEC CURR

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “TEC_CURR” | CHAR |  |
| COLUMN_NUMBER | 8 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 100 | INT |  |
| DESCRIPTION | “THERMO ELECTRICAL COOLER CURRENT.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 5000.0 | INT |  |
| VALID_MINIMUM | 0.0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON5A

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON5A” | CHAR |  |
| COLUMN_NUMBER | 9 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 109 | INT |  |
| DESCRIPTION | “VCC CURRENT.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 120.0 | REAL |  |
| VALID_MINIMUM | 30.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON1D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON1D” | CHAR |  |
| COLUMN_NUMBER | 10 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 118 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 1.5V.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 250.0 | REAL |  |
| VALID_MINIMUM | 170.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON3D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON3D” | CHAR |  |
| COLUMN_NUMBER | 11 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | IEEE_REAL | ID |  |
| START_BYTE | 127 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 3.3V.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 450.0 | REAL |  |
| VALID_MINIMUM | 10.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CMON5D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CMON5D” | CHAR |  |
| COLUMN_NUMBER | 12 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 136 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT CURRENT 5.0V.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “mA” | CHAR |  |
| VALID_MAXIMUM | 10.0 | REAL |  |
| VALID_MINIMUM | 0.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON5A

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON5A” | CHAR |  |
| COLUMN_NUMBER | 13 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 145 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT VCC VOLTAGE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 5.3 | REAL |  |
| VALID_MINIMUM | 5.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### V14 Volt

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “V14_VOLT” | CHAR |  |
| COLUMN_NUMBER | 14 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 154 | INT |  |
| DESCRIPTION | “14 V VOLTAGE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 15.0 | REAL |  |
| VALID_MINIMUM | 13.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON3D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON3D” | CHAR |  |
| COLUMN_NUMBER | 15 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 163 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 3.3V VOLTAGE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 3.4 | REAL |  |
| VALID_MINIMUM | 3.2 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### ISU PDA TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ISU_PDA_TEMP” | CHAR |  |
| COLUMN_NUMBER | 16 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 172 | INT |  |
| DESCRIPTION | “PHOTO DIODE ARRAY TEMPERATURE (ON-CHIP THERMISTOR).” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | REAL |  |
| VALID_MINIMUM | -70.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### ISU PCB TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ISU_PCB_TEMP” | CHAR |  |
| COLUMN_NUMBER | 17 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 181 | INT |  |
| DESCRIPTION | “INSTRUMENT SENSOR UNIT PCB TEMPERATURE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 50.0 | REAL |  |
| VALID_MINIMUM | -50.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### PSU PCB TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “PSU_PCB_TEMP” | CHAR |  |
| COLUMN_NUMBER | 18 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 190 | INT |  |
| DESCRIPTION | “POWER SUPPLY UINT PCB TEMPERATURE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 55.0 | REAL |  |
| VALID_MINIMUM | 10.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### COOL TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “COOL_TEMP” | CHAR |  |
| COLUMN_NUMBER | 19 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 199 | INT |  |
| DESCRIPTION | “COOLING CONSOLE TEMPERATURE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | INT |  |
| VALID_MINIMUM | -70.0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### QUA TEMP

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “QUA_TEMP” | CHAR |  |
| COLUMN_NUMBER | 20 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 208 | INT |  |
| DESCRIPTION | “QUARTZ-BODY TEMPERATURE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “C” | CHAR |  |
| VALID_MAXIMUM | 35.0 | REAL |  |
| VALID_MINIMUM | -70.0 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON5D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON5D” | CHAR |  |
| COLUMN_NUMBER | 21 | INT |  |
| BYTES | 8 | INT |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 217 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 5.0V VOLTAGE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 5.4 | REAL |  |
| VALID_MINIMUM | 4.9 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### VMON1D

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “VMON1D” | CHAR |  |
| COLUMN_NUMBER | 22 | REAL |  |
| BYTES | 8 | REAL |  |
| DATA_TYPE | ASCII_REAL | ID |  |
| START_BYTE | 226 | INT |  |
| DESCRIPTION | “INSTRUMENT CONTROL UNIT 1.5V VOLTAGE.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “V” | CHAR |  |
| VALID_MAXIMUM | 1.55 | REAL |  |
| VALID_MINIMUM | 1.4 | REAL |  |
| END_OBJECT | COLUMN | ID |  |

##### CCSDS Counter

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “CCSDS_COUNTER” | CHAR |  |
| COLUMN_NUMBER | 23 | INT |  |
| BYTES | 5 | INT |  |
| DATA_TYPE | ASCII_INTEGER | ID |  |
| START_BYTE | 235 | INT |  |
| DESCRIPTION | “PACKET SEQUENCE COUNT IN THE PACKET PRIMARY HEADER (CCSDS STANDARD). THE PACKET SEQUENCE COUNT FIELD PROVIDES THE SEQUENTIAL BINARY COUNT OF EACH SPACE PACKET GENERATED BY THE USER APPLICATION IDENTIFIED BY THEAPID(APPLICATIONIDENTIFIER).” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 16383 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |

##### ERRORID

| OBJECT | COLUMN | ID |  |
| --- | --- | --- | --- |
| NAME | “ERRORID” | CHAR |  |
| COLUMN_NUMBER | 24 | INT |  |
| BYTES | 3 | INT |  |
| DATA_TYPE | MSB_UNSIGNED_INTEGER | ID |  |
| START_BYTE | 241 | INT |  |
| DESCRIPTION | “ERROR IDENTIFIER FROM THE LAST IHDM (INSTRUMENT HEALTH DATA MESSAGE) THAT WAS SENT. SEE SIR2 ICD FOR MORE DETAILED INFORMATION.” | CHAR |  |
| FORMAT | “F8.2” | CHAR |  |
| UNIT | “N/A” | CHAR |  |
| VALID_MAXIMUM | 11 | INT |  |
| VALID_MINIMUM | 0 | INT |  |
| END_OBJECT | COLUMN | ID |  |
| END |  |  |  |

## Sample Labels

### PDS version

PDS_VERSION_ID	=	PDS3

### File Characteristics and data elements

The following keywords are defined for this section:

/*** FILE CHARACTERISTICS ***/

FILE_NAME				= “CH1SIR2_NE2_SC_R01971.LBL "

RECORD_TYPE			= FIXED_LENGTH

RECORD_BYTES			= 2880

FILE_RECORDS			= 3077

INTERCHANGE_FORMAT		= BINARY

Most of the keywords in this part are standard PDS keywords. A detailed description is given in the PDS Standard Reference

Data Object Pointers

/*** POINTERS TO OBJECTS IN FILE ***/

^SIR2_SC_HEADER = (“CH1SIR2_NE2_SC_R01971.FIT,1<BYTES>”)

^SIR2_SC_TABLE     = (“CH1SIR2_NE2_SC_R01971.FIT,14401<BYTES>”)

### Identification Data Elements

/*** IDENTIFICATION DATA ELEMENTS ***/

DATA_SET_ID                                = "CH1ORB-L-SIR2-2-NPO-V1.0"

DATA_SET_NAME                        = "CHANDRAYAAN-1-ORBITER LUNAR SIR2 2 NPO V1.0"

PRODUCT_ID                                 = "CH1SIR2_NE2_SC_R01971"

PRODUCT_CREATION_TIME     = 2009-04-25T13:44:14

PRODUCT_TYPE                           = EDR

PROCESSING_LEVEL_ID             = 2

PROCESSING_LEVEL_DESC       = "EXPERIMENT DATA RECORD"

| PRODUCER_ID | = SIR2_TEAM |
| --- | --- |
| PRODUCER_FULL_NAME | = "DR. URS MALL" |
| PRODUCER_INSTITUTION_NAME | = "MAX PLANCK INSTITUTE FOR SOLAR SYSTEM RESEARCH" |
| DATA_QUALITY_ID | = 4 |
| DATA_QUALITY_DESC | = "THE DATA QUALITY ID FOR ALL SPECTRA IN THIS DATA PRODUCT IS CALCULATED AS THE AVERAGE OF THE INDIVIDUAL QUALITY ID FOR EACH SPECTRUM ROUNDED TO AN INTEGER VALUE. (SEE THE DATA_QUALITY_ID COLUMN OBJECT BELOW)" |
| MISSION_ID | = CH1 |
| MISSION_NAME | = "CHANDRAYAAN-1" |
| INSTRUMENT_HOST_ID | = CH1ORB |
| NSTRUMENT_HOST_NAME | ="CHANDRAYAAN-1-ORBITER" |
| MISSION_PHASE_NAME | = "NORMAL PHASE OPERATIONS" |
| TARGET_NAME | = "MOON" |
| TARGET_TYPE | = SATELLITE |
| START_TIME | = 2009-04-19T22:46:48 |
| STOP_TIME | = 2009-04-20T00:44:11 |
| SPACECRAFT_CLOCK_START_COUNT | = "9/1032586.19" |
| SPACECRAFT_CLOCK_STOP_COUNT | = "9/1039629.885" |
| ORBIT_NUMBER | = 1971 |

### Descriptive related parameters

| INSTRUMENT_ID |  | = SIR2 |
| --- | --- | --- |
| INSTRUMENT_NAME |  | ="INFRARED SPECTROMETER 2" |
| INSTRUMENT_TYPE |  | = "INFRARED SPECTROMETER" |
| INSTRUMENT_MODE_ID |  | = OPERATING |
| INSTRUMENT_MODE_DESC |  | = "OPERATING" |
| NOTE | = "THIS DATA PRODUCT HAS BEEN GENERATED BY THE GDP SOFTWARE. SPACE PACKET FILE AND CONFIGURATION FILES USED: CH1_SIR_SC_0050_2009110072324446_D18.dat CH1_SIR2_50_SCDP.tcf CH1_SIR2_50_SCDP_V3.0.dcf CH1_SIR2_50_SCDP_V3.0.pcf SPICE KERNELS USED: NAIF0009.TLS PCK00008.TPC moon_pa_de421_1900-2050.bpc moon_080317.tf moon_assoc_me.tf EARTH_TOPO_050714.TF RSSD0002.TF de421.bsp CH1_V01.TF CH1_SIR2_V01.TI isro_2008295235703_2010365000000_00.sclk isro_21_day_eph_2008298_00.bsp isro_21_day_eph_2008315_00.bsp |  |

### Data objects definition

| OBJECT | = SIR2_SC_HEADER |
| --- | --- |
| BYTES | = 14400 |
| HEADER_TYPE | = FITS |
| INTERCHANGE_FORMAT | = BINARY |
| RECORDS | = 5 |
| DESCRIPTION | = "SIR2 FITS HEADER" |
| END_OBJECT | = SIR2_SC_HEADER |
| OBJECT | = SIR2_SC_TABLE |
| NAME | ="SIR2 SPECTRAL OBSERVATIONS" |
| INTERCHANGE_FORMAT | = BINARY |
| COLUMNS | =45 |
| ROWS | = 12423 |
| ROW_BYTES | = 712 |
| DESCRIPTION | = "SIR2 TIME-TAGGED SPECTRAL OBSERVATIONS IN ORIGINAL DIGITAL NUMBER" |
| OBJECT | = COLUMN |
| NAME | = "UTC_TIME" |
| COLUMN_NUMBER | = 1 |
| BYTES | = 23 |
| DATA_TYPE | = TIME |
| START_BYTE | = 1 |
| DESCRIPTION | = "START TIME OF MEASUREMENT (UTC)." |
| UNIT | = "UT" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "SCET_TIME" |
| COLUMN_NUMBER | = 2 |
| BYTES | = 17 |
| DATA_TYPE | = CHARACTER |
| START_BYTE | = 24 |
| DESCRIPTION | = "START TIME OF MEASUREMENT (SCET)." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "EXPOSURE_TIME" |
| COLUMN_NUMBER | = 3 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 41 |
| DESCRIPTION | = "DETECTOR EXPOSURE TIME SPECIFIED IN THIS PARAMETER IS GIVEN IN UNITS OF 8 CLOCK PERIODS. THIS VALUE IS THEN DEPENDING ON THE SENSOR READOUT CLOCK DIVISOR PARAMETER (READDIV), SEE COLUMN OBJECT NAMED READDIV. THE REAL EXPOSURE TIME CAN BE CALCULATED WITH THIS FORMULA: EXPOSURE_TIME * 8 / (20MHz / READDIV) ï¿½s WHERE READDIV IS THE SENSOR READOUT FREQUENCY DIVISOR." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 65535 |
| VALID_MINIMUM | = 1 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | ="REAL_EXPOSURE_TIME" |
| COLUMN_NUMBER | = 4 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 43 |
| DESCRIPTION | ="DETECTOR EXPOSURE TIME IN MILLISECONDS." |
| UNIT | = "MS" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "SPECTRUM" |
| COLUMN_NUMBER | = 5 |
| BYTES | = 512 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 47 |
| ITEMS | = 256 |
| ITEM_BYTES | = 2 |
| DESCRIPTION | = "DETECTOR 256 PIXEL VALUES PHOTO DIODE ARRAY MEASUREMENTS IN ORIGINAL DIGITALNUMBERS." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "DATA_QUALITY_ID" |
| COLUMN_NUMBER | = 6 |
| BYTES | = 1 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 559 |
| DESCRIPTION | = "THE DATA QUALITY IS EVALUATED ACCORDING TO THE NUMBER OF SATURATED PIXELS OUT OF THE 256 PIXELS. - 4 IS HIGHEST QUALITY, NO SATURATED PIXELS. - 3 IS HIGH QUALITY, 14 OR LESS - SATURATED. - 2 IS MEDIUM QUALITY, 58 OR LESS SATURATED. - 1 IS LOW QUALITY, 132 OR LESS SATURATED. - 0 IS LOWEST QUALITY, MORE THAN 132 SATURATED. - -1 IS NON EXISTENT." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 4 |
| VALID_MINIMUM | = -1 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "LONGITUDE" |
| COLUMN_NUMBER | = 7 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 560 |
| DESCRIPTION | = "MOON LONGITUDE AT TARGET POINT" |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 180 |
| VALID_MINIMUM | = -180 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "LATITUDE" |
| COLUMN_NUMBER | = 8 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 564 |
| DESCRIPTION | = "MOON LATITUDE AT TARGET POINT" |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 90 |
| VALID_MINIMUM | = -90 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "INCIDENCE_ANGLE" |
| COLUMN_NUMBER | = 9 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 568 |
| DESCRIPTION | = "THE ANGLE BETWEEN THE TARGET POINT NORMAL VECTOR AND THE TARGET POINT - SUN VECTOR." |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 180 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "PHASE_ANGLE" |
| COLUMN_NUMBER | = 10 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 572 |
| DESCRIPTION | = "THE ANGLE BETWEEN THE TARGET POINT - SPACECRAFT VECTOR AND THE TARGET POINT - SUN VECTOR." |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 180 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "EMISSION_ANGLE" |
| COLUMN_NUMBER | = 11 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 576 |
| DESCRIPTION | = "THE ANGLE BETWEEN THE TARGET POINT NORMAL VECTOR AND THE TARGET POINT - SPACECRAFT VECTOR." |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 180 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "LOCAL_HOUR_ANGLE" |
| COLUMN_NUMBER | = 12 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 580 |
| DESCRIPTION | = "THE ANGLE BETWEEN THE LOCAL SURFACE NORMAL PROJECTED ONTO THE ECLIPTIC PLANE AND THE DIRECTION VECTOR TO THE SUN. IT IS GIVEN FOR THE VIEWPOINT ON THE LUNAR SURFACE WITH ABERRATION CORRECTION. THE VALUE IS ALWAYS 0 DEG AT NOON, 90 DEG AT SUNSET, AND 270 DEG AT SUNRISE. THE LOCAL HOUR ANGLE DOES NOT ADVANCE WITH A CONSTANT RATE. " |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 360 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "SUB_SPACECRAFT_LONGITUDE" |
| COLUMN_NUMBER | = 13 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 584 |
| DESCRIPTION | = "SUB SPACECRAFT POINT LONGITUDE." |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 180 |
| VALID_MINIMUM | = -180 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "SUB_SPACECRAFT_LATITUDE" |
| COLUMN_NUMBER | = 14 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 588 |
| DESCRIPTION | = "SUB SPACECRAFT POINT LATITUDE." |
| UNIT | = "DEGREES" |
| VALID_MAXIMUM | = 90 |
| VALID_MINIMUM | = -90 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "SPACECRAFT_ALTITUDE" |
| COLUMN_NUMBER | = 15 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 592 |
| DESCRIPTION | = "SPACECRAFT ALTITUDE." |
| UNIT | = "Km" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "SITIME" |
| COLUMN_NUMBER | = 16 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 596 |
| DESCRIPTION | = "SAMPLE INTERVAL TIME. TIME PERIOD BETWEEN TWO SPECTRA TAKEN IN UNITS OF 10 MILLISECONDS." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 65535 |
| VALID_MINIMUM | = 5 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "MMODE" |
| COLUMN_NUMBER | = 17 |
| BYTES | = 1 |
| DATA_TYPE | = MSB_UNSIGNED_INTEGER |
| START_BYTE | = 598 |
| DESCRIPTION | = "MEASUREMENT MODE. THERE ARE TWO POSSIBLE MODES: NORMAL MODE AND AVERAGE MODE. IN NORMAL MODE, SINGLE SPECTRA ARE SENT WITH NO PROCESSING. IN AVERAGE MODE, A CONFIGURABLE NUMBER OF SPECTRA ARE AVERAGED (ARITHMETIC MEAN) BEFORE THEY ARE SENT. 0 MEANS NORMAL MODE AND 1 MEANS AVERAGE MODE. THE NUMBER OF SPECTRA TO AVERAGE IS SPECIFIED IN THE SPECTRA TOAVERAGE PARAMATER (AVERAG), SEE THE COLUMN OBJECT NAMED AVERAG." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 1 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "AVERAG" |
| COLUMN_NUMBER | = 18 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 599 |
| DESCRIPTION | = "NUMBER OF SPECTRA TO AVERAGE IN AVERAGE MODE. THIS VALUE IS IGNORED IN NORMAL MODE. THE AVERAGING METHOD USED IS THE ARITHMETIC MEAN. THE NUMBER IS A POWER OF TWO." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 32768 |
| VALID_MINIMUM | = 2 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "ADCDEL" |
| COLUMN_NUMBER | = 19 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 601 |
| DESCRIPTION | = "ADC TRIGGER DELAY. THE NUMBER OF SENSOR CLOCK PERIODS TO WAIT BETWEEN THE ANALOG PIXEL VALUE BEING AVAILABLE, AND IT BEING READ." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "READDIV" |
| COLUMN_NUMBER | = 20 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 603 |
| DESCRIPTION | = "SENSOR READOUT FREQUENCY DIVISOR.DEFINES WHICH NUMBER THE SYSTEM CLOCK (20MHz) HAS BEEN DIVIDED BY TO GET THE SENSOR READOUT CLOCK.READOUT FREQUENCY (MHZ) = 20MHZ/READDIV." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 65535 |
| VALID_MINIMUM | = 13 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "NSPEC" |
| COLUMN_NUMBER | = 21 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 605 |
| DESCRIPTION | = "WHEN IN MEASUREMENT MORE THIS PARAMETER SPECIFIES THE NUMBER OF SPECTRA TO OBTAIN BEFORE RETURNING TO PREPARATION MODE. IF SET TO ZERO, THE INSTRUMENT WILL STAY IN MEASUREMENT MODE INDEFINITELY." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "PXPER" |
| COLUMN_NUMBER | = 22 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 607 |
| DESCRIPTION | = "PIXEL PERIOD TIME. THE TIME ELAPSED AFTER A PIXEL HAS BEEN SHIFTED OUT UNTIL THE NEXT PIXEL BEING SHIFTED OUT. THE PIXEL PERIOD IS SPECIFIED IN NUMBER OF SENSOR READOUT CLOCK PERIODS (SEE THE COLUMN OBJECT NAMED READDIV)." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "NSMPL" |
| COLUMN_NUMBER | = 23 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 609 |
| DESCRIPTION | = "NUMBER OF SAMPLES TAKEN PER PIXEL. IF MORE THAN ONE, ALL SAMPLES WILL BE AVERAGED (ARITHMETIC MEAN) BY THE INSTRUMENT BEFORE BEING SENT." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "CCSDS_COUNTER" |
| COLUMN_NUMBER | = 24 |
| BYTES | = 2 |
| DATA_TYPE | = MSB_INTEGER |
| START_BYTE | = 611 |
| DESCRIPTION | = "PACKET SEQUENCE COUNT IN THE PACKET PRIMARY HEADER (CCSDS STANDARD). THE PACKET SEQUENCE COUNT FIELD PROVIDES THE SEQUENTIAL BINARY COUNT OF EACH SPACE PACKET GENERATED BY THE USER APPLICATION IDENTIFIED BY THE APID (APPLICATIONIDENTIFIER)." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 16383 |
| VALID_MINIMUM | = 0 |
| OFFSET | = 32768 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "HK_CLOCK" |
| COLUMN_NUMBER | = 25 |
| BYTES | = 13 |
| DATA_TYPE | = CHARACTER |
| START_BYTE | = 613 |
| DESCRIPTION | = "HK CLOCK AT TIME OF MEASUREMENT STARTS. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "HK_CLOCK_OBT_SYNCRONIZATION" |
| COLUMN_NUMBER | = 26 |
| BYTES | = 13 |
| DATA_TYPE | = CHARACTER |
| START_BYTE | = 626 |
| DESCRIPTION | = "HK CLOCK AT TIME OF OBT SYNCHRONIZATION. ONE TICK OF THE HK CLOCK CORRESPONDS TO 10 MICROSECONDS." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "OBT_SYNCRONIZATION" |
| COLUMN_NUMBER | = 27 |
| BYTES | = 8 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 639 |
| DESCRIPTION | = "OBT AT TIME OF SYNCHRONIZATION IN MS." |
| UNIT | = "ms" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "TEC" |
| COLUMN_NUMBER | = 28 |
| BYTES | = 1 |
| DATA_TYPE | = MSB_UNSIGNED_INTEGER |
| START_BYTE | = 647 |
| DESCRIPTION | = "THERMO ELECTRICAL COOLER SET VALUE. WHEN THE TEC IS CONTROLLED MANUALLY THIS VALUE GIVES A TEC CURRENT LINEAR VALUE." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = "N/A" |
| VALID_MINIMUM | = "N/A" |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "TEC_VOLT" |
| COLUMN_NUMBER | = 29 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 648 |
| DESCRIPTION | = "THERMO ELECTRICAL COOLER VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 1.5 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "TEC_CURR" |
| COLUMN_NUMBER | = 30 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 652 |
| DESCRIPTION | = "THERMO ELECTRICAL COOLER CURRENT." |
| UNIT | = "mA" |
| VALID_MAXIMUM | = 5000 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "CMON5A" |
| COLUMN_NUMBER | = 31 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 656 |
| DESCRIPTION | = "VCC CURRENT." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 120 |
| VALID_MINIMUM | = 30 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "CMON1D" |
| COLUMN_NUMBER | = 32 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 660 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT CURRENT 1.5V." |
| UNIT | = "mA" |
| VALID_MAXIMUM | = 250 |
| VALID_MINIMUM | = 170 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "CMON3D" |
| COLUMN_NUMBER | = 33 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 664 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT CURRENT 3.3V." |
| UNIT | = "mA" |
| VALID_MAXIMUM | = 450 |
| VALID_MINIMUM | = 10 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "CMON5D" |
| COLUMN_NUMBER | = 34 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 668 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT CURRENT 5.0V." |
| UNIT | = "mA" |
| VALID_MAXIMUM | = 10 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "VMON5A" |
| COLUMN_NUMBER | = 35 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 672 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT VCC VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 5.3 |
| VALID_MINIMUM | = 5 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "V14_VOLT" |
| COLUMN_NUMBER | = 36 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 676 |
| DESCRIPTION | = "14V VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 15 |
| VALID_MINIMUM | = 13 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "VMON3D" |
| COLUMN_NUMBER | = 37 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 680 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT 3.3V VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 3.4 |
| VALID_MINIMUM | = 3.2 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "ISU_PDA_TEMP" |
| COLUMN_NUMBER | = 38 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 684 |
| DESCRIPTION | = "PHOTO DIODE ARRAY TEMPERATURE (ON-CHIP THERMISTOR)." |
| UNIT | = "C" |
| VALID_MAXIMUM | = 35 |
| VALID_MINIMUM | = -70 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "ISU_PCB_TEMP" |
| COLUMN_NUMBER | = 39 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 688 |
| DESCRIPTION | = "INSTRUMENT SENSOR UNIT PCB TEMPERATURE." |
| UNIT | = "C" |
| VALID_MAXIMUM | = 50 |
| VALID_MINIMUM | = -50 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "PSU_PCB_TEMP" |
| COLUMN_NUMBER | = 40 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 692 |
| DESCRIPTION | = "POWER SUPPLY UNIT PCB TEMPERATURE." |
| UNIT | = "C" |
| VALID_MAXIMUM | = 55 |
| VALID_MINIMUM | = 10 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "COOL_TEMP" |
| COLUMN_NUMBER | = 41 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 696 |
| DESCRIPTION | = "COOLING CONSOLE TEMPERATURE." |
| UNIT | = "C" |
| VALID_MAXIMUM | = 35 |
| VALID_MINIMUM | = -70 |
| END_OBJECT | = COLUMN |

| OBJECT | = COLUMN |
| --- | --- |
| NAME | = "QUA_TEMP" |
| COLUMN_NUMBER | = 42 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 700 |
| DESCRIPTION | = "QUARTZ-BODY TEMPERATURE." |
| UNIT | = "C" |
| VALID_MAXIMUM | = 35 |
| VALID_MINIMUM | = -70 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "VMON5D" |
| COLUMN_NUMBER | = 43 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 704 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT 5.0V VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 5.4 |
| VALID_MINIMUM | = 4.9 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "VMON1D" |
| COLUMN_NUMBER | = 44 |
| BYTES | = 4 |
| DATA_TYPE | = IEEE_REAL |
| START_BYTE | = 708 |
| DESCRIPTION | = "INSTRUMENT CONTROL UNIT 1.5V VOLTAGE." |
| UNIT | = "V" |
| VALID_MAXIMUM | = 1.55 |
| VALID_MINIMUM | = 1.4 |
| END_OBJECT | = COLUMN |
| OBJECT | = COLUMN |
| NAME | = "ERRORID" |
| COLUMN_NUMBER | = 45 |
| BYTES | = 1 |
| DATA_TYPE | = MSB_UNSIGNED_INTEGER |
| START_BYTE | = 712 |
| DESCRIPTION | = "ERROR IDENTIFIER FROM THE LAST IHDM (INSTRUMENT HEALTH DATA MESSAGE) THAT WAS SENT. SEE SIR2 ICD FOR MORE DETAILED INFORMATION." |
| UNIT | = "N/A" |
| VALID_MAXIMUM | = 11 |
| VALID_MINIMUM | = 0 |
| END_OBJECT | = COLUMN |
| END_OBJECT | = SIR2_SC_TABLE |
| END |  |

# Appendix: Available Software to read PDS files

The software for reading PDS files is depending on the individual operating system which is used. For IDL users with an UNIX operating system the sbnidl 4.0 package is recommended. This is available athttp://pdssbn.astro.umd.edu/nodehtml/software.shtml.  With some modifications it can also be used for other operating systems and IDL versions.

# Appendix: Directory Listing of Data Set CH1ORB-SIR2-2-NPO-V1.0

[CH1ORB-SIR2-2-NPO-V1.0]

|

|- AAREADME.TXT                 Overview of the contents and

|                               organization of the volume

|

|- VOLDESC.CAT                  The file containing a description of

|                               the VOLUME object

|

|- [CATALOG]                    A directory containing top level

|     |                         information about the SIR2 dataset in

|     |                         a set of ASCII files

|     |

|     |- CATINFO.TXT            Description of files in the CATALOG

|     |                         directory

|     |

|     |- DATASET.CAT            ASCII file providing information

|     |                         about this dataset

|     |

|     |- INST.CAT               ASCII file providing information

|     |                         about the SIR2 instrument

|     |

|     |- INSTHOST.CAT           ASCII file providing information

|     |                         about the CHANDRAYAAN-1 spacecraft

|     |

|     |- MISSION.CAT            ASCII file providing information

|     |                         about the CHANDRAYAAN-1 mission

|     |

|     |- PERSON.CAT             ASCII file providing information

|     |                         about SIR2 personnel

|     |

|     |- REF.CAT                ASCII file providing

|     |                         SIR2 references

|     |

|     |- SOFT.CAT               This is a dummy software catalog

|     |                         which is required by the PDS standard

|     |                         although no software directory is

|     |                         present in this dataset.

|

|- [DATA]                       The directory containing the actual

|     |                         SIR2 data products, organized in

|     |                         subdirectories

|     |

|     |- [ORBIT_mmmmm_TO_nnnnn] The subdirectory containing the data

|     |     |                   products from orbit number mmmmm to

|     |     |                   orbit number nnnnn

|     |     |

|     |     |- CH1SIR2_NE2_tt_Rnnnnn.FIT

|     |     |                   Data of type tt (either SC for science

|     |     |                   data including housekeeping data

|     |     |                   or HK for housekeeping data only, in

|     |     |                   binary or ASCII format, respectively)

|     |     |                   taken in orbit nnnnn

|     |     |

|     |     |- CH1SIR2_NE2_tt_Rnnnnn.LBL

|     |     |                   PDS label file of the respective FITS

|     |     |                   file

|

|- [DOCUMENT]                   A directory containing documents

|     |                         providing information about the

|     |                         SIR2 dataset

|     |

|     |- ALIGNMENT.PDF          Assessment of the alignment of the

|     |                         SIR2 instrument

|     |

|     |- CLEMENTINE_OFFSET.PDF  This report addresses considerable

|     |                         offsets which have been reported to

|     |                         occur between AMIE images projected

|     |                         onto the Moon's surface and the

|     |                         Clementine basemap.

|     |

|     |- DATA_HANDLING.PDF      Description of the content and

|     |                         format of the SIR2 telemetry data

|     |

|     |- DOCINFO.TXT            Description of files in the DOCUMENT

|     |                         directory

|     |

|     |- EAICD.DOC              Experiment to Archive Interface

|     |                         Control Document, describing how the

|     |                         data from the experiment is organized

|     |                         in this archive

|     |

|     |- GEOMETRY_INDEX.PDF     Description of the concept and the

|     |                         format of the GEO_MOON_INDEX_TABLE

|     |                         provided in the INDEX directory

|     |

|     |- OTHERDOCS.LBL          PDS label file for all documents

|     |                         in this directory without

|     |                         individual label file

|     |

|     |- USER_MANUAL.PDF        Description of the SIR2 instrument,

|     |                         its interfaces, handling, and

|     |                         operations

|

|- [INDEX]                      This directory contains the indices

|     |                         for all data products in the volume

|     |

|     |- GEO_MOON.LBL           Detached PDS label file for the

|     |                         table file GEO_MOON.TAB.  The table

|     |                         provides various geometrical parameters

|     |                         for a sample set of all measurements.

|     |                         The sample set does reasonably encompass

|     |                         the footprints of the complete set of

|     |                         all measurements, cf. the GEOMETRY_INDEX

|     |                         document in the DOCUMENT directory.

|     |

|     |- GEO_MOON.TAB           ASCII table file with one row for

|     |                         each measurement of a sample set of

|     |                         all measurements.  The format and

|     |                         content of each column is described

|     |                         in the PDS label file GEO_MOON.LBL.

|     |

|     |- INDEX.LBL              Detached PDS label file for the

|     |                         table file INDEX.TAB. The table

|     |                         provides a complete listing of all

|     |                         data products in the dataset.

|     |

|     |- INDEX.TAB              Table file with one row for each

|     |                         data product.  The format and content

|     |                         of each column is described in the

|     |                         PDS label file INDEX.LBL.

|     |

|     |- INDXINFO.TXT           Description of files in the INDEX

|     |                         directory