Mars Express Bistatic Radar Experiment

Operations Plan and Report

22 January 2006

Summary

=======

DSN Antenna:                                          14

Pass:                                                966

Orbit:                                              2606

Orbit Start Time:                        2006-01-22T06:04:44 SCET

HGA Pointing:                         Specular Track (first attempt)

Specular Condition (ERT):            06:20:00    06:50:00    07:20:00

Target Latitude (deg N):               38.64       43.35       48.33

Longitude (deg E):              95.10       91.12       88.90

Rp (km):                                  3383.823

Incidence/Reflection Angle (deg):      55.21       60.06       65.46

Slant Range (km):                      11299       11171       10622

Slew Angle:                            69.59       59.97       49.08

Doppler (carrier, fd; Hz):            -30671      -27984      -23094

Doppler (echo, fr; Hz):                -4804        1232        6934

Doppler Difference (fdd; Hz):         -25867      -29216      -30028

Earth-Mars Distance (m):                        1.448E+11

Experiment Set Up

=================

This experiment was conducted using Goldstone's DSS 14.  The support team at JPL included Danny Kahan, Gene Goltz, Essam Marouf, Sami Asmar, and Dick Simpson.  Jesse Velasco was in the NOPE area.

The MEX MOC briefing message included instructions for both uplink commanding and downlink telemetry.  Goltz confirmed with the NOPE and DSS 14 that neither was required; MEX MOC concurred after some review.

For the first time the spacecraft HGA was programmed to track the specular point.  This meant that there were surface echoes for the full hour of BSR recording (plus a little more on either end).

Performance Problems and Notes

==============================

During the experiment on 2006/021, the S-RCP maser failed.  Station personnel had investigated the problem but not found a solution in time for this experiment.  There was no substitute LNA, so the S-RCP channel was abandoned.  DR G106461 documented the data loss.

For unknown reasons, the X-RCP noise diode was enabled twice during the first mini-cal; and there was no enabling of the X-LCP diode.  So parts of steps 2-4 were repeated to get the X-LCP diode signal after the first surface echoes appeared.

There was occasional interference on X-RCP, presumably from MGS, throughout the pass.

The X-RCP and X-LCP quasi-specular echoes appeared on schedule, drifted slightly during the hour, and disappeared on schedule.  The S-LCP echo was probably there, but only detectable in a transient sense using the real-time display.

There was a shift change at 08:00; the new crew had a little trouble getting up to speed given complications such as the red lines in the post-cal procedure resulting from loss of the S-RCP channel.

It appears that the S-LCP ND was turned on about 08:06, before the receivers all went to AMB; this means there is AMB+ND, SKY, and SKY_ND calibration during the post-cal but no AMB.

Data Acquisition

================

RSRs were configured as in Table 1:

Table 1

---------------------------------------------

RSR  Channel Mode  ATT FGAIN    Operator

dB

----- ------- ---- ---- ----- ---------------

RSR3A  S-LCP   1-W auto   60    Kahan

RSR3B  X-LCP   1-W auto   60    Kahan

RSR2A   N/A    1-W auto   60    Goltz

RSR2B  X-RCP   1-W auto   68    Goltz

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Step/  RSR3A  RSR3B  RSR2A  RSR2B

Time  Action  S-LCP  X-LCP   N/A   X-RCP  Ancillary Information

----- -------- -----  -----  -----  -----  ----------------------------------

FRO      0      0      0      0   RSR frequency offset (Hz)

********************************** PRE-CAL **********************************

03:50     01   -10.1  -10.1  -26.7  -10.1  Velasco says DSS 14 is ready

18.5   11.0    0.0   10.5  RSR ATT after AUTO reset

03:57          -13.3  -10.1  -42.8  -10.2  Goltz changes S-RCP ATT setting

03:58     02   -11.5   -0.2  -45.2   -0.0  X-Band to sky

04:00     03   -16.9   -0.2  -43.8    0.0  SL to sky

04:02          -10.1  -10.0    --   -10.0  ADC amplitude (dB)

21.5   23.0          23.0  Reset ATT using AUTO

Ambient load physical temperatures

S1 = 16.44C

S2 = 19.06C

X1 = 13.25C

DSS 14 weather:

T = 48F

H = 47 percent

P = 904 mb

W = 0 mph

sky = clear

04:07     05   -10.0   -9.9    --    -9.9  Start 25 kHz recording (SCHAN=3)

04:10     05   -10.0  -10.0    --    -9.7  XR ND ON

04:15     07   -10.0  -20.9    --   -20.1  X-Band to sky

04:20     08   -10.1  -22.2    --   -22.7  XR ND OFF

04:25     09   -10.0  -19.8    --   -22.8  XL ND OFF; DSS 14 conf ND=12.5

Ambient load physical temperatures

S1 = 16.19C

S2 = 18.69C

X1 = 13.05C

04:30     10   -10.0   -9.9    --   -10.0  X-Band to AMB

04:35     11   -10.0   -9.9    --   -10.0  XL ND OFF

04:40     12   -10.0  -22.1    --   -22.6  X-Band to sky

04:42     13                               Reconfigure DTT01 for X-RCP

04:46     15   -10.0  -22.6    --   -22.6  Reconfigure DTT02 for S-LCP

DSS 14 confirms ND=12.5

04:47     16    -9.9  -22.1    --   -22.6  SL ND ON

04:50     17   -17.8  -22.2    --   -22.6  SL to sky

04:55     18   -19.2  -22.2    --          SL ND OFF

Ambient load physical temperatures

S1 = 15.94C

S2 = 18.44C

X1 = 13.00C

05:05          -18.0  -22.2    --   -22.6  Stop 25 kHz recordings

*************************** BOT, BCAL #1, SLEW #1 ***************************

05:06                                      DSS 14 going to point

05:16                 -21.9    --   -20.9  DSS 14 on point

05:21          -18.8  -21.9    --   -21.0  DSS 14 X-RCP in lock

05:22                                      X+S 1-way, TLM OFF

47.5    --    73.0  Carrier-to-noise (dB/Hz)

-2452    --   -2471  Carrier offset (Hz)

05:35                                      Start 25 kHz recording (SCHAN=3)

05:50                  47.4    --    73.4  CNR (dB/Hz)

-2852    --   -2868  Carrier offset (Hz)

06:05          -18.5  -21.7    --   -20.9  ADC amplitude (dB)

15.7   47.2    --    73.3  CNR (dB/Hz)

-1800  -3018    --   -3041  Carrier offset (Hz)

06:06                                      Begin Slew #1

06:08    FRO   +3000 +25000    --  +25000  Center echo in passband (Hz)

********************************_MINI-CAL #1 ********************************

06:09     01   -20.4  -21.6    --   -19.7  XR ND ON

06:12     02   -20.7  -21.6    --   -21.9  XR ND OFF

06:15     03   -20.4  -21.5    --   -19.3  XR ND ON (wrong diode)

Echoes on all 3 channels

06:18     04   -18.8  -21.6    --   -21.5  XR ND OFF; SL ND ON

06:21     05   -20.7  -21.7    --   -21.5  SL ND OFF

Start 100 kHz recordings (SCHAN=4)

06:24     02                               Repeat lines 3-7 (DTT01 to X-LCP)

06:25     03   -20.6  -19.5    --   -21.5  Repeat line 2 (XL ND ON)

06:28     04   -20.4  -21.3    --   -21.6  Repeat lines 1,3-7 (XL ND OFF)

************************************ BSR ************************************

06:28                   32     --     26   Echo SNR (dB/Hz)

-900    --   -1000  Echo offset (Hz)

1500                Echo bandwidth (Hz)

06:51          -20.5  -21.3    --   -21.4  ADC amplitude (dB)

34            33   Echo SNR (dB/Hz)

1220          1051  Echo offset (Hz)

1000          1000  Echo bandwidth (Hz)

07:00                                      XR interfering signal drifting -50

Hz/sec, merges with surface echo

07:10                                      Another interfering signal, 40 kHz

07:17                                      End 100 kHz recordings

*********************** MINICAL #2, SLEW #2, BCAL #2 ************************

07:17     01   -20.3  -21.1    --   -19.3  XR ND ON

07:20     02   -20.3  -21.2    --   -21.2  XR ND OFF

Begin Slew #2 back to Earth point

End surface echoes

07:23     03   -20.2  -19.2    --   -21.1  XL ND ON

07:26     04   -18.6  -21.2    --   -21.0  XL ND OFF; SL ND ON

07:29     05   -20.4  -21.1    --   -21.1  SL ND OFF

07:32           19.2   46.0    --    72.2  CNR (dB/Hz)

-7000 -28200    --  -28200  Carrier offset (Hz)

End Slew #2

07:35    FRO      0      0     --      0   Return carrier to center (Hz)

13.0   46.3    --    72.3  CNR (dB/Hz)

-1419  -3206         -3208  Carrier offset (Hz)

********************************* POST-CAL *********************************

07:50                                      EOT

07:55                                      Going to zenith

08:11     01    -9.7   -9.9    --    -9.8  All to AMB

Ambient load physical temperatures

S1 = 13.70C

S2 = 16.12C

X1 = 13.00C

08:17     02    -9.8   -9.9    --    -9.6  XR ND ON

08:20     03    -9.8  -10.0    --    -9.9  XR ND OFF

08:23     04    -9.7   -9.8    --    -9.9  XL ND ON; config DTT02 for S-LCP

08:26     05    -9.7  -10.0    --    -9.9  XL ND OFF; SL ND ON (no change)

08:29           -9.5  -10.0    --    -9.9  Late XL ND ON?

08:30     06   -18.8  -22.2    --   -22.4  All to sky

08:32     07   -18.9  -19.8    --   -22.5  SL ND OFF(?); XL ND ON

08:34          -19.4  -19.8    --   -22.5  Late SL ND OFF?

08:36     08   -20.3  -22.0    --   -22.5  XL ND OFF

08:38     09   -20.4  -22.2    --   -20.1  XR ND ON

08:41     10   -20.6  -22.1    --   -22.6  XR ND OFF

08:42     05   -18.5                       SL ND ON (Step 5, line 2)

08:43     07   -20.4                       SL ND OFF (Step 7, line 1)

08:45                                      Stop 25 kHz recordings

Ambient load physical temperatures

S1 = 13.3C

S2 = 15.8C

X1 = 12.7C

Weather at DSS 14:

T = 32F

H = 36 percent

P = 904 mb

W = 4 MPH from W

sky = partly cloudy

End of activity

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LNA = low noise amplifier

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2006-01-22

Added orbit start time to summary and LNA to acronym list: 2006-07-04