Gene, Danny:

This is my recommendation for the MEX bistatic radar operation Saturday night.  This is less challenging than Friday's experiment; but there will be good echoes for at least an hour in all four channels and the X-band FRO puts the carrier outside the 25 kHz passband, so we'll be busy.  We'll run 100 kHz sampling for slightly less than an hour (twice what we have been used to).  Essam may join us again.

Dick

- - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2006/022

-----

03:20  Begin Pre-cal at DSS 14

X-RCP FGAIN = 68 dB; all others = 60 dB

FRO_x = 0; FRO_s = 0 (changes later)

Start 4-channel 25 kHz recordings after Step 4

Stop 25 kHz recording after Step 18

05:20  Beginning of track (DL = S+X)

05:22  X TLM OFF

05:35  Begin 4-channel 25 kHz recordings

06:06  Begin Slew #1

06:08  FRO_x = +25000 Hz

FRO_s = +3000 Hz

06:09  Begin Mini-Cal #1

06:19  End Slew #1

Begin echoes (LCP > RCP)

06:21  Begin 4-channel 100 kHz recordings

06:24  End Mini-Cal #1 (end of Step 5)

07:17  End 100 kHz recordings (53 minutes)

Begin Mini-Cal #2

07:20  Begin Slew #2 (back to Earth point)

End echoes (RCP > LCP)

07:32  End Mini-Cal #2 (end of Step 5)

End Slew #2

07:35  FRO_x = 0

FRO_s = 0

08:09  S TX OFF

08:10  End of track

Begin post-cal

09:10  End of activity (if not sooner)

Stop 25 kHz recordings