From JVelasco@jftl.jpl.nasa.gov Fri Aug  4 13:57:56 2006

Return-Path: <JVelasco@jftl.jpl.nasa.gov>

Received: from jftl.jpl.nasa.gov (jftl.jpl.nasa.gov [128.149.207.22])

by magellan.stanford.edu (8.11.7p2+Sun/8.11.7) with SMTP id k74KvtF17310

for <rsimpson@magellan.Stanford.EDU>; Fri, 4 Aug 2006 13:57:55 -0700 (PDT)

Received: from jftl.jpl.nasa.gov (Not Verified[128.149.207.21]) by jftl.jpl.nasa.gov with MailMarshal (v5,0,3,100)

id <B000425e73>; Fri, 04 Aug 2006 14:00:01 -0700

Received: by jftl-mm.jpl.nasa.gov with Internet Mail Service (5.5.2657.72)

id <Q121ZWAH>; Fri, 4 Aug 2006 13:57:46 -0700

Message-ID: <88AB5DE488E2F84493F94DF423EE26D029DE6A@jftl-mm.jpl.nasa.gov>

From: "Velasco, Jesse" <JVelasco@jftl.jpl.nasa.gov>

To: DSN-NOCC <dsn-nocc@jjpl.jpl.nasa.gov>,

"Asmar, Sami W" <asmar@mail.jpl.nasa.gov>,

"Goltz, Gene L" <ggoltz@mail.jpl.nasa.gov>,

DSN-PREDICTS <DSN-PREDICTS@jjpl.jpl.nasa.gov>,

DSN-RMDCT <DSN-RMDCT@jjpl.jpl.nasa.gov>,

"'John.Reynolds@esa.int'" <John.Reynolds@esa.int>,

"Kahan, Daniel S" <kahan@mail.jpl.nasa.gov>,

DSNO&M-DSOT <DSNO&M-DSOT@jftl.jpl.nasa.gov>,

"'Dick Simpson 650-723-3525'" <rsimpson@magellan.stanford.edu>,

CDSCC <CDSCC@cdscc.nasa.gov>

Cc: "Holmes, Dwight P" <dholmes@mail.jpl.nasa.gov>,

"Thompson, Thomas W" <twthomps@mail.jpl.nasa.gov>,

"Angold, Nigel G" <nangold@mail.jpl.nasa.gov>,

DSN-MPSETD <DSN-MPSETD@jjpl.jpl.nasa.gov>,

"'Gary.Whitehead-esa.int'" <Gary.Whitehead@esa.int>,

"'Alan.Moorhouse@esa.int'" <Alan.Moorhouse@esa.int>,

"Wallick, Jack" <JWallick@jftl.jpl.nasa.gov>,

"Landon, Arthur J" <ALandon@jftl.jpl.nasa.gov>,

"Berman, Allen L" <alberman@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Beyer, Patrick E" <pbeyer@mail.jpl.nasa.gov>,

"Williams, Raymond C" <rcw@mail.jpl.nasa.gov>,

"Medina, Margaret" <MMedina@jftl.jpl.nasa.gov>,

"Berner, Jeff B" <jbberner@mail.jpl.nasa.gov>,

"Guerrero, Anamaria P" <ana@mail.jpl.nasa.gov>,

"Buck, Robert D" <rbuck@mail.jpl.nasa.gov>,

"Klose, J C" <cklose@mail.jpl.nasa.gov>,

"Recce, David J" <drecce@mail.jpl.nasa.gov>,

"Marina, Miguel" <mmarina@mail.jpl.nasa.gov>,

"Hodgin, Wendy K" <whodgin@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Eaton, Curtis A" <ceaton@mail.jpl.nasa.gov>,

"Davis, Esker B" <eskerb@mail.jpl.nasa.gov>,

"Kurtik, Susan C" <skurtik@mail.jpl.nasa.gov>

Subject: BRIEFING MESSAGE FOR MEX BI STATIC RADAR SUPPORT DOY 222 DSS-43

Date: Fri, 4 Aug 2006 13:57:18 -0700

MIME-Version: 1.0

X-Mailer: Internet Mail Service (5.5.2657.72)

Content-Type: multipart/mixed;

boundary="----_=_NextPart_000_01C6B807.8F4081C0"

Content-Length: 336975

Status: RO

Mars Express (MEX)

Briefing Message

Level 3

DSS-43 BiStatic Radar Experiment

Prepared by: J Velasco / MEX / DSN-MPSETD

Origination Date: DOY 214/2000 UTC

Revision : Original

DSS-43 DOY 222/0330 UTC

ROUTING: JFTL / JJPL / ANBE

FROM: JFTL / MPSETD / J Velasco

SUBJECT:  Mars Express Bi-Static Radar Support

NOP: 871-049-041, REV.F

SCHEDULE: Refer to DSN 7-Day Schedule for actual support times.

DOY      BOA     BOT    EOT    EOA      DSS   USER         ACTIVITY     PASS

222     0330    0530   0825   0925    DSS-43   MEX    T/P BISTATIC R/SE 1167

LEVEL / TYPE: Level 3 / Bi-Static Radar support

1. OVERVIEW:

Bi Static Radar (BSR) experiment employs the MEX spacecraft's transmitter

and a DSN 70-m antenna to receive weak S-band and X-band dual polarized RF

signals and Radio Science Receivers (RSRs) to simultaneously receive and

record these four signals. At a predetermined time during the pass, the

spacecraft slews toward Mars so that its transmitted S-band and X-band RCP

signals illuminate the planet's surface. As these RCP signals bounce off the

surface, part of the signals change polarization to Left Circular

Polarization (LCP) effectively creating four signals; 1) S-band RCP, 2)

S-band LCP, 3) X-band RCP and 4) X-band LCP.

DSN Mars Express Bistatic Radar (BSR) calibration procedures are performed

in three parts: 1) Pre-calibration, 2) Mini- calibration and 3)

Post-calibration. Section-5 details the pre-calibration procedures,

section-6 details mini-calibration procedures, and section-7 details post

calibration procedures.

2. SUPPORT DOCUMENTATION:

a. This briefing message

b. Bi-Static Radar SOE (see attachment) supersedes DSN SOE. Radio

Science shall provide start/stop times of BSR mini-pre-calibrations

near real-time.

3. SUPPORT REQUIREMENTS:

DOWNLINK:

1) Configure one DTT with table 7041HRX to support X-band operation and

to control the 12.5k noise diode.

2) Configure a second DTT with table 7041HRXS for S-band carrier only

and to control the 12.5k noise diode.

UPLINK:

No uplink requirment

COMMAND:

No commanding required

RADIO SCIENCE:

a. For remote operations the FSPR must NOT be assigned to the DSS

link. The RSRs are controlled and configured remotely by the Radio

Science group located at JPL.

b. The RSR operator will configure four radio science receivers to

simultaneously receive and record S-band and X-band RCP and LCP

polarized signals. The RSR equipment requires both S-band and

X-band 1-way and 2-way downlink predicts. The actual RSR resources

configured for support will depend on availability.

NOCT:

Ops-Chief / TSS:

Transmit X-band up/down, and X-up/S-band down predicts to the station.

Comm-Chief:

Configure a voice line: DSS /AMMOS / MEX MOC/ RS

DSOT:

No SLE telemetry return link services required

4. SPACECRAFT CONFIGURATION:

The spacecraft shall be reconfigured for X-band and S-band downlink carrier

only in accordance with the BSR SOE (see attachment).

5. PRE-CALIBRATION PROCEDURES AND CONFIGURATION

a. Pre-calibration procedures are detailed below in step-1 through

step-18. The times shown are to illustrate event chronology and event

duration:

b. Pre-calibration requires configuring the 70m-antenna microwave

subsystem so that each receiving downlink channel (S-RCP, S-LCP, X-RCP,

X-LCP) can be cycled in sequence through the four configurations listed

below.  The antenna must be pointed to zenith for pre-calibration.

1. Receiving channel into sky (zenith) with noise diode off

2. Receiving channel into sky (zenith) with noise diode on

3. Receiving channel into ambient load with noise diode off

4. Receiving channel into ambient load with noise diode on

NOTE 1:

In this briefing message DTT01 is used for X-band and DTT02 is used for

S-band, but any available DTT resource can be used.

NOTE 2:

Do not time stack Operator Directives (ODs). Pass experience has shown

that time stacking may result in rejection of ODs or delayed OD

execution.

c. Link controller perform the procedures in step 1 and advise radio

science when completed; wait at this point until directed by radio

science to proceed to step 2.

Step-01  00:00:00  US43 MOD 102 A        Confirm dichroic reflector in place

US43 CNF XRA1O1       X-RCP (LNA-1) to Sky

US43 CNF XLA2O2       X-LCP (LNA-2) to Sky

US43 CNF SLLA2C2O2    S-LCP (LNA-2) low noise to sky

US43 CNF SRDA1C1O1    S-RCP (LNA-1) via diplexer to sky

AP SCAN OFF           CONSCAN Off

AP TRK SR             Sub-reflector in Tracking Mode

Radio science operator set each RSR attenuator using the

AUTO feature. After checking for proper signal levels

(about -10.0 dB ADC amplitude on each RSR), direct the

link controller to proceed to step 2.

Step-02  00:04:00  US43 CNF XA1A2O2      LNA-2 to ambient load-1 (X-LCP)

US43 CNF XA1A1O1      LNA-1 to ambient load-1 (X-RCP)

Step-03  00:06:00  US43 CNF SA2A2C2O2    LNA-2 to ambient load-2 (S-LCP)

Step-04  00:08:00  US43 CNF SA1A1C1O1    LNA-1 to ambient load-1 (S-RCP)

Link controller and radio science operator after

completing steps 2 through 4, perform the following

procedures, (1) through (5):

(1) Link Controller report ambient load temperatures (S1,

S2, and X1) from the PRFREA display and report current

weather conditions, (temperature, humidity, and sky

conditions) to the Radio Science operator.

(2) Radio Science operator set each RSR attenuator using

AUTO feature

(3) RSR operator run radio science recorders

(4) RSR operator report to Link Controller when radio

science recorders are recording

(5) Link controller execute the following procedures at

5 min intervals and report the completion of each

step to the RSR operator:

Step-05  00:10:00  DC01 DISC

DC01 CONN DSS43XO1    Configure DTT 01 to X-RCP

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC

DC02 CONN DSS43SO1    Configure DTT 02 to S-RCP

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-06  00:15:00  DC01 SNT E            Enable X-RCP noise diode

Step-07  00:20:00  US43 CNF XRA1O1       X-RCP to sky

US43 CNF XLA2O2       X-LCP to sky

Step-08  00:25:00  DC01 SNT D            Disable X-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-LCP

DC01 CONN DSS43XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:30:00  DC01 SNT E            Enable X-LCP noise diode

Confirm noise diode selection is 12.5

Step-10  00:35:00  US43 CNF XA1A1O1      X-RCP (LNA-1) to ambient load

US43 CNF XA1A2O2      X-LCP (LNA-2) to ambient load

00:36:00  Link controller report ambient load temp (S1, S2, X1)

from the PRFREA display to the RS operator.

Step-11  00:40:00  DC01 SNT D            Disable X-LCP noise diode

Step-12  00:45:00  US43 CNF XRA1O1       X-RCP (LNA-1) to sky

US43 CNF XLA2O2       X-LCP (LNA-2) to sky

Step-13  00:50:00  DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-RCP

DC01 CONN DSS43XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-14  00:55:00  US43 CNF SRDA1C1O1    S-RCP (LNA-1) via diplexer to sky

Step-15  01:00:00  DC02 SNT D            Disable S-RCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-LCP

DC02 CONN DSS43SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Confirm noise diode is 12.5

Step-16  01:05:00  DC02 SNT E            Enable S-LCP noise diode

Step-17  01:10:00  US43 CNF SLLA2C2O2    S-LCP (LNA-2) to sky

Step-18  01:15:00  DC02 SNT D            Disable S-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-RCP

DC02 CONN DSS43SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Link controller report ambient load physical

temperatures (S1,S2, and X1) from PRFREA display to

the RS operator.

01:20:00  End of BSR noise measurement pre-cal procedure

6. MINI CALIBRATION PROCEDURES AND TIMELINE

a. Link controller, shortly after MEX begins its slew and before the

BSR surface observations begin, confirm DTT 01 and DTT 02 configurations.

b. Link controller confirm DTT 01 is configured for X-RCP; if not,

perform the following operator directives:

(1) DC01 DISC

(2) DC01 CONN DSS43XO1

(3) DC01 SET SNT NDS=12.5

(4) DC01 SET SNT MFQ=ON

c. Link controller confirm DTT 02 is configured for S-RCP; if not,

perform the following operator directives:

(1) DC02 DISC

(2) DC02 CONN DSS43SO1

(3) DC02 SET SNT NDS=12.5

(4) DC02 SET SNT MFQ=ON

d. RSR operator ensure all radio science Recorders are recording.

e. Link controller, at direction from RSR operator, begin the BSR

mini-noise calibration procedures. The times shown are to illustrate

event chronology and event duration:

Step-01  00:00:00  DC01 SNT E            Enable X-RCP noise diode

Confirm noise diode value is 12.5

Step-02  00:03:00  DC01 SNT D            Disable X-RCP noise diode

DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-LCP

DC01 CONN DSS43XO2

US43 D XMAP           Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-03  00:06:00  DC02 SNT D            Disable S-RCP noise diode

DC01 SNT E            Enable X-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-LCP

DC02 CONN DSS43SO2

US43 D SMAP           Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-04  00:09:00  DC01 SNT D            Disable X-LCP noise diode

DC02 SNT E            Enable S-LCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-RCP

DC01 CONN DSS43XO1

US43 D XMAP           Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-05  00:12:00  DC02 SNT D            Disable S-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-RCP

DC02 CONN DSS43SO1

US43 D SMAP           Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

00:15:00                        End of BSR Mini-Noise Cal.

Step-06            Link controller, repeat the BSR mini-noise calibration

procedures steps 1-5 after MEX has completed its surface

observations and before it returns to Earth point. The

start time will be provided by radio science.

7. POST CALIBRATION PROCEDURES AND TIMELINE:

Step-01  00:00:00  Link controller move antenna to zenith

US43 CNF SA1A1C1O1    S-RCP (LNA-1) to ambient load

US43 CNF SA2A2C2O2    S-LCP (LNA-2) to ambient load

US43 CNF XA1A1O1      X-RCP (LNA-1) to ambient load

US43 CNF XA1A2O2      X-LCP (LNA-2) to ambient load

DC01 DISC             Confirm DTT 01 for X-RCP

DC01 CONN DSS43XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC             Confirm DTT 02 for S-RCP

DC02 CONN DSS43SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

a. Link controller advise the RSR operator when the antenna is at

zenith and the configurations listed above have been completed.

b. Link controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display.

c. RSR operator run all radio science Recorders and record ambient

loads for 3 min. At completion, advise the link controller to proceed

with the following steps:

Step-02  00:03:00  DC01 SNT E           Enable X-RCP noise diode

Step-03  00:06:00  DC01 SNT D           Disable X-RCP noise diode

DC02 SNT E           Enable S-RCP noise diode

DC01 DISC            Reconfigure DTT 01 for X-LCP

DC01 CONN DSS43XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-04  00:09:00  DC02 SNT D           Disable S-RCP noise diode

DC01 SNT E           Enable X-LCP noise diode

DC02 DISC            Reconfigure DTT 02 for S-LCP

DC02 CONN DSS43SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-05  00:12:00  DC01 SNT D           Disable X-LCP noise Diode

DC02 SNT E           Enable S-LCP noise diode

Step-06  00:15:00  US43 CNF SRDA1C1O1   S-RCP (LNA-1) to sky

US43 CNF SLLA2C2O2   S-LCP (LNA-2) to sky

US43 CNF XRA1O1      X-RCP (LNA-1) to sky

US43 CNF XLA2O2      X-LCP (LNA-2) to sky

Step-07  00:18:00  DC02 SNT D           Disable S-LCP noise diode

DC01 SNT E           Enable X-LCP noise diode

DC02 DISC            Reconfigure DTT 02 for S-RCP

DC02 CONN DSS43SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-08  00:21:00  DC01 SNT D           Disable X-LCP noise diode

DC02 SNT E           Enable S-RCP noise diode

DC01 DISC            Reconfigure DTT 01 for X-RCP

DC01 CONN DSS43XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:24:00  DC02 SNT D           Disable S-RCP noise diode

DC01 SNT E           Enable X-RCP noise diode

Step-10  00:27:00  DC01 SNT D           Disable X-RCP noise diode

00:30:00                       End

a. Link Controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display and report the current

weather (temperature, humidity, and sky conditions) to radio science.

b. Radio Science operator stop RSRs.

c. End of BSR noise post-calibration procedures.

d. Link Controller proceed with normal tear-down procedures.

e. Re-initialization of the DTTs is recommended.

MPSETD POINTS OF CONTACT: DSN-MPSETD@jjpl.jpl.nasa.gov

<mailto:DSN-MPSETD@jjpl.jpl.nasa.gov>

MPSETD                     OFFICE    OPS WORK STATION

Jose Valencia (LEAD)  (626) 305-6308  (626) 305-6370

(626) 305-6368

Jesse Velasco (NOPE)  (626) 305-6315

Romy Paredes (NOPE)   (626) 305-6309

Greg Hewitt (NOPE)    (626) 305-6292

RADIO SCIENCE WORK STATION            (818) 393-0666