Mars Express Bistatic Radar Experiment

Operations Plan and Report

25 January 2008

Summary

=======

DSN Antenna:                                                     63

Pass:                                                          1700

Orbit:                                                         5219

Orbit Start Time:                               2008-01-24T19:47:36

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):            01:05:00   01:35:00   02:06:00

Target Latitude (deg N):               -44.89     -36.08     -28.02

Longitude (deg E):              357.81     343.01     331.64

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       56.78      57.05      57.35

Slant Range (km):                       8985.     10376.     11193.

Slew Angle (deg):                       66.45      65.90      65.29

Doppler (carrier, fd; Hz):             -11064      -8246      -5556

Doppler (echo, fr; Hz):                -24926     -16345      -8870

Doppler Difference (fdd; Hz):           13863       8099       3314

Earth-Mars Distance (m):            1.086E+11  1.086E+11  1.086E+11

Experiment Set Up

=================

This experiment was conducted using Madrid's DSS 63.  Gene Goltz, Danny Kahan, and Dick Simpson were in the Radio Science Support Area at JPL; Kahan operated the RSR's while Goltz handled communications.  Greg Hewitt was in the NOPE area until about 01:30.

The voice net was not working.  Greg Hewitt set up a telephone conference call including MEX MOC, DSS 63, the NOPE, and Radio Science.  Starting at 02:05 the call was joined by DSS 15, which was setting up for a MEX telemetry pass.  At 02:55 DSS 15 reported a fiber optics system failure; DSS 24 joined the call at 03:22 to substitute.

Performance Problems and Notes

==============================

This was the first use of DSS 63 for either Mars Express or Venus Express bistatic radar since April 2006.

Between BOT and the MiniCal #1, the X-RCP amplitude of the ADC samples rose by about 5 dB; this was not matched by increases on any of the other three channels, nor did it correlate with presence or absence of the X-RCP carrier.  During MiniCal #1, the ADC level rose about 1 dB when the noise diode was enabled; it dropped about 5 dB (to the expected level) when the diode was turned off.  By 01:38, during the surface observations, the levels had crept up again.  They remained higher than expected through MiniCal #2.  When the X-Band FRO was reset to zero after the HGA returned to Earth point, the ADC amplitude dropped by about 2 dB.  The cause for these changes is not known.

X-Band echoes were seen throughout the experiment with LCP being the stronger.  The X-Band carrier (direct) signal was intermittent, usually stronger on RCP when present.  Echo bandwidth was 1-2 kHz, meaning it was beam-limited.  Both the S-Band carrier and surface echo could be seen (after the amount of integration was increased to 10 s), with LCP generally the stronger.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    Kahan

RSR1B  X-LCP   1-W auto   60    Kahan

RSR2A  S-RCP   1-W auto   60    Kahan

RSR2B  X-RCP   1-W auto   65    Kahan

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

2008/024

--------

Pre-Cal  22:20     1      -9.7   -9.9  -10.0   -9.6  All to sky (dB)

29.0   34.0   28.0   30.0  ATT settings

22:24     2      -9.8   -0.3  -10.2   -0.1  X-Band to AMB

22:25     3      -0.5   -0.3  -10.1   -0.1  S-LCP to AMB

22:26     4      -0.6   -0.3   -0.4    0.1  S-RCP to AMB

AMB load physical temps:

S1=13.81

S2=17.12

X1=14.06

Weather conditions:

T=10.1C

H=58.8 percent

sky = partly cloudy

22:28            -9.6   -9.3   -9.9  -10.2  ADC amp after AUTO reset

30.0   31.5   29.0   28.0  ATT settings

22:30:02                                    Begin 25 kHz recording

22:32     5      -9.7   -9.2   -9.9  -10.1  Configure DTT's

22:35     6      -9.7   -9.4   -9.9   -9.9  X-RCP ND ON

22:40     7      -9.8  -21.2   -9.9  -20.0  X-Band to sky

22:45     8      -9.7  -21.1  -10.1  -22.1  X-RCP ND OFF

22:50     9      -9.7  -17.9  -10.1  -22.3  XL ND ON

22:51                                       Confirmed ND = 12.5K

22:55    10      -9.6   -9.0   -9.7  -10.1  X-Band to AMB

AMB physical temps:

S1=13.88

S2=16.75

X1=14.00

23:00    11      -9.7   -9.4   -9.8   -9.9  X-LCP ND OFF

23:05    12      -9.6  -21.2   -9.7  -22.3  X-Band to sky

23:10    13      -9.6  -21.1   -9.6  -22.3  S-RCP ND ON

23:15    14      -9.7  -21.1  -19.4  -22.2  S-RCP to sky

23:20    15      -9.8  -21.2  -21.8  -22.2  S-RCP ND OFF

23:25    16      -9.2  -21.1  -21.8  -22.2  S-LCP ND ON

23:30    17     -16.7  -21.2  -21.7  -22.3  S-LCP to sky

23:35    18     -20.9  -21.2  -21.8  -22.3  S-LCP ND OFF

AMB physical temps:

S1=14.25

S2=16.62

X1=14.25

23:40                                       Stop recording

23:55   AOS                                 S-Band visible

23:58   AOS                                 X-Band appears

23:59            26.9   39.4   52.5   67.6  CNR (dB/Hz)

-1240  -2567  -1238  -2557  Carrier offset (Hz)

2008/025

--------

00:00                                       Resume 25 kHz recording

DSS 62 in lock on S/X

00:06                                       Ranging modulation OFF

00:26            26.3   47.5   52.5   74.2  CNR (dB/Hz)

-3106  Carrier offset (Hz)

00:34                                       Begin SLEW #1

00:38            26.0   43.5   50.9   54.0  CNR (dB/Hz)

-1422  -3223  -1425  -3226  Carrier offset (Hz)

00:40           -20.8  -20.6  -21.7  -17.7  ADC amp (dB) (XR high?)

00:44           -20.7  -20.7  -21.7  -17.8  ADC amp (dB)

MiniCal1 00:45     1     -20.6  -20.6  -21.7  -16.7  XR ND ON; conf ND=12.5K

00:48     2     -20.7  -20.7  -19.4  -21.6  XR ND OFF; SR ND ON

S-Band carrier gone

XR carrier 26 dB/Hz

00:51     3     -20.7  -17.8  -21.7  -21.6  SR ND OFF; XL ND ON

00:54     4     -16.6  -20.5  -21.6  -21.5  XL ND OFF; SL ND ON

00:57     5     -20.7  -21.2  -21.4  -20.6  SL ND OFF

01:02    FRO          -12000        -12000  X-Band RX change (Hz)

33.0          25.0  X-Band echo max dB/Hz

-4900         -4900  Echo peak offset (Hz)

BSR      01:05                                       Begin 100 kHz recording

01:30    FRO           -7000         -7000  X-Band RX change (Hz)

01:38           -20.5  -20.4  -21.6  -17.3  ADC amplitude (dB)

01:58           -20.4  -20.2  -21.4  -17.5  ADC AMPLITUDE (Hz)

01:59            22.0   35.0          34.0  Echo maximum dB/Hz

02:06                                       Begin SLEW #2

End 100 kHz recordings

02:09                                       X-Band echoes gone

MiniCal2 02:10     1     -20.4  -20.4  -21.4  -16.6  XR ND ON

02:13     2     -20.4  -20.2  -19.4  -17.5  XR ND OFF; SR ND ON

02:16     3     -20.2  -17.4  -21.3  -17.6  SR ND OFF; XL ND ON

02:19     4     -16.6  -20.1  -21.3  -17.5  XL ND OFF; SL ND ON

02:22     5     -20.1  -20.2  -21.3  -17.6  SL ND OFF

02:26            19.4   19.8   22.9   37.3  Carrier to noise (dB/Hz)

-1415   3813  -1417   3815  Carrier offset (Hz)

02:36                                       End SLEW #2

02:37           -20.1  -20.0  -21.2  -16.9  ADC amplitude (dB)

26.5   47.2   51.8   74.2  Carrier to noise (dB/Hz)

-1331   4122  -1331   4124  Carrier offset (Hz)

02:40   FRO         0      0      0      0  Receiver offset (Hz)

02:42            26.3   46.4   51.8   74.2  Carrier to noise (dB/Hz)

-1328  -2867  -1327  -2867  Carrier offset (Hz)

02:44                  -19.9  -21.2  -19.9  ADC amplitude (dB)

Post-Cal 02:50   EOT                                 DSS 63 moving to zenith

02:55           -21.0  -21.2  -22.0  -22.2  DSS 63 at zenith

02:58     1      -9.7   -9.2  -10.3  -10.0  All to AMB

AMB phys temps:

S1=16.19

S2=16.19

X1=15.38

03:00     2      -9.7   -9.1  -10.1   -9.8  XR ND ON

03:03     3      -9.8   -9.1  -10.0  -10.1  XR ND OFF; SR ND ON

03:06     4      -9.8   -8.9  -10.2   -9.9  SR ND OFF; XL ND ON

03:09     5      -9.3   -9.3  -10.0  -10.0  XL ND OFF; SL ND ON

03:12     6     -16.8  -21.1  -21.9  -22.1  All to sky

03:15     7     -21.0  -17.9  -21.9  -22.2  SL ND OFF; XL ND ON

03:18     8     -20.9  -21.1  -19.7  -22.2  XL ND OFF; SR ND ON

03:21     9     -21.1  -21.3  -22.0  -19.8  SR ND OFF; XR ND ON

03:24    10     -21.2  -21.1  -21.9  -22.0  XR ND OFF

AMB physical temps:

S1=16.12

S2=16.38

X1=15.38

Weather conditions:

T=6.9C

H=65.1 percent

sky=partly cloudy

EOA      03:30                                       Stop 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2008-01-25

Added orbit start time: 2008-01-31

Typo corrected: 2008-02-11