Mars Express Bistatic Radar Experiment

Operations Plan and Report

03 Febuary 2008

Summary

=======

DSN Antenna:                                                     63

Pass:                                                          1709

Orbit:                                                         5251

Orbit Start Time:                               2008-02-02T22:43:46

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):            00:35:00   01:06:00   01:37:00

Target Latitude (deg N):                 7.24      22.20      45.88

Longitude (deg E):               70.49      65.01      69.14

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       59.79      60.87      62.77

Slant Range (km):                       8853.      6835.      4094.

Slew Angle (deg):                       60.42      58.27      54.46

Doppler (carrier, fd; Hz):               8380      13573      20337

Doppler (echo, fr; Hz):                 24903      34573      44928

Doppler Difference (fdd; Hz):          -16522     -21000     -24591

Earth-Mars Distance (m):            1.187E+11  1.187E+11  1.187E+11

Experiment Set Up

=================

This experiment was conducted using Madrid's DSS 63.  Gene Goltz and Danny Kahan were in the Radio Science Support Area at JPL.  Dick Simpson had a cold and checked in by phone from time to time.

Performance Problems and Notes

==============================

There were puzzlin g changes in the X-RCP noise level; these are indicated by "???" in Table 3.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60

RSR1B  X-LCP   1-W auto   60

RSR2A  S-RCP   1-W auto   60

RSR2B  X-RCP   1-W auto   65

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-------------------------------------------------------------------------------

Activity  Time    Step#   S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

-------- -----  --------- -----  -----  -----  -----  -------------------------

Set-Up   21:25   channel    1A     1B     2A     2B

21:25    fgain     60     60     60     65

Pre-Cal  21:41      1

21:42   att auto  -9.7   -9.9  -10.0  -10.1

21:43      2      -9.7   -0.2  -10.0   -0.2

21:45      3      -0.5   -0.3  -10.0   -0.3

21:46      4      -0.5   -0.3   -0.5   -0.3

21:47   att auto (26.0) (31.5) (23.0) (31.0) Attenuator settings (dB)

-10.0   -9.2  -10.0  -10.0  ADC amplitude (dB)

Ambient load phys temps:

S1=16.69

S2=16.19

X1=14.62

Weather:

T=5C

H=33 percent

sky=clear

21:50   rec 3 e                              Begin 25 kHz recording

21:50      5     -10.1   -9.3   -9.9   -9.9

21:55      6     -10.0   -9.1   -9.9   -9.7

22:00      7     -10.0  -21.1  -10.0  -19.7

22:05      8     -10.0  -21.1   -9.8  -22.3

22:10      9     -10.0  -18.0  -10.0  -22.1  Confirm 12.5 ND selection

22:15     10      -9.9   -8.9  -10.0  -10.0  Ambient load phys temps:

S1=16.62

S2=16.12

X1=14.56

22:20     11      -9.9   -9.3   -9.8  -10.0

22:25     12      -9.9  -21.1   -9.9  -22.1

22:30     13     -10.0  -21.2   -9.7  -22.2

22:35     14     -10.0  -21.1  -19.5  -22.1

22:40     15     -10.0  -21.0  -21.6  -22.0  Confirm 12.5 ND selection

22:45     16      -9.5  -21.0  -21.6  -22.1

22:50     17     -17.0  -21.0  -21.6  -22.1

22:55     18     -21.1  -21.1  -21.7  -22.1  Ambient load phys temps:

S1=16.94

S2=16.06

X1=14.75

23:00    end     -21.1  -21.0  -21.5  -22.1  Stop 25 kHz recording

23:15:20 ???                          -17.2

23:30    BOT     -20.9  -20.5  -21.7  -17.2  resume 25 kHz recording

23:57:44 ???                          -20.5

MiniCal1 00:15      1     -20.8  -20.5  -21.6  -16.2  confirm 12.5 ND selection

00:18      2     -20.9  -20.5  -19.4  -17.2

00:21      3     -20.9  -17.6  -21.5  -17.3

00:24      4     -16.9  -20.6  -21.5  -17.1

00:27      5     -20.8  -20.5  -21.5  -17.1

00:30    end     -20.9  -20.5  -21.5  -17.1

BSR      00:32  sfro 3/SX   +2    +18     +2    +18   Receiver offset (kHz)

00:35  begin BSR -20.8  -20.3  -21.4  -17.3  begin 100 kHz recording

01:37  end BSR   -20.5  -19.5  -21.3  -16.9  stop 100 kHz recording

MiniCal2 01:45      1     -20.6  -19.7  -21.2  -16.1  confirm 12.5 ND selection

01:48      2     -20.5  -19.3  -19.2  -17.0

01:50:09 ???                          -20.4

01:51      3     -20.5  -17.1  -21.1  -20.4

01:54      4     -16.8  -19.6  -21.1  -20.5

01:57      5     -20.5  -19.7  -21.2  -20.6

02:00    end     -20.4  -19.7  -21.1  -20.6

02:02  sfro 3/SX    0      0      0      0   Receiver offset (kHz)

02:07:26 ???                          -16.8

02:20  EOT/stow  -20.2  -19.4  -20.9  -16.7

02:20:43 ???                          -21.4

Post-Cal 02:34      1      -9.9   -9.1  -10.0   -9.9  Ambient load phys temps:

S1=16.56

S2=15.62

X1=14.25

02:38      2     -10.0   -9.2  -10.0   -9.6

02:41      3     -10.0   -9.2   -9.8   -9.9

02:44      4     -10.0   -8.9   -9.9   -9.9

02:47      5      -9.5   -9.2  -10.1   -9.8

02:50      6     -17.1  -20.8  -21.7  -21.7

02:53      7     -21.1  -17.5  -21.6  -21.7

02:56      8     -21.1  -20.7  -19.3  -21.5

02:59      9     -21.0  -20.5  -21.7  -19.0

03:02     10     -21.1  -20.4  -21.6  -21.2  Ambient load phys temps:

S1=16.44

S2=15.62

X1=14.12

Weather:

T=5C

H=28 percent

sky=clear

03:08  rec 3 d                               Stop 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original from spreadsheet: 2008-05-20