Mars Express Bistatic Radar Experiment

Operations Plan and Report

26 June 2009

Summary

=======

DSN Antenna:                                                     14

Pass:                                                          2218

Orbit:                                                         7033

Orbit Start Time:                               2009-06-26T07:35:01

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):               10:45      11:17      11:53

Target Latitude (deg N):                43.34     -22.19     -59.25

Longitude (deg E):              291.49     323.47     199.51

Rp (km):                         3375.      3375.      3375.

Incidence/Reflection Angle (deg):       62.16      62.88      57.84

Slant Range (km):                       35.31       635.      3640.

Slew Angle (deg):                       55.69      54.25      64.32

Doppler (carrier, fd; Hz):             26874.      8859     -21788.

Doppler (echo, fr; Hz):                49829.      9598.    -49475.

Doppler Difference (fdd; Hz):         -22955.      -739.     27687.

Earth-Mars Distance (m):            2.858E+11  2.858E+11  2.858E+11

Experiment Set Up

=================

This experiment was conducted using Goldstone's DSS 14.  Danny Kahan, Gene Goltz, and Dick Simpson were in the Radio Science Support Area (RSSA) at JPL.  Jesse Velasco was in the NOPE area.  Kahan handled RSRs, Goltz did communications.

Performance Problems and Notes

==============================

This was the first MEX bistatic radar experiment conducted using a calibration procedure written into the MEX Network Operations Plan rather than a briefing message sent to the station before the observation.

Echoes were visible in both X-Band polarizations before the experiment officially began.  They were visible intermittently in S-Band.  Echo strength varied during the experiment, seemingly being weaker as the specular point passed the Viking Lander 1 site and peaking as it passed the Eberswalde and Holden MSL sites.  There was a brief strong S-Band echo at 11:26, after which all echoes disappeared until about 11:33.

The ADC amplitude readings on S-LCP were not stable.  This has been seen before without there being a problem with RSR data; so no fixes were attempted in real time.

Learned that the antenna continues to 'track' if simply moved to zenith.  If 'stowed' the brakes are put on.

The S-LCP noise diode was supposed to be turned off in step 7 of the post-cal (12:58).  That did not happen, so there is no 'dark sky' measurement at the end of the post-cal.  The NMC log shows two 'snt enable' commands at 12:58 instead of a 'disable' followed by an 'enable'; so this was probably an operator error.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    Kahan

RSR1B  X-LCP   1-W auto   60    Kahan

RSR3A  S-RCP   1-W auto   60    Kahan

RSR3B  X-RCP   1-W auto   60    Kahan

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Recording bandwidth for occultations (RCP only)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-RCP  X-LCP  S-LCP  X-RCP  Notes / Comments

2008/231                 RSR2A  RSR2B  RSR3A  RSR3B

-------- ----- --------- -----  -----  -----  -----  ------------------------

Pre-Cal 08:35       1    -15.1   -9.2  -11.5   -9.9  Orig ADC amplitudes (dB)

08:40             17.5   12.5    4.0   11.5  Attens after AUTO reset

08:41       2    -10.5   -0.5   -9.9    0.1  X-Band to ambient load

08:43       3     -6.0   -0.2  -10.0    0.1  SL to ambient load

08:45       4     -6.1   -0.3   -0.6    0.1  SR to ambient load

Ambient load phys temps:

S1 = 19.19 C

S2 = 23.25 C

X1 = 16.12 C

T = 21.7 C

H = 22.3 percent

sky clear

08:49             21.5   24.5   15.5   24.5  Attens after AUTO reset

08:50                                        Begin 25 kHz recording

08:52       5    -10.1   -9.9  -10.5  -10.1  ADC amplitude (dB)

08:55       6    -10.1  -10.1  -10.4   -9.9  XR ND ON

09:00       7    -10.0  -22.1  -10.5  -20.3  X-Band to sky

09:05       8    -10.0  -22.1  -10.4  -22.9  XR ND OFF

09:10       9    -10.0  -19.8  -10.4  -22.8  XL ND ON

12.5 ND confirmed

09:15      10    -10.1   -9.7  -10.4  -10.2  X-Band to AMB load

AMB phys temps:

S1 = 18.75 C

S2 = 22.81 C

X1 = 15.69 C

09:20      11    -10.1   -9.8  -10.4  -10.2  XL ND OFF

09:25      12    -10.0  -22.1  -10.5  -22.8  X-Band to sky

09:30      13    -10.0  -22.1  -10.2  -22.7  SR ND ON

09:35      14     -9.9  -22.2  -19.5  -22.8  SR to sky

09:40      15     -9.9  -22.0  -21.5  -22.7  SR ND OFF

09:45      16     -9.9  -22.1  -21.3  -22.9  SL ND ON

09:50      17    -16.2  -22.2  -21.3  -22.8  SL to sky

09:55      18    -16.8  -22.2  -21.3  -22.9  SL ND OFF

12.5 ND confirmed

Amb load phys temps:

S1 = 18.12 C

S2 = 22.19

X1 = 15.12

10:00            -15.3  -22.2  -21.3  -22.7  SL unstable(?)

Begin moving to point

10:09            -17.3  -18.7  -19.2  -19.0  On point

BOT     10:16                                        Tracking

10:17            -16.9  -18.7  -19.2  -19.0  ADC amplitude (dB)

18.4   25.4   41.8   53.1  Pc/No (dB/Hz)

-1424  -3230  -1428  -3238  fc (carrier offset, Hz)

10:21                                        telemetry OFF

10:24             19.6   32.8   42.1   60.4  Pc/No (dB/Hz)

10:28                                        planetary coords loaded

10:29                           42.0   56.7  Pc/No, SLEW #1 beginning

10:30            23000  22000  23000  22000  FRO (SCHAN 3)

-6000         -6000  FRO (SCHAN 4)

MINICAL1            1    -16.0  -19.4  -19.6  -18.2  XR ND ON

10:33       2    -18.2  -19.6  -18.4  -19.6  XR ND OFF, SR ND ON

10:36       3    -16.9  -18.3  -19.8  -19.8  SR ND OFF, XL ND ON

10:39       4    -15.4  -19.1  -19.8  -19.8  XL ND OFF, SL ND ON

10:42       5    -18.1  -19.8  -19.9  -19.8  SL ND OFF

BSR     10:44                    7500          7500  echo frequency (Hz)

10:45                                        Begin 100 kHz recordings

10:48                    30.0          30.0  echo pwr density (dB/Hz)

11:00            -15.7  -20.4  -20.3  -20.2  ADC amplitude (dB)

25.0                echo pwr density (dB/Hz)

6674                echo peak frequency

11:02                    19.2   15.1         echo pwr density (dB/Hz)

6000   6400         echo peak frequency (Hz)

11:08                    28.0                echo pwr density (dB/Hz)

-548                echo peak frequency

11:09                    6000          6000  FRO (SCHAN 3)

11:12                                        Consistent S/X echoes

11:13                       0             0  FRO (SCHAN 3)

11:15                   -4000         -4000  FRO (SCHAN 3)

11:17           -11000  -9000 -11000  -9000  FRO (SCHAN 3)

-16.5  -20.8  -20.5  -20.5  ADC amplitude

25.0                       echo pwr density (dB/Hz)

5036                       echo peak frequency (Hz)

11:21                  -18000        -18000  FRO (SCHAN 3)

11:24                  -25000        -25000  FRO (SCHAN 3)

11:29                  -33000        -33000  FRO (SCHAN 3)

11:52            -15.2  -21.1  -20.8  -21.8  ADC amplitude (dB)

21.0                echo pwr density (dB/Hz)

2500                echo peak frequency (Hz)

11:53            23000  22000  23000  22000  FRO (SCHAN 3)

End 100 kHz recordings

Begin SLEW #2

MINICAL2 11:54      1    -13.2  -21.3  -20.9  -19.7  XR ND ON

11:57       2    -13.3  -21.3  -19.3  -21.8  XR ND OFF, SR ND ON

12:00       3    -12.8  -19.4  -20.9  -21.8  SR ND OFF, XL ND ON

12:03       4    -13.6  -21.3  -20.9  -21.7  XL ND OFF, SL ND ON

372  Weak XR MEX image (Hz)

12:04                                        End SLEW #2

12:06       5    -14.8  -21.4  -20.9  -21.7  SL ND OFF

12:09                                        End mini-cal

BCAL    12:10                0      0      0      0  FRO (SCHAN 3)

12:11            -17.1  -21.4  -20.9  -21.6  ADC amplitude (dB)

18.6   42.0   42.0   67.3  Pc/No (dB/Hz)

-1259  -2618  -1259  -2622  Peak carrier freq (Hz)

12:26            -13.2  -21.5  -21.0  -20.9  S-Band OFF

60.7  X ranging ON

EOT     12:30                                        X-Band OFF

PostCal 12:37            -17.6  -22.1  -21.4  -22.9  Antenna at stow

12:40        1   -10.1  -10.0  -10.4  -10.2  All to AMB

Ambient load phys temps:

S1 = 16.56 C

S2 = 20.56 C

X1 = 14.25 C

12:43        2   -10.0   -9.9  -10.4  -10.0  XR ND ON

12:46        3   -10.1  -10.0  -10.1  -10.2  XR ND OFF, SR ND ON

12:49        4   -10.0   -9.8  -10.5  -10.2  SR ND OFF, XL ND ON

12:52        5    -9.8   -9.8  -10.5  -10.1  XL ND OFF, SL ND ON

12:55        6   -16.8  -22.2  -21.4  -22.8  All to sky

12:58        7   -16.7  -19.9  -21.3  -22.8  SL ND OFF, XL ND ON

13:01        8   -17.0  -22.1  -19.5  -22.8  XL ND OFF, SR ND ON

13:04        9   -17.0  -22.0  -21.4  -20.3  SR ND OFF, XR ND ON

13:07       10   -16.4  -22.2  -21.5  -22.8  XR ND OFF

Ambient load phys temps:

S1 = 16.30 C

S2 = 20.38 C

X1 = 14.12 C

Local Weather:

T = 20.0 C

H = 26.5 percent

sky clear

13:10                                        End 25 kHz recordings

End of activity

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post-Analysis Summary

The specular track began at Tempe Fossae (10:45, 43N, 69W); the illumination ellipse was 79 x 37 km, with the spacecraft 29 degrees east of north viewed from the specular point.  The specular point moved southeast across cratered terrain until entering Chryse Planitia (11:00, 26N, 49W).  It passed close to the Viking Lander 1 site (11:02, 22N, 47W), then ran parallel to and somewhat west of Simud Vallis.  It crossed the eastern ends of Capri Chasma and Eos Chasma (11:13, 10S, 38W), then moved into cratered terrain.  It passed about 200 km west of Holden Crater (11:18, 27S, 38W) at periapsis (illumination ellipse 14 x 7 km, oriented 87 degrees east of north) and crossed Nirgal Vallis (11:19, 30S, 38W).  It crossed Nereidum Montes (the north rim of Argyre Plantia (11:22, 40S, 40W), the center of Argyre (11:25, 50S, 43W; illumination ellipse 18 x 9 km, oriented 104 degrees east of north), and Charityum Montes (the south rim) (11:27, 57S, 50W).  In southern cratered terrain it grazed the northwest rim of Von Karman Crater (11:30, 65S, 60W) and the northwest rim of Schmidt Crater (11:34, 71S, 80W) and passed through the northern half of Stoney Crater (11:44, 69S, 138W) before ending at (11:53, 60S, 161W; illumination ellipse 72 x 38 km, with a spacecraft azimuth of 120 degrees east of north at the specular point).

Incidence angle started at 62.2 degrees, peaked at 63.3 degrees (11:12), and ended at 58 degrees.

Slant range started at 3531 km, dropped to 629 km (11:18), and ended at 3640 km.

There were large fluctuations in X-Band echo power (10 dB) until the specular point entered the southern rim of Argyre (11:27); X-RCP and X-LCP echo powers had very similar variations, and the dielectric constant was 4.0+/-0.5.  After 11:27, the powers increased slowly and the dielectric constant was mostly 2.2-2.5.  The changes in echo power probably resulted from changes in rms surface slopes (and the evolving observational geometry) since there are apparently only two primary values for dielectric constant.

The VL-1 site appears to be in a locally rough area (echo power 300 zW, compared with 1600 zW over 18-20N); the region east of Holden Crater appears to be locally smoother (echo power 2300 zW).  Strongest echoes were from an elevated cratered planeau north of Capri Chasma (3600 zW), weakest echoes were from the south rim of Argyre (55 zW).  The center of Argyre was surpisingly smooth (2400 zW).

S-Band echoes and dielectric constant were also variable, but harder to interpret because of the higher relative noise.  Strongest echoes were obtained at 11:12 (elevated cratered platau north of Capri Chasma and 11:26 (center of Argyre).  The S- and X-Band dielectric constants are comparable on the plateau (4.0); in Argyre, the S-Band value is 4-5 while the X-Band value is 2.8-4.

Dick Simpson

Original: 2009-06-26

Filled in summary table at beginning: 2009-06-29

Added note about S-LCP noise diode failure during post-cal: 2009-07-07

Post-analysis summary added: 2009-07-28

Updated pass number: 2009-08-12