Mars Express Bistatic Radar Experiment

Operations Plan and Report

3 July 2009

Summary

=======

DSN Antenna:                                                     14

Pass:                                                          2225

Orbit:                                                         7058

Orbit Start Time:                               2009-07-03T11:57:16

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):               14:58      15:32      16:05

Target Latitude (deg N):                49.10       6.34     -70.03

Longitude (deg E):              278.91     321.47     240.01

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       61.42      60.14      58.86

Slant Range (km):                       4425.       861.      2577.

Slew Angle (deg):                       57.15      59.73      62.29

Doppler (carrier, fd; Hz):             29226.     17405.    -27093.

Doppler (echo, fr; Hz):                48990.     33606.    -53256.

Doppler Difference (fdd; Hz):         -19763.    -16201.     26164.

Earth-Mars Distance (m):            2.818E+11  2.818E+11  2.818E+11

Experiment Set Up

=================

This experiment was conducted using Goldstone's DSS 14.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL.  Dick Simpson was available by phone.

Performance Problems and Notes

==============================

DSS 14 personnel were not familiar with BSR procedures.  There was some confusion about step 5 in the pre-cal; the X-RCP receiver was not left looking at the ambient load for the normal time.  The first mini-cal was not completed; there are no X-LCP or S-LCP calibration measurements.  The surface observations themselves and all of the post-experiment calibrations appear to be nominal, however.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    UNK

RSR1B  X-LCP   1-W auto   60    UNK

RSR2A  S-RCP   1-W auto   60    UNK

RSR2B  X-RCP   1-W auto   60    UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

2009/184                 RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

Pre-Cal  12:20     1      -8.9   -9.7  -11.4  -10.0

12:21  att auto -10.1  -10.2   -9.9   -9.9

14.0   12.5    4.5   11.0  Attenuator settings (dB)

12:23     2     -10.0   -0.2   -9.8   -0.0

12:28     3      -3.0   -0.3   -9.9   +0.0

12:30     4      -3.1   -0.3   -0.6   -0.0

12:31  att auto -10.1  -10.0   -9.9   -9.9

21.5   24.5   15.5   23.5  Attenuator settings (dB)

AMB load phys temps:

S1=15.81

S2=20.75

X1=13.31"

Weather:

T=22.2C

H=29.1 percent

sky=clear

12:33   rec 3 e                             Begin 25 kHz recording

12:34     5     -10.1  -10.0   -9.8   -9.7

12:40     6     -10.1  -10.0   -9.9   -9.6

12:45     7     -10.2  -22.1   -9.9  -20.1

12:50     8     -10.2  -22.1  -10.0  -22.5

12:55     9     -10.1  -19.8   -9.9  -22.4  Confirm 12.5K ND

13:05    10     -10.1   -9.7   -9.9   -9.8  AMB load phys temps:

S1=15.56

S2=20.50

X1=13.25

13:10    11     -10.1  -10.1  -10.0   -9.9

13:15    12     -10.1  -22.1   -9.9  -22.4

13:20    13     -10.1  -22.2   -9.7  -22.4

13:25    14     -10.0  -22.1  -21.2  -22.4

13:30    15     -10.1  -22.1  -21.1  -22.5  Confirm 12.5K ND

13:35    16      -9.9  -22.1  -21.1  -22.5

13:40    17     -14.9  -22.1  -21.1  -22.4

13:45    18     -16.6  -22.1  -21.2  -22.5  AMB load phys temps:

S1=15.44

S2=20.31

X1=13.06

13:50   end     -17.2  -22.1  -21.2  -22.5  Stop 25 kHz recording

BOT      14:00                                       Collecting telemetry

until 14:30

14:30 rec 3 e   -14.5  -22.0  -21.0  -20.8  Resume 25 kHz recording

14:40 sfro 3/SX  +11k   +10k   +11k   +10k

MiniCal1 14:41     1     -15.9  -21.9  -21.1  -19.1  Confirm 12.5K ND

14:44     2                             problems - step not executed

14:47     3                             problems - step not executed

14:50     4                             problems - step not executed

14:53     5                             problems - step not executed

14:56   end                             confirm configurations and

continue

BSR      14:58 sfro 3/SX   -2k   +14k    -2k   +14k  Begin 100 kHz recording

15:26                                       Pass VL-1 site

15:31  sfro 3/X          +7k           +7k

15:35  sfro 3/X            0             0

15:38  sfro 3/X          -7k           -7k

15:40  sfro 3/S   -9k           -9l

15:41  sfro 3/X         -14k          -14k

15:44  sfro 3/X         -21k          -21k

15:47  sfro 3/X         -28k          -28k

15:54  sfro 3/X         -35k          -35k

16:05  sfro 3/SX +11k   +10k   +11k   +10k  End 100 kHz recording

MiniCal2 16:05     1     -14.1  -22.0  -21.2  -20.1  Confirm 12.5K ND

16:08     2     -13.1  -22.1  -19.3  -22.3

16:11     3     -14.2  -19.9  -21.1  -22.4

16:14     4     -14.5  -22.1  -21.2  -22.4

16:17     5     -15.0  -22.1  -21.1  -21.1

16:20   end     -14.8  -22.1  -21.1  -21.1

16:20 sfro 3/SX     0      0      0      0

16:24                                       Switch to spacecraft

pointing predicts

16:30  rec 3 d  -13.4  -22.1  -21.1  -21,1  Stop 25 kHz recording

16:30                                       Collecting telemetry

until 17:10

EOT      17:10           -14.7  -22.3  -21.3  -22.6  Resume 25 kHz recording

Post-Cal           1     -10.2  -10.1  -10.0   -9.9  AMB load phys temps:

S1=17.69

S2=23.25

X1=14.81

2     -10.1  -10.1  -10.0   -9.8

3     -10.2  -10.1   -9.9  -10.1

4     -10.1   -9.9  -10.1  -10.1

5      -9.9  -10.0  -10.3  -10.1

6     -16.0  -22.2  -21.2  -22.6

7     -16.3  -19.9  -21.3  -22.5

8     -17.0  -22.3  -19.4  -22.5

9     -16.5  -22.1  -21.3  -20.1

10     -17.5  -22.3  -21.2  -22.6  AMB load phys temps:

S1=18.12

S2=23.38

X1=15.12

Weather:

T=33.0C

H=12.5 percent

sky=clear

EOA                      -17.5  -22.1  -21.2  -22.6  End 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post-Analysis Summary

The first table below lists the time intervals over which X-Band data were processed by the MOVIE program.  Intervals were chosen primarily on the basis of times and magnitudes of receiver frequency offsets (FRO).

'Split echo' is an interval in which the FRO caused the echo to appear at two distinct frequencies.  The echo power estimate is usually correct (or nearly so) if the echo integration bandwidth was chosen large enough.  That appears

to be the case for the split echoes noted here.

'Carrier Suppression' was used when the residual carrier propagating directly from the spacecraft fell within the bandwidth over which the echo power was calculated.  Carrier signals in these data are small relative to the echo powers; the correction process has been validated previously.  For these data, carrier suppression is a minor correction.

'MOVIE Run' lists the spectra over which the MOVIE program was run and from which its output was saved.  For previous experiments, all spectra were calculated with all processing parameter selections; for 2009 that would have led to a large number of useless results, so only the parameter choices for the intervals over which they were valid were calculated and saved.

The second table gives the S-Band processing intervals (for 60 s average spectra)

X-Band  Spectra       Times       Split      Carrier     MOVIE

Interval                            Echo    Suppression    Run

-------- ------- ----------------- -----    -----------  -------

A    001-198 14:58:00-15:31:00                       001-199

B    199-240 15:31:00-15:38:00   199                 199-241

223

C    241-277 15:38:00-15:44:10   241         Y       241-277

259         Y

277         Y

D    278-337 15:44:10-15:54:10   295                 277-337

337

E    338-402 15:54:10-16:05:00                       337-402

S-Band  Spectra       Times       Split      Carrier     MOVIE

Interval                            Echo    Suppression    Run

-------- ------- ----------------- -----    -----------  -------

A    001-032 14:58:00-15:30:00                       001-032

B    033-042 15:30:00-15:40:00               Y       030-043

C    043-053 15:40:00-15:51:00               Y       043-053

D    054-067 15:51:00-16:05:10                       053-067

Dick Simpson

Original: 2009-08-10

Added Post-Analysis Summary: 2009-08-17

Various edits: 2009-09-23