Mars Express Bistatic Radar Experiment

Operations Plan and Report

10 February 2010

Summary

=======

DSN Antenna:                                                     14

Pass:                                                           N/A

Orbit:                                                         7830

Orbit Start Time:                              2010-02-10T07:18:34

HGA Pointing:                                              Specular

Start    Mid Point      End

--------  ----------  --------

Specular Condition (ERT):               07:29      08:00      08:31

Target Latitude (deg N):               -31.61     -36.63     -42.18

Longitude (deg E):              306.47     300.74     295.83

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       45.12      50.08      55.67

Slant Range (km):                      11124.     11004.     10451.

Slew Angle (deg):                       89.76      79.83      68.65

Doppler (carrier, fd; Hz):            -31177.    -32315.    -31455.

Doppler (echo, fr; Hz):                -4485.      1497.      7149.

Doppler Difference (fdd; Hz):         -26691.    -33812.    -38603.

Earth-Mars Distance (m):            1.024E+11  1.024E+11  1.024E+11

NB: The DSN is no longer routinely using pass numbers; so this entry in the

summary will not be maintained in this and future reports.

Experiment Set Up

=================

This experiment was conducted using the Goldstone DSS 14.  Danny Kahan, Gene Goltz, and Dick Simpson were in the Radio Science Support Area (RSSA) at JPL.  No one was in the NOPE area.

Performance Problems and Notes

==============================

The S-RCP maser was red, so no S-RCP data were collected (DR N105983).  Instead, the RSR normally used to collect S-RCP data was configured to capture a backup copy of S-LCP.  S-RCP is expected to be available again on 2010/046.

The first operator at DSS 14 (until 08:00 UTC) was rusty on radio science procedures and was using a newer version of the Network Operations Plan (NOP, Rev. M).  Numbering of steps in the pre-cal was different, causing some confusion in the RSSA.  Goltz downloaded the newer version of the NOP section, and the pre-cal was going more smoothly by Step 10.  The pre-cal required more than two hours to complete, but there was extra time because BSR activities did not start until 50 minutes into the track.

Real-time monitoring during Mini-Cal #2 indicated that the X-RCP noise diode was not turned off at 08:35 and may have remained on through the early parts of the post-cal.

The three identical ambient load physical temperatures at the beginning of the post-cal are very suspicious.  Goltz commented on this at the time, but the operator did not respond.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR3A  S-LCP   1-W auto   60    UNK

RSR3B  X-LCP   1-W auto   60    UNK

RSR2A  S-LCP   1-W auto   60    UNK

RSR2B  X-RCP   1-W auto   60    UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  Steps are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-LCP  X-RCP  Notes / Comments

2010/041       (new NOP) RSR3A  RSR3B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

FGAIN                       60     60     60     60  Set FGAIN

PreCal   04:35      1     -6.7   -9.8   -6.0  -10.2  ADC amplitudes (dB)

04:36 att auto  -10.1   -9.8   -7.9   -9.6  ADC amplitudes (dB)

04:42      2    -10.1   -2.5   -7.9   -2.2  X-Band to AMB load

04:43      3     -5.4   -2.4   -1.7   -2.1  S-LCP to AMB load

04:45      4     -5.4   -2.5   -3.7   -2.2  S-RCP to AMB (no op)

04:48 rec 3 e                               Begin 25 kHz recording

04:49      5                                AMB load physical temps:

S1= 9.69C

S2=13.31C

X1=11.12C

Local weather:

T= 3.2C

H=92.6 percent

Light-heavy rain

05:00      6     -9.7  -10.1   -9.8   -9.9  ADC amplitudes (dB)

26.0   14.0   27.0   22.5  Atten settings (dB)

05:02      7     -9.8  -10.1  -10.0   -9.7  X-RCP ND ON

05:07      8     -9.8  -18.3  -10.0  -17.1  X-Band to sky

05:12      9     -9.7  -18.4  -10.1  -18.4  X-RCP ND OFF

05:17     10     -9.8  -17.5   -9.9  -18.6  X-LCP ND ON

Confirmed 12.5K ND

05:19     12                                AMB load physical temps:

S1= 9.69C

S2=13.25C

X1=11.00C

05:22     11     -9.8  -10.0   -9.9  -10.0  X-Band to AMB

05:27     13     -9.7  -10.1  -10.0   -9.9  X-LCP ND OFF

05:32     14     -9.8  -18.5  -10.0  -18.6  X-Band to sky

05:37     15     -9.7  -18.5   -9.9  -18.5  S-RCP ND ON (no op)

05:43     16                                S-RCP to sky (no op)

05:47     17     -9.8  -18.1   -9.9  -18.0  S-RCP ND OFF (no op)

Confirmed 12.5K ND

05:53     18     -9.8  -18.0   -9.9  -17.8  S-LCP ND ON

05:58     19    -14.3  -17.2  -14.1  -17.1  S-LCP to sky

06:03     20    -14.7  -17.2  -14.7  -17.1  S-LCP ND OFF

AMB load physical temps:

S1= 9.56C

S2=13.06C

X1=11.00C

06:08     21    -14.5  -17.3  -14.6  -17.1  End pre-cal

Stop 25 kHz recording

BOT      06:11                                       DSS 14 on point

06:16           -10.2  -17.3  -10.2  -17.2  No MEX signal

06:33           -10.5  -16.2  -10.5  -16.3  No MEX signal

06:39           -11.3  -16.3  -11.4  -16.5  ADC amplitude (dB)

-1143  -2191         -2191  Carrier offset (Hz)

28.0   40.5          69.0  Carrier-to-noise (dB/Hz)

06:40            28.0   35.5          62.0  Carrier-to-noise (dB/Hz)

06:47                                       MEX MOC wants telemetry;

says project expects it

06:49                                       MEX MOC backs off,

withdraws request

06:50           -11.8  -16.2  -12.3  -14.6  ADC amplitude (dB)

29.3   35.2   29.2   62.2  Carrier-to-noise (dB/Hz)

06:50 rec 3 e                               Resume 25 kHz recording

06:54            29.6   41.3   29.6   69.1  CNR (dB/Hz); TLM OFF

Clipping on XR SCHAN 2,

No clipping on SCHAN 3

07:05            29.5                 68.8  CNR (dB/Hz)

Begin SLEW #1

07:08            10.5                 39.5  CNR (dB/Hz)

07:10                                       DSS 14 switch to

planetary coordinates

07:11 sfro 3/SX   +5K +29.6K    +5K +29.6 K Rcvr tuning offset (Hz)

MiniCal1 07:12      1    -13.8  -16.5  -13.6  -14.5  XR ND ON (ADC amp, dB)

07:14           -6340                       SL carrier offset (Hz)

07:15      2    -15.1  -16.5  -15.8  -15.2  XR ND OFF, SR ND ON

07:18      3                                SR ND OFF, XL ND ON

07:21      4    -15.1  -16.5  -16.0  -14.7  XL ND OFF, SL ND ON

07:26      5    -12.6  -16.6  -13.8  -15.3  SL ND OFF

BSR      07:29            20.0   32.0          25.0  Echo to noise (dB/Hz)

rec 4 e                               Begin 100 kHz recording

End SLEW #1

07:33            1160  -4800                Echo offset (Hz)

-6400                       Carrier offset (Hz)

07:57                   -191   2501    -31  Echo offset (Hz)

33.1   23.3   27.9  Echo peak (dB/Hz)

-6502                       Carrier offset (Hz)

19.8                       Carrier peak (dB/Hz)

08:13                   3111   3230   2965  Echo offset (Hz)

30.9   20.3   26.3  Echo peak (dB/Hz)

-6525                       Carrier offset (Hz)

20.3                       Carrier peak (dB/Hz)

08:27                   4960   3970   4913  Echo offset (Hz)

31.6   21.4   27.8  Echo peak (dB/Hz)

08:30           -17.1  -16.4  -16.7  -16.5  ADC amplitude (dB)

08:31 rec 4 d                               End 100 kHz recording

Begin SLEW #2 (to Earth)

MiniCal2 08:32      1    -16.3  -16.5  -15.9  -14.4  XR ND ON

08:35      2    -16.2  -16.4  -15.6  -14.8  XR ND OFF, SR ND ON

08:38      3    -14.0  -15.7  -14.3  -14.7  SR ND OFF, XL ND ON

08:41            22.0                       CNR (dB/Hz)

-6554                       Carrier offset (Hz)

08:41      4    -16.9  -16.5  -17.0  -14.6  XL ND OFF, SL ND ON

08:44      5    -17.1  -16.6  -16.9  -14.6  SL ND OFF

08:47     end                               DSS 14 switch back to

spacecraft pointing

08:48               0      0      0      0  Rcvr tuning offset (Hz)

26.2   54.2  CNR (dB/Hz)

-1560  -3719  Carrier offset (Hz)

08:50                                       End SLEW #2

08:51                          25.8   70.4  CNR (dB/Hz); clipping on

X-RCP SCHAN 3

EOT      09:07                                       Early end of track

09:12  Zenith   -15.0  -16.3  -14.9  -16.2  ADC amplitudes (dB)

PostCal  09:26      1     -9.7  -10.1   -9.9  -10.0  All to AMB

09:27                                       AMB load physical temps:

S1=10.75C

S2=10.75C

X1=10.75C

These are suspicious ...

09:30      2     -9.8  -10.1  -10.1   -9.9  XR ND ON

09:33      3     -9.8  -10.1  -10.0   -9.9  XR ND OFF, SR ND ON

09:36      4     -9.7  -10.0   -9.8   -9.9  SR ND OFF, XL ND ON

09:39      5     -9.8  -10.0   -9.9   -9.9  XL ND OFF, SL ND ON

09:42      6    -14.3  -16.5  -14.3  -16.4  All to sky

09:45      7    -14.4  -15.7  -14.6  -16.5  SL ND OFF, XL ND ON

09:48      8    -14.5  -16.5  -14.5  -16.4  XL ND OFF, SR ND ON

09:51      9    -14.4  -16.6  -14.5  -15.7  SR ND OFF, XR ND ON

09:54     10    -14.3  -16.5  -14.9  -16.5  XR ND OFF

AMB load physical temps:

S1= 9.31C

S2=12.56C

X1=10.75C

Local weather:

T= 3.7C

H=89.1 percent

W=5.6 mph

Sky=partly cloudy

EOA      10:00  end                                  End 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

NOP = Network Operations Plan

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

s/c = spacecraft

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post Analysis Summary

Good echoes throughout.

Some echoes may not be beam-limited.  For example at 07:48:54 the S-Band half-power width is less than 363 Hz (possibly as little as 150 Hz).  The X-Band width is 448 Hz.  The X-Band width is less than half the beam-limited width for 0.1 rms slopes.  The inferred X-Band rms slope is 2.43 deg (35S, 57W).

X-Band system temperatures were 40-70K, with an unexplained rise while the S-Band pre-cal was being completed that persisted through the remainder of the pass.  There may be a small contribution from Mars to X-Band Tsys. S-LCP seemed normal (25-28K).

There was strong correlation between X-RCP and X-LCP noise whenever the LNAs were looking out the horn (during tracking and also during the pre-cal).  The correlation was not present when the LNAs were looking at the ambient load.

Dick Simpson

Original: 2010-02-10

Minor edits; added Post-Analysis Summary: 2010-02-17

Minor edits for odd characters: 2011-05-10