Mars Express Bistatic Radar Experiment

Operations Plan and Report

07 February 2012

Summary

=======

DSN Antenna:                                                     14

Orbit:                                                        10325

Orbit Start Time:                               2012-02-07T02:57:55

HGA Pointing:                                              Specular

Start   Ascend EQX     End

--------  ----------  --------

Specular Condition (ERT):               05:45      06:34      07:14

Target Latitude (deg N):               -40.14      -0.19      69.56

Longitude (deg E):              128.78     213.69     130.61

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       70.74      64.47      47.31

Slant Range (km):                       5553.       697.      3808.

Slew Angle (deg):                       38.53      51.07      85.37

Doppler (carrier, fd; Hz):             25743.     42993.    -10035.

Doppler (echo, fr; Hz):                40634.     36661.    -48836.

Doppler Difference (fdd; Hz):         -14891.      6331.     38801.

Earth-Mars Distance (m):            1.129E+11  1.129E+11  1.129E+11

Experiment Set Up

=================

This experiment was conducted using the Goldstone DSS 14.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL.  Jesse Velasco was in the JPL NOPE area.  Dick Simpson monitored by telephone from Stanford.

Performance Problems and Notes

==============================

Kahan noted that there is no longer an S-Band maser at DSS 14; both LNAs are HEMT devices.  Simpson requested performance data on the HEMTs, which Kahan will track down.

Echoes were visible in all four channels as soon as the experiment began.

There was brief, but strong, S-Band interference at 06:17-06:18.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

-----------------------------------------------------

RSR   Channel Mode  ATT FGAIN        Operator

dB

------ ------- ---- ---- ----- ----------------------

RSR1A   S-LCP   1-W auto   60    UNK

RSR1B   X-LCP   1-W auto   60    UNK

RSR2A   S-RCP   1-W auto   60    UNK

RSR2B   X-RCP   1-W auto   65    UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  Steps are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

2012/038                 RSR2A  RSR2B  RSR3A  RSR3B

-------- ----- --------- -----  -----  -----  -----  ------------------------

Set-Up   01:50   FGAIN      60     60     60     65  FGAIN settings (dB)

Pre-Cal  03:07     1     -16.9   -9.6  -11.8  -10.2  All rcvrs to SKY

03:08  att auto -10.1  -10.3  -10.2  -10.1  ADC amplitude

5.0   12.5    9.5   11.5  Attenuator settings (dB)

03:10     2     -10.2   -0.3  -10.3   -0.1  XR and XL to AMB

03:11     3      -2.7   -0.2  -10.2   -0.0  SL to AMB

03:13     4      -2.7   -0.3   -1.8   -0.1  SR to AMB

03:15  att auto  -9.7  -10.0  -10.0  -10.0  Reset attenuators

12.5   24.5   18.5   24.0  Attenuator settings (dB)

Ambient load temps:

S1=17.88C

S2=20.31C

X1=16.62C

Local weather:

T=9.8C

H=21.2 percent

sky=partly cloudy

03:18  rec 3 e                              Begin 25 kHz recording

03:20     5      -9.7  -10.0   -9.9  -10.0  Configure noise diodes

03:25     6      -9.7  -10.1  -10.1   -9.8  XR=AMB+ND

03:30     7      -9.7  -22.1   -9.9  -20.3  XR=SKY+ND; XL=SKY

03:35     8      -9.6  -22.3  -10.0  -22.6  XR=SKY

03:40     9      -9.7  -20.5   -9.8  -22.6  XL+SKY+ND

Confirmed 12.5K ND

03:45    10      -9.8   -9.9   -9.9   -9.9  XR=AMB; XL=AMB+ND

Ambient load temps:

S1=17.75C

S2=20.12C

X1=16.38C

03:50    11      -9.7  -10.0   -9.8   -9.9  XL=AMB

03:55    12      -9.7  -22.2   -9.8  -22.6  XR=SKY; XL=SKY

04:00    13      -9.7  -22.1   -9.7  -22.5  SR=AMB+ND

04:05    14      -9.7  -22.1  -18.0  -22.6  SR=SKY+ND

04:10    15      -9.6  -22.1  -19.2  -22.5  SR=SKY

Confirmed 12.5K ND

04:15    16      -9.5  -22.2  -19.2  -22.4  SL=AMB+ND

04:20    17     -16.6  -22.2  -19.0  -22.4  SL=SKY+ND

04:25    18     -17.6  -22.3  -19.2  -22.5  SL=SKY

Confirmed 12.5 K ND

Ambient load temps:

S1=17.62C

S2=20.00C

X1=16.12C

04:30   end     -17.7  -22.2  -19.2  -22.5  Stop 25 kHz recording

BOT      05:00           -17.3  -20.1  -18.4  -16.5  X-Band AOS

05:07                                       S-Band AOS

05:22 rec 3 e   -17.3  -20.5  -18.7  -17.0  TLM off

05:23                                       Resume 25 kHz recording

05:28                                       DSS 14 switch to Mars

pointing predicts

05:29 sfro 3        0   +10k      0   +10k  25 kHz RSR offsets, Hz

sfro 4        0   -18k      0   -18k  100 kHz offsets, Hz

MiniCal1 05:30     1     -16.5  -20.8  -18.7  -16.1  XR=SKY+ND

Confirmed 12.5K ND

05:33     2     -14.8  -20.7  -17.6  -16.8  XR=SKY; SR=SKY+ND

05:36     3     -14.2  -19.5  -18.8  -17.3  SR=SKY; XL=SKY+ND

05:39     4     -12.9  -20.7  -18.6  -17.4  XL=SKY; SL=SKY+ND

05:42     5     -13.8  -20.9  -18.7  -17.4  SL=SKY

05:45   end     -13.8  -20.9  -18.7  -17.5

BSR      05:45  rec 4 e                              Begin 100 kHz recording

06:15  sfro 3            +2k           +2k  RSR tuning offset (Hz)

06:29  sfro 3            -5k           -5k  RSR tuning offset (Hz)

06:30  sfro 3     -4k           -4k         RSR tuning offset (Hz)

06:34  sfro 3           -13k          -13k  RSR tuning offset (Hz)

06:36  sfro 3     -7k           -7k         RSR tuning offset (Hz)

06:37  sfro 3           -21k          -21k  RSR tuning offset (Hz)

06:39  sfro 3           -27k          -27k  RSR tuning offset (Hz)

06:40  sfro 3    -10k          -10k         RSR tuning offset (Hz)

06:41  sfro 3           -33k          -33k  RSR tuning offset (Hz)

06:43  sfro 3           -39k          -39k  RSR tuning offset (Hz)

06:44  sfro 3    -13k          -13k         RSR tuning offset (Hz)

06:47  sfro 3           -48k          -48k  RSR tuning offset (Hz)

06:51  sfro 3    -15k          -15k         RSR tuning offset (Hz)

07:11  sfro 3           -45k          -45k  RSR tuning offset (Hz)

07:14 rec 4 d                               End 100 kHz recording

MiniCal2 07:15     1     -14.0  -21.6  -19.2  -16.6  XR=SKY+ND

Confirm 12.5K ND

07:18     2     -14.0  -21.7  -17.9  -17.4  XR=SKY; SR=SKY+ND

07:21     3     -13.9  -20.1  -19.1  -17.5  SR=SKY; XL=SKY+ND

07:24     4     -13.0  -21.6  -19.1  -17.4  XL=SKY; SL=SKY+ND

07:27     5     -14.0  -21.7  -19.0  -17.5  SL=SKY

07:30   end                                 DSS 14 return to S/C

pointing predicts

07:31 sfro 3        0      0      0      0  All tuning offsets 0

EOT      07:44           -14.0  -22.2  -19.3  -22.4  Move antenna to zenith

(do not stow)

Post-Cal 07:52     1      -6.0  -10.0   -9.9   -9.9  All to AMB

Ambient load temps:

S1=16.75C

S2=18.19C

X1=15.31C

07:55     2      -6.0  -10.0   -9.9   -9.7  XR=AMB+ND

07:58     3      -6.0  -10.2   -9.7   -9.9  XR=AMB; SR=AMB+ND

08:01     4      -5.9   -9.8   -9.9   -9.9  SR=AMB; XL=AMB+ND

08:04     5      -5.8  -10.0   -9.9   -9.9  XL=AMB; SL=AMB+ND

08:07     6     -13.0  -22.1  -19.0  -22.4  All to SKY; SL=SKY+ND

08:10     7     -14.0  -20.4  -19.0  -22.3  SL=SKY; XL=SKY+ND

08:13     8     -13.9  -22.2  -17.8  -22.6  XL=SKY; SR=SKY+ND

08:16     9     -14.1  -22.1  -19.0  -20.2  SR=SKY; XR=SKY+ND

08:19    10     -14.0  -22.2  -19.0  -22.5  XR=SKY

Ambient load temps:

S1=16.69C

S2=18.12C

X1=15.19C

Local weather:

T=8.3C

H=24.3 percent

sky=cloudy

EOA      08:23   end

AMB = ambient load

BOT = Beginning of Track

BW = bandwidth

C = Celsius

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

k = kilo, thousand

K = Kelvin

LOS = loss of signal

ND = noise diode

No = noise power

NOP = Network Operations Plan

occn = occultation

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

s/c = spacecraft

SKY = receiver looking a sky

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post Analysis Summary

There were major signal dynamics in both amplitude and Doppler during this experiment.  X-Band echo power was computed by defining an echo window and letting the FRO changes keep the signal within it.  This appears to have been successful except for X-Band over about 06:19-06:31 when the echoes were strongest and moving fastest and may have spilled over into the noise window(s).  The error in Tsys appears to be less than 5 percent and the error in echo power is probably no more than that since most power within the echo window was echo rather than noise (error will be in scaling X-RCP and X-LCP power and not in removing the noise pedestal – and it should be comparable in both polarizations.

There is a Brewster angle transition in both the S- and X-Band data, but it is a little hard to tell exactly when it occurs because the signal powers are decreasing quickly at about the same time as the dominant polarization is switching.  Everything happens as the specular point crosses Stealth near periapsis (northward equator crossing is at about 147 W longitude) and the incidence angle passes through 60 degrees (at about 13 N latitude).

10 s average X-Band spectra merged with 60 s S-Band average spectra appears to be a good combination.

S-Band dielectric constant may be slightly higher than the X-Band value until about 06:10.  Signals are reasonably strong and stable over this time; so these should be reliable values.  Until about 06:24, the S-Band dielectric constant holds at about 3, then drops linearly to 2 just before 06:30.  Meanwhile, X-Band dielectric constant is increasing to almost 5 at 06:24, then shoots upward to almost 7 before dropping sharply by 06:30.  All four channels have weak echoes by 06:35, rise slightly around 06:37, drop again around 06:40, and then have modest and approximately constant values for the remainder of the experiment.  After about 06:30, S-Band dielectric constant approximately tracks the X-Band value, though the weak echo signals (especially S-LCP) are reminders that we should be cautious in deriving dielectric constant.

Noise power density looks reasonable in all four channels.  S-LCP was by far the largest.  S-RCP and X-LCP were comparable, and X-RCP was lowest.  The two X-Band noise power densities drop more sharply with elevation angle, as expected.

Dick Simpson

Original: 2012-02-08

Updated comment about S-Band interference: 2012-04-10

Added Post Analysis Summary: 2013-06-19

Minor edits to Post Analysis Summary: 2013-06-26