Mars Express Bistatic Radar Experiment

Operations Plan and Report

08 April 2012

Summary

=======

DSN Antenna:                                                      43

Orbit:                                                         10542

Orbit Start Time:                                2012-04-10T08:44:42

HGA Pointing:                                               Specular

Start  Olympus Mons     End

--------  ----------   --------

Specular Condition (ERT):               10:30       10:41      11:01

Target Latitude (deg N):                22.01       18.33      10.13

Longitude (deg E):              232.35      229.21     233.90

Rp (km):                         3394.       3394.      3394.

Incidence/Reflection Angle (deg):       51.72       52.97      55.82

Slant Range (km):                       9200.       8660.      7502.

Slew Angle (deg):                       76.56       74.07      68.37

Doppler (carrier, fd; Hz):            -12707.     –12604.    -11822.

Doppler (echo, fr; Hz):                19243.      21689.     26045.

Doppler Difference (fdd; Hz):         -31950.     -34292    -37867.

Earth-Mars Distance (m):            1.194E+11   1.194E+11  1.194E+11

Experiment Set Up

=================

This experiment was conducted using the Canberra DSS 43.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL.  Dick Simpson monitored by telephone from Stanford.

Performance Problems and Notes

==============================

At 09:40 Kahan reported that the S-RCP and S-LCP channels looked very similar; the station reported a similar observation five minutes later.  After some adjustments by DSS 43 personnel, the S-LCP channel looked better (09:51).  There may have been an incorrect operator directive at the end of the pre-cal, which connected the S-LCP RSR to the S-RCP LNA.

The station requested permission to switch to planetary pointing predicts at about 09:56 and reported the action completed at 09:59.  Kahan reported a 20 dB drop in X-RCP during the change, but most of it was recovered by 09:59.

X-Band echoes were seen from the beginning with LCP slightly stronger than RCP; S-Band echoes were seen intermittently.

X-RCP FGAIN was changed from 65 to 70 dB to prevent clipping during the first spacecraft calibration (09:51-10:09).  There was no explicit change during the second spacecraft calibration (11:25-11:36), but Kahan said he would make the correction if it appeared necessary.  He said nothing, so the assumption is that FGAIN remained at 65 except during the first spacecraft calibration.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

-----------------------------------------------------

RSR   Channel Mode  ATT FGAIN        Operator

dB

------ ------- ---- ---- ----- ----------------------

RSR1A   S-LCP   1-W auto   60      UNK

RSR1B   X-LCP   1-W auto   60      UNK

RSR2A   S-RCP   1-W auto   60      UNK

RSR2B   X-RCP   1-W auto 65-70     UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  Steps are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

2012/101                 RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

07:25   fgain      60     60     60     65  Set FGAIN attenuation (dB)

PreCalc  08:00     1      -9.3   -7.9  -13.0   -9.9  ADC voltage levels (dB)

08:03  att auto  -9.8   -9.8   -9.9  -10.1  Auto reset attenuation

17.0   13.5   11.5    9.5  Attenuator setting (dB)

08:05     2      -9.7   -0.1   -9.8   -0.1  X-Band to ambient load

08:06     3      -1.4   -0.1   -9.9   -0.1  S-LCP to ambient load

08:08     4      -1.4   -0.1   -0.4   -0.0  S-RCP to ambient load

08:10  att auto -10.0   -9.7   -9.9  -10.0  Reset attenuators

26.5   25.5   23.0   22.0  Attenuator settings (dB)

Ambient load phys temps:

S1=19.50C

S2=20.88C

X1=20.56C

Local weather

T=9.9C

H=52.6 percent

sky=clear

08:12  rec 3 e                              Begin 25 kHz recording

08:15     5     -10.1   -9.6   -9.8  -10.1  All to ambient load

08:20     6      -9.9   -9.7   -9.8   -9.8  XR ND ON

08:25     7     -10.0  -22.0   -9.8  -18.9  X-Band to sky

08:30     8      -9.9  -22.0   -9.8  -22.6  XR ND OFF

08:35     9      -9.8  -19.6   -9.9  -22.5  XL ND ON

Confirm 12.5K ND

08:40    10      -9.9   -9.5   -9.8  -10.0  X-Band to ambient load

Ambient load phys temps:

S1=18.94C

S2=20.56C

X1=20.12C

08:45    11      -9.9   -9.7   -9.8  -10.1  XL ND OFF

08:50    12      -9.9  -22.1   -9.7  -22.5  X-Band to sky

08:55    13     -10.0  -22.1   -9.6  -22.6  SR ND ON

09:00    14      -9.9  -21.7  -19.6  -22.6  SR to sky

09:05    15      -9.9  -21.7  -21.2  -22.6  SR ND OFF

Confirm 12.5K ND

09:10    16      -9.6  -21.8  -21.3  -22.5  SL ND ON

09:15    17     -18.0  -21.8  -21.3  -22.5  SL to sky

09:20    18     -26.3  -21.8  -21.3  -22.6  SL ND OFF

Confirm 12.5K ND

Ambient load phys temps:

S1=19.06C

S2=20.25C

X1=20.06C

end      09:25  rec 3 d  -26.5  -21.5  -21.3  -22.6  End 25 kHz recording;

antenna to point

BOT      09:45           -19.1  -21.5  -21.3  -17.6  S/X AOS, X-TLM OFF

09:51                                   70  Reset XR FGAIN (dB)

09:53  rec 3 e                              Resume 25 kHz recording

09:59           -19.1  -21.0  -20.9  -17.7  DSS 43 to Mars antenna

pointing predicts

10:09 sfro 3/XS   +8k   +32k    +8k   +32k  Rcvr tuning offsets (Hz)

65  Reset XR FGAIN (dB)

MiniCal1 10:10     1     -19.2  -21.2  -20.8  -15.9  XR ND ON

Confirm 12.5K ND

10:13     2     -19.1  -21.2  -19.3  -17.7  XR ND OFF, SR ND ON

-23.6

10:16     3     -19.0  -19.2  -21.0  -21.7  SR ND OFF, XL ND ON

10:19     4     -17.8  -21.3  -20.9  -17.8  XL ND OFF, SL ND ON

10:22     5     -19.1  -21.2  -20.9  -17.9  SL ND OFF

10:22   end     -19.1  -21.1  -20.9  -17.9  All to sky

BSR      10:30  rec 4 e  -19.1  -21.2  -21.0  -17.7  Begin 100 kHz recording

end      11:01  rec 4 d  -19.1  -21.2  -21.0  -17.8  End 100 kHz recording

MiniCal2 11:02     1     -19.2  -21.2  -21.0  -16.0  XR ND ON

Confirm 12.5K ND

11:05     2     -19.1  -21.3  -19.2  -17.8  XR ND OFF, SR ND ON

11:08     3     -19.1  -19.3  -21.0  -17.8  SR ND OFF, XL ND ON

11:11     4     -17.9  -21.2  -20.9  -21.6  XL ND OFF, SL ND ON

11:14     5     -19.2  -21.3  -21.0  -21.6  SL ND OFF

11:17   end     -19.1  -21.3  -21.0  -21.5  All to sky

11:18 sfro 3/XS     0      0      0      0  Remove all FROs

DSS 43 returns to s/c

pointing predicts

EOT      11:36           -19.1  -21.3  -20.9  -20.2  Move antenna to zenith;

do not stow

PostCal  11:41     1     -10.0   -9.7   -9.7   -9.6  All to ambient loads

Ambient load phys temps:

S1=19.19C

S2=19.44C

X1=20.12C

11:48     2      -9.9   -9.6   -9.6   -9.2  XR ND ON

11:51     3      -9.8   -9.7   -9.5   -9.8  XR ND OFF, SR ND ON

11:54     4     -10.0   -9.5   -9.7   -9.7  SR ND OFF, XL ND ON

11:57     5      -9.7   -9.6   -9.6   -9.6  XL ND OFF, SL ND ON

12:00     6     -18.1  -21.7  -21.3  -22.0  All to sky

12:04     7     -19.3  -19.4  -21.3  -22.3  SL ND OFF, XL ND ON

12:08     8     -19.2  -21.7  -19.4  -22.2  XL ND OFF, SR ND ON

12:11     9     -19.2  -21.6  -21.4  -18.4  SR ND OFF, XR ND ON

12:14    10     -19.2  -21.7  -21.2  -22.3  XR ND OFF

Ambient load phys temps:

S1=19.19C

S2=19.25C

X1=20.12C

Local weather:

T= 4.3C

H=81.8 percent

sky=clear

EOA      12:18  rec 3 d  -19.3  -21.7  -21.3  -22.1  End 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

BW = bandwidth

C = Celsius

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

k = kilo, thousand

K = Kelvin

LOS = loss of signal

ND = noise diode

No = noise power

NOP = Network Operations Plan

occn = occultation

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

s/c = spacecraft

SKY = receiver looking a sky

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post Analysis Summary

X-Band echoes were easily detectable in both polarizations at both 10 and 60 second averaging; the measurement procedure appears to have been good.  X-RCP system temperature was only 0.3K lower than X-LCP system temperature during the track; from the pre- and post-cals, the difference was 0.5-1.5K.  X-LCP may have been under-estimated by a few percent; this would make the X-Band dielectric constant slightly larger than calculated, where the values are already surprisingly large.

S-Band echoes are barely detectable.  With 60 second averaging, the S-LCP echo can be seen during the final 6-8 minutes and (perhaps) during the first couple minutes.  The S-RCP echo is almost impossible to see except for the final 3-4 minutes.  If either echo is smaller than 20 zW, it is probably not worth talking about dielectric constant; that means none of the S-Band measurements can be turned into dielectric constant, since none of the S-RCP values ever reached 20 zW.

S-RCP system temperature during the track was about 2K higher than during the pre- and post-cals.  S-LCP system temperature was about 5K higher during the track than during the pre- and post-cals.  This seems too high.  Further, S-LCP was typically 15K higher than S-RCP, implying that the microwave configuration at S-Band was not correct.

It would be interesting to estimate how much of the total echo was captured in X-RCP and X-LCP: that is, what fraction of the oblique forward scattering was specular.

Dick Simpson

Original: 2012-04-10

Post Analysis Summary added: 2013-05-08