Mars Express Bistatic Radar Experiment

Operations Plan and Report

20 April 2012

Summary

=======

DSN Antenna:                                                      63

Orbit:                                                         10578

Orbit Start Time:                                2012-04-20T20:24:52

HGA Pointing:                                               Specular

Start    Gale/MSL       End

--------  ----------   --------

Specular Condition (ERT):               22:31       10:41      23:59

Target Latitude (deg N):                20.03       -4.94     -47.62

Longitude (deg E):              146.53      136.56     234.82

Rp (km):                         3394.       3394.      3394.

Incidence/Reflection Angle (deg):       53.42       62.59      84.43

Slant Range (km):                       8124.       5183.      1275.

Slew Angle (deg):                       73.16       54.83      11.14

Doppler (carrier, fd; Hz):            -13590.      -9217.     30497.

Doppler (echo, fr; Hz):                23669.      30357.     30645.

Doppler Difference (fdd; Hz):         -37260.     -39574.      1085.

Earth-Mars Distance (m):            1.299E+11   1.299E+11  1.299E+11

Experiment Set Up

=================

This experiment was conducted using the Madrid DSS 63.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL.  Dick Simpson monitored by telephone from Stanford.

Performance Problems and Notes

==============================

There were no significant problems before, during, or after the experiment. Echoes were seen during real time in both S- and X-Band and on both polarizations.

In order to characterize the receiving system better, we stepped through some FGAIN and ATT settings after the pre-cal and again after the post-cal.  These noise data will be studied for better understanding of where the post-antenna noise sources lie and how the FGAIN and ATT settings affect their appearance in the data.  The RSRs continued to record data through these tests, so there is no gap between the pre-cal and tracking data for this experiment.

Danny Kahan noticed that X-RCP was clipping when the spacecraft antenna was pointed toward Earth and telemetry was turned off; he increased the X-RCP FGAIN setting to 70 until the beginning of SLEW #1.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

-----------------------------------------------------

RSR   Channel Mode  ATT FGAIN        Operator

dB

------ ------- ---- ---- ----- ----------------------

RSR1A   S-LCP   1-W auto   60      UNK

RSR1B   X-LCP   1-W auto   60      UNK

RSR2A   S-RCP   1-W auto   60      UNK

RSR2B   X-RCP   1-W auto 65-70     UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  Steps are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

2012/111                 RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

Set-up   19:15   FGAIN      60     60     60     65  FGAIN settings (dB)

Pre-Cal  19:56      1     -7.7  -15.9  -12.8   -9.2  ADC amplitudes (dB)

20:00 att auto   -9.8  -10.2   -9.8   -9.6  Auto reset attenuators

22.5   18.0   15.5   25.0  Attenuator settings (dB)

20:02      2     -9.8   -0.3   -9.9   -0.4  X-Band to ambient load

20:03      3     -0.6   -0.5   -9.9   -0.4  S-LCP to ambient load

20:04      4     -0.7   -0.4   -1.0   -0.4  S-RCP to ambient load

20:05 att auto   -8.0   -9.8   -9.9   -5.0  Auto reset attenuators

31.5   29.5   26.0   31.5  Attenuator settings (dB)

Ambient load phys temps:

S1=19.69C

S2=20.75C

X1=16.38C

Local weather:

T=14.1C

H=55.0 percent

sky=partly cloudy

20:08  rec 3 e                              Begin 25 kHz recording

20:08      5     -8.0   -9.9   -9.8   -4.9  ADC amplitudes (dB)

20:13      6     -8.0   -9.8   -9.9   -4.9  X-RCP ND ON

20:18      7     -8.0  -21.6   -9.9  -15.4  X-Band to sky

20:23      8     -7.9  -21.7   -9.7  -17.2  X-RCP ND OFF

20:28      9     -8.0  -19.3    9.8  -17.3  X-LCP ND ON

Confirm 12.5K ND

20:33     10     -8.0   -9.6   -9.8   -5.1  X-Band to ambient load

Ambient load phys temps:

S1=19.69C

S2=20.56C

X1=16.12C

20:38     11     -8.0   -9.9   -9.8   -5.1  X-LCP ND OFF

29:43     12     -8.0  -21.7   -9.9  -17.2  X-Band to sky

20:48     13     -8.0  -21.7   -9.7  -17.3  S-RCP ND ON

20:53     14     -8.0  -21.7  -19.4  -17.2  S-RCP to sky

20:58     15     -8.0  -21.7  -21.2  -17.3  S-RCP ND OFF

Confirm 12.5K ND

21:03     16     -7.7  -21.7  -21.2  -17.2  S-LCP ND ON

21:08     17    -16.6  -21.7  -21.2  -17.2  S-LCP to sky

21:13     18    -18.8  -21.7  -21.3  -17.2  S-LCP ND OFF

Confirm 12.5K ND

Ambient load phys temps:

S1=19.56C

S2=20.38C

end                                                  X1=15.69C

ATT test 21:18   FGAIN      55     55     55     55  Reset FGAIN

21:21    ATT    -13.8  -16.6  -16.3  -12.0  Manual ATT reset

26.5   24.5   21.0   26.5  Attenuator settings (dB)

21:24   FGAIN      60     60     60     65  Restore FGAIN settings

21:27    ATT    -18.8  -21.6  -21.3  -17.2  Manual ATT restore

31.5   29.5   26.0   31.5  Attenuator settings (dB)

end      21:30                                       Move DSS 63 to point

BOT      21:45           -18.7  -21.4  -21.3  -12.3  ADC amplitudes (dB)

AOS      21:53                                       X-AOS, S-AOS, TLM off

22:00           -18.8  -21.3  -21.3  -15.6  DSS 63 to Mars pointing

22:08 sfro 3/SX   +4k   +31k    +4k   +31k  SCHAN3 RSR offsets (Hz)

sfro 4/X          +20k          +20k  SCHAN4 RSR offsets (Hz)

MINICAL1 22:13      1    -18.7  -21.2  -21.2  -11.8  XR ND ON

Confirm 12.5K ND

22:16      2    -18.8  -21.4  -19.4  -12.5  XR ND OFF, SR ND ON

22:19      3    -18.8  -19.2  -21.2  -12.4  SR ND OFF, XL ND ON

22:22      4    -16.6  -21.2  -21.2  -12.3  XL ND OFF, SL ND ON

22:25      5    -18.8  -21.3  -21.2  -12.2  SL ND OFF

END      22:28    end    -18.6  -21.3  -21.2  -12.2  ADC amplitudes (dB)

BSR      22:31  rec 4 e                              Begin 100 kHz recording

23:26 sfro 3/X          +24k          +24k  SCHAN3 RSR offsets (Hz)

23:34 sfro 3/X          +17k          +17k  SCHAN3 RSR offsets (Hz)

23:39 sfro 3/X          +11k          +11k  SCHAN3 RSR offsets (Hz)

23:44 sfro 3/X           +4k           +4k  SCHAN3 RSR offsets (Hz)

23:49 sfro 3/X           -2k           -2k  SCHAN3 RSR offsets (Hz)

23:55 sfro 3/X           -6k           -6k  SCHAN3 RSR offsets (Hz)

23:59  rec 4 d  -18.4  -21.0  -20.9  -16.3  End 100 kHz recording

END BSR        sfro 3/SX  +60k   +60k   +60k   +60k  SCHAN3 RSR offsets (Hz)

MINICAL2 00:03      1    -18.7  -21.1  -20.9  -11.6  XR ND ON

Confirm 12.5K ND

00:06      2    -18.3  -20.9  -19.2  -12.2  XR ND OFF, SR ND ON

00:09      3    -18.4  -18.9  -20.9  -11.7  SR ND OFF, XL ND ON

00:12      4    -16.4  -21.0  -20.9  -11.7  XL ND OFF, SL ND ON

00:15      5    -18.5  -20.8  -20.9  -11.7  SL ND OFF

END      00:18    end    -18.4  -20.9  -20.8  -11.8  DSS 63 to s/c pointing

sfro 3/SX     0      0      0      0  SCHAN3 RSR offsets (Hz)

EOT      00:21           -18.4  -20.8  -20.8  -11.7  ADC amplitudes (dB)

-16.3  DSS 63 to zenith

PostCal  00:26      1     -7.8   -9.6   -9.9   -5.0  All to ambient loads

Ambient load phys temps:

S1=19.69C

S2=18.81C

X1=16.19C

00:30      2     -7.9   -9.8  -10.1   -4.8  XR ND ON

00:33      3     -7.8   -9.8   -9.8   -4.9  XR ND OFF, SR ND ON

00:36      4     -7.9   -9.6  -10.0   -5.0  SR ND OFF, XL ND ON

00:39      5     -7.7   -9.8   -9.9   -5.0  XL ND OFF, SL ND ON

00:42      6    -16.6  -21.6  -21.3  -17.0  All to sky

00:45      7    -18.8  -19.3  -21.2  -17.1  SL ND OFF, XL ND ON

00:48      8    -18.8  -21.5  -19.5  -17.0  XL ND OFF, SR ND ON

00:51      9    -18.7  -21.7  -21.3  -15.5  SR ND OFF, XR ND ON

00:54     10    -18.9  -21.7  -21.2  -17.0  XR ND OFF

Ambient load phys temps:

S1=19.44C

S2=18.81C

X1=16.12C

Local weather:

T=11.2C

H=65.6 percent

END                                                   sky=partly cloudy

ATT Test 01:00  FGAIN       55     55     55     55  Reset FGAIN

01:05   ATT     -13.7  -16.4  -16.2  -11.8  Manual ATT reset

26.5   24.5   21.0   26.5  Attenuator settings (dB)

01:10  FGAIN       60     60     60     65  Restore FGAIN

01:15   ATT     -18.8  -21.5  -21.2  -17.0  Manual ATT restore

31.5   29.5   26.0   31.5  Attenuator settings (dB)

EOA      01:20 rec 3 d   -18.7  -21.6  -21.2  -17.0  End 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

BW = bandwidth

C = Celsius

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

k = kilo, thousand

K = Kelvin

LOS = loss of signal

ND = noise diode

No = noise power

NOP = Network Operations Plan

occn = occultation

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

s/c = spacecraft

SKY = receiver looking a sky

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post Analysis Summary

Results appear pretty nominal.  On both S and X there is a Brewster angle transition at about 23:15, other changes mostly show up in all four channels at once, RCP increases until almost the end when it drops slightly as incidence angle starts to decrease again, LCP starts to drop after 23:20 and plunges after 23:38 (the S-LCP echo is lost in noise after about 23:45).

Dielectric constant at S-Band is larger than at X-Band until about 23:15 and is less than X-Band dielectric constant over 23:15-23:45.

Noise power density increases approximately linearly throughout the experiment except that it increases slightly faster in X-RCP after 23:30; the difference from linear is probably no more than 5 percent, however.

Dick Simpson

Original: 2012-04-21

Corrected typos: 2013-04-04

Added Post-Analysis Summary: 2013-06-17

Minor edits for bad characters: 2013-07-08