820-013
Deep Space Mission System (DSMS)
External Interface Specification

- JPL D-16765

0158-Monitor

Deep Space Mission System Interface for Mission Monitor Data

Original Release:  April 30, 2002

Revision A:  June 10, 2003

| Document Owner: ______________________________ ________ M. Levesque Date Service System Development Engineer Telecommunications Services |  | Approved By: ______________________________ ________ S. Kurtik Date Service System Manager Telecommunications Services |
| --- | --- | --- |
| Approved by: ______________________________ ________ W. Tai Date Manager, IND Systems Engineering and Standards Office |  | Approved by: ______________________________ ________ J. Stipanuk Date Interface Engineer and Release Authority |

- Prepared By:

| ______________________________ ________ K. Levister Date MIA CDE |  |  |
| --- | --- | --- |

- Reviewed By:

| ______________________________ ________ K. Levister Date MIA CDE |  | ______________________________ ________ E. Barkley Date Service System Development Engineer Monitor and Control Services |
| --- | --- | --- |
| ______________________________ ________ V. I. Altunin Date Telecommunications & Mission Systems Manager |  | ______________________________ ________ R. D. Benson Date Telecommunications & Mission Systems Manager |
| ______________________________ ________ K. I. Moyd Date Telecommunications & Mission Systems Manager |  | ______________________________ ________ P. E. Beyer Date Telecommunications & Mission Systems Manager |
| ______________________________ ________ A. F. Chang Date Telecommunications & Mission Systems Manager |  | ______________________________ ________ R. L. Gillete Date Telecommunications & Mission Systems Manager |
| ______________________________ ________ S. Waldherr Date Telecommunications & Mission Systems Manager |  | ______________________________ ________ D. P. Holmes Date Telecommunications & Mission Systems Manager |
| ______________________________ ________ P. T. Poon Date Telecommunications & Mission Systems Manager |  | ______________________________ ________ B. G. Yetter Date Telecommunications & Mission Systems Manager |
| ______________________________ ________ D. Recce Date DSMS Chief Operations Engineer |  | ______________________________ ________ C. Chang Date DSMS System Engineering |
| ______________________________ ________ J. Berner Date Service System Development Engineer Telecommunications Services |  |  |

This page is intentionally left blank.

- Change Log

| Revision | Issue Date | Pages / Sections Affected | Change Summary |
| --- | --- | --- | --- |
| New issue | 04/30/2002 | All |  |
| Revision A | 06/10/2003 | 1.5.2.3 section 2 section 3 Appendix A Appendix B | Added new monitor parameter format. Added additional description to the functional overview. Restructured SFDU description to reference CHDO descriptions in other DSMS documents. Restricted max SFDU size to 4500 bytes. Restricted channel source to 13. Added 26M monitor channels; added lots of new monitor channels for the 34M/70M subsystems; reformatted and reorganized tables of monitor channels. New. |

This page is intentionally left blank.

- Contents

Section	Page

1.	Introduction	1-1

1.1	Purpose and Scope	1-1

1.1.1	Applicability of This Release	1-1

1.2	Revision and Control	1-1

1.3	Relationship to Other DSMS Documents	1-1

1.4	Applicable Documents	1-1

1.5	Notation and Conventions	1-2

1.5.1	Notations	1-2

1.5.2	Conventions	1-2

1.5.2.1	Conventions for Figures	1-2

1.5.2.2	SFDU Field Formats	1-3

1.5.2.3	Monitor Parameter Formats	1-3

1.6	Abbreviations	1-5

2.	Functional Overview	2-1

3.	Detailed Interface Description	3-1

3.1	Data Exchange	3-1

3.1.1	Data Transfer	3-1

3.1.2	Data Format	3-1

3.1.2.1	0158-Monitor SFDU Format	3-1

4.	Appendix A.  List of Monitor Parameters	4-1

5.	Appendix B Guide to the 34M/70M Subsystems	5-1

- Figures

Figure	Page

Figure 3-1 Format of 0158-Monitor SFDU	3-2

Figure 3-2. Format of 0158-Monitor Secondary CHDO (73)	3-4

- Tables

Table	Page

Table 3-1.  Format of 0158-Monitor Secondary CHDO (73)	3-5

Table A-1 26M Monitor Data	4-1

Table A-2 34M/70M Monitor Data	4-8

Table A-3 Channel ID Lookup	4-155

This page is intentionally left blank.

# Introduction

- Section 1

- Introduction

## Purpose and Scope

This module defines the format and distribution of the real-time, mission monitor data generated by the DSN ground data systems of the Deep Space Mission System (DSMS).  The 0158-Monitor records are produced by the Monitor Interface Assembly (MIA) of the Network Monitor and Control Subsystem (NMC) and by the 26M Monitor and Control Processor (MCP). The 0158-Monitor record is generated for missions to analyze the quality of the telecom link and to confirm events and configuration for automated telemetry and command service operations.

### Applicability of This Release

Revision A supercedes the original release of this module.  Revision A adds additional monitor parameters which provide configuration, status, and performance information for the new subsystems delivered as part of the DSMS upgrade.  Appendix A has been reorganized to provide more information about each monitor parameter and to more clearly indicate each parameter’s origin and applicability.  There are no changes to the data format, but section 3 has been reorganized to eliminate duplication of information contained in other DSMS documents.

## Revision and Control

Revisions or changes to the information herein presented may be initiated according to the procedure specified in the Introduction to document 820-013.

## Relationship to Other DSMS Documents

None.

## Applicable Documents

| 813-109, D-17818 | Preparation Guidelines and Procedures for Deep Space Mission System (DSMS) Interface Specifications (DSMS Internal Use Only) |
| --- | --- |
| 810-047 | DSN Antenna and Facility Identifiers |
| 820-013, D-16765 | DSMS External Interface Specification |
|  | OPS-6-21 Standard Code Assignments |
|  | 0171-Telecomm-NJPL JPL Created SFDU Structures (formerly known as SFOC-5-SYS-*DU-NJPL) |
|  | 0172-Telecomm-CHDO DSMS Created CHDO Structures |
|  | 0172-Telecomm-CHDO-000 NULL Tertiary |
|  | 0172-Telecomm-CHDO-001 Aggregate |
|  | 0172-Telecomm-CHDO-002 Primary |
|  | 0172-Telecomm-CHDO-027 Channelized Data Quaternary |
|  | 0172-Telecomm-CHDO-028 Channelized Data Area |
| 820-019 | DSN System Requirement, Detailed Interface Design Standards (DSMS Internal Use Only) |
|  | DFL-1-1 Network-Level Data Flow Standard (DSMS Internal Use Only) |
| ANSI/IEEE Std 754-1985 | IEEE Standard for Binary Floating Point Arithmetic |

## Notation and Conventions

### Notations

| CHDO | A Compressed Header Data Object (CHDO) is a Type-Length-Value Object (TLVO) that has been specifically formatted to reduce the overhead introduced by the SFDU standard for structure within a single “logical” data object. For a more detailed explanation of CHDOs, please refer to the 820-013, 0171-Telecomm-NJPL software interface specification. |
| --- | --- |
| channel ID | An identifier for a monitor parameter consisting of a letter and a number. The channel ID for a monitor parameter is included in the monitor record along with the parameter value, making the monitor parameter self-identifying. |

### Conventions

#### Conventions for Figures

The following conventions are used in figures defining the format of a data record or piece of a data record.

 All byte offsets are assumed to be relative to the beginning of a structure or substructure.  The first byte of a structure is called offset 0 (normally shown as the left byte of two); the second byte is at offset 1 (normally shown as the right byte of two), etc.

 If a field in the SFDU requires more than one byte, the most significant byte is at the lowest-number byte offset with each succeeding byte in the next higher byte offset so that the least significant byte is in the highest-number byte offset.

 Bits in a byte are labeled 0 through 7, where the 0th bit is the most-significant or sign bit (left-most bit) and the 7th bit is the least-significant bit (right-most bit).  For fields using more than one byte, bits are labeled 0 - n correspondingly.

#### SFDU Field Formats

The formats of the fields in the 0158-Monitor SFDU are described in section 3.8, Field Types, in 820-013, 0172-Telecomm-CHDO.

#### Monitor Parameter Formats

In the list of monitor parameters in Appendix A, each monitor item’s format is described as one of the following: integer, float, modcomp, double, or string.  In the 0158-Monitor SFDU, the monitor parameter value will appear in the data CHDO in either the length_value or lc_value fields.  (See 820-013, 0172-Telecomm-CHDO-028 for a description of these fields.)  The format, units/precision, and range of values information in Appendix A allows the customer to correctly interpret the channel value in the 0158-Monitor SFDU.  The following subsections describe the meaning of monitor parameter formats.

##### Integer

An integer format is used to to express integral quantities, using two’s complement notation.  In general, the range for an integer is [-1 * (2n/2)] to [(2n/2) – 1], where n is the number of bits and can be a maximum of 32.

When creating the 0158-Monitor SFDU, if a particular monitor parameter value will fit into less than 32 bits, then the monitor value will be stored in either the length_value field, which is always 8 bits long, or in the lc_value field using the least number of bits needed.  For example, if a monitor parameter’s value is 12, it will be stored in the length_value field and the lc_value field won’t exist for that monitor parameter.  If the monitor value is 300, the value will be stored in the lc_value field, which will be 16 bits long (the minimum allowable size for lc_value).  The maximum size for an integer value is 32 bits.

##### Float

Float is the 32-bit IEEE floating point format defined in document ANSI/IEEE Std 754-1985.

##### Modcomp

Modcomp is a 32-bit floating point format which can express quantities in the approximate range of -2255 (about -1076) to +2255 (about +1076) with a precision of 22 bits (over six significant decimal digits).  Floating point numbers will be expressed with a sign, a 9-bit exponent, and a 22-bit mantissa.  The bits are arranged as follows:

Word 1		Bit 1:		Sign

Bits 2-10:	Exponent

Bits 11-16:	Mantissa

Word 2		Bits 1-16:	Mantissa (continued)

- The exponent value is defined as follows:

Exponent Value (HEX)	Floating Point Value

000 				2-256 x Fraction Value (1 > F > 0)

- 20 x Fraction Value

1FF				2255 x Fraction Value

A negative number is represented as the two's complement of the entire 32-bit pattern.

##### Double

Double is the 64-bit IEEE floating point format defined in document ANSI/IEEE Std 754-1985.

##### String

A string is 1 or more ASCII characters.  The maximum size for a string is 242 characters.  If a DSN subsystem produces a monitor parameter of string format whose value is longer than 242 characters, it will be truncated to 242 when it is put into the 0158-Monitor SFDU.  DSN subsystems can produce strings with 0 characters.  In this case the corresponding channel value in the 0158-Monitor SFDU will be a single ASCII NULL character.

Similar to the integer format, the least number of bytes needed will be used when storing the monitor parameter value in the 0158-Monitor SFDU.  So a 1 character string will appear in the length_value field and strings 2 or more characters long will appear in the lc_value field.

## Abbreviations

Abbreviations used in this document are defined with the first textual use of the term. Abbreviations appearing in this module are:

| AMMOS | Advanced Multimission Operations System |
| --- | --- |
| ASCII | American Standard Code for Information Interchange |
| CHDO | Compressed Header Data Object |
| DSMS | Deep Space Mission System |
| DSN | Deep Space Network |
| ID | Identifier |
| LVO | Label Value Object |
| MCP | Monitor and Control Processor |
| MIA | Monitor Interface Assembly |
| NMC | Network Monitor and Control |
| PDW | Parameter Descriptor Word |
| SFDU | Standard Formatted Data Unit |

This page is intentionally left blank.

# Functional Overview

- Section 2

- Functional Overview

The 0158-Monitor SFDUs contain a collection of monitor data generated by the DSN subsystems.  For each spacecraft being tracked, the latest available monitor data is periodically packaged into an SFDU and delivered to the customer.  Monitor record generation can be set to on or off for each spacecraft supported by the DSMS.  If on, the generation interval is 5 seconds.  If off, no monitor records are created for that spacecraft.

The 0158-Monitor record is in a CHDO-structured SFDU format.  Each SFDU consists of a fixed-length header and a variable-length data CHDO containing self-identifying monitor parameters.  To understand this data format, it may be helpful to think of it as being like packet telemetry.  The SFDU is the equivalent of the transfer frame.  The SFDU label and CHDOs up to the data CHDO are like the transfer frame header and the data CHDO is like the transfer frame data field. Each monitor parameter is formatted into a little structure similar in concept to a packet.  These "packets" consist of an identifier and the monitor data value.  The data CHDO will contain one or more monitor "packets."  Like packet telemetry, the monitor "packets" are not guaranteed to appear in every SFDU and the order in which they appear may vary from SFDU to SFDU.  Unlike packet telemetry the monitor "packets" are not allowed to span SFDUs.  Because the number of monitor "packets" can vary, the size of the data CHDO can vary, causing the overall SFDU size to vary from one record to the next.

The monitor parameter identifier is a channel ID.  Channels and channel IDs are concepts borrowed from telemetry.  In telemetry data, a channel is a single measurement taken by an instrument or sensor onboard a spacecraft.  In the context of this document, a channel is equivalent to a monitor parameter.  The channel identifier consists of a letter and number.  For monitor data the letter is always M; the valid range for numbers is 0 - 4095.  The set of monitor parameters, along with their channel IDs, are listed in Appendix A of this document.  This set is used for all missions receiving 0158-Monitor records.

When interpreting the monitor data, the user should be aware that some channels have another channel associated with them that indicates the validity of the data.  Whether a channel has an associated validity channel depends on the originating DSN subsystem and how it generates and outputs its monitor data.  In Appendix A the description for validity channels will list the channels to which a validity channel applies.

This page is intentionally left blank.

# Detailed Interface Description

- Section 3

- Detailed Interface Description

## Data Exchange

### Data Transfer

The 0158-Monitor SFDUs contain a set of monitor data describing the current status, configuration, and performance of the DSN equipment supporting a spacecraft pass.  The 0158-Monitor SFDUs are created at 5-second intervals, forming a monitor data stream.  A separate stream is generated for each antenna providing support and for each spacecraft supported by that antenna.

### Data Format

The 0158-Monitor record is delivered to the end user encapsulated in a Standard DSN Block (SDB).  An SDB consists of a DSN Data Delivery (DDD) header, a data portion, and a DDD trailer.  The data portion in this case is the 0158-Monitor SFDU described by this module.  Module DFL-1-1 of document 820-019 defines the format of the DDD header and trailer.

#### 0158-Monitor SFDU Format

The 0158-Monitor SFDU has a fixed format in the sense that an SFDU will always consist of the CHDOs defined below in Figure 3-1.  The data CHDO will normally contain the list of monitor parameters documented in Appendix A.  If the value for a monitor parameter is unavailable or invalid at the time a particular SFDU is being created, that monitor parameter will not be included in the data CHDO.  Therefore, the length of the data CHDO, and thus the entire SFDU, can vary.

The 0158-Monitor SFDU is made up of the following components: an SFDU label, an aggregation CHDO, a primary CHDO, a secondary CHDO, a tertiary CHDO, a quaternary CHDO, and a data CHDO.  Figure 3-1 shows the overall format of the 0158-Monitor SFDU.  The SFDU label and the aggregation, primary, tertiary, quaternary, and data CHDOs are used in many SFDUs created by the DSMS so this document will only specify the field values contained within these components that are unique to the 0158-Monitor SFDU.  The secondary CHDO is unique to the 0158-Monitor SFDU and its format and description are documented below.

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                      CCSDS SFDU Label                         |

|                        NJPL2I000093                           |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                       Aggregation CHDO                        |

|                            Type = 1                           |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                          Primary CHDO                         |

|                             Type = 2                          |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                          Secondary CHDO                       |

|                             Type = 73                         |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                          Tertiary CHDO                        |

|                             Type = 0                          |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                          Quaternary CHDO                      |

|                             Type = 27                         |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

|                                                               |

|                              Data CHDO                        |

|                              Type = 28                        |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

Figure 3-1 Format of 0158-Monitor SFDU

##### SFDU Label

The format of the SFDU label is documented in 820-013, 0171-Telecomm-NJPL.  The following fields within the label will have these values in the 0158-Monitor SFDU.

| Field Name | Value |
| --- | --- |
| control_authority_id | NJPL |
| version_id | 2 |
| class_id | I |
| ddp_id | 0093 |
| block_length | The maximum size is restricted to 4480 bytes in the 0158-Monitor SFDU. |

##### Aggregation CHDO

The format of the aggregation CHDO is documented in 820-013, 0172-Telecomm-CHDO-001.

##### Primary CHDO

The format of the primary CHDO is documented in 820-013, 0172-Telecomm-CHDO-002.  The following fields within the primary CHDO will have these values in the 0158-Monitor SFDU.

| Field Name | Value |
| --- | --- |
| major | 11 |
| minor | 2 |
| mission | See table 3-4, Spacecraft Identifier Code Assignments, in 820-013, OPS-6-21. |
| format | 5 |

##### Secondary CHDO

This CHDO is unique to the 0158-Monitor SFDU and is documented here.  Figure 3-2 shows the CHDO format and Table 3-1 describes each field in the CHDO.

0   1   2   3   4   5   6   7   0   1   2   3   4   5   6   7

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

Byte 0 |                          chdo_type                            |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

2 |                          chdo_length                          |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

4 |        originator             |          last_modifier        |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

6 |        scft_id                |          data_source          |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

8 |                             spare                             |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

10 |                                                               |

|--                                                           --|

12 |                              mst                              |

|--                                                           --|

14 |                                                               |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

16 |      source_code              |            spare              |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

18 |                             spare                             |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

20 |                             spare                             |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

22

- Figure 3-2. Format of 0158-Monitor Secondary CHDO (73)

- Table 3-1.  Format of 0158-Monitor Secondary CHDO (73)

| Byte Offset | Item Name and Description | Format | Units/ Precision | Range |
| --- | --- | --- | --- | --- |
| 0-1 | chdo_type Secondary header CHDO type code. | Binary unsigned integer. | N/A | 73 |
| 2-3 | chdo_length Length of CHDO value field (bytes 4 - 21). | Binary unsigned integer. | N/A | 18 |
| 4 | originator Originator ID. See 820-013, 0172-Telecomm-CHDO for more information about this field. | Binary unsigned integer. | N/A | 48 |
| 5 | last_modifier Last Modifier ID. See 820-013, 0172-Telecomm-CHDO for more information about this field. | Binary unsigned integer. | N/A | 48 |
| 6 | scft_id Spacecraft identifier code. | Binary unsigned integer. | N/A | See 820-013, OPS-6-21 for values. |
| 7 | data_source Antenna/Facility Identifier. | Binary unsigned integer. | N/A | See document 810-047 for values. |
| 8-9 | spare | Binary unsigned integer. | N/A | 0 |
| 10-15 | mst Monitor sample time. This is the time the record was created. Bits 0 - 15 contain the number of days since 01 Jan 1958. (0 = 01 Jan 1958.) Bits 16 - 47 contain the milliseconds of the day. | Binary unsigned integer. Binary unsigned integer. | seconds milliseconds | 0 - 65535 0 - 86399999 |
| 16 | source_code Source code. | Binary unsigned integer. | N/A | See table 3-3 in 820-013, OPS-6-21 for values. |
| 17-21 | spare | Binary unsigned integer. | N/A | 0 |

##### Tertiary CHDO

The format of the tertiary CHDO is documented in 820-013, 0172-Telecomm-CHDO-000.

##### Quaternary CHDO

The format of the quaternary CHDO is documented in 820-013, 0172-Telecomm-CHDO-027.  The following fields within the quaternary CHDO will have these values in the 0158-Monitor SFDU.

| Field Name | Value |
| --- | --- |
| decom_flags | These flags are not used in the 0158-Monitor SFDU and should be ignored. |
| map_id | FFFF hexadecimal. |

##### Data CHDO

The format of the data CHDO is documented in 820-013, 0172-Telecomm-CHDO-028.  The following fields within the data CHDO will have these values in the 0158-Monitor SFDU.

| Field Name | Value |
| --- | --- |
| source | 13 |
| bad_data | 0 |
| decom_error | 0 |

# Appendix A.  List of Monitor Parameters

- Appendix A

- List of Monitor Parameters

Table A-1 lists all the monitor channels for the equipment associated with the 26M antennas.  The data is sorted by section and by channel ID within each section.  A section is a group of monitor data that apply to a particular functionality of the 26M equipment, e.g., antenna pointing.

Table A-1 26M Monitor Data

| Section | Channel ID | Description | Format | Units/ Precision | Range of Values |
| --- | --- | --- | --- | --- | --- |
| Antenna Pointing | M-0654 | Interpretation of Angle. Indicates the units of Angle 1 (M-0655) and Angle 2 (M-0656). | Integer | N/A | 0 = Angle 1 is azimuth, Angle 2 is elevation. 1 = Angle 1 is hour angle, Angle 2 is declination. 2 = Angle 1 is x-angle, Angle 2 is y-angle, with +x pointing East. 3 = Angle 1 is x-angle, Angle 2 is y-angle, with +x pointing South. |
| Antenna Pointing | M-0655 | Angle 1. | Integer | 1/1024 degree increments |  |
| Antenna Pointing | M-0656 | Angle 2. | Integer | 1/1024 degree increments |  |
| Command Data Accountability | M-0541 | Total number of command elements received. | Integer | N/A | 0 to 65535 |
| Command Data Accountability | M-0542 | Confirmed Blocks. This is the number of command blocks received, verified, and radiated by the DSCC Command Subsystem. | Integer | N/A | 0 to 65535 |
| Command Status and Performance | M-0536 | Carrier Suppression | String | dB | "x.yz", "xy.z", or "UNCL" |
| Command Status and Performance | M-0535 | Command Modulation | String | N/A | "FM", "PM", "PSK", "FSK" |
| Receiver Status | M-0100 | TPA Channel 1 Receiver Number | Integer | N/A |  |
| Receiver Status | M-0200 | TPA Channel 2 Receiver Number | Integer | N/A |  |
| Receiver Status | M-0349 | Receiver 1 Signal Level | Modcomp | dBm | -170 to -50 |
| Receiver Status | M-0379 | Receiver 2 Signal Level | Modcomp | dBm | -170 to -50 |
| Receiver Status | M-0961 | Receiver Frequency. | String | MHz | 2200.00 to 2299.99 |
| Receiver Status | M-0962 | Receiver AGC Mode (AGC Response). | String | milliseconds | "3", "30", "300", "3000", "MAN" |
| Support Description | M-0037 | Spacecraft Number | Integer | N/A | 1 to 255 |
| Support Description | M-0049 | Pass Number | Integer | N/A | -1 to 32767; -1 implies no pass number assigned. |
| Support Description | M-0064 | Master Antenna ID | Integer | N/A | 0 to 99 |
| Telemetry Data Accountability, Channel 1 | M-0182 | Total Frames. Total number of telemetry frames output during this pass for DTM channel 1. For spacecraft which do not use variable-length Standard Format Data Units (SFDUs), this is the number of NASCOM blocks output. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0183 | Sync-Mode Frames. This item reflects the number of telemetry transfer frames received in all frame synchronization modes (search, verify, in-lock, or flywheel) for all virtual channels for DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0190 | Good Frames 0. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 0 and DTM channel 1. A "good" frame synchronization Status is defined as in-lock or flywheel, or with a bit slip or bit stuff of no more than 2 bits. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0191 | Good Frames 1. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 1 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0192 | Good Frames 2. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 2 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0193 | Good Frames 3. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 3 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0194 | Good Frames 4. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 4 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0195 | Good Frames 5. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 5 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0196 | Good Frames 6. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 6 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 1 | M-0197 | Good Frames 7. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 7 and DTM channel 1. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0282 | Total Frames. Total number of telemetry frames output during this pass for DTM channel 2. For spacecraft which do not use variable-length Standard Format Data Units (SFDUs), this is the number of NASCOM blocks output. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0283 | Sync-Mode Frames. This item reflects the number of telemetry transfer frames received in all frame synchronization modes (search, verify, in-lock, or flywheel) for all virtual channels for DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0290 | Good Frames 0. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 0 and DTM channel 2. A "good" frame synchronization Status is defined as in-lock or flywheel, or with a bit slip or bit stuff of no more than 2 bits. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0291 | Good Frames 1. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 1 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0292 | Good Frames 2. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 2 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0293 | Good Frames 3. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 3 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0294 | Good Frames 4. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 4 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0295 | Good Frames 5. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 5 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0296 | Good Frames 6. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 6 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Data Accountability, Channel 2 | M-0297 | Good Frames 7. Total number of telemetry frames with "good" frame synchronization Status processed during the pass for virtual channel 7 and DTM channel 2. | Integer | frames | 0 to 2147483647 |
| Telemetry Status and Performance | M-0104 | Channel 1 telemetry bit rate | Float | bps | 0 to 2.2e6 |
| Telemetry Status and Performance | M-0147 | Channel 1 Lock Status | Integer | N/A | 0 = in lock (lock or flywheel), 1 = out of lock (search or verify) |
| Telemetry Status and Performance | M-0204 | Channel 2 telemetry bit rate | Float | bps | 0 to 2.2e6 |
| Telemetry Status and Performance | M-0247 | Channel 2 Lock Status | Integer | N/A | 0 = in lock (lock or flywheel), 1 = out of lock (search or verify) |
| Tracking Status | M-0311 | Doppler Mode Antenna 'A' Channel 1, S Band | String | N/A | "1" to "4" where "1" = one-way, "2" = two-way, "3" = three-way, "4" = three-way coherent |
| Tracking Status | M-0314 | Doppler Mode Antenna 'A' Channel 2, X Band | String | N/A | "1" to "4" where "1" = one-way, "2" = two-way, "3" = three-way, "4" = three-way coherent |
| Transmitter Status | M-0524 | Exciter Frequency | String | kHz | 12 characters |
| Transmitter Status | M-0526 | S-band Exciter Availability | String | N/A | "UNAV", "TEST", "OFF", "PROG", "ON", "FLT" |
| Transmitter Status | M-0527 | S-band Exciter Composite Modulation Level | Integer | radians |  |

Table A-2 lists all the monitor channels for the subsystems associated with the 34M and 70M antennas.  The monitor data is sorted by subsystem and by channel ID within each subsystem.  Appendix B has definitions for each subsystem acronym and a brief description of what the subsystem does.

Table A-2 34M/70M Monitor Data

| Subsystem | Channel ID | Description | Format | Units | Range of Values |
| --- | --- | --- | --- | --- | --- |
| apa | M-0301 | PDW validity field for elevation angle (M-0324) | Integer | N/A | 0 = valid, 1 = invalid |
| apa | M-0302 | PDW analysis field for elevation angle | Integer | N/A | 0 = disregard, the subsystem has not analyzed this field; 1 = normal, the parameter has a normal, reasonable, or expected value; 2 = high warning limit has been exceeded; 3 = high critical limit has been exceeded; 4 = low warning limit has been exceeded; 5 = low critical limit has been exceeded. |
| apa | M-0303 | PDW source field for elevation angle | String | N/A | String containing the functional address of the observing subsystem |
| apa | M-0308 | Conscan Drive Status | String | N/A | "ON", "OFF" |
| apa | M-0309 | Conscan Loop Status | String | N/A | "OPEN", "CLOS" |
| apa | M-0324 | Elevation angle for 34M HEF and 70M antennas. The elevation angle is expressed as degrees times 10,000. | Float | degrees * 104 | 0.0 to 999999.0 |
| apa | M-0325 | Azimuth angle for 34M HEF and 70M antennas. The azimuth angle is expressed as degrees times 10,000. | Float | degrees * 104 | 0.0 to 3599999.0 |
| apa | M-0326 | Azimuth angle residual for 34M HEF and 70M antennas. The azimuth angle residual is expressed as degrees times 10,000. | Float | degrees * 104 | -1800000.0 to 1800000.0 |
| apa | M-0327 | Elevation angle residual for 34M HEF and 70M antennas. The elevation angle residual is expressed as degrees times 10,000. | Float | degrees * 104 | -990000.0 to 990000.0 |
| apc | M-0301 | PDW validity field for elevation angle (M-0304) | Integer | N/A | 0 = valid, 1 = invalid |
| apc | M-0302 | PDW analysis field for elevation angle | Integer | N/A | 0 = disregard, the subsystem has not analyzed this field; 1 = normal, the parameter has a normal, reasonable, or expected value; 2 = high warning limit has been exceeded; 3 = high critical limit has been exceeded; 4 = low warning limit has been exceeded; 5 = low critical limit has been exceeded. |
| apc | M-0303 | PDW source field for elevation angle | String | N/A | String containing the functional address of the observing subsystem |
| apc | M-0304 | Elevation angle for 34M BWG antennas | Double | degrees | 0.0 to 99.9999 |
| apc | M-0305 | Azimuth angle for 34M BWG antennas | Double | degrees | 0.0 to 359.999 |
| apc | M-0306 | Azimuth angle residual for 34M BWG antennas | Float | degrees | -180.0 to 180.0 |
| apc | M-0307 | Elevation angle residual for 34M BWG antennas | Float | degrees | -99.0 to 99.0 |
| apc | M-0308 | Conscan Drive Status | String | N/A | "ON", "OFF" |
| apc | M-0309 | Conscan Loop Status | String | N/A | "OPEN", "CLOS" |
| apc | M-0804 | Average monopulse receiver estimated azimuth error (first second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0805 | Average monopulse receiver estimated elevation error (first second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0806 | Average monopulse receiver estimated azimuth error (second second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0807 | Average monopulse receiver estimated elevation error (second second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0808 | Average monopulse receiver estimated azimuth error (third second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0809 | Average monopulse receiver estimated elevation error (third second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0810 | Average monopulse receiver estimated azimuth error (fourth second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0811 | Average monopulse receiver estimated elevation error (fourth second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0812 | Average monopulse receiver estimated azimuth error (fifth second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0813 | Average monopulse receiver estimated elevation error (fifth second) | Float | degrees | -0.1 to 0.1 |
| apc | M-0814 | Accumulated azimuth correction | Float | degrees | -0.1 to 0.1 |
| apc | M-0815 | Accumulated elevation correction | Float | degrees | -0.1 to 0.1 |
| apc | M-0816 | Aberration angle off boresight | Float | degrees | 0.0 to 1.0 |
| apc | M-0817 | Aberration clock angle of uplink beam | Float | degrees | 0.0 to 360.0 |
| apc | M-0818 | Conscan type | String | N/A | "NONE", "CONICAL", "MONOPULSE" |
| apc | M-0819 | Monopulse Receiver SNR | Float | dB | -300.0, 0.0 to 90.0; -300.0 indicates out of lock. |
| apc | M-0820 | Aberration table x-position | Float | N/A | -150.0 to 150.0 |
| apc | M-0821 | Aberration table y-position | Float | N/A | -150.0 to 150.0 |
| apc | M-0822 | Aberration Correction | String | N/A | "ON", "OFF", "ERR" |
| ce | M-0037 | Spacecraft Number. This is the spacecraft ID entered into the Network Monitor and Control (NMC) subsystem by the operator. During normal operations, this S/C ID is automatically sent by the NMC to the uplink and downlink subsystems when they are assigned to the connection. | Integer | N/A | 1 to 255 |
| ce | M-0042 | DSS ID. This is the antenna ID entered into the NMC by the operator when configuring the connection. | Integer | N/A | 0 to 99 |
| cpa | M-0500 | Transmitting Antenna ID | Integer | N/A | 0 to 99 |
| cpa | M-0534 | CMA Mode | String | N/A | "C1", "C2", "I1", "I2", "AC", "AB" |
| cpa | M-0536 | Carrier Suppression | String | N/A | "x.yz" dB, "xy.z" dB, or "UNCL" |
| cpa | M-0537 | Command Bit Rate | String | bps | "1.0000" to "9999.9999" |
| dcc1 (Carrier Loop) | M-1290 | Time tag of carrier performance data. This channel contains the seconds portion of the total time. Channels M-1290, M-1291, and M-1292 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Carrier Loop) | M-1291 | Time tag of carrier performance data. This channel contains the nanoseconds portion of the total time. Channels M-1290, M-1291, and M-1292 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1290 | >=0 |
| dcc1 (Carrier Loop) | M-1292 | Time tag of carrier performance data. This channel contains the leap seconds portion of the total time. Channels M-1290, M-1291, and M-1292 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Carrier Loop) | M-1293 | Time that the last carrier acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1293, M-1294, and M-1295 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Carrier Loop) | M-1294 | Time that the last carrier acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1293, M-1294, and M-1295 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1293 | >=0 |
| dcc1 (Carrier Loop) | M-1295 | Time that the last carrier acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1293, M-1294, and M-1295 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Carrier Loop) | M-1296 | Time that the carrier lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1296, M-1297, and M-1298 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Carrier Loop) | M-1297 | Time that the carrier lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1296, M-1297, and M-1298 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1296 | >=0 |
| dcc1 (Carrier Loop) | M-1298 | Time that the carrier lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1296, M-1297, and M-1298 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Carrier Loop) | M-1299 | Carrier loop tracking bandwidth. | Float | Hz | 0.100 to 1000.000 |
| dcc1 (Carrier Loop) | M-1300 | The carrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc1 (Carrier Loop) | M-1301 | Carrier tracking loop lock status. Status is out-of-lock unless the loop status (M-1302) is IN LOCK. | Integer | N/A | 0 = Out of lock, 1 = In lock. |
| dcc1 (Carrier Loop) | M-1302 | Carrier tracking loop status. This is the result of the acquisition command or enabling the loop. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc1 (Carrier Loop) | M-1303 | Data tracking loop status (combined status of subcarrier and symbol loops). | Integer | N/A | 0 = Out of lock, 1= In lock, 2 = Off (Idle), 3 = Acquiring, 4 = Open |
| dcc1 (Carrier Loop) | M-1304 | The measured carrier frequency. | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc1 (Carrier Loop) | M-1305 | Carrier frequency residual. The measured carrier frequency minus the predicted value. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc1 (Carrier Loop) | M-1306 | The predicted carrier frequency minus the measured value. This is commonly called the doppler residual. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc1 (Carrier Loop) | M-1307 | The measured carrier frequency rate (first derivative of the frequency). | Float | Hz/sec | -108000.0 to 108000.0 |
| dcc1 (Carrier Loop) | M-1308 | The measured carrier frequency acceleration (second derivative of the frequency). | Float | Hz/sec2 | -2600.0 to 2600.0 |
| dcc1 (Carrier Loop) | M-1309 | The standard deviation of the detrended frequency residuals. This is commonly called the doppler noise. | Float | Hz | 0.000 to 1000.000 |
| dcc1 (Carrier Loop) | M-1310 | Carrier power at performance time tag (channels M-1290, M-1291, and M-1292). If value not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc1 (Carrier Loop) | M-1311 | Carrier power residual. The estimated carrier power minus the predicted value. | Float | dB | -190.0 to 190.0 |
| dcc1 (Carrier Loop) | M-1312 | The measured data power. If value is not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc1 (Carrier Loop) | M-1313 | Data power residual. The measured data power minus the predicted value. | Float | dBm | -190.0 to 190.0 |
| dcc1 (Carrier Loop) | M-1314 | The carrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc1 (Carrier Loop) | M-1315 | The carrier power to noise spectral density ratio (Pc/NO). If the value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc1 (Carrier Loop) | M-1316 | Pc/NO residual. The measured Pc/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc1 (Carrier Loop) | M-1317 | Carrier tracking mode (source of carrier error signal). | String | N/A | "RESIDUAL", "SUPPRESSED", "SIDEBAND", "QPSK", "OQPSK", "OPEN" |
| dcc1 (Carrier Loop) | M-1318 | The system noise temperature (SNT). | Float | kelvin | 10.0 to 2000.0 |
| dcc1 (Carrier Loop) | M-1319 | SNT residual. The measured system noise temperature minus the predicted value. | Float | kelvin | 10.0 to 2000.0 |
| dcc1 (Carrier Loop) | M-1320 | The predicted carrier frequency. | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc1 (Carrier Loop) | M-1321 | Count of the number of cycle slips detected during this pass. | Float | number of slips | 0.0 to 10000.0 |
| dcc1 (Carrier Loop) | M-1322 | Carrier acquisition FFT usage switch. If enabled, the carrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Carrier Loop) | M-1323 | Indicates whether or not the carrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1240 | Downlink channel providing the data. | Integer | N/A | 1 to 24 |
| dcc1 (Configuration) | M-1241 | Antenna being used from the perspective of the DTT (Downlink Tracking and Telemetry) subsystem. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 0 to 99 |
| dcc1 (Configuration) | M-1242 | Configured spacecraft ID from the perspective of the DTT. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 1 to 255 |
| dcc1 (Configuration) | M-1243 | Pass number. | Integer | N/A | 0 to 9999 |
| dcc1 (Configuration) | M-1244 | Mission ID for spacecraft. | Integer | N/A | 0 to 255 |
| dcc1 (Configuration) | M-1245 | Downlink carrier band. | String | N/A | "NONE", "L", "S", "X", "KA", "S26" |
| dcc1 (Configuration) | M-1246 | Uplink carrier band used for the downlink processing. It is possible that this is not the actual uplink band. | String | N/A | "NONE", "L", "S", "X", "KA" |
| dcc1 (Configuration) | M-1247 | Receiver-Telemetry configuration table name. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc1 (Configuration) | M-1248 | Receiver radiometrics (frequency) predicts mode. NPX => no predicts used, 1W => one-way, 2W => two-way, 3W => three-way. | String | N/A | "NPX", "1W", "2W", "3W" |
| dcc1 (Configuration) | M-1250 | Uplink antenna for three-way predicts. | Integer | N/A | 0 to 99 |
| dcc1 (Configuration) | M-1251 | Predicted carrier frequency | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc1 (Configuration) | M-1252 | Predicted subcarrier frequency | Double | Hz | 0.000 to 2000000.000 |
| dcc1 (Configuration) | M-1253 | Predicted symbol rate | Double | Hz | 4.000 to 26400000.000 |
| dcc1 (Configuration) | M-1254 | SNT measurement switch. If the measurement is disabled, the value in M-1318 is the predicted value. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1255 | Monopulse tracking processing switch. Used only when there is a Ka-band downlink (M-1245 is equal to "KA"). | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1256 | Name of the radiometric predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc1 (Configuration) | M-1257 | Time that the last radiometric predict mode change was done. This channel contains the seconds portion of the total time. Channels M-1257, M-1258, and M-1259 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Configuration) | M-1258 | Time that the last radiometric predict mode change was done. This channel contains the nanoseconds portion of the total time. Channels M-1257, M-1258, and M-1259 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1257 | >=0 |
| dcc1 (Configuration) | M-1259 | Time that the last radiometric predict mode change was done. This channel contains the leap seconds portion of the total time. Channels M-1257, M-1258, and M-1259 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Configuration) | M-1260 | Telemetry predicts usage mode. If enabled, telemetry predicts are used if the radiometric predict mode (M-1248) is not NPX. | String | N/A | "DISABLED", "ENABLED" |
| dcc1 (Configuration) | M-1261 | Name of the telemetry predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc1 (Configuration) | M-1262 | Range state | String | N/A | "Enable", "Disable" |
| dcc1 (Configuration) | M-1263 | Carrier phase (tracking) data output switch. If enabled, carrier phase data is being sent (count of blocks given in M-1508). | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1264 | Range data output switch. If enabled, range data is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1265 | Telemetry data output switch. If enabled, telemetry is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Configuration) | M-1266 | The LNA (Low Noise Amplifier) number from which the channel is receiving the signal. | String | N/A | "A1", "A2", "A3", "A4", "N/A" |
| dcc1 (Configuration) | M-1267 | The polarization of the signal that the downlink channel is tracking. | String | N/A | "LCP", "RCP", "UNKN", "N/A" |
| dcc1 (Configuration) | M-1268 | The configuration of the microwave for uplink. Possible configurations are low noise (LONO), diplexed (DPLX), radar configuration (RADR), and non-diplexed (NDPX, another name for low noise). | String | N/A | "LONO", "RADR", "DPLX", "NPDX", "UNKN", "N/A" |
| dcc1 (Ranging) | M-1500 | Time that the ranging acquisition started. Also referred to as the T0 time. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc1 (Ranging) | M-1503* | Time that the correlation validity last changed. This channel contains the seconds portion of the total time. Channels M-1503, M-1504, and M-1505 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Ranging) | M-1504* | Time that the correlation validity last changed. This channel contains the nanoseconds portion of the total time. Channels M-1503, M-1504, and M-1505 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1503 | >=0 |
| dcc1 (Ranging) | M-1505* | Time that the correlation validity last changed. This channel contains the leap seconds portion of the total time. Channels M-1503, M-1504, and M-1505 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Ranging) | M-1506 | Indicates whether or not the last ranging correlation was valid. | String | N/A | "OFF", "OUT OF LOCK", "IN LOCK" |
| dcc1 (Ranging) | M-1507 | The number of range data blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc1 (Ranging) | M-1508 | The number of carrier phase blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc1 (Ranging) | M-1509 | Number of ranging points that have been measured. Value is reset to zero every time ranging acquisition is initiated. | Integer | N/A | 0 to (231 - 1) |
| dcc1 (Ranging) | M-1510 | The Figure of Merit, expressed as a percentage, of the range measurement. | Float | percent | 0.0 to 100.0 |
| dcc1 (Ranging) | M-1511 | Range residual. The measured range value minus the predicted value. | Double | range units | -230 to 230 |
| dcc1 (Ranging) | M-1512 | Ranging power to noise spectral density ratio (Pr/NO). | Float | dB-Hz | -10.0 to 90.0 |
| dcc1 (Ranging) | M-1513 | Pr/NO residual. The measured Pr/N0 minus the predicted value. | Float | dB-Hz | -100.0 to 100.0 |
| dcc1 (Ranging) | M-1514 | Type of ranging being performed. SSIN => Sequential with sinewave clock, SSQ => Sequential with squarewave clock, PSIN => PN with sinewave clock, PSQ => PN with squarewave clock, PCON => PN, no clock. | String | N/A | "SSIN", "SSQ", "PSIN", "PSQ", "PCON" |
| dcc1 (Ranging) | M-1515 | Round trip light time used. Valid only if ranging type (channel M-1514) is Sequential. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc1 (Ranging) | M-1516 | Highest component of sequential ranging code (also called the clock component). Frequency is approximately 66*2-n MHz, where n is the first component value. Valid only if ranging type (channel M-1514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc1 (Ranging) | M-1517 | Lowest component of sequential ranging code. Frequency is approximately 66*2-n MHz, where n is the last component value. Valid only if ranging type (channel M-1514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc1 (Ranging) | M-1518 | Component value that is used for chopping the lower components at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-1514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc1 (Ranging) | M-1519 | Component value that the chopping starts at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-1514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc1 (Ranging) | M-1520 | Length of time that the first component is transmitted. Only valid if ranging type (channel M-1514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc1 (Ranging) | M-1521 | Length of time that the all components except the first are transmitted. Only valid if ranging type (channel M-1514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc1 (Ranging) | M-1522 | Length of time that the first component is transmitted for each DRVID measurement. Only valid if ranging type (channel M-1514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc1 (Ranging) | M-1523 | Number of DRVID measurements per ranging cycle. Only valid if ranging type (channel M-1514) is Sequential. | Integer | N/A | 0 to 255 |
| dcc1 (Ranging) | M-1524 | The integration time, in PN cycle periods, that the measurement was made. Valid only if ranging type (channel M-1514) is PN. | Integer | seconds | 1 to (231 - 1) |
| dcc1 (Ranging) | M-1525 | The reference signal is divided by this value to get the modulation clock. Valid only if ranging type (channel M-1514) is PN. | Integer | N/A | 1 to 64 (normally 64) |
| dcc1 (Ranging) | M-1526 | Length of the 1st PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1527 | The symbol (chip) pattern of the 1st PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1528 | The Boolean operation associated with the 1st PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1529 | Length of the 2nd PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1530 | The symbol (chip) pattern of the 2nd PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1531 | The Boolean operation associated with the 2nd PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1532 | Length of the 3rd PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1533 | The symbol (chip) pattern of the 3rd PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1534 | The Boolean operation associated with the 3rd PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1535 | Length of the 4th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1536 | The symbol (chip) pattern of the 4th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1537 | The Boolean operation associated with the 4th PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1538 | Length of the 5th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1539 | The symbol (chip) pattern of the 5th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1540 | The Boolean operation associated with the 5th PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1541 | Length of the 6th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc1 (Ranging) | M-1542 | The symbol (chip) pattern of the 6th PN sequence. Valid only if ranging type (channel M-1514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc1 (Ranging) | M-1543 | The Boolean operation associated with the 6th PN sequence. Valid only if ranging type (channel M-1514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc1 (Ranging) | M-1544 | Differenced Range versus Integrated Doppler (DRVID) measurement in Range Units. | Double | range units | -230 to 230 |
| dcc1 (Ranging) | M-1545 | Current operational mode of the ranging process. | String | N/A | "IDLE", "TRK", "CAL", "DIAG" |
| dcc1 (Ranging) | M-1546 | Time of the range and DRVID measurements. | String | N/A | "hh:mm:ss" |
| dcc1 (Ranging) | M-1547 | Range measurement in Range Units. | Double | range units | 0.00 to 230 |
| dcc1 (Ranging) | M-1548 | The desired ranging Figure of Merit. | Float | Percentage | 0.0 to 100.0 |
| dcc1 (Status) | M-1200 | Status code for the overall downlink channel status. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1201 | Status qualifier for the overall downlink channel status. This qualifies the status code (M-1200). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1202 | Status code for the overall receiver equipment controller status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1203 | Status qualifier for the overall receiver equipment controller status. This qualifies the status code (M-1202). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1204 | Status of the receiving function of the downlink channel. "Out of Service" indicates the receiving function is not installed. | Integer | N/A | 0 = Out Of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1205 | Status qualifier for the receiving function of the downlink channel. No qualifiers are used to qualify the status code (M-1204). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1206 | Status code for the status of the downlink channel controller of the downlink channel. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1207 | Status qualifier for the status of the downlink channel controller of the downlink channel. No qualifiers are used to qualify the status code (M-1206). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1208* | Status code for the status of the monopulse function of the downlink channel. "Out of Service" indicates the monopulse function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1209* | Status qualifier for the status of the monopulse function of the downlink channel. No qualifiers are used to qualify the status code (M-1208). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1210 | Status code for the status of the ranging function of the downlink channel. "Out of Service" indicates the ranging function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1211 | Status qualifier for the status of the ranging function of the downlink channel. No qualifiers are used to qualify the status code (M-1210). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1212 | Status code for the status of the standard telemetry function of the downlink channel. "Out of Service" indicates the telemetry function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1213 | Status qualifier for the status of the standard telemetry function of the downlink channel. No qualifiers are used to qualify the status code (M-1212). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1214* | Status code for the status of the MCD3 decoder function of the downlink channel. "Out of Service" indicates the MCD3 function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1215* | Status qualifier for the status of the MCD3 decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-1214). | Integer | N/A | 0 = None |
| dcc1 (Status) | M-1216* | Status code for the status of the turbo decoder function of the downlink channel. "Out of Service" indicates the turbo decoder function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc1 (Status) | M-1217* | Status qualifier for the status of the turbo decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-1216). | Integer | N/A | 0 = None |
| dcc1 (Subcarrier Loop) | M-1340 | The time of the last change in the predicted subcarrier frequency. This channel contains the seconds portion of the total time. Channels M-1340, M-1341, and M-1342 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Subcarrier Loop) | M-1341 | The time of the last change in the predicted subcarrier frequency. This channel contains the nanoseconds portion of the total time. Channels M-1340, M-1341, and M-1342 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1340 | >=0 |
| dcc1 (Subcarrier Loop) | M-1342 | The time of the last change in the predicted subcarrier frequency. This channel contains the leap seconds portion of the total time. Channels M-1340, M-1341, and M-1342 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Subcarrier Loop) | M-1343 | The measured subcarrier frequency | Double | Hz | 500.000 to 6000000.000 |
| dcc1 (Subcarrier Loop) | M-1344 | Subcarrier frequency residual. The measured subcarrier frequency minus the predicted value. | Float | Hz | -1.000e6 to 1.000e6 |
| dcc1 (Subcarrier Loop) | M-1345 | Subcarrier loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc1 (Subcarrier Loop) | M-1346 | The subcarrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc1 (Subcarrier Loop) | M-1347 | The windowing on the subcarrier phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc1 (Subcarrier Loop) | M-1348 | Subcarrier tracking loop lock status. Status is out-of-lock unless the loop status (M-1352) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc1 (Subcarrier Loop) | M-1349* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the seconds portion of the total time tag. Channels M-1349, M-1350, and M-1351 together contain the total time tag. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Subcarrier Loop) | M-1350* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time tag. Channels M-1349, M-1350, and M-1351 together contain the total time tag. | Integer | number of nanoseconds since the seconds value in M-1349 | >=0 |
| dcc1 (Subcarrier Loop) | M-1351* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time tag. Channels M-1349, M-1350, and M-1351 together contain the total time tag. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Subcarrier Loop) | M-1352 | Subcarrier tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc1 (Subcarrier Loop) | M-1353 | The subcarrier data power to noise spectral density ratio (Pd/NO). If value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc1 (Subcarrier Loop) | M-1354 | Pd/NO residual. The measured subcarrier Pd/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc1 (Subcarrier Loop) | M-1355 | The subcarrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc1 (Subcarrier Loop) | M-1356 | Subcarrier waveform. | String | N/A | “SIN”, “SQ”, “D” |
| dcc1 (Subcarrier Loop) | M-1357 | Predicted subcarrier frequency (including predicted Doppler shifts). | Double | Hz | 500.000 to 6000000.000 |
| dcc1 (Subcarrier Loop) | M-1358 | Subcarrier acquisition FFT usage switch. If enabled, the subcarrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Subcarrier Loop) | M-1359 | Indicates whether or not the subcarrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Symbol Loop) | M-1380 | The time of the last change in the predicted symbol frequency. This channel contains the seconds portion of the total time. Channels M-1380, M-1381, and M-1382 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Symbol Loop) | M-1381 | The time of the last change in the predicted symbol frequency. This channel contains the nanoseconds portion of the total time. Channels M-1380, M-1381, and M-1382 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1380 | >=0 |
| dcc1 (Symbol Loop) | M-1382 | The time of the last change in the predicted symbol frequency. This channel contains the leap seconds portion of the total time. Channels M-1380, M-1381, and M-1382 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Symbol Loop) | M-1383 | The measured symbol rate | Double | sps | 4.000 to 26400000.000 |
| dcc1 (Symbol Loop) | M-1384 | Symbol rate residual. The measured symbol rate minus the predicted value. | Float | sps | -26400000.000 to 26400000.000 |
| dcc1 (Symbol Loop) | M-1385 | Symbol loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc1 (Symbol Loop) | M-1386 | The symbol loop filter order. | Integer | N/A | 1 to 3 |
| dcc1 (Symbol Loop) | M-1387 | The windowing on the symbol phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc1 (Symbol Loop) | M-1388 | Symbol tracking loop lock status. Status is out-of-lock unless the loop status (M-1392) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc1 (Symbol Loop) | M-1389* | Time that the symbol tracking loop lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1389, M-1390, and M-1391 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Symbol Loop) | M-1390* | Time that the symbol tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1389, M-1390, and M-1391 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1389 | >=0 |
| dcc1 (Symbol Loop) | M-1391* | Time that the symbol tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1389, M-1390, and M-1391 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Symbol Loop) | M-1392 | Symbol tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc1 (Symbol Loop) | M-1393 | The measured symbol signal to noise ratio (SSNR). | Float | dB | -10.0 to 50.0 |
| dcc1 (Symbol Loop) | M-1394 | SSNR residual. The measured SSNR minus the predicted value. | Float | dB | -50.0 to 50.0 |
| dcc1 (Symbol Loop) | M-1395 | The symbol tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc1 (Symbol Loop) | M-1396 | Modulation format of the symbols. | String | N/A | “NRZL”, “NRZM”, “NRZS”, “BIPS”, “BIPL”, “BIPM” |
| dcc1 (Symbol Loop) | M-1397 | Configuration of the symbol smoothing algorithm (low threshold telemetry mode). | String | N/A | “ENABLED”, “DISABLED” |
| dcc1 (Symbol Loop) | M-1398 | Predicted symbol frequency (including predicted Doppler shifts). | Double | sps | 4.000 to 26400000.000 |
| dcc1 (Symbol Loop) | M-1399 | Symbol acquisition FFT usage switch. If enabled, the symbol acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Symbol Loop) | M-1400 | Indicates whether or not the symbol FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Telemetry) | M-0190 | Number of Virtual Channel 0 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0191 | Number of Virtual Channel 1 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0192 | Number of Virtual Channel 2 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0193 | Number of Virtual Channel 3 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0194 | Number of Virtual Channel 4 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0195 | Number of Virtual Channel 5 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0196 | Number of Virtual Channel 6 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-0197 | Number of Virtual Channel 7 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1415 | Operational state of the Reed-Solomon decoder. In Primary (P) mode, the corrected bits are output to the end user. In Secondary (S) mode, the decoding statistics are provided for the operator, but the bit corrections are not output. | String | N/A | "D", “P”, “S” |
| dcc1 (Telemetry) | M-1416 | Indicator of whether the input signal is an arrayed signal or not. A value of 0 indicates not arrayed; values of 1 and 2 correspond to the arraying equipment physical number. | Integer | N/A | 0 = Unarrayed, 1 = Arrayed with FSPA1 as the source, 2 = Arrayed with FSPA2 as the source |
| dcc1 (Telemetry) | M-1417 | Frame sync operational mode. | String | N/A | “D”, “P” |
| dcc1 (Telemetry) | M-1418 | The pattern of bits of the frame sync word. | String | N/A | Any pattern expressed in hexadecimal |
| dcc1 (Telemetry) | M-1419 | Length of the frame sync word. | Integer | bits | 8 to 64 |
| dcc1 (Telemetry) | M-1420 | Predicted telemetry bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc1 (Telemetry) | M-1421 | Measured bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc1 (Telemetry) | M-1422 | Overall telemetry lock status (includes decoders and frame synchronizer). | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc1 (Telemetry) | M-1424 | Telemetry processing operational status | String | N/A | "GO", "NOGO" |
| dcc1 (Telemetry) | M-1425* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1425, M-1426, and M-1427 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Telemetry) | M-1426* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1425, M-1426, and M-1427 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1425 | >=0 |
| dcc1 (Telemetry) | M-1427* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1425, M-1426, and M-1427 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Telemetry) | M-1428* | Time that the decoder lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1428, M-1429, and M-1430 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Telemetry) | M-1429* | Time that the decoder lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1428, M-1429, and M-1430 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1428 | >=0 |
| dcc1 (Telemetry) | M-1430* | Time that the decoder lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1428, M-1429, and M-1430 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Telemetry) | M-1431 | Telemetry decoder lock status. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc1 (Telemetry) | M-1433 | Type of decoder used. | String | N/A | "MCD2", "MCD3", "TURBO", "UNC", "SYMB", "GSSR" |
| dcc1 (Telemetry) | M-1434 | The output bit signal to noise ratio for the decoder. | Double | dB | -10.0 to 50.0 |
| dcc1 (Telemetry) | M-1436 | In-lock threshold for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | Integer | N/A | 1 to 255 |
| dcc1 (Telemetry) | M-1437 | Vector set for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | String | N/A | "CCSDS", "DSN" |
| dcc1 (Telemetry) | M-1438 | In-lock threshold for MCD3 decoder. Only valid if decoder type is MCD3 decoder. | Integer | N/A | 1 to 256 |
| dcc1 (Telemetry) | M-1439 | Vector set for the MCD3 decoder. Only valid if decoder type is MCD3 decoder. | String | N/A | "NONE", "CCSDS", "DSN", "A15R6", "CAS", "MAP" |
| dcc1 (Telemetry) | M-1440 | Frame sync primary frame length | Integer | bits | 8 to 65536 |
| dcc1 (Telemetry) | M-1441 | Frame sync in lock bit error tolerance. The number of bit errors allowed in the frame sync word while still being considered in lock. | Integer | bits | 0 to 15 |
| dcc1 (Telemetry) | M-1442 | Frame sync out of lock bit error tolerance (the minimum number of bit errors required to declare out of lock). | Integer | bits | 0 to 15 |
| dcc1 (Telemetry) | M-1443 | Frame sync in lock threshold (the number of consecutive in sync frames verified before lock is declared). | Integer | frames | 1 to 15 |
| dcc1 (Telemetry) | M-1444 | Frame sync out of lock threshold (number of consecutive out of sync frames before out-of-lock is declared). | Integer | frames | 1 to 15 |
| dcc1 (Telemetry) | M-1445* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1445, M-1446, and M-1447 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Telemetry) | M-1446* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1445, M-1446, and M-1447 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1445 | >=0 |
| dcc1 (Telemetry) | M-1447* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1445, M-1446, and M-1447 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Telemetry) | M-1448* | Time that the frame sync lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1448, M-1449, and M-1450 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc1 (Telemetry) | M-1449* | Time that the frame sync lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1448, M-1449, and M-1450 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1448 | >=0 |
| dcc1 (Telemetry) | M-1450* | Time that the frame sync lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1448, M-1449, and M-1450 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc1 (Telemetry) | M-1451 | Frame sync lock status. | String | N/A | "IDLE", "SEARCH", "VERIFY", "LOCK", "FLYWHEEL" |
| dcc1 (Telemetry) | M-1452 | The number of bits the frame synchronizer processed in its last (if in-lock) or current (if out-of-lock) acquisition. When the frame sync goes to out-of-lock, the value is reset to zero. | Integer | bits | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1453 | Number of telemetry blocks output since the start of the connection. | Integer | blocks | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1455 | Count of total number of frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1456 | Count of the number of good frames (verify, flywheel, and lock) since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1457 | Count of the number of bad (search) frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1458 | Lock status of the Cyclic Redundancy Code (CRC) check. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc1 (Telemetry) | M-1459 | Configuration of the pseudo derandomizer. | String | N/A | "ENABLED", "DISABLED" |
| dcc1 (Telemetry) | M-1460 | Count of total number of frames that have had the CRC checked since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1461 | Count of the number of frames that have passed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1462 | Count of the number of frames that have failed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1463 | Lock status of the Reed-Solomon decoder. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc1 (Telemetry) | M-1464 | Count of total number of Reed-Solomon code words that have been checked since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1465 | Count of the number of Reed-Solomon code words that have been decoded since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1466 | Count of the number of Reed-Solomon code words that have failed to decode since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1467 | Ratio of bad frames to total frames for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc1 (Telemetry) | M-1468 | Ratio of bad code words to total code words for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc1 (Telemetry) | M-1469 | Count of total number of locked (flywheel and lock) frames since start of connection. Count rolls over to zero. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1470 | The number of frames with flywheel status in the last (or current) in-lock period. When lock is first acquired, the value is reset to zero. | Integer | frames | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1471 | Name of the format table used by the default telemetry stream. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc1 (Telemetry) | M-1474 | Out-of-lock threshold for Viterbi decoder (7, _). | Integer | bits | 1 to 65535 |
| dcc1 (Telemetry) | M-1475 | Out-of-lock threshold for the MCD3 decoder. | Integer | bits | 1 to 255 |
| dcc1 (Telemetry) | M-1476 | Configuration of the differential decoder (disabled, mark, or space). | String | N/A | “D”, “NM”, "NS" |
| dcc1 (Telemetry) | M-1477 | State of the Viterbi decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc1 (Telemetry) | M-1478 | State of the MCD3 decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc1 (Telemetry) | M-1479 | The number of times the metric normalization count exceeded the threshold since the TLP was put in the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1480 | Percentage of symbol errors detected over the last accumulation period. | Double | Percentage | 0.0 to 100.0 |
| dcc1 (Telemetry) | M-1481 | Viterbi decoder node sync change algorithm. | String | N/A | “D”, “SER”, “MNR” |
| dcc1 (Telemetry) | M-1482 | Number of node sync changes since the TLP was put into the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1483 | Position that the MCD3 decoder has locked onto. | Integer | N/A | 0 to 5 |
| dcc1 (Telemetry) | M-1484 | Frame sync secondary frame length (alternate frame length). | Integer | bits | 0 to 65528 |
| dcc1 (Telemetry) | M-1485 | Detected frame size. | Integer | bits | 8 to 65536 |
| dcc1 (Telemetry) | M-1486 | Status of the frame synchronizer hardware. | String | N/A | "GO", "NOGO" |
| dcc1 (Telemetry) | M-1487 | The order of the sequence for searching for frame synchronization (e.g. left-to-right (MSB) or right-to-left (LSB)). Allows handling of spacecraft that transmit recorded data backwards. M => MSB, L => LSB, and B => Both. | String | N/A | “M”, “L”, “B” |
| dcc1 (Telemetry) | M-1488 | Number of bits over or under the frame length the synchronizer will accept as a correct length frame. | Integer | bits | 0 to 3 |
| dcc1 (Telemetry) | M-1489 | The bit error rate (BER) of the frame sync words for the in-lock frames. | Double | Percentage | 0.0 to 100.0 |
| dcc1 (Telemetry) | M-1490 | The detected bit sequence. | String | N/A | “MSB”, “LSB” |
| dcc1 (Telemetry) | M-1491 | The polarity of the input data to the frame synchronizer. | String | N/A | “NORMAL”, “INVERTED” |
| dcc1 (Telemetry) | M-1492 | The number of bit slips detected since the start of processing. This count is the number of bits slipped, not the number of bit slip occurrences. | Integer | bits | 0 to (231 - 1) |
| dcc1 (Telemetry) | M-1493 | The number of polarity changes detected since the start of processing. | Integer | N/A | 0 to 65535 |
| dcc1 (Telemetry) | M-1494 | The number of times since the start of the processing that the frame synchronizer has transitioned to the LOCK state. | Integer | N/A | 0 to 65535 |
| dcc1 (Telemetry) | M-1495 | Number of Reed-Solomon codewords interleaved into a transfer frame. | Integer | Codewords | 0 to 5 |
| dcc1 (Telemetry) | M-1496 | Reed-Solomon decoding algorithm used (Berlekamp versus Conventional). | String | N/A | “B”, “C” |
| dcc1 (Telemetry) | M-1497 | Number of zero pad symbols (bytes) used in the Reed-Solomon encoding. This is also known as the virtual fill. | Integer | Reed-Solomon symbols | 0 to 223 |
| dcc1 (Telemetry) | M-1498 | The number of symbol errors detected divided by the number of symbols received. Codewords that cannot be corrected are not counted. | Double | Percentage | 0.0 to 100.0 |
| dcc1 (Telemetry) | M-1499 | Reed-Solomon decoder hardware status. | String | N/A | “GO”, “NOGO” |
| dcc2 (Carrier Loop) | M-1790 | Time tag of carrier performance data. This channel contains the seconds portion of the total time. Channels M-1790, M-1791, and M-1792 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Carrier Loop) | M-1791 | Time tag of carrier performance data. This channel contains the nanoseconds portion of the total time. Channels M-1790, M-1791, and M-1792 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1790 | >=0 |
| dcc2 (Carrier Loop) | M-1792 | Time tag of carrier performance data. This channel contains the leap seconds portion of the total time. Channels M-1790, M-1791, and M-1792 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Carrier Loop) | M-1793 | Time that the last carrier acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1793, M-1794, and M-1795 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Carrier Loop) | M-1794 | Time that the last carrier acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1793, M-1794, and M-1795 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1793 | >=0 |
| dcc2 (Carrier Loop) | M-1795 | Time that the last carrier acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1793, M-1794, and M-1795 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Carrier Loop) | M-1796 | Time that the carrier lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1796, M-1797, and M-1798 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Carrier Loop) | M-1797 | Time that the carrier lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1796, M-1797, and M-1798 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1796 | >=0 |
| dcc2 (Carrier Loop) | M-1798 | Time that the carrier lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1796, M-1797, and M-1798 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Carrier Loop) | M-1799 | Carrier loop tracking bandwidth. | Float | Hz | 0.100 to 1000.000 |
| dcc2 (Carrier Loop) | M-1800 | The carrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc2 (Carrier Loop) | M-1801 | Carrier tracking loop lock status. Status is out-of-lock unless the loop status (M-1802) is IN LOCK. | Integer | N/A | 0 = Out of lock, 1 = In lock. |
| dcc2 (Carrier Loop) | M-1802 | Carrier tracking loop status. This is the result of the acquisition command or enabling the loop. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc2 (Carrier Loop) | M-1803 | Data tracking loop status (combined status of subcarrier and symbol loops). | Integer | N/A | 0 = Out of lock, 1= In lock, 2 = Off (Idle), 3 = Acquiring, 4 = Open |
| dcc2 (Carrier Loop) | M-1804 | The measured carrier frequency. | Double | Hz | 2200000000.000 to 32,300,000,000.000 |
| dcc2 (Carrier Loop) | M-1805 | Carrier frequency residual. The measured carrier frequency minus the predicted value. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc2 (Carrier Loop) | M-1806 | The predicted carrier frequency minus the measured value. This is commonly called the doppler residual. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc2 (Carrier Loop) | M-1807 | The measured carrier frequency rate (first derivative of the frequency). | Float | Hz/sec | -108000.0 to 108000.0 |
| dcc2 (Carrier Loop) | M-1808 | The measured carrier frequency acceleration (second derivative of the frequency). | Float | Hz/sec2 | -2600.0 to 2600.0 |
| dcc2 (Carrier Loop) | M-1809 | The standard deviation of the detrended frequency residuals. This is commonly called the doppler noise. | Float | Hz | 0.000 to 1000.000 |
| dcc2 (Carrier Loop) | M-1810 | Carrier power at performance time tag (channels M-1790, M-1791, and M-1792). If value not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc2 (Carrier Loop) | M-1811 | Carrier power residual. The estimated carrier power minus the predicted value. | Float | dB | -190.0 to 190.0 |
| dcc2 (Carrier Loop) | M-1812 | The measured data power. If value is not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc2 (Carrier Loop) | M-1813 | Data power residual. The measured data power minus the predicted value. | Float | dBm | -190.0 to 190.0 |
| dcc2 (Carrier Loop) | M-1814 | The carrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc2 (Carrier Loop) | M-1815 | The carrier power to noise spectral density ratio (Pc/NO). If the value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc2 (Carrier Loop) | M-1816 | Pc/NO residual. The measured Pc/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc2 (Carrier Loop) | M-1817 | Carrier tracking mode (source of carrier error signal). | String | N/A | "RESIDUAL", "SUPPRESSED", "SIDEBAND", "QPSK", "OQPSK", "OPEN" |
| dcc2 (Carrier Loop) | M-1818 | The system noise temperature (SNT). | Float | kelvin | 10.0 to 2000.0 |
| dcc2 (Carrier Loop) | M-1819 | SNT residual. The measured system noise temperature minus the predicted value. | Float | kelvin | 10.0 to 2000.0 |
| dcc2 (Carrier Loop) | M-1820 | The predicted carrier frequency. | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc2 (Carrier Loop) | M-1821 | Count of the number of cycle slips detected during this pass. | Float | number of slips | 0.0 to 10000.0 |
| dcc2 (Carrier Loop) | M-1822 | Carrier acquisition FFT usage switch. If enabled, the carrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Carrier Loop) | M-1823 | Indicates whether or not the carrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1740 | Downlink channel providing the data. | Integer | N/A | 1 to 24 |
| dcc2 (Configuration) | M-1741 | Antenna being used from the perspective of the DTT (Downlink Tracking and Telemetry) subsystem. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 0 to 99 |
| dcc2 (Configuration) | M-1742 | Configured spacecraft ID from the perspective of the DTT. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 1 to 255 |
| dcc2 (Configuration) | M-1743 | Pass number. | Integer | N/A | 0 to 9999 |
| dcc2 (Configuration) | M-1744 | Mission ID for spacecraft. | Integer | N/A | 0 to 255 |
| dcc2 (Configuration) | M-1745 | Downlink carrier band. | String | N/A | "NONE", "L", "S", "X", "KA", "S26" |
| dcc2 (Configuration) | M-1746 | Uplink carrier band used for the downlink processing. It is possible that this is not the actual uplink band. | String | N/A | "NONE", "L", "S", "X", "KA" |
| dcc2 (Configuration) | M-1747 | Receiver-Telemetry configuration table name. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc2 (Configuration) | M-1748 | Receiver radiometrics (frequency) predicts mode. NPX => no predicts used, 1W => one-way, 2W => two-way, 3W => three-way. | String | N/A | "NPX", "1W", "2W", "3W" |
| dcc2 (Configuration) | M-1750 | Uplink antenna for three-way predicts. | Integer | N/A | 0 to 99 |
| dcc2 (Configuration) | M-1751 | Predicted carrier frequency | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc2 (Configuration) | M-1752 | Predicted subcarrier frequency | Double | Hz | 0.000 to 2000000.000 |
| dcc2 (Configuration) | M-1753 | Predicted symbol rate | Double | Hz | 4.000 to 26400000.000 |
| dcc2 (Configuration) | M-1754 | SNT measurement switch. If the measurement is disabled, the value in M-1818 is the predicted value. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1755 | Monopulse tracking processing switch. Used only when there is a Ka-band downlink (M-1745 is equal to "KA"). | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1756 | Name of the radiometric predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc2 (Configuration) | M-1757 | Time that the last radiometric predict mode change was done. This channel contains the seconds portion of the total time. Channels M-1757, M-1758, and M-1759 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Configuration) | M-1758 | Time that the last radiometric predict mode change was done. This channel contains the nanoseconds portion of the total time. Channels M-1757, M-1758, and M-1759 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1757 | >=0 |
| dcc2 (Configuration) | M-1759 | Time that the last radiometric predict mode change was done. This channel contains the leap seconds portion of the total time. Channels M-1757, M-1758, and M-1759 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Configuration) | M-1760 | Telemetry predicts usage mode. If enabled, telemetry predicts are used if the radiometric predict mode (M-1748) is not NPX. | String | N/A | "DISABLED", "ENABLED" |
| dcc2 (Configuration) | M-1761 | Name of the telemetry predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc2 (Configuration) | M-1762 | Range state | String | N/A | "Enable", "Disable" |
| dcc2 (Configuration) | M-1763 | Carrier phase (tracking) data output switch. If enabled, carrier phase data is being sent (count of blocks given in M-2008). | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1764 | Range data output switch. If enabled, range data is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1765 | Telemetry data output switch. If enabled, telemetry is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Configuration) | M-1766 | The LNA (Low Noise Amplifier) number from which the channel is receiving the signal. | String | N/A | "A1", "A2", "A3", "A4", "N/A" |
| dcc2 (Configuration) | M-1767 | The polarization of the signal that the downlink channel is tracking. | String | N/A | "LCP", "RCP", "UNKN", "N/A" |
| dcc2 (Configuration) | M-1768 | The configuration of the microwave for uplink. Possible configurations are low noise (LONO), diplexed (DPLX), radar configuration (RADR), and non-diplexed (NDPX, another name for low noise). | String | N/A | "LONO", "RADR", "DPLX", "NPDX", "UNKN", "N/A" |
| dcc2 (Ranging) | M-2000 | Time that the ranging acquisition started. Also referred to as the T0 time. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc2 (Ranging) | M-2003* | Time that the correlation validity last changed. This channel contains the seconds portion of the total time. Channels M-2003, M-2004, and M-2005 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Ranging) | M-2004* | Time that the correlation validity last changed. This channel contains the nanoseconds portion of the total time. Channels M-2003, M-2004, and M-2005 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2003 | >=0 |
| dcc2 (Ranging) | M-2005* | Time that the correlation validity last changed. This channel contains the leap seconds portion of the total time. Channels M-2003, M-2004, and M-2005 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Ranging) | M-2006 | Indicates whether or not the last ranging correlation was valid. | String | N/A | "OFF", "OUT OF LOCK", "IN LOCK" |
| dcc2 (Ranging) | M-2007 | The number of range data blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc2 (Ranging) | M-2008 | The number of carrier phase blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc2 (Ranging) | M-2009 | Number of ranging points that have been measured. Value is reset to zero every time ranging acquisition is initiated. | Integer | N/A | 0 to (231 - 1) |
| dcc2 (Ranging) | M-2010 | The Figure of Merit, expressed as a percentage, of the range measurement. | Float | percent | 0.0 to 100.0 |
| dcc2 (Ranging) | M-2011 | Range residual. The measured range value minus the predicted value. | Double | range units | -230 to 230 |
| dcc2 (Ranging) | M-2012 | Ranging power to noise spectral density ratio (Pr/NO). | Float | dB-Hz | -10.0 to 90.0 |
| dcc2 (Ranging) | M-2013 | Pr/NO residual. The measured Pr/N0 minus the predicted value. | Float | dB-Hz | -100.0 to 100.0 |
| dcc2 (Ranging) | M-2014 | Type of ranging being performed. SSIN => Sequential with sinewave clock, SSQ => Sequential with squarewave clock, PSIN => PN with sinewave clock, PSQ => PN with squarewave clock, PCON => PN, no clock. | String | N/A | "SSIN", "SSQ", "PSIN", "PSQ", "PCON" |
| dcc2 (Ranging) | M-2015 | Round trip light time used. Valid only if ranging type (channel M-2014) is Sequential. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc2 (Ranging) | M-2016 | Highest component of sequential ranging code (also called the clock component). Frequency is approximately 66*2-n MHz, where n is the first component value. Valid only if ranging type (channel M-2014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc2 (Ranging) | M-2017 | Lowest component of sequential ranging code. Frequency is approximately 66*2-n MHz, where n is the last component value. Valid only if ranging type (channel M-2014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc2 (Ranging) | M-2018 | Component value that is used for chopping the lower components at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-2014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc2 (Ranging) | M-2019 | Component value that the chopping starts at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-2014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc2 (Ranging) | M-2020 | Length of time that the first component is transmitted. Only valid if ranging type (channel M-2014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc2 (Ranging) | M-2021 | Length of time that the all components except the first are transmitted. Only valid if ranging type (channel M-2014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc2 (Ranging) | M-2022 | Length of time that the first component is transmitted for each DRVID measurement. Only valid if ranging type (channel M-2014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc2 (Ranging) | M-2023 | Number of DRVID measurements per ranging cycle. Only valid if ranging type (channel M-2014) is Sequential. | Integer | N/A | 0 to 255 |
| dcc2 (Ranging) | M-2024 | The integration time, in PN cycle periods, that the measurement was made. Valid only if ranging type (channel M-2014) is PN. | Integer | seconds | 1 to (231 - 1) |
| dcc2 (Ranging) | M-2025 | The reference signal is divided by this value to get the modulation clock. Valid only if ranging type (channel M-2014) is PN. | Integer | N/A | 1 to 64 (normally 64) |
| dcc2 (Ranging) | M-2026 | Length of the 1st PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2027 | The symbol (chip) pattern of the 1st PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2028 | The Boolean operation associated with the 1st PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2029 | Length of the 2nd PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2030 | The symbol (chip) pattern of the 2nd PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2031 | The Boolean operation associated with the 2nd PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2032 | Length of the 3rd PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2033 | The symbol (chip) pattern of the 3rd PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2034 | The Boolean operation associated with the 3rd PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2035 | Length of the 4th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2036 | The symbol (chip) pattern of the 4th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2037 | The Boolean operation associated with the 4th PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2038 | Length of the 5th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2039 | The symbol (chip) pattern of the 5th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2040 | The Boolean operation associated with the 5th PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2041 | Length of the 6th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc2 (Ranging) | M-2042 | The symbol (chip) pattern of the 6th PN sequence. Valid only if ranging type (channel M-2014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc2 (Ranging) | M-2043 | The Boolean operation associated with the 6th PN sequence. Valid only if ranging type (channel M-2014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc2 (Ranging) | M-2044 | Differenced Range versus Integrated Doppler (DRVID) measurement in Range Units. | Double | range units | -230 to 230 |
| dcc2 (Ranging) | M-2045 | Current operational mode of the ranging process. | String | N/A | "IDLE", "TRK", "CAL", "DIAG" |
| dcc2 (Ranging) | M-2046 | Time of the range and DRVID measurements. | String | N/A | "hh:mm:ss" |
| dcc2 (Ranging) | M-2047 | Range measurement in Range Units. | Double | range units | 0.00 to 230 |
| dcc2 (Ranging) | M-2048 | The desired ranging Figure of Merit. | Float | Percentage | 0.0 to 100.0 |
| dcc2 (Status) | M-1700 | Status code for the overall downlink channel status. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1701 | Status qualifier for the overall downlink channel status. This qualifies the status code (M-1700). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1702 | Status code for the overall receiver equipment controller status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1703 | Status qualifier for the overall receiver equipment controller status. This qualifies the status code (M-1702). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1704 | Status of the receiving function of the downlink channel. "Out of Service" indicates the receiving function is not installed. | Integer | N/A | 0 = Out Of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1705 | Status qualifier for the receiving function of the downlink channel. No qualifiers are used to qualify the status code (M-1704). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1706 | Status code for the status of the downlink channel controller of the downlink channel. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1707 | Status qualifier for the status of the downlink channel controller of the downlink channel. No qualifiers are used to qualify the status code (M-1706). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1708* | Status code for the status of the monopulse function of the downlink channel. "Out of Service" indicates the monopulse function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1709* | Status qualifier for the status of the monopulse function of the downlink channel. No qualifiers are used to qualify the status code (M-1708). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1710 | Status code for the status of the ranging function of the downlink channel. "Out of Service" indicates the ranging function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1711 | Status qualifier for the status of the ranging function of the downlink channel. No qualifiers are used to qualify the status code (M-1710). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1712 | Status code for the status of the standard telemetry function of the downlink channel. "Out of Service" indicates the telemetry function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1713 | Status qualifier for the status of the standard telemetry function of the downlink channel. No qualifiers are used to qualify the status code (M-1712). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1714* | Status code for the status of the MCD3 decoder function of the downlink channel. "Out of Service" indicates the MCD3 function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1715* | Status qualifier for the status of the MCD3 decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-1714). | Integer | N/A | 0 = None |
| dcc2 (Status) | M-1716* | Status code for the status of the turbo decoder function of the downlink channel. "Out of Service" indicates the turbo decoder function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc2 (Status) | M-1717* | Status qualifier for the status of the turbo decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-1716). | Integer | N/A | 0 = None |
| dcc2 (Subcarrier Loop) | M-1840 | The time of the last change in the predicted subcarrier frequency. This channel contains the seconds portion of the total time. Channels M-1840, M-1841, and M-1842 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Subcarrier Loop) | M-1841 | The time of the last change in the predicted subcarrier frequency. This channel contains the nanoseconds portion of the total time. Channels M-1840, M-1841, and M-1842 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1840 | >=0 |
| dcc2 (Subcarrier Loop) | M-1842 | The time of the last change in the predicted subcarrier frequency. This channel contains the leap seconds portion of the total time. Channels M-1840, M-1841, and M-1842 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Subcarrier Loop) | M-1843 | The measured subcarrier frequency | Double | Hz | 500.000 to 6000000.000 |
| dcc2 (Subcarrier Loop) | M-1844 | Subcarrier frequency residual. The measured subcarrier frequency minus the predicted value. | Float | Hz | -1.000e6 to 1.000e6 |
| dcc2 (Subcarrier Loop) | M-1845 | Subcarrier loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc2 (Subcarrier Loop) | M-1846 | The subcarrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc2 (Subcarrier Loop) | M-1847 | The windowing on the subcarrier phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc2 (Subcarrier Loop) | M-1848 | Subcarrier tracking loop lock status. Status is out-of-lock unless the loop status (M-1852) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc2 (Subcarrier Loop) | M-1849* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the seconds portion of the total time tag. Channels M-1849, M-1850, and M-1851 together contain the total time tag. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Subcarrier Loop) | M-1850* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time tag. Channels M-1849, M-1850, and M-1851 together contain the total time tag. | Integer | number of nanoseconds since the seconds value in M-1849 | >=0 |
| dcc2 (Subcarrier Loop) | M-1851* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time tag. Channels M-1849, M-1850, and M-1851 together contain the total time tag. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Subcarrier Loop) | M-1852 | Subcarrier tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc2 (Subcarrier Loop) | M-1853 | The subcarrier data power to noise spectral density ratio (Pd/NO). If value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc2 (Subcarrier Loop) | M-1854 | Pd/NO residual. The measured subcarrier Pd/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc2 (Subcarrier Loop) | M-1855 | The subcarrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc2 (Subcarrier Loop) | M-1856 | Subcarrier waveform. | String | N/A | “SIN”, “SQ”, “D” |
| dcc2 (Subcarrier Loop) | M-1857 | Predicted subcarrier frequency (including predicted Doppler shifts). | Double | Hz | 500.000 to 6000000.000 |
| dcc2 (Subcarrier Loop) | M-1858 | Subcarrier acquisition FFT usage switch. If enabled, the subcarrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Subcarrier Loop) | M-1859 | Indicates whether or not the subcarrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Symbol Loop) | M-1880 | The time of the last change in the predicted symbol frequency. This channel contains the seconds portion of the total time. Channels M-1880, M-1881, and M-1882 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Symbol Loop) | M-1881 | The time of the last change in the predicted symbol frequency. This channel contains the nanoseconds portion of the total time. Channels M-1880, M-1881, and M-1882 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1880 | >=0 |
| dcc2 (Symbol Loop) | M-1882 | The time of the last change in the predicted symbol frequency. This channel contains the leap seconds portion of the total time. Channels M-1880, M-1881, and M-1882 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Symbol Loop) | M-1883 | The measured symbol rate | Double | sps | 4.000 to 26400000.000 |
| dcc2 (Symbol Loop) | M-1884 | Symbol rate residual. The measured symbol rate minus the predicted value. | Float | sps | -26400000.000 to 26400000.000 |
| dcc2 (Symbol Loop) | M-1885 | Symbol loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc2 (Symbol Loop) | M-1886 | The symbol loop filter order. | Integer | N/A | 1 to 3 |
| dcc2 (Symbol Loop) | M-1887 | The windowing on the symbol phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc2 (Symbol Loop) | M-1888 | Symbol tracking loop lock status. Status is out-of-lock unless the loop status (M-1892) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc2 (Symbol Loop) | M-1889* | Time that the symbol tracking loop lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1889, M-1890, and M-1891 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Symbol Loop) | M-1890* | Time that the symbol tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1889, M-1890, and M-1891 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1889 | >=0 |
| dcc2 (Symbol Loop) | M-1891* | Time that the symbol tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1889, M-1890, and M-1891 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Symbol Loop) | M-1892 | Symbol tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc2 (Symbol Loop) | M-1893 | The measured symbol signal to noise ratio (SSNR). | Float | dB | -10.0 to 50.0 |
| dcc2 (Symbol Loop) | M-1894 | SSNR residual. The measured SSNR minus the predicted value. | Float | dB | -50.0 to 50.0 |
| dcc2 (Symbol Loop) | M-1895 | The symbol tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc2 (Symbol Loop) | M-1896 | Modulation format of the symbols. | String | N/A | “NRZL”, “NRZM”, “NRZS”, “BIPS”, “BIPL”, “BIPM” |
| dcc2 (Symbol Loop) | M-1897 | Configuration of the symbol smoothing algorithm (low threshold telemetry mode). | String | N/A | “ENABLED”, “DISABLED” |
| dcc2 (Symbol Loop) | M-1898 | Predicted symbol frequency (including predicted Doppler shifts). | Double | sps | 4.000 to 26400000.000 |
| dcc2 (Symbol Loop) | M-1899 | Symbol acquisition FFT usage switch. If enabled, the symbol acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Symbol Loop) | M-1900 | Indicates whether or not the symbol FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Telemetry) | M-0290 | Number of Virtual Channel 0 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0291 | Number of Virtual Channel 1 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0292 | Number of Virtual Channel 2 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0293 | Number of Virtual Channel 3 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0294 | Number of Virtual Channel 4 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0295 | Number of Virtual Channel 5 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0296 | Number of Virtual Channel 6 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-0297 | Number of Virtual Channel 7 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1915 | Operational state of the Reed-Solomon decoder. In Primary (P) mode, the corrected bits are output to the end user. In Secondary (S) mode, the decoding statistics are provided for the operator, but the bit corrections are not output. | String | N/A | "D", “P”, “S” |
| dcc2 (Telemetry) | M-1916 | Indicator of whether the input signal is an arrayed signal or not. A value of 0 indicates not arrayed; values of 1 and 2 correspond to the arraying equipment physical number. | Integer | N/A | 0 = Unarrayed, 1 = Arrayed with FSPA1 as the source, 2 = Arrayed with FSPA2 as the source |
| dcc2 (Telemetry) | M-1917 | Frame sync operational mode. | String | N/A | “D”, “P” |
| dcc2 (Telemetry) | M-1918 | The pattern of bits of the frame sync word. | String | N/A | Any pattern expressed in hexadecimal |
| dcc2 (Telemetry) | M-1919 | Length of the frame sync word. | Integer | bits | 8 to 64 |
| dcc2 (Telemetry) | M-1920 | Predicted telemetry bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc2 (Telemetry) | M-1921 | Measured bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc2 (Telemetry) | M-1922 | Overall telemetry lock status (includes decoders and frame synchronizer). | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc2 (Telemetry) | M-1924 | Telemetry processing operational status | String | N/A | "GO", "NOGO" |
| dcc2 (Telemetry) | M-1925* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1925, M-1926, and M-1927 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Telemetry) | M-1926* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1925, M-1926, and M-1927 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1925 | >=0 |
| dcc2 (Telemetry) | M-1927* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1925, M-1926, and M-1927 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Telemetry) | M-1928* | Time that the decoder lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1928, M-1929, and M-1930 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Telemetry) | M-1929* | Time that the decoder lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1928, M-1929, and M-1930 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1928 | >=0 |
| dcc2 (Telemetry) | M-1930* | Time that the decoder lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1928, M-1929, and M-1930 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Telemetry) | M-1931 | Telemetry decoder lock status. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc2 (Telemetry) | M-1933 | Type of decoder used. | String | N/A | "MCD2", "MCD3", "TURBO", "UNC", "SYMB", "GSSR" |
| dcc2 (Telemetry) | M-1934 | The output bit signal to noise ratio for the decoder. | Double | dB | -10.0 to 50.0 |
| dcc2 (Telemetry) | M-1936 | In-lock threshold for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | Integer | N/A | 1 to 255 |
| dcc2 (Telemetry) | M-1937 | Vector set for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | String | N/A | "CCSDS", "DSN" |
| dcc2 (Telemetry) | M-1938 | In-lock threshold for MCD3 decoder. Only valid if decoder type is MCD3 decoder. | Integer | N/A | 1 to 256 |
| dcc2 (Telemetry) | M-1939 | Vector set for the MCD3 decoder. Only valid if decoder type is MCD3 decoder. | String | N/A | "NONE", "CCSDS", "DSN", "A15R6", "CAS", "MAP" |
| dcc2 (Telemetry) | M-1940 | Frame sync primary frame length | Integer | bits | 8 to 65536 |
| dcc2 (Telemetry) | M-1941 | Frame sync in lock bit error tolerance. The number of bit errors allowed in the frame sync word while still being considered in lock. | Integer | bits | 0 to 15 |
| dcc2 (Telemetry) | M-1942 | Frame sync out of lock bit error tolerance (the minimum number of bit errors required to declare out of lock). | Integer | bits | 0 to 15 |
| dcc2 (Telemetry) | M-1943 | Frame sync in lock threshold (the number of consecutive in sync frames verified before lock is declared). | Integer | frames | 1 to 15 |
| dcc2 (Telemetry) | M-1944 | Frame sync out of lock threshold (number of consecutive out of sync frames before out-of-lock is declared). | Integer | frames | 1 to 15 |
| dcc2 (Telemetry) | M-1945* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-1945, M-1946, and M-1947 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Telemetry) | M-1946* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-1945, M-1946, and M-1947 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1945 | >=0 |
| dcc2 (Telemetry) | M-1947* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-1945, M-1946, and M-1947 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Telemetry) | M-1948* | Time that the frame sync lock status was last changed. This channel contains the seconds portion of the total time. Channels M-1948, M-1949, and M-1950 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc2 (Telemetry) | M-1949* | Time that the frame sync lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-1948, M-1949, and M-1950 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-1948 | >=0 |
| dcc2 (Telemetry) | M-1950* | Time that the frame sync lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-1948, M-1949, and M-1950 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc2 (Telemetry) | M-1951 | Frame sync lock status. | String | N/A | "IDLE", "SEARCH", "VERIFY", "LOCK", "FLYWHEEL" |
| dcc2 (Telemetry) | M-1952 | The number of bits the frame synchronizer processed in its last (if in-lock) or current (if out-of-lock) acquisition. When the frame sync goes to out-of-lock, the value is reset to zero. | Integer | bits | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1953 | Number of telemetry blocks output since the start of the connection. | Integer | blocks | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1955 | Count of total number of frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1956 | Count of the number of good frames (verify, flywheel, and lock) since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1957 | Count of the number of bad (search) frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1958 | Lock status of the Cyclic Redundancy Code (CRC) check. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc2 (Telemetry) | M-1959 | Configuration of the pseudo derandomizer. | String | N/A | "ENABLED", "DISABLED" |
| dcc2 (Telemetry) | M-1960 | Count of total number of frames that have had the CRC checked since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1961 | Count of the number of frames that have passed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1962 | Count of the number of frames that have failed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1963 | Lock status of the Reed-Solomon decoder. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc2 (Telemetry) | M-1964 | Count of total number of Reed-Solomon code words that have been checked since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1965 | Count of the number of Reed-Solomon code words that have been decoded since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1966 | Count of the number of Reed-Solomon code words that have failed to decode since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1967 | Ratio of bad frames to total frames for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc2 (Telemetry) | M-1968 | Ratio of bad code words to total code words for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc2 (Telemetry) | M-1969 | Count of total number of locked (flywheel and lock) frames since start of connection. Count rolls over to zero. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1970 | The number of frames with flywheel status in the last (or current) in-lock period. When lock is first acquired, the value is reset to zero. | Integer | frames | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1971 | Name of the format table used by the default telemetry stream. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc2 (Telemetry) | M-1974 | Out-of-lock threshold for Viterbi decoder (7, _). | Integer | bits | 1 to 65535 |
| dcc2 (Telemetry) | M-1975 | Out-of-lock threshold for the MCD3 decoder. | Integer | bits | 1 to 255 |
| dcc2 (Telemetry) | M-1976 | Configuration of the differential decoder (disabled, mark, or space). | String | N/A | “D”, “NM”, "NS" |
| dcc2 (Telemetry) | M-1977 | State of the Viterbi decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc2 (Telemetry) | M-1978 | State of the MCD3 decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc2 (Telemetry) | M-1979 | The number of times the metric normalization count exceeded the threshold since the TLP was put in the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1980 | Percentage of symbol errors detected over the last accumulation period. | Double | Percentage | 0.0 to 100.0 |
| dcc2 (Telemetry) | M-1981 | Viterbi decoder node sync change algorithm. | String | N/A | “D”, “SER”, “MNR” |
| dcc2 (Telemetry) | M-1982 | Number of node sync changes since the TLP was put into the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1983 | Position that the MCD3 decoder has locked onto. | Integer | N/A | 0 to 5 |
| dcc2 (Telemetry) | M-1984 | Frame sync secondary frame length (alternate frame length). | Integer | bits | 0 to 65528 |
| dcc2 (Telemetry) | M-1985 | Detected frame size. | Integer | bits | 8 to 65536 |
| dcc2 (Telemetry) | M-1986 | Status of the frame synchronizer hardware. | String | N/A | "GO", "NOGO" |
| dcc2 (Telemetry) | M-1987 | The order of the sequence for searching for frame synchronization (e.g. left-to-right (MSB) or right-to-left (LSB)). Allows handling of spacecraft that transmit recorded data backwards. M => MSB, L => LSB, and B => Both. | String | N/A | “M”, “L”, “B” |
| dcc2 (Telemetry) | M-1988 | Number of bits over or under the frame length the synchronizer will accept as a correct length frame. | Integer | bits | 0 to 3 |
| dcc2 (Telemetry) | M-1989 | The bit error rate (BER) of the frame sync words for the in-lock frames. | Double | Percentage | 0.0 to 100.0 |
| dcc2 (Telemetry) | M-1990 | The detected bit sequence. | String | N/A | “MSB”, “LSB” |
| dcc2 (Telemetry) | M-1991 | The polarity of the input data to the frame synchronizer. | String | N/A | “NORMAL”, “INVERTED” |
| dcc2 (Telemetry) | M-1992 | The number of bit slips detected since the start of processing. This count is the number of bits slipped, not the number of bit slip occurrences. | Integer | bits | 0 to (231 - 1) |
| dcc2 (Telemetry) | M-1993 | The number of polarity changes detected since the start of processing. | Integer | N/A | 0 to 65535 |
| dcc2 (Telemetry) | M-1994 | The number of times since the start of the processing that the frame synchronizer has transitioned to the LOCK state. | Integer | N/A | 0 to 65535 |
| dcc2 (Telemetry) | M-1995 | Number of Reed-Solomon codewords interleaved into a transfer frame. | Integer | Codewords | 0 to 5 |
| dcc2 (Telemetry) | M-1996 | Reed-Solomon decoding algorithm used (Berlekamp versus Conventional). | String | N/A | “B”, “C” |
| dcc2 (Telemetry) | M-1997 | Number of zero pad symbols (bytes) used in the Reed-Solomon encoding. This is also known as the virtual fill. | Integer | Reed-Solomon symbols | 0 to 223 |
| dcc2 (Telemetry) | M-1998 | The number of symbol errors detected divided by the number of symbols received. Codewords that cannot be corrected are not counted. | Double | Percentage | 0.0 to 100.0 |
| dcc2 (Telemetry) | M-1999 | Reed-Solomon decoder hardware status. | String | N/A | “GO”, “NOGO” |
| dcc3 (Carrier Loop) | M-2290 | Time tag of carrier performance data. This channel contains the seconds portion of the total time. Channels M-2290, M-2291, and M-2292 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Carrier Loop) | M-2291 | Time tag of carrier performance data. This channel contains the nanoseconds portion of the total time. Channels M-2290, M-2291, and M-2292 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2290 | >=0 |
| dcc3 (Carrier Loop) | M-2292 | Time tag of carrier performance data. This channel contains the leap seconds portion of the total time. Channels M-2290, M-2291, and M-2292 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Carrier Loop) | M-2293 | Time that the last carrier acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2293, M-2294, and M-2295 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Carrier Loop) | M-2294 | Time that the last carrier acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2293, M-2294, and M-2295 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2293 | >=0 |
| dcc3 (Carrier Loop) | M-2295 | Time that the last carrier acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2293, M-2294, and M-2295 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Carrier Loop) | M-2296 | Time that the carrier lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2296, M-2297, and M-2298 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Carrier Loop) | M-2297 | Time that the carrier lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2296, M-2297, and M-2298 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2296 | >=0 |
| dcc3 (Carrier Loop) | M-2298 | Time that the carrier lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2296, M-2297, and M-2298 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Carrier Loop) | M-2299 | Carrier loop tracking bandwidth. | Float | Hz | 0.100 to 1000.000 |
| dcc3 (Carrier Loop) | M-2300 | The carrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc3 (Carrier Loop) | M-2301 | Carrier tracking loop lock status. Status is out-of-lock unless the loop status (M-2302) is IN LOCK. | Integer | N/A | 0 = Out of lock, 1 = In lock. |
| dcc3 (Carrier Loop) | M-2302 | Carrier tracking loop status. This is the result of the acquisition command or enabling the loop. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc3 (Carrier Loop) | M-2303 | Data tracking loop status (combined status of subcarrier and symbol loops). | Integer | N/A | 0 = Out of lock, 1= In lock, 2 = Off (Idle), 3 = Acquiring, 4 = Open |
| dcc3 (Carrier Loop) | M-2304 | The measured carrier frequency. | Double | Hz | 2200000000.000 to 32,300,000,000.000 |
| dcc3 (Carrier Loop) | M-2305 | Carrier frequency residual. The measured carrier frequency minus the predicted value. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc3 (Carrier Loop) | M-2306 | The predicted carrier frequency minus the measured value. This is commonly called the doppler residual. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc3 (Carrier Loop) | M-2307 | The measured carrier frequency rate (first derivative of the frequency). | Float | Hz/sec | -108000.0 to 108000.0 |
| dcc3 (Carrier Loop) | M-2308 | The measured carrier frequency acceleration (second derivative of the frequency). | Float | Hz/sec2 | -2600.0 to 2600.0 |
| dcc3 (Carrier Loop) | M-2309 | The standard deviation of the detrended frequency residuals. This is commonly called the doppler noise. | Float | Hz | 0.000 to 1000.000 |
| dcc3 (Carrier Loop) | M-2310 | Carrier power at performance time tag (channels M-2290, M-2291, and M-2292). If value not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc3 (Carrier Loop) | M-2311 | Carrier power residual. The estimated carrier power minus the predicted value. | Float | dB | -190.0 to 190.0 |
| dcc3 (Carrier Loop) | M-2312 | The measured data power. If value is not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc3 (Carrier Loop) | M-2313 | Data power residual. The measured data power minus the predicted value. | Float | dBm | -190.0 to 190.0 |
| dcc3 (Carrier Loop) | M-2314 | The carrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc3 (Carrier Loop) | M-2315 | The carrier power to noise spectral density ratio (Pc/NO). If the value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc3 (Carrier Loop) | M-2316 | Pc/NO residual. The measured Pc/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc3 (Carrier Loop) | M-2317 | Carrier tracking mode (source of carrier error signal). | String | N/A | "RESIDUAL", "SUPPRESSED", "SIDEBAND", "QPSK", "OQPSK", "OPEN" |
| dcc3 (Carrier Loop) | M-2318 | The system noise temperature (SNT). | Float | kelvin | 10.0 to 2000.0 |
| dcc3 (Carrier Loop) | M-2319 | SNT residual. The measured system noise temperature minus the predicted value. | Float | kelvin | 10.0 to 2000.0 |
| dcc3 (Carrier Loop) | M-2320 | The predicted carrier frequency. | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc3 (Carrier Loop) | M-2321 | Count of the number of cycle slips detected during this pass. | Float | number of slips | 0.0 to 10000.0 |
| dcc3 (Carrier Loop) | M-2322 | Carrier acquisition FFT usage switch. If enabled, the carrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Carrier Loop) | M-2323 | Indicates whether or not the carrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2240 | Downlink channel providing the data. | Integer | N/A | 1 to 24 |
| dcc3 (Configuration) | M-2241 | Antenna being used from the perspective of the DTT (Downlink Tracking and Telemetry) subsystem. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 0 to 99 |
| dcc3 (Configuration) | M-2242 | Configured spacecraft ID from the perspective of the DTT. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 1 to 255 |
| dcc3 (Configuration) | M-2243 | Pass number. | Integer | N/A | 0 to 9999 |
| dcc3 (Configuration) | M-2244 | Mission ID for spacecraft. | Integer | N/A | 0 to 255 |
| dcc3 (Configuration) | M-2245 | Downlink carrier band. | String | N/A | "NONE", "L", "S", "X", "KA", "S26" |
| dcc3 (Configuration) | M-2246 | Uplink carrier band used for the downlink processing. It is possible that this is not the actual uplink band. | String | N/A | "NONE", "L", "S", "X", "KA" |
| dcc3 (Configuration) | M-2247 | Receiver-Telemetry configuration table name. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc3 (Configuration) | M-2248 | Receiver radiometrics (frequency) predicts mode. NPX => no predicts used, 1W => one-way, 2W => two-way, 3W => three-way. | String | N/A | "NPX", "1W", "2W", "3W" |
| dcc3 (Configuration) | M-2250 | Uplink antenna for three-way predicts. | Integer | N/A | 0 to 99 |
| dcc3 (Configuration) | M-2251 | Predicted carrier frequency | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc3 (Configuration) | M-2252 | Predicted subcarrier frequency | Double | Hz | 0.000 to 2000000.000 |
| dcc3 (Configuration) | M-2253 | Predicted symbol rate | Double | Hz | 4.000 to 26400000.000 |
| dcc3 (Configuration) | M-2254 | SNT measurement switch. If the measurement is disabled, the value in M-2318 is the predicted value. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2255 | Monopulse tracking processing switch. Used only when there is a Ka-band downlink (M-2245 is equal to "KA"). | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2256 | Name of the radiometric predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc3 (Configuration) | M-2257 | Time that the last radiometric predict mode change was done. This channel contains the seconds portion of the total time. Channels M-2257, M-2258, and M-2259 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Configuration) | M-2258 | Time that the last radiometric predict mode change was done. This channel contains the nanoseconds portion of the total time. Channels M-2257, M-2258, and M-2259 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2257 | >=0 |
| dcc3 (Configuration) | M-2259 | Time that the last radiometric predict mode change was done. This channel contains the leap seconds portion of the total time. Channels M-2257, M-2258, and M-2259 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Configuration) | M-2260 | Telemetry predicts usage mode. If enabled, telemetry predicts are used if the radiometric predict mode (M-2248) is not NPX. | String | N/A | "DISABLED", "ENABLED" |
| dcc3 (Configuration) | M-2261 | Name of the telemetry predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc3 (Configuration) | M-2262 | Range state | String | N/A | "Enable", "Disable" |
| dcc3 (Configuration) | M-2263 | Carrier phase (tracking) data output switch. If enabled, carrier phase data is being sent (count of blocks given in M-2508). | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2264 | Range data output switch. If enabled, range data is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2265 | Telemetry data output switch. If enabled, telemetry is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Configuration) | M-2266 | The LNA (Low Noise Amplifier) number from which the channel is receiving the signal. | String | N/A | "A1", "A2", "A3", "A4", "N/A" |
| dcc3 (Configuration) | M-2267 | The polarization of the signal that the downlink channel is tracking. | String | N/A | "LCP", "RCP", "UNKN", "N/A" |
| dcc3 (Configuration) | M-2268 | The configuration of the microwave for uplink. Possible configurations are low noise (LONO), diplexed (DPLX), radar configuration (RADR), and non-diplexed (NDPX, another name for low noise). | String | N/A | "LONO", "RADR", "DPLX", "NPDX", "UNKN", "N/A" |
| dcc3 (Ranging) | M-2500 | Time that the ranging acquisition started. Also referred to as the T0 time. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc3 (Ranging) | M-2503* | Time that the correlation validity last changed. This channel contains the seconds portion of the total time. Channels M-2503, M-2504, and M-2505 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Ranging) | M-2504* | Time that the correlation validity last changed. This channel contains the nanoseconds portion of the total time. Channels M-2503, M-2504, and M-2505 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2503 | >=0 |
| dcc3 (Ranging) | M-2505* | Time that the correlation validity last changed. This channel contains the leap seconds portion of the total time. Channels M-2503, M-2504, and M-2505 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Ranging) | M-2506 | Indicates whether or not the last ranging correlation was valid. | String | N/A | "OFF", "OUT OF LOCK", "IN LOCK" |
| dcc3 (Ranging) | M-2507 | The number of range data blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc3 (Ranging) | M-2508 | The number of carrier phase blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc3 (Ranging) | M-2509 | Number of ranging points that have been measured. Value is reset to zero every time ranging acquisition is initiated. | Integer | N/A | 0 to (231 - 1) |
| dcc3 (Ranging) | M-2510 | The Figure of Merit, expressed as a percentage, of the range measurement. | Float | percent | 0.0 to 100.0 |
| dcc3 (Ranging) | M-2511 | Range residual. The measured range value minus the predicted value. | Double | range units | -230 to 230 |
| dcc3 (Ranging) | M-2512 | Ranging power to noise spectral density ratio (Pr/NO). | Float | dB-Hz | -10.0 to 90.0 |
| dcc3 (Ranging) | M-2513 | Pr/NO residual. The measured Pr/N0 minus the predicted value. | Float | dB-Hz | -100.0 to 100.0 |
| dcc3 (Ranging) | M-2514 | Type of ranging being performed. SSIN => Sequential with sinewave clock, SSQ => Sequential with squarewave clock, PSIN => PN with sinewave clock, PSQ => PN with squarewave clock, PCON => PN, no clock. | String | N/A | "SSIN", "SSQ", "PSIN", "PSQ", "PCON" |
| dcc3 (Ranging) | M-2515 | Round trip light time used. Valid only if ranging type (channel M-2514) is Sequential. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc3 (Ranging) | M-2516 | Highest component of sequential ranging code (also called the clock component). Frequency is approximately 66*2-n MHz, where n is the first component value. Valid only if ranging type (channel M-2514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc3 (Ranging) | M-2517 | Lowest component of sequential ranging code. Frequency is approximately 66*2-n MHz, where n is the last component value. Valid only if ranging type (channel M-2514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc3 (Ranging) | M-2518 | Component value that is used for chopping the lower components at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-2514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc3 (Ranging) | M-2519 | Component value that the chopping starts at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-2514) is Sequential. | Integer | N/A | 1 to 24 |
| dcc3 (Ranging) | M-2520 | Length of time that the first component is transmitted. Only valid if ranging type (channel M-2514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc3 (Ranging) | M-2521 | Length of time that the all components except the first are transmitted. Only valid if ranging type (channel M-2514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc3 (Ranging) | M-2522 | Length of time that the first component is transmitted for each DRVID measurement. Only valid if ranging type (channel M-2514) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc3 (Ranging) | M-2523 | Number of DRVID measurements per ranging cycle. Only valid if ranging type (channel M-2514) is Sequential. | Integer | N/A | 0 to 255 |
| dcc3 (Ranging) | M-2524 | The integration time, in PN cycle periods, that the measurement was made. Valid only if ranging type (channel M-2514) is PN. | Integer | seconds | 1 to (231 - 1) |
| dcc3 (Ranging) | M-2525 | The reference signal is divided by this value to get the modulation clock. Valid only if ranging type (channel M-2514) is PN. | Integer | N/A | 1 to 64 (normally 64) |
| dcc3 (Ranging) | M-2526 | Length of the 1st PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2527 | The symbol (chip) pattern of the 1st PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2528 | The Boolean operation associated with the 1st PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2529 | Length of the 2nd PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2530 | The symbol (chip) pattern of the 2nd PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2531 | The Boolean operation associated with the 2nd PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2532 | Length of the 3rd PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2533 | The symbol (chip) pattern of the 3rd PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2534 | The Boolean operation associated with the 3rd PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2535 | Length of the 4th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2536 | The symbol (chip) pattern of the 4th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2537 | The Boolean operation associated with the 4th PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2538 | Length of the 5th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2539 | The symbol (chip) pattern of the 5th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2540 | The Boolean operation associated with the 5th PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2541 | Length of the 6th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc3 (Ranging) | M-2542 | The symbol (chip) pattern of the 6th PN sequence. Valid only if ranging type (channel M-2514) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc3 (Ranging) | M-2543 | The Boolean operation associated with the 6th PN sequence. Valid only if ranging type (channel M-2514) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc3 (Ranging) | M-2544 | Differenced Range versus Integrated Doppler (DRVID) measurement in Range Units. | Double | range units | -230 to 230 |
| dcc3 (Ranging) | M-2545 | Current operational mode of the ranging process. | String | N/A | "IDLE", "TRK", "CAL", "DIAG" |
| dcc3 (Ranging) | M-2546 | Time of the range and DRVID measurements. | String | N/A | "hh:mm:ss" |
| dcc3 (Ranging) | M-2547 | Range measurement in Range Units. | Double | range units | 0.00 to 230 |
| dcc3 (Ranging) | M-2548 | The desired ranging Figure of Merit. | Float | Percentage | 0.0 to 100.0 |
| dcc3 (Status) | M-2200 | Status code for the overall downlink channel status. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2201 | Status qualifier for the overall downlink channel status. This qualifies the status code (M-2200). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2202 | Status code for the overall receiver equipment controller status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2203 | Status qualifier for the overall receiver equipment controller status. This qualifies the status code (M-2202). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2204 | Status of the receiving function of the downlink channel. "Out of Service" indicates the receiving function is not installed. | Integer | N/A | 0 = Out Of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2205 | Status qualifier for the receiving function of the downlink channel. No qualifiers are used to qualify the status code (M-2204). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2206 | Status code for the status of the downlink channel controller of the downlink channel. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2207 | Status qualifier for the status of the downlink channel controller of the downlink channel. No qualifiers are used to qualify the status code (M-2206). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2208* | Status code for the status of the monopulse function of the downlink channel. "Out of Service" indicates the monopulse function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2209* | Status qualifier for the status of the monopulse function of the downlink channel. No qualifiers are used to qualify the status code (M-2208). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2210 | Status code for the status of the ranging function of the downlink channel. "Out of Service" indicates the ranging function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2211 | Status qualifier for the status of the ranging function of the downlink channel. No qualifiers are used to qualify the status code (M-2210). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2212 | Status code for the status of the standard telemetry function of the downlink channel. "Out of Service" indicates the telemetry function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2213 | Status qualifier for the status of the standard telemetry function of the downlink channel. No qualifiers are used to qualify the status code (M-2212). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2214* | Status code for the status of the MCD3 decoder function of the downlink channel. "Out of Service" indicates the MCD3 function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2215* | Status qualifier for the status of the MCD3 decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-2214). | Integer | N/A | 0 = None |
| dcc3 (Status) | M-2216* | Status code for the status of the turbo decoder function of the downlink channel. "Out of Service" indicates the turbo decoder function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc3 (Status) | M-2217* | Status qualifier for the status of the turbo decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-2216). | Integer | N/A | 0 = None |
| dcc3 (Subcarrier Loop) | M-2340 | The time of the last change in the predicted subcarrier frequency. This channel contains the seconds portion of the total time. Channels M-2340, M-2341, and M-2342 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Subcarrier Loop) | M-2341 | The time of the last change in the predicted subcarrier frequency. This channel contains the nanoseconds portion of the total time. Channels M-2340, M-2341, and M-2342 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2340 | >=0 |
| dcc3 (Subcarrier Loop) | M-2342 | The time of the last change in the predicted subcarrier frequency. This channel contains the leap seconds portion of the total time. Channels M-2340, M-2341, and M-2342 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Subcarrier Loop) | M-2343 | The measured subcarrier frequency | Double | Hz | 500.000 to 6000000.000 |
| dcc3 (Subcarrier Loop) | M-2344 | Subcarrier frequency residual. The measured subcarrier frequency minus the predicted value. | Float | Hz | -1.000e6 to 1.000e6 |
| dcc3 (Subcarrier Loop) | M-2345 | Subcarrier loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc3 (Subcarrier Loop) | M-2346 | The subcarrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc3 (Subcarrier Loop) | M-2347 | The windowing on the subcarrier phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc3 (Subcarrier Loop) | M-2348 | Subcarrier tracking loop lock status. Status is out-of-lock unless the loop status (M-2352) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc3 (Subcarrier Loop) | M-2349* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the seconds portion of the total time tag. Channels M-2349, M-2350, and M-2351 together contain the total time tag. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Subcarrier Loop) | M-2350* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time tag. Channels M-2349, M-2350, and M-2351 together contain the total time tag. | Integer | number of nanoseconds since the seconds value in M-2349 | >=0 |
| dcc3 (Subcarrier Loop) | M-2351* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time tag. Channels M-2349, M-2350, and M-2351 together contain the total time tag. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Subcarrier Loop) | M-2352 | Subcarrier tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc3 (Subcarrier Loop) | M-2353 | The subcarrier data power to noise spectral density ratio (Pd/NO). If value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc3 (Subcarrier Loop) | M-2354 | Pd/NO residual. The measured subcarrier Pd/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc3 (Subcarrier Loop) | M-2355 | The subcarrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc3 (Subcarrier Loop) | M-2356 | Subcarrier waveform. | String | N/A | “SIN”, “SQ”, “D” |
| dcc3 (Subcarrier Loop) | M-2357 | Predicted subcarrier frequency (including predicted Doppler shifts). | Double | Hz | 500.000 to 6000000.000 |
| dcc3 (Subcarrier Loop) | M-2358 | Subcarrier acquisition FFT usage switch. If enabled, the subcarrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Subcarrier Loop) | M-2359 | Indicates whether or not the subcarrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Symbol Loop) | M-2380 | The time of the last change in the predicted symbol frequency. This channel contains the seconds portion of the total time. Channels M-2380, M-2381, and M-2382 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Symbol Loop) | M-2381 | The time of the last change in the predicted symbol frequency. This channel contains the nanoseconds portion of the total time. Channels M-2380, M-2381, and M-2382 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2380 | >=0 |
| dcc3 (Symbol Loop) | M-2382 | The time of the last change in the predicted symbol frequency. This channel contains the leap seconds portion of the total time. Channels M-2380, M-2381, and M-2382 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Symbol Loop) | M-2383 | The measured symbol rate | Double | sps | 4.000 to 26400000.000 |
| dcc3 (Symbol Loop) | M-2384 | Symbol rate residual. The measured symbol rate minus the predicted value. | Float | sps | -26400000.000 to 26400000.000 |
| dcc3 (Symbol Loop) | M-2385 | Symbol loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc3 (Symbol Loop) | M-2386 | The symbol loop filter order. | Integer | N/A | 1 to 3 |
| dcc3 (Symbol Loop) | M-2387 | The windowing on the symbol phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc3 (Symbol Loop) | M-2388 | Symbol tracking loop lock status. Status is out-of-lock unless the loop status (M-2392) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc3 (Symbol Loop) | M-2389* | Time that the symbol tracking loop lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2389, M-2390, and M-2391 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Symbol Loop) | M-2390* | Time that the symbol tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2389, M-2390, and M-2391 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2389 | >=0 |
| dcc3 (Symbol Loop) | M-2391* | Time that the symbol tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2389, M-2390, and M-2391 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Symbol Loop) | M-2392 | Symbol tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc3 (Symbol Loop) | M-2393 | The measured symbol signal to noise ratio (SSNR). | Float | dB | -10.0 to 50.0 |
| dcc3 (Symbol Loop) | M-2394 | SSNR residual. The measured SSNR minus the predicted value. | Float | dB | -50.0 to 50.0 |
| dcc3 (Symbol Loop) | M-2395 | The symbol tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc3 (Symbol Loop) | M-2396 | Modulation format of the symbols. | String | N/A | “NRZL”, “NRZM”, “NRZS”, “BIPS”, “BIPL”, “BIPM” |
| dcc3 (Symbol Loop) | M-2397 | Configuration of the symbol smoothing algorithm (low threshold telemetry mode). | String | N/A | “ENABLED”, “DISABLED” |
| dcc3 (Symbol Loop) | M-2398 | Predicted symbol frequency (including predicted Doppler shifts). | Double | sps | 4.000 to 26400000.000 |
| dcc3 (Symbol Loop) | M-2399 | Symbol acquisition FFT usage switch. If enabled, the symbol acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Symbol Loop) | M-2400 | Indicates whether or not the symbol FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Telemetry) | M-2407 | Number of Virtual Channel 0 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2408 | Number of Virtual Channel 1 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2409 | Number of Virtual Channel 2 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2410 | Number of Virtual Channel 3 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2411 | Number of Virtual Channel 4 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2412 | Number of Virtual Channel 5 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2413 | Number of Virtual Channel 6 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2414 | Number of Virtual Channel 7 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2415 | Operational state of the Reed-Solomon decoder. In Primary (P) mode, the corrected bits are output to the end user. In Secondary (S) mode, the decoding statistics are provided for the operator, but the bit corrections are not output. | String | N/A | "D", “P”, “S” |
| dcc3 (Telemetry) | M-2416 | Indicator of whether the input signal is an arrayed signal or not. A value of 0 indicates not arrayed; values of 1 and 2 correspond to the arraying equipment physical number. | Integer | N/A | 0 = Unarrayed, 1 = Arrayed with FSPA1 as the source, 2 = Arrayed with FSPA2 as the source |
| dcc3 (Telemetry) | M-2417 | Frame sync operational mode. | String | N/A | “D”, “P” |
| dcc3 (Telemetry) | M-2418 | The pattern of bits of the frame sync word. | String | N/A | Any pattern expressed in hexadecimal |
| dcc3 (Telemetry) | M-2419 | Length of the frame sync word. | Integer | bits | 8 to 64 |
| dcc3 (Telemetry) | M-2420 | Predicted telemetry bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc3 (Telemetry) | M-2421 | Measured bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc3 (Telemetry) | M-2422 | Overall telemetry lock status (includes decoders and frame synchronizer). | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc3 (Telemetry) | M-2424 | Telemetry processing operational status | String | N/A | "GO", "NOGO" |
| dcc3 (Telemetry) | M-2425* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2425, M-2426, and M-2427 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Telemetry) | M-2426* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2425, M-2426, and M-2427 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2425 | >=0 |
| dcc3 (Telemetry) | M-2427* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2425, M-2426, and M-2427 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Telemetry) | M-2428* | Time that the decoder lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2428, M-2429, and M-2430 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Telemetry) | M-2429* | Time that the decoder lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2428, M-2429, and M-2430 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2428 | >=0 |
| dcc3 (Telemetry) | M-2430* | Time that the decoder lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2428, M-2429, and M-2430 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Telemetry) | M-2431 | Telemetry decoder lock status. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc3 (Telemetry) | M-2433 | Type of decoder used. | String | N/A | "MCD2", "MCD3", "TURBO", "UNC", "SYMB", "GSSR" |
| dcc3 (Telemetry) | M-2434 | The output bit signal to noise ratio for the decoder. | Double | dB | -10.0 to 50.0 |
| dcc3 (Telemetry) | M-2436 | In-lock threshold for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | Integer | N/A | 1 to 255 |
| dcc3 (Telemetry) | M-2437 | Vector set for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | String | N/A | "CCSDS", "DSN" |
| dcc3 (Telemetry) | M-2438 | In-lock threshold for MCD3 decoder. Only valid if decoder type is MCD3 decoder. | Integer | N/A | 1 to 256 |
| dcc3 (Telemetry) | M-2439 | Vector set for the MCD3 decoder. Only valid if decoder type is MCD3 decoder. | String | N/A | "NONE", "CCSDS", "DSN", "A15R6", "CAS", "MAP" |
| dcc3 (Telemetry) | M-2440 | Frame sync primary frame length | Integer | bits | 8 to 65536 |
| dcc3 (Telemetry) | M-2441 | Frame sync in lock bit error tolerance. The number of bit errors allowed in the frame sync word while still being considered in lock. | Integer | bits | 0 to 15 |
| dcc3 (Telemetry) | M-2442 | Frame sync out of lock bit error tolerance (the minimum number of bit errors required to declare out of lock). | Integer | bits | 0 to 15 |
| dcc3 (Telemetry) | M-2443 | Frame sync in lock threshold (the number of consecutive in sync frames verified before lock is declared). | Integer | frames | 1 to 15 |
| dcc3 (Telemetry) | M-2444 | Frame sync out of lock threshold (number of consecutive out of sync frames before out-of-lock is declared). | Integer | frames | 1 to 15 |
| dcc3 (Telemetry) | M-2445* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2445, M-2446, and M-2447 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Telemetry) | M-2446* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2445, M-2446, and M-2447 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2445 | >=0 |
| dcc3 (Telemetry) | M-2447* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2445, M-2446, and M-2447 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Telemetry) | M-2448* | Time that the frame sync lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2448, M-2449, and M-2450 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc3 (Telemetry) | M-2449* | Time that the frame sync lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2448, M-2449, and M-2450 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2448 | >=0 |
| dcc3 (Telemetry) | M-2450* | Time that the frame sync lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2448, M-2449, and M-2450 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc3 (Telemetry) | M-2451 | Frame sync lock status. | String | N/A | "IDLE", "SEARCH", "VERIFY", "LOCK", "FLYWHEEL" |
| dcc3 (Telemetry) | M-2452 | The number of bits the frame synchronizer processed in its last (if in-lock) or current (if out-of-lock) acquisition. When the frame sync goes to out-of-lock, the value is reset to zero. | Integer | bits | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2453 | Number of telemetry blocks output since the start of the connection. | Integer | blocks | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2455 | Count of total number of frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2456 | Count of the number of good frames (verify, flywheel, and lock) since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2457 | Count of the number of bad (search) frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2458 | Lock status of the Cyclic Redundancy Code (CRC) check. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc3 (Telemetry) | M-2459 | Configuration of the pseudo derandomizer. | String | N/A | "ENABLED", "DISABLED" |
| dcc3 (Telemetry) | M-2460 | Count of total number of frames that have had the CRC checked since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2461 | Count of the number of frames that have passed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2462 | Count of the number of frames that have failed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2463 | Lock status of the Reed-Solomon decoder. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc3 (Telemetry) | M-2464 | Count of total number of Reed-Solomon code words that have been checked since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2465 | Count of the number of Reed-Solomon code words that have been decoded since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2466 | Count of the number of Reed-Solomon code words that have failed to decode since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2467 | Ratio of bad frames to total frames for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc3 (Telemetry) | M-2468 | Ratio of bad code words to total code words for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc3 (Telemetry) | M-2469 | Count of total number of locked (flywheel and lock) frames since start of connection. Count rolls over to zero. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2470 | The number of frames with flywheel status in the last (or current) in-lock period. When lock is first acquired, the value is reset to zero. | Integer | frames | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2471 | Name of the format table used by the default telemetry stream. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc3 (Telemetry) | M-2474 | Out-of-lock threshold for Viterbi decoder (7, _). | Integer | bits | 1 to 65535 |
| dcc3 (Telemetry) | M-2475 | Out-of-lock threshold for the MCD3 decoder. | Integer | bits | 1 to 255 |
| dcc3 (Telemetry) | M-2476 | Configuration of the differential decoder (disabled, mark, or space). | String | N/A | “D”, “NM”, "NS" |
| dcc3 (Telemetry) | M-2477 | State of the Viterbi decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc3 (Telemetry) | M-2478 | State of the MCD3 decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc3 (Telemetry) | M-2479 | The number of times the metric normalization count exceeded the threshold since the TLP was put in the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2480 | Percentage of symbol errors detected over the last accumulation period. | Double | Percentage | 0.0 to 100.0 |
| dcc3 (Telemetry) | M-2481 | Viterbi decoder node sync change algorithm. | String | N/A | “D”, “SER”, “MNR” |
| dcc3 (Telemetry) | M-2482 | Number of node sync changes since the TLP was put into the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2483 | Position that the MCD3 decoder has locked onto. | Integer | N/A | 0 to 5 |
| dcc3 (Telemetry) | M-2484 | Frame sync secondary frame length (alternate frame length). | Integer | bits | 0 to 65528 |
| dcc3 (Telemetry) | M-2485 | Detected frame size. | Integer | bits | 8 to 65536 |
| dcc3 (Telemetry) | M-2486 | Status of the frame synchronizer hardware. | String | N/A | "GO", "NOGO" |
| dcc3 (Telemetry) | M-2487 | The order of the sequence for searching for frame synchronization (e.g. left-to-right (MSB) or right-to-left (LSB)). Allows handling of spacecraft that transmit recorded data backwards. M => MSB, L => LSB, and B => Both. | String | N/A | “M”, “L”, “B” |
| dcc3 (Telemetry) | M-2488 | Number of bits over or under the frame length the synchronizer will accept as a correct length frame. | Integer | bits | 0 to 3 |
| dcc3 (Telemetry) | M-2489 | The bit error rate (BER) of the frame sync words for the in-lock frames. | Double | Percentage | 0.0 to 100.0 |
| dcc3 (Telemetry) | M-2490 | The detected bit sequence. | String | N/A | “MSB”, “LSB” |
| dcc3 (Telemetry) | M-2491 | The polarity of the input data to the frame synchronizer. | String | N/A | “NORMAL”, “INVERTED” |
| dcc3 (Telemetry) | M-2492 | The number of bit slips detected since the start of processing. This count is the number of bits slipped, not the number of bit slip occurrences. | Integer | bits | 0 to (231 - 1) |
| dcc3 (Telemetry) | M-2493 | The number of polarity changes detected since the start of processing. | Integer | N/A | 0 to 65535 |
| dcc3 (Telemetry) | M-2494 | The number of times since the start of the processing that the frame synchronizer has transitioned to the LOCK state. | Integer | N/A | 0 to 65535 |
| dcc3 (Telemetry) | M-2495 | Number of Reed-Solomon codewords interleaved into a transfer frame. | Integer | Codewords | 0 to 5 |
| dcc3 (Telemetry) | M-2496 | Reed-Solomon decoding algorithm used (Berlekamp versus Conventional). | String | N/A | “B”, “C” |
| dcc3 (Telemetry) | M-2497 | Number of zero pad symbols (bytes) used in the Reed-Solomon encoding. This is also known as the virtual fill. | Integer | Reed-Solomon symbols | 0 to 223 |
| dcc3 (Telemetry) | M-2498 | The number of symbol errors detected divided by the number of symbols received. Codewords that cannot be corrected are not counted. | Double | Percentage | 0.0 to 100.0 |
| dcc3 (Telemetry) | M-2499 | Reed-Solomon decoder hardware status. | String | N/A | “GO”, “NOGO” |
| dcc4 (Carrier Loop) | M-2790 | Time tag of carrier performance data. This channel contains the seconds portion of the total time. Channels M-2790, M-2791, and M-2792 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Carrier Loop) | M-2791 | Time tag of carrier performance data. This channel contains the nanoseconds portion of the total time. Channels M-2790, M-2791, and M-2792 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2790 | >=0 |
| dcc4 (Carrier Loop) | M-2792 | Time tag of carrier performance data. This channel contains the leap seconds portion of the total time. Channels M-2790, M-2791, and M-2792 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Carrier Loop) | M-2793 | Time that the last carrier acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2793, M-2794, and M-2795 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Carrier Loop) | M-2794 | Time that the last carrier acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2793, M-2794, and M-2795 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2793 | >=0 |
| dcc4 (Carrier Loop) | M-2795 | Time that the last carrier acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2793, M-2794, and M-2795 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Carrier Loop) | M-2796 | Time that the carrier lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2796, M-2797, and M-2798 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Carrier Loop) | M-2797 | Time that the carrier lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2796, M-2797, and M-2798 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2796 | >=0 |
| dcc4 (Carrier Loop) | M-2798 | Time that the carrier lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2796, M-2797, and M-2798 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Carrier Loop) | M-2799 | Carrier loop tracking bandwidth. | Float | Hz | 0.100 to 1000.000 |
| dcc4 (Carrier Loop) | M-2800 | The carrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc4 (Carrier Loop) | M-2801 | Carrier tracking loop lock status. Status is out-of-lock unless the loop status (M-2802) is IN LOCK. | Integer | N/A | 0 = Out of lock, 1 = In lock. |
| dcc4 (Carrier Loop) | M-2802 | Carrier tracking loop status. This is the result of the acquisition command or enabling the loop. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc4 (Carrier Loop) | M-2803 | Data tracking loop status (combined status of subcarrier and symbol loops). | Integer | N/A | 0 = Out of lock, 1= In lock, 2 = Off (Idle), 3 = Acquiring, 4 = Open |
| dcc4 (Carrier Loop) | M-2804 | The measured carrier frequency. | Double | Hz | 2200000000.000 to 32,300,000,000.000 |
| dcc4 (Carrier Loop) | M-2805 | Carrier frequency residual. The measured carrier frequency minus the predicted value. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc4 (Carrier Loop) | M-2806 | The predicted carrier frequency minus the measured value. This is commonly called the doppler residual. | Float | Hz | -1000000.000 to 1000000.000 |
| dcc4 (Carrier Loop) | M-2807 | The measured carrier frequency rate (first derivative of the frequency). | Float | Hz/sec | -108000.0 to 108000.0 |
| dcc4 (Carrier Loop) | M-2808 | The measured carrier frequency acceleration (second derivative of the frequency). | Float | Hz/sec2 | -2600.0 to 2600.0 |
| dcc4 (Carrier Loop) | M-2809 | The standard deviation of the detrended frequency residuals. This is commonly called the doppler noise. | Float | Hz | 0.000 to 1000.000 |
| dcc4 (Carrier Loop) | M-2810 | Carrier power at performance time tag (channels M-2790, M-2791, and M-2792). If value not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc4 (Carrier Loop) | M-2811 | Carrier power residual. The estimated carrier power minus the predicted value. | Float | dB | -190.0 to 190.0 |
| dcc4 (Carrier Loop) | M-2812 | The measured data power. If value is not valid, -300.0 is output. | Float | dBm | -300.0, -190.0 to 85.0 |
| dcc4 (Carrier Loop) | M-2813 | Data power residual. The measured data power minus the predicted value. | Float | dBm | -190.0 to 190.0 |
| dcc4 (Carrier Loop) | M-2814 | The carrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc4 (Carrier Loop) | M-2815 | The carrier power to noise spectral density ratio (Pc/NO). If the value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc4 (Carrier Loop) | M-2816 | Pc/NO residual. The measured Pc/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc4 (Carrier Loop) | M-2817 | Carrier tracking mode (source of carrier error signal). | String | N/A | "RESIDUAL", "SUPPRESSED", "SIDEBAND", "QPSK", "OQPSK", "OPEN" |
| dcc4 (Carrier Loop) | M-2818 | The system noise temperature (SNT). | Float | kelvin | 10.0 to 2000.0 |
| dcc4 (Carrier Loop) | M-2819 | SNT residual. The measured system noise temperature minus the predicted value. | Float | kelvin | 10.0 to 2000.0 |
| dcc4 (Carrier Loop) | M-2820 | The predicted carrier frequency. | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc4 (Carrier Loop) | M-2821 | Count of the number of cycle slips detected during this pass. | Float | number of slips | 0.0 to 10000.0 |
| dcc4 (Carrier Loop) | M-2822 | Carrier acquisition FFT usage switch. If enabled, the carrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Carrier Loop) | M-2823 | Indicates whether or not the carrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2740 | Downlink channel providing the data. | Integer | N/A | 1 to 24 |
| dcc4 (Configuration) | M-2741 | Antenna being used from the perspective of the DTT (Downlink Tracking and Telemetry) subsystem. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 0 to 99 |
| dcc4 (Configuration) | M-2742 | Configured spacecraft ID from the perspective of the DTT. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 1 to 255 |
| dcc4 (Configuration) | M-2743 | Pass number. | Integer | N/A | 0 to 9999 |
| dcc4 (Configuration) | M-2744 | Mission ID for spacecraft. | Integer | N/A | 0 to 255 |
| dcc4 (Configuration) | M-2745 | Downlink carrier band. | String | N/A | "NONE", "L", "S", "X", "KA", "S26" |
| dcc4 (Configuration) | M-2746 | Uplink carrier band used for the downlink processing. It is possible that this is not the actual uplink band. | String | N/A | "NONE", "L", "S", "X", "KA" |
| dcc4 (Configuration) | M-2747 | Receiver-Telemetry configuration table name. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc4 (Configuration) | M-2748 | Receiver radiometrics (frequency) predicts mode. NPX => no predicts used, 1W => one-way, 2W => two-way, 3W => three-way. | String | N/A | "NPX", "1W", "2W", "3W" |
| dcc4 (Configuration) | M-2750 | Uplink antenna for three-way predicts. | Integer | N/A | 0 to 99 |
| dcc4 (Configuration) | M-2751 | Predicted carrier frequency | Double | Hz | 2200000000.000 to 32300000000.000 |
| dcc4 (Configuration) | M-2752 | Predicted subcarrier frequency | Double | Hz | 0.000 to 2000000.000 |
| dcc4 (Configuration) | M-2753 | Predicted symbol rate | Double | Hz | 4.000 to 26400000.000 |
| dcc4 (Configuration) | M-2754 | SNT measurement switch. If the measurement is disabled, the value in M-2818 is the predicted value. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2755 | Monopulse tracking processing switch. Used only when there is a Ka-band downlink (M-2745 is equal to "KA"). | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2756 | Name of the radiometric predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc4 (Configuration) | M-2757 | Time that the last radiometric predict mode change was done. This channel contains the seconds portion of the total time. Channels M-2757, M-2758, and M-2759 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Configuration) | M-2758 | Time that the last radiometric predict mode change was done. This channel contains the nanoseconds portion of the total time. Channels M-2757, M-2758, and M-2759 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2757 | >=0 |
| dcc4 (Configuration) | M-2759 | Time that the last radiometric predict mode change was done. This channel contains the leap seconds portion of the total time. Channels M-2757, M-2758, and M-2759 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Configuration) | M-2760 | Telemetry predicts usage mode. If enabled, telemetry predicts are used if the radiometric predict mode (M-2748) is not NPX. | String | N/A | "DISABLED", "ENABLED" |
| dcc4 (Configuration) | M-2761 | Name of the telemetry predict set being used. The name is composed of the Franz code plus the revision level. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc4 (Configuration) | M-2762 | Range state | String | N/A | "Enable", "Disable" |
| dcc4 (Configuration) | M-2763 | Carrier phase (tracking) data output switch. If enabled, carrier phase data is being sent (count of blocks given in M-3008). | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2764 | Range data output switch. If enabled, range data is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2765 | Telemetry data output switch. If enabled, telemetry is being sent by the DTT. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Configuration) | M-2766 | The LNA (Low Noise Amplifier) number from which the channel is receiving the signal. | String | N/A | "A1", "A2", "A3", "A4", "N/A" |
| dcc4 (Configuration) | M-2767 | The polarization of the signal that the downlink channel is tracking. | String | N/A | "LCP", "RCP", "UNKN", "N/A" |
| dcc4 (Configuration) | M-2768 | The configuration of the microwave for uplink. Possible configurations are low noise (LONO), diplexed (DPLX), radar configuration (RADR), and non-diplexed (NDPX, another name for low noise). | String | N/A | "LONO", "RADR", "DPLX", "NPDX", "UNKN", "N/A" |
| dcc4 (Ranging) | M-3000 | Time that the ranging acquisition started. Also referred to as the T0 time. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc4 (Ranging) | M-3003* | Time that the correlation validity last changed. This channel contains the seconds portion of the total time. Channels M-3003, M-3004, and M-3005 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Ranging) | M-3004* | Time that the correlation validity last changed. This channel contains the nanoseconds portion of the total time. Channels M-3003, M-3004, and M-3005 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-3003 | >=0 |
| dcc4 (Ranging) | M-3005* | Time that the correlation validity last changed. This channel contains the leap seconds portion of the total time. Channels M-3003, M-3004, and M-3005 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Ranging) | M-3006 | Indicates whether or not the last ranging correlation was valid. | String | N/A | "OFF", "OUT OF LOCK", "IN LOCK" |
| dcc4 (Ranging) | M-3007 | The number of range data blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc4 (Ranging) | M-3008 | The number of carrier phase blocks sent out by the DTT. | Integer | blocks | 0 to (231 - 1) |
| dcc4 (Ranging) | M-3009 | Number of ranging points that have been measured. Value is reset to zero every time ranging acquisition is initiated. | Integer | N/A | 0 to (231 - 1) |
| dcc4 (Ranging) | M-3010 | The Figure of Merit, expressed as a percentage, of the range measurement. | Float | percent | 0.0 to 100.0 |
| dcc4 (Ranging) | M-3011 | Range residual. The measured range value minus the predicted value. | Double | range units | -230 to 230 |
| dcc4 (Ranging) | M-3012 | Ranging power to noise spectral density ratio (Pr/NO). | Float | dB-Hz | -10.0 to 90.0 |
| dcc4 (Ranging) | M-3013 | Pr/NO residual. The measured Pr/N0 minus the predicted value. | Float | dB-Hz | -100.0 to 100.0 |
| dcc4 (Ranging) | M-3014 | Type of ranging being performed. SSIN => Sequential with sinewave clock, SSQ => Sequential with squarewave clock, PSIN => PN with sinewave clock, PSQ => PN with squarewave clock, PCON => PN, no clock. | String | N/A | "SSIN", "SSQ", "PSIN", "PSQ", "PCON" |
| dcc4 (Ranging) | M-3015 | Round trip light time used. Valid only if ranging type (channel M-3014) is Sequential. | Integer | The time is expressed in the following format: HH * 10000 + MM * 100 + SS, where HH is 0 to 23, MM is 0 to 59, and SS is 0 to 60. | >=0 |
| dcc4 (Ranging) | M-3016 | Highest component of sequential ranging code (also called the clock component). Frequency is approximately 66*2-n MHz, where n is the first component value. Valid only if ranging type (channel M-3014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc4 (Ranging) | M-3017 | Lowest component of sequential ranging code. Frequency is approximately 66*2-n MHz, where n is the last component value. Valid only if ranging type (channel M-3014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc4 (Ranging) | M-3018 | Component value that is used for chopping the lower components at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-3014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc4 (Ranging) | M-3019 | Component value that the chopping starts at. Frequency is approximately 66*2-n MHz, where n is the chop component value. Valid only if ranging type (channel M-3014) is Sequential. | Integer | N/A | 1 to 24 |
| dcc4 (Ranging) | M-3020 | Length of time that the first component is transmitted. Only valid if ranging type (channel M-3014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc4 (Ranging) | M-3021 | Length of time that the all components except the first are transmitted. Only valid if ranging type (channel M-3014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc4 (Ranging) | M-3022 | Length of time that the first component is transmitted for each DRVID measurement. Only valid if ranging type (channel M-3014) is Sequential. | Float | seconds | 1.0 to 1.0e6 |
| dcc4 (Ranging) | M-3023 | Number of DRVID measurements per ranging cycle. Only valid if ranging type (channel M-3014) is Sequential. | Integer | N/A | 0 to 255 |
| dcc4 (Ranging) | M-3024 | The integration time, in PN cycle periods, that the measurement was made. Valid only if ranging type (channel M-3014) is PN. | Integer | seconds | 1 to (231 - 1) |
| dcc4 (Ranging) | M-3025 | The reference signal is divided by this value to get the modulation clock. Valid only if ranging type (channel M-3014) is PN. | Integer | N/A | 1 to 64 (normally 64) |
| dcc4 (Ranging) | M-3026 | Length of the 1st PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3027 | The symbol (chip) pattern of the 1st PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3028 | The Boolean operation associated with the 1st PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3029 | Length of the 2nd PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3030 | The symbol (chip) pattern of the 2nd PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3031 | The Boolean operation associated with the 2nd PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3032 | Length of the 3rd PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3033 | The symbol (chip) pattern of the 3rd PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3034 | The Boolean operation associated with the 3rd PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3035 | Length of the 4th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3036 | The symbol (chip) pattern of the 4th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3037 | The Boolean operation associated with the 4th PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3038 | Length of the 5th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3039 | The symbol (chip) pattern of the 5th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3040 | The Boolean operation associated with the 5th PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3041 | Length of the 6th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | symbols (chips) | 0 to 32 |
| dcc4 (Ranging) | M-3042 | The symbol (chip) pattern of the 6th PN sequence. Valid only if ranging type (channel M-3014) is PN. | Integer | Symbols in 32-bit binary | 0 to 0xFFFF |
| dcc4 (Ranging) | M-3043 | The Boolean operation associated with the 6th PN sequence. Valid only if ranging type (channel M-3014) is PN. | String | N/A | "AND", "OR", "XOR", "MVT" |
| dcc4 (Ranging) | M-3044 | Differenced Range versus Integrated Doppler (DRVID) measurement in Range Units. | Double | range units | -230 to 230 |
| dcc4 (Ranging) | M-3045 | Current operational mode of the ranging process. | String | N/A | "IDLE", "TRK", "CAL", "DIAG" |
| dcc4 (Ranging) | M-3046 | Time of the range and DRVID measurements. | String | N/A | "hh:mm:ss" |
| dcc4 (Ranging) | M-3047 | Range measurement in Range Units. | Double | range units | 0.00 to 230 |
| dcc4 (Ranging) | M-3048 | The desired ranging Figure of Merit. | Float | Percentage | 0.0 to 100.0 |
| dcc4 (Status) | M-2700 | Status code for the overall downlink channel status. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2701 | Status qualifier for the overall downlink channel status. This qualifies the status code (M-2700). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2702 | Status code for the overall receiver equipment controller status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2703 | Status qualifier for the overall receiver equipment controller status. This qualifies the status code (M-2702). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2704 | Status of the receiving function of the downlink channel. "Out of Service" indicates the receiving function is not installed. | Integer | N/A | 0 = Out Of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2705 | Status qualifier for the receiving function of the downlink channel. No qualifiers are used to qualify the status code (M-2704). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2706 | Status code for the status of the downlink channel controller of the downlink channel. | Integer | N/A | 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2707 | Status qualifier for the status of the downlink channel controller of the downlink channel. No qualifiers are used to qualify the status code (M-2706). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2708* | Status code for the status of the monopulse function of the downlink channel. "Out of Service" indicates the monopulse function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2709* | Status qualifier for the status of the monopulse function of the downlink channel. No qualifiers are used to qualify the status code (M-2708). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2710 | Status code for the status of the ranging function of the downlink channel. "Out of Service" indicates the ranging function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2711 | Status qualifier for the status of the ranging function of the downlink channel. No qualifiers are used to qualify the status code (M-2710). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2712 | Status code for the status of the standard telemetry function of the downlink channel. "Out of Service" indicates the telemetry function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2713 | Status qualifier for the status of the standard telemetry function of the downlink channel. No qualifiers are used to qualify the status code (M-2712). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2714* | Status code for the status of the MCD3 decoder function of the downlink channel. "Out of Service" indicates the MCD3 function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2715* | Status qualifier for the status of the MCD3 decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-2714). | Integer | N/A | 0 = None |
| dcc4 (Status) | M-2716* | Status code for the status of the turbo decoder function of the downlink channel. "Out of Service" indicates the turbo decoder function is not installed. | Integer | N/A | 0 = Out of Service, 1 = Operational, 3 = Deviation, 4 = Marginal, 5 = Critical |
| dcc4 (Status) | M-2717* | Status qualifier for the status of the turbo decoder function of the downlink channel. No qualifiers are used to qualify the status code (M-2716). | Integer | N/A | 0 = None |
| dcc4 (Subcarrier Loop) | M-2840 | The time of the last change in the predicted subcarrier frequency. This channel contains the seconds portion of the total time. Channels M-2840, M-2841, and M-2842 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Subcarrier Loop) | M-2841 | The time of the last change in the predicted subcarrier frequency. This channel contains the nanoseconds portion of the total time. Channels M-2840, M-2841, and M-2842 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2840 | >=0 |
| dcc4 (Subcarrier Loop) | M-2842 | The time of the last change in the predicted subcarrier frequency. This channel contains the leap seconds portion of the total time. Channels M-2840, M-2841, and M-2842 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Subcarrier Loop) | M-2843 | The measured subcarrier frequency | Double | Hz | 500.000 to 6000000.000 |
| dcc4 (Subcarrier Loop) | M-2844 | Subcarrier frequency residual. The measured subcarrier frequency minus the predicted value. | Float | Hz | -1.000e6 to 1.000e6 |
| dcc4 (Subcarrier Loop) | M-2845 | Subcarrier loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc4 (Subcarrier Loop) | M-2846 | The subcarrier loop filter order. | Integer | N/A | 1 to 3 |
| dcc4 (Subcarrier Loop) | M-2847 | The windowing on the subcarrier phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc4 (Subcarrier Loop) | M-2848 | Subcarrier tracking loop lock status. Status is out-of-lock unless the loop status (M-2852) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc4 (Subcarrier Loop) | M-2849* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the seconds portion of the total time tag. Channels M-2849, M-2850, and M-2851 together contain the total time tag. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Subcarrier Loop) | M-2850* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time tag. Channels M-2849, M-2850, and M-2851 together contain the total time tag. | Integer | number of nanoseconds since the seconds value in M-2849 | >=0 |
| dcc4 (Subcarrier Loop) | M-2851* | Time that the subcarrier tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time tag. Channels M-2849, M-2850, and M-2851 together contain the total time tag. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Subcarrier Loop) | M-2852 | Subcarrier tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc4 (Subcarrier Loop) | M-2853 | The subcarrier data power to noise spectral density ratio (Pd/NO). If value is not valid, -300.0 is output. | Float | dB-Hz | -300.0, 0.0 to 90.0 |
| dcc4 (Subcarrier Loop) | M-2854 | Pd/NO residual. The measured subcarrier Pd/N0 minus the predicted value. | Float | dB | -90.0 to 90.0 |
| dcc4 (Subcarrier Loop) | M-2855 | The subcarrier tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc4 (Subcarrier Loop) | M-2856 | Subcarrier waveform. | String | N/A | “SIN”, “SQ”, “D” |
| dcc4 (Subcarrier Loop) | M-2857 | Predicted subcarrier frequency (including predicted Doppler shifts). | Double | Hz | 500.000 to 6000000.000 |
| dcc4 (Subcarrier Loop) | M-2858 | Subcarrier acquisition FFT usage switch. If enabled, the subcarrier acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Subcarrier Loop) | M-2859 | Indicates whether or not the subcarrier FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Symbol Loop) | M-2880 | The time of the last change in the predicted symbol frequency. This channel contains the seconds portion of the total time. Channels M-2880, M-2881, and M-2882 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Symbol Loop) | M-2881 | The time of the last change in the predicted symbol frequency. This channel contains the nanoseconds portion of the total time. Channels M-2880, M-2881, and M-2882 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2880 | >=0 |
| dcc4 (Symbol Loop) | M-2882 | The time of the last change in the predicted symbol frequency. This channel contains the leap seconds portion of the total time. Channels M-2880, M-2881, and M-2882 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Symbol Loop) | M-2883 | The measured symbol rate | Double | sps | 4.000 to 26400000.000 |
| dcc4 (Symbol Loop) | M-2884 | Symbol rate residual. The measured symbol rate minus the predicted value. | Float | sps | -26400000.000 to 26400000.000 |
| dcc4 (Symbol Loop) | M-2885 | Symbol loop tracking bandwidth. | Float | Hz | 0.001 to 25.000 |
| dcc4 (Symbol Loop) | M-2886 | The symbol loop filter order. | Integer | N/A | 1 to 3 |
| dcc4 (Symbol Loop) | M-2887 | The windowing on the symbol phase data. The windowing value is 2-n, where n is the value reported. | Integer | N/A | 0 to 15 |
| dcc4 (Symbol Loop) | M-2888 | Symbol tracking loop lock status. Status is out-of-lock unless the loop status (M-2892) is IN LOCK. | Integer | N/A | 0 = Out of Lock, 1 = In Lock |
| dcc4 (Symbol Loop) | M-2889* | Time that the symbol tracking loop lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2889, M-2890, and M-2891 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Symbol Loop) | M-2890* | Time that the symbol tracking loop lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2889, M-2890, and M-2891 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2889 | >=0 |
| dcc4 (Symbol Loop) | M-2891* | Time that the symbol tracking loop lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2889, M-2890, and M-2891 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Symbol Loop) | M-2892 | Symbol tracking loop status. | String | N/A | "OFF", "OPEN", "FREQ SEARCH", "LOOP OPT", "WAIT FOR LOCK", "OUT OF LOCK", "IN LOCK" |
| dcc4 (Symbol Loop) | M-2893 | The measured symbol signal to noise ratio (SSNR). | Float | dB | -10.0 to 50.0 |
| dcc4 (Symbol Loop) | M-2894 | SSNR residual. The measured SSNR minus the predicted value. | Float | dB | -50.0 to 50.0 |
| dcc4 (Symbol Loop) | M-2895 | The symbol tracking loop static phase error. | Float | degrees | -90.0 to 90.0 |
| dcc4 (Symbol Loop) | M-2896 | Modulation format of the symbols. | String | N/A | “NRZL”, “NRZM”, “NRZS”, “BIPS”, “BIPL”, “BIPM” |
| dcc4 (Symbol Loop) | M-2897 | Configuration of the symbol smoothing algorithm (low threshold telemetry mode). | String | N/A | “ENABLED”, “DISABLED” |
| dcc4 (Symbol Loop) | M-2898 | Predicted symbol frequency (including predicted Doppler shifts). | Double | sps | 4.000 to 26400000.000 |
| dcc4 (Symbol Loop) | M-2899 | Symbol acquisition FFT usage switch. If enabled, the symbol acquisition will use an FFT as part of the acquisition process. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Symbol Loop) | M-2900 | Indicates whether or not the symbol FFT is currently running. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Telemetry) | M-2907 | Number of Virtual Channel 0 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2908 | Number of Virtual Channel 1 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2909 | Number of Virtual Channel 2 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2910 | Number of Virtual Channel 3 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2911 | Number of Virtual Channel 4 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2912 | Number of Virtual Channel 5 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2913 | Number of Virtual Channel 6 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2914 | Number of Virtual Channel 7 frames detected. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2915 | Operational state of the Reed-Solomon decoder. In Primary (P) mode, the corrected bits are output to the end user. In Secondary (S) mode, the decoding statistics are provided for the operator, but the bit corrections are not output. | String | N/A | "D", “P”, “S” |
| dcc4 (Telemetry) | M-2916 | Indicator of whether the input signal is an arrayed signal or not. A value of 0 indicates not arrayed; values of 1 and 2 correspond to the arraying equipment physical number. | Integer | N/A | 0 = Unarrayed, 1 = Arrayed with FSPA1 as the source, 2 = Arrayed with FSPA2 as the source |
| dcc4 (Telemetry) | M-2917 | Frame sync operational mode. | String | N/A | “D”, “P” |
| dcc4 (Telemetry) | M-2918 | The pattern of bits of the frame sync word. | String | N/A | Any pattern expressed in hexadecimal |
| dcc4 (Telemetry) | M-2919 | Length of the frame sync word. | Integer | bits | 8 to 64 |
| dcc4 (Telemetry) | M-2920 | Predicted telemetry bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc4 (Telemetry) | M-2921 | Measured bit rate. | Double | bps | 2.000 to 26.4e6 |
| dcc4 (Telemetry) | M-2922 | Overall telemetry lock status (includes decoders and frame synchronizer). | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc4 (Telemetry) | M-2924 | Telemetry processing operational status | String | N/A | "GO", "NOGO" |
| dcc4 (Telemetry) | M-2925* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2925, M-2926, and M-2927 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Telemetry) | M-2926* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2925, M-2926, and M-2927 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2925 | >=0 |
| dcc4 (Telemetry) | M-2927* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2925, M-2926, and M-2927 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Telemetry) | M-2928* | Time that the decoder lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2928, M-2929, and M-2930 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Telemetry) | M-2929* | Time that the decoder lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2928, M-2929, and M-2930 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2928 | >=0 |
| dcc4 (Telemetry) | M-2930* | Time that the decoder lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2928, M-2929, and M-2930 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Telemetry) | M-2931 | Telemetry decoder lock status. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc4 (Telemetry) | M-2933 | Type of decoder used. | String | N/A | "MCD2", "MCD3", "TURBO", "UNC", "SYMB", "GSSR" |
| dcc4 (Telemetry) | M-2934 | The output bit signal to noise ratio for the decoder. | Double | dB | -10.0 to 50.0 |
| dcc4 (Telemetry) | M-2936 | In-lock threshold for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | Integer | N/A | 1 to 255 |
| dcc4 (Telemetry) | M-2937 | Vector set for the Viterbi decoder. Only valid if decoder type is Viterbi decoder. | String | N/A | "CCSDS", "DSN" |
| dcc4 (Telemetry) | M-2938 | In-lock threshold for MCD3 decoder. Only valid if decoder type is MCD3 decoder. | Integer | N/A | 1 to 256 |
| dcc4 (Telemetry) | M-2939 | Vector set for the MCD3 decoder. Only valid if decoder type is MCD3 decoder. | String | N/A | "NONE", "CCSDS", "DSN", "A15R6", "CAS", "MAP" |
| dcc4 (Telemetry) | M-2940 | Frame sync primary frame length | Integer | bits | 8 to 65536 |
| dcc4 (Telemetry) | M-2941 | Frame sync in lock bit error tolerance. The number of bit errors allowed in the frame sync word while still being considered in lock. | Integer | bits | 0 to 15 |
| dcc4 (Telemetry) | M-2942 | Frame sync out of lock bit error tolerance (the minimum number of bit errors required to declare out of lock). | Integer | bits | 0 to 15 |
| dcc4 (Telemetry) | M-2943 | Frame sync in lock threshold (the number of consecutive in sync frames verified before lock is declared). | Integer | frames | 1 to 15 |
| dcc4 (Telemetry) | M-2944 | Frame sync out of lock threshold (number of consecutive out of sync frames before out-of-lock is declared). | Integer | frames | 1 to 15 |
| dcc4 (Telemetry) | M-2945* | Time that the last decoder acquisition command was issued. This channel contains the seconds portion of the total time. Channels M-2945, M-2946, and M-2947 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Telemetry) | M-2946* | Time that the last decoder acquisition command was issued. This channel contains the nanoseconds portion of the total time. Channels M-2945, M-2946, and M-2947 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2945 | >=0 |
| dcc4 (Telemetry) | M-2947* | Time that the last decoder acquisition command was issued. This channel contains the leap seconds portion of the total time. Channels M-2945, M-2946, and M-2947 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Telemetry) | M-2948* | Time that the frame sync lock status was last changed. This channel contains the seconds portion of the total time. Channels M-2948, M-2949, and M-2950 together contain the total time. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| dcc4 (Telemetry) | M-2949* | Time that the frame sync lock status was last changed. This channel contains the nanoseconds portion of the total time. Channels M-2948, M-2949, and M-2950 together contain the total time. | Integer | number of nanoseconds since the seconds value in M-2948 | >=0 |
| dcc4 (Telemetry) | M-2950* | Time that the frame sync lock status was last changed. This channel contains the leap seconds portion of the total time. Channels M-2948, M-2949, and M-2950 together contain the total time. | Integer | number of leap seconds in the time | >=0 |
| dcc4 (Telemetry) | M-2951 | Frame sync lock status. | String | N/A | "IDLE", "SEARCH", "VERIFY", "LOCK", "FLYWHEEL" |
| dcc4 (Telemetry) | M-2952 | The number of bits the frame synchronizer processed in its last (if in-lock) or current (if out-of-lock) acquisition. When the frame sync goes to out-of-lock, the value is reset to zero. | Integer | bits | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2953 | Number of telemetry blocks output since the start of the connection. | Integer | blocks | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2955 | Count of total number of frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2956 | Count of the number of good frames (verify, flywheel, and lock) since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2957 | Count of the number of bad (search) frames since the start of connection. Count rolls over to zero. Only valid if frame sync is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2958 | Lock status of the Cyclic Redundancy Code (CRC) check. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc4 (Telemetry) | M-2959 | Configuration of the pseudo derandomizer. | String | N/A | "ENABLED", "DISABLED" |
| dcc4 (Telemetry) | M-2960 | Count of total number of frames that have had the CRC checked since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2961 | Count of the number of frames that have passed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2962 | Count of the number of frames that have failed the CRC check since the start of connection. Count rolls over to zero. Only valid if CRC check is enabled. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2963 | Lock status of the Reed-Solomon decoder. | String | N/A | "IDLE", "IN LOCK", "OUT OF LOCK" |
| dcc4 (Telemetry) | M-2964 | Count of total number of Reed-Solomon code words that have been checked since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2965 | Count of the number of Reed-Solomon code words that have been decoded since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2966 | Count of the number of Reed-Solomon code words that have failed to decode since the start of connection. Count rolls over to zero. Only valid if Reed-Solomon decoder is enabled. | Integer | code words | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2967 | Ratio of bad frames to total frames for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc4 (Telemetry) | M-2968 | Ratio of bad code words to total code words for Reed-Solomon decoder. Only valid if Reed-Solomon decoder is enabled. | Double | Percentage | 0.00 to 100.00 |
| dcc4 (Telemetry) | M-2969 | Count of total number of locked (flywheel and lock) frames since start of connection. Count rolls over to zero. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2970 | The number of frames with flywheel status in the last (or current) in-lock period. When lock is first acquired, the value is reset to zero. | Integer | frames | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2971 | Name of the format table used by the default telemetry stream. | String | N/A | Any ASCII string, up to 242 characters long |
| dcc4 (Telemetry) | M-2974 | Out-of-lock threshold for Viterbi decoder (7, _). | Integer | bits | 1 to 65535 |
| dcc4 (Telemetry) | M-2975 | Out-of-lock threshold for the MCD3 decoder. | Integer | bits | 1 to 255 |
| dcc4 (Telemetry) | M-2976 | Configuration of the differential decoder (disabled, mark, or space). | String | N/A | “D”, “NM”, "NS" |
| dcc4 (Telemetry) | M-2977 | State of the Viterbi decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc4 (Telemetry) | M-2978 | State of the MCD3 decoder alternate symbol inversion. | String | N/A | “ENABLED”, “DISABLED” |
| dcc4 (Telemetry) | M-2979 | The number of times the metric normalization count exceeded the threshold since the TLP was put in the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2980 | Percentage of symbol errors detected over the last accumulation period. | Double | Percentage | 0.0 to 100.0 |
| dcc4 (Telemetry) | M-2981 | Viterbi decoder node sync change algorithm. | String | N/A | “D”, “SER”, “MNR” |
| dcc4 (Telemetry) | M-2982 | Number of node sync changes since the TLP was put into the active state. | Integer | N/A | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2983 | Position that the MCD3 decoder has locked onto. | Integer | N/A | 0 to 5 |
| dcc4 (Telemetry) | M-2984 | Frame sync secondary frame length (alternate frame length). | Integer | bits | 0 to 65528 |
| dcc4 (Telemetry) | M-2985 | Detected frame size. | Integer | bits | 8 to 65536 |
| dcc4 (Telemetry) | M-2986 | Status of the frame synchronizer hardware. | String | N/A | "GO", "NOGO" |
| dcc4 (Telemetry) | M-2987 | The order of the sequence for searching for frame synchronization (e.g. left-to-right (MSB) or right-to-left (LSB)). Allows handling of spacecraft that transmit recorded data backwards. M => MSB, L => LSB, and B => Both. | String | N/A | “M”, “L”, “B” |
| dcc4 (Telemetry) | M-2988 | Number of bits over or under the frame length the synchronizer will accept as a correct length frame. | Integer | bits | 0 to 3 |
| dcc4 (Telemetry) | M-2989 | The bit error rate (BER) of the frame sync words for the in-lock frames. | Double | Percentage | 0.0 to 100.0 |
| dcc4 (Telemetry) | M-2990 | The detected bit sequence. | String | N/A | “MSB”, “LSB” |
| dcc4 (Telemetry) | M-2991 | The polarity of the input data to the frame synchronizer. | String | N/A | “NORMAL”, “INVERTED” |
| dcc4 (Telemetry) | M-2992 | The number of bit slips detected since the start of processing. This count is the number of bits slipped, not the number of bit slip occurrences. | Integer | bits | 0 to (231 - 1) |
| dcc4 (Telemetry) | M-2993 | The number of polarity changes detected since the start of processing. | Integer | N/A | 0 to 65535 |
| dcc4 (Telemetry) | M-2994 | The number of times since the start of the processing that the frame synchronizer has transitioned to the LOCK state. | Integer | N/A | 0 to 65535 |
| dcc4 (Telemetry) | M-2995 | Number of Reed-Solomon codewords interleaved into a transfer frame. | Integer | Codewords | 0 to 5 |
| dcc4 (Telemetry) | M-2996 | Reed-Solomon decoding algorithm used (Berlekamp versus Conventional). | String | N/A | “B”, “C” |
| dcc4 (Telemetry) | M-2997 | Number of zero pad symbols (bytes) used in the Reed-Solomon encoding. This is also known as the virtual fill. | Integer | Reed-Solomon symbols | 0 to 223 |
| dcc4 (Telemetry) | M-2998 | The number of symbol errors detected divided by the number of symbols received. Codewords that cannot be corrected are not counted. | Double | Percentage | 0.0 to 100.0 |
| dcc4 (Telemetry) | M-2999 | Reed-Solomon decoder hardware status. | String | N/A | “GO”, “NOGO” |
| etc | M-0504 | ID of transmitter #1 | String | N/A | "S20K", "X4K", "X20K", "S400K", "S200W", "S2K", "N/A" |
| etc | M-0505 | Transmitter #1 beam | String | N/A | "ON", "OFF", "SAFE" |
| etc | M-0509 | Transmitter #1 power level | Float | watts | 0 to 600000 |
| etc | M-0535 | Command Modulation | String | N/A | "OFF", "MOD1", "MOD2", "MOD3", "UNKN" |
| etc | M-0543 | Transmitter in use. | Integer | N/A | 0, 1, 2, 3. 0 means not in use. |
| etc | M-0544 | ID of transmitter #2 | String | N/A | "S20K", "X4K", "X20K", "S400K", "S200W", "S2K", "N/A" |
| etc | M-0545 | Transmitter #2 beam | String | N/A | "ON", "OFF", "SAFE" |
| etc | M-0546 | Transmitter #2 power level | Float | watts | 0 to 600000 |
| etc | M-0547 | ID of transmitter #3 | String | N/A | "S20K", "X4K", "X20K", "S400K", "S200W", "S2K", "N/A" |
| etc | M-0548 | Transmitter #3 beam | String | N/A | "ON", "OFF", "SAFE" |
| etc | M-0549 | Transmitter #3 power level | Float | watts | 0 to 600000 |
| etc | M-0616 | Ranging Modulation Status | String | N/A | "OFF", "MOD1", "MOD2", "UNKN" |
| etc | M-0640 | Tuning Configuration, Tuning Rate. | Double | Hz/sec | 0 to 80000 |
| etc | M-0641 | Tuning Configuration, Sweep Range. | Double | Hz | 0 to 6000000 |
| etc | M-0642 | Tuning Configuration, Dwell Time 0. | Integer | seconds | 0 to 999 |
| etc | M-0643 | Tuning Configuration, Dwell Time 1. | Integer | seconds | 0 to 999 |
| etc | M-0644 | Tuning Configuration, Dwell Time 2. | Integer | seconds | 0 to 999 |
| etc | M-0645 | Tuning Configuration, T0 for template. Set to 0 if no data and not valid. This channel contains the seconds portion of the time. Channels M-0645, M-0646, and M-0647 contain the total time tag. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| etc | M-0646 | Tuning Configuration, T0 for template. Set to 0 if no data and not valid. This channel contains the nanoseconds portion of the time. Channels M-0645, M-0646, and M-0647 contain the total time tag. | Integer | number of nanoseconds since the seconds value in M-0645 | >=0 |
| etc | M-0647 | Tuning Configuration, T0 for template. Set to 0 if no data and not valid. This channel contains the leap seconds portion of the time. Channels M-0645, M-0646, and M-0647 contain the total time tag. | Integer | number of leap seconds in the time | >=0 |
| etc | M-0885 | Uplink carrier frequency | Double | Hz | 0 to 35e9 |
| mda | M-0311 | Doppler Mode Antenna 'A' Channel 1, S Band | String | N/A | "1" to "4" where "1" = one-way, "2" = two-way, "3" = three-way, "4" = three-way coherent |
| mda | M-0314 | Doppler Mode Antenna 'A' Channel 2, X Band | String | N/A | "1" to "4" where "1" = one-way, "2" = two-way, "3" = three-way, "4" = three-way coherent |
| mda | M-0611 | Channel 1 Doppler noise | Float | Hz |  |
| mda | M-0614 | Channel 1 Doppler residual | Float | Hz |  |
| mda | M-0615 | Channel 1 Range residual | Integer | Range Units |  |
| mda | M-0617 | Channel 1 Uplink Carrier suppression | Float | dB |  |
| mda | M-0618 | Channel 1 Range Power Over Noise Ratio | Float | N/A |  |
| mda | M-0619 | Channel 1 Last ranging component | Integer | N/A | 2 to 24 |
| mda | M-0620 | Channel 1 T1 clock integration time | Integer | seconds |  |
| mda | M-0621 | Channel 1 T2 component integration time | Integer | seconds |  |
| mda | M-0622 | Channel 1 T3 DRVID integration time | Integer | seconds |  |
| mda | M-0623 | Channel 1 Doppler Frequency | Double | Hz |  |
| rcc | M-0346 | Downlink band for logical receiver 1 | String | N/A | "S", "X", "Ka", "L" |
| rcc | M-0373 | Loop Status qualifier for logical receiver 2. M-0373 and M-0374 together give the loop Status for logical receiver 2. | Integer | N/A | 0 = None, 1 = Off line, 5 = In-lock, 6 = Out-of-lock, 17 = Acquiring |
| rcc | M-0374 | Loop Status code for logical receiver 2. M-0373 and M-0374 together give the loop Status for logical receiver 2. | Integer | N/A | 0 = Out of service, 1 = Operational, 5 = Critical |
| rcc | M-0376 | Downlink band for logical receiver 2 | String | N/A | "S", "X", "Ka", "L" |
| rcc | M-0714 | Loop Status qualifier for logical receiver 1. M-0714 and M-0716 together give the loop Status for logical receiver 1. | Integer | N/A | 0 = None, 1 = Off line, 5 = In-lock, 6 = Out-of-lock, 17 = Acquiring |
| rcc | M-0716 | Loop Status code for logical receiver 1. M-0714 and M-0716 together give the loop Status for logical receiver 1. | Integer | N/A | 0 = Out of service, 1 = Operational, 5 = Critical |
| rcc | M-0717 | Carrier loop lock Status for logical receiver 1 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0718 | Subcarrier loop #1 lock Status for logical receiver 1 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0719 | Symbol stream #1 lock Status for logical receiver 1 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0720 | Subcarrier loop #2 lock Status for logical receiver 1 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0721 | Symbol stream #2 lock Status for logical receiver 1 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0723 | Channel Carrier Power to Noise ratio for logical receiver 1 | Float | dB-Hz | 0 to 90 |
| rcc | M-0724 | PDW validity field for SNT from logical receiver 1 (M-0725) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0725 | System noise temperature (SNT) for logical receiver 1 | Float | kelvin | 10 to 2000 |
| rcc | M-0727 | Carrier Power for logical receiver 1 | Float | dBm | -185 to -45 |
| rcc | M-0729 | Subcarrier #1 Data Power to Noise Ratio, Pd/N0, for logical receiver #1 | Float | dB-Hz | 0 to 90 |
| rcc | M-0730 | PDW validity field for symbol stream #1 SNR from logical receiver 1 (M-0731) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0731 | Symbol Stream #1 SNR for logical receiver 1 | Float | dB | -20 to 20 |
| rcc | M-0733 | Subcarrier #2 Data Power to Noise Ratio, Pd/N0, for logical receiver 1 | Float | dB-Hz | 0 to 90 |
| rcc | M-0734 | PDW validity field for symbol stream #2 SNR from logical receiver 1 (M-0735) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0735 | Symbol Stream #2 SNR for logical receiver 1 | Float | dB | -20 to 20 |
| rcc | M-0737 | Carrier Static Phase Error for logical receiver 1 | Float | degrees | -90 to 90 |
| rcc | M-0739 | Carrier loop bandwidth for logical receiver 1 | Float | Hz | 0.01 to 500 |
| rcc | M-0741 | Carrier loop noise bandwidth for logical receiver 1 | Float | Hz | 0.01 to 500 |
| rcc | M-0742 | Subcarrier #1 loop noise bandwidth for logical receiver 1 | Float | Hz | 0.001 to 10 |
| rcc | M-0743 | Symbol stream #1 loop noise bandwidth for logical receiver 1 | Float | Hz | 0.001 to 10 |
| rcc | M-0744 | Subcarrier #2 loop noise bandwidth for logical receiver 1 | Float | Hz | 0.001 to 10 |
| rcc | M-0745 | Symbol stream #2 loop noise bandwidth for logical receiver 1 | Float | Hz | 0.001 to 10 |
| rcc | M-0746 | Carrier frequency for logical receiver 1 | Double | Hz | 0.0 to 32.3e9 |
| rcc | M-0748 | Symbol stream #1 rate for logical receiver 1 | Double | N/A | 4 sps to 26.4 Msps |
| rcc | M-0749 | Symbol stream #2 rate for logical receiver 1 | Double | N/A | 4 sps to 26.4 Msps |
| rcc | M-0767 | Carrier loop lock Status for logical receiver 2 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0768 | Subcarrier loop #1 lock Status for logical receiver 2 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0769 | Symbol stream #1 lock Status for logical receiver 2 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0770 | Subcarrier loop #2 lock Status for logical receiver 2 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0771 | Symbol stream #2 lock Status for logical receiver 2 | String | N/A | "OFF", "ON", "OPEN", "ACQ", "IN", "OUT" |
| rcc | M-0773 | Channel Carrier Power to Noise ratio for logical receiver 2 | Float | dB-Hz | 0 to 90 |
| rcc | M-0774 | PDW validity field for SNT from logical receiver 2 (M-0775) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0775 | System noise temperature (SNT) for logical receiver 2 | Float | kelvin | 10 to 2000 |
| rcc | M-0777 | Carrier Power for logical receiver 2 | Float | dBm | -185 to -45 |
| rcc | M-0779 | Subcarrier #1 Data Power to Noise Ratio, Pd/N0, for logical receiver #2 | Float | dB-Hz | 0 to 90 |
| rcc | M-0780 | PDW validity field for symbol stream #1 SNR from logical receiver 2 (M-0781) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0781 | Symbol Stream #1 SNR for logical receiver 2 | Float | dB | -20 to 20 |
| rcc | M-0783 | Subcarrier #2 Data Power to Noise Ratio, Pd/N0, for logical receiver 2 | Float | dB-Hz | 0 to 90 |
| rcc | M-0784 | PDW validity field for symbol stream #2 SNR from logical receiver 2 (M-0785) | Integer | N/A | 0 = valid, 1 = invalid |
| rcc | M-0785 | Symbol Stream #2 SNR for logical receiver 2 | Float | dB | -20 to 20 |
| rcc | M-0787 | Carrier Static Phase Error for logical receiver 2 | Float | degrees | -90 to 90 |
| rcc | M-0789 | Carrier loop bandwidth for logical receiver 2 | Float | Hz | 0.01 to 500 |
| rcc | M-0791 | Carrier loop noise bandwidth for logical receiver 2 | Float | Hz | 0.01 to 500 |
| rcc | M-0792 | Subcarrier #1 loop noise bandwidth for logical receiver 2 | Float | Hz | 0.001 to 10 |
| rcc | M-0793 | Symbol stream #1 loop noise bandwidth for logical receiver 2 | Float | Hz | 0.001 to 10 |
| rcc | M-0794 | Subcarrier #2 loop noise bandwidth for logical receiver 2 | Float | Hz | 0.001 to 10 |
| rcc | M-0795 | Symbol stream #2 loop noise bandwidth for logical receiver 2 | Float | Hz | 0.001 to 10 |
| rcc | M-0796 | Carrier frequency for logical receiver 2 | Double | Hz | 0.0 to 32.3e9 |
| rcc | M-0798 | Symbol stream #1 rate for logical receiver 2 | Double | N/A | 4 sps to 26.4 Msps |
| rcc | M-0799 | Symbol stream #2 rate for logical receiver 2 | Double | N/A | 4 sps to 26.4 Msps |
| rcc | M-0800 | Receiver Channel Processor #1 Predicts Mode | String | N/A | "1W", "2W", "3Wxx", "NPX" (xx => uplink DSS, NPX => no predicts) |
| rcc | M-0801 | Receiver Channel Processor #2 Predicts Mode | String | N/A | "1W", "2W", "3Wxx", "NPX" (xx => uplink DSS, NPX => no predicts) |
| rcc | M-0802 | Receiver Channel Processor #1 Configuration Table ID | String | N/A | valid table label (up to 8 characters) |
| rcc | M-0803 | Receiver Channel Processor #2 Configuration Table ID | String | N/A | valid table label (up to 8 characters) |
| tgc | M-0101 | PDW validity field for channel 1 bit rate and subcarrier frequency (M-0104 and M-0109) | Integer | N/A | 0 = valid, 1 = invalid |
| tgc | M-0104 | Channel 1 telemetry bit rate | Float | bps | 0 to 2.2e6 |
| tgc | M-0109 | Channel 1 subcarrier frequency | Double | Hz | 6.0 to 4.0e6 |
| tgc | M-0115 | Real-time telemetry combiner (RTC) channel 1 Status qualifier. M-0115 and M-0116 together give the RTC channel 1 Status. | Integer | N/A | 0 = none, 5 = in lock, 6 = out of lock |
| tgc | M-0116 | Real-time telemetry combiner (RTC) channel 1 Status code. M-0115 and M-0116 together give the RTC channel 1 Status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 5 = Critical |
| tgc | M-0125 | MCD channel 1 lock Status qualifier. M-0125 and M-0126 together give the MCD channel 1 Status. | Integer | N/A | 0 = none, 5 = in lock, 6 = out of lock |
| tgc | M-0126 | MCD channel 1 lock Status code. M-0125 and M-0126 together give the MCD channel 1 Status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 5 = Critical |
| tgc | M-0130 | MCD channel 1 bit SNR | Double | N/A | 0.0 to 30.0 |
| tgc | M-0140 | Channel 1 In-lock bit error tolerance | Integer | N/A | 0 to 15 |
| tgc | M-0141 | Channel 1 Out-of-lock bit error tolerance | Integer | N/A | 0 to 15 |
| tgc | M-0142 | Channel 1 in-lock threshold | Integer | N/A | 1 to 15 |
| tgc | M-0143 | Channel 1 Out-of-lock threshold | Integer | N/A | 1 to 15 |
| tgc | M-0144 | PDW validity field for channel 1 FSS Status data (M-0147, M-0148, M-0149, M-0152, and M-0153) | Integer | N/A | 0 = valid, 1 = invalid |
| tgc | M-0147 | Channel 1 Lock Status | Integer | N/A | 0 = in lock (lock or flywheel), 1 = out of lock (search or verify) |
| tgc | M-0148 | Channel 1 FSS Out of Lock Reason | Integer | N/A | 0 = Not out-of-lock, 1 = Flywheel threshold expired (or never in-lock), 2 = Forced into Search, 3 = both 1 and 2 |
| tgc | M-0149 | Channel 1 In-lock frame count | Integer | frames | 0 to 65535 |
| tgc | M-0152 | Channel 1 Bits to acquire | Integer | bits | 0 to 4294967295 |
| tgc | M-0153 | Channel 1 Flywheel frame count | Integer | frames | 0 to 65535 |
| tgc | M-0169 | Channel 1 Reed-Solomon input bit SNR | Float | decibels | 0.00 to 40.00 |
| tgc | M-0201 | PDW validity field for channel 2 bit rate and subcarrier frequency (M-0204 and M-0209) | Integer | N/A | 0 = valid, 1 = invalid |
| tgc | M-0204 | Channel 2 telemetry bit rate | Float | bps | 0 to 2.2e6 |
| tgc | M-0209 | Channel 2 subcarrier frequency | Double | Hz | 6.0 to 4.0e6 |
| tgc | M-0215 | Real-time telemetry combiner (RTC) channel 2 Status qualifier. M-0215 and M-0216 together give the RTC channel 2 Status. | Integer | N/A | 0 = none, 5 = in lock, 6 = out of lock |
| tgc | M-0216 | Real-time telemetry combiner (RTC) channel 2 Status code. M-0215 and M-0216 together give the RTC channel 2 Status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 5 = Critical |
| tgc | M-0225 | MCD channel 2 lock Status qualifier. M-0225 and M-0226 together give the MCD channel 2 Status. | Integer | N/A | 0 = none, 5 = in lock, 6 = out of lock |
| tgc | M-0226 | MCD channel 2 lock Status code. M-0225 and M-0226 together give the MCD channel 2 Status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 5 = Critical |
| tgc | M-0230 | MCD channel 2 bit SNR | Double | N/A | 0.0 to 30.0 |
| tgc | M-0240 | Channel 2 In-lock bit error tolerance | Integer | N/A | 0 to 15 |
| tgc | M-0241 | Channel 2 Out-of-lock bit error tolerance | Integer | N/A | 0 to 15 |
| tgc | M-0242 | Channel 2 in-lock threshold | Integer | N/A | 1 to 15 |
| tgc | M-0243 | Channel 2 Out-of-lock threshold | Integer | N/A | 1 to 15 |
| tgc | M-0244 | PDW validity field for channel 2 FSS Status data (M-0247, M-0248, M-0249, M-0252, and M-0253) | Integer | N/A | 0 = valid, 1 = invalid |
| tgc | M-0247 | Channel 2 Lock Status | Integer | N/A | 0 = in lock (lock or flywheel), 1 = out of lock (search or verify) |
| tgc | M-0248 | Channel 2 FSS Out of Lock Reason | Integer | N/A | 0 = Not out-of-lock, 1 = Flywheel threshold expired (or never in-lock), 2 = Forced into Search, 3 = both 1 and 2 |
| tgc | M-0249 | Channel 2 In-lock frame count | Integer | frames | 0 to 65535 |
| tgc | M-0252 | Channel 2 Bits to acquire | Integer | bits | 0 to 4294967295 |
| tgc | M-0253 | Channel 2 Flywheel frame count | Integer | frames | 0 to 65535 |
| tgc | M-0269 | Channel 2 Reed-Solomon input bit SNR | Float | decibels | 0.00 to 40.00 |
| tx_hp | M-0514 | High Power Transmitter ID | String | N/A | "SIM", "SNNN", "RDRS", "RDRX" |
| tx_hp | M-0515 | High Power Transmitter Beam | String | N/A | "OFF", "ON", "SAFE" |
| tx_hp | M-0519 | High Power Transmitter Power | String | kW | up to 12 characters |
| ulc | M-0969 | Subsystem version ID. This is the software version of the uplink subsystem. | String | N/A | "aVx.y.z" where a is a letter identifying the operational revision and x.y.z is a number indicating the program version. |
| ulc | M-0970 | Subsystem Status code. Overall subsystem Status, based on assembly Status and current activities. This monitor item is only available when using the UPL-6167-OPCV3.2.0 version of uplink. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical, 6 = Emergency |
| ulc | M-0971 | Subsystem Status qualifier. Overall subsystem Status, based on assembly Status and current activities. This monitor item is only available when using the UPL-6167-OPCV3.2.0 version of uplink. | Integer | N/A | 0 = None |
| ulc | M-0972 | This item is a human-readable text string that says why the overall Status (channels M-0970 and M-0971) has the value it has. If the situation is nominal, its value is the empty string. This monitor item is only available when using the UPL-6167-OPCV3.2.0 version of uplink. | String | N/A | up to 242 characters long |
| ulc | M-0973 | The uplink assembly providing data. | Integer | N/A | 0 = Invalid/Unknown, 1 = S-/X-band uplink, 2 = Ka-band uplink |
| ulc | M-0974 | When the subsystem is "In Service", the spacecraft number for which the subsystem is configured; otherwise 0. Note that due to manual configuration by the operator, this may be different than the NMC settings. | Integer | N/A | 1 to 255, 0 when not configured. |
| ulc | M-0975 | When the subsystem is "In Service", the pass number for which the subsystem is configured; otherwise 0. | Integer | N/A | >=1, 0 when not configured. |
| ulc/ccp | M-0850 | Source of abort directive (SLE) | String | N/A | "POCC", "Controller", "None" |
| ulc/ccp | M-0851 | ETX Command Modulation. Indicates whether the CCP thinks the Exciter command modulation is on or off for the port to which the CMG is attached. | String | N/A | "Off", "On" |
| ulc/ccp | M-0852 | CSMC Control Mode. For UPL-6167-OPBV2.0.1, the control mode is either remote or local. For UPL-6167-OPCV3.2.0, the CMD function operates in one of these four modes: normal radiation (RAD), configuration (CNF), emergency file radiation (EFRAD), or emergency file recording (EFREC). | String | N/A | For UPL-6167-OPBV2.0.1: "Remote", "Local" For UPL-6167-OPCV3.2.0: "CNF", "RAD", "EFREC", "EFRAD" |
| ulc/ccp | M-0853 | Front end monitoring Status code. Indicates whether monitoring of the front end (exciter/transmitter) is enabled from the exciter onwards. This flag is controlled by the FEM directive. M-0853 and M-0854 together give the front end monitoring status. | Integer | N/A | 1 = Operational, 3 = Deviation |
| ulc/ccp | M-0854 | Front end monitoring Status qualifier. Indicates whether monitoring of the front end (exciter/transmitter) is enabled from the exciter onwards. This flag is controlled by the FEM directive. M-0853 and M-0854 together give the front end monitoring status. | Integer | N/A | 11 = Enabled, 12 = Disabled |
| ulc/ccp | M-0855 | Transmitter monitoring Status code. Indicates whether or not monitoring of the front end is enabled from the transmitter onwards. If exciter monitoring (M-0853 and M-0854) is disabled, then this will be disabled as well. M-0855 and M-0856 give the transmitter monitoring status. | Integer | N/A | 1 = Operational, 3 = Deviation |
| ulc/ccp | M-0856 | Transmitter monitoring Status qualifier. Indicates whether or not monitoring of the front end is enabled from the transmitter onwards. If exciter monitoring (M-0853 and M-0854) is disabled, then this will be disabled, too. M-0855 and M-0856 give the transmitter monitoring status. | Integer | N/A | 11 = Enabled, 12 = Disabled |
| ulc/ccp | M-0857 | Transmitter Output State. This value is received from the exciter and indicates whether a carrier signal is going out the horn ("On"), into a water load ("Water"), or not being produced ("Off"). The CMD function uses this to determine whether or not commands can be radiated by the transmitter. | String | N/A | "Off", "On", "Water" |
| ulc/ccp | M-0858 | The number of CLTUs successfully radiated during the current support. Note that Throughput Command blocks are reformatted as CLTUs by the CCP prior to radiation by the CMG. This statistic is computed for all command services. | Integer | N/A | >=0 |
| ulc/ccp | M-0859 | Total number of Throughput command blocks received and validated during the current support (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0860 | Total number of Throughput command blocks received during the current support (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0861 | Total number of Throughput blocks rejected during the current support (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0862 | Number of times that radiation of a command block was aborted during the current support (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0863 | Number of command blocks flushed from the queue due to aborts during the current support. Note that the flushed blocks (M-0863) plus the aborted blocks (M-0862) is the number of command blocks accepted but not completely radiated. (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0864 | Warning alarm sent by the CCP when the queue reaches a certain percentage capacity. Only applies to Throughput service mode (CMD-4-9). | Integer | percent | 0 to 100 |
| ulc/ccp | M-0865 | Percentage of the queue presently in use. Only applies to the Throughput service mode (CMD-4-9). | Integer | percent | 0 to 100 |
| ulc/ccp | M-0882 | The SLE Initiator ID of the CLTU service user. This is used to generate security credential to check against incoming PDUs. | String | N/A | up to 72 characters long |
| ulc/ccp | M-0883 | The SLE Responder ID of the CLTU service provider. This is used to generate security credential for PDUs sent to the user. | String | N/A | up to 72 characters long |
| ulc/ccp | M-0884 | Configured service mode. Indicates the service mode for which the CCP is configured, if any. | String | N/A | For UPL-6167-OPBV2.0.1: "Throughput", "CLTU", "File", "Packet", "Unknown" For UPL-6167-OPCV3.2.0: "Throughput", "CLTU", "Unknown" |
| ulc/ccp | M-0886 | The number of CLTUs which were aborted at the CCP/CMG during the CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0887 | The number of CLTUs accepted by the CCP during this CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0888 | The number of CLTUs which expired at the CCP/CMG during this CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0889 | The number of CLTUs flushed from the queue due to aborts during the current CLTU support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0890 | The number of CLTUs processed by the CCP during this CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0891 | The number of CLTUs received from the CLTU user during this CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0892 | The number of CLTUs rejected by the CCP during this CLTU service support. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0893 | The number of CLTUs in the queue. (SLE) | Integer | N/A | >=0 |
| ulc/ccp | M-0894 | Total number of Throughput test blocks received during the current support. (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-0895 | The total number of valid Throughput blocks (including both command and test blocks) received during the current support. (CMD-4-9) | Integer | N/A | >=0 |
| ulc/ccp | M-1014 | Command Modulation Change Time. Gives the time when the exciter command modulation (M-0851) monitor data item last changed. This channel contains the number of seconds since January 1, 1970 UTC, in the time. The total time tag is contained in M-1014, M-1015, and M-1016. This monitor data item is only available when using the UPL-6167-OPCV3.2.0 version of the uplink subsystem. | Integer | number of seconds since January 1, 1970 UTC, excluding leap seconds | >=0 |
| ulc/ccp | M-1015 | Command Modulation Change Time. Gives the time when the exciter command modulation (M-0851) monitor data item last changed. This channel contains the the number nanoseconds since the seconds value (M-1014) in the time. The total time tag is contained in M-1014, M-1015, and M-1016. This monitor data item is only available when using the UPL-6167-OPCV3.2.0 version of the uplink subsystem. | Integer | number of nanoseconds since the seconds value in M-1014 | >=0 |
| ulc/ccp | M-1016 | Command Modulation Change Time. Gives the time when the exciter command modulation (M-0851) monitor data item last changed. This channel contains the count of leap seconds in the time. The total time tag is contained in M-1014, M-1015, and M-1016. This monitor data item is only available when using the UPL-6167-OPCV3.2.0 version of the uplink subsystem. | Integer | number of leap seconds in the time | >=0 |
| ulc/ccp | M-1017 | Indicates if a CLTU is currently being radiated by the CMG. This monitor item is only available when using the UPL-6167-OPCV3.2.0 version of the uplink subsystem. | String | N/A | "Radiating", "Not Radiating" |
| ulc/ccp | M-1018 | Name of the service table used to configure the subsystem. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1019 | Service Instance Id used for CLTU configuration. A period-delimited list of the service instance Id. This string needs to match the user service instance Id before CCP will accept the BIND request. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1020 | CLTU Authentication Trigger. The CLTU user must send a password for authentication; this item indicates when the password will be checked. | String | N/A | For UPL-6167-OPBV2.0.1: "Never", "On Bind", "Always" For UPL-6167-OPCV3.2.0: "NONE", "BIND", "ALL" |
| ulc/ccp | M-1021 | The configuration set name of the service table. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1022 | For UPL-6167-OPBV2.0.1: This is the Status code for the overall Status of the Command subsystem. For UPL-6167-OPCV3.2.0: This is the Status code for the overall Status of the CCP assembly. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical, 6 = Emergency |
| ulc/ccp | M-1023 | For UPL-6167-OPBV2.0.1: This is the Status qualifier for the overall Status of the Command subsystem. For UPL-6167-OPCV3.2.0: This is the Status qualifier for the overall Status of the CCP assembly. | Integer | N/A | 0 = None |
| ulc/ccp | M-1024 | This item is a human-readable text string that says why the overall Status (channels M-1022 and M-1023) has the value it has. If the situation is nominal, its value is the empty string. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1025 | Status code for the CCP Configuration Status. | Integer | N/A | 1 = Operational |
| ulc/ccp | M-1026 | Status qualifier for the CCP Configuration Status. Offline indicates the CCP is not configured for commanding. Awaiting operator indicates command service has been requested, but configuration failed due to a configuration error or a lack of configuration information. Online indicates command service has been requested, and configuration is proceeding normally (or is complete). | Integer | N/A | 1 = Offline, 2 = Online, 9 = Awaiting operator |
| ulc/ccp | M-1027 | Text explanation of CCP configuration Status (channels M-1025 and M-1026). This item is a human-readable text string that explains and amplifies CCP configuration Status. It may describe the current configuration activity; if configuration failed, it explains why. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1028 | Status code of command service Status. Indicates the health of the command service. | Integer | N/A | 1 = Operational, 3 = Deviation, 5 = Critical |
| ulc/ccp | M-1029 | Status qualifier of command service Status. Indicates the health of the command service. | Integer | N/A | 0 = None |
| ulc/ccp | M-1030 | Text explanation of command service Status (channels M-1028 and M-1029). A human-readable string that explains why the command service Status has the value it has. If Status is nominal, this string is empty. | String | N/A | up to 242 characters long |
| ulc/ccp | M-1031 | CLTU Service State. The current state of the SLE service provider. | String | N/A | "Unbound", "Bind pending", "Ready", "Unbind pending", "Start pending", "Active", "Stop pending" |
| ulc/ccp | M-1032 | CLTU Production State. The current production state of the SLE service provider. | String | N/A | "Waiting", "Active", "Halted" |
| ulc/ccp | M-1033 | Acquisition Sequence Trigger. When the acquisition sequence should be sent to the spacecraft. | String | N/A | "Never", "When Configured", "When Ready", "When Active" |
| ulc/ccp | M-1034 | Maximum CLTU size. The maximum command block (CLTU) size, in bytes. | Integer | bytes | >=0 |
| ulc/ccp | M-1035 | Minimum CLTU size. The minimum command block (CLTU) size, in bytes. | Integer | bytes | >=0 |
| ulc/ccp | M-1036 | The ID of the latest CLTU received during this CLTU service support. | Integer | N/A | >=0 |
| ulc/ccp | M-1037 | The ID of the latest CLTU accepted by the CCP during this CLTU service support. | Integer | N/A | >=0 |
| ulc/ccp | M-1038 | The ID of the latest CLTU radiated during this CLTU service support. | Integer | N/A | >=0 |
| ulc/ccp | M-1077 | Status code for the transmitter output status. This value is received from the exciter. Critical means no carrier signal is being produced. Operational means a carrier signal is going out the horn. Marginal indicates a carrier signal is going into a water load. | Integer | N/A | 1 = Operational, 4 = Marginal, 5 = Critical |
| ulc/ccp | M-1078 | Status qualifier for the transmitter output status. This qualifies the status code (M-1077). | Integer | N/A | 0 = None |
| ulc/ccp | M-1079 | Transmitter Output Diagnostic Text. This value explains why M-0857 has the value it does. If the state is nominal, this item's value is the empty string. | String | N/A | up to 242 characters long |
| ulc/cmg | M-0866 | The command carrier suppression in dBs. This value derives from the modulation index. | Double | dB | 0.2 to 6.0 (Sine Wave), 0.4 to 12.0 (Square Wave) |
| ulc/cmg | M-0867 | Actual radiation data rate in bits per second. | Double | bps | 1.0 to 4120 |
| ulc/cmg | M-0868 | Reason for failure to load a CLTU. If not blank, then the CCP failed to load a CLTU into the CMG. The CLTU ID is included in this message. (SLE) | String | N/A | "", "Expired Time, CLTU n", "Checksum Error, CLTU n", "Invalid Size, CLTU n", "Buffer Exceeded, CLTU n", "Unknown Reason, CLTU n" In the above messages, n is the CLTU ID number. |
| ulc/cmg | M-0869 | Whether the CMG is currently modulating or not, i.e., sending a signal to the ETC. Note that the CMG is only told to modulate when ETC command modulation is enabled. | String | N/A | "Off", "On" |
| ulc/cmg | M-0870 | The CMG's current operating mode. Test means the CMG is performing a pre-cal test to verify communications with the ETC; Ready means the CMG is sending idle pattern if idle is defined for this spacecraft; Active means the CMG is sending CLTUs. | String | N/A | "Test", "Off", "Ready", "Active" |
| ulc/cmg | M-0871 | Loss of Reference Signal by CMG. This item is non-blank when the CMG notifies the CCP that it has lost its 10 MHz reference signal from the FTS. | String | N/A | "", "10MHz Reference Signal Failure on CMG" |
| ulc/cmg | M-0872 | Overall CMG Status code. This is the overall Status of the CMG assembly. M-0872 and M-0873 together give the overall CMG Status. | Integer | N/A | 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical |
| ulc/cmg | M-0873 | Overall CMG Status qualifier. This is the overall Status of the CMG assembly. M-0872 and M-0873 together give the overall CMG Status. | Integer | N/A | 0 = None |
| ulc/cmg | M-0874 | TCT Error. This item is non-blank when the CMG notifies the CCP that it has detected a TCT failure. | String | N/A | "", "TCT Failure Reported by CMG" |
| ulc/cmg | M-0875 | This item is non-blank when the CMG notifies the CCP that the verification feedback signal failed to match the transmitted signal. | String | N/A | "", "Out of Tolerance" |
| ulc/cmg | M-0876 | This item is non-blank when the CMG fails to communicate to the CCP within a predetermined time period. | String | N/A | "", "Watch dog timer expired" |
| ulc/cmg | M-0877 | Command subcarrier phase angle to be applied to the command subcarrier in degrees. Note: the modulation index for X-band carriers must be limited to 80.21 degrees peak (1.40 radians) if verification feedback from the Exciter to the CMG is used. | Double | degrees | For UPL-6167-OPBV2.0.1: 20 to 85. For UPL-6167-OPCV3.2.0: 17.18 to 89.95 |
| ulc/cmg | M-0878 | CMG subcarrier frequency | Double | Hz | sine wave: 999 to 250075. square wave: 100 to 1000. |
| ulc/cmg | M-0879 | Status of CMG amplitude verification. The status will always be operational if verify mode is disabled. If verify mode is enabled, the status is critical if the verify amplitude is outside of the range, and operational otherwise. | Integer | N/A | 1 = operational, 5 = critical |
| ulc/cmg | M-0880 | Status qualifier of CMG amplitude verification. | Integer | N/A | 0 = none |
| ulc/cmg | M-0881 | Command subcarrier phase angle to be applied to the command subcarrier in radians. Note that the modulation index for X-band carriers must be limited to 1.40 radians (80.21 degrees) if verification feedback from the Exciter to the CMG is used. | Double | radians | For UPL-6167-OPBV2.0.1: 0.35 to 1.48. For UPL-6167-OPCV3.2.0: 0.30 to 1.57. |
| ulc/cmg | M-1039 | Radiation bit format. | String | N/A | "NRZ_L", "NRZ_M", "NRZ_S", "BIP_L", "BIP_M", "BIP_S" |
| ulc/cmg | M-1040 | This is the currently configured CMG subcarrier waveform. | String | N/A | "Sine", "Square" |
| ulc/cmg | M-1041 | The requested radiation data rate in bits per second. | Double | bps |  |
| ulc/cmg | M-1042 | Intercommand Modulation type. What to radiate when there are no command bits to radiate. | String | N/A | "Idle", "Subcarrier Only", "No Output" |
| ulc/cmg | M-1043 | Idle sequence bit pattern. Bit pattern to radiate when there are no command bits to radiate and intercommand modulation type (M-1042) is "Idle". | Integer | N/A | 0 to 255 |
| ulc/cmg | M-1044 | Text explanation of overall CMG Status (channels M-0872 and M-0873). This item is a human-readable text string that says why overall CMG Status has the value it has. If the situation is nominal, its value is the empty string. | String | N/A | up to 242 characters long |
| ulc/etc | M-1045 | This is the Status code for the Exciter/Transmitter function Status. This is the status of the ETC hardware and software. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical, 6 = Emergency |
| ulc/etc | M-1046 | This is the Status qualifier for the Exciter/Transmitter function Status. | Integer | N/A | 0 = None |
| ulc/etc | M-1047 | Text explanation of the Exciter/Transmitter function Status (channels M-1045 and M-1046). This item is a human-readable text string that says why Exciter/Transmitter function Status has the value it has. If the situation is nominal, its value is the empty string. | String | N/A | up to 242 characters long |
| ulc/etc | M-1049 | Current Carrier Ramping Frequency. This is the current ramp frequency being generated by the Exciter. | Double | Hz | 0 to 35000000000 |
| ulc/etc | M-1050 | Command Port on OS2 set to receive command data. This is the hardware port between the exciter and the CMG. | String | N/A | "OFF", "MOD1", "MOD2", "MOD3", "UNKN" |
| ulc/etc | M-1051 | This is the current status from the ETC of the exciter command modulation connection telling if the command signal can be modulated onto the exciter carrier. | Integer | N/A | 0 = Off, 1 = On |
| ulc/etc | M-1052 | Time at which the modulation Status (M-1051) value last changed. | String | N/A | "hh:mm:ss" |
| ulc/etc | M-1053 | Ranging port on OS2 set to receive ranging data. | String | N/A | "OFF", "MOD1", "MOD2", "UNKN" |
| ulc/etc | M-1054 | This is the current Status of the exciter range modulation connection telling if the ranging signal can be modulated onto the exciter carrier. | Integer | N/A | 0 = Off, 1 = On |
| ulc/etc | M-1055 | Time at which the Exciter Range Modulation Status (channel M-1054) last changed. | String | N/A | "hh:mm:ss" |
| ulc/etc | M-1056 | Current tuning frequency rate. This is the current tuning frequency rate. | Double | Hz/sec | 0.0 to 80000.0 |
| ulc/etc | M-1057 | Current tuning frequency sweep range. This is the current tuning frequency sweep range. | Double | Hz | 0 to 6000000 |
| ulc/etc | M-1058 | Dwell 0 value. This is the first Dwell time setting. | Integer | seconds | 0 to 999 |
| ulc/etc | M-1059 | Dwell 1 value. This is the second Dwell time setting. | Integer | seconds | 0 to 999 |
| ulc/etc | M-1060 | Dwell 2 value. This is the third Dwell time setting. | Integer | seconds | 0 to 999 |
| ulc/etc | M-1061 | This is the day of year the first ramp starts. The effect on downlink will occur one Round-Trip Light Time (RTLT) later. | Integer | day of year | 0 to 366 |
| ulc/etc | M-1062 | This is the seconds of day (channel M-1061 gives the day of year) the first ramp starts. The effect on downlink will occur one Round-Trip Light Time (RTLT) later. | Integer | seconds | 0 to 86401 |
| ulc/etc | M-1063 | The Day of Year that the last ramp ends. The effect on downlink will occur one Round Trip Light Time (RTLT) later. | Integer | day of year | 0 to 366 |
| ulc/etc | M-1064 | This is the seconds of day (channel M-1063 give the day of year) the last ramp ends. The effect on downlink will occur one Round Trip Light Time (RTLT) later. | Integer | seconds | 0 to 86401 |
| ulc/etc | M-1065 | Identifier of the current transmitter. Contains the name of the currently scheduled transmitter. | String | N/A | "X4K", "S400K", "K800W", "S20K", "X20K", "TXR20K", "N/A" |
| ulc/etc | M-1066 | Indicates whether the transmitter amplifier is on or not. ON means the beam is on and the transmitter has the capability to amplify the drive signal. OFF means there is no high voltage power to the amplifier. In this state, the beam can be turned on by the network operator. SAFE means the beam is off and the transmitter is locked into the safe mode. In this state the beam cannot be turned on by the network operator. This state occurs when technicians/maintenance personnel are working on or near the transmitter hardware. | String | N/A | "OFF", "ON", "SAFE" |
| ulc/etc | M-1067 | Scheduled transmitter power output to antenna horn. Contains the measured power output to the antenna horn of the scheduled transmitter. | Float | kW | 0 to 500 |
| ulc/etc | M-1068 | Current State of the Scheduled Transmitter. Current State of the Scheduled Transmitter. | String | N/A | "OFF", "WARM", "CALIBRATE", "STANDBY", "TUNE", "OPERATE" |
| ulc/etc | M-1069 | Numerical indication of which transmitter is in use. | Integer | N/A | 0 = No transmitter is selected, 1 = Transmitter #1 is selected, 2 = Transmitter #2 is selected, 3 = Transmitter #3 is selected. |
| ulc/etc | M-1070 | This is the current Status of the transmitter. It indicates whether the carrier is going out of the horn. | Integer | N/A | 0 = Off, 1 = On |
| ulc/etc | M-1071 | Time at which the transmitter Status (M-1070) last changed. | String | N/A | "hh:mm:ss" |
| ulc/etc | M-1072 | This indicator shows the Status of the current ramps. | Integer | N/A | 0 = Off (no ramps scheduled),1 = On (ramps are scheduled) |
| ulc/etc | M-1073 | Day of year of the uplink carrier phase data time tag. | Integer | day of year | 0 to 366 |
| ulc/etc | M-1074 | Seconds of day (channel M-1073 is the day of year) of the uplink carrier phase data time tag. | Integer | seconds | 0 to 86401 |
| ulc/etc | M-1076 | This is the band that the Uplink carrier is currently set for. | Integer | N/A | 0 = Unknown, 1 = S-band, 2 = X-band, 3 = Ka-band |
| ulc/etc | M-1080 | Scheduled transmitter current output switch setting. This indicates whether the exciter is configured to send the signal out the antenna or to a water load. | String | N/A | "ANT", "WTR", "ERR" |
| ulc/etc | M-1081 | File name of the uplink predicts file being used. | String | N/A | any ASCII string up to 242 characters |
| ulc/etc | M-1082 | Current Exciter Template File. This contains the file name for the current Template File that is loaded on the UPL. | String | N/A | any ASCII string up to 242 characters |
| ulc/etc | M-1083 | Exciter Drive Setting. This is the current exciter drive setting. | String | N/A | "OFF", "ON", "UNKNOWN" |
| ulc/etc | M-1084 | This is the nominal spacecraft frequency setting generated by the Exciter hardware (at T0). | Double | Hz | 0 to 35000000000 |
| ulc/etc | M-1085 | This is the frequency offset added to the frequency predicts. | Float | Hz | -10000000.0 to 10000000.0 |
| ulc/urc | M-0976 | This is the Status code for the Uplink Ranging function Status. | Integer | N/A | 0 = Out of Service, 1 = Operational, 2 = Busy, 3 = Deviation, 4 = Marginal, 5 = Critical, 6 = Emergency |
| ulc/urc | M-0977 | This is the Status qualifier for the Uplink Ranging function Status. | Integer | N/A | 0 = None |
| ulc/urc | M-0978 | Text explanation of the Uplink Ranging function Status (channels M-0976 and M-0977). This item is a human-readable text string that says why Uplink Ranging function Status has the value it has. If the situation is nominal, its value is the empty string. | String | N/A | up to 242 characters long |
| ulc/urc | M-0979 | Day of year the ranging modulation was started; the first range transmit T0. | Integer | day of year | 0 to 366 |
| ulc/urc | M-0980 | Seconds of day (channel M-0979 contains the day of year) the ranging modulation was started; the first range transmit T0. | Integer | seconds | 0 to 86401 |
| ulc/urc | M-0981 | Current mode of the uplink ranging function. | String | N/A | "IDLE", "TRK", "CAL", "DIAG" |
| ulc/urc | M-0982 | Type of ranging modulation. SSIN = Sequential, Sinewave clock. SSQ = Sequential, Squarewave clock. PSIN = Pseudo-Noise, Sinewave clock. PSQ = Pseudo-Noise, Squarewave clock. PCON = Pseudo-Noise, No clock. This has a continuous spread spectrum modulation. | String | N/A | "SSIN", "SSQ", "PSIN", "PSQ", "PCON" |
| ulc/urc | M-0983 | Spacecraft ID from the support data ranging configuration file. There is one configuration file per spacecraft so the spacecraft ID indicates which configuration file is being used. | Integer | N/A | 0 to 255 |
| ulc/urc | M-0984 | The polarity of the uplink ranging modulation. PLUS = plus, same polarity as a +sinewave. MINUS = minus, same polarity as a -sinewave. | String | N/A | "PLUS", "MINUS" |
| ulc/urc | M-0985 | Suppression of carrier by ranging modulation, from ranging configuration table. | Float | dB | 0.0 to 9.0 |
| ulc/urc | M-0986 | T1 setting. T1 is the time duration for the clock or highest frequency tone. It is the first segment of sequential tone ranging. | Float | seconds | 0.0 to 9999.9 |
| ulc/urc | M-0987 | T2 setting. T2 is the time duration for each succession of lower and lower frequency tones transmitted. It is the second segment of sequential tone ranging. | Float | seconds | 0.0 to 9999.9 |
| ulc/urc | M-0988 | T3 setting. T3 is the time duration for the clock segment again and is the last portion of the sequential tone ranging. | Float | seconds | 0.0 to 9999.9 |
| ulc/urc | M-0989 | Clock component number. | Integer | N/A | 1 to 24 |
| ulc/urc | M-0990 | Last component number. | Integer | N/A | 1 to 24 |
| ulc/urc | M-0991 | Chop component number. | Integer | N/A | 1 to 24 |
| ulc/urc | M-0992 | Component number where the chopping starts being applied. | Integer | N/A | 1 to 25 |
| ulc/urc | M-0993 | Number of DRVID measurements. | Integer | N/A | 0 to 999 |
| ulc/urc | M-0994 | The reference signal is divided by this value to get the modulation clock. | Integer | N/A | 1 to 64 (normally 64) |
| ulc/urc | M-0995 | Length of the 1st PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-0996 | The symbol (chip) pattern of the 1st PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-0997 | The Boolean operation associated with the 1st PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-0998 | Length of the 2nd PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-0999 | The symbol (chip) pattern of the 2nd PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-1000 | The Boolean operation associated with the 2nd PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-1001 | Length of the 3rd PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-1002 | The symbol (chip) pattern of the 3rd PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-1003 | The Boolean operation associated with the 3rd PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-1004 | Length of the 4th PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-1005 | The symbol (chip) pattern of the 4th PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-1006 | The Boolean operation associated with the 4th PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-1007 | Length of the 5th PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-1008 | The symbol (chip) pattern of the 5th PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-1009 | The Boolean operation associated with the 5th PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-1010 | Length of the 6th PN sequence. | Integer | symbols (chips) | 0 to 32 |
| ulc/urc | M-1011 | The symbol (chip) pattern of the 6th PN sequence. | Integer | N/A | 0 to 0xffff |
| ulc/urc | M-1012 | The Boolean operation associated with the 6th PN sequence. | String | N/A | "AND", "OR", "XOR", "MVT" |
| ulc/urc | M-1013 | 1 = ON, the exciter ranging port is set to the URC, and the URC is producing range tones. 0 = OFF, no range tones being modulated on the carrier (the default). | Integer | N/A | 0, 1 |
| ulc/urc | M-1086 | Ranging state. Indicates whether uplink ranging is enabled or disabled. | String | N/A | "DISABLED","ENABLED" |
| ulc/urc | M-1087 | Ranging data output switch. If enabled, range data is being sent out. | String | N/A | "DISABLED", "ENABLED" |
| ulc/urc | M-1088 | Number of range blocks sent by the ranging assembly of uplink. | Integer | blocks | 0 to (231 - 1) |
| ulc/urc | M-1089 | Carrier data output switch. If enabled, carrier data is being sent out. | String | N/A | "DISABLED", "ENABLED" |
| ulc/urc | M-1090 | Number of carrier blocks sent by the ranging assembly of uplink. | Integer | blocks | 0 to (231 - 1) |
| uwv | M-3300 | Rainblower water pressure sensors for 34M HEF antennas. | String | N/A | "ON", "OFF", "N/A", "UNKN" |
| uwv | M-3301 | Rainblower air pressure for 34M HEF antennas. | String | N/A | "ON", "OFF", "N/A", "UNKN" |
| uwv | M-3302 | Rainblower water pressure sensors for 70M antennas. | String | N/A | "ON", "OFF", "N/A", "UNKN" |
| uwv | M-3303 | Rainblower air pressure for 70M antennas. | String | N/A | "ON", "OFF", "N/A", "UNKN" |
| uwv | M-3304 | The polarization of the transmission signal for S-band. | String | N/A | "RCP", "LCP", "N/A", "UNKN" |
| uwv | M-3305 | The polarization of the transmission signal for X-band. | String | N/A | "RCP", "LCP", "N/A", "UNKN" |
| uwv | M-3306 | The polarization of the transmission signal for Ka-band. | String | N/A | "RCP", "LCP", "N/A", "UNKN" |
| uwv | M-3307 | The polarization of the transmission signal for L-band. | String | N/A | "RCP", "LCP", "N/A", "UNKN" |

*The monitor data in these channels have not yet been implemented by the originating DSN subsystem.  The next delivery of the subsystem should include these items.  Until that time, though, these channels will not appear in the 0158-Monitor SFDUs.

Table A-3 lists the monitor channel IDs in channel ID order and shows the origin of the channel: a 34M/70M subsystem, the 26M equipment, or both.  This table is meant as an aid to the user who knows a channel ID and wants to look up its description in tables A-1 and/or A-2.

Table A-3 Channel ID Lookup

| Channel ID | 34M/70M Subsystem | 26M Subsystem | 34M/70M DSN Parameter Name |
| --- | --- | --- | --- |
| M-0037 | ce | Support Description | SpacecraftNumber |
| M-0042 | ce |  | DssList |
| M-0049 |  | Support Description |  |
| M-0064 |  | Support Description |  |
| M-0100 |  | Receiver Status |  |
| M-0101 | tgc |  | C1PDW |
| M-0104 | tgc | Telemetry Status and Performance | C1RATE |
| M-0109 | tgc |  | C1SCFR1 |
| M-0115 | tgc |  | C1BCSTS |
| M-0116 | tgc |  | C1BCSTS |
| M-0125 | tgc |  | C1MCSTS |
| M-0126 | tgc |  | C1MCSTS |
| M-0130 | tgc |  | C1MCSNR |
| M-0140 | tgc |  | C1ILBET |
| M-0141 | tgc |  | C1OLBET |
| M-0142 | tgc |  | C1ILTH |
| M-0143 | tgc |  | C1OLTH |
| M-0144 | tgc |  | C1FSSSTV |
| M-0147 | tgc | Telemetry Status and Performance | C1FSSSTS |
| M-0148 | tgc |  | C1OLCODE |
| M-0149 | tgc |  | C1ILFRMS |
| M-0152 | tgc |  | C1BITSAQ |
| M-0153 | tgc |  | C1FLYWHS |
| M-0169 | tgc |  | C1RSSNR |
| M-0182 |  | Telemetry Data Accountability, Channel 1 |  |
| M-0183 |  | Telemetry Data Accountability, Channel 1 |  |
| M-0190 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount1 |
| M-0191 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount2 |
| M-0192 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount3 |
| M-0193 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount4 |
| M-0194 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount5 |
| M-0195 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount6 |
| M-0196 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount7 |
| M-0197 | dcc1 (Telemetry) | Telemetry Data Accountability, Channel 1 | tlm.perf.fmt.vcCount8 |
| M-0200 |  | Receiver Status |  |
| M-0201 | tgc |  | C2PDW |
| M-0204 | tgc | Telemetry Status and Performance | C2RATE |
| M-0209 | tgc |  | C2SCFR1 |
| M-0215 | tgc |  | C2BCSTS |
| M-0216 | tgc |  | C2BCSTS |
| M-0225 | tgc |  | C2MCSTS |
| M-0226 | tgc |  | C2MCSTS |
| M-0230 | tgc |  | C2MCSNR |
| M-0240 | tgc |  | C2ILBET |
| M-0241 | tgc |  | C2OLBET |
| M-0242 | tgc |  | C2ILTH |
| M-0243 | tgc |  | C2OLTH |
| M-0244 | tgc |  | C2FSSSTV |
| M-0247 | tgc | Telemetry Status and Performance | C2FSSSTS |
| M-0248 | tgc |  | C2OLCODE |
| M-0249 | tgc |  | C2ILFRMS |
| M-0252 | tgc |  | C2BITSAQ |
| M-0253 | tgc |  | C2FLYWHS |
| M-0269 | tgc |  | C2RSSNR |
| M-0282 |  | Telemetry Data Accountability, Channel 2 |  |
| M-0283 |  | Telemetry Data Accountability, Channel 2 |  |
| M-0290 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount1 |
| M-0291 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount2 |
| M-0292 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount3 |
| M-0293 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount4 |
| M-0294 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount5 |
| M-0295 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount6 |
| M-0296 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount7 |
| M-0297 | dcc2 (Telemetry) | Telemetry Data Accountability, Channel 2 | tlm.perf.fmt.vcCount8 |
| M-0301 | apa, apc |  | apa: ELAPD70, ELAPDHE; apc: ELAPD |
| M-0302 | apa, apc |  | apa: ELAPD70, ELAPDHE; apc: ELAPD |
| M-0303 | apa, apc |  | apa: ELAPD70, ELAPDHE; apc: ELAPD |
| M-0304 | apc |  | ELANG |
| M-0305 | apc |  | AZANG |
| M-0306 | apc |  | AZRES |
| M-0307 | apc |  | ELRES |
| M-0308 | apa, apc |  | apa: CNSCN70, CNSCNHE; apc: CNSCNDRV |
| M-0309 | apa, apc |  | apa: LPSTAT70, LPSTATHE; apc: LPSTAT |
| M-0311 | mda | Tracking Status | WAY1 |
| M-0314 | mda | Tracking Status | WAY2 |
| M-0324 | apa |  | ELANG70, ELANGHE |
| M-0325 | apa |  | AZANG70, AZANGHE |
| M-0326 | apa |  | AZRES70, AZRESHE |
| M-0327 | apa |  | ELRES70, ELRESHE |
| M-0346 | rcc |  | BAND1 |
| M-0349 |  | Receiver Status |  |
| M-0373 | rcc |  | CLSTAT2 |
| M-0374 | rcc |  | CLSTAT2 |
| M-0376 | rcc |  | BAND2 |
| M-0379 |  | Receiver Status |  |
| M-0500 | cpa |  | DSSID |
| M-0504 | etc |  | TXR1 |
| M-0505 | etc |  | TBEAM1 |
| M-0509 | etc |  | TXPWR1 |
| M-0514 | tx_hp |  | HITXRID |
| M-0515 | tx_hp |  | HIBEAM |
| M-0519 | tx_hp |  | HIPWR |
| M-0524 |  | Transmitter Status |  |
| M-0526 |  | Transmitter Status |  |
| M-0527 |  | Transmitter Status |  |
| M-0534 | cpa |  | CMAMDE |
| M-0535 | etc | Command Status and Performance | CMDMOD |
| M-0536 | cpa | Command Status and Performance | CARSUP |
| M-0537 | cpa |  | DRATE |
| M-0541 |  | Command Data Accountability |  |
| M-0542 |  | Command Data Accountability |  |
| M-0543 | etc |  | TXROP |
| M-0544 | etc |  | TXR2 |
| M-0545 | etc |  | TBEAM2 |
| M-0546 | etc |  | TXPWR2 |
| M-0547 | etc |  | TXR3 |
| M-0548 | etc |  | TBEAM3 |
| M-0549 | etc |  | TXPWR3 |
| M-0611 | mda |  | DOPNS1 |
| M-0614 | mda |  | DOPRS1 |
| M-0615 | mda |  | RNGRS1 |
| M-0616 | etc |  | RNGMOD |
| M-0617 | mda |  | RNGSUP1 |
| M-0618 | mda |  | PRNO1 |
| M-0619 | mda |  | LAST1 |
| M-0620 | mda |  | RNGT1_1 |
| M-0621 | mda |  | RNGT2_1 |
| M-0622 | mda |  | RNGT3_1 |
| M-0623 | mda |  | DOPFRQ1 |
| M-0640 | etc |  | TR |
| M-0641 | etc |  | SR |
| M-0642 | etc |  | DWL0 |
| M-0643 | etc |  | DWL1 |
| M-0644 | etc |  | DWL2 |
| M-0645 | etc |  | T0 |
| M-0646 | etc |  | T0 |
| M-0647 | etc |  | T0 |
| M-0654 |  | Antenna Pointing |  |
| M-0655 |  | Antenna Pointing |  |
| M-0656 |  | Antenna Pointing |  |
| M-0714 | rcc |  | CLSTAT1 |
| M-0716 | rcc |  | CLSTAT1 |
| M-0717 | rcc |  | CARRLST1 |
| M-0718 | rcc |  | SUB1LST1 |
| M-0719 | rcc |  | SYM1LST1 |
| M-0720 | rcc |  | SUB2LST1 |
| M-0721 | rcc |  | SYM2LST1 |
| M-0723 | rcc |  | PCNO1 |
| M-0724 | rcc |  | SNTPD1 |
| M-0725 | rcc |  | SNT1 |
| M-0727 | rcc |  | PC1 |
| M-0729 | rcc |  | PDNO11 |
| M-0730 | rcc |  | SS1SNPD1 |
| M-0731 | rcc |  | SSNR11 |
| M-0733 | rcc |  | PDNO21 |
| M-0734 | rcc |  | SS2SNPD1 |
| M-0735 | rcc |  | SSNR21 |
| M-0737 | rcc |  | SPE1 |
| M-0739 | rcc |  | CARRLPB1 |
| M-0741 | rcc |  | CARRBW1 |
| M-0742 | rcc |  | SUB1BW1 |
| M-0743 | rcc |  | SYM1BW1 |
| M-0744 | rcc |  | SUB2BW1 |
| M-0745 | rcc |  | SYM2BW1 |
| M-0746 | rcc |  | FC1 |
| M-0748 | rcc |  | SYM1RAT1 |
| M-0749 | rcc |  | SYM2RAT1 |
| M-0767 | rcc |  | CARRLST2 |
| M-0768 | rcc |  | SUB1LST2 |
| M-0769 | rcc |  | SYM1LST2 |
| M-0770 | rcc |  | SUB2LST2 |
| M-0771 | rcc |  | SYM2LST2 |
| M-0773 | rcc |  | PCNO2 |
| M-0774 | rcc |  | SNTPD2 |
| M-0775 | rcc |  | SNT2 |
| M-0777 | rcc |  | PC2 |
| M-0779 | rcc |  | PDNO12 |
| M-0780 | rcc |  | SS1SNPD2 |
| M-0781 | rcc |  | SSNR12 |
| M-0783 | rcc |  | PDNO22 |
| M-0784 | rcc |  | SS2SNPD2 |
| M-0785 | rcc |  | SSNR22 |
| M-0787 | rcc |  | SPE2 |
| M-0789 | rcc |  | CARRLPB2 |
| M-0791 | rcc |  | CARRBW2 |
| M-0792 | rcc |  | SUB1BW2 |
| M-0793 | rcc |  | SYM1BW2 |
| M-0794 | rcc |  | SUB2BW2 |
| M-0795 | rcc |  | SYM2BW2 |
| M-0796 | rcc |  | FC2 |
| M-0798 | rcc |  | SYM1RAT2 |
| M-0799 | rcc |  | SYM2RAT2 |
| M-0800 | rcc |  | R1PDXMOD |
| M-0801 | rcc |  | R2PDXMOD |
| M-0802 | rcc |  | R1TBLID |
| M-0803 | rcc |  | R2TBLID |
| M-0804 | apc |  | AZESTMP1 |
| M-0805 | apc |  | ELESTMP1 |
| M-0806 | apc |  | AZESTMP2 |
| M-0807 | apc |  | ELESTMP2 |
| M-0808 | apc |  | AZESTMP3 |
| M-0809 | apc |  | ELESTMP3 |
| M-0810 | apc |  | AZESTMP4 |
| M-0811 | apc |  | ELESTMP4 |
| M-0812 | apc |  | AZESTMP5 |
| M-0813 | apc |  | ELESTMP5 |
| M-0814 | apc |  | AZCORMP |
| M-0815 | apc |  | ELCORMP |
| M-0816 | apc |  | AALPHA |
| M-0817 | apc |  | ABETA |
| M-0818 | apc |  | CNSCNTYP |
| M-0819 | apc |  | RCVSNR |
| M-0820 | apc |  | ABCX |
| M-0821 | apc |  | ABCY |
| M-0822 | apc |  | ABER |
| M-0850 | ulc/ccp |  | CltuAbortSource |
| M-0851 | ulc/ccp |  | CmdMod |
| M-0852 | ulc/ccp |  | ControlMode |
| M-0853 | ulc/ccp |  | ExcMonitoring, FrontEndMonitoring |
| M-0854 | ulc/ccp |  | ExcMonitoring, FrontEndMonitoring |
| M-0855 | ulc/ccp |  | TxrMonitoring |
| M-0856 | ulc/ccp |  | TxrMonitoring |
| M-0857 | ulc/ccp |  | TxrOutput |
| M-0858 | ulc/ccp |  | TotalCltusRadiated |
| M-0859 | ulc/ccp |  | TpBlocksCmd |
| M-0860 | ulc/ccp |  | TpBlocksReceived |
| M-0861 | ulc/ccp |  | TpBlocksRejected |
| M-0862 | ulc/ccp |  | TpCltusAborted |
| M-0863 | ulc/ccp |  | TpCltusFlushed |
| M-0864 | ulc/ccp |  | TpOverflowWarning |
| M-0865 | ulc/ccp |  | TpPctQueueFull |
| M-0866 | ulc/cmg |  | CarrierSup |
| M-0867 | ulc/cmg |  | ActualDataRate |
| M-0868 | ulc/cmg |  | CmgCltuLoadError |
| M-0869 | ulc/cmg |  | Modulation |
| M-0870 | ulc/cmg |  | Mode |
| M-0871 | ulc/cmg |  | ReferenceSignalFailure |
| M-0872 | ulc/cmg |  | Status |
| M-0873 | ulc/cmg |  | Status |
| M-0874 | ulc/cmg |  | TctError |
| M-0875 | ulc/cmg |  | VerificationSignalError |
| M-0876 | ulc/cmg |  | WatchDogError |
| M-0877 | ulc/cmg |  | ModIndexDegrees |
| M-0878 | ulc/cmg |  | SubcarFrequency |
| M-0879 | ulc/cmg |  | VerifyTolStatus |
| M-0880 | ulc/cmg |  | VerifyTolStatus |
| M-0881 | ulc/cmg |  | ModIndexRadians |
| M-0882 | ulc/ccp |  | CltuInitiatorId |
| M-0883 | ulc/ccp |  | CltuResponderId |
| M-0884 | ulc/ccp |  | ServiceMode |
| M-0885 | etc |  | UPFRQ |
| M-0886 | ulc/ccp |  | CltuCltusAborted |
| M-0887 | ulc/ccp |  | CltuCltusAccepted |
| M-0888 | ulc/ccp |  | CltuCltusExpired |
| M-0889 | ulc/ccp |  | CltuCltusFlushed |
| M-0890 | ulc/ccp |  | CltuCltusProcessed |
| M-0891 | ulc/ccp |  | CltuCltusReceived |
| M-0892 | ulc/ccp |  | CltuCltusRejected |
| M-0893 | ulc/ccp |  | CltuNumInQueue |
| M-0894 | ulc/ccp |  | TpBlocksTest |
| M-0895 | ulc/ccp |  | TpBlocksValid |
| M-0961 |  | Receiver Status |  |
| M-0962 |  | Receiver Status |  |
| M-0969 | ulc |  | VersionID |
| M-0970 | ulc |  | Status |
| M-0971 | ulc |  | Status |
| M-0972 | ulc |  | StatusText |
| M-0973 | ulc |  | uplinkAssemblyNumber |
| M-0974 | ulc |  | ConfiguredSpacecraft |
| M-0975 | ulc |  | ConfiguredPass |
| M-0976 | ulc/urc |  | Status |
| M-0977 | ulc/urc |  | Status |
| M-0978 | ulc/urc |  | StatusText |
| M-0979 | ulc/urc |  | upRngStartDay |
| M-0980 | ulc/urc |  | upRngStartSec |
| M-0981 | ulc/urc |  | rng.perf.currentMode |
| M-0982 | ulc/urc |  | rng.config.patternType |
| M-0983 | ulc/urc |  | rng.config.scId |
| M-0984 | ulc/urc |  | rng.config.modPolarity |
| M-0985 | ulc/urc |  | rng.config.csv |
| M-0986 | ulc/urc |  | rng.config.t1Xmit |
| M-0987 | ulc/urc |  | rng.config.t2Xmit |
| M-0988 | ulc/urc |  | rng.config.t3Xmit |
| M-0989 | ulc/urc |  | rng.config.clock |
| M-0990 | ulc/urc |  | rng.config.last |
| M-0991 | ulc/urc |  | rng.config.chop |
| M-0992 | ulc/urc |  | rng.config.chopStrt |
| M-0993 | ulc/urc |  | rng.config.drvn |
| M-0994 | ulc/urc |  | rng.config.clockDiv |
| M-0995 | ulc/urc |  | rng.config.comp1Len |
| M-0996 | ulc/urc |  | rng.config.comp1Def |
| M-0997 | ulc/urc |  | rng.config.comp1Bool |
| M-0998 | ulc/urc |  | rng.config.comp2Len |
| M-0999 | ulc/urc |  | rng.config.comp2Def |
| M-1000 | ulc/urc |  | rng.config.comp2Bool |
| M-1001 | ulc/urc |  | rng.config.comp3Len |
| M-1002 | ulc/urc |  | rng.config.comp3Def |
| M-1003 | ulc/urc |  | rng.config.comp3Bool |
| M-1004 | ulc/urc |  | rng.config.comp4Len |
| M-1005 | ulc/urc |  | rng.config.comp4Def |
| M-1006 | ulc/urc |  | rng.config.comp4Bool |
| M-1007 | ulc/urc |  | rng.config.comp5Len |
| M-1008 | ulc/urc |  | rng.config.comp5Def |
| M-1009 | ulc/urc |  | rng.config.comp5Bool |
| M-1010 | ulc/urc |  | rng.config.comp6Len |
| M-1011 | ulc/urc |  | rng.config.comp6Def |
| M-1012 | ulc/urc |  | rng.config.comp6Bool |
| M-1013 | ulc/urc |  | rng.config.rngModTones |
| M-1014 | ulc/ccp |  | CmdModChangeTime |
| M-1015 | ulc/ccp |  | CmdModChangeTime |
| M-1016 | ulc/ccp |  | CmdModChangeTime |
| M-1017 | ulc/ccp |  | RadiationStatus |
| M-1018 | ulc/ccp |  | ServiceTableFile |
| M-1019 | ulc/ccp |  | CltuServiceInstanceId |
| M-1020 | ulc/ccp |  | CltuCheckCredential |
| M-1021 | ulc/ccp |  | ConfiguredSet |
| M-1022 | ulc/ccp |  | Status |
| M-1023 | ulc/ccp |  | Status |
| M-1024 | ulc/ccp |  | StatusText |
| M-1025 | ulc/ccp |  | ConfigStatus |
| M-1026 | ulc/ccp |  | ConfigStatus |
| M-1027 | ulc/ccp |  | ConfigStatusText |
| M-1028 | ulc/ccp |  | ServiceStatus |
| M-1029 | ulc/ccp |  | ServiceStatus |
| M-1030 | ulc/ccp |  | ServiceStatusText |
| M-1031 | ulc/ccp |  | CltuServiceState |
| M-1032 | ulc/ccp |  | CltuProductionState |
| M-1033 | ulc/ccp |  | CltuSendAcqSeq |
| M-1034 | ulc/ccp |  | MaxCltuSize |
| M-1035 | ulc/ccp |  | MinCltuSize |
| M-1036 | ulc/ccp |  | CltuLastCltuReceived |
| M-1037 | ulc/ccp |  | CltuLastCltuAccepted |
| M-1038 | ulc/ccp |  | CltuLastCltuRadiated |
| M-1039 | ulc/cmg |  | BitFormat |
| M-1040 | ulc/cmg |  | SubcarWaveForm |
| M-1041 | ulc/cmg |  | RequestedDataRate |
| M-1042 | ulc/cmg |  | InterModulationType |
| M-1043 | ulc/cmg |  | IdleSequence |
| M-1044 | ulc/cmg |  | StatusText |
| M-1045 | ulc/etc |  | Status |
| M-1046 | ulc/etc |  | Status |
| M-1047 | ulc/etc |  | StatusText |
| M-1049 | ulc/etc |  | CarrierFrequency |
| M-1050 | ulc/etc |  | CommandModulation |
| M-1051 | ulc/etc |  | CmdModStatus |
| M-1052 | ulc/etc |  | CmdModStatusTime |
| M-1053 | ulc/etc |  | RangeModulation |
| M-1054 | ulc/etc |  | RngModStatus |
| M-1055 | ulc/etc |  | RngModStatusTime |
| M-1056 | ulc/etc |  | TuningRate |
| M-1057 | ulc/etc |  | SweepRange |
| M-1058 | ulc/etc |  | Dwell0 |
| M-1059 | ulc/etc |  | Dwell1 |
| M-1060 | ulc/etc |  | Dwell2 |
| M-1061 | ulc/etc |  | tuneStartDay |
| M-1062 | ulc/etc |  | tuneStartSec |
| M-1063 | ulc/etc |  | tuneEndDay |
| M-1064 | ulc/etc |  | tuneEndSec |
| M-1065 | ulc/etc |  | TransmitterName |
| M-1066 | ulc/etc |  | TransmitterBeamSetting |
| M-1067 | ulc/etc |  | TransmitterPowerOutput |
| M-1068 | ulc/etc |  | TransmitterState |
| M-1069 | ulc/etc |  | txrOper |
| M-1070 | ulc/etc |  | TxrOutputStatus |
| M-1071 | ulc/etc |  | TxrOutputStatusTime |
| M-1072 | ulc/etc |  | upAcqIndicator |
| M-1073 | ulc/etc |  | upCarDay |
| M-1074 | ulc/etc |  | upCarSec |
| M-1076 | ulc/etc |  | uplinkBand |
| M-1077 | ulc/ccp |  | TxrOutputStatus |
| M-1078 | ulc/ccp |  | TxrOutputStatus |
| M-1079 | ulc/ccp |  | TxrOutputText |
| M-1080 | ulc/etc |  | TransmitterSwitch |
| M-1081 | ulc/etc |  | upPredictsID |
| M-1082 | ulc/etc |  | ExciterTemplateId |
| M-1083 | ulc/etc |  | ExciterDrive |
| M-1084 | ulc/etc |  | SpacecraftFrequency |
| M-1085 | ulc/etc |  | upPredictsFrequencyOffset |
| M-1086 | ulc/urc |  | rng.config.state |
| M-1087 | ulc/urc |  | rng.config.rangeOutput |
| M-1088 | ulc/urc |  | rng.perf.blocksSent |
| M-1089 | ulc/urc |  | rng.config.trkDataOutput |
| M-1090 | ulc/urc |  | rng.perf.carBlocksSent |
| M-1200 | dcc1 (Status) |  | smon.status.sum.sys.status |
| M-1201 | dcc1 (Status) |  | smon.status.sum.sys.status |
| M-1202 | dcc1 (Status) |  | smon.status.sum.rec.general |
| M-1203 | dcc1 (Status) |  | smon.status.sum.rec.general |
| M-1204 | dcc1 (Status) |  | smon.status.sum.rrp.general |
| M-1205 | dcc1 (Status) |  | smon.status.sum.rrp.general |
| M-1206 | dcc1 (Status) |  | smon.status.sum.dcc.general |
| M-1207 | dcc1 (Status) |  | smon.status.sum.dcc.general |
| M-1208* | dcc1 (Status) |  | smon.status.sum.rrp.mp.general |
| M-1209* | dcc1 (Status) |  | smon.status.sum.rrp.mp.general |
| M-1210 | dcc1 (Status) |  | smon.status.sum.rng.general |
| M-1211 | dcc1 (Status) |  | smon.status.sum.rng.general |
| M-1212 | dcc1 (Status) |  | smon.status.sum.tlp.general |
| M-1213 | dcc1 (Status) |  | smon.status.sum.tlp.general |
| M-1214* | dcc1 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-1215* | dcc1 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-1216* | dcc1 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-1217* | dcc1 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-1240 | dcc1 (Configuration) |  | DccChannel |
| M-1241 | dcc1 (Configuration) |  | ChannelDss |
| M-1242 | dcc1 (Configuration) |  | rcv.config.scDep.scId |
| M-1243 | dcc1 (Configuration) |  | ChannelPass |
| M-1244 | dcc1 (Configuration) |  | dc.perf.missionId |
| M-1245 | dcc1 (Configuration) |  | rcv.config.scDep.downBand |
| M-1246 | dcc1 (Configuration) |  | rcv.config.scDep.upBand |
| M-1247 | dcc1 (Configuration) |  | rcv.config.tableId |
| M-1248 | dcc1 (Configuration) |  | rcv.config.scDep.pdxMode |
| M-1250 | dcc1 (Configuration) |  | rcv.config.scDep.uplinkDss |
| M-1251 | dcc1 (Configuration) |  | rcv.config.car.freqPred |
| M-1252 | dcc1 (Configuration) |  | rcv.config.sub.freqPred |
| M-1253 | dcc1 (Configuration) |  | rcv.config.sym.ratePredicts |
| M-1254 | dcc1 (Configuration) |  | rcv.config.nar.sntMode |
| M-1255 | dcc1 (Configuration) |  | rcv.config.rxDep.mpSwitch |
| M-1256 | dcc1 (Configuration) |  | dc.config.trkSetName |
| M-1257 | dcc1 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1258 | dcc1 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1259 | dcc1 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1260 | dcc1 (Configuration) |  | rcv.config.rxDep.useTpdx |
| M-1261 | dcc1 (Configuration) |  | dc.config.tlmSetName |
| M-1262 | dcc1 (Configuration) |  | rng.config.state |
| M-1263 | dcc1 (Configuration) |  | rng.config.trkDataOutput |
| M-1264 | dcc1 (Configuration) |  | rng.config.rangeOutput |
| M-1265 | dcc1 (Configuration) |  | tlm.config.gen.output |
| M-1266 | dcc1 (Configuration) |  | dc.perf.signal.lnaId |
| M-1267 | dcc1 (Configuration) |  | dc.perf.signal.polarization |
| M-1268 | dcc1 (Configuration) |  | dc.perf.signal.path |
| M-1290 | dcc1 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1291 | dcc1 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1292 | dcc1 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1293 | dcc1 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1294 | dcc1 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1295 | dcc1 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1296 | dcc1 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1297 | dcc1 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1298 | dcc1 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1299 | dcc1 (Carrier Loop) |  | rcv.perf.car.loopBw |
| M-1300 | dcc1 (Carrier Loop) |  | rcv.config.car.loopType |
| M-1301 | dcc1 (Carrier Loop) |  | dc.perf.autom.carLockStatus |
| M-1302 | dcc1 (Carrier Loop) |  | rcv.perf.car.loopStatus |
| M-1303 | dcc1 (Carrier Loop) |  | dc.perf.autom.dataLockStatus |
| M-1304 | dcc1 (Carrier Loop) |  | rcv.perf.car.freqEst |
| M-1305 | dcc1 (Carrier Loop) |  | dc.perf.resid.carFreq |
| M-1306 | dcc1 (Carrier Loop) |  | dc.perf.resid.dopp |
| M-1307 | dcc1 (Carrier Loop) |  | rcv.perf.car.rateEst |
| M-1308 | dcc1 (Carrier Loop) |  | rcv.perf.car.accelEst |
| M-1309 | dcc1 (Carrier Loop) |  | rng.perf.dopNoise |
| M-1310 | dcc1 (Carrier Loop) |  | rcv.perf.car.carPwrEst |
| M-1311 | dcc1 (Carrier Loop) |  | dc.perf.resid.pc |
| M-1312 | dcc1 (Carrier Loop) |  | rcv.perf.sub.pdEst |
| M-1313 | dcc1 (Carrier Loop) |  | dc.perf.resid.pd |
| M-1314 | dcc1 (Carrier Loop) |  | rcv.perf.car.spe |
| M-1315 | dcc1 (Carrier Loop) |  | rcv.perf.car.pcnoEst |
| M-1316 | dcc1 (Carrier Loop) |  | dc.perf.resid.pcno |
| M-1317 | dcc1 (Carrier Loop) |  | rcv.perf.car.trackingMode |
| M-1318 | dcc1 (Carrier Loop) |  | rcv.perf.car.snt |
| M-1319 | dcc1 (Carrier Loop) |  | dc.perf.resid.snt |
| M-1320 | dcc1 (Carrier Loop) |  | rcv.perf.car.freqPredicts |
| M-1321 | dcc1 (Carrier Loop) |  | rng.perf.cycleSlips |
| M-1322 | dcc1 (Carrier Loop) |  | rcv.config.carAcq.useFft |
| M-1323 | dcc1 (Carrier Loop) |  | rcv.perf.state.carFft |
| M-1340 | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1341 | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1342 | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1343 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.freqEst |
| M-1344 | dcc1 (Subcarrier Loop) |  | dc.perf.resid.subFreq |
| M-1345 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.loopBandwidth |
| M-1346 | dcc1 (Subcarrier Loop) |  | rcv.config.sub.loopType |
| M-1347 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.currentWindowWidth |
| M-1348 | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subLockStatus |
| M-1349* | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1350* | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1351* | dcc1 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1352 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.loopStatus |
| M-1353 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.pdnoEst |
| M-1354 | dcc1 (Subcarrier Loop) |  | dc.perf.resid.pdno |
| M-1355 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.spe |
| M-1356 | dcc1 (Subcarrier Loop) |  | rcv.config.sub.waveForm |
| M-1357 | dcc1 (Subcarrier Loop) |  | rcv.perf.sub.freqPredicts |
| M-1358 | dcc1 (Subcarrier Loop) |  | rcv.config.subAcq.useFft |
| M-1359 | dcc1 (Subcarrier Loop) |  | rcv.perf.state.subFft |
| M-1380 | dcc1 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1381 | dcc1 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1382 | dcc1 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1383 | dcc1 (Symbol Loop) |  | rcv.perf.sym.rateEst |
| M-1384 | dcc1 (Symbol Loop) |  | dc.perf.resid.symRate |
| M-1385 | dcc1 (Symbol Loop) |  | rcv.perf.sym.loopBandwidth |
| M-1386 | dcc1 (Symbol Loop) |  | rcv.config.sym.loopType |
| M-1387 | dcc1 (Symbol Loop) |  | rcv.perf.sym.currentWindowWidth |
| M-1388 | dcc1 (Symbol Loop) |  | dc.perf.autom.symLockStatus |
| M-1389* | dcc1 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1390* | dcc1 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1391* | dcc1 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1392 | dcc1 (Symbol Loop) |  | rcv.perf.sym.loopStatus |
| M-1393 | dcc1 (Symbol Loop) |  | rcv.perf.sym.snrEst |
| M-1394 | dcc1 (Symbol Loop) |  | dc.perf.resid.symSnr |
| M-1395 | dcc1 (Symbol Loop) |  | rcv.perf.sym.spe |
| M-1396 | dcc1 (Symbol Loop) |  | rcv.config.sym.format |
| M-1397 | dcc1 (Symbol Loop) |  | rcv.config.sym.smoothing |
| M-1398 | dcc1 (Symbol Loop) |  | rcv.perf.sym.ratePred |
| M-1399 | dcc1 (Symbol Loop) |  | rcv.config.symAcq.useFft |
| M-1400 | dcc1 (Symbol Loop) |  | rcv.perf.state.symFft |
| M-1415 | dcc1 (Telemetry) |  | tlm.config.fan.frameMode |
| M-1416 | dcc1 (Telemetry) |  | tlm.perf.gen.rrpArrayStatus |
| M-1417 | dcc1 (Telemetry) |  | tlm.config.fs.mode |
| M-1418 | dcc1 (Telemetry) |  | tlm.config.fs.pattern |
| M-1419 | dcc1 (Telemetry) |  | tlm.config.fs.patternLength |
| M-1420 | dcc1 (Telemetry) |  | tlm.perf.fs.predictedBitRate |
| M-1421 | dcc1 (Telemetry) |  | tlm.perf.fs.dataRate |
| M-1422 | dcc1 (Telemetry) |  | tlm.perf.gen.tlpLockStatus |
| M-1424 | dcc1 (Telemetry) |  | tlm.perf.gen.chanStatus |
| M-1425* | dcc1 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1426* | dcc1 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1427* | dcc1 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1428* | dcc1 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1429* | dcc1 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1430* | dcc1 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1431 | dcc1 (Telemetry) |  | tlm.perf.gen.decoderLockStatus |
| M-1433 | dcc1 (Telemetry) |  | tlm.config.gen.decoderType |
| M-1434 | dcc1 (Telemetry) |  | tlm.perf.gen.decoderSNR |
| M-1436 | dcc1 (Telemetry) |  | tlm.config.b2cd.ilThreshold |
| M-1437 | dcc1 (Telemetry) |  | tlm.config.b2cd.vecSet |
| M-1438 | dcc1 (Telemetry) |  | tlm.config.b3cd.ilThreshold |
| M-1439 | dcc1 (Telemetry) |  | tlm.config.b3cd.vecSet |
| M-1440 | dcc1 (Telemetry) |  | tlm.config.fs.frameLengthPrimary |
| M-1441 | dcc1 (Telemetry) |  | tlm.config.fs.ilBitErrorThreshold |
| M-1442 | dcc1 (Telemetry) |  | tlm.config.fs.oolBitErrorThreshold |
| M-1443 | dcc1 (Telemetry) |  | tlm.config.fs.ilThreshold |
| M-1444 | dcc1 (Telemetry) |  | tlm.config.fs.oolThreshold |
| M-1445* | dcc1 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1446* | dcc1 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1447* | dcc1 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1448* | dcc1 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1449* | dcc1 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1450* | dcc1 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1451 | dcc1 (Telemetry) |  | tlm.perf.fs.lockStatus |
| M-1452 | dcc1 (Telemetry) |  | tlm.perf.fs.bitToAcquire |
| M-1453 | dcc1 (Telemetry) |  | tlm.perf.gen.blocksOutput |
| M-1455 | dcc1 (Telemetry) |  | tlm.perf.fs.frameTotalCount |
| M-1456 | dcc1 (Telemetry) |  | tlm.perf.fs.frameGoodCount |
| M-1457 | dcc1 (Telemetry) |  | tlm.perf.fs.frameBadCount |
| M-1458 | dcc1 (Telemetry) |  | tlm.perf.crc.crcLockStatus |
| M-1459 | dcc1 (Telemetry) |  | tlm.config.fan.pnDecode |
| M-1460 | dcc1 (Telemetry) |  | tlm.perf.crc.crcTotalFrameCount |
| M-1461 | dcc1 (Telemetry) |  | tlm.perf.crc.crcGoodFrameCount |
| M-1462 | dcc1 (Telemetry) |  | tlm.perf.crc.crcBadFrameCount |
| M-1463 | dcc1 (Telemetry) |  | tlm.perf.rsd.lockStatus |
| M-1464 | dcc1 (Telemetry) |  | tlm.perf.rsd.totalCodeWordCount |
| M-1465 | dcc1 (Telemetry) |  | tlm.perf.rsd.goodCodeWordCount |
| M-1466 | dcc1 (Telemetry) |  | tlm.perf.rsd.badCodeWordCount |
| M-1467 | dcc1 (Telemetry) |  | tlm.perf.rsd.frameErrorRate |
| M-1468 | dcc1 (Telemetry) |  | tlm.perf.rsd.codeWordErrorRate |
| M-1469 | dcc1 (Telemetry) |  | tlm.perf.fs.totalInLockFrameCount |
| M-1470 | dcc1 (Telemetry) |  | tlm.perf.fs.flywheelFrameCount |
| M-1471 | dcc1 (Telemetry) |  | tlm.config.dft.formatTable |
| M-1474 | dcc1 (Telemetry) |  | tlm.config.b2cd.oolThreshold |
| M-1475 | dcc1 (Telemetry) |  | tlm.config.b3cd.oolThreshold |
| M-1476 | dcc1 (Telemetry) |  | tlm.config.gen.diffDecoder |
| M-1477 | dcc1 (Telemetry) |  | tlm.config.b2cd.altSymInv |
| M-1478 | dcc1 (Telemetry) |  | tlm.config.b3cd.symSignInversion |
| M-1479 | dcc1 (Telemetry) |  | tlm.perf.b2cd.metricNormalization |
| M-1480 | dcc1 (Telemetry) |  | tlm.perf.b2cd.estimatedSymbolErrorRate |
| M-1481 | dcc1 (Telemetry) |  | tlm.config.b2cd.nodeSyncChange |
| M-1482 | dcc1 (Telemetry) |  | tlm.perf.b2cd.nodeSyncChangeCount |
| M-1483 | dcc1 (Telemetry) |  | tlm.perf.b3cd.nodeSyncPosition |
| M-1484 | dcc1 (Telemetry) |  | tlm.config.fs.frameLengthSecondary |
| M-1485 | dcc1 (Telemetry) |  | tlm.perf.fs.correctFrameLength |
| M-1486 | dcc1 (Telemetry) |  | tlm.perf.fs.selfTestResult |
| M-1487 | dcc1 (Telemetry) |  | tlm.config.fs.bitSequence |
| M-1488 | dcc1 (Telemetry) |  | tlm.config.fs.bitSlipWindow |
| M-1489 | dcc1 (Telemetry) |  | tlm.perf.fs.inLockBer |
| M-1490 | dcc1 (Telemetry) |  | tlm.perf.fs.syncDirection |
| M-1491 | dcc1 (Telemetry) |  | tlm.perf.fs.currentPolarity |
| M-1492 | dcc1 (Telemetry) |  | tlm.perf.fs.bitSlip |
| M-1493 | dcc1 (Telemetry) |  | tlm.perf.fs.polarityChange |
| M-1494 | dcc1 (Telemetry) |  | tlm.perf.fs.acquisitionCount |
| M-1495 | dcc1 (Telemetry) |  | tlm.config.fan.interleave |
| M-1496 | dcc1 (Telemetry) |  | tlm.config.fan.algorithm |
| M-1497 | dcc1 (Telemetry) |  | tlm.perf.rsd.virtualFill |
| M-1498 | dcc1 (Telemetry) |  | tlm.perf.rsd.symbolErrorRate |
| M-1499 | dcc1 (Telemetry) |  | tlm.perf.rsd.selfTestResult |
| M-1500 | dcc1 (Ranging) |  | rng.config.t0Trk |
| M-1503* | dcc1 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-1504* | dcc1 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-1505* | dcc1 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-1506 | dcc1 (Ranging) |  | rng.perf.rangeLock |
| M-1507 | dcc1 (Ranging) |  | rng.perf.blocksSent |
| M-1508 | dcc1 (Ranging) |  | rng.perf.carBlocksSent |
| M-1509 | dcc1 (Ranging) |  | rng.perf.totalAcqCompleted |
| M-1510 | dcc1 (Ranging) |  | rng.perf.rangeConfidence |
| M-1511 | dcc1 (Ranging) |  | rng.perf.residRuValue |
| M-1512 | dcc1 (Ranging) |  | rng.perf.rangePwrToNoise |
| M-1513 | dcc1 (Ranging) |  | rng.perf.pwrNoiseResidual |
| M-1514 | dcc1 (Ranging) |  | rng.config.patternType |
| M-1515 | dcc1 (Ranging) |  | rng.config.rtlt |
| M-1516 | dcc1 (Ranging) |  | rng.config.clock |
| M-1517 | dcc1 (Ranging) |  | rng.config.last |
| M-1518 | dcc1 (Ranging) |  | rng.config.chop |
| M-1519 | dcc1 (Ranging) |  | rng.config.chopStrt |
| M-1520 | dcc1 (Ranging) |  | rng.config.t1Xmit |
| M-1521 | dcc1 (Ranging) |  | rng.config.t2Xmit |
| M-1522 | dcc1 (Ranging) |  | rng.config.t3Xmit |
| M-1523 | dcc1 (Ranging) |  | rng.config.drvn |
| M-1524 | dcc1 (Ranging) |  | rng.config.intTime |
| M-1525 | dcc1 (Ranging) |  | rng.config.clockDiv |
| M-1526 | dcc1 (Ranging) |  | rng.config.comp1Len |
| M-1527 | dcc1 (Ranging) |  | rng.config.comp1Def |
| M-1528 | dcc1 (Ranging) |  | rng.config.comp1Bool |
| M-1529 | dcc1 (Ranging) |  | rng.config.comp2Len |
| M-1530 | dcc1 (Ranging) |  | rng.config.comp2Def |
| M-1531 | dcc1 (Ranging) |  | rng.config.comp2Bool |
| M-1532 | dcc1 (Ranging) |  | rng.config.comp3Len |
| M-1533 | dcc1 (Ranging) |  | rng.config.comp3Def |
| M-1534 | dcc1 (Ranging) |  | rng.config.comp3Bool |
| M-1535 | dcc1 (Ranging) |  | rng.config.comp4Len |
| M-1536 | dcc1 (Ranging) |  | rng.config.comp4Def |
| M-1537 | dcc1 (Ranging) |  | rng.config.comp4Bool |
| M-1538 | dcc1 (Ranging) |  | rng.config.comp5Len |
| M-1539 | dcc1 (Ranging) |  | rng.config.comp5Def |
| M-1540 | dcc1 (Ranging) |  | rng.config.comp5Bool |
| M-1541 | dcc1 (Ranging) |  | rng.config.comp6Len |
| M-1542 | dcc1 (Ranging) |  | rng.config.comp6Def |
| M-1543 | dcc1 (Ranging) |  | rng.config.comp6Bool |
| M-1544 | dcc1 (Ranging) |  | rng.perf.drvidRuValue |
| M-1545 | dcc1 (Ranging) |  | rng.perf.currentMode |
| M-1546 | dcc1 (Ranging) |  | rng.perf.acquisitionTime |
| M-1547 | dcc1 (Ranging) |  | rng.perf.rangeRuValue |
| M-1548 | dcc1 (Ranging) |  | rng.config.tolerance |
| M-1700 | dcc2 (Status) |  | smon.status.sum.sys.status |
| M-1701 | dcc2 (Status) |  | smon.status.sum.sys.status |
| M-1702 | dcc2 (Status) |  | smon.status.sum.rec.general |
| M-1703 | dcc2 (Status) |  | smon.status.sum.rec.general |
| M-1704 | dcc2 (Status) |  | smon.status.sum.rrp.general |
| M-1705 | dcc2 (Status) |  | smon.status.sum.rrp.general |
| M-1706 | dcc2 (Status) |  | smon.status.sum.dcc.general |
| M-1707 | dcc2 (Status) |  | smon.status.sum.dcc.general |
| M-1708* | dcc2 (Status) |  | smon.status.sum.rrp.mp.general |
| M-1709* | dcc2 (Status) |  | smon.status.sum.rrp.mp.general |
| M-1710 | dcc2 (Status) |  | smon.status.sum.rng.general |
| M-1711 | dcc2 (Status) |  | smon.status.sum.rng.general |
| M-1712 | dcc2 (Status) |  | smon.status.sum.tlp.general |
| M-1713 | dcc2 (Status) |  | smon.status.sum.tlp.general |
| M-1714* | dcc2 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-1715* | dcc2 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-1716* | dcc2 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-1717* | dcc2 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-1740 | dcc2 (Configuration) |  | DccChannel |
| M-1741 | dcc2 (Configuration) |  | ChannelDss |
| M-1742 | dcc2 (Configuration) |  | rcv.config.scDep.scId |
| M-1743 | dcc2 (Configuration) |  | ChannelPass |
| M-1744 | dcc2 (Configuration) |  | dc.perf.missionId |
| M-1745 | dcc2 (Configuration) |  | rcv.config.scDep.downBand |
| M-1746 | dcc2 (Configuration) |  | rcv.config.scDep.upBand |
| M-1747 | dcc2 (Configuration) |  | rcv.config.tableId |
| M-1748 | dcc2 (Configuration) |  | rcv.config.scDep.pdxMode |
| M-1750 | dcc2 (Configuration) |  | rcv.config.scDep.uplinkDss |
| M-1751 | dcc2 (Configuration) |  | rcv.config.car.freqPred |
| M-1752 | dcc2 (Configuration) |  | rcv.config.sub.freqPred |
| M-1753 | dcc2 (Configuration) |  | rcv.config.sym.ratePredicts |
| M-1754 | dcc2 (Configuration) |  | rcv.config.nar.sntMode |
| M-1755 | dcc2 (Configuration) |  | rcv.config.rxDep.mpSwitch |
| M-1756 | dcc2 (Configuration) |  | dc.config.trkSetName |
| M-1757 | dcc2 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1758 | dcc2 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1759 | dcc2 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-1760 | dcc2 (Configuration) |  | rcv.config.rxDep.useTpdx |
| M-1761 | dcc2 (Configuration) |  | dc.config.tlmSetName |
| M-1762 | dcc2 (Configuration) |  | rng.config.state |
| M-1763 | dcc2 (Configuration) |  | rng.config.trkDataOutput |
| M-1764 | dcc2 (Configuration) |  | rng.config.rangeOutput |
| M-1765 | dcc2 (Configuration) |  | tlm.config.gen.output |
| M-1766 | dcc2 (Configuration) |  | dc.perf.signal.lnaId |
| M-1767 | dcc2 (Configuration) |  | dc.perf.signal.polarization |
| M-1768 | dcc2 (Configuration) |  | dc.perf.signal.path |
| M-1790 | dcc2 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1791 | dcc2 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1792 | dcc2 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-1793 | dcc2 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1794 | dcc2 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1795 | dcc2 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-1796 | dcc2 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1797 | dcc2 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1798 | dcc2 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-1799 | dcc2 (Carrier Loop) |  | rcv.perf.car.loopBw |
| M-1800 | dcc2 (Carrier Loop) |  | rcv.config.car.loopType |
| M-1801 | dcc2 (Carrier Loop) |  | dc.perf.autom.carLockStatus |
| M-1802 | dcc2 (Carrier Loop) |  | rcv.perf.car.loopStatus |
| M-1803 | dcc2 (Carrier Loop) |  | dc.perf.autom.dataLockStatus |
| M-1804 | dcc2 (Carrier Loop) |  | rcv.perf.car.freqEst |
| M-1805 | dcc2 (Carrier Loop) |  | dc.perf.resid.carFreq |
| M-1806 | dcc2 (Carrier Loop) |  | dc.perf.resid.dopp |
| M-1807 | dcc2 (Carrier Loop) |  | rcv.perf.car.rateEst |
| M-1808 | dcc2 (Carrier Loop) |  | rcv.perf.car.accelEst |
| M-1809 | dcc2 (Carrier Loop) |  | rng.perf.dopNoise |
| M-1810 | dcc2 (Carrier Loop) |  | rcv.perf.car.carPwrEst |
| M-1811 | dcc2 (Carrier Loop) |  | dc.perf.resid.pc |
| M-1812 | dcc2 (Carrier Loop) |  | rcv.perf.sub.pdEst |
| M-1813 | dcc2 (Carrier Loop) |  | dc.perf.resid.pd |
| M-1814 | dcc2 (Carrier Loop) |  | rcv.perf.car.spe |
| M-1815 | dcc2 (Carrier Loop) |  | rcv.perf.car.pcnoEst |
| M-1816 | dcc2 (Carrier Loop) |  | dc.perf.resid.pcno |
| M-1817 | dcc2 (Carrier Loop) |  | rcv.perf.car.trackingMode |
| M-1818 | dcc2 (Carrier Loop) |  | rcv.perf.car.snt |
| M-1819 | dcc2 (Carrier Loop) |  | dc.perf.resid.snt |
| M-1820 | dcc2 (Carrier Loop) |  | rcv.perf.car.freqPredicts |
| M-1821 | dcc2 (Carrier Loop) |  | rng.perf.cycleSlips |
| M-1822 | dcc2 (Carrier Loop) |  | rcv.config.carAcq.useFft |
| M-1823 | dcc2 (Carrier Loop) |  | rcv.perf.state.carFft |
| M-1840 | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1841 | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1842 | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-1843 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.freqEst |
| M-1844 | dcc2 (Subcarrier Loop) |  | dc.perf.resid.subFreq |
| M-1845 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.loopBandwidth |
| M-1846 | dcc2 (Subcarrier Loop) |  | rcv.config.sub.loopType |
| M-1847 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.currentWindowWidth |
| M-1848 | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subLockStatus |
| M-1849* | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1850* | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1851* | dcc2 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-1852 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.loopStatus |
| M-1853 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.pdnoEst |
| M-1854 | dcc2 (Subcarrier Loop) |  | dc.perf.resid.pdno |
| M-1855 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.spe |
| M-1856 | dcc2 (Subcarrier Loop) |  | rcv.config.sub.waveForm |
| M-1857 | dcc2 (Subcarrier Loop) |  | rcv.perf.sub.freqPredicts |
| M-1858 | dcc2 (Subcarrier Loop) |  | rcv.config.subAcq.useFft |
| M-1859 | dcc2 (Subcarrier Loop) |  | rcv.perf.state.subFft |
| M-1880 | dcc2 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1881 | dcc2 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1882 | dcc2 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-1883 | dcc2 (Symbol Loop) |  | rcv.perf.sym.rateEst |
| M-1884 | dcc2 (Symbol Loop) |  | dc.perf.resid.symRate |
| M-1885 | dcc2 (Symbol Loop) |  | rcv.perf.sym.loopBandwidth |
| M-1886 | dcc2 (Symbol Loop) |  | rcv.config.sym.loopType |
| M-1887 | dcc2 (Symbol Loop) |  | rcv.perf.sym.currentWindowWidth |
| M-1888 | dcc2 (Symbol Loop) |  | dc.perf.autom.symLockStatus |
| M-1889* | dcc2 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1890* | dcc2 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1891* | dcc2 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-1892 | dcc2 (Symbol Loop) |  | rcv.perf.sym.loopStatus |
| M-1893 | dcc2 (Symbol Loop) |  | rcv.perf.sym.snrEst |
| M-1894 | dcc2 (Symbol Loop) |  | dc.perf.resid.symSnr |
| M-1895 | dcc2 (Symbol Loop) |  | rcv.perf.sym.spe |
| M-1896 | dcc2 (Symbol Loop) |  | rcv.config.sym.format |
| M-1897 | dcc2 (Symbol Loop) |  | rcv.config.sym.smoothing |
| M-1898 | dcc2 (Symbol Loop) |  | rcv.perf.sym.ratePred |
| M-1899 | dcc2 (Symbol Loop) |  | rcv.config.symAcq.useFft |
| M-1900 | dcc2 (Symbol Loop) |  | rcv.perf.state.symFft |
| M-1915 | dcc2 (Telemetry) |  | tlm.config.fan.frameMode |
| M-1916 | dcc2 (Telemetry) |  | tlm.perf.gen.rrpArrayStatus |
| M-1917 | dcc2 (Telemetry) |  | tlm.config.fs.mode |
| M-1918 | dcc2 (Telemetry) |  | tlm.config.fs.pattern |
| M-1919 | dcc2 (Telemetry) |  | tlm.config.fs.patternLength |
| M-1920 | dcc2 (Telemetry) |  | tlm.perf.fs.predictedBitRate |
| M-1921 | dcc2 (Telemetry) |  | tlm.perf.fs.dataRate |
| M-1922 | dcc2 (Telemetry) |  | tlm.perf.gen.tlpLockStatus |
| M-1924 | dcc2 (Telemetry) |  | tlm.perf.gen.chanStatus |
| M-1925* | dcc2 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1926* | dcc2 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1927* | dcc2 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-1928* | dcc2 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1929* | dcc2 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1930* | dcc2 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-1931 | dcc2 (Telemetry) |  | tlm.perf.gen.decoderLockStatus |
| M-1933 | dcc2 (Telemetry) |  | tlm.config.gen.decoderType |
| M-1934 | dcc2 (Telemetry) |  | tlm.perf.gen.decoderSNR |
| M-1936 | dcc2 (Telemetry) |  | tlm.config.b2cd.ilThreshold |
| M-1937 | dcc2 (Telemetry) |  | tlm.config.b2cd.vecSet |
| M-1938 | dcc2 (Telemetry) |  | tlm.config.b3cd.ilThreshold |
| M-1939 | dcc2 (Telemetry) |  | tlm.config.b3cd.vecSet |
| M-1940 | dcc2 (Telemetry) |  | tlm.config.fs.frameLengthPrimary |
| M-1941 | dcc2 (Telemetry) |  | tlm.config.fs.ilBitErrorThreshold |
| M-1942 | dcc2 (Telemetry) |  | tlm.config.fs.oolBitErrorThreshold |
| M-1943 | dcc2 (Telemetry) |  | tlm.config.fs.ilThreshold |
| M-1944 | dcc2 (Telemetry) |  | tlm.config.fs.oolThreshold |
| M-1945* | dcc2 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1946* | dcc2 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1947* | dcc2 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-1948* | dcc2 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1949* | dcc2 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1950* | dcc2 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-1951 | dcc2 (Telemetry) |  | tlm.perf.fs.lockStatus |
| M-1952 | dcc2 (Telemetry) |  | tlm.perf.fs.bitToAcquire |
| M-1953 | dcc2 (Telemetry) |  | tlm.perf.gen.blocksOutput |
| M-1955 | dcc2 (Telemetry) |  | tlm.perf.fs.frameTotalCount |
| M-1956 | dcc2 (Telemetry) |  | tlm.perf.fs.frameGoodCount |
| M-1957 | dcc2 (Telemetry) |  | tlm.perf.fs.frameBadCount |
| M-1958 | dcc2 (Telemetry) |  | tlm.perf.crc.crcLockStatus |
| M-1959 | dcc2 (Telemetry) |  | tlm.config.fan.pnDecode |
| M-1960 | dcc2 (Telemetry) |  | tlm.perf.crc.crcTotalFrameCount |
| M-1961 | dcc2 (Telemetry) |  | tlm.perf.crc.crcGoodFrameCount |
| M-1962 | dcc2 (Telemetry) |  | tlm.perf.crc.crcBadFrameCount |
| M-1963 | dcc2 (Telemetry) |  | tlm.perf.rsd.lockStatus |
| M-1964 | dcc2 (Telemetry) |  | tlm.perf.rsd.totalCodeWordCount |
| M-1965 | dcc2 (Telemetry) |  | tlm.perf.rsd.goodCodeWordCount |
| M-1966 | dcc2 (Telemetry) |  | tlm.perf.rsd.badCodeWordCount |
| M-1967 | dcc2 (Telemetry) |  | tlm.perf.rsd.frameErrorRate |
| M-1968 | dcc2 (Telemetry) |  | tlm.perf.rsd.codeWordErrorRate |
| M-1969 | dcc2 (Telemetry) |  | tlm.perf.fs.totalInLockFrameCount |
| M-1970 | dcc2 (Telemetry) |  | tlm.perf.fs.flywheelFrameCount |
| M-1971 | dcc2 (Telemetry) |  | tlm.config.dft.formatTable |
| M-1974 | dcc2 (Telemetry) |  | tlm.config.b2cd.oolThreshold |
| M-1975 | dcc2 (Telemetry) |  | tlm.config.b3cd.oolThreshold |
| M-1976 | dcc2 (Telemetry) |  | tlm.config.gen.diffDecoder |
| M-1977 | dcc2 (Telemetry) |  | tlm.config.b2cd.altSymInv |
| M-1978 | dcc2 (Telemetry) |  | tlm.config.b3cd.symSignInversion |
| M-1979 | dcc2 (Telemetry) |  | tlm.perf.b2cd.metricNormalization |
| M-1980 | dcc2 (Telemetry) |  | tlm.perf.b2cd.estimatedSymbolErrorRate |
| M-1981 | dcc2 (Telemetry) |  | tlm.config.b2cd.nodeSyncChange |
| M-1982 | dcc2 (Telemetry) |  | tlm.perf.b2cd.nodeSyncChangeCount |
| M-1983 | dcc2 (Telemetry) |  | tlm.perf.b3cd.nodeSyncPosition |
| M-1984 | dcc2 (Telemetry) |  | tlm.config.fs.frameLengthSecondary |
| M-1985 | dcc2 (Telemetry) |  | tlm.perf.fs.correctFrameLength |
| M-1986 | dcc2 (Telemetry) |  | tlm.perf.fs.selfTestResult |
| M-1987 | dcc2 (Telemetry) |  | tlm.config.fs.bitSequence |
| M-1988 | dcc2 (Telemetry) |  | tlm.config.fs.bitSlipWindow |
| M-1989 | dcc2 (Telemetry) |  | tlm.perf.fs.inLockBer |
| M-1990 | dcc2 (Telemetry) |  | tlm.perf.fs.syncDirection |
| M-1991 | dcc2 (Telemetry) |  | tlm.perf.fs.currentPolarity |
| M-1992 | dcc2 (Telemetry) |  | tlm.perf.fs.bitSlip |
| M-1993 | dcc2 (Telemetry) |  | tlm.perf.fs.polarityChange |
| M-1994 | dcc2 (Telemetry) |  | tlm.perf.fs.acquisitionCount |
| M-1995 | dcc2 (Telemetry) |  | tlm.config.fan.interleave |
| M-1996 | dcc2 (Telemetry) |  | tlm.config.fan.algorithm |
| M-1997 | dcc2 (Telemetry) |  | tlm.perf.rsd.virtualFill |
| M-1998 | dcc2 (Telemetry) |  | tlm.perf.rsd.symbolErrorRate |
| M-1999 | dcc2 (Telemetry) |  | tlm.perf.rsd.selfTestResult |
| M-2000 | dcc2 (Ranging) |  | rng.config.t0Trk |
| M-2003* | dcc2 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2004* | dcc2 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2005* | dcc2 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2006 | dcc2 (Ranging) |  | rng.perf.rangeLock |
| M-2007 | dcc2 (Ranging) |  | rng.perf.blocksSent |
| M-2008 | dcc2 (Ranging) |  | rng.perf.carBlocksSent |
| M-2009 | dcc2 (Ranging) |  | rng.perf.totalAcqCompleted |
| M-2010 | dcc2 (Ranging) |  | rng.perf.rangeConfidence |
| M-2011 | dcc2 (Ranging) |  | rng.perf.residRuValue |
| M-2012 | dcc2 (Ranging) |  | rng.perf.rangePwrToNoise |
| M-2013 | dcc2 (Ranging) |  | rng.perf.pwrNoiseResidual |
| M-2014 | dcc2 (Ranging) |  | rng.config.patternType |
| M-2015 | dcc2 (Ranging) |  | rng.config.rtlt |
| M-2016 | dcc2 (Ranging) |  | rng.config.clock |
| M-2017 | dcc2 (Ranging) |  | rng.config.last |
| M-2018 | dcc2 (Ranging) |  | rng.config.chop |
| M-2019 | dcc2 (Ranging) |  | rng.config.chopStrt |
| M-2020 | dcc2 (Ranging) |  | rng.config.t1Xmit |
| M-2021 | dcc2 (Ranging) |  | rng.config.t2Xmit |
| M-2022 | dcc2 (Ranging) |  | rng.config.t3Xmit |
| M-2023 | dcc2 (Ranging) |  | rng.config.drvn |
| M-2024 | dcc2 (Ranging) |  | rng.config.intTime |
| M-2025 | dcc2 (Ranging) |  | rng.config.clockDiv |
| M-2026 | dcc2 (Ranging) |  | rng.config.comp1Len |
| M-2027 | dcc2 (Ranging) |  | rng.config.comp1Def |
| M-2028 | dcc2 (Ranging) |  | rng.config.comp1Bool |
| M-2029 | dcc2 (Ranging) |  | rng.config.comp2Len |
| M-2030 | dcc2 (Ranging) |  | rng.config.comp2Def |
| M-2031 | dcc2 (Ranging) |  | rng.config.comp2Bool |
| M-2032 | dcc2 (Ranging) |  | rng.config.comp3Len |
| M-2033 | dcc2 (Ranging) |  | rng.config.comp3Def |
| M-2034 | dcc2 (Ranging) |  | rng.config.comp3Bool |
| M-2035 | dcc2 (Ranging) |  | rng.config.comp4Len |
| M-2036 | dcc2 (Ranging) |  | rng.config.comp4Def |
| M-2037 | dcc2 (Ranging) |  | rng.config.comp4Bool |
| M-2038 | dcc2 (Ranging) |  | rng.config.comp5Len |
| M-2039 | dcc2 (Ranging) |  | rng.config.comp5Def |
| M-2040 | dcc2 (Ranging) |  | rng.config.comp5Bool |
| M-2041 | dcc2 (Ranging) |  | rng.config.comp6Len |
| M-2042 | dcc2 (Ranging) |  | rng.config.comp6Def |
| M-2043 | dcc2 (Ranging) |  | rng.config.comp6Bool |
| M-2044 | dcc2 (Ranging) |  | rng.perf.drvidRuValue |
| M-2045 | dcc2 (Ranging) |  | rng.perf.currentMode |
| M-2046 | dcc2 (Ranging) |  | rng.perf.acquisitionTime |
| M-2047 | dcc2 (Ranging) |  | rng.perf.rangeRuValue |
| M-2048 | dcc2 (Ranging) |  | rng.config.tolerance |
| M-2200 | dcc3 (Status) |  | smon.status.sum.sys.status |
| M-2201 | dcc3 (Status) |  | smon.status.sum.sys.status |
| M-2202 | dcc3 (Status) |  | smon.status.sum.rec.general |
| M-2203 | dcc3 (Status) |  | smon.status.sum.rec.general |
| M-2204 | dcc3 (Status) |  | smon.status.sum.rrp.general |
| M-2205 | dcc3 (Status) |  | smon.status.sum.rrp.general |
| M-2206 | dcc3 (Status) |  | smon.status.sum.dcc.general |
| M-2207 | dcc3 (Status) |  | smon.status.sum.dcc.general |
| M-2208* | dcc3 (Status) |  | smon.status.sum.rrp.mp.general |
| M-2209* | dcc3 (Status) |  | smon.status.sum.rrp.mp.general |
| M-2210 | dcc3 (Status) |  | smon.status.sum.rng.general |
| M-2211 | dcc3 (Status) |  | smon.status.sum.rng.general |
| M-2212 | dcc3 (Status) |  | smon.status.sum.tlp.general |
| M-2213 | dcc3 (Status) |  | smon.status.sum.tlp.general |
| M-2214* | dcc3 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-2215* | dcc3 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-2216* | dcc3 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-2217* | dcc3 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-2240 | dcc3 (Configuration) |  | DccChannel |
| M-2241 | dcc3 (Configuration) |  | ChannelDss |
| M-2242 | dcc3 (Configuration) |  | rcv.config.scDep.scId |
| M-2243 | dcc3 (Configuration) |  | ChannelPass |
| M-2244 | dcc3 (Configuration) |  | dc.perf.missionId |
| M-2245 | dcc3 (Configuration) |  | rcv.config.scDep.downBand |
| M-2246 | dcc3 (Configuration) |  | rcv.config.scDep.upBand |
| M-2247 | dcc3 (Configuration) |  | rcv.config.tableId |
| M-2248 | dcc3 (Configuration) |  | rcv.config.scDep.pdxMode |
| M-2250 | dcc3 (Configuration) |  | rcv.config.scDep.uplinkDss |
| M-2251 | dcc3 (Configuration) |  | rcv.config.car.freqPred |
| M-2252 | dcc3 (Configuration) |  | rcv.config.sub.freqPred |
| M-2253 | dcc3 (Configuration) |  | rcv.config.sym.ratePredicts |
| M-2254 | dcc3 (Configuration) |  | rcv.config.nar.sntMode |
| M-2255 | dcc3 (Configuration) |  | rcv.config.rxDep.mpSwitch |
| M-2256 | dcc3 (Configuration) |  | dc.config.trkSetName |
| M-2257 | dcc3 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2258 | dcc3 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2259 | dcc3 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2260 | dcc3 (Configuration) |  | rcv.config.rxDep.useTpdx |
| M-2261 | dcc3 (Configuration) |  | dc.config.tlmSetName |
| M-2262 | dcc3 (Configuration) |  | rng.config.state |
| M-2263 | dcc3 (Configuration) |  | rng.config.trkDataOutput |
| M-2264 | dcc3 (Configuration) |  | rng.config.rangeOutput |
| M-2265 | dcc3 (Configuration) |  | tlm.config.gen.output |
| M-2266 | dcc3 (Configuration) |  | dc.perf.signal.lnaId |
| M-2267 | dcc3 (Configuration) |  | dc.perf.signal.polarization |
| M-2268 | dcc3 (Configuration) |  | dc.perf.signal.path |
| M-2290 | dcc3 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2291 | dcc3 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2292 | dcc3 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2293 | dcc3 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2294 | dcc3 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2295 | dcc3 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2296 | dcc3 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2297 | dcc3 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2298 | dcc3 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2299 | dcc3 (Carrier Loop) |  | rcv.perf.car.loopBw |
| M-2300 | dcc3 (Carrier Loop) |  | rcv.config.car.loopType |
| M-2301 | dcc3 (Carrier Loop) |  | dc.perf.autom.carLockStatus |
| M-2302 | dcc3 (Carrier Loop) |  | rcv.perf.car.loopStatus |
| M-2303 | dcc3 (Carrier Loop) |  | dc.perf.autom.dataLockStatus |
| M-2304 | dcc3 (Carrier Loop) |  | rcv.perf.car.freqEst |
| M-2305 | dcc3 (Carrier Loop) |  | dc.perf.resid.carFreq |
| M-2306 | dcc3 (Carrier Loop) |  | dc.perf.resid.dopp |
| M-2307 | dcc3 (Carrier Loop) |  | rcv.perf.car.rateEst |
| M-2308 | dcc3 (Carrier Loop) |  | rcv.perf.car.accelEst |
| M-2309 | dcc3 (Carrier Loop) |  | rng.perf.dopNoise |
| M-2310 | dcc3 (Carrier Loop) |  | rcv.perf.car.carPwrEst |
| M-2311 | dcc3 (Carrier Loop) |  | dc.perf.resid.pc |
| M-2312 | dcc3 (Carrier Loop) |  | rcv.perf.sub.pdEst |
| M-2313 | dcc3 (Carrier Loop) |  | dc.perf.resid.pd |
| M-2314 | dcc3 (Carrier Loop) |  | rcv.perf.car.spe |
| M-2315 | dcc3 (Carrier Loop) |  | rcv.perf.car.pcnoEst |
| M-2316 | dcc3 (Carrier Loop) |  | dc.perf.resid.pcno |
| M-2317 | dcc3 (Carrier Loop) |  | rcv.perf.car.trackingMode |
| M-2318 | dcc3 (Carrier Loop) |  | rcv.perf.car.snt |
| M-2319 | dcc3 (Carrier Loop) |  | dc.perf.resid.snt |
| M-2320 | dcc3 (Carrier Loop) |  | rcv.perf.car.freqPredicts |
| M-2321 | dcc3 (Carrier Loop) |  | rng.perf.cycleSlips |
| M-2322 | dcc3 (Carrier Loop) |  | rcv.config.carAcq.useFft |
| M-2323 | dcc3 (Carrier Loop) |  | rcv.perf.state.carFft |
| M-2340 | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2341 | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2342 | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2343 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.freqEst |
| M-2344 | dcc3 (Subcarrier Loop) |  | dc.perf.resid.subFreq |
| M-2345 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.loopBandwidth |
| M-2346 | dcc3 (Subcarrier Loop) |  | rcv.config.sub.loopType |
| M-2347 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.currentWindowWidth |
| M-2348 | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subLockStatus |
| M-2349* | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2350* | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2351* | dcc3 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2352 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.loopStatus |
| M-2353 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.pdnoEst |
| M-2354 | dcc3 (Subcarrier Loop) |  | dc.perf.resid.pdno |
| M-2355 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.spe |
| M-2356 | dcc3 (Subcarrier Loop) |  | rcv.config.sub.waveForm |
| M-2357 | dcc3 (Subcarrier Loop) |  | rcv.perf.sub.freqPredicts |
| M-2358 | dcc3 (Subcarrier Loop) |  | rcv.config.subAcq.useFft |
| M-2359 | dcc3 (Subcarrier Loop) |  | rcv.perf.state.subFft |
| M-2380 | dcc3 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2381 | dcc3 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2382 | dcc3 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2383 | dcc3 (Symbol Loop) |  | rcv.perf.sym.rateEst |
| M-2384 | dcc3 (Symbol Loop) |  | dc.perf.resid.symRate |
| M-2385 | dcc3 (Symbol Loop) |  | rcv.perf.sym.loopBandwidth |
| M-2386 | dcc3 (Symbol Loop) |  | rcv.config.sym.loopType |
| M-2387 | dcc3 (Symbol Loop) |  | rcv.perf.sym.currentWindowWidth |
| M-2388 | dcc3 (Symbol Loop) |  | dc.perf.autom.symLockStatus |
| M-2389* | dcc3 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2390* | dcc3 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2391* | dcc3 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2392 | dcc3 (Symbol Loop) |  | rcv.perf.sym.loopStatus |
| M-2393 | dcc3 (Symbol Loop) |  | rcv.perf.sym.snrEst |
| M-2394 | dcc3 (Symbol Loop) |  | dc.perf.resid.symSnr |
| M-2395 | dcc3 (Symbol Loop) |  | rcv.perf.sym.spe |
| M-2396 | dcc3 (Symbol Loop) |  | rcv.config.sym.format |
| M-2397 | dcc3 (Symbol Loop) |  | rcv.config.sym.smoothing |
| M-2398 | dcc3 (Symbol Loop) |  | rcv.perf.sym.ratePred |
| M-2399 | dcc3 (Symbol Loop) |  | rcv.config.symAcq.useFft |
| M-2400 | dcc3 (Symbol Loop) |  | rcv.perf.state.symFft |
| M-2407 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount1 |
| M-2408 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount2 |
| M-2409 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount3 |
| M-2410 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount4 |
| M-2411 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount5 |
| M-2412 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount6 |
| M-2413 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount7 |
| M-2414 | dcc3 (Telemetry) |  | tlm.perf.fmt.vcCount8 |
| M-2415 | dcc3 (Telemetry) |  | tlm.config.fan.frameMode |
| M-2416 | dcc3 (Telemetry) |  | tlm.perf.gen.rrpArrayStatus |
| M-2417 | dcc3 (Telemetry) |  | tlm.config.fs.mode |
| M-2418 | dcc3 (Telemetry) |  | tlm.config.fs.pattern |
| M-2419 | dcc3 (Telemetry) |  | tlm.config.fs.patternLength |
| M-2420 | dcc3 (Telemetry) |  | tlm.perf.fs.predictedBitRate |
| M-2421 | dcc3 (Telemetry) |  | tlm.perf.fs.dataRate |
| M-2422 | dcc3 (Telemetry) |  | tlm.perf.gen.tlpLockStatus |
| M-2424 | dcc3 (Telemetry) |  | tlm.perf.gen.chanStatus |
| M-2425* | dcc3 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2426* | dcc3 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2427* | dcc3 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2428* | dcc3 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2429* | dcc3 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2430* | dcc3 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2431 | dcc3 (Telemetry) |  | tlm.perf.gen.decoderLockStatus |
| M-2433 | dcc3 (Telemetry) |  | tlm.config.gen.decoderType |
| M-2434 | dcc3 (Telemetry) |  | tlm.perf.gen.decoderSNR |
| M-2436 | dcc3 (Telemetry) |  | tlm.config.b2cd.ilThreshold |
| M-2437 | dcc3 (Telemetry) |  | tlm.config.b2cd.vecSet |
| M-2438 | dcc3 (Telemetry) |  | tlm.config.b3cd.ilThreshold |
| M-2439 | dcc3 (Telemetry) |  | tlm.config.b3cd.vecSet |
| M-2440 | dcc3 (Telemetry) |  | tlm.config.fs.frameLengthPrimary |
| M-2441 | dcc3 (Telemetry) |  | tlm.config.fs.ilBitErrorThreshold |
| M-2442 | dcc3 (Telemetry) |  | tlm.config.fs.oolBitErrorThreshold |
| M-2443 | dcc3 (Telemetry) |  | tlm.config.fs.ilThreshold |
| M-2444 | dcc3 (Telemetry) |  | tlm.config.fs.oolThreshold |
| M-2445* | dcc3 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2446* | dcc3 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2447* | dcc3 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2448* | dcc3 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2449* | dcc3 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2450* | dcc3 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2451 | dcc3 (Telemetry) |  | tlm.perf.fs.lockStatus |
| M-2452 | dcc3 (Telemetry) |  | tlm.perf.fs.bitToAcquire |
| M-2453 | dcc3 (Telemetry) |  | tlm.perf.gen.blocksOutput |
| M-2455 | dcc3 (Telemetry) |  | tlm.perf.fs.frameTotalCount |
| M-2456 | dcc3 (Telemetry) |  | tlm.perf.fs.frameGoodCount |
| M-2457 | dcc3 (Telemetry) |  | tlm.perf.fs.frameBadCount |
| M-2458 | dcc3 (Telemetry) |  | tlm.perf.crc.crcLockStatus |
| M-2459 | dcc3 (Telemetry) |  | tlm.config.fan.pnDecode |
| M-2460 | dcc3 (Telemetry) |  | tlm.perf.crc.crcTotalFrameCount |
| M-2461 | dcc3 (Telemetry) |  | tlm.perf.crc.crcGoodFrameCount |
| M-2462 | dcc3 (Telemetry) |  | tlm.perf.crc.crcBadFrameCount |
| M-2463 | dcc3 (Telemetry) |  | tlm.perf.rsd.lockStatus |
| M-2464 | dcc3 (Telemetry) |  | tlm.perf.rsd.totalCodeWordCount |
| M-2465 | dcc3 (Telemetry) |  | tlm.perf.rsd.goodCodeWordCount |
| M-2466 | dcc3 (Telemetry) |  | tlm.perf.rsd.badCodeWordCount |
| M-2467 | dcc3 (Telemetry) |  | tlm.perf.rsd.frameErrorRate |
| M-2468 | dcc3 (Telemetry) |  | tlm.perf.rsd.codeWordErrorRate |
| M-2469 | dcc3 (Telemetry) |  | tlm.perf.fs.totalInLockFrameCount |
| M-2470 | dcc3 (Telemetry) |  | tlm.perf.fs.flywheelFrameCount |
| M-2471 | dcc3 (Telemetry) |  | tlm.config.dft.formatTable |
| M-2474 | dcc3 (Telemetry) |  | tlm.config.b2cd.oolThreshold |
| M-2475 | dcc3 (Telemetry) |  | tlm.config.b3cd.oolThreshold |
| M-2476 | dcc3 (Telemetry) |  | tlm.config.gen.diffDecoder |
| M-2477 | dcc3 (Telemetry) |  | tlm.config.b2cd.altSymInv |
| M-2478 | dcc3 (Telemetry) |  | tlm.config.b3cd.symSignInversion |
| M-2479 | dcc3 (Telemetry) |  | tlm.perf.b2cd.metricNormalization |
| M-2480 | dcc3 (Telemetry) |  | tlm.perf.b2cd.estimatedSymbolErrorRate |
| M-2481 | dcc3 (Telemetry) |  | tlm.config.b2cd.nodeSyncChange |
| M-2482 | dcc3 (Telemetry) |  | tlm.perf.b2cd.nodeSyncChangeCount |
| M-2483 | dcc3 (Telemetry) |  | tlm.perf.b3cd.nodeSyncPosition |
| M-2484 | dcc3 (Telemetry) |  | tlm.config.fs.frameLengthSecondary |
| M-2485 | dcc3 (Telemetry) |  | tlm.perf.fs.correctFrameLength |
| M-2486 | dcc3 (Telemetry) |  | tlm.perf.fs.selfTestResult |
| M-2487 | dcc3 (Telemetry) |  | tlm.config.fs.bitSequence |
| M-2488 | dcc3 (Telemetry) |  | tlm.config.fs.bitSlipWindow |
| M-2489 | dcc3 (Telemetry) |  | tlm.perf.fs.inLockBer |
| M-2490 | dcc3 (Telemetry) |  | tlm.perf.fs.syncDirection |
| M-2491 | dcc3 (Telemetry) |  | tlm.perf.fs.currentPolarity |
| M-2492 | dcc3 (Telemetry) |  | tlm.perf.fs.bitSlip |
| M-2493 | dcc3 (Telemetry) |  | tlm.perf.fs.polarityChange |
| M-2494 | dcc3 (Telemetry) |  | tlm.perf.fs.acquisitionCount |
| M-2495 | dcc3 (Telemetry) |  | tlm.config.fan.interleave |
| M-2496 | dcc3 (Telemetry) |  | tlm.config.fan.algorithm |
| M-2497 | dcc3 (Telemetry) |  | tlm.perf.rsd.virtualFill |
| M-2498 | dcc3 (Telemetry) |  | tlm.perf.rsd.symbolErrorRate |
| M-2499 | dcc3 (Telemetry) |  | tlm.perf.rsd.selfTestResult |
| M-2500 | dcc3 (Ranging) |  | rng.config.t0Trk |
| M-2503* | dcc3 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2504* | dcc3 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2505* | dcc3 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-2506 | dcc3 (Ranging) |  | rng.perf.rangeLock |
| M-2507 | dcc3 (Ranging) |  | rng.perf.blocksSent |
| M-2508 | dcc3 (Ranging) |  | rng.perf.carBlocksSent |
| M-2509 | dcc3 (Ranging) |  | rng.perf.totalAcqCompleted |
| M-2510 | dcc3 (Ranging) |  | rng.perf.rangeConfidence |
| M-2511 | dcc3 (Ranging) |  | rng.perf.residRuValue |
| M-2512 | dcc3 (Ranging) |  | rng.perf.rangePwrToNoise |
| M-2513 | dcc3 (Ranging) |  | rng.perf.pwrNoiseResidual |
| M-2514 | dcc3 (Ranging) |  | rng.config.patternType |
| M-2515 | dcc3 (Ranging) |  | rng.config.rtlt |
| M-2516 | dcc3 (Ranging) |  | rng.config.clock |
| M-2517 | dcc3 (Ranging) |  | rng.config.last |
| M-2518 | dcc3 (Ranging) |  | rng.config.chop |
| M-2519 | dcc3 (Ranging) |  | rng.config.chopStrt |
| M-2520 | dcc3 (Ranging) |  | rng.config.t1Xmit |
| M-2521 | dcc3 (Ranging) |  | rng.config.t2Xmit |
| M-2522 | dcc3 (Ranging) |  | rng.config.t3Xmit |
| M-2523 | dcc3 (Ranging) |  | rng.config.drvn |
| M-2524 | dcc3 (Ranging) |  | rng.config.intTime |
| M-2525 | dcc3 (Ranging) |  | rng.config.clockDiv |
| M-2526 | dcc3 (Ranging) |  | rng.config.comp1Len |
| M-2527 | dcc3 (Ranging) |  | rng.config.comp1Def |
| M-2528 | dcc3 (Ranging) |  | rng.config.comp1Bool |
| M-2529 | dcc3 (Ranging) |  | rng.config.comp2Len |
| M-2530 | dcc3 (Ranging) |  | rng.config.comp2Def |
| M-2531 | dcc3 (Ranging) |  | rng.config.comp2Bool |
| M-2532 | dcc3 (Ranging) |  | rng.config.comp3Len |
| M-2533 | dcc3 (Ranging) |  | rng.config.comp3Def |
| M-2534 | dcc3 (Ranging) |  | rng.config.comp3Bool |
| M-2535 | dcc3 (Ranging) |  | rng.config.comp4Len |
| M-2536 | dcc3 (Ranging) |  | rng.config.comp4Def |
| M-2537 | dcc3 (Ranging) |  | rng.config.comp4Bool |
| M-2538 | dcc3 (Ranging) |  | rng.config.comp5Len |
| M-2539 | dcc3 (Ranging) |  | rng.config.comp5Def |
| M-2540 | dcc3 (Ranging) |  | rng.config.comp5Bool |
| M-2541 | dcc3 (Ranging) |  | rng.config.comp6Len |
| M-2542 | dcc3 (Ranging) |  | rng.config.comp6Def |
| M-2543 | dcc3 (Ranging) |  | rng.config.comp6Bool |
| M-2544 | dcc3 (Ranging) |  | rng.perf.drvidRuValue |
| M-2545 | dcc3 (Ranging) |  | rng.perf.currentMode |
| M-2546 | dcc3 (Ranging) |  | rng.perf.acquisitionTime |
| M-2547 | dcc3 (Ranging) |  | rng.perf.rangeRuValue |
| M-2548 | dcc3 (Ranging) |  | rng.config.tolerance |
| M-2700 | dcc4 (Status) |  | smon.status.sum.sys.status |
| M-2701 | dcc4 (Status) |  | smon.status.sum.sys.status |
| M-2702 | dcc4 (Status) |  | smon.status.sum.rec.general |
| M-2703 | dcc4 (Status) |  | smon.status.sum.rec.general |
| M-2704 | dcc4 (Status) |  | smon.status.sum.rrp.general |
| M-2705 | dcc4 (Status) |  | smon.status.sum.rrp.general |
| M-2706 | dcc4 (Status) |  | smon.status.sum.dcc.general |
| M-2707 | dcc4 (Status) |  | smon.status.sum.dcc.general |
| M-2708* | dcc4 (Status) |  | smon.status.sum.rrp.mp.general |
| M-2709* | dcc4 (Status) |  | smon.status.sum.rrp.mp.general |
| M-2710 | dcc4 (Status) |  | smon.status.sum.rng.general |
| M-2711 | dcc4 (Status) |  | smon.status.sum.rng.general |
| M-2712 | dcc4 (Status) |  | smon.status.sum.tlp.general |
| M-2713 | dcc4 (Status) |  | smon.status.sum.tlp.general |
| M-2714* | dcc4 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-2715* | dcc4 (Status) |  | smon.status.sum.tlp.mcd3.general |
| M-2716* | dcc4 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-2717* | dcc4 (Status) |  | smon.status.sum.tlp.turbo.general |
| M-2740 | dcc4 (Configuration) |  | DccChannel |
| M-2741 | dcc4 (Configuration) |  | ChannelDss |
| M-2742 | dcc4 (Configuration) |  | rcv.config.scDep.scId |
| M-2743 | dcc4 (Configuration) |  | ChannelPass |
| M-2744 | dcc4 (Configuration) |  | dc.perf.missionId |
| M-2745 | dcc4 (Configuration) |  | rcv.config.scDep.downBand |
| M-2746 | dcc4 (Configuration) |  | rcv.config.scDep.upBand |
| M-2747 | dcc4 (Configuration) |  | rcv.config.tableId |
| M-2748 | dcc4 (Configuration) |  | rcv.config.scDep.pdxMode |
| M-2750 | dcc4 (Configuration) |  | rcv.config.scDep.uplinkDss |
| M-2751 | dcc4 (Configuration) |  | rcv.config.car.freqPred |
| M-2752 | dcc4 (Configuration) |  | rcv.config.sub.freqPred |
| M-2753 | dcc4 (Configuration) |  | rcv.config.sym.ratePredicts |
| M-2754 | dcc4 (Configuration) |  | rcv.config.nar.sntMode |
| M-2755 | dcc4 (Configuration) |  | rcv.config.rxDep.mpSwitch |
| M-2756 | dcc4 (Configuration) |  | dc.config.trkSetName |
| M-2757 | dcc4 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2758 | dcc4 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2759 | dcc4 (Configuration) |  | dc.perf.autom.rpdxChangeTime |
| M-2760 | dcc4 (Configuration) |  | rcv.config.rxDep.useTpdx |
| M-2761 | dcc4 (Configuration) |  | dc.config.tlmSetName |
| M-2762 | dcc4 (Configuration) |  | rng.config.state |
| M-2763 | dcc4 (Configuration) |  | rng.config.trkDataOutput |
| M-2764 | dcc4 (Configuration) |  | rng.config.rangeOutput |
| M-2765 | dcc4 (Configuration) |  | tlm.config.gen.output |
| M-2766 | dcc4 (Configuration) |  | dc.perf.signal.lnaId |
| M-2767 | dcc4 (Configuration) |  | dc.perf.signal.polarization |
| M-2768 | dcc4 (Configuration) |  | dc.perf.signal.path |
| M-2790 | dcc4 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2791 | dcc4 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2792 | dcc4 (Carrier Loop) |  | rcv.perf.car.timeTag |
| M-2793 | dcc4 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2794 | dcc4 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2795 | dcc4 (Carrier Loop) |  | dc.perf.autom.acqStartTime |
| M-2796 | dcc4 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2797 | dcc4 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2798 | dcc4 (Carrier Loop) |  | dc.perf.autom.carLockStatusTime |
| M-2799 | dcc4 (Carrier Loop) |  | rcv.perf.car.loopBw |
| M-2800 | dcc4 (Carrier Loop) |  | rcv.config.car.loopType |
| M-2801 | dcc4 (Carrier Loop) |  | dc.perf.autom.carLockStatus |
| M-2802 | dcc4 (Carrier Loop) |  | rcv.perf.car.loopStatus |
| M-2803 | dcc4 (Carrier Loop) |  | dc.perf.autom.dataLockStatus |
| M-2804 | dcc4 (Carrier Loop) |  | rcv.perf.car.freqEst |
| M-2805 | dcc4 (Carrier Loop) |  | dc.perf.resid.carFreq |
| M-2806 | dcc4 (Carrier Loop) |  | dc.perf.resid.dopp |
| M-2807 | dcc4 (Carrier Loop) |  | rcv.perf.car.rateEst |
| M-2808 | dcc4 (Carrier Loop) |  | rcv.perf.car.accelEst |
| M-2809 | dcc4 (Carrier Loop) |  | rng.perf.dopNoise |
| M-2810 | dcc4 (Carrier Loop) |  | rcv.perf.car.carPwrEst |
| M-2811 | dcc4 (Carrier Loop) |  | dc.perf.resid.pc |
| M-2812 | dcc4 (Carrier Loop) |  | rcv.perf.sub.pdEst |
| M-2813 | dcc4 (Carrier Loop) |  | dc.perf.resid.pd |
| M-2814 | dcc4 (Carrier Loop) |  | rcv.perf.car.spe |
| M-2815 | dcc4 (Carrier Loop) |  | rcv.perf.car.pcnoEst |
| M-2816 | dcc4 (Carrier Loop) |  | dc.perf.resid.pcno |
| M-2817 | dcc4 (Carrier Loop) |  | rcv.perf.car.trackingMode |
| M-2818 | dcc4 (Carrier Loop) |  | rcv.perf.car.snt |
| M-2819 | dcc4 (Carrier Loop) |  | dc.perf.resid.snt |
| M-2820 | dcc4 (Carrier Loop) |  | rcv.perf.car.freqPredicts |
| M-2821 | dcc4 (Carrier Loop) |  | rng.perf.cycleSlips |
| M-2822 | dcc4 (Carrier Loop) |  | rcv.config.carAcq.useFft |
| M-2823 | dcc4 (Carrier Loop) |  | rcv.perf.state.carFft |
| M-2840 | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2841 | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2842 | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subFreqChangeTime |
| M-2843 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.freqEst |
| M-2844 | dcc4 (Subcarrier Loop) |  | dc.perf.resid.subFreq |
| M-2845 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.loopBandwidth |
| M-2846 | dcc4 (Subcarrier Loop) |  | rcv.config.sub.loopType |
| M-2847 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.currentWindowWidth |
| M-2848 | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subLockStatus |
| M-2849* | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2850* | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2851* | dcc4 (Subcarrier Loop) |  | dc.perf.autom.subLockStatusTime |
| M-2852 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.loopStatus |
| M-2853 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.pdnoEst |
| M-2854 | dcc4 (Subcarrier Loop) |  | dc.perf.resid.pdno |
| M-2855 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.spe |
| M-2856 | dcc4 (Subcarrier Loop) |  | rcv.config.sub.waveForm |
| M-2857 | dcc4 (Subcarrier Loop) |  | rcv.perf.sub.freqPredicts |
| M-2858 | dcc4 (Subcarrier Loop) |  | rcv.config.subAcq.useFft |
| M-2859 | dcc4 (Subcarrier Loop) |  | rcv.perf.state.subFft |
| M-2880 | dcc4 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2881 | dcc4 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2882 | dcc4 (Symbol Loop) |  | dc.perf.autom.symRateChangeTime |
| M-2883 | dcc4 (Symbol Loop) |  | rcv.perf.sym.rateEst |
| M-2884 | dcc4 (Symbol Loop) |  | dc.perf.resid.symRate |
| M-2885 | dcc4 (Symbol Loop) |  | rcv.perf.sym.loopBandwidth |
| M-2886 | dcc4 (Symbol Loop) |  | rcv.config.sym.loopType |
| M-2887 | dcc4 (Symbol Loop) |  | rcv.perf.sym.currentWindowWidth |
| M-2888 | dcc4 (Symbol Loop) |  | dc.perf.autom.symLockStatus |
| M-2889* | dcc4 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2890* | dcc4 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2891* | dcc4 (Symbol Loop) |  | dc.perf.autom.symLockStatusTime |
| M-2892 | dcc4 (Symbol Loop) |  | rcv.perf.sym.loopStatus |
| M-2893 | dcc4 (Symbol Loop) |  | rcv.perf.sym.snrEst |
| M-2894 | dcc4 (Symbol Loop) |  | dc.perf.resid.symSnr |
| M-2895 | dcc4 (Symbol Loop) |  | rcv.perf.sym.spe |
| M-2896 | dcc4 (Symbol Loop) |  | rcv.config.sym.format |
| M-2897 | dcc4 (Symbol Loop) |  | rcv.config.sym.smoothing |
| M-2898 | dcc4 (Symbol Loop) |  | rcv.perf.sym.ratePred |
| M-2899 | dcc4 (Symbol Loop) |  | rcv.config.symAcq.useFft |
| M-2900 | dcc4 (Symbol Loop) |  | rcv.perf.state.symFft |
| M-2907 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount1 |
| M-2908 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount2 |
| M-2909 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount3 |
| M-2910 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount4 |
| M-2911 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount5 |
| M-2912 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount6 |
| M-2913 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount7 |
| M-2914 | dcc4 (Telemetry) |  | tlm.perf.fmt.vcCount8 |
| M-2915 | dcc4 (Telemetry) |  | tlm.config.fan.frameMode |
| M-2916 | dcc4 (Telemetry) |  | tlm.perf.gen.rrpArrayStatus |
| M-2917 | dcc4 (Telemetry) |  | tlm.config.fs.mode |
| M-2918 | dcc4 (Telemetry) |  | tlm.config.fs.pattern |
| M-2919 | dcc4 (Telemetry) |  | tlm.config.fs.patternLength |
| M-2920 | dcc4 (Telemetry) |  | tlm.perf.fs.predictedBitRate |
| M-2921 | dcc4 (Telemetry) |  | tlm.perf.fs.dataRate |
| M-2922 | dcc4 (Telemetry) |  | tlm.perf.gen.tlpLockStatus |
| M-2924 | dcc4 (Telemetry) |  | tlm.perf.gen.chanStatus |
| M-2925* | dcc4 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2926* | dcc4 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2927* | dcc4 (Telemetry) |  | dc.perf.autom.decoderStartTime |
| M-2928* | dcc4 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2929* | dcc4 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2930* | dcc4 (Telemetry) |  | dc.perf.autom.decoderLockStatusTime |
| M-2931 | dcc4 (Telemetry) |  | tlm.perf.gen.decoderLockStatus |
| M-2933 | dcc4 (Telemetry) |  | tlm.config.gen.decoderType |
| M-2934 | dcc4 (Telemetry) |  | tlm.perf.gen.decoderSNR |
| M-2936 | dcc4 (Telemetry) |  | tlm.config.b2cd.ilThreshold |
| M-2937 | dcc4 (Telemetry) |  | tlm.config.b2cd.vecSet |
| M-2938 | dcc4 (Telemetry) |  | tlm.config.b3cd.ilThreshold |
| M-2939 | dcc4 (Telemetry) |  | tlm.config.b3cd.vecSet |
| M-2940 | dcc4 (Telemetry) |  | tlm.config.fs.frameLengthPrimary |
| M-2941 | dcc4 (Telemetry) |  | tlm.config.fs.ilBitErrorThreshold |
| M-2942 | dcc4 (Telemetry) |  | tlm.config.fs.oolBitErrorThreshold |
| M-2943 | dcc4 (Telemetry) |  | tlm.config.fs.ilThreshold |
| M-2944 | dcc4 (Telemetry) |  | tlm.config.fs.oolThreshold |
| M-2945* | dcc4 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2946* | dcc4 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2947* | dcc4 (Telemetry) |  | dc.perf.autom.fsStartTime |
| M-2948* | dcc4 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2949* | dcc4 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2950* | dcc4 (Telemetry) |  | dc.perf.autom.fsLockStatusTime |
| M-2951 | dcc4 (Telemetry) |  | tlm.perf.fs.lockStatus |
| M-2952 | dcc4 (Telemetry) |  | tlm.perf.fs.bitToAcquire |
| M-2953 | dcc4 (Telemetry) |  | tlm.perf.gen.blocksOutput |
| M-2955 | dcc4 (Telemetry) |  | tlm.perf.fs.frameTotalCount |
| M-2956 | dcc4 (Telemetry) |  | tlm.perf.fs.frameGoodCount |
| M-2957 | dcc4 (Telemetry) |  | tlm.perf.fs.frameBadCount |
| M-2958 | dcc4 (Telemetry) |  | tlm.perf.crc.crcLockStatus |
| M-2959 | dcc4 (Telemetry) |  | tlm.config.fan.pnDecode |
| M-2960 | dcc4 (Telemetry) |  | tlm.perf.crc.crcTotalFrameCount |
| M-2961 | dcc4 (Telemetry) |  | tlm.perf.crc.crcGoodFrameCount |
| M-2962 | dcc4 (Telemetry) |  | tlm.perf.crc.crcBadFrameCount |
| M-2963 | dcc4 (Telemetry) |  | tlm.perf.rsd.lockStatus |
| M-2964 | dcc4 (Telemetry) |  | tlm.perf.rsd.totalCodeWordCount |
| M-2965 | dcc4 (Telemetry) |  | tlm.perf.rsd.goodCodeWordCount |
| M-2966 | dcc4 (Telemetry) |  | tlm.perf.rsd.badCodeWordCount |
| M-2967 | dcc4 (Telemetry) |  | tlm.perf.rsd.frameErrorRate |
| M-2968 | dcc4 (Telemetry) |  | tlm.perf.rsd.codeWordErrorRate |
| M-2969 | dcc4 (Telemetry) |  | tlm.perf.fs.totalInLockFrameCount |
| M-2970 | dcc4 (Telemetry) |  | tlm.perf.fs.flywheelFrameCount |
| M-2971 | dcc4 (Telemetry) |  | tlm.config.dft.formatTable |
| M-2974 | dcc4 (Telemetry) |  | tlm.config.b2cd.oolThreshold |
| M-2975 | dcc4 (Telemetry) |  | tlm.config.b3cd.oolThreshold |
| M-2976 | dcc4 (Telemetry) |  | tlm.config.gen.diffDecoder |
| M-2977 | dcc4 (Telemetry) |  | tlm.config.b2cd.altSymInv |
| M-2978 | dcc4 (Telemetry) |  | tlm.config.b3cd.symSignInversion |
| M-2979 | dcc4 (Telemetry) |  | tlm.perf.b2cd.metricNormalization |
| M-2980 | dcc4 (Telemetry) |  | tlm.perf.b2cd.estimatedSymbolErrorRate |
| M-2981 | dcc4 (Telemetry) |  | tlm.config.b2cd.nodeSyncChange |
| M-2982 | dcc4 (Telemetry) |  | tlm.perf.b2cd.nodeSyncChangeCount |
| M-2983 | dcc4 (Telemetry) |  | tlm.perf.b3cd.nodeSyncPosition |
| M-2984 | dcc4 (Telemetry) |  | tlm.config.fs.frameLengthSecondary |
| M-2985 | dcc4 (Telemetry) |  | tlm.perf.fs.correctFrameLength |
| M-2986 | dcc4 (Telemetry) |  | tlm.perf.fs.selfTestResult |
| M-2987 | dcc4 (Telemetry) |  | tlm.config.fs.bitSequence |
| M-2988 | dcc4 (Telemetry) |  | tlm.config.fs.bitSlipWindow |
| M-2989 | dcc4 (Telemetry) |  | tlm.perf.fs.inLockBer |
| M-2990 | dcc4 (Telemetry) |  | tlm.perf.fs.syncDirection |
| M-2991 | dcc4 (Telemetry) |  | tlm.perf.fs.currentPolarity |
| M-2992 | dcc4 (Telemetry) |  | tlm.perf.fs.bitSlip |
| M-2993 | dcc4 (Telemetry) |  | tlm.perf.fs.polarityChange |
| M-2994 | dcc4 (Telemetry) |  | tlm.perf.fs.acquisitionCount |
| M-2995 | dcc4 (Telemetry) |  | tlm.config.fan.interleave |
| M-2996 | dcc4 (Telemetry) |  | tlm.config.fan.algorithm |
| M-2997 | dcc4 (Telemetry) |  | tlm.perf.rsd.virtualFill |
| M-2998 | dcc4 (Telemetry) |  | tlm.perf.rsd.symbolErrorRate |
| M-2999 | dcc4 (Telemetry) |  | tlm.perf.rsd.selfTestResult |
| M-3000 | dcc4 (Ranging) |  | rng.config.t0Trk |
| M-3003* | dcc4 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-3004* | dcc4 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-3005* | dcc4 (Ranging) |  | dc.perf.autom.correlValidTime |
| M-3006 | dcc4 (Ranging) |  | rng.perf.rangeLock |
| M-3007 | dcc4 (Ranging) |  | rng.perf.blocksSent |
| M-3008 | dcc4 (Ranging) |  | rng.perf.carBlocksSent |
| M-3009 | dcc4 (Ranging) |  | rng.perf.totalAcqCompleted |
| M-3010 | dcc4 (Ranging) |  | rng.perf.rangeConfidence |
| M-3011 | dcc4 (Ranging) |  | rng.perf.residRuValue |
| M-3012 | dcc4 (Ranging) |  | rng.perf.rangePwrToNoise |
| M-3013 | dcc4 (Ranging) |  | rng.perf.pwrNoiseResidual |
| M-3014 | dcc4 (Ranging) |  | rng.config.patternType |
| M-3015 | dcc4 (Ranging) |  | rng.config.rtlt |
| M-3016 | dcc4 (Ranging) |  | rng.config.clock |
| M-3017 | dcc4 (Ranging) |  | rng.config.last |
| M-3018 | dcc4 (Ranging) |  | rng.config.chop |
| M-3019 | dcc4 (Ranging) |  | rng.config.chopStrt |
| M-3020 | dcc4 (Ranging) |  | rng.config.t1Xmit |
| M-3021 | dcc4 (Ranging) |  | rng.config.t2Xmit |
| M-3022 | dcc4 (Ranging) |  | rng.config.t3Xmit |
| M-3023 | dcc4 (Ranging) |  | rng.config.drvn |
| M-3024 | dcc4 (Ranging) |  | rng.config.intTime |
| M-3025 | dcc4 (Ranging) |  | rng.config.clockDiv |
| M-3026 | dcc4 (Ranging) |  | rng.config.comp1Len |
| M-3027 | dcc4 (Ranging) |  | rng.config.comp1Def |
| M-3028 | dcc4 (Ranging) |  | rng.config.comp1Bool |
| M-3029 | dcc4 (Ranging) |  | rng.config.comp2Len |
| M-3030 | dcc4 (Ranging) |  | rng.config.comp2Def |
| M-3031 | dcc4 (Ranging) |  | rng.config.comp2Bool |
| M-3032 | dcc4 (Ranging) |  | rng.config.comp3Len |
| M-3033 | dcc4 (Ranging) |  | rng.config.comp3Def |
| M-3034 | dcc4 (Ranging) |  | rng.config.comp3Bool |
| M-3035 | dcc4 (Ranging) |  | rng.config.comp4Len |
| M-3036 | dcc4 (Ranging) |  | rng.config.comp4Def |
| M-3037 | dcc4 (Ranging) |  | rng.config.comp4Bool |
| M-3038 | dcc4 (Ranging) |  | rng.config.comp5Len |
| M-3039 | dcc4 (Ranging) |  | rng.config.comp5Def |
| M-3040 | dcc4 (Ranging) |  | rng.config.comp5Bool |
| M-3041 | dcc4 (Ranging) |  | rng.config.comp6Len |
| M-3042 | dcc4 (Ranging) |  | rng.config.comp6Def |
| M-3043 | dcc4 (Ranging) |  | rng.config.comp6Bool |
| M-3044 | dcc4 (Ranging) |  | rng.perf.drvidRuValue |
| M-3045 | dcc4 (Ranging) |  | rng.perf.currentMode |
| M-3046 | dcc4 (Ranging) |  | rng.perf.acquisitionTime |
| M-3047 | dcc4 (Ranging) |  | rng.perf.rangeRuValue |
| M-3048 | dcc4 (Ranging) |  | rng.config.tolerance |
| M-3300 | uwv |  | HEFRBLR |
| M-3301 | uwv |  | HEFAIRP |
| M-3302 | uwv |  | RAINBLR |
| M-3303 | uwv |  | AIRPRES |
| M-3304 | uwv |  | SXMITPOL |
| M-3305 | uwv |  | XXMITPOL |
| M-3306 | uwv |  | KXMITPOL |
| M-3307 | uwv |  | LXMITPOL |

*The monitor data in these channels have not yet been implemented by the originating DSN subsystem.  The next delivery of the subsystem should include these items.  Until that time, though, these channels will not appear in the 0158-Monitor SFDUs.

# Appendix B Guide to the 34M/70M Subsystems

Appendix B

Guide to the 34M/70M Subsystems

This appendix provides definitions for the subsystem acronyms used in Table A-2, 34M/70M Monitor Data, along with a brief description of what each subsystem does. Starting in October, 2002, the DSMS will begin upgrading and replacing equipment at each antenna in the 34M/70M network.  The new subsystems, referred to as NSP subsystems because they've been developed as part of the Network Simplification Project, will be replacing some of the old subsystems, referred to as legacy subsystems.  Other legacy subsystems will remain in service after the upgrade is complete.

The upgrade, also known as the NSP rollout, will occur on an antenna by antenna basis.  Once an antenna is upgraded, the monitor data will be from the NSP subsystems and the legacy subsystems which are not being replaced.  Before the upgrade the monitor data will be from the legacy subsystems.  So during the transition period you will need to be prepared to get monitor data from all the subsystems listed in Table A-2.  Once the upgrade is complete, the legacy subsystems which are being replaced will be obselete and you will no longer receive monitor data from those subsystems.  The first antenna is scheduled to go operational with the NSP equipment in December, 2002, and the upgrade will be complete in November, 2003.  The subsystem descriptions in this appendix will identify which subsystems are legacy and which are NSP.  For the legacy subsystems the description will indicate whether the subsystem will be replaced by an NSP subsystem.

| apa | Antenna Pointing Assembly. The apa is part of the Antenna Mechanical (ANT) Subsystem. The apa controls the antenna pointing for the 34M HEF and 70M antennas. The apa uses predicts or feedback from the receiver to point the antenna at the spacecraft. The apa is a legacy subsystem. It will eventually be replaced by the apc, but that won’t occur until 2004 or 2005 at the earliest. |
| --- | --- |
| apc | Antenna Pointing Controller. The apc is the antenna pointing controller for the Antenna Mechanical (ANT) Subsystem. The apc controls the antenna pointing for the 34M BWG antennas. The apc uses predicts or feedback from the receiver to point the antenna at the spacecraft. The apc is a legacy subsystem which will remain in service after the NSP rollout is complete. It is not being replaced by an NSP subsystem. Eventually, the apc will replace the apa at the 34M HEF and 70M antennas. |
| ce | Connection Engine. The connection engine is an assembly of the Network Monitor and Control subsystem, which is an NSP subsystem. The connection engine receives directives from the NMC operator and sends directives to other the subsystems, assigning them to the connection/link and configuring them to support a spacecraft pass. |
| cpa | Command Processor Assembly. The cpa is the subsystem controller for the DSCC Command (DCD) subsystem. The CPA receives the command data and passes it to the command modulation assembly, which generates the subcarrier and modulates the command data onto the subcarrier. The DCD is a legacy subsystem which will be replaced by the UPL. |
| dcc | Downlink Channel Controller. This is the Downlink Tracking and Telemetry (DTT) subsystem. The DTT consists of receiver, telemetry processor, and downlink ranging assemblies controlled by the dcc, which is the subsystem controller. The DTT demodulates the carrier, the subcarrier, and the symbol stream. It converts the symbols to bits and performs frame synchronization and decoding. It transmits the processed telemetry to the projects. It also performs ranging correlation and provides downlink carrier phase data for Doppler measurements. It is an NSP subsystem which will replace the BVR, DTM, and MDA subsystems. A single DTT provides all the processing capabilities for one downlink channel. Up to 4 DTTs can be assigned to a single connection/link to provide 4 downlink channels. In the 0158-Monitor data there are 4 sets of DTT channels defined, one set for each logical downlink channel. |
| etc | Exciter-Transmitter Controller. The etc is the subsystem controller for the Exciter-Transmitter (ETX) subsystem. The ETX receives the subcarrier, modulates the subcarrier onto the carrier, and transmits it to the spacecraft. The ETX is a legacy subsystem which will be replaced by the UPL. |
| mda | Metric Data Assembly. The MDA receives the downlink carrier frequency from the BVR and compares it to the predicted frequency, generating Doppler residuals. It also receives ranging data from the SRA (Sequential Ranging Assembly), packages it for output, and sends it out. This is a legacy subsystem which will be replaced by the DTT and the UPL. |
| rcc | Receiver Control Computer. The rcc is the subsystem controller for the Block V Receiver (BVR) subsystem. The BVR demodulates the carrier, the subcarrier, and the symbol stream and converts the symbols to bits. The BVR is a legacy subsystem which will be replaced by the DTT. |
| tgc | Telemetry Group Controller. The tgc is the subsystem controller for the DSCC Telemetry (DTM) subsystem. The DTM performs frame synchronization and decoding and sends the processed telemetry data to the projects. The DTM is a legacy subsystem which will be replaced by the DTT. |
| tx_hp | High Power Transmitter. The high power transmitter receives the subcarrier, and modulates it onto the carrier, and transmits it to the spacecraft. |
| ulc | Uplink Control. The ulc is the subsystem controller for the Uplink (UPL) subsystem. The UPL consists of the following assemblies, which are controlled by the ulc: the Command Control Processor (ccp), the Command Modulation Generator (cmg), the Exciter-Transmitter Controller (etc), and the Uplink Ranging Controller (urc). The UPL receives directives and command data from the end user, modulates the uplink carrier with the command data and ranging modulation, amplifies the modulated carrier for transmission, and provides uplink carrier phase data for Doppler measurement and ranging phase data for ranging measurement. There are currently two versions of the UPL subsystem. UPL-6167-OPBV2.0.1 (a.k.a. Delivery 2 or D2) is the first operational version. It is already deployed. UPL-6167-OPCV3.2.0 (a.k.a Delivery 3 or D3) will be deployed during the NSP rollout, replacing UPL-6167-OPBV2.0.1. Delivery 2 of the UPL consists only of the ccp and cmg assemblies. It is used with the legacy ETX subsystem. Delivery 3 adds the etc and urc assemblies, which replace the legacy ETX and the MDA subsystems. The UPL is an NSP subsystem which will replace the DCD, ETX, and MDA subsystems. |
| uwv | Antenna Microwave Subsystem. The antenna microwave subsystem consists of the feed equipment group and the low noise amplifiers. It controls the connections between the low noise amplifiers and the receivers. |