Mars Express Bistatic Radar Experiment

Operations Plan and Report

24 April 2005

Summary

=======

DSN Antenna:                                          14

Pass:                                                693

Orbit:                                              1631

Orbit Start Time:                    2005-04-24T07:46:30

HGA Pointing:                                   Inertial

Specular Condition (ERT):                          11:46

Target Latitude (deg N):                          13.84N

Longitude (deg E):                         99.07W

Rp (km):                                 3399.128

Incidence/Reflection Angle (deg):                  67.58

Slant Range (km):                                   2922

Slew Angle (deg):                                  44.91

Doppler (carrier, fd; Hz):                         27965

Doppler (echo, fr; Hz):                           -13568

Doppler Difference (fdd; Hz):                      41533

Earth-Mars Distance (m):                        2.143E11

Data Acquisition

================

This experiment was supported by DSS 14 staff at the Goldstone DSN complex.  The support team at JPL included Gene Goltz, Danny Kahan, and Dick Simpson.  In the NOPE area was Romy Paredes.

Gene Goltz misunderstood the start time for the activities and didn't show up until after the pre-calibration had been completed.  Since Danny and I had just returned from the MaRS Team meeeting in Munich and neither of us had received the briefing message, there was some initial scrambling to get basic information into place.  Danny set up the voice net with the Comm Chief while I negotiated with Romy to get the briefing message.  MEX MOC wanted to know whether DSS 14 would support telemetry after the bistatic radar; this had been the subject of some debate the previous week, and I was hoping that Gene would have the answers.  After about 10 minutes MEX MOC withdrew the request, and there was agreement that DSS 14 would support BSR only.  The spacecraft did begin to transmit data after the BSR, however.

The experiment began with pre-calibration activities starting at 07:50; the pre-cal was completed 45 minutes ahead of schedule.  The bistatic surface observations were collected in the interval 11:36-11:49, but echoes were seen during only about two minutes centered on 11:46.  At 12:19 the spacecraft returned to Earth point and was one-way with no telemetry modulation for about half an hour.  At about 12:50 the S-band transmitter was turned off and X-band telemetry modulation was turned on.  The track ended about 10 minutes later, and the post-calibration was completed before 14:00.

Jose Valencia had adapted the calibration procedure and briefing message so that it was appropriate for DSS 14 rather than DSS 43, which had been used for the previous three experiments.  There were occasional, minor problems relating to our inability to see clearly when the noise diodes were turned on and off.  During the post-calibration, it also appeared that the S-LCP channel was unstable; levels fluctuated by a factor of two whether the noise diode was on or off.  We attempted to confirm the S-LCP levels by cycling the noise diode one more time after the procedure was completed but without success; the fluctuations continued.  There were no companion fluctuations on S-RCP.  Neither of these problems requires changes to the procedures.

RSRs were configured as follows.

Table 1

---------------------------------------------

RSR  Channel Mode  ATT FGAIN    Operator

dB

----- ------- ---- ---- ----- ---------------

RSR1A  S-LCP   1-W auto   50    Kahan

RSR1B  X-LCP   1-W auto   50    Kahan

RSR2A  S-RCP   1-W auto   50    Kahan/Goltz

RSR2B  X-RCP   1-W auto   50    Kahan/Goltz

Subchannels were defined as follows:

Table 2

---------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps   Used for seeking weak carrier signals

2          8 ksps

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording (able to capture both X-band

residual carrier and surface echo)

Predicts were provided by NSS, but the first time was 10:42, so the RSR's used their default tuning during the pre-calibration period..

Table 3 lists ADC RMS/peak levels read from RSR displays during the pre-calibration.  ADC peak values varied too much to be useful in confirming receiver configuration; a different system was employed for the surface observations and post-calibration.  Times are in UTC read from a wall clock and may not reflect actual times when configurations changed.  Acronyms and abbreviations are explained after Table 4.

Table 3

------------------------------------------------------------------------------

TIME (ERT)  RSR1A  RSR1B  RSR2A  RSR2B                COMMENTS

2005-04-10  S-LCP  X-LCP  S-RCP  X-RCP

RMS/pk RMS/pk RMS/pk RMS/pk

---------- ------ ------ ------ ------ ---------------------------------------

08:00                                 Voice net established (MEX MOC, MEX

NOPE, DSS 14, and RSSG)

08:10                                 MEX MOCC asks whether DSS 14 will

support telemetry; answer, after

investigation, is no.

08:37     29     29     29     29     All channels to sky; set RSR

attenuators using AUTO feature

08:43            88            91     X-band to AMB

08:45     79                          S-LCP to AMB

08:47                   86            S-RCP to AMB

08:48                                 DSS 14 reports ambient load physical

temperatures:

S1=13.52C,

S2=19.75,

X1=15.56

Sky overcast,

T=10.4C,

Humidity = 47.1%

08:49     29/114 28/127 29/118 30/122 Reset attenuators using AUTO feature

08:50                                 Start RSR recording, subchannel 3

08:56     29/113 28/127 30/118 30/127 All channels to AMB

09:00                          30/113 X-RCP ND ON

09:07     29/120  7/28  30/127  9/37  X-band to sky

09:10     29/109  9/36  30/124  7/27  Disable X-RCP ND

09:12                                 DSS 14 confirms X-RCP ND is OFF

09:15     29/101  9/40  30/110  7/30  Enable X-LCP ND

09:20     29/109 28/109 30/111 29/127 X-band to AMB

09:22                                 DSS 14 reports ambient load physical

temperatures:

S1=17.50,

S2=19.69,

X1=16.56

09:25     29/126 28/101 30/121 29/124 Disable X-LCP ND

09:30     28/120  7/25  29/127  7/27  X-band to sky

09:35     28/108  7/33  31/110  7/30  S-RCP ND ON

09:40     29/114  7/29  10/48   7/28  S-RCP to sky

09:45     29/116  7/26   8/28   7/28  Disable S-RCP ND

09:50     29/107  7/29   8/33   7/28  S-LCP ND ON

09:55     10/44   7/26   8/30   7/29  S-LCP to sky

10:00      8/33   7/27   8/33   7/28  Disable S-LCP ND

10:01                                 DSS 14 reports ambient load physical

temperatures:

S1=17.50,

S2=19.75,

X1=16.44

10:05                                 Stop RSR recording

Table 4 lists ADC RMS and ADC "amplitude" (dB) levels read from RSR displays during the bistatic observations (including the mini-calibrations) and the post-calibration.  Times are in UTC and may not reflect actual time when configurations changed.  Acronyms and abbreviations are explained below.

Table 4

------------------------------------------------------------------------------

TIME (ERT)   RSR1A    RSR1B    RSR2A    RSR2B             COMMENTS

2005-04-10   S-LCP    X-LCP    S-RCP    X-RCP

RMS/ampl RMS/ampl RMS/ampl RMS/ampl

---------- -------- -------- -------- -------- -------------------------------

10:58                                         Enter X-band FRO = -44000 Hz;

Enter S-band FRO =  -7000 Hz

10:59     10/-18.7  9/-19.6 10/-19.2 10/-19.0 Begin RSR recording subch 3

11:00                                         Spacecraft enters eclipse

11:00     10/-18.7  9/-19.6 10/-19.2 10/-19.0 Begin mini-calibration #1;

Enable X-RCP ND

11:04     10/-19.2  9/-19.7 12/-17.7 10/-19.2 Disable X-RCP ND;

Enable S-RCP ND

11:07     10/-19.2 11/-18.5 9/-19.7  10/-19.5 Disable S-RCP ND;

Enable X-LCP ND

11:10     12/-17.5 9/-20.0  9/-19.7  9/-19.6  Disable X-LCP ND;

Enable S-LCP ND

11:13     10/-19.0 9/-20.1  9/-19.8  9/-19.8  Disable S-LCP ND

11:15                                         Spacecraft begins Slew #1

11:18                                         Spacecraft occultation ingress

11:16                                         Spacecraft S-band TX ON (CW)

11:20                                         Spacecraft exits eclipse;

Spacecraft periapsis

11:24                                         Spacecraft occultation egress;

Spacecraft end Slew #1

11:30     10/-19.0 8/-20.6  9/-20.1  9/-20.1

11:32                                         Spacecraft X-band TX ON

11:34                                         Spacecraft X-band TLM ON

11:35                                         Begin RSR recording,

subchannel 4 (100 kHz)

11:36                                         Spacecraft X-band TLM OFF (CW)

11:46                                         Echoes peak in X-RCP at approx

28 dB SNR, drifting toward

positive frequencies; visible

but weaker (20 dB) in X-LCP.

Nothing seen in either S-band

channel.

11:49                                         Spacecraft begin Slew #2

11:55                                         End subchannel 4 recordings

11:59     10/-19.2 8/-21.0  9/-21.0  8/020.6  Setting up for Mini-Cal #2

12:00     9/-20.0  8/-21.0  8/-20.5  10/-18.7 X-RCP ND ON

12:04     9/-20.0  8/-21.2  11/-18.2 8/-20.6  Disable X-RCP ND;

Enable S-RCP ND

12:07     10/-19.3 10/-19.4 9/-20.5  8/-20.6  Disable S-RCP ND;

Enable X-LCP ND

12:10     11/-18.2 8/-21.2  8/-20.5  8/-20.8  Disable X-LCP ND;

Enable S-LCP ND

12:11     9/-19.6  8/-21.2  8/-20.5  8/-20.8  S-RCP carrier briefly visible

12:13                                         S-LCP ND OFF; end Mini-Cal #2

12:18                                         Set all FRO = 0;

S-LCP SNR=19 dB at -1322 Hz;

X-LCP SNR=37 dB at -2856 Hz

12:19                                         Spacecraft end Slew #2

12:19                                         X-RCP clipping; set FGAIN=70.

12:21                                         S-RCP SNR=47 dB at -1325 Hz

X-RCP SBR=69 dB at -2860 Hz

12:30                                         DSS 14 locks to S,X carriers at

request of MEX NOPE

12:35     13/-16.9 8/-21.5  8/-20.8  8/-20.8  S-LCP SNR=19 dB at -1334 Hz

X-LCP SNR=37 dB at -3000 Hz

S-RCP SNR=47 dB at -1335 Hz

X-RCP SNR=69 dB at -2898 Hz

12:49                                         S-band LOS (Tx OFF)

12:51     10/-19.4 7/-21.5  8/-20.7  8/-20.8  X-RCP SNR=69 dB at -2846 Hz

12:53                                         Spacecraft X-band TLM ON

12:54     10/-19.2 8/-21.6  8/-20.7  8/-20.7  X-RCP SNR=62 dB at -2850 Hz

13:02                                         DSS 14 moving to zenith; X-band

carrier disappears

13:03                                         X-RCP FGAIN set to 50

13:06     10/-19.5 7/-22.2  8/-21.2  7/-22.1  Prepare for post-calibration

13:13     29/-9.9  27/-10.3 20/-9.7  30/-9.5  All channels to AMB; DSS 14

reports ambient load physical

temperatures:

S1=16.31,

S2=18.50,

X1=14.90

13:17     28/-10.0 28/-10.3 29/-9.8  30/-9.5  Enable X-RCP ND

13:20     29/-9.9  27/-10.3 30/-9.5  30/-9.7  Disable X-RCP ND;

Enable S-RCP ND

13:23     29/-10.0 28/-10.0 28/-9.7  28/-9.7  Disable S-RCP ND;

Enable X-LCP ND

13:26     29/-9.7  28/-10.2 29/-9.8  29/-9.7  Disable X-LCP ND;

Enable S-LCP ND

13:29     11/-18.1 7/-22.3  8/-21.1  7/-22.2  All channels to sky

13:32       ???    9/-20.1  10/-18.7 7/-22.1  Disable S-LCP ND;

Enable X-LCP ND;

S-LCP unstable (rms varying

from 10 to 15)

13:35     12/-17.4 7/-22.3  11/-18.6 7/-22.1  Disable X-LCP ND;

Enable S-RCP ND

13:38     11/-18.0 7/-22.3  8/-21.4  9/-19.8  Disable S-RCP ND;

Enable X-RCP ND

13:41       ???    7/-22.3  8/-21.4  7/-22.1  Disable X-RCP ND;

S-LCP rms levels range from 9

to 12.

DSS 14 reports ambient load

physical temperatures:

S1=16.12,

S2=18.19,

X1=14.62.

Sky overcast,

T=8.3C,

humidity=59.3%

13:47     10/-18.7 7/-22.3  8/-21.2  7/-22.2  DC02 reconfigured for S-LCP

13:48     12/-17.2 7/-22.4  8/-21.3  7/-22.1  S-LCP ND ON

13:50     17/-14.4 7/-22.3  8/-20.6  7/-22.1  S-LCP ND OFF

13:52                                         Stop RSR recordings

AMB = ambient load

BOT = Beginning of Track

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Original: 2005-02-27

Summary section added: 2006-04-12

Summary section updated: 2006-05-19