Mars Express Bistatic Radar Experiment

Operations Plan and Report

01 October 2005

Summary

=======

DSN Antenna:                 14

Pass:                        0853

Orbit:                       2202

Orbit Start Time:            2005-10-01T04:12:25

HGA Pointing:                Inertial

Specular Condition:          2005-10-01T08:34:00 ERT

Target:                      (49.65N, 118.82E, 3381225 m)

Incidence/Reflection Angle:  60.57 deg

Slant Range:                 5649 km

Slew Angle:                  58.86 deg

Doppler (carrier, fd):       -60321 Hz

Doppler (echo, fr):          -56381 Hz

Doppler Difference (fdd):    -3940 Hz

Earth-Mars Distance:         7.840E10 m

Experiment Set Up

=================

This experiment was conducted using Goldstone's DSS 14.  The support team at JPL included Gene Goltz, Danny Kahan, and Dick Simpson.  In the NOPE area was Greg Hewitt.

Performance Problems

====================

As has been the case on previous experiments at DSS 14 since July, there were problems with the pre-calibration on X-LCP.  These were later traced to a typo in the procedure which erroneously connected the X-band ambient load to the X-RCP receiver and then to the X-LCP I/F.  During the pre-cal for this experiment, we repeated the noise diode disable command and successfully returned the X-LCP channel to its proper configuration before the pre-cal was completed.  The NOPE requested a DR from the station (G106121).

Also as on recent DSS 14 observations, the S-LCP channel seemed less stable than the other three in the sense that ADC amplitudes jumped around when constant levels were expected and it was harder to detect changes in noise diode status.

During analysis of the data it became apparent that there were serious interference problems on the X-RCP channel during this experiment.  Although the ground antenna elevation was more than 50 degrees (and increasing) throughout the BSR observation, the noise level increased by 5 percent at about 08:24 and by another 20 percent at about 08:29.  There were also a number of spurs starting at about 08:29, some moving quickly and others not.  A check of our MGS radio occultation data collection showed that MGS emerged from occultation at about 08:24:27 and its telemetry was turned on at about 08:28:41, explaining both changes in the noise level.  Since the noise was "normal" during the interval 08:14-08:28, we can use that as our reference for determining X-RCP system temperature.  Using 1024-point power spectra we have identified a frequency window that can serve as a noise power reference during the time when we were collecting surface echoes -- bins 809:908 (19714-22133 Hz) are relatively stable (albeit elevated by about 25 percent) during the time interval 08:28:15-08:41:30.  The surface echo power can be measured with some confidence during 08:31:30-08:36:00 in a sliding window that starts at bins 390:524 and ends at bins 533:667.  The surface echo overlaps the residual carrier during approximately 08:30-08:31.  There is no hint of interference in the X-LCP channel or in either of the S-band channels.

There is a 28 Hz increase in the S-Band carrier frequency at 08:39:45.

Data Acquisition

================

The set up period began at 04:00.  DSS 14 announced that it was ready to begin the pre-calibrations about 10 minutes later.

Except for the X-LCP noise diode problem and the S-LCP stability problem noted above, the pre-calibration was nominal.

RSRs were configured as in Table 1:

Table 1

---------------------------------------------

RSR  Channel Mode  ATT FGAIN    Operator

dB

----- ------- ---- ---- ----- ---------------

RSR3A  S-LCP   1-W auto   60    Kahan

RSR3B  X-LCP   1-W auto   60    Kahan

RSR2A  S-RCP   1-W auto   68    Goltz

RSR2B  X-RCP   1-W auto   60    Goltz

RSR subchannels (SCHAN) were defined as follows:

Table 2

---------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps

2          8 ksps

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Where available, signal-to-noise ratios, frequency offsets, and other measurements are also listed. Times are in UTC read from a wall clock and may not reflect actual times when configurations changed.  "Steps" are as defined in the briefing message.  Acronyms and abbreviations are explained after Table 3.

Table 3

------------------------------------------------------------------------------

TIME (ERT)  RSR3A  RSR3B  RSR2A  RSR2B                COMMENTS

2005-10-01  S-LCP  X-LCP  S-RCP  X-RCP

ADC    ADC    ADC    ADC

(dB)   (dB)   (dB)   (dB)

---------- ------ ------ ------ ------ ---------------------------------------

End-to-End Configuration Check

04:10       -9.3  -10.3   -9.9  -10.1 Step 01: DSS 14 ready to proceed;

RSSG sets ATT usding AUTO feature

04:13                                 MEX MOC and NOPE agree on no telemetry

04:15       -2.7   -0.3   -0.9   -0.0 Steps 02, 03, and 04

04:17                                 RSSG requests return to Step 01

04:19      -10.6  -10.2   -9.9  -10.1 Step 01: All channels to sky

04:21      -12.4   -0.3   -9.9   -0.1 Step 02: X-band to AMB

04:22       -2.6   -0.3   -9.8   -0.0 Step 03: S-LCP to AMB

NOPE reminds DSS 14 to type OD's as in

briefing message and not to use time

stacking.

04:24       -2.2   -0.3   -0.4   -0.1 Step 04: S-RCP to AMB

Pre-Calibration

04:25                                 Ambient load temperatures:

S1 = 18.19C

S2 = 23.62C

X1 = 14.81C

Weather conditions:

T = 32C

P = 901 mbar

H = 13%

Sky partly cloudy

04:25       -9.7  -10.1  -10.4  -10.0 Reset all ATTEN using auto feature

21.5   16.0   15.5   23.0 ATTEN settings

04:30:01                              All RSR's recording 25 kHz

04:33      -10.0  -10.0  -10.3  -10.0 Step 05: Configure ND controllers

04:36       -9.7   -9.9  -10.2   -9.8 Step 06: XR ND on

04:41       -9.7  -22.1  -10.3  -20.1 Step 07: X-band to sky

04:47       -9.6  -22.2  -10.3  -22.6 Step 08: XR ND off

04:48       -9.7  -20.8  -10.3  -22.6 Spontaneous X-LCP change;

DSS 14 confirms SNT D

04:51       -9.6  -19.8  -10.2  -22.6 Step 09: XL ND on

DSS 14 confirms 12.5K ND selected

04:56       -9.7  -10.6  -10.2   -9.9 Step 10: X-band to AMB (XL is too low)

Ambient load temperatures:

S1 = 18.00C

S2 = 23.25C

X1 = 15.12C

05:01       -9.8  -10.5  -10.3   -9.9 Step 11: XL ND off (no change in XL)

05:06       -9.6  -19.6  -10.1  -22.6 Step 12: X-band to sky (XL is too high)

05:11       -9.6  -22.1  -10.2  -22.4 Repeat Step 11: X-LCP ND now off

05:16       -9.7  -21.9  -10.0  -22.7 Step 13: SR ND on

05:21       -9.7  -21.9  -19.5  -22.6 Step 14: SR to sky

05:27       -9.7  -22.0  -22.0  -22.5 Step 15: SR ND off

DSS 14 confirms 12.5 ND selected

05:32       -9.4  -21.9  -22.1  -22.6 Step 16: SL ND on

05:37      -15.0  -22.0  -22.1  -22.7 Step 17: SL to sky

05:38      -18.8  -21.9  -22.1  -22.7 Unexpected change in S-LCP level

05:42      -20.2  -22.0  -22.1  -22.6 Step 18: SL ND off

Ambient load temperatures:

S1 = 18.94C

S2 = 23.19C

X1 = 17.00C

05:45                                 Stop RSR 25 kHz recording

Spacecraft Calibration (BCAL #1) and Slew #1

05:59                           -19.3 X-RCP signal acquired (ADC amplitude, dB)

69.0 Px/No

-2302 Offset from center (Hz)

06:41                                 DSS 14 reports X-band carrier in lock

07:05      -12.7  -21.0  -21.7    ?   X-band 3-way; S-LCP is very high

07:09      -10.0  -21.0  -21.6  -19.4 X-band, 1-way (ADC ampl, dB)

69.0 Px/No (dB)

-2669 Offset from center (Hz)

07:10                                 Begin RSR 25 kHz recording

07:11      -18.0  -21.0  -21.7  -19.5 S-band in lock

29     42     56     69  Px/No (dB)

-1280  -2697  -1279  -2695 Offset (Hz)

07:23      -17.4  -21.0  -21,5  -19.4 ADC amplitude (dB)

29     43     56     69  Px/No (dB)

-1325  -2867  -1332  -2888 Offset (Hz)

07:28      -17.4  -21.0  -21.2  -19.3 ADC amplitude (dB)

29     50     56     76  Px/No (dB)

-1367  -3017  -1361  -2998 Offset (Hz)

Mini-Cal #1

07:43                                 DSS 14 confirms DTT 01 and DTT 02

configurations for MiniCal #1

07:46                                 Begin Slew #1

07:47      -13.5  -21.0  -21.5  -19.5 Step 01: XR ND on (SL high)

07:50      -16.8  -21.0  -19.1  -21.4 Step 02: XR ND off, SR ND on

30     38     24     39  Px/No (dB)

-1447  -3310  -1447  -3305 Offset (Hz)

07:53      -17.6  -19.1  -21.3  -21.4 Step 03: SR ND off, XL ND on

17     24     24     37  Px/No (dB)

-2486  -3284  -1444  -3290 Offset (Hz)

07:56      -16.0  -21.0  -21.2  -21.6 Step 04: XL ND off, SL ND on

18     25     11     23  Px/No

-1400  -3260  -7920  -3250 Offset (Hz)

07:59      -17.0  -21.1  -21.7  -21.9 Step 05: SL ND off

20     25     13     21  Px/No (dB)

-1428  -3240  10171  -3238 Offset (Hz)

08:03                                 End Slew #1

Bistatic Radar Activities

08:04                                 Begin RSR SCHAN=4 recording (100 kHz)

08:28:30                              First echoes visible in X-LCP

08:30                                 Echoes in all channels

08:32      -19.2  -21.0  -21.9  -21.2 Echo, multiple peaks at X-band

27     37     28     37  Px/No (dB)

-600   -650   -610   -400 Offset (Hz)

08:34      -16.8  -20.9  -22.0  -21.2 Echo, 2 kHz wide at X-band

26     39     28     39  Px/No (dB)

-462    434   -400    550 Offset (Hz)

08:35      -18.0  -21.0  -22.0  -21.1 Echo

28     40     27     38  Px/No (dB)

-340   1163   -231   1355 Offset (Hz)

08:36      -18.9  -21.1  -22.1  -21.1 Echo fading, 2 peaks

25     20     26     28  Px/No (dB)

-251   2500    -27   3200 Offset (Hz)

08:41                                 X-band echo has 2 peaks at +3 and +7 kHz

08:45                                 Echoes gone

08:47      -16.8  -21.1  -22.0  -20.9 Carrier ADC amplitude (dB)

16     28            18  Px/No

-1356  -2970         -2966 Offset (Hz)

09:04                                 End RSR 100 kHz recording

Mini-Cal #2, Slew #2, and BCAL #2

09:04      -19.1  -21.6  -22.0  -19.2 Step 01: XR ND on

09:04:11                              Begin Slew #2

09:06                                 Interfering signal at upper edge of

XR spectrum moving at -5 kHz per minute

09:07      -19.9  -21.0  -19.7  -20.8 Step 02: XR ND off, SR ND on

09:08                                 Interference abruptly disappears at

center of XR passband

09:10      -19.5  -18.9  -22.1  -20.9 Step 03: SR ND off, XL ND on

09:13      -17.9  -21.0  -21.9  -20.8 Step 04: XL ND off, SL ND on

09:16      -19.1  -21.0  -22.0  -20.8 Step 05: SL ND off

29     41     33     44  Carrier Px/No (dB)

-1327  -2875  -1329  -2530 Carrier offiset (Hz)

09:22      -19.8  -21.0  -22.0  -19.0 Slew #2 complete

32     46     56     77  Carrier Px/No (dB)P

-1248  -2574  -1247  -2575 Carrier offset (Hz)

10:06      -20.0  -21.0  -22.0  -19.8 S-band TX off

77  Carrier Px/No (dB)

-2529 Carrier offset (Hz)

10:08                    -22.1  -19.7 X-band telemetry on

70  Carrier Px/No (dB)

-2524 Carrier offset (Hz)

Post-Calibration

10:16                                 DSS 14 going to zenith

10:25       -9.6   -9.4  -10.3   -9.9 Step 01: All channels to AMB at zenith

Ambient load temperatures:

S1 = 19.31C

S2 = 22.31C

X1 = 17.94C

10:27       -9.6   -9.5  -10.3   -9.7 Step 02: XR ND on

10:30       -9.5   -9.4  -10.0  -10.0 Step 03: XR ND off, SR ND on

10:33       -9.5   -9.2  -10.3  -10.0 Step 04: SR ND off, XL ND on

10:36       -9.4   -9.4  -10.4  -10.0 Step 05: XL ND off, SL ND on

10:39      -18.0  -21.6  -22.1  -22.6 Step 06: All to sky

10:42      -19.8  -19.3  -22.0  -22.5 Step 07: SL ND off, XL ND on

10:45      -19.2  -21.7  -19.5  -22.6 Step 08: XL ND off, SR ND on

10:48      -19.1  -21.6  -22.0  -20.1 Step 09: SR ND off, XR ND on

10:51      -17.9  -21.6  -22.1  -22.6 Step 10: XR ND off

Ambient load temperatures:

S1 = 19.25C

S2 = 22.31C

X1 = 17.69C

Weather conditions:

T = 19.6C

P = 899.8 mbar

H = 20.0%

Sky clear, winds calm

10:55                                 End of track

Stop RSR recording (SCHAN=3)

AMB = ambient load

BOT = Beginning of Track

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2005-10-01

Edits: 2005-12-29

Added notes on X-RCP interference: 2006-01-17