From JVelasco@jftl.jpl.nasa.gov Mon Dec  5 09:26:58 2005

Return-Path: <JVelasco@jftl.jpl.nasa.gov>

Received: from jftl.jpl.nasa.gov (jftl.jpl.nasa.gov [128.149.207.22])

by magellan.stanford.edu (8.11.7p1+Sun/8.11.7) with SMTP id jB5HQuF29210

for <rsimpson@magellan.Stanford.EDU>; Mon, 5 Dec 2005 09:26:56 -0800 (PST)

Received: from jftl.jpl.nasa.gov (Not Verified[128.149.207.21]) by jftl.jpl.nasa.gov with MailMarshal (v5,0,3,100)

id <B00033ec23>; Mon, 05 Dec 2005 09:28:01 -0800

Received: by jftl-mm.jpl.nasa.gov with Internet Mail Service (5.5.2657.72)

id <YGMDK57C>; Mon, 5 Dec 2005 09:26:28 -0800

Message-ID: <12373B687CF8324F843BE0CA103AA6F4068D98AC@jftl-mm.jpl.nasa.gov>

From: "Velasco, Jesse" <JVelasco@jftl.jpl.nasa.gov>

To: DSN-NOCC <dsn-nocc@jjpl.jpl.nasa.gov>,

"Asmar, Sami W" <asmar@mail.jpl.nasa.gov>,

"Goltz, Gene L" <ggoltz@mail.jpl.nasa.gov>,

DSN-PREDICTS <DSN-PREDICTS@jjpl.jpl.nasa.gov>,

DSN-RMDCT <DSN-RMDCT@jjpl.jpl.nasa.gov>,

"'John.Reynolds@esa.int'" <John.Reynolds@esa.int>,

"Kahan, Daniel S" <kahan@mail.jpl.nasa.gov>,

DSNO&M-DSOT <DSNO&M-DSOT@jftl.jpl.nasa.gov>,

"'Dick Simpson 650-723-3525'" <rsimpson@magellan.stanford.edu>,

"GDSCC Operations," <GDSCCOperations@gdscc.nasa.gov>

Cc: "Holmes, Dwight P" <dholmes@mail.jpl.nasa.gov>,

"Thompson, Thomas W" <twt@ampersand.jpl.nasa.gov>,

"Angold, Nigel G" <nangold@mail.jpl.nasa.gov>,

DSN-MPSETD <DSN-MPSETD@jjpl.jpl.nasa.gov>,

"'Gary.Whitehead-esa.int'" <Gary.Whitehead@esa.int>,

"'Alan.Moorhouse@esa.int'" <Alan.Moorhouse@esa.int>,

"Wallick, Jack" <JWallick@jftl.jpl.nasa.gov>,

"Landon, Arthur J" <ALandon@jftl.jpl.nasa.gov>,

"Berman, Allen L" <alberman@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Beyer, Patrick E" <pbeyer@mail.jpl.nasa.gov>,

"Williams, Raymond C" <rcw@mail.jpl.nasa.gov>,

"Medina, Margaret" <MMedina@jftl.jpl.nasa.gov>,

"Berner, Jeff B" <jbberner@mail.jpl.nasa.gov>,

"Guerrero, Anamaria P" <ana@mail.jpl.nasa.gov>,

"Buck, Robert D" <rbuck@mail.jpl.nasa.gov>,

"Klose, J C" <cklose@mail.jpl.nasa.gov>,

"Recce, David J" <drecce@mail.jpl.nasa.gov>,

"Marina, Miguel" <mmarina@mail.jpl.nasa.gov>,

"Hodgin, Wendy K" <whodgin@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Eaton, Curtis A" <ceaton@mail.jpl.nasa.gov>,

"Davis, Esker B" <eskerb@mail.jpl.nasa.gov>,

"Kurtik, Susan C" <skurtik@mail.jpl.nasa.gov>

Subject: BRIEFING MESSAGE FOR MARS EXPRESS BISTATIC RADAR SUPPORT DOY 344 DSS-14

Date: Mon, 5 Dec 2005 09:26:14 -0800

MIME-Version: 1.0

X-Mailer: Internet Mail Service (5.5.2657.72)

Content-Type: multipart/mixed;boundary="----_=_NextPart_000_01C5F9C0.FCA4F550"

Content-Length: 303769

Status: RO

Mars Express

Briefing Message

Level 3

Bistatic Radar Experiment Support

Prepared by: J Velasco / MEX / NOPE / MPSETD

Origination Date: DOY 334 / 23:00 UTC

Revision: Original

DSS-14   DOY 344 / 0210 UTC

ROUTING: JFTL / JJPL / JGTS

FROM: MPSETD J Velasco

SUBJECT: DSS-14 Mars Express Bistatic Radar Support

NOP: 871-049-041, REV.C

SCHEDULE: Refer to DSN 7-Day Schedule for actual support times.

DOY   SOA   BOT   EOT   EOA  FACILITY  USER      ACTIVITY      PASS

344  0210  0510  0810  0910   DSS-14    MEX  T/P BISTATIC R/S  0923

TYPE: Level 3 / Bi-static Radar Experiment Support

1. OVERVIEW:

Bi Static Radar (BSR) experiment employs the MEX spacecraft's transmitter

and a DSN 70-m antenna to receive weak S-band and X-band dual polarized RF

signals and Radio Science Receivers (RSRs) to simultaneously receive and

record these four signals.  At a predetermined time during the pass, the

spacecraft slews toward Mars so that its transmitted S-band and X-band RCP

signals illuminate the planet's surface.  As these RCP signals bounce off

the surface, part of the signals change polarization to Left Circular

Polarization (LCP) effectively creating four signals; 1) S-band RCP, 2)

S-band LCP, 3) X-band RCP and 4) X-band LCP.

DSN Mars Express Bistatic Radar (BSR) calibration procedures are performed

in three parts: 1) Pre-calibration, 2) Mini- calibration and 3)

Post-calibration. Section-8 details the pre-calibration procedures,

section-9 details mini-calibration procedures, and section-10 details post

calibration procedures.

2. SUPPORT DOCUMENTATION:

a. Mars Express NOP 871-049-041, Rev. C

b. This briefing message

c. Bi-static Radar SOE (see attachment) supersedes DSN SOE. Radio

Science shall provide start/stop times of BSR mini-pre-calibrations near

real-time.

3. SUPPORT REQUIREMENTS:

a. DOWNLINK SUPPORT ONLY

b. DTTS (DOWNLINK TRACKING AND TELEMETRY) RECEIVERS:

* One DTT shall be configured with table 7041HRX to support X-band, and to

control the 12.5k noise diode on/off at predetermined times during pre-pass

calibrations, real-time BSR activities and post calibrations activities.

* A second DTT shall be configured with table 7041HRXS to support S-band

carrier only and to control the 12.5 k noise diode on/off at predetermined

times during pre-pass calibrations, real-time BSR activities and post

calibrations activities.

c. UPLINK

* There are NO uplink requirements

d. COMMAND:

There are NO command requirements

e. RADIO SCIENCE RECEIVERS (RSRS):

NOTE: For remote operations the FSPR must not be assigned to the DSS link.

The RSRs are controlled and configured remotely by the Radio Science group

located at JPL.  The RSR operator will configure four radio science

receivers to simultaneously receive and record S-band and X-band RCP and LCP

polarized signals.  The RSR equipment requires both S-band and X-band 1-way

and 2-way downlink predicts.  The actual RSR resources configured for

support will depend on availability.

1. R1B IFS 14_S_RCP

2. R2A IFS 14_S_LCP

3. R1A IFS 14_X_RCP

4. R2B IFS 14_X_LCP

4. NOCT:

Transmit X-band up/down, and X-up/S-band down predicts to the station

5. GCF:

Voice: DSS /ROC MOPS/AMMOS/MEX MOC/ DSN OPS (RS)

6. DSOT

There is NO SLE telemetry return link services required

7. SPACECRAFT CONFIGURATION:

The spacecraft shall be reconfigured for X-band and S-band downlink carrier

only in accordance with the BSR SOE (see attachment)

8. PRE-CALIBRATION PROCEDURES AND CONFIGURATION

a. Pre-calibration requires configuring the 70m-antenna microwave subsystem

so that each receiving downlink channel (S-RCP, S-LCP, X-RCP, X-LCP) can

be cycled in sequence through the four configurations listed here:

1. Receiving channel into ambient load with noise diode off

2. Receiving channel into ambient load with noise diode on

3. Receiving channel into sky (zenith) with noise diode on

4. Receiving channel into sky (zenith) with noise diode off

NOTE: DTT01 is used for X-band and DTT02 is used for S-band, but any

available DTT resource can be used.

b. The pre-calibration is done with the antenna pointed to zenith

c. Link Controller perform the procedures in step-01 (below) and advise Radio

Science when completed; wait at this point until directed by Radio Science

to proceed to step-02.

Step-01  00:00:00  US14 MOD 102 A        Confirm dichroic reflector in place

US14 CNF XRA1O1       X-RCP (LNA-1) to Sky

US14 CNF XLA2O2       X-LCP (LNA-2) to Sky

US14 CNF SLLA2C2O2    S-LCP (LNA-2) low noise to Sky

US14 CNF SRDA1C1O1    S-RCP (LNA-1) via Diplexer to Sky

AP SCAN D             Conscan Off

AP TRK SR             Sub-reflector in Tracking Mode

* NOPE use the connection log display to verify DSS-14's microwave

switches are configured in accordance with step-1

*  Radio science operator set each RSR attenuator using the AUTO

feature. After checking for proper signal levels (about -10.0 dB

amplitude on each RSR), direct the link controller to proceed to step 2.

Step-02  00:04:00  US14 CNF XA1A2O2      LNA2 to ambient load-1 (X-LCP)

US14 CNF XA1A1O1      LNA1 to ambient load-1 (X-RCP)

Step-03  00:06:00  US14 CNF SA2A2C2O2    LNA2 to ambient load-2 (S-LCP)

Step-04  00:08:00  US14 CNF SA1A1C1O1    LNA1 to ambient load-1 (S-RCP)

* After steps 2, 3, and 4 (above) have been completed, perform

procedures given below in steps a through e:

a. Link Controller report ambient load temperatures (S1, S2, and X1) from

the PRFREA display and report current weather conditions, (temperature,

humidity, and sky conditions) to the Radio Science operator.

b. Radio Science operator set each RSR attenuator using AUTO feature

c. RSR operator run radio science recorders

d. RSR operator report to Link Controller when radio science recorders

are recording

e. Link controller execute the following procedures at 5 min intervals

and report the completion of each step to the RSR operator:

Step-05  00:10:00  DC01 DISC

DC01 CONN DSS14XO1    Configure DTT 01 to X-RCP

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC

DC02 CONN DSS14SO1    Configure DTT 02 to S-RCP

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-06  00:15:00  DC01 SNT E            Enable X-RCP noise diode

Step-07  00:20:00  US14 CNF XRA1O1       X-RCP to sky

US14 CNF XLA2O2       X-LCP to sky

Step-08  00:25:00  DC01 SNT D            Disable X-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:30:00  DC01 SNT E            Enable X-LCP noise diode

CONFIRM NOISE DIODE SELECTION IS 12.5

Step-10  00:35:00  US14 CNF XA1A1O1      X-RCP (LNA-1) to ambient load

US14 CNF XA1A1O2      X-LCP (LNA-1) to ambient load

00:36:00  Link Controller report ambient load temp (S1, S2, X1)

from the PRFREA display to the RS operator.

Step-11  00:40:00  DC01 SNT D            Disable X-LCP noise diode

Step-12  00:45:00  US14 CNF XRA1O1       X-RCP (LNA-1) to sky

US14 CNF XLA2O2       X-LCP (LNA-2) to sky

Step-13  00:50:00  DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-14  00:55:00  US14 CNF SRDA1C1O1    S-RCP (LNA-1) via diplexer to sky

Step-15  01:00:00  DC02 SNT D            Disable S-RCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

CONFIRM NOISE DIODE IS 12.5

Step-16  01:05:00  DC02 SNT E            Enable S-LCP noise diode

Step-17  01:10:00  US14 CNF SLLA2C2O2    S-LCP (LNA-2) to sky

Step-18  01:15:00  DC02 SNT D            Disable S-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Link Controller report ambient load temp (S1, S2, X1) from

the PRFREA display to the RS operator.

01:20:00  End of BSR pre-calibration procedures, configure for

real-time and wait for acquisition of signal

9. MINI CALIBRATION PROCEDURES AND TIMELINE

Link Controller, shortly after MEX begins its slew and before the BSR

surface observations begin, confirm DTT 01 and DTT 02 configurations.

a. Link Controller confirm DTT 01 is configured for X-RCP; if not,

perform the following operator directives:

* DC01 DISC

* DC01 CONN DSS14XO1

* DC01 SET SNT NDS=12.5

* DC01 SET SNT MFQ=ON

b. Link Controller confirm DTT 02 is configured for S-RCP; if not, perform

the following operator directives:

* DC02 DISC

* DC02 CONN DSS14SO1

* DC02 SET SNT NDS=12.5

* DC02 SET SNT MFQ=ON

c. RSR operator ensures all Radio Science Recorders are recording.

d. Link Controller, at direction from RSR operator begin the BSR

mini-noise calibration procedures (step-1 through step-6. The times shown

are to illustrate event chronology and event duration:

Step-01  00:00:00  DC01 SNT E            Enable X-RCP noise diode

CONFIRM NOISE DIODE VALUE IS 12.5

Step-02  00:03:00  DC01 SNT D            Disable X-RCP noise diode

DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

US14 D XMAP           Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-03  00:06:00  DC02 SNT D            Disable S-RCP noise diode

DC01 SNT E            Enable X-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

US14 D SMAP           Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-04  00:09:00  DC01 SNT D            Disable X-LCP noise diode

DC02 SNT E            Enable S-LCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

US14 D XMAP           Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-05  00:12:00  DC02 SNT D            Disable S-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

US14 D SMAP           Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

00:15:00                        End of BSR Mini-Noise Cal

Step-06             Link Controller, repeat the BSR mini-noise calibration

procedures steps 1-5 after MEX has completed its

surface observations and before it returns to Earth

point. The start time will be provided by radio science.

10. POST CALIBRATION PROCEDURES AND TIMELINE:

Link Controller, at direction from RSR operator begin post-calibration

procedures (step-01 through step-10). The times shown are to illustrate event

chronology and event duration:

Step-01  00:00:00  Link controller move antenna to zenith (do not use stow

command, sub-reflector is to remain tracking)

US14 CNF SA1A1C1O1    LNA1 to ambient load1 (S-RCP)

US14 CNF SA2A2C2O2    LNA2 to ambient load2 (S-LCP)

US14 CNF XA1A1O1      LNA1 to ambient load1 (X-RCP)

US14 CNF XA1A2O2      LNA2 to ambient load1 (X-LCP)

DC01 DISC             Configure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC             Configure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

a. Link Controller advise the RSR operator when the antenna is at

zenith and the configurations listed above have been completed.

b. Link Controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display.

c. RSR operator shall run all Radio Science Recorders and record

ambient loads for 3 min. At completion, advise the Link Controller to

proceed with the following steps:

Step-02  00:03:00  DC01 SNT E            Enable X-RCP noise diode

Step-03  00:06:00  DC01 SNT D            Disable X-RCP noise diode

DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-04  00:09:00  DC02 SNT D            Disable S-RCP noise diode

DC01 SNT E            Enable X-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-05  00:12:00  DC01 SNT D            Disable X-LCP noise Diode

DC02 SNT E            Enable S-LCP noise diode

Step-06  00:15:00  US14 CNF SRDA1C1O1    S-RCP (LNA-1) to sky

US14 CNF SLLA2C2O2    S-LCP (LNA-2) to sky

US14 CNF XRA1O1=      X-RCP (LNA-1) to sky

US14 CNF XLA2O2       X-LCP (LNA-2) to sky

Step-07  00:18:00  DC02 SNT D            Disable S-LCP noise diode

DC01 SNT E            Enable X-LCP noise diode

DC02 DISC             Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-08  00:21:00  DC01 SNT D            Disable X-LCP noise diode

DC02 SNT E            Enable S-RCP noise diode

DC01 DISC             Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:24:00  DC02 SNT D            Disable S-RCP noise diode

DC01 SNT E            Enable X-RCP noise diode

Step-10  00:27:00  DC01 SNT D            Disable X-RCP noise diode

00:30:00  End

a. Link Controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display and report the current

weather (temperature, humidity, and sky conditions) to radio science

b. Radio Science operator stop RSRs

c. End of BSR noise post-calibration procedures

d. Link Controller proceed with normal tear-down procedures

e. Re-initialization of the DTTs is recommended

MPSETD POINTS OF CONTACT: DSN-MPSETD@jjpl.jpl.nasa.gov

<mailto:DSN-MPSETD@jjpl.jpl.nasa.gov>

MPSETD         OFFICE      OPS WORK STATION

--------------------  --------------  ----------------

Jose Valencia (LEAD)  (626) 305-6308   (626) 305-6370

(626) 305-6368

Jesse Velasco (NOPE)  (626) 305-6315

Romy Paredes (NOPE)   (626) 305-6309

Greg Hewitt (NOPE     (626) 305-6292

RADIO SCIENCE WORK STATION  (818) 393-0666