DOCUMENT NUMBER:	PY-SSP-OU-UM-001

ISSUE DATE:			22 October 97

Revision Date:			12/7/06

HUYGENS

SURFACE SCIENCE PACKAGE

ON-BOARD SOFTWARE USERS MANUAL

ISSUE 2.2

|  |  | Rev | Date: |
| --- | --- | --- | --- |
| PREPARED BY: | D J Parker | 1.0 | 22/10/1997 |
| REVISED BY: | M R. Patel and L. R. Hobbs M. R. Leese M. R. Leese and B. Hathi | 2.0 2.1 2.2 | 13/08/2004 12/12/2005 12/07/2006 |

TABLE OF CONTENTS

1.	introduction	5

1.1	Administrative	5

1.2	SSP MEMORY CONFIGURATION	7

2.	software architecture	8

2.1	Overall Software structure	8

2.2	Boot Up	9

2.2.1	Boot Up Sequence	9

2.2.2	Use of EEPROM for Patching	9

2.3	Software Execution	11

2.3.1	Task Scheduler	11

2.3.2	Main Polling Loop	13

2.3.3	Main Processing	13

2.4	SSP Modes	14

2.4.1	Mode 0 or 8 (EMC)	14

2.4.2	Mode 1 (Upper Atmosphere)	14

2.4.3	Mode 2 (Mid Atmosphere)	15

2.4.4	Mode 3 (Lower Atmosphere)	15

2.4.5	Mode 4 (Proximity)	15

2.4.6	Mode 5 (Surface)	15

2.4.7	Mode 6 (Extended Surface)	16

2.4.8	Mode 7 (Diagnostic)	16

2.5	Proximity Mode and Impact Detection Algorithm	16

3.	data HANDLING	19

3.1	Mission Status - The Descent Data Broadcast	19

3.1.1	Mission Time	19

3.1.2	SSP Time	19

3.1.3	Mission Phase	19

3.1.4	Altitude	20

3.1.5	Spin	20

3.2	Packet Telemetry	21

3.2.1	Concept	21

3.2.2	Packet Headers	21

3.2.3	ACC-E	22

3.2.4	ACC-I	24

3.2.5	API-S	25

3.2.6	API-V	27

3.2.7	DEN	28

3.2.8	PER and CON	28

3.2.9	REF	28

3.2.10	THP	29

3.2.11	TIL	29

3.2.12	Housekeeping	30

3.2.13	Impact Packet	32

3.2.14	Engineering	33

3.3	SSP Status Words	34

3.4	Commands	36

3.4.1	Command Packet Format	36

3.4.2	SSP Command Format	36

3.4.3	SSP Commands	38

4.	SPACECRAFT INTERFACE HANDLING	51

4.1	Telecommands and DDBs	51

4.1.1	Command Handler	51

4.1.2	Descent Data Broadcasts	52

4.1.3	SSP Command Types	55

4.2	Use of Broadcast Pulse	56

4.3	Telemetry	56

4.3.1	Telemetry Handler	56

5.	SENSOR INTERFACE HANDLING	57

5.1	ACC-I	57

5.1.1	ACC-I Module	57

5.1.2	Sampling	57

5.1.3	Checkout	57

5.1.4	Atmospheric Modes	59

5.1.5	Proximity Mode	59

5.1.6	Surface Modes	60

5.2	DEN	60

5.2.1	Den Module	61

5.2.2	Sampling	61

5.2.3	Processing	61

5.2.4	Task Timeline	61

5.3	PER and CON	62

5.3.1	Per Module	62

5.3.2	Sampling	62

5.3.3	Processing	62

5.3.4	Task Timeline	63

5.4	REF	63

5.4.1	REF Module	63

5.4.2	Checkout	63

5.4.3	Upper Atmosphere	63

5.4.4	Mid and Lower Atmospheric Modes	64

5.4.5	Proximity Mode	66

5.4.6	Impact	66

5.4.7	Surface Modes	66

5.5	TIL	66

5.5.1	TIL Module	66

5.5.2	Sampling	67

5.5.3	Checkout	67

5.5.4	Processing	67

5.5.5	Operation - All Modes	67

5.5.6	Task Timeline	67

5.6	THP	68

5.6.1	THP Module	68

5.6.2	Sampling	68

5.6.3	Processing	69

5.6.4	Safety Mechanisms	69

5.6.5	Checkout	69

5.6.6	Upper Atmosphere Mode (1)	69

5.6.7	Mid Atmosphere Mode (2)	69

5.6.8	Low Atmosphere Mode (3)	69

5.6.9	Proximity Mode (4)	70

5.6.10	Surface Mode (5)	70

5.6.11	Extended Surface Mode (6)	70

5.6.12	Sample Timings	70

5.7	ACC-E	71

5.8	API-V	71

5.8.1	API-V Module	71

5.8.2	Checkout	71

5.8.3	Operation - All Modes	71

5.8.4	Sampling	72

5.8.5	Processing	72

5.8.6	Task Time	72

5.9	API-S	72

5.9.1	API-S Module	72

5.9.2	Checkout	72

5.9.3	Upper Atmosphere Mode	72

5.9.4	Mid and Lower Atmospheric Modes	73

5.9.5	Proximity Mode	74

5.9.6	Surface Mode	74

5.9.7	Extended Surface Mode	74

6.	routing bus	75

6.1	Overview	75

6.2	Routing Bus Handlers	75

6.2.1	Address Map	76

6.2.2	Low Level Handler	76

6.2.3	12 Bit Board Handler	76

6.2.4	Spacecraft Interface Handler	77

6.2.5	16 Bit Board Handler	78

# introduction

## Administrative

Scope

This document provides the flight software design for the Huygens Surface Science Package.  This document satisfies the requirement detailed in the Huygens Document Requirements Description.

Purpose

This document describes the design and internal functioning of the flight software and relates this information into the applicable requirements specifications(s). The description is presented in a top-down fashion beginning with the software subsystem as a whole and successively describing lower level components. The data and procedure structures as well as the operational data flows supported by these structures are explained. Software external interfaces and the module to module interfaces are described.  Provisions for error isolation and verification characteristics are given.

Applicable Documents

Huygens Experiment Interface Document Part A

PY-SSP-RAL-PL-0001		Huygens Surface Science Package Product Assurance Plan

PY-SSP-UKC-SP-005	Huygens Surface Science Package Software Requirements Document

MIL-STD-1750A		Sixteen-Bit Computer Instruction Set Architecture

BAe Low Power Processor User Manual

Document Conventions and Abbreviations

Flag			1 bit

Byte			8 bits

Word			16 Bits

Long Word		32 Bits

LSB			Least Significant Bit	      Note: Bit 0 is the LSB and bit 15 is the MSB of a

MSB			Most Significant Bit              word, this is opposite to the 1750a specification.

BCP			Broadcast Pulse

CDMS			Command Data Management System

EID			Experiment Interface Document

SSP			Surface Science Package

PDS			Planetary Database System

Sensors

Acc-E			External Accelerometer

Acc-I			Internal Accelerometer

Api-S			Acoustic Properties Instrument-Sounder

Api-V			Acoustic Properties Instrument - Velocity

Den			Density Subsystem

Per			Permittivity Subsystem

Ref			Refractometer Subsystem

Thp			Thermal Properties Subsystem

TIL			TILT Subsystem

HK			Housekeeping

System

ADC			Analogue to Digital Converter

DAC			Digital to analogue Converter

DRB			Data Routing Bus

DDB			Descent Data Broadcast Message

EEPROM		Electrically Erasable Programmable Read Only Memory

EPROM		Erasable Programmable Read Only Memory

FIFO			First in First Out

LPP			Low Power Processor

ML			Memory Load

PROM			Programmable Read Only Memory

RAM			Random Access Memory

SOS			Silicon On Sapphire

TM			Telemetry

Documentation

TBD			To be Determined

TBC			To be Confirmed

## SSP MEMORY CONFIGURATION

The diagram below shows the physical connection of SSP memory. Note that only PROM and RAM are directly connected to the processor bus, which enhances the reliability. The EEPROM, which can be updated by telecommand, is accessed via an I/O port.

Telemetry

Channel A

eeprom

BIPOLAR

PROM

SOS

RAM

L P P

Primary Routing Bus				        Processor Bus

Redundant Routing Bus

SOS

RAM

Telemetry

Channel B

On power up program is copied from PROM to RAM. The EEPROM is then checked to see if it contains any patches. If it does then the program in RAM is updated from EEPROM.

System reliability is also increased by the use of dual Data Routing Busses. Each bus is connected to half of the sensor conditioning circuits, and one spacecraft interface. The EEPROM is on one bus only.

# software architecture

## Overall Software structure

The diagram below shows the main modules of the SSP code. There are two phases, Boot and Normal Operation. During normal operation synchronous tasks are scheduled by the Task Handler while others are scheduled in sequence by the Main Polling Loop.

FIRST BOOT

initialisation

TASK HANDLER

MAIN POLLING

LOOP

Commands

Telemetry

PATCH

SECOND BOOT

MISSION TIME

BCP HANDLER

WATCHDOG

## Boot Up

### Boot Up Sequence

On power-up SSP enters a boot sequence as shown below. During most of the steps in this sequence, a value is written to the Mode field of the SSP Status Word, which can be a useful diagnostic aid.

Step		Mode				Action

1		N/A		Save stack pointer for diagnostics

2		N/A		Set up stack pointer

3		N/A		Save registers RO-R14 for diagnostics

4		N/A		Clear all of high ram area

5		N/A		Disable Interrupts

6		N/A		Initialise routing bus

7		0		Initialise parallel I/O ports

8		0		Run initialisation patch (if present)

9		1		Check Ram

11		2		Copy PROM Code to RAM

12		3		Turn 12V Converter A on

13		4		Start Task Scheduler

14		5		Start watchdog and BCP handlers

15		6		Reset command handler and synchronise DDB to BCP

16		0		Wait for 10 seconds to allow EEPROM override

commands

17		1		Read patches from EEPROM

18		2		Initialise telemetry handler

19		2		Still going flag toggled

20		1		Set current SSP mode in status word (nominally 1)

21		1		Jump to main polling loop.

### Use of EEPROM for Patching

The EEPROM is intended to hold updates to the baseline SSP flight software.  The updates are held as individual patches.  These patches are read into memory during initialisation.

There are two types of patches:

a)  The initialisation patch which is loaded into memory as soon as the processor is able to access the EEPROM and then the patch is executed.  The function of this patch is to be able to rectify problems that occur during the initialisation phase of the SSP s/w.  The patch can either make small modifications to the flight s/w or can replace entire sections of the flight s/w.

b) The second type of patch is loaded once the flight s/w has executed its initialisation sequence and has been copied from PROM to RAM.  There is a ten second delay between initialising the command handler and command FIFO, and loading the patches to allow the patch loading to be overridden by a telecommand.

There can be several patches each of which contains modification to the flight s/w or additions to it.  In each case the patch is read from the EEPROM into spare memory where the patch CRC is checked.  If the patch CRC is OK then the patch is copied to the appropriate memory location.

The EEPROM Memory Map is as follows:

| Address | Label | Comments |
| --- | --- | --- |
| 0 | EE-Patch List | Start address of patch list |

The patch List Structure is as follows:-

| Offset (Words from EE-PatchList) | Field | Comments |
| --- | --- | --- |
| 0 | PatchNumber | Number of patches in list |
| 1 | Address Patch 1 | EEPROM address of 1st patch |
| 2 | Address patch 2 | EEPROM address of 2nd patch |
| : | : | : : : |
| n | Address patch n | EEPROM address of last patch |

The structure of a patch is as follows:-

| Offset | Field | Comments |
| --- | --- | --- |
| 0 | Length | Length of patch data including CRC |
| 1 | Memory Address | Address to write patch to when loaded from EEPROM |
| 2 | Patch Data | 1st Word of Patch Data |
| 3 | Patch Data | 2nd Word of Patch Data |
| : | : : | : : : |
| Length+1 | Patch Data | Last Word of Patch Data |
| Length+2 | CRC | CRC |

These commands associated with patches:-

Write Patch	Writes patch data to the EEPROM, the patch CRC is verified before the patch is written.

Read Patch	Reads a patch and transmits the patch data via the engineering datastream.

Delete Patch	Deletes a patch by clearing the patch address in the patch list.

Clear EEPROM 	Can be used to remove all patches.

## Software Execution

After boot up, the SSP software performs some initialisation then enters its normal execution.  Tasks are divided into two categories: Time critical and not so time critical.  The critical ones are executed under interrupt control via a 'Task Scheduler'.  The others run in a sequential fashion via the 'Main Polling Loop' during the gaps allowed by the task scheduler.

### Task Scheduler

Module 	TK.

Directory 	KERNEL

Files:	Code		Task.ASM

Variables	Task.VAR

Constants	Task.COM

Macros		Task.MAC

Make		Task.MAK

The task scheduler enables a variety of data acquisition tasks that have different sampling strategies to be serviced whilst still maintaining accurate timing. A task has certain requirements:

i)	The task has to be run at pre-defined intervals e.g. a complete sample every 10 seconds.  This has been defined as the cycle time.

ii)	The task consists of a number of functions that have to be carried out in a certain order e.g. initialise sensor, take n readings, process data, transmit data.

iii)	Each function may have to be run at a specific time e.g. 10 readings taken at 1ms intervals.

The task scheduler uses Processor Timer A to generate an interrupt every 1ms. When the scheduler is activated by the interrupt it scans the current task table.

Each entry in a task table consists of 4 fields.

TK-Task Address		The address to start executing the task

TK-Counter			The number of timeslices used from the current cycle

TK-Time Slices Used	The number of timeslices left before the task is rescheduled

TK-Cycle	The cycle time of the task in timeslices

For each entry in the task table the scheduler will decrement the counter TK-Counter.  If the counter becomes 0 then the code at the address in TK-Task Address is executed. The task runs until it hands back control to the Task Scheduler.  The task is able to use registers RO-R6 as they are saved by the Timer A interrupt handler.  Register R6 holds the address of the current task entry in the task table.  All remaining registers should be preserved.

A task can hand back control to the task scheduler via one of several macros:

TK-Pause xx	Suspends execution of the task for xx timeslices.  The value xx is loaded into TK-Counter and added to TK-Time Slices Used.  Control is then handed back to the task scheduler.

TK-Wait Rx	Same as TK-Pause except the number of timeslices to suspend execution for is held in register Rx.  Only registers R0-R5 can be used.

TK-Cycle	Suspends execution of the task until a known time from the time the current task cycle was started.  The number of time slices used since the start of the task, TK-TimeSlicesUsed is subtracted from the overall cycle time TK-Cycle.  The result is then located into the task counter TK-Counter.  The number of time slices used.  TK-TimeSlicesUsed is set to 0.

TK-LongWait Rx Ry-	Suspends execution of a task for longer than 65536 timeslices.  This is similar to TK-Wait except Rx and Ry hold the 1sw and msw of the delay time respectively.  TK-Counter is used as before being loaded with 1sw of the delay time.  TK-Time Slices Used is aliased as TK-CounterHi and is loaded with the msw +1 of the delay time.  Every time the task scheduler schedules the task the macro code decrements TK-CounterHi until it also becomes zero and the task is resumed.

Note that TK-Cycle will not take account of the time TK-LongWait.

Task Start-up

At each SSP mode change a different sampling strategy is required.  To do this the task table has to be changed to reflect the change in the sampling strategy.

At a mode change the Task Handler loads TK-Task Table with a new task table held in program memory.  Originally these will have been in PROM and will have been loaded into RAM at power on.  They can be modified either by telecommand or with replacements held in EEPROM. These start-up task tables are identical in form to the running task table and are simply copied to TK-Task Table at the appropriate time.

The format of a startup task table entry is as follows:

TK-TaskAddress		Start address of the task

TK-TimeSlicesUsed		0

TK-Count	The number of timeslices after the table is loaded before the task actually runs.

TK-Cycle	The cycle time of the task.  This is set here and is not altered (usually) by the task.

The start-up task table has to be constructed so that there is minimum interference between tasks.  Effectively this means that no two tasks should be scheduled to run in the same time slice.  This is not always possible but in the majority of cases where exact timing is necessary then this should be achievable.  Most of the SSP tasks run at fairly well defined intervals such as every second or some whole number of seconds.  Here the initial value of TK-Count can be used to stagger these tasks so that they do not conflict.  There are complications caused by tasks that run for a large successive number of timescales e.g. a REF readout and those that have non uniform sampling timing e.g. THP and those that have to be sampled very fast e.g. ACC-I.

One problem with reloading the task table is that it is not known what state the existing tasks were in.  Ordinarily, this does not matter but it will impact the handling of the data streams.  If task is terminated then the data stream that it is loading to may not have a complete sample in it.  Any incomplete data stream packets are padded out to their normal length to simplify data processing.

### Main Polling Loop

The following tasks are executed in sequence:

START:			Command Packet Handler

Telemetry Packet Handler

ACC-E Impact Processing (if impact detected)

Main Processing (see below)

Telemetry Packet Handler

Increment Poll Count (reset on BCP)

Jump to Start.

### Main Processing

These background tasks are executed in priority order when the main polling loop reaches "Main Processing".  The tasks are:

1.	ACC-I		Impact Processing

2.	ACC-I		Data Processing (all modes)

3.	API-S		Data Processing

4.	REF		Data processing.

## SSP Modes

The SSP experiment has eight modes.

| MODE | DESCRIPTION | STARTS AT |
| --- | --- | --- |
| 0 (or 8) | EMC Test | Command Only |
| 1 | Upper Atmosphere | to |
| 2 | Mid Atmosphere | to + 10 Minutes |
| 3 | Lower Atmosphere | to + 85 Minutes |
| 4 | Proximity | Attitude = 7km or to + 120 min. |
| 5 | Surface | Impact or Proximity + 31 Min. |
| 6 | Extended Surface | Impact + 3 Minutes |
| 7 | Diagnostic | Command Only |

The modes each define a different data sampling scheme. A summary may be found in Appendix D - Excel sheet FMDATA.XLS

Modes 1-6 only are used during the nominal mission.  The modes are entered automatically in response to the Descent Data Broadcast (except for Proximity and Impact explained later).

The instrument may be commanded into any mode. In this case the command contains a parameter which can be used to freeze SSP into that mode; otherwise it may change autonomously based on certain criteria.

### Mode 0 or 8 (EMC)

In Mode 0 only housekeeping packets are generated.  All available packets are filled, so the housekeeping is sampled much faster than normal. This mode is used during ground test and is especially useful during EMC testing.

Mode 0 is not used during a nominal descent.

### Mode 1 (Upper Atmosphere)

When SSP is switched on it starts in Mode 1.  After it has locked to the DDB, the mode may then change if appropriate.

In Mode 1 DEN, PER, TIL and HK are regularly sampled.  There are also stimulation (self test) sequences for ACC-E, ACC-I and THP sensors.  There are 5 THP samples:

Wire 0		:	A/D Converter Cross-Calibration

Wire 1		:	Low Current Only

Wire 2		:	Low Current Only

Wire 3		:	Low Current Only

Wire 4		:	Low Current Only

Mode 1 corresponds to the period when the CDMU polling rate is 3 packets/16 sec. SSP leaves Mode 1 at To + 10 minutes when CDMU polling changes to 8 packets/16 sec.

### Mode 2 (Mid Atmosphere)

On entering Mode 2, ACC-E is stimulated (and packets generated) for checkout purposes.  Thereafter the normal sampling scheme is followed for all sensors; see Appendix D.

In Mode 2 SSP can adapt to an increase in polling rate if this occurs, by increasing the number of API-S samples.  This will generate an extra 1.5 SSP packets/16 secs at most.

SSP leaves Mode 2 at To  + 85 minutes when CDMU polling changes to Step 3 (11 SSP packets/16 sec).

### Mode 3 (Lower Atmosphere)

On entering Mode 3, ACC-E is stimulated for checkout purposes.  Other sensors continue regular sampling.  API-S is already sampling at its maximum rate of 0.7 Hz so no extra packets can be generated. This means that SSP can only fill about 90% of its nominal allocation (970 out of 1138).

### Mode 4 (Proximity)

SSP enters proximity mode at an altitude of 7 km or at a time out of (To + 120 minutes).

ACC-I is sampled continuously at 500 samples/sec and REF sampling drops to only one sample per 15 mins.

API-S reverts to adaptive sampling and transmits extra data in proximity mode.  At an altitude of 1km (reported in the probe DDB) the API-S pulse length changes to 2 mS for enhanced accuracy (N.B. This change is only made if in the DDB Word 5 bit 15 is set to 0, i.e. “measured altitude”. This prevents the 2 ms operation taking place if the ≤ 1km altitude is an estimated value rather than the real altitude derived from the RAU A and RAU B. The practical outcome of this is that for the Titan mission data the API-S pulse width would be reduced to 2ms at 1km indicated altitude, but would switch back to 10 ms pulse width at a lower altitude once the RAU were unable to function and the DDB altitude bit was reset to 1, i.e. “predicted altitude”.).

SSP can increase its packet usage to approx 22 packets/16 secs in Mode 4.

### Mode 5 (Surface)

When impact is detected SSP enters Mode 5 (the impact detection algorithm is discussed later). At this point some special processing takes place to build an Impact Packet before normal sensor sampling recommences.

The Impact Packet contains processed ACC-E and ACC-I impact data plus new samples from the other sensors. Impact times as detected by ACC-E and ACC-I are included. Impact Packet building takes about 1.5 secs.  As a high priority the spacecraft interface FIFOs are loaded with eight copies of the impact data packet.

Normal packet building and sensor sampling then recommences.  There will be some data left over from pre-impact (say 6 packets) still to be transmitted, and the complete ACC-E and ACC-I impact buffers and the surface REF readout also join the data output queue. At (T impact + 15 secs) a THP sample is taken.

Transmission of packets immediately after impact:

| TYPE | NUMBER OF PACKETS | APPROX. DURATION SECS |
| --- | --- | --- |
| Last Pre-impact | 1 or 2 | 2 |
| Impact | 4 | 5 |
| Remaining pre-impact | 6 max | 8 |
| ACC-E | 5 | 22 |
| ACC-I | 7 | 22 |
| REF | 12 | 22 |
| THP | 1 | 22 |

Impact Processing

After Impact has been detected some special processing takes place to build the Impact Packet before normal sensor sampling resumes.  The tasks are listed below:-

Send some existing data streams

PER sample

Trigger APIS and APIV

2 DEN Samples - write to Impact Packet

TIL		- write to Impact Packet

Send Tele

REF X3 			→	REF datastream

REF Calc 		→	Impact Packet

Send tele

Process ACC-I 			Impact Packet

ACC-I	 Buff		→	datastream

Process ACC-E		→	Impact Packet

Send ACC-E buff if non-zero

Read APIV 		→	Impact Packet

Send Tele

Process AP1S 		→	Impact Packet

Send APIS buff

Send Tele

Read PER and CON 	→	Impact Packet

Send IMP x 4

### Mode 6 (Extended Surface)

Three minutes after impact SSP starts Mode 6.  The principal change is that API-S switches to a mode optimised for ocean depth sounding.  Ten pulses are emitted followed by a break of 20 seconds.

### Mode 7 (Diagnostic)

In Mode 7 no telemetry packets are generated except on command.  This mode may be useful for debugging hardware faults which may develop during cruise.

## Proximity Mode and Impact Detection Algorithm

Changing into Proximity Mode

The primary concern is to ensure that Proximity Mode is entered before the Probe impacts the surface for the minimum descent scenario and maximum Titan topography, for whatever failures occur on the Radar Altimeters or Descent Data Broadcast (DDB).  However, entry into Proximity Mode at too high an altitude is to be avoided since the REF sensor is not read in Proximity Mode and any potential atmospheric data from the ACC-I sensor is reduced.

No notice is taken of whether the Altimeter data in the DDB is real or predicted.  The expected errors in the predicted altitude at 7km are small provided that at least one radar altimeter has functioned by that altitude.  For example, if the altimeter worked but failed at 10km then the error at 7km from plotting the nominal descent gradient would be -200m for the Minimum Descent and +100m for the maximum descent.

By increasing the nominal altitude for the SSP mode switch into proximity mode to 7km and selecting a timeout of 120 minutes we can achieve this.  For any failure Proximity Mode is entered at a minimum altitude of 2.5km above the nominal 0km.

Allowing for the 10% accuracy on the radar altimeters, the maximum altitude at which SSP can enter Proximity Mode is 7.7km.

Changing into Surface Mode

The primary concern here is not to enter Surface Mode too early, or else the impact data could be missed.  However we definitely need to get to Surface mode before the end of the mission, so a timeout is required in case of failure of the impact detection mechanisms. If Surface Mode is never entered then any data frozen in the ACC-E buffer would be lost, and other sensors would not sample in their surface modes.

After Proximity Mode is entered there is no requirement to know by which method the mode was selected.  The change into Surface Mode should be initiated by an SSP algorithm which uses the signal from the internal accelerometer (ACC-I); this will detect a solid or liquid impact. If this should fail the timeout comes into operation; this timeout is set to 31 minutes after Proximity mode, the maximum descent time expected.

In the event that switching to Surface Mode is initiated by the timeout, the time that the Probe may have been on a liquid surface at the nominal 0km altitude would be between 6.7 and 24 minutes.  This depends on the actual descent profile and whether Proximity Mode was entered by altitude or timeout.  For proximity mode entry by altitude data the maximum time on a liquid surface would be 12.9 minutes (which would be a realistic time for Probe/Experiment survival).  Further details are given in Figure 2.4.1.

Mode selection flow chart:

LOWER

ATMOSPHERE

N

ALTITUDE

< 7 KM ?

T > 120 MIN

N

Y

6.3 - 7.7 KM

Y

above terrain

due to 10% acc

2.5 -7 km above nom 0km

PROXIMITY

MODE

N

ACC-I

TRIGGER

TIN PROX

> 31 MIN

N

Y

Y

SURFACE

MODE

136.5 - 151 MIN

Figure 2.4.1     SSP Mode Change Fault Tree (7 km Proximity Altitude)

# data HANDLING

## Mission Status - The Descent Data Broadcast

Mission status is obtained from the descent data broadcast (DDB) message that is sent to SSP from the CDMS every two seconds.  There are four items of information contained in this message; the mission time, the altitude of the spacecraft, the mission phase and the spin rate of the spacecraft.  These items are copied into the SSP housekeeping packets.

### Mission Time

The mission time within the DDB contains 2 bits which define before or after T0. As SSP is always switched on after T0 these two LSBs are ignored.  The remaining 14 bits define the mission time to a resolution of 2 seconds, with full scale of 16384.

| 15 8 | 7 0 |  |
| --- | --- | --- |
| t t t t t t t t t t |  | t t t t t t x x |

t - 14 bit mission time

x - don't care

### SSP Time

The mission time is combined with the 10 bit time from the onboard millisecond counter to form a 24 bit time SSP Time value that has a resolution of 2ms. Full scale of SSP time would be 16384 secs. or 4 hours, 33 minutes, 4 seconds.

| 23 16 | 15 8 | 7 0 |
| --- | --- | --- |
| t t t t t t t t | t t t t t t m m | m m m m m m m m |

t   - 14 bit mission time

m - 10 bit millisecond counter time

### Mission Phase

This is an 8 bit value that is defined by ESA as follows:

00	-	Entry/Descent

03	-	Ground Checkout

0c	-	Ground Checkout Suspended

OF	-	Ground Checkout De-activated

F3	-	Cruise Checkout

FC	-	Cruise Checkout Suspended

FF	-	Cruise Checkout De-activated

See EID Part A Section 3.6.3.8.

The only use that SSP makes of this information is check for "Cruise Checkout" phase.  In this case TIL is not enabled and THP high current is not switched on.  "Suspend" is not sensed; SSP detects changes in mission time only and makes appropriate mode changes.

In ground checkout phase a nominal descent mission is followed with timeout to Surface Mode (no impact), unless ACC-I is stimulated by command, electronically or physically as part of a test.

### Altitude

The altitude is stored as a 16 bit word as defined in EID Part A Section 3.6.3.8.  The altitude will be used to enter proximity mode at a height of 7km.

MSB

MSB 15

0

1 = Predicted Altitude

0 = Measured Altitude

LSB = 10 m

### Spin

The spin will be stored in an 8 bit byte, at present it is not used.

15

0

Spin

7

Mission phase

LSB = 0.1 rpm

## Packet Telemetry

### Concept

Huygens uses a packet telemetry system as defined by ESA standards. Huygens packets as sent by SSP to the CDMU are 126 bytes long, of which 118 bytes is available for actual sensor data. The sensors are required to be sampled at rates varying from 1 second to 15 minutes and each data sample can have a size varying from 2 to 1k bytes.

To provide a uniform method of data handling we invented the 'Data Stream' concept, in which each sensor fills its own buffer until a predetermined number of bytes have accumulated. This number can be different for each sensor and mode of SSP. The data are combined with a header and trailer to form a 'Datastream Packet'. Datastream Packets fit within the Huygens packets (in the SSP data field) and are arranged to be an exact fit wherever possible; however for sensors generating large volumes of data per sample (THP, API-S, REF, ACC-E, ACC-I) they spread over several Huygens packets.

The data analysis system for SSP must recognise first Huygens packets, then identify the Datastream type and synchronise on the Datastream packet structure.

For a pictorial overview of data sample formats and complete packets see Appendix A.

### Packet Headers

#### Huygens Packets

The Huygens packet consists of 126 bytes :-

| Packet ID ( 2 bytes) | Sequence Control ( 2 bytes) | Packet Length (2 bytes) | SSP Data Source (2 bytes) | SSP Data (118 bytes) |
| --- | --- | --- | --- | --- |

Packet ID		Value = 0x0F94 for CDMU-A

Value = 0x0FB4 for CDMU-B

Sequence Control	Value = 11xxxxxx xxxxxxxx

where       xxxxxx xxxxxxxx is a 14 bit Huygens packet counter.

Packet Length		Value = 0x0077

SSP Data Source	Value = cccccccc cccciiii

where    cccccccc cccc       is a 12 bit datastream packet counter.

where                          iiii   is the Datasteam ID, see below:

SSP Data		is the field used by datastream packets, see the next section.

| Datastream ID | Data stream |
| --- | --- |
| 0 | Engineering |
| 1 | Impact |
| 2 | ACC-I |
| 3 | API-S |
| 4 | API-V |
| 5 | DEN |
| 6 | PER |
| 7 | REF |
| 8 | THP |
| 9 | TIL |
| A | Housekeeping |
| B | ACC-E |

#### Datastream Packets

The datastream packet format is the same for each stream, however the length is variable. Headers and trailers are fixed (8 bytes total). Details of each datastream packet type may be found under the section for each sensor. To fit in a Huygens packet, the data length would be 110 bytes.

| Start Sync (2 bytes) | Time ( 3 bytes) | Mode (1 byte) | Data Samples (variable) | End Sync (2 bytes) |
| --- | --- | --- | --- | --- |

Start Sync		Value = 0x8888

Time			is the SSP Time when the first data sample was made.

Mode			Value = ffff mmmm

where   ffff                 is mostly unused (uses listed under sensors).

where          mmmm is the SSP mode when the samples were taken.

Data Samples		detailed under sensor sections following.

End Sync		Value = 0x9999

### ACC-E

#### Sensor Sampling

An ACC-E sample consists of 512 x 8 bit values. Sampling stops at impact so that the FIFO buffer contains the impact signature.

#### ACC-E Datastream Packet, ID = 1 (Modes 0, 5, 7)

Datastream Packet length = 520 bytes

Measurements/packet = 1 impact signature

This packet contains the full impact signature and is transmitted during the descent sequence only following the impact.

The packet can be identified by the Mode field in the Datastream Header i.e.

Mode	Value = ffff mmmm

where   ffff                  is the datastream packet ID = 0001

where          mmmm is the SSP mode when the samples were taken.

| Datastream Header (6 bytes) | Sensor Samples (512 bytes) | End Sync (2 bytes) |
| --- | --- | --- |

The Sensor Samples data field consists of the raw 512 x 8 bit sensor samples.

#### ACC-E Datastream packet, ID = 2 (Modes 1,2,3)

Datastream Packet length = 110 bytes

Measurements/packet = 1 compressed impact signature

This packet contains a compressed impact signature consisting of 4 calculated values plus a selection of raw data around the peak response.

Calculated values are :-

a)	The peak value of the impact signature.

b)	The offset of the peak value from the start of the impact signature.

c)	The offset from the start of the impact signature of the 1/2 maximum point before the peak.

d)	The offset from the start of the impact signature of the 1/2 maximum point after the peak.

These are stored in 4 x 16 bit words.  These values are followed by 102 bytes of raw data from the impact buffer:  the first four, 94 around the peak, and the last four; giving a total of 110 bytes which fits in a Huygens packet.

| Field | Offset | Size (Bytes) |
| --- | --- | --- |
| Max Signal | 0 | 2 |
| Max Position | 2 | 2 |
| Pre-Max FWHM position | 4 | 2 |
| Post-Max FWHM position | 6 | 2 |
| First 4 samples in buffer | 8 | 4 |
| 94 peak samples | 12 | 94 |
| last 4 samples in buffer | 106 | 4 |

Figure 3.2.1.1 ACC-E Compressed Impact Packet data field

### ACC-I

#### Sensor Sampling

A single data sample from ACC-I consists of a 12-bit value that will be stored in a 16-bit word. The sample rate is 500 samples/sec.

#### ACC-I Datastream packet, ID = 0  (Modes 1,2,3,6)

Datastream Packet length = 110 bytes

Measurements/packet = 18

In these modes, one measurement consists of 200 samples, processed to give three 16 bit words which hold the maximum sample value, the mean and the variance.  Used in Modes 1, 2, 3, 5, 6.

| Field | Offset (Bytes) | Size (Bytes) |
| --- | --- | --- |
| Maximum Value | 0 | 2 |
| Mean | 2 | 2 |
| Variance | 4 | 2 |

The packet consists of 18 processed samples as above followed by 2 bytes padding with a value of 0x9999,  giving a total datastream packet length of 118 bytes.

#### ACC-I Datastream packet, ID = 1  (Modes 0,5,7)

Datastream Packet length = 768 bytes

Measurements/packet = 1 impact signature

The ACC-I impact signature consists of 512 single data samples.  ACC-I is sampled at a rate of 500Hz during the proximity phase.  The samples are stored in a 512 x 16-bit word FIFO.  The sampling is frozen on impact so that the 512 samples obtained show the acceleration profile when impact occurred.  Once the FIFO has been frozen the impact signature is packed into a 512 x 12 bit word array for transmission.

#### ACC-I Datastream packet, ID = 2  (Mode 1)

Datastream Packet length = 110 bytes

Measurements/packet = 1 compressed impact signature.

An ACC-I compressed impact signature has the same format as the ACC-I compressed signature. Four 16 bit values are calculated, these are:

a)	The peak value of the impact signature.

b)	the offset of the peak value from the start of the impact signature.

c)	The offset from the start of the impact signature of the 1/2 maximum point before the peak.

d)	The offset from the start of the impact signature of the 1/2 maximum point after the peak.

| Field | Offset | Size (Bytes) |
| --- | --- | --- |
| Max Signal | 0 | 2 |
| Max Position | 2 | 2 |
| FWHM position pre-peak | 4 | 2 |
| FWHM position post-peak | 6 | 2 |
| First 4 samples in buffer | 8 | 6 |
| 60 peak samples | 14 | 90 |
| last 4 samples in buffer | 104 | 6 |

Figure 3.2.1.2c ACC-I Compressed Impact Signature

### API-S

#### Sensor sampling

An API-S return signal consists of 1024 samples, where each sample is an 8 bit value (i.e. raw data held in a 1 kilobyte array). Time step between each sample is 1 milli-second. To accommodate limited transmission bandwidth,  three data reduction strategies are used depending on the mission phase as described below:-

#### Checkout (raw) Sample (mode = 0, 8)

Only used during in-flight/cruise or ground testing.

NOT USED DURING THE TITAN MISSION.

Datastream Packet length = 1010 bytes

Measurements/packet = 1

| Field | Offset (Bytes) | Size (Bytes) | Description |
| --- | --- | --- | --- |
| Sync | 0 | 2 | Sync Word = 8888 hex |
| Time | 2 | 3 |  |
| Mode | 5 | 1 | SSP Mode |
| Peak | 6 | 2 | Peak Position |
| Data | 8 | 1000 | Raw Data |
| Sync | 1008 | 2 | 9999 hex |

#### Atmosphere Sample (modes 2, 3)

Datastream Packet length = 36 bytes

Measurements/packet = 1 binned return

Samples 50 to 570 are extracted from the API-S return signal and binned into 26 bins of 20 (i.e. each bin gives the average of 20 samples).

For modes 2 and 3 ONLY the bins packing order in the telemetry packets is swapped pair-wise as shown by the figure below (i.e. “Little Endian” storage format).

(N.B. Titan PDS data are corrected but any older, non PDS complaint data sets may need correcting).

Actual order of bins  (where each bin is average of 20 samples)

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |

Order of bins in telemetry packets

| 2 | 1 | 4 | 3 | 6 | 5 |
| --- | --- | --- | --- | --- | --- |

#### Proximity Sample (modes 4, 5)

Datastream Packet length = 146 bytes

Measurements/packet = 1 compressed return

The Apis return signal is reduced using the following strategy:-

- Discard first 20 samples. (After this step, the 1st sample = sample 21st from raw data set).

- Determine the peak position (i.e. position of the API-S sample with the highest value) from  remaining  samples.

- Store peak position in the telemetry  packet marked by “Peak”. Please note to recover correct peak position, an offset of 20 needs to be added to the peak position (due to 20 discarded samples).

- If peak < 100, then set the peak to 100, for data reduction purposes. Else keep the actual peak position.

- Transmit the first +/- 30 samples  around the peak without compression (i.e. raw data).

- Transmit the remaining samples between peak +/- 100  as bins of 4 (i.e. average 4 samples to give 1 value).

- Transmit all remaining samples as bins of 20.

In algorithm terms, the compression around the peak can be expressed as:-

Starting at first sample

while this sample number is less than the total no of samples

if this sample is within 30 samples of the peak sample

compressed data equals raw data

look at next sample if this sample is within 100 samples of the peak sample

compressed data equals the mean of the next 4 samples

look at fourth sample from this sample

if this sample is not within 100 samples of the peak sample

compressed data equals the mean of the next 20 samples

look at 20th sample from this sample.

The net effect of this reduction strategy is to retain features around the peak from the API-S return signature (as long as the peak signature is after 120 milli-seconds from the start of the return).

#### Extended Surface Sample (mode 6)

Datastream Packet length = 646 bytes

Measurements/packet = 1, consisting of 10 pulses.

Every Depth Sounding:

10 API-S pulses are sent  approximately 1 second apart. The data from all 10 measurements are compressed by binning each into 50 bins of 20 samples. Since the first 20 samples are discarded, the 1st bin represents the average of API-S return from 21ms to 40 ms .

The return which has the maximum signal is also processed as for proximity mode.

| Field | Offset (Bytes) | Size (Bytes) | Description |
| --- | --- | --- | --- |
| Sync | 0 | 2 | Sync Word = 8888 hex |
| Time | 2 | 3 | Time at which sample taken |
| Mode | 5 | 1 | Mode = 6 |
| Peak | 6 | 2 | Peak Position |
| API-S Data | 8 | 136 | Peak Sample |
| API-S Data | 144 | 500 | 10 Compressed Samples |
| Sync | 644 | 2 | Sync Word = 9999 |

### API-V

#### Sensor sampling

An API-V measurement consists of two 12 bit values.  These are a measure of the local velocity of sound in forward and reverse directions.

| 12 Bits | 12 Bits |
| --- | --- |
| VEL 1 | VEL 2 |

Figure 3.2.2.1a API-V Sample

Once two measurements have been collected these will be compressed into 3 x 16 bit words:-

word                 0	             1	     2

| Each D Each digit represents 4 bits --> | 1 1 1 1 1 1 2 | 2 2 1 1 | 1 2 2 2 |
| --- | --- | --- | --- |

#### API-V Datastream packet (all modes)

Datastream Packet length = 118 bytes

Measurements/packet = 36

### DEN

#### Sensor Sampling

A DEN measurement consists of a single 12 bit value.

Four samples are compressed into 3 x 16 bit words.

A Den offset measurement will also be made and placed in the housekeeping packet.

#### DEN Datastream Packet

Datastream Packet length = 118 bytes

Measurements/packet = 72

2 bytes of padding, value = 0x9999 are appended to the end of the packet.

### PER and CON

#### Sensor Sampling

PER and CON measurements are each single 12 bit values.

Four 12 bit values are packed into 3 x 16 bit words:

1	2	3

| ( | p e r c | o n p e | r c o n | ) = 1 measurement |
| --- | --- | --- | --- | --- |

PER and CON offset measurements will be placed in the housekeeping packet.

#### PER and CON Datastream packet

Datastream Packet length = 118 bytes

Measurements/packet = 36

2 bytes of padding, value = 0x9999 are appended to the end of the packet.

### REF

#### REF Sensor Sampling

A REF sample consists of 512 x 8 bit values.  A complete REF readout sequence consists of 3 samples, one with the internal illumination LED on, one with no illumination and a third with the external illumination LED on.

#### REF  Datastream packet

Datastream Packet length = 520 bytes

Measurements/packet = 1

Each packet contains a single REF readout. The illumination mode is indicated in byte 6 of the datastream header:

Mode	Value = iiii mmmm

where    iiii                is the illumination mode.	0 = dark

1 = external

2 = internal

where         mmmm  is the SSP mode.

### THP

#### THP Sensor Sample

A THP sample consists of a series of 16 bit measurements: the preamplifier offset,  initial temperature value, and 60 x temperature values, each value being the average of 16 ADC samples.

#### THP Datastream packet, wires 1-4  (ID = 0/1)

Datastream Packet length = 132 bytes

Measurements/packet = 1

For THP the wire number is indicated in byte 6 of the datastream header:

Mode	Value = wwww mmmm

where    wwww                is the wire number	1 = 25 microns diameter

2 = 25 microns diameter

3 = 10 microns diameter

4 = 10 microns diameter

where                 mmmm  is the SSP mode.

Data field :

| Offset (bytes) | Length (bytes) | Name |
| --- | --- | --- |
| 0 | 2 | Preamp offset |
| 2 | 2 | Initial Temperature |
| 4 | 120 | 60 Averaged Temperature Values |

#### THP Datastream packet, wires 0 (ID = 2)

Datastream Packet length = 132 bytes

Measurements/packet = 31 samples, 12bit + 16 bit

In mode 1 a single checkout packet is generated to inter-calibrate the 12 bit and 16 bit A/D converters in SSP. The THP D/A converter is used as a signal source to generate a staircase waveform of 31 levels. Readings from each A/D converter are assembled into a packet which has the wire number set to zero. ('wwww' field above).

### TIL

#### TIL Sensor sample   (modes 1-4 @ 1Hz)
                                                    (modes 5 + 6 @ 2Hz)

A TIL data sample consists of 4 x 12 bit values which correspond to a voltage range of -5V to +5V.  These values are stored in 16 bit words:

6 Bytes

| TLXH | TLXL | TLYH | TLYL |
| --- | --- | --- | --- |

In addition, the excitation voltages and offsets are measured and placed in the Housekeeping packet.

TIL Excitation Voltages:

6 Bytes

| TOPXH | TOPXL | TOPYH | TOPYL |
| --- | --- | --- | --- |

TILOffset Voltages:

6 Bytes

| TLXO | TLYO | Topxo | TOPYO |
| --- | --- | --- | --- |

#### TIL Datastream packet

Datastream Packet length = 118 bytes

Measurements/packet = 18

2 bytes of padding, value = 0x9999 are appended to the end of the packet.

### Housekeeping

#### Housekeeping Samples

There are 11 temperature sensors sampled by SSP; eight diodes, one IC and two AD590s.  They are all digitised to 12 bit accuracy.

The majority of voltage monitors are digitised to 12 bit accuracy.  The exceptions are three values sampled by the 16 bit converter.

Various processor status words and bytes are also included in the housekeeping packet.

#### Housekeeping Datastream packet

Datastream Packet length = 110 bytes

Measurements/packet = 1

The housekeeping packet is built at the time it is required using variables currently in memory; the current values are updated by the main processing task.  The 12 bit samples are packed (4 samples in 3 x 16 bit words). A list of housekeeping parameters and their position in the packet is given below. Where the parameter does not occupy a whole byte or word, the mask column shows which are bits contain the parameter.

| Offset (Bytes) |  | Description |  | Size (Bytes) |  | Mask |  | Mnemonic |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | Huygens Packet Header - ID |  | 2 |  |  |  | HUYPKTID |
| 2 |  | Huygens Packet Header - Counter |  | 2 |  |  |  | HUYPKTCNT |
| 4 |  | Huygens Packet Header - Length |  | 2 |  |  |  | HUYPKTLEN |
| 6 |  | SSP Data Stream Count |  | 2 |  | 0xFFF0 |  | STRMCNT |
| 6 |  | SSP Data Stream ID (=0x0A) |  | 2 |  | 0x000F |  | STRMID |
| 8 |  | Start Sync = 0x8888 |  | 2 |  |  |  | STARTSYNC |
| 10 |  | SSP Time |  | 3 |  |  |  | SSPTIME |
| 13 |  | SSP Mode |  | 1 |  |  |  | MODE |
| 14 |  | Altitude |  | 2 |  |  |  | ALTITUDE |
| 16 |  | Spin Rate |  | 1 |  |  |  | SPIN |
| 17 |  | Mission Phase |  | 1 |  |  |  | PHASE |
| 18 |  | Time Last Mode Change |  | 3 |  |  |  | TIMELM |
| 21 |  | Last Mode |  | 1 |  |  |  | LASTMODE |
| 22 |  | Altitude Last Mode Change |  | 2 |  |  |  | ALTLM |
| 24 |  | SSP Command Count |  | 2 |  |  |  | SSPCMDCNT |
| 26 |  | Broadcast Count A |  | 2 |  |  |  | BCASTCNTA |
| 28 |  | Broadcast Count B |  | 2 |  |  |  | BCASTCNTB |
| 30 |  | Command Packet Count A |  | 2 |  |  |  | CMDPKTCNTA |
| 32 |  | Command Packet Count B |  | 2 |  |  |  | CMDPKTCNTB |
| 34 |  | Telemetry Packet Count A |  | 2 |  |  |  | TMPKCNTA |
| 36 |  | Telemetry Packet Count B |  | 2 |  |  |  | TMPKCNTB |
| 38 |  | Command Error Count (Total) |  | 1 |  |  |  | CMDERRCNT |
| 39 |  | Command Error Code (Last Error) |  | 1 |  |  |  | CMDERRCD |
| 40 |  | Last Command Sequence Number |  | 1 |  |  |  | LCMDSQNO |
| 41 |  | Last Command Code |  | 1 |  |  |  | LCMDCODE |
| 42 |  | Engineering Packet Count |  | 2 |  | 0xFFF0 |  | ENGPKTCNT |
| 44 |  | Impact Packet Count |  | 2 |  | 0xFFF0 |  | IMPPKTCNT |
| 46 |  | ACC-I Packet Count |  | 2 |  | 0xFFF0 |  | ACCIPKTCNT |
| 48 |  | API-S Packet Count |  | 2 |  | 0xFFF0 |  | APISPKTCNT |
| 50 |  | API-V Packet Count |  | 2 |  | 0xFFF0 |  | APIVPKTCNT |
| 52 |  | DEN Packet Count |  | 2 |  | 0xFFF0 |  | DENPKTCNT |
| 54 |  | PER Packet Count |  | 2 |  | 0xFFF0 |  | PERPKTCNT |
| 56 |  | REF Packet Count |  | 2 |  | 0xFFF0 |  | REFPKTCNT |
| 58 |  | THP Packet Count |  | 2 |  | 0xFFF0 |  | THPPKTCNT |
| 60 |  | TIL Packet Count |  | 2 |  | 0xFFF0 |  | TILPKTCNT |
| 62 |  | HK Packet Count |  | 2 |  | 0xFFF0 |  | HKPKTCNT |
| 64 |  | ACC-E Packet Count |  | 2 |  | 0xFFF0 |  | ACCEPKTCNT |
| 66 |  | Temp 1 (THP Sensor Body) |  | 3 |  | 0xFFF000 |  | THPT |
| 66 |  | Temp 2 (REF Sensor Board) |  | 3 |  | 0x000FFF |  | REFSENT |
| 69 |  | Temp 3 (REF Prism Tip) |  | 3 |  | 0xFFF000 |  | REFPRTIPT |
| 69 |  | Temp 4 (REF Prism Base) |  | 3 |  | 0x000FFF |  | REFPRBASET |
| 72 |  | Temp 5 (PER Sensor Body) |  | 3 |  | 0xFFF000 |  | PERT |
| 72 |  | Temp 6 (Top Hat Foam) |  | 3 |  | 0x000FFF |  | TOPHATT |
| 75 |  | Temp 7 (SSPE 1 - 16 bit A/D) |  | 3 |  | 0xFFF000 |  | THPADCT |
| 75 |  | Temp 8 (SSPE 7 - 5v Converter A) |  | 3 |  | 0x000FFF |  | CONVT |
| 78 |  | Temp 9 (TIL Sensor) |  | 3 |  | 0xFFF000 |  | TILTT |
| 78 |  | Temp 10 (SSPE Reference) |  | 3 |  | 0x000FFF |  | SSPEBOXT |
| 81 |  | Temp 11 (2.5v Ref) |  | 3 |  | 0xFFF000 |  | 2V5REFT |
| 81 | 2.5v Ref Mon |  | 3 |  | 0x000FFF |  | 2V5 |  |
| 84 | 4.5v REF Mon |  | 3 |  | 0xFFF000 |  | 4V5 |  |
| 84 | -9v Ref Mon |  | 3 |  | 0x000FFF |  | M9V |  |
| 87 | +12v Mon |  | 3 |  | 0xFFF000 |  | P12V |  |
| 87 | -12v Mon |  | 3 |  | 0x000FFF |  | M12V |  |
| 90 | +5v Mon |  | 3 |  | 0xFFF000 |  | P5V |  |
| 90 | Test Input Via 12 Bit A/D |  | 3 |  | 0x000FFF |  | TEST |  |
| 93 | THP Voltage Ref - Gas (Via 12 Bit) |  | 3 |  | 0xFFF000 |  | VREFG |  |
| 93 | THP Voltage Ref - Liquid (Via 12 Bit) |  | 3 |  | 0x000FFF |  | VREFL |  |
| 96 | ACC-E Pre-amp Output |  | 3 |  | 0xFFF000 |  | ACCEPRE |  |
| 96 | DEN Offset |  | 3 |  | 0x000FFF |  | DENOFF |  |
| 99 | CON Offset |  | 3 |  | 0xFFF000 |  | CONOFF |  |
| 99 | PER Offset |  | 3 |  | 0x000FFF |  | PEROFF |  |
| 102 | TIL Excitation X, +ve |  | 3 |  | 0xFFF000 |  | TOPXH |  |
| 102 | TIL Excitation X, -ve |  | 3 |  | 0x000FFF |  | TOPXL |  |
| 105 | TIL Excitation Y, +ve |  | 3 |  | 0xFFF000 |  | TOPYH |  |
| 105 | TIL Excitation Y, -ve |  | 3 |  | 0x000FFF |  | TOPYL |  |
| 108 | TIL Offset X, Top |  | 3 |  | 0xFFF000 |  | TOPXO |  |
| 108 | TIL Offset Y, Top |  | 3 |  | 0x000FFF |  | TOPYO |  |
| 111 | TIL Offset X |  | 3 |  | 0xFFF000 |  | TLXO |  |
| 111 | TIL Offset Y |  | 3 |  | 0x000FFF |  | TLYO |  |
| 114 | ACC-I Offset |  | 2 |  |  |  | ACCIOFF |  |
| 116 | Errors |  | 1 |  |  |  | ERRORS |  |
| 117 | Status Byte - (Port C + A?) |  | 1 |  |  |  | STATBYTE |  |
| 118 | THP Voltage Ref - Gas (Via 16 Bit) |  | 2 |  |  |  | VREFG16 |  |
| 120 | Test Input via 16 Bit A/D |  | 2 |  |  |  | TEST16 |  |
| 122 | +5v Mon Via 16 Bit A/D |  | 2 |  |  |  | P5V16 |  |
| 124 | End Sync = 0x9999 |  | 2 |  |  |  | ENDSYNC |  |

Bit Allocation for ERRORS - byte 116

| Bit | Function |
| --- | --- |
| 7 | Timer Over-run (Task has exceeded 1mS) |
| 6 | EEPROM write timeout (Verification after 10mS) |
| 5 | BCP Failure |
| 4 | 2 Sec Failure |
| 3 | DDB Failure |
| 2 | No DDB Sync |
| 1 | Memory Error |
| 0 (LSB) | THP Wire Broken |

### Impact Packet

#### Impact Packet sample

The Impact packet contains a summary data set from all sensors, especially the accelerometer impact signatures. Section 2.4.6 describes how the packet is built at the beginning of mode 5. The packet is transmitted 8 times to maximise the chances of collecting this most important data from SSP.

#### Impact Datastream packet

Datastream Packet length = 118 bytes

Measurements/packet = 1

| Parameter | Data Stream Packet Offset | Size (Bytes) | Description |
| --- | --- | --- | --- |
| Start Sync | 0 | 2 | = 0 x 8888 |
| Impact Time | 2 | 3 | Mission Time at ACC-I Impact |
| MODE | 5 | 1 | = XY, where X=Packet_ID, Y=Mode |
| ACC-I Peak Position | 6 | 2 |  |
| ACC-I Pre-Position | 8 | 2 | Half Height Point |
| ACC-I Post-Position | 10 | 2 | Half Height Point |
| ACC-I Samples | 12 | 12 | 8 x 12 bit samples around peak |
| Spare | 14 | 2 |  |
| ACC-E Sample Time | 26 | 3 | Mission Time at ACC-E Impact |
| Mode | 29 | 1 | = XY, where X=not used, Y=Mode |
| ACC-E Peak Position | 30 | 2 |  |
| ACC-E Pre-Position | 32 | 2 | Half Height Point |
| ACC-E Post-Position | 34 | 2 | Half Height Point |
| ACC-E Samples | 36 | 10 | 10 Samples Around Peak |
| REF Internal RI Point | 46 | 2 | Max Differential |
| REF Internal Raw Data | 48 | 10 | 10 Samples Around RI Point |
| REF External RI Point | 58 | 2 | Max Differential |
| REF External Raw Data | 60 | 10 | 10 Samples Around RI Point |
| API-S Sample Time | 70 | 3 | Mission Time |
| Mode | 73 | 1 | = 5 |
| API-S Peak Signal | 74 | 2 |  |
| API-S Peak Position | 76 | 2 |  |
| API-S Pre-Position | 78 | 2 | Half Height Point |
| API-S Post-Position | 80 | 2 | Half Height Point |
| API-V Sample | 82 | 4 | 3 Bytes APIV, + 1 spare |
| DEN Sample Time | 86 | 3 | Mission Time |
| Mode | 89 | 1 | = 5 |
| DEN Sample | 90 | 2 |  |
| PER Sample | 92 | 4 | per + con, 3 Bytes + 1 spare |
| TIL Sample Time | 96 | 3 |  |
| Mode | 99 | 1 | = 5 |
| TIL Sample | 100 | 6 | 4 x 12 Bits |
| Temps 2 - 8 | 106 | 10 | 7 x 12 Bits |
| End Sync | 116 | 2 | = 0x9999 |

### Engineering

#### Engineering Samples

Engineering samples are taken in response to commands only, generally used for memory-dump and fault diagnostic purposes.

#### Engineering Datastream Packets

Engineering packets have the following general format. Contents of the data field depend on the command which requested the engineering packet. Details are given in the following sections, with the command which requests the information.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | Sync Word = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | ID of Command Executed |
| Command Sequence Number | 2 | Number of Command Executed |
| Data | Variable | Data Generated by Command |
| Sync | 2 | Sync Word = 9999 hex |

Figure 3.2.14.2  SSP Command Reply Format

## SSP Status Words

| CDMU B | Bit No. | CDMU A |
| --- | --- | --- |
| Software Impact | 15 (MSB) | Software Impact |
| Sounder Mode | 14 | Processor Watchdog OK |
| All OK | 13 | All OK |
| Command Count LSB | 12 | Command Count LSB |
| DDB Loss | 11 | SSP Mode MSB |
| BCP Loss | 10 | SSP Mode NMSB |
| Redundant 12v Enabled | 9 | SSP Mode LSB |
| Nominal 12V Enabled | 8 | 12V OK |
| ! Processor Valid | 7 | ! Processor Valid |
| Impact | 6 | Impact |
| ROM +5V | 5 | ROM +5V |
| Bus B + 5V | 4 | Bus B + 5V |
| Bus A + 5V | 3 | Bus A + 5V |
| ! ML FIFO Empty | 2 | ! ML FIFO Empty |
| ! TM FIFO B Empty | 1 | ! TM FIFO B Empty |
| ! TM FIFO A Empty | 0 (LSB) | ! TM FIFO A Empty |

Meaning of Status Flag Bits

'!' indicates an active low status.

Bits 0 - 7 (identical in both words) are derived from hardware only.

Bits 8 - 15 (CDMU A) are software status, routed via J4-J5.

Bits 8 - 15 (CDMU B) are control bits and software status routed via J4-J5.

CDMU-A

B15	SW Impact			1 = LPP Algorithm based on ACC-I data has detected 						impact.

B14	-				-

B13	All OK				1 = SSP Self Test Complete.

B12	Cmd Count LSB		The LSB of the command counter, toggles on receipt of a

valid command

B11-9	SSP Mode			0 = Checkout

1 = Upper Atmosphere

2 = Mid Atmosphere

3 = Lower Atmosphere

4 = Proximity

5 = Surface

6 = Extended Surface

7 = Diagnostic

B8	12V OK

B7	! Processor Valid		0 = CDMU-A valid,   1 = CDMU-B

B6	Impact				1 = ACC-E or ACC-I has detected impact

B5	ROM+5V			1 = Processor is booting from PROM, 0 = Running from 					RAM.

B4	Bus B +5V			1 = Supply OK, 0 = latchup protection has tripped.

B3	Bus A +5V			1 = Supply OK, 0 = latchup protection has tripped.

B2	!ML FIFO Empty		0 = Command Buffer Empty

B1	!TM FIFO B Empty		0 = Telemetry Buffer B Empty

B0	!TM FIFO A Empty		0 = Telemetry Buffer Empty

CDMU-B

B15	S W Impact			1 = LPP Algorithm based on ACC-I data has detected

impact.

B14	Sounder Mode			0 = Atmosphere; 1 = Proximity.

B13	All OK				1 = Self Test Complete

B12	Cmd Count LSB		The LSB of the command counter; toggles on receipt of a

valid command

B11	DDB Loss			1 = loss, 0 = OK

B10	BCP Loss			1 = loss, 0 = OK

B9 	Redundant 12V Enabled	1 = Enabled, 0 = Disabled

B8	Nominal 12V Enabled		1 = Enabled, 0 = Disabled

B7	! Processor Valid		0 = CDMUA, 1 = CDMU B

B6	Impact				1 = ACC-E or ACC-I has detected impact

B5	ROM +5V			1 = Processor is booting from PROM, 0 = Running from

RAM.

B4	Bus B +5V			1 = Supply OK, 0 = Latchup Protection has Tripped

B3	Bus A +5V			1 = Supply OK, 0 = Latchup Protection has Tripped

B2	! ML FIFO Empty		0 = Command Buffer Empty

B1	! TM FIFO B Empty		0 = Telemetry Buffer Empty

B0	! TM FIFO A Empty		0 = Telemetry Buffer Empty

## Commands

Commands are not used during the descent to Titan as there is no command link from earth. However they may be used during cruise, most likely to send software patches to SSP or to check those already in memory at launch.

### Command Packet Format

The telecommand protocol is defined in EID Part A Section 3.6.3.6. The packet has a standard 6 byte Huygens Packet Header followed by SSP data as required. The first word of SSP Command Data indicates the number of SSP commands within this Huygens Command Packet. More than one SSP command may sometimes be used, for example when patching, as long as the total length is less than 256 bytes.

| Huygens Packet Header |  |  | SSP Command Data |  | Error checking |
| --- | --- | --- | --- | --- | --- |
| Packet Identification (2 bytes) | Packet Sequence Control (2 bytes) | Packet Length (2 bytes) | Data Field Header (2 bytes) | Command Data (Variable) | CRC (2 bytes) |

Packet Identification		Value = 0x1F94 for CDMU-A

Value = 0x1FB4 for CDMU-B

Packet Sequence Control  	Value = 11cccccc cccccccc

where    11		    are bits fixed at ‘1’

where        cccccc cccccccc is a sequence counter, normally not used.

Packet Length			Value = length of data field in bytes + 1

Data Field Header		Value = 0xnnmm

where        nn        is the number of commands in this packet.

where	         mm  is the number of command words.

Command Data			See next section

CRC				Cyclic Redundancy code, calculated from whole packet contents, is 				checked within SSP on reception.

### SSP Command Format

Within the Command Data field (above) can be a single, or multiple commands of the format given below:

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | see list of Command IDs in table below. |
| Sequence Number | 2 | may be used to identify a number of command of the same ID. |
| Parameters | Variable | Command Parameters |

A summary of all SSP commands so far implemented is given below.  Details of each command may be found in the following sections.

| Command ID (Code) | Function | Parameters | Eng. Packet? |
| --- | --- | --- | --- |
| 0 | Read RAM | Address, Length | Y |
| 1 | Write Ram | Address, Length, Data |  |
| 2 | Read EEPROM | Address, Length | Y |
| 3 | Write EEPROM | Address, Length, Data |  |
| 4 | Read PROM | Address, Length | Y |
| 5 | Verify RAM | Block (Low or High) | Y |
| 6 | Verify EEPROM | - | Y |
| 7 | Verify PROM | - | Y |
| 8 | Clear EEPROM | - |  |
| 10 | Read DRB | DRB Address | Y |
| 11 | Write DRB | DRB Address, Data Word |  |
| 12 | Execute DRB Sequence | Length, Commands | Y |
| 12 | Read I/O | I/O Address | Y |
| 14 | Write I/O | I/O Address, Data Word |  |
| 15 | Execute I/O Sequence | Length, Commands | Y |
| 16 | Read Parallel Port | Port Number | Y |
| 17 | Write Parallel Port | Port, Data, Mask |  |
| 18 | Write Telemetry | Channel, Length, Data Words |  |
| 19 | Reset Data Stream | Channel Mask |  |
| 20 | Read Processor Status | - | Y |
| 21 | Execute Code | Length, Code Words |  |
| 22 | Send Housekeeping Pkt | - |  |
| 23 | Read 12 bit A/D | Multiplexer Address, # Samples | Y |
| 24 | Read 16 bit A/D | Multiplexer Address, # Samples | Y |
| 30 | Stimulate ACC-E | Pulse Length |  |
| 31 | Test ACC_E | Pulse Length |  |
| 32 | Readout ACC-E | - |  |
| 33 | Test ACC_I | Pulse Length |  |
| 34 | Stimulate ACC-I | Pulse Length |  |
| 35 | Readout ACC-I | - |  |
| 36 | Readout API-S | Mode (for tele packet format) |  |
| 37 | Sample API-V | # Samples |  |
| 38 | Sample Den | # Samples |  |
| 39 | Sample Per | # Samples |  |
| 40 | Readout Ref | Illumination |  |
| 41 | Readout THP | Wire Number / Mode |  |
| 42 | Sample TIL |  |  |
| 50 | Set SSP Mode | Mode Number, Lock Option |  |
| 51 | Write Patch | Patch Number, Data |  |
| 52 | Read Patch | Patch Number | Y |
| 53 | Delete Patch | Patch Number |  |

### SSP Commands

#### Read RAM - 0

This command will dump any area of memory into an Engineering packet.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0000 |
| Sequence Number | 2 |  |
| Address | 2 | Address of RAM to be read |
| Length | 2 | Length of Data to be read in Words |

Engineering Packet Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 0x8888 |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID executed | 2 | = 0 |
| Command Number | 2 | Number of Command Executed |
| Length | 2 | Length of Data Returned in Words |
| Data | Length * 2 | Data Read from Ram |
| Sync | 2 | Sync Word = 9999 hex |

#### Write RAM - 1

This command will write directly to RAM. Patching would normally be used instead.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0001 |
| Sequence Number | 2 |  |
| Address | 2 | Address of Ram to be Written |
| Length | 2 | Length of Data to be Written in Words |
| Data | Length * 2 | Data to be Written to Ram |

#### Read EEPROM - 2

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0002 |
| Sequence Number | 2 |  |
| Address | 2 | Address of EEPROM |
| Length | 2 | Length of Data to be Read in Words |

Engineering Packet Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command Code | 2 | Code of Command Executed |
| Command Number | 2 | Number of Command Executed |
| Length | 2 | Length of Data Returned in Worlds |
| Data | Length * 2 | Data Read from EEPROM |
| Sync | 2 | Sync World = 9999 hex |

#### Write EEPROM - 3

This command will write directly to EEPROM. Patching would normally be used instead.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0003 |
| Sequence Number | 2 |  |
| Address | 2 | Address of EEPROM to write |
| Length | 2 | Length of Data to be Read in Words |
| Data | Length * 2 | Data to be Written to EEPROM |

#### Read PROM - 4

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0004 |
| Sequence number | 2 |  |
| Address | 2 | Address of PROM to be Read |
| Length | 2 | Length of Data to be Read in Words |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | Sync World = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command Code | 2 | Code of Command Executed |
| Command Number | 2 | Number of Command Executed |
| Length | 2 | Length of Data Returned in Worlds |
| Data | Length * 2 | Data Read from PROM |
| Sync | 2 | Sync World = 9999 hex |

#### Verify RAM Block - 5

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0005 |
| Sequence Number | 2 |  |
| Block number | 2 | 0 = Low Ram Block, 1 = High Ram Block |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command Code | 2 | = 0005 |
| Command Number | 2 | Number of Command Executed |
| Number of Errors | 2 | Number of Errors Found |
| 1st Error Address | 2 | Address at which 1st Error Occurred |
| 1st Error | 2 | Type of Error |
| 2nd Error Address | 2 | Address at which 2nd Error Occurred |
| 2nd Error | 2 | Type of Error |
| : | : | : |
| : | : | : |
| nt. Error Address | 2 | Address at which nt. Error Occurred |
| nt. Error | 2 | Type of Error |
| Sync | 2 | Sync World = 9999 hex |

#### Verify EEPROM

This command calculates a CRC (checksum) of its contents and dumps via an engineering packet.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0006 |
| Command Number | 2 | Sequence Number of Command |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result 0 = OK |
| Command Code | 2 | = 0x0006 |
| Command Number | 2 | Number of Command Executed |
| CRC | 2 | CRC Value of EEPROM |
| Sync | 2 | = 9999 hex |

#### Verify PROM Contents

N.B.	The 12V supply has to be turned off during PROM Verification

(Ground Test or Cruise Phase only)

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0007 |
| Command Number | 2 | Sequence Number of Command |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID executed | 2 | = 0x0007 |
| Command Number | 2 | Number of Command Executed |
| CRC | 2 | CRC Value of PROM |
| Sync | 2 | = 9999 hex |

#### Clear EEPROM - 8

N.B.	All Telemetry is Stopped for 2 Minutes

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | =0x0008 |
| Command Number | 2 |  |

#### Read Data Routing Bus - 10

This command is a low level read for debugging and was never used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x000A |
| Sequence Number | 2 |  |
| Address | 2 | DRB Address to be Read |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command Code | 2 | = 0x000A |
| Command Number | 2 | Number of Command Executed |
| Data | 2 | Data Read from DRB |
| Sync | 2 | = 9999 hex |

#### Write DRB - 11

Low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | =0x000B |
| Sequence Number | 2 |  |
| Address | 2 | DRB Address to be Written to |
| Data | 2 | Data to be Written to DRB |

#### Execute DRB Sequence

Low level - not used

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x000C |
| Sequence Number | 2 |  |
| Length | 2 | Number of DRB Commands to Execute |
| DRB Command | 2 or 4 | 1st DRB Command |
| DRB Command | 2 or 4 | 2nd DRB Command |
| : | : | : |
| : | : | : |
| DRB Command | 2 or 4 | nth DRB Command |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | = 0x000C |
| Command Number | 2 | Number of Commands Executed |
| Length | 2 | Length of Data Returned (Words) |
| Data | Length * 2 | Data Generated by Sequence |
| Sync | 2 | = 9999 hex |

#### Read I/O

Low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x000D |
| Sequence Number | 2 |  |
| Address | 2 | I/O Address to be Read |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | Sync Word = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | ID of Command Executed |
| Command Number | 2 | Number of Commands Executed |
| Data | 2 | Data Read from 1/0 Port |
| Sync | 2 | Sync Word = 9999 hex |

#### Write I/O

Low Level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x000E |
| Sequence Number | 2 |  |
| Address | 2 | I/O Address to be Written to |
| Data | 2 | Data to be written to I/O |

#### Execute I/O Sequence

Low level - not used

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x000F |
| Sequence Number | 2 |  |
| Length | 2 | Number of I/O Commands to Execute |
| I/O Command | 2 or 4 | 1st I/O Command |
| I/O Command | 2 or 4 | 2nd I/O Command |
| : | : | : |
|  |  | : |
| I/O Command | 2 or 4 | nth I/O Command |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | = 0x000F |
| Command Number | 2 | Number of Commands Executed |
| Length | 2 | Length of Data Returned (Words) |
| Data | Length * 2 | Data Generated by Sequence |
| Sync | 2 | = 9999 hex |

#### Read Parallel Port

Low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0010 |
| Sequence Number | 2 |  |
| Port | 2 | Port to Read |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0= OK |
| Command ID | 2 | 0x0010 |
| Command Number | 2 | Number of Commands Executed |
| Data | 2 | Data Read from Parallel Port |
| Sync | 2 | = 9999 hex |

#### Write Parallel Port

Low level - not used

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0011 |
| Sequence Number | 2 |  |
| Port | 2 | Port to Read |
| Data | 1 | Data to be Written to Port/Register |
| Mask | 1 | Bit Mask to use to Write to Port |

#### Write Telemetry (to Data Stream)

low level - not used

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0012 |
| Sequence Number | 2 | Sequence Number of Command |
| Channel | 2 | Channel to Write to (Data Stream ID) |
| Length | 2 | Number of Words of Data to Write |
| Data | Length * 2 | Data to Write to Telemetry Channel |

#### Clear Telemetry Stream(s)

Low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0013 |
| Sequence Number | 2 | Sequence Number of Command |
| Channel Mast | 2 | Mask Selects Data Streams to be Cleared |

#### Read Processor Status

low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0014 |
| Sequence Number | 2 |  |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | ID of Command Executed |
| Command Number | 2 | Number of Commands Executed |
| Registers | 32 | Contents of Registers R-R15 |
| Status Word | 2 | Contents of Status Word |
| Fault Register | 2 | Contents of Fault Register |
| Interrupt Mask | 2 | Contents of Interrupt Mask Register |
| Pending Interrupt | 2 | Contents of Pending Interrupt Register |
| Timer A | 2 | Timer A Value |
| Timer B | 2 | Timer B Value |
| m Counter | 2 | Millisecond Counter Value |
| Sync | 2 | = 9999 hex |

#### Execute Code

low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0015 |
| Sequence Number | 2 |  |
| Length | 2 | Size of Code (Words) |
| Data | Length * 2 | Code to be executed |

#### Send Housekeeping Packet

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0016 |
| Sequence Number | 2 |  |

#### Read 12 Bit ADC

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | = 0x0017 |
| Sequence Number | 2 |  |
| Mux Address | 2 | 12 Bit Multiplexer Address |
| Number of Samples | 2 | Number of Samples |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | = 0x0017 |
| Command Number | 2 | Number of Command Executed |
| Number of Samples | 2 |  |
| Data | 2 * No of Samples | Data Sampled from 12 Bit ADC |
| Sync | 2 | = 9999 |

#### Read 16 Bit ADC

low level - not used.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0018 |
| Sequence Number | 2 |  |
| Mux Address | 2 | 16 Bit Multiplexer Address |
| Samples | 2 | Number of Samples |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 8888 hex |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID executed | 2 | =0x0018 |
| Command Number | 2 | Number of Commands Executed |
| Number of Samples | 2 |  |
| Samples | 2 * No of Samples | Data Sampled from 16 Bit ADC |
| Sync | 2 | Sync Word = 9999 |

#### Stimulate ACC-E

This command generates a voltage pulse at the ACC-E sensor stimulation electrode, which is enough to initiate an impact packet if SSP is in the correct mode.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x001E |
| Sequence Number | 2 |  |
| Duration | 2 | Pulse Length N x 4 S |

#### Test ACC-E

Not used with the FM software

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x001F |
| Sequence Number | 2 |  |
| Count | 2 | Pulse Length = N x S approx. |

#### Readout ACC-E

Not used with FM software.

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0020 |
| Sequence Number | 2 |  |

#### Test ACC-I

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0021 |
| Sequence Number | 2 |  |
| Count | 2 | Pulse Length in mS |

#### Stimulate ACC-I

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0022 |
| Sequence Number | 2 |  |
| Length | 2 | Pulse Length in mS |

#### Readout ACC-I

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0023 |
| Sequence Number | 2 | Sequence Number of Command |

#### Readout API-S

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0024 |
| Sequence Number | 2 |  |
| Mode | 2 | Readout Data Format |

#### Sample API-V

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0025 |
| Sequence Number | 2 |  |
| Number of Samples | 2 | Number of Samples to be Taken |

#### Sample DEN

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0026 |
| Sequence Number | 2 |  |
| Number of Samples | 2 | Number of Samples Taken |

#### Sample PER

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0027 |
| Sequence Number | 2 |  |
| Number of Samples | 2 | Number of Samples to be Taken |

#### Readout REF

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0028 |
| Sequence Number | 2 |  |
| Illumination | 2 | 0 = Dark, 2 = Internal, 1 = External |

#### Readout THP

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0029 |
| Sequence Number | 2 |  |
| Wire Number | 1 | 1-4, or 0 = A/D Calibration |
| Mode | 1 | Determines Sample Table |

#### Sample TIL

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x002A |
| Sequence Number | 2 |  |
| Number of Samples | 2 | Number of Samples to be Taken |

#### Set SSP Mode

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0032 |
| Sequence Number | 2 |  |
| Mode | 2 | SSP Mode + Lock Flag |

Normal unlocked modes (1-6) are used in descent.

Checkout modes are 7  (no housekeeping) and 0 (housekeeping).

The locked modes are 8-15.

#### Write Patch

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0033 |
| Sequence Number | 2 |  |
| Patch Number | 2 | Number of Patch to write |
| EEPROM Address | 2 | Address in EEPROM to write patch to |
| Length | 2 | Length of patch data including CRC (excluding address) |
| Memory Address | 2 | Address to patch in memory |
| Data | 2n | Patch data |
| CRC | 2 | CRC Check |

#### Read Patch

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0034 |
| Sequence Number | 2 |  |
| Patch Number | 2 | Patch number to read |

Reply

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Sync | 2 | = 0x8888 |
| Time | 3 | Time Command Executed |
| Result | 1 | Command Result, 0 = OK |
| Command ID | 2 | = 0x0034 |
| Sequence Number | 2 | of Command Executed |
| Length | 2 | Length of patch |
|  | 2 |  |
|  | 2 |  |
| Data | Length 2 |  |

#### Delete Patch

| Field | Size (Bytes) | Description |
| --- | --- | --- |
| Command ID | 2 | 0x0035 |
| Sequence Number | 2 |  |
| Patch Number | 2 | Patch number to delete |

# SPACECRAFT INTERFACE HANDLING

## Telecommands and DDBs

Telecommands and DDBs transmitted from either CDMU A or CDMU B are stored in a h/w FIFO which is part of the SSP spacecraft interface electronics.  The LPP can determine if there is any data in the FIFO by reading the FIFO empty flag.  The processor can also determine if the CDMU is in the process of transmitting data into the FIFO.  When data is present the LPP can read data from the FIFO and process it.  The LPP can also reset the FIFO, this is done on power up to clear any "garbage" that may be present in the FIFO.

### Command Handler

Module		CM

Directory	COMMAND

Files:		Code			Command.ASM

Variables		Cm.VAR

Constants		Cm.CON

Make			Command.MAK

The command handler is responsible for reading telecommands and DDBs from the telecommand FIFO and processing them.

The command handler is run from the main background polling loop.  It is therefore interruptible by the task scheduler.

On entry the command handler checks to see if there is any data in the telecommand FIFO.  If the FIFO is empty then the handler exits.

If there is data in the FIFO then the handler checks to see if data is still being loaded into the FIFO by the CDMU.  If it is then the handler exits.  This is done to ensure that the handler does not read incomplete packets.

If there is a whole packet in the FIFO, i.e. the FIFO has data in it and is not being loaded then the packet is read from the FIFO.  If the packet length field and the amount of data in the FIFO are inconsistent then an error is signalled and the handler exits discarding the corrupted packet.  If a complete packet has been received the process packet ID is examined to see if it is a telecommand or a DDB.  The packet count is checked against the process ID counter to check that the packet has not already been received.  If the packet count is less than or equal to the process ID counter then an error is signalled and the packet is discarded.  However, if the packet count is 0 then it is accepted and the process ID counter is reset.

If the packet count is valid then the process ID counter is loaded with the packet count.

In all there are 4 process ID counters for received data, these are:-

HK_Broadcast_CounterA		DDB Channel A Process ID Counter

HK_Broadcast_CounterB		DDB Channel A Process ID Counter

HK_Command_CounterA		Telecommand Channel A Process ID Counter

HK_Command_CounterB		Telecommand Channel B Process ID Counter

If the packet is a DDB then the mission time, altitude and phase are updated.  The new mission time and altitude are used to see if SSP should change its mode.  If a mode change is detected then the task handler is activated to change the task table.

If the packet is a command packet a reply header the command count is checked to ensure that the same command is not executed more than once.  If the command count is OK then the command is executed via a vector table.  The command ID is used as an offset into the command table CM_Command Table, the value read at this offset is the address of the code that implements the command.  This code is jumped to.

Any output from the command is transmitted as telemetry via a sensor datastream in the case of sensor commands or the engineering data stream in the case of engineering commands.

If an error is detected then the command number and an error code is written to the housekeeping data area for transmission with the next housekeeping packet, if the incoming packet has been recognised as a command, as opposed to a DDB or not recognised at all, then an error packet is returned via the engineering datastream.

Error Codes are as follows:-

0	All OK

1	Packet already received

2	Packet length error

3	Packet checksum invalid

4	Unrecognised Command

5	Command already executed

6	Packet header invalid ID

7	Not able to execute this command in current mode

8	Command executed but failed.

### Descent Data Broadcasts

#### Mission Time

The mission time is updated each time a DDB is processed, this should occur every 2 seconds.  A 10 bit timer is synchronised with the DDB so that a 24 bit mission time with a resolution of 2ms is achieved.  This time can be used to time stamp sensor sample times and packet transmission times.

The mission time is also used to switch the SSP mode as certain events occur.  The default switching points are as follows, note that all times are assumed to be after time T0 and some mode changes will be instigated by external events:

0		Power On, Switch from cruise checkout to Upper Atmosphere Mode

10 minutes	Switch from Upper Atmosphere to Mid Atmosphere Mode

85 minutes	Switch from Mid Atmosphere to Lower Atmosphere Mode

120 minutes	Switch from Lower Atmosphere to Proximity Mode

151 minutes	Switch from Proximity Mode to Surface Mode

154 minutes	Switch from Impact to Extended Surface Mode

See Section 4.3 for a description of the DDB handling.

Timer Handler

Module		TI

Directory	TIMER

Files:		Macro		Timer.MAC

Variables	Ti.VAR

The timer handler provides a selection of macros which enable the use and updating of the mission time.  The mission time as received in the DDB is stored in the variable TI_MissionTime.  The updating of  and reading of this variable is available to all modules via the timer handler macros.

TI_Set Mission Time Rx	Updates TI_Mission Time with the contents of Rx which hold the mission time as received from the DDB.

TI_GetMission Time Rx	Loads the contents of TI_Misson Time into register Rx.

TI_Get2ms Timer Rx	Loads Rx with the value of the 2ms timer that is synchronised to the DDB.

TI_Reset2ms Timer	Resets the 2ms timer to 0, this enables the timer to be synchronised with the DDB.

TI_ReadMission Time Rx Rx+1	Reads the full 24 bit mission time into register Rx and the top 8 bits of register Rx+1.  This is a combination of the TI_Mission Time and the 2ms timer.  See Section 3.1.1.  Used to time stamp sensor samples.

#### Altitude

The altitude of the spacecraft from the surface of Titan is transmitted to SSP via the DDB, SSP copies the altitude into the variable HK_Altitude.  The altitude measured is used to switch SSP from Lower Atmosphere mode to Proximity mode 3km above the surface.

#### Mission Phase

The mission phase is one of the following:

Hex		Mission Phase

00		Entry/Descent

03		Ground Checkout

0C		Ground Checkout Suspended

0F		Ground Checkout Deactive

F3		Cruise Checkout

FC		Cruise Checkout Suspended

FF		Cruise Checkout Deactivated

SSP shall use the mission phase to:

Ensure that any experiment that may be damaged by operation in vacuum or 0g is operated in any phase in a manner such that damage will not be sustained. The mission phase is stored by SSP in the bottom 8 bits of the variable HK_Phase.

#### Spin

This is transmitted to SSP via the DDB and indicates the rate of spin of the spacecraft.  This is not used by SSP but is stored in the top 8 bits of the variable HK_Phase.

#### CRC

SSP checks the CRC on all received DDBs and telecommands.  An invalid CRC results in the DDB/telecommand being rejected. The same CRC algorithm is also used to check the integrity of EEPROM patches and the contents of the PROM and EEPROM.

### SSP Command Types

| Command ID (Code) | Function | Parameters | Eng. Packet? |
| --- | --- | --- | --- |
| 0 | Read RAM | Address, Length | Y |
| 1 | Write Ram | Address, Length, Data |  |
| 2 | Read EEPROM | Address, Length | Y |
| 3 | Write EEPROM | Address, Length, Data |  |
| 4 | Read PROM | Address, Length | Y |
| 5 | Verify RAM | Block (Low or High) | Y |
| 6 | Verify EEPROM | - | Y |
| 7 | Verify PROM | - | Y |
| 8 | Clear EEPROM | - |  |
| 10 | Read DRB | DRB Address | Y |
| 11 | Write DRB | DRB Address, Data Word |  |
| 12 | Execute DRB Sequence | Length, Commands | Y |
| 12 | Read I/O | I/O Address | Y |
| 14 | Write I/O | I/O Address, Data Word |  |
| 15 | Execute I/O Sequence | Length, Commands | Y |
| 16 | Read Parallel Port | Port Number | Y |
| 17 | Write Parallel Port | Port, Data, Mask |  |
| 18 | Write Telemetry | Channel, Length, Data Words |  |
| 19 | Reset Data Stream | Channel Mask |  |
| 20 | Read Processor Status | - | Y |
| 21 | Execute Code | Length, Code Words |  |
| 22 | Send Housekeeping Pkt | - |  |
| 23 | Read 12 bit A/D | Multiplexer Address, # Samples | Y |
| 24 | Read 16 bit A/D | Multiplexer Address, # Samples | Y |
| 30 | Stimulate ACC-E | Pulse Length |  |
| 31 | Test ACC_E | Pulse Length |  |
| 32 | Readout ACC-E | - |  |
| 33 | Test ACC_I | Pulse Length |  |
| 34 | Stimulate ACC-I | Pulse Length |  |
| 35 | Readout ACC-I | - |  |
| 36 | Readout API-S | Mode (for tele packet format) |  |
| 37 | Sample API-V | # Samples |  |
| 38 | Sample Den | # Samples |  |
| 39 | Sample Per | # Samples |  |
| 40 | Readout Ref | Illumination |  |
| 41 | Readout THP | Wire Number / Mode |  |
| 42 | Sample TIL |  |  |
| 50 | Set SSP Mode | Mode Number, Lock Option |  |
| 51 | Write Patch | Patch Number, Data |  |
| 52 | Read Patch | Patch Number | Y |
| 53 | Delete Patch | Patch Number |  |

## Use of Broadcast Pulse

A broadcast pulse is sent from the CDMU to SSP every 1/8th of a second, this pulse is made available to the processor as an interrupt.  The SSP spacecraft interface counts 16 broadcast pulses and generated a 2 sec interrupt.  The s/w can synchronise this pulse so that the processor is interrupted when a broadcast message is received.  In practice these two interrupts are not enabled but the interrupt pending flags are used to indicate that a broadcast message or pulse has occurred.  This enables the mission time to be kept up to date.

The 2 sec interrupt is also used to reset an onboard 10 bit counter.  This gives the SSP time a resolution of 2ms.

During initialisation a watchdog interrupt, the BCP 1/8th sec interrupt and BCP 2 sec interrupt are started.

Then the BCP is synchronised to the DDB such that the 2 sec interrupt occurs on the BCP pulse after the DB has been received.  If no DDBs occur in 8 consecutive DDB time periods then an error flag is set in the housekeeping indicating that there is no DDB/BCP synchronisation.

Once SSP is running, the three interrupts service the BCP as follows:

The 2 sec BCP interrupt checks to see that a DDB has been received, if it has then the interrupt updates the mission time from the DDB.  If it has not then the mission time is incremented by 2 seconds.  After DDB failures then the DDB is ignored, a DDB failure is flagged in the housekeeping and the mission time updated automatically by 2 seconds on each 2 second interrupt.

The 1/8th sec BCP interrupt triggers a watchdog timer interrupt.  The watchdog timer interrupt checks to see if a 2 sec interrupt should have occurred.  If it has not then the Watchdog timer simulates a 2 sec BCP interrupt.

The timer is set up to timeout after 350ms (350ms = 2 BCP times + 100ms).  If the timer times out then the 1/8th sec BCP interrupt has probably failed.  the timer flags a 1/8th sec BCP failure in the housekeeping and reprograms itself to timeout every 125ms (1/8th sec) so taking over from the 1/8th sec interrupt.

## Telemetry

Telemetry packets are transmitted via a telemetry FIFO to the CDMS.  There is a separate FIFO for each of the two telemetry channels.  The processor can determine if the last telemetry packet has been transmitted by reading the FIFO empty flag.  When the processor recognises that the last packet has been transmitted it can load the FIFO with a new packet.  The processor can reset the FIFOS clearing them of any data.  The FIFOS will be reset on power up.

### Telemetry Handler

Module		TM

Directory	TELEM

Files:		Code		Telem.ASM

Variables	Tm.VAR

Constants	Tm.CON

Make		Telem.MAK

The telemetry handler is responsible for taking data from the data streams inserting this data into a packet and then loading the packet into the two telemetry FIFOS.

The telemetry handler is executed by the main background program loop, so it can be interrupted by the task scheduler if a sensor needs servicing.

When the handler starts executing it first checks to see if either of the telemetry FIFOS are empty.  If they are both full then the handler exits.

If one or both FIFOS are empty then the handler checks the impact data stream, if there is enough data in the datastream to fill a packet then an impact packet is constructed and loaded into both telemetry FIFOS.  A FIFO is not loaded if there is not enough data in the impact data stream to fill a packet then the other data streams are polled in turn.  If any data stream has enough data to fill a packet then the data is inserted into a packet and the packet is loaded into one or both FIFOS.  If the engineering data stream is polled then if it has any data in it then this data is loaded into a packet.  The empty packet space is padded out with (AAAAhex).  The packet is then loaded to one or both telemetry FIFOS.

The handler is designed so that if one telemetry channel fails then it shall transmit all the telemetry on the other channel.  In normal operation the telemetry shall be transmitted on both channels.

# SENSOR INTERFACE HANDLING

## ACC-I

The ACC-I sensor is an accelerometer that measures the acceleration of the spacecraft.  This sensor is used as the main impact detector.

### ACC-I Module

Module		ACC-I

Directory	ACC-I

Files:		Code		ACC-I.ASM

Variables	ACC-VAR

Constants	ACC-I.CON

Make		ACC-I.MAK

### Sampling

Functions:	ACC_I_Sample

ACC_I_ 500 Hz Sample

The sample function selects the ACC-I sensor input and reads the output of the sensor.  The reading is returned in register RO.

The 500 Hz sampling function takes a reading of the ACC-I sensor using ACC_I_ Sample, the reading has the ACC-I offset subtracted from it and the resulting value is stored in a circular buffer.  This function is used by the ACC-I command functions and by the ACC-I proximity task.  The ACC-I offset is subtracted from the ACC-I reading so that the impact detection mechanism does not trigger on the offset signal.

### Checkout

Functions:	ACC_I_ Checkout Task

ACC_I_ Checkout Process

ACC_I_ Process Impact Data

The ACC-I checkout task is run when SSP enters Mode 1, this task applies a stimulation signal to the sensor electronics and reads the sensor at 500 Hz for 512 samples.  when the task terminates the ACC-I ready flag is set to indicate that there is data to be processed.  The background ACC-I process detects the ACC-I ready flag and processes the ACC-I data buffer.  The peak signal position and amplitude is found as well as the FWHM positions, this data is saved in the checkout packet.  The rest of the checkout packet is filled with data from the buffer, 4 samples are taken from the beginning and end of the buffer and the region around the peak is also copied to the checkout packet.  As the data is 12 bit data it is compressed such that 4 samples are compressed into three 16 bit words.  The checkout packet fits exactly into a Huygens packet so that all the data is transmitted in a single packet.

### Atmospheric Modes

TBD

Function:	ACC_I_ Sample

ACC_I_ Task

ACC_I_ Atmosphere Process

The ACC-I sensor is sampled at 500 Hz for 0.4 seconds.  Once finished the ACC-I ready flag is set.  The background process detects the ready flag and processes the data.  The maximum, mean and variance of the 200 samples are found.  These values are loaded to the ACC-I data stream.  The mean value is added to a running total of mean values and an ACC-I offset is calculated from this which is saved in the housekeeping data.

This measurement is repeated every second (i.e. the data contains the max, mean and variance of the first 0.4 seconds of every second).

#### Task Timeline

Function:	ACC_I_ Task

Time (ms)			Action

0			Determine Sample Time

2			Read 1st Sample

4			Read 2nd Sample

6			Read 3rd Sample

:			:		:

308		Read 199th Sample

400		Read 200th Sample and Set ACC-I Ready Flag

### Proximity Mode

Function:		ACC_I_ 500 Hz Sample

ACC_I_ Proximity Task

ACC_I_ Look for Impact

ACC_I_ Process Impact

ACC_I_ Process Impact Data

A sample of the ACC-I sensor is taken every 2ms (500 Hz) using the 12 bit ADC and is stored in a circular buffer (512 word FIFO). After each acquisition, the latest sample is compared to the detection threshold and, if greater, the Acc_I_TotalSignal count is incremented. The latest value but 5 is also compared to the threshold and, if greater, the Acc_I_TotalSignal count is decremented. Thus Acc_I_Total signal contains a count of the number of samples exceeding the threshold in the last 5.

The threshold for impact detection is initialised in the code to 80 TM counts which corresponds to approximately 3.5 g (2048 counts = approx 90g)

The number of samples above threshold to indicate impact, Acc_I_ExceedLimit, is initialised in the code to 3. An impact is assumed if Acc_I_Total reaches 3.

After impact is detected the sensor continues to be read at 500 Hz for a further 448 samples.

#### Task Timeline

Task Name:		ACC_I_ Proximity Task

Time (ms)			Action

0				Read Sample Calculate Total Signal

2				Read Sample Calculate Total Signal

4				Read Sample Calculate Total Signal

:				:		:		:

Impact-2		Read Sample Calculate Total Signal

Impact			Freeze ACC-I

Impact + 2		Read Sample

Impact + 4		Read Sample

:			:		:

Impact + 894	Read Sample

Impact + 896	Set Impact Detected Flag, Terminate Task

### Surface Modes

See Atmospheric Modes Section 5.1.4.

## DEN

The Den sensor measures the density of the liquid in which the Den sensor is immersed.  The sensor is operational throughout the mission at 1 Hz. The atmospheric measurements allowing offset drift and temperature changes to be monitored (this may also provide secondary information on turbulence and probe accelerations).

### Den Module

Module		Den

Directory	Den

Files:		Code		Den.ASM

Variables	Den.VAR

Constants	Den.CON

Make		Den.MAK

### Sampling

Function:	Den_Sample

The Den sensor is read simply by selecting the multiplexer address for Den and reading the output voltage of the sensor using the 12 bit ADC.  The sensor is read 8 times and the average of the 8 reading is calculated.

### Processing

Function:	Den_Process Sample

Den_Task

Firstly a Den sample is taken with Den disabled; this reading is saved in the housekeeping packet data area and is the Den offset.  Den is then sampled 4 times with Den being enabled for each reading.  These 4 samples are compressed into 3 16 bit words and loaded to the Den datastream for transmission.

### Task Timeline

Function:	Den_Task

Time (ms)			Action

0			Determine Sample Time, Sample Den Offset and then Enable Den

4			Sample Den, Disable Den

1000		Enable Den

1004		Sample Den, Disable Den

2000		Enable Den

2004		Sample Den, Disable Den

3000		Enable Den

3004		Sample Den, Disable Den

3006		Process Sample

3008		Transmit Sample

## PER and CON

The PER sensor measures the permittivity of the fluid in which the sensor is immersed.  The sensor is operational throughout the mission.

The conductivity of the fluid can be measured by the PER sensor when it is not clocking, this is known as CON.

### Per Module

Module		PER

Directory	PER

Files:		Code			PER.ASM

Variables		PER.VAR

Constants		PER.CON

Make			PER.MAK

### Sampling

Functions:	PER_Sample

CON_Sample

The PER sensor is read by reading the sensor via the 12 bit ADC, CON has to be disabled and PER has to be clocked for 1 second before taking a reading.

The CON sensor is read by reading the PER input via the 12 bit ADC, CON has to be enabled for 1 second before taking the reading and PER clocking has to be disabled during the ADC conversion.

### Processing

Functions:	PER_Process

PER_Task

When the PER task starts at each mode change PER and CON are read with CON disabled and PER not clocking, these two readings are the offset measurements for PER and CON and are saved in the housekeeping data area.

CON is then enabled and sampled followed by PER being enabled (CON disabled) and sampled.  CON and PER are sampled again and then the 4 samples are compressed into 3 16 bit words before being transmitted.  This process is repeated such that PER and CON are read once every 10 seconds.

### Task Timeline

Function:	PER_Task

Time (ms)			Action

0				Disable CON, Disable PER Clocking

1000			Sample PER and CON offsets, Enable CON

10000			Sample CON, Enable PER

11000			Sample PER, Enable CON

20000			Sample CON, Enable PER

21000			Sample PER, Enable CON, Process Samples, Transmit Data

30000			Sample CON, Enable PER

31000			Sample PER, Enable CON

40000			Sample CON, Enable PER

41000			Sample PER, Enable CON, Process Samples, Transmit Data

:				:		:		:		:		:

## REF

The REF sensor is a light sensitive array; this array has 512 pixels and is illuminated by light refracted by a prism.  The light source is provided by two LEDs.  A light/dark boundary is seen in the refracted light.  The position of this is determined by the refractive index of the liquid in which the prism is immersed (N.B. The REF sensor can only determine refractive index, R.I.  if immersed in a liquid, or if a thin film of liquid condenses on the REF prism – in which case only the internal mode can detect R.I.).

### REF Module

Module		REF

Directory	REF

Files:		Code			REF.ASM

Variables		REF.VAR

Constants		REF.CON

Make			REF.MAK

### Checkout

None.

### Upper Atmosphere

The REF sensor does not operate during upper atmosphere mode.

### Mid and Lower Atmospheric Modes

#### Sampling

Functions:	REF_Reset

REF_Clear

REF_Illuminate

REF_Readout

For each full sample the REF sensor is read out 3 times using different illumination modes.

The three modes are:

i)	Dark		-	No Illumination.

ii)	External	-	Prism Illuminated by the External LED.

iii)	Internal	-	-	Prism Illuminated by the Internal LED.

The sensor is read out as follows:

i)	Sensor readout register is initialised by clocking the start and pixel clocks.

ii)	Sensor array is cleared by clocking each pixel out but not digitising it.

iii)	The readout time is taken, the prism is illuminated as required.

iv)	Array is exposed to light for a pre-calculated exposure time.

v)	Illumination is switched off.

vi)	Sensor readout register is initialised by clocking the start and pixel clocks.

vii)	Array is readout by clocking the pixel clock and digitising each pixel with the 12 bit ADC.

#### Processing

The background task detects the REF data ready flag which is set when the REF task has completed readout and processes the REF data.  Each 12 bit pixel value is compressed to 8 bits by shifting each value left by 3 bits (dividing by 8) and storing the LSB which loses the most significant bit.  The 512 pixels are therefore compressed into 512 bytes which are loaded to the REF datastream for transmission.

#### Task Timeline

Function:	REF_Task

Time (ms)			Action

0				Reset Sensor

1				Clear 6 Pixels

2				Clear 6 Pixels

:				:	:	:

85				Clear Last 6 Pixels

86				Reset sensor, internally illuminate prism for pre-calculated internal exposure time, read sample

time, expose

Exposure		Turn off illumination

Exposure +1		Read 6 Pixels

Exposure + 2		Read 6 Pixels

:				:	:	:

Exposure + 85		Read Last 8 Pixels

Exposure + 86		Set REF Data Ready Flag

The above process is repeated at 16000 ms with no illumination, and at pre-calculated external exposure time (this equals 50 x internal exposure time) with external illumination.

#### Pre-Calculated Exposure Time

The REF module has a table of 330 points giving the required internal mode exposure time for a measured sensor temperature in 1 K steps from 0 K to 329 K (although, as initialised, the exposure only changes at 10K intervals). The measured temperature used is nominally taken from the REF_SENSE_DIODE. If REF_SENSE_DIODE were to fail then the software would default to the other REF temperature diodes (first REF_PRBASE and then REF_PRTIP).

The following equation is used by the software to derive diode temperature (in K) from the diode sensor output (in bits). (N.B. This may differ very slightly to the preferred conversion formula for the REF SENSE DIODE, but was the best available when software was written. In fact the difference is less than 4 K at around 100 K temperature).

REF_SENSE_DIODE_TEMP (K) = 433.085 - 0.204*REF_SENSE Output (bits)

The table is given below:

;       Exposure times in ms for internal illumination mode for temperatures 0K - 329K

;       External mode illumination time  = Internal mode illumination time * 50

Ref_Internal Mode Exposure Times in ms:	       ; Temperature in K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 0K - 9K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 10K - 19K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 20K - 29K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 30K - 39K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 40K - 49K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 50K - 59K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 60K - 69K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 70K - 79K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 80K - 89K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 90K - 99K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 100K - 109K

DW      0020,0020,0020,0020,0020,0020,0020,0020,0020,0020       ; 110K - 119K

DW      0022,0022,0022,0022,0022,0022,0022,0022,0022,0022       ; 120K

DW      0022,0022,0022,0022,0022,0022,0022,0022,0022,0022       ; 130K

DW      0025,0025,0025,0025,0025,0025,0025,0025,0025,0025       ; 140K

DW      0030,0030,0030,0030,0030,0030,0030,0030,0030,0030       ; 150K

DW      0040,0040,0040,0040,0040,0040,0040,0040,0040,0040       ; 160K

DW      0040,0040,0040,0040,0040,0040,0040,0040,0040,0040       ; 170K

DW      0050,0050,0050,0050,0050,0050,0050,0050,0050,0050       ; 180K

DW      0050,0050,0050,0050,0050,0050,0050,0050,0050,0050       ; 190K

DW      0060,0060,0060,0060,0060,0060,0060,0060,0060,0060       ; 200K

DW      0077,0077,0077,0077,0077,0077,0077,0077,0077,0077       ; 210K

DW      0087,0087,0087,0087,0087,0087,0087,0087,0087,0087       ; 220K

DW      0095,0095,0095,0095,0095,0095,0095,0095,0095,0095       ; 230K

DW      0115,0115,0115,0115,0115,0115,0115,0115,0115,0115       ; 240K

DW      0120,0120,0120,0120,0120,0120,0120,0120,0120,0120       ; 250K

DW      0140,0140,0140,0140,0140,0140,0140,0140,0140,0140       ; 260K

DW      0160,0160,0160,0160,0160,0160,0160,0160,0160,0160       ; 270K

DW      0180,0180,0180,0180,0180,0180,0180,0180,0180,0180       ; 280K

DW      0230,0230,0230,0230,0230,0230,0230,0230,0230,0230       ; 290K - 299K

DW      0250,0250,0250,0250,0250,0250,0250,0250,0250,0250       ; 300K - 309K

DW      0275,0275,0275,0275,0275,0275,0275,0275,0275,0275       ; 310K - 319K

DW      0300,0300,0300,0300,0300,0300,0300,0300,0300,0300       ; 320K - 329K

### Proximity Mode

As for atmospheric modes except that only 1 pixel is read per timeslice.

### Impact

REF is read out in all three modes and the data transmitted via the REF datastream.  The data is used to calculate the refractive index of the liquid and this is inserted into the impact packet along with samples from the boundary points.

The internal refractive index is calculated by subtracting the dark readout data from the internal readout data.  The resulting values are then smoothed by averaging each value over 8 values.  The dark/light boundary can be determined by finding the maximum positive gradient of the smoothed data.  The external refractive index is found in a similar way except that the light/dark boundary is determined by finding the maximum negative gradient.

Note that due to the REF linearity range (first 400 pixels only) only the first 400 pixels are processed.

### Surface Modes

As for Atmospheric Modes.

## TIL

The TIL sensor measures the tilt of the spacecraft in two orthogonal directions relative to the local gravity vector.  The sensor is operational throughout the mission.

### TIL Module

Module		TIL

Directory	TIL

Files:		Code		TIL.ASM

Variables	TIL.VAR

Constants	TIL.CON

Make		TIL.MAK

### Sampling

Function:	TIL_Sample

The TIL hardware is clocked by a 500 Hz square wave.  There are 4 measurements that can be taken, these are Tilt X, Tilt Y, Top x and Top Y.  The Tilt X and Y value indicate the tilt of the spacecraft in X and Y and the Top measurements indicate the driving voltage to each direction.  Each value is measured twice, once during the +ve clock phase and again 1 ms later during the -ve phase.  To read the sensor the software has to perform at least one read during each phase of the cycle.  To do this the sampling routine reads the output from the sensor twice, if the two value differ by more than 8 then the samples were taken whilst the clock was changing phase so a third sample is taken if both samples differ by less than 8 the first sample is used.  If the valid reading is positive then it is saved in the positive location for the sample otherwise the negative location is used.

### Checkout

Functions:	TIL_Disable

TIL_Sample Offsets

The electronic offset of the sensor sub-system is measured by disabling the TIL input and reading TIL as if it were operational.  The readings taken are saved in the housekeeping data.  These offsets are read at the start of each SSP mode change.

### Processing

Function:	TIL_Process

The eight 12 bit reading are compressed into six 16 bit words before transmission.

### Operation - All Modes

Function:	TIL_Task

The TIL task starts by disabling the TIL sensor and measuring the offsets.  Once this has been done TIL is re-enabled and the main task loop begins.

The task loop starts by measuring the TIL X and TIL Y values.  This process is then repeated 1 ms later for the next phase of the TIL clock.  The Top X and Top Y values are measured similarly and finally the values are compressed and loaded to the TIL datastream for transmission.

The task loop is repeated at intervals of 1 second.

### Task Timeline

Function:	TIL_Task

Time (ms)			Action

0				Disable TIL

1000			Sample TIL Offsets and Re-Enable TIL

2000			Sample TIL X and Y

2001			Sample TIL X and Y

2002			Sample Top X and Y

2003			Sample Top X and Y

2005			Compress Data and Transmit It

3000			Sample TIL X and Y

:			:	:	:

## THP

The THP sensor is designed to measure the thermal conductivity of the fluid the sensor is immersed in.  The measurement is obtained by heating one of the 4 wires and measuring the wire's resistance change over time.

### THP Module

Module		THP

Directory	THP

Files:		Code			THP.ASM

Variables		THP.VAR

Constants		THP.CON

Make			THP.MAK

### Sampling

Functions:	THP_Sample,THP_Startup, THP_SelectWire, THP_Sampling

The THP sensor is sampled as follows:

i)	Wire 1, 2 3 or 4 is selected, if the selected wire is broken then an alternative wire is chosen.

ii)	A measurement is taken with no current, this is the THP offset measurement.

iii)	Low current is applied to the wire.

iv)	The initial resistance of the wire is measured.

v)	The initial measurement has a gain factor applied and an offset added and is loaded into the offset DAC.

vi)	High current is applied to the wire.

vii)	60 readings are taken over a time period, the readings are spaced logarithmically and the time period depends on the SSP mode.

### Processing

Functions:	THP_Sampling

Each reading is the mean of 16 measurements of the sensor.  If the mean exceeds a value of 7FF016 then the high current is switched off and the rest of the data sample is filled with the last reading.

### Safety Mechanisms

Functions:	RB_16_High Current

High Current will NOT be applied to any THP wire if:

i)	The DDB indicates that we are in cruise checkout.

ii)	The DDB indicates that we are in descent phase AND the mission time is <T0 + 10 minutes.

### Checkout

Function:	THP_Checkout Task

For the first 'THP' sample in Mode 1 a check of the A/D calibration is performed.

The checkout consists of loading a series of 31 increasing values to the DAC; both the 12 and 16 bit ADCs are used to measure output.  The results are stored in a standard THP sample with the first 31 samples being the 16 bit results and the second 31 values the 12 bit results.  The results can be used to determine if both ADCs and the DAC are operating correctly.

The 31 values are (in hex):

| 0000 | 000C | 0018 | 0024 | 0030 | 003C | 0048 | 0054 | 0060 | 006C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0078 | 0084 | 0090 | 009C | 00A8 | 00B4 | 00C0 | 00CC | 00D8 | 00E4 |
| 00F0 | 00FC | 0108 | 0114 | 0120 | 012C | 0138 | 0144 | 0160 | 0168 |
| 0174 |  |  |  |  |  |  |  |  |  |

### Upper Atmosphere Mode (1)

Function:	THP_Upper Task

Wires 3 and 4 are sampled for a period of 60 seconds.

Note that in Cruise checkout and Descent low current only will be used.  In ground checkout high current will be used.  See 5.2.1.4 above.

### Mid Atmosphere Mode (2)

Function:	THP_Mid Task

A wire is sampled every 60 seconds.  Wires 3 and 4 are each sampled for 15 seconds followed by 105 seconds off.

### Low Atmosphere Mode (3)

Function:	THP_Lower Task

A wire is sampled every 60 seconds.  Wires 3 and 4 are each sampled for a period of 15 seconds followed by 105 seconds off.  If both wires are broken then wires 1 and 2 are used.

### Proximity Mode (4)

Function:	THP_Proximity Task

A wire is sampled every 60 seconds.  Wires 3 and 4 are each sampled for a period of 15 seconds followed by 105 seconds off.  If both wires are broken then wires 1 and 2 are used.

### Surface Mode (5)

Function:	THP_Surface Task

A wire is sampled every 15 seconds.  All 4 wires are each sampled for a period of 5 seconds followed by 55 seconds off.  Wires 1 and 2 are sampled alternately with wires 3 and 4 i.e. the sampling sequence is 1, 3, 2, 4...(or 2, 3, 1 ,4 depending on which wire was sampled first at the start of the mode).

### Extended Surface Mode (6)

Function:	THP_Extended Task

Wires 1 and 2 are sampled for 15 seconds and wires 3 and 4 sampled for 25 seconds.  Again wires 1 and 2 are sampled alternately with wires 3 and 4 i.e. the sampling sequence is 1, 3, 2, 4...(or 2, 3, 1, 4 depending on which wire was sampled first at the start of the mode).  So wire 1 or 2 will be on for 15 seconds, all off for 30 seconds, 3 or 4 on for 25 seconds, all off for 30 seconds.

### Sample Timings

These tables define the time gap in ms between each of the 60 samples.

| 5 Second Sample |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 3 | 4 | 4 | 6 | 6 | 7 | 9 | 9 |
| 11 | 13 | 15 | 18 | 20 | 23 | 27 | 31 | 37 | 42 |
| 48 | 56 | 66 | 75 | 87 | 102 | 116 | 136 | 158 | 182 |
| 210 | 244 | 284 | 328 | 378 | 440 | 510 | 590 | 682 |  |

| 15 Second Sample |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 4 |
| 4 | 5 | 6 | 7 | 8 | 10 | 11 | 13 | 16 | 19 |
| 22 | 26 | 31 | 36 | 43 | 51 | 59 | 71 | 83 | 98 |
| 116 | 136 | 162 | 190 | 226 | 266 | 312 | 370 | 438 | 514 |
| 608 | 718 | 848 | 1000 | 1182 | 1392 | 1646 | 1942 | 2290 |  |

| 25 Second Sample |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 4 | 4 |
| 5 | 7 | 7 | 9 | 11 | 12 | 15 | 18 | 21 | 26 |
| 30 | 36 | 42 | 51 | 61 | 72 | 86 | 102 | 122 | 146 |
| 172 | 206 | 246 | 290 | 348 | 414 | 494 | 586 | 698 | 832 |
| 992 | 1180 | 1404 | 1674 | 1992 | 2372 | 2824 | 3364 | 4004 |  |

| 60 Second Sample |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 2 | 3 | 5 | 5 | 6 |
| 8 | 9 | 11 | 14 | 16 | 20 | 24 | 29 | 35 | 42 |
| 52 | 61 | 75 | 91 | 108 | 132 | 160 | 194 | 232 | 282 |
| 342 | 412 | 498 | 602 | 730 | 880 | 1064 | 1286 | 1556 | 1880 |
| 2274 | 2746 | 3322 | 4016 | 4854 | 5868 | 7096 | 8574 | 10368 |  |

## ACC-E

ACC-E is sampled at 10 kHz by hardware.  The ACC-E analogue data is converted to an 8 bit value using a hardware pseudo logarithmic conversion technique.  The converted data is then loaded into a 512 byte FIFO.  When the hardware detects an impact (Set in hardware as a 177 mV threshold at the output of the pseudo logarithmic amplifier. This equates to a signal of 80 mV at the ACC-E output, equivalent to an input force of 3.9 N at room temperature or 7.2 N at liquid nitrogen temperature) the data is frozen in the FIFO so that approximately 64 samples have been taken before the impact and the rest have been taken after impact.  The hardware can indicate to the processor that impact has occurred and the processor can then read the contents of the FIFO.  The processor can restart the ACC-E hardware once an impact has been detected.

## API-V

The API-V sensor measures the local velocity of sound.

### API-V Module

Module		API-V

Directory	API

Files:		Code			API-V.ASM

Variables		API.VAR

Constants		API.COn

Make			API.MAK

### Checkout

None.

### Operation - All Modes

Sampling Rate:		1 Hz

Data Rate:		1.67 bytes/second

### Sampling

Functions:	API_V_Trigger

API_V_Read Sample

The API-V sensor is triggered by reading the API-V trigger register on the API sensor electronic board.  Two velocity readings are taken in opposite directions.  These results are available 10 ms after triggering and can be read as two 12 bit numbers from the data registers on the sensor electronics board

### Processing

Function:	API_V_Compress Sample

Two samples of two 12 bit measurements are compressed into three 16 bit words.

### Task Time

Function:	API_V_Task

Time (ms)			Action

0	Determine Sample Time and Trigger First API-V Sample

10	Read 1st Sample and Trigger Second API-V Sample

20	Read 2nd Sample

22	Process Samples

24	Transmit Samples

## API-S

The API-S sensor is an acoustic sounder.  It emits acoustic signals and receives any reflections of the signal from solid or semi-solid objects.

The API-S sensor is triggered by writing to the API-S trigger register on the API sensor electronics board.  The sensor emits a signal and waits for the return signal.  The return signal is available to be read 1 second after triggering.  The return signal is stored in a 1000 byte buffer on the sensor electronic board and can be read by reading the API-S data register.

### API-S Module

Module		API-S

Directory	API-S

Files:		Code			API-S.ASM

Variables		API.VAR

Constants		API.CON

Make			API.MAK

### Checkout

None.

### Upper Atmosphere Mode

There is no operation of the API-S sensor during upper atmosphere mode.

### Mid and Lower Atmospheric Modes

Sampling Rate:	1 Sample every 2 seconds (minimum).

#### Sampling

Functions:	API_S_Trigger

API_S_Read Sample

The sensor is triggered every two seconds during mid and lower atmosphere modes.  One second after triggering API-S the API-S ready flag is set.  This indicates to the background process that the data can be processed.  Once the data has been processed the flag is cleared.  The background process as well as processing the data can judge if there is any spare data rate available for API-S and speed up or slow down the sampling cycle as required.  When the ready flag is cleared the cycle time is updated and a new sample taken at the start of the next sample cycle.

#### Processing

Function:	API_S_Atmosphere Compress

Only bytes 50 to 570 are used of the total 1000 byte sample.  These 520 bytes are compressed into 26 bytes by taking the mean of each of 26 blocks of 20 bytes.

For Modes 2 and 3 ONLY the bins packing order in telemetry packets is swapped pair-wise as shown by the figure below (i.e. “Little Endian” storage format).

(N.B. Titan PDS data are corrected but any older, non PDS complaint data sets may need correcting).

Actual order of bins  (where each bin is average of 20 samples)

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |

Order of bins in telemetry packets

| 2 | 1 | 4 | 3 | 6 | 5 |
| --- | --- | --- | --- | --- | --- |

#### Task Timeline

Function:	API_S_Task

Time (ms)			Action

0				Get Sample Time and Trigger API-S Sample

1000			Set API-S Ready Flag

1100			See if API-S Ready flags Clear - Repeat every 100 ms until Clear

Modify API-S Cycle Time.

Note that a time slice slot is reserved for API every 100 ms, this enables the cycle time to be varied by multiples of 100 ms.

### Proximity Mode

Sampling Rate:	1 Sample every 2 seconds (Minimum)

#### Sampling

Function:	API_S_Pulse Length

API_S_Trigger

API_S_Read Sample

As for atmospheric mode except the pulse width may be set to short (2 ms) at an altitude below 1 km (provided that the “measured altitude” flag is set in the DDB. See section 2.4.5.) by setting the pulse width signal to the API-S sensor electronics board.

#### Processing

Function:	API_S_Proximity Compress

The peak signal (see section 3.2.5.4) is found in the return signal then the data is compressed as follows :

Peak Signal 	  30 Values 	-	Not compressed (61 bytes)

Peak Signal 100 Values		-	Compress into bins of 4 samples/bin (35 bytes)

Rest				-	Compress into bins of 20 samples/bin (42 bytes)

#### Task Timeline

As for Mid and Lower Atmosphere.

### Surface Mode

As for proximity mode.  Data for first sample in this mode is further analysed on the data stored in the impact packet.

### Extended Surface Mode

Sampling Rate:	10 Samples at one second intervals every 30 seconds

Data Rate:		26.17 bytes/sec.

#### Sampling

As for atmospheric mode except cycle time is nominally 10 seconds followed by a 20 second gap.

#### Processing

Function:	API_S_Extended Process

The sample which contains the return signal is processed as for proximity mode, all 10 samples are compressed into 50 bins of 20 samples.

#### Task Time

Function:	API_S_Surface Task

Time (ms)				Action

0			Get sample time and trigger 1st API-S sample

1000			Set API-S ready flag

1100	If ready flag clear trigger 2nd sample (100 ms repeat if flag not clear)

2100	Set API-S ready flag

2200	If ready flag clear trigger 3rd sample (100 ms repeat if flag not clear)

3200	Set API-S ready flag

3300	If ready flag clear trigger 4th sample (100 ms repeat if flag not clear)

4300	Set API-S ready flag

4400	If ready flag clear trigger 5th sample (100 ms repeat if flag not clear)

5400	Set API-S ready flag

5500	If ready flag clear trigger 6th sample (100 ms repeat if flag not clear)

6500	Set API-S ready flag

6600	If ready flag clear trigger 7th sample (100 ms repeat if flag not clear)

7600	Set API-S ready flag

7700	If ready flag clear trigger 8th sample (100 ms repeat if flag not clear)

8700	Set API-S ready flag

8800	If ready flag clear trigger 9th sample (100 ms repeat if flag not clear)

9800	Set API-S ready flag

9900	If ready flag clear trigger 10th sample (100 ms repeat if flag not clear)

10900	Set API-S ready flag

11000	If ready clear wait for 60 seconds before restarting sampling

Count reset to starting a new sample.  This means that at a mode change there will be some incomplete samples so the EGSE must be aware of this.  In the case of the continuous data streams the data samples may lose synchronisation with the packets.

# routing bus

## Overview

The routing bus provides a redundant control/data path between the LPP and the various peripherals.  The data routing bus is entirely separate to the main processor bus so that any problems on the data routing bus do not affect the processor.  The two buses are nominated the primary bus and the redundant bus.  The SST experiment splits the experiment electronics between the two buses as follows:

Primary Routing Bus:			Spacecraft Interface A

12 Bit Analogue Board

Redundant Routing Bus:		Spacecraft Interface B, including EEPROM

16 Bit Analogue Board

API Board

## Routing Bus Handlers

Module		ROUTING

### Address Map

Filename:	RB_Mem.MAP

The Routing Bus is mapped into the processor's memory space at address FC0016 the sub-system addresses are as follows:

Spacecraft I/F			FC70

LPP				FC78

16 Bit Board			FCB0

12 Bit Board			FCD0

API Board			FCE0

Note that the A or B spacecraft interface is selected by using the primary or redundant routing bus.

### Low Level Handler

Filenames:		Code		RB_Utils.ASM

Macro		RB_Utils.MAC

Make		RB.MAK

The low level handler provides a set of functions and macros to enable other handlers to use the bus.

Function					Description

RB_Initialise	Initialises the data routing bus by mapping it into memory, the 12 bit board is then reset.  This function has to be called before any access to the routing bus can be made.

Macro	Parameters			Description

RB_Write	Rx.Address	Writes the data held in register Rx to routing bus address.  Note that the routing bus base address is automatically added to Address before writing the data.  No check is made to see if the data was received OK.

RB_Write	Rx.Address, Ry	Same as above except the address written to is Address + Contents of Ry.

RB_Read	Rx.Address	Reads a value from address of the routing bus and returns the value read in Rx.  Note that no check is made to see if data is really present.

RB_Read	Rx.Address, Ry	Same as above except the address read from is the sum of Address + Contents of Ry.

RB_Set Active Bus	Bus	Activates the Primary or Redundant bus.  Bus should be RB_Primary Bus or RB_Redundant Bus.

### 12 Bit Board Handler

Module		RB_12

Filenames:		Code			RB_Adc.ASM

Macro			RB_Adc.MAC

Variable		RB_Adc.VAR

Make			RB.MAK

The 12 bit board handler provides a set of functions and macros to enable the 12 bit sensor handlers to control and sample 12 bit sensors.

Function						Description

RB_12_SetMuxAddr	This sets the multiplexer address to the address held in register R1, this enables the ADC to sample data from more than one device.  The mux address must be set up before reading data from the ADC.  See RB_Mem.MAP for valid multiplexer addresses.

RB_12_ReadAdc	This function uses the 12 bit ADC to convert the signal at the current multiplexer address.  The value of the signal sampled is returned in register RO.

RB_12_FastReadAdc	As the ADC requires up to 30s to perform a sample this function enables processing to continue whilst the ADC is performing a conversion.  The first call will start the ADC conversion process and the second call will return the data sampled by the ADC and also start a second conversion.  A third call will return the data from the second conversion and start a third.  There must be 30 s between each call to this function to give the ADC time to convert.

RB_12_Reset	This function resets the 12 bit board and sets the multiplexers and control signals to a known state.

Macro	Parameters			Description

RB_12_Raise	Signal	Raises the signal control line signal.  See RB_Mem.MAP for signal definitions.  Note that the signal is not actually raised until RB_12_apply Signals is called.

RB_12_Lower	Signal	As above except signal is lowered.

RB_12_Toggle	Signal	As above except signal state is inverted.

RB_12_Set Signals Rx		Sets the state of all the signals to the contents of Rx where each bit in Rx refers to a specific signal.  Note RB_12_ApplySignals should be used afterwards to set the physical control signals.

RB_12_GetSignalsRx		Returns the current state of the control signals in Rx.  This will reflect the state of the physical control signals only if RB_12_ApplySignals was last used.

RB_12_Apply Signals Rx	Applies any control signal changes using the above macros to the physical control signals.  The current state of the control signals is returned in Rx.

### Spacecraft Interface Handler

Module			RB_SC

Filenames:		Code		RB_SC.ASM

Macro		RB_SC.MAC

Make		RB.MAK

The spacecraft interface handler provides a set of functions and macros to enable the command and telemetry sensor handlers to manage control and data to and from the main spacecraft.

Function					Description

RB_SC_Read/ML.MLFifo	Reads the command FIFO, the number of words to read is passed in R0 and the address to write the data in R2.  On exit R0 holds the number of data words not read by the routine, i.e. it should be 0 if all data was read.

RB_SC_WriteTMFifo	Writes data to the telemetry FIFO, the number of words to write is passed in R0 and the address of the data in R2.  R0 should be 0 if all the data was written to the FIFO.

RB_SC_TMFifosEmpty	This function checks both telemetry FIFO and sets the zero flag if one or both of them are empty.

Macro	Parameters			Description

RB_SC_Status	Rx	Returns the contents of the spacecraft interface status register.  This indicates the current state of the command and telemetry FIFOs.

RB_SC_FifoEmptyFifo		Rx	Sets the zero flag if FIFO is empty.  FIFO should be TM or CM.  The state of the zero flag is returned in register Rx.

RB_SC_FifoLoading 	Rx	Sets the zero flag if the command FIFO is being loaded.  The state of the loading flag is returned in register Rx.

RB_SC_TmWrite Word 		Rx		Write the contents of Rx to the telemetry FIFO.

RB_SC_MIReadByte 	Rx		Reads a byte from the command FIFO into Rx.

RB_SC_PacketReady		Sets the packet ready signal

### 16 Bit Board Handler

Module	RB_16

Filenames:		Code		RB_16.ASM

Macro		RB_16.MAC

Variable	RB_16.VAR

Make		RB.MAK

The 16 bit board handler provides a set of functions and macros to enable the 16 bit sensor handlers to control and sample 16 bit sensors.