ROSETTA-ALICE

TO PLANETARY SCIENCE ARCHIVE

INTERFACE CONTROL DOCUMENT

February 2018

SwRI® Project 02353

Document No. 8225-EAICD-01

Contract JPL 960549

Prepared by

Department of Space Studies

1050 Walnut Street, Suite 400

Boulder, CO 80302

(303) 546-9670

ROSETTA-ALICE

TO PLANETARY SCIENCE ARCHIVE

INTERFACE CONTROL DOCUMENT

SwRI Project 02353

Document No. 8225-EAICD-01

Contract JPL 960549

Prepared by:  Joel Wm. Parker					16 June 2004

Andrew J. Steffl					26 March 2010

Andrew J. Steffl					07 November 2016

David E. Kaufmann					18 February 2018

Approved by:  ____________________________________	Date:  ____________

S. Alan Stern

Approved by:  ____________________________________	Date:  ____________

L. D. McCullough, SwRI Project CM

TABLE OF CONTENTS

Page

1.	Introduction	1

1.1	Purpose and Scope	1

1.2	Archiving Authorities	1

1.2.1	ESA’s Planetary Science Archive (PSA)	1

1.3	Contents	1

1.4	Intended Readership	1

1.5	Applicable Documents	1

1.6	Relationships to Other Interfaces	2

1.7	Acronyms and Abbreviations	2

1.8	Contact names and Addresses	2

2.	Overview of Instrument Design, Data Handling, and Product Generation	2

2.1	Instrument Design	2

2.1.1	Hardware and Characteristics	2

2.1.2	Science Modes	5

2.2	Scientific Objectives	6

2.3	Data Handling Process	7

2.3.1	Responsible Individuals	7

2.3.2	Data Flow	8

2.3.3	Processing Software	8

2.3.4	Archive Data Levels	8

2.4	Overview of Data Products	9

2.4.1	Pre-Flight Data Products	9

2.4.2	Instrument Calibrations	9

2.4.3	In-Flight Data Products	9

2.4.4	Software	14

2.4.5	Documentation	14

2.4.6	Derived and other Data Products	14

2.4.7	Ancillary Data Usage	14

3.	Archive Format and Content	14

3.1	Format and Conventions	14

3.1.1	Deliveries and Archive Volume Format	14

3.1.2	Data Set ID Formation	14

3.1.3	Data Directory Naming Convention	15

3.1.4	Filenaming Convention	16

3.2	Standards Used in Data Product Generation	17

3.2.1	PDS Standards	17

3.2.2	Time Standards	17

3.2.3	Reference Systems	19

3.2.4	Other Applicable Standards	19

3.3	Data Validation	19

3.4	Content	19

3.4.1	Volume Set	19

3.4.2	Data Set	19

3.4.3	Directories	20

4.	Detailed Interface Specifications: data product design	23

4.1	HIS, Level 2 (engineering histogram data)	23

4.2	HIS, Level 3 (science histogram data)	26

4.3	HIS, Level4  (linearized histogram data)	30

4.4	PIX, Level 2 (engineering pixel list data)	34

4.5	PIX, Level 3 (science pixel list data)	37

4.6	PIX, Level 4 (Linearized pixel list data)	41

4.7	CNT, Level 2 (engineering count rate data)	45

4.8	CNT, Level 3 (science count rate data)	47

4.9	HKTM (housekeeping telemetry data)	48

4.10	Sample FITS header	62

REVISION NOTICE

Initial Issue:	January 2004

Revision 1:	January 2007		Major revision to reflect new format of Alice data products

Revision 2:	April 2007		Multiple  revisions to correct inaccuracies in the description of the Alice data products. Removed all TBD items from document

Revision 3:	March 2010		Added description of LIN data product

Revision 4:	November 2016		Revised descriptions of data products

Revision 5:	February 2018		Revised Section 3.1.3 to include information about LIN data product directories; revised Section 3.4.3.5 to indicate BROWSE directory is used for LIN data archives; revised Section 3.4.3.12 to include reference to LIN data; fixed typo in Section 2.3.4.

TBD LIST

| Section | Description |
| --- | --- |

# Introduction

## Purpose and Scope

The purpose of this EAICD (Experimenter to (Science) Archive Interface Control Document) is twofold. First it provides users of the Alice instrument on the Rosetta mission with detailed description of the product and a description of how it was generated, including data sources and destinations. Secondly, it is the official interface between the Alice instrument team and the archiving authority.

## Archiving Authorities

The Planetary Data System Standard is used as archiving standard by

- NASA for U.S. planetary missions, implemented by PDS

- ESA for European planetary missions, implemented by the Research and Scientific Support Department (RSSD) of ESA

### ESA’s Planetary Science Archive (PSA)

ESA implements an online science archive, the PSA,

- to support and ease data ingestion

- to offer additional services to the scientific user community and science operations teams as e.g.

- search queries that allow searches across instruments, missions and scientific disciplines

- several data delivery options as

- direct download of data products, linked files and data sets

- ftp download of data products, linked files and data sets

The PSA aims for online ingestion of logical archive volumes and will offer the creation of physical archive volumes on request.

## Contents

This document describes the data flow of the Alice instrument on Rosetta from the s/c until the insertion into the PDS-SBN for NASA and PSA for ESA. It includes information on how data were processed, formatted, labeled and uniquely identified. The document discusses general naming schemes for data volumes, data sets, data and label files. Standards used to generate the product are explained. Software that may be used to access the product are explained.

The design of the data set structure and the data product is given. Examples of these are given in the appendix.

## Intended Readership

The staff of archiving authority (Planetary Data System for NASA, Planetary Science Archive for ESA) design team and any potential user of the Rosetta-Alice data.

## Applicable Documents

- Planetary Data System Data Preparation Workbook, 1995 February 17, Version 3.1, JPL, D-7669, Part 1

- Planetary Data System Standards Reference, 2003 August 1, Version 3.6, JPL, D-7669, Part 2

- Rosetta Archive Generation, Validation and Transfer Plan, 2003 December 22, Issue 2, Revision 1, RO-EST-PL-5011

- Rosetta-Alice Data Processing Software, 2003 April 22

- Alice Experiment User Manual, 2002 August 2, Version 1.3, RO-8225-EUM-01_3_ALICE_UM

- Data Delivery Interface Document (DDID), 2003 October 23, Issue B6, RO-ESC-IF-5003

- Rosetta Time Handling, 2004 Januaryu 9, Draft 2, RO-EST-TN-3165

## Relationships to Other Interfaces

Any change in this ICD may need to be reflected in the data processing software (LIMA, MIKE, and NOVEMBER) and the document that describes the software (AD4).  Correspondingly, this ICD may need to be changed to reflect changes in the software or changes in the understanding of requirements for archiving imposed either by NASA or by NASA’s Planetary DataSystem and ESA’s Planetary Science Archive.

## Acronyms and Abbreviations

APID		Application Process ID

DDS		Data Distribution System

FITS		Flexible Image Transport System

FOV		field-of-view

HK		house keeping

ICD		interface control document
MCP		microchannel plate

OBT		Onboard Time

PDS		Planetary Data System

PSA		Planetary Science Archive

SFDU		Standard Format Data Unit

SCET		Spacecraft Elapsed Time

TC		telecommand

TM		telemetry

UV		ultraviolet

## Contact names and Addresses

- Joel Parker, Southwest Research Institute, joel@boulder.swri.edu, +1 303-546-9670

- Maarten Versteeg, Southwest Research Institute, mversteeg@swri.org, +1 210-522-5029

# Overview of Instrument Design, Data Handling, and Product Generation

## Instrument Design

### Hardware and Characteristics

The Alice UV spectrometer is a very simple instrument. An opto-mechanical layout of the instrument is shown in Figure 2-1. Light enters the telescope section through a 40 x 40 mm2 entrance aperture and is collected and focused by an f/3 off-axis paraboloidal (OAP) primary mirror onto the approximately 0.1° x 6° spectrograph entrance slit. After passing through entrance slit, the light falls onto a toroidal holographic grating, where it is dispersed onto a microchannel plate (MCP) detector that uses a double-delay line (DDL) readout scheme. The 2-D (1024 x 32)-pixel format, MCP detector uses dual, side-by-side, solar-blind photocathodes of potassium bromide (KBr) and cesium iodide (CsI). The predicted spectral resolving power () of Alice is in the range of 105–330 for an extended source that fills the instantaneous field-of-view (IFOV) defined by the size of the entrance slit. Alice is controlled by an SA 3865 microprocessor, and utilizes lightweight, compact, surface mount electronics to support the science detector, as well as the instrument support and interface electronics. Figure 2-2 shows a 3D external view of Alice. The resulting design is highly systems-engineered to minimize mass and complexity, and enjoys strong parts-level heritage from previous UV spectrometers.

Figure 2-1: The opto-mechanical layout of Alice.

Figure 2-2: External view of Alice.

| Table 2-1: Pre-flight Alice Characteristics & Performance Overview |  |
| --- | --- |
| Bandpass | 700-2050 Å |
| Spectral Resolution ( FWHM) | 10 Å (700 Å) – 13 Å (2050 Å) for extended sources 3–5 Å for point sources |
| Spatial Resolution | 0.1° x 0.5° |
| Nominal Sensitivity | 0.5 (1900 Å) – 7.8 (1150 Å) counts s-1 R-1 |
| Field of View | 0.1° x 6.0° |
| Pointing | Boresight with OSIRIS, VIRTIS |
| Observation Types | Nucleus Imaging and Spectroscopy; Coma Spectroscopy |
|  | Jet and Grain Spectrophotometry; Stellar Occultations (secondary observations) |
| Telescope/Spectrograph | Off-axis telescope, 0.15-m diameter Rowland circle spectrograph |
| Detector Type | 2-D Microchannel Plate with delay-line anode readout |

### Science Modes

The instrument has three modes of taking data: histogram, pixel list, and count rate (“photometer”). For the first two, the time of arrival of a charge at both ends of the orthogonal delay lines is used to determine the pixel on which a photon landed.

- In histogram mode, the instrument uses a histogram memory, with memory locations that map one-to-one to pixels on the detector, to store the total number of counts in each pixel. This mode integrates an image and is used at high count rates, and will likely be the most commonly-used mode.  Images are 1024x32 pixels (the long axis corresponds to wavelength, the short axis is the spatial dimension), but the first 100 columns of the image are and “unexposed” part of the detector.  This region contains the pulse height array, and also will provide monitoring of background/cosmic ray and dark counts.  The histogram values (counts) saturate at 65535 counts.

- In pixel list mode, the instrument records the x-y location of each photon and inserts time tags in this list at regular (user-definable) intervals to provide the timing information for the photons. This mode is used at low count rates.  The maximum number of entries is 32678 (including photons and time tags), but may be less depending on the brightness of the target and the exposure time of the particular observation.  Once the memory is filled to this limit, no more data are recorded.

- In count rate mode (also called “photometer” mode), the x-y information for each photon is ignored and a series of counts is summed with time tags inserted at uniform intervals to provide the timing information. This mode is likely to not be widely used but is particularly useful for observing very bright ultraviolet stars, e.g. at the limb of the cometary nucleus.  The maximum number of entries is 32678, but may be less depending on the time interval and total exposure time of the particular observation. (Note that the number of entries is independent of the the brightness of the target.)  Values (summed counts per time interval) saturate at 65535 counts.

In all modes, data from portions of the array can be “windowed”. E.g., in histogram mode, transmission of data to Earth can be of subarrays rather than of the complete array.  This allows for minimizing bandwidth by reading subsets (windows) of data stored in memory.  The data processing software will consider each of these windows to be separate data sets, so that multiple reads of separate windows of a single observation will be saved into separate files.  For this reason, the file naming convention (Section 3.1.4) includes a “dump number” for each file/image.

## Scientific Objectives

See Alice EID-B, § 1.1.1.

The scientific objectives of the Alice investigation are to characterize the composition of the nucleus and coma, and the coma/nucleus coupling of comet 67P/Churyumov-Gerasimenko. This will be accomplished through the observation of spectral features in the 700-2050 Å EUV/FUV region. Alice will provide measurements of noble gas abundances in the coma, the atomic budget in the coma, major ion abundances in the tail and in the region where solar wind particles interact with the ionosphere of the comet, determine the production rates, variability, the structure of H2O, CO and CO2 gas surrounding the nucleus, and the far-UV properties of solid grains in the coma. Alice will also map the cometary nucleus in the FUV, and study Mars and the Rosetta asteroid flyby targets while en route to 67P/Churyumov-Gerasimenko.

Ultraviolet spectroscopy is a powerful tool for studying astrophysical objects, and has been applied with dramatic success to the study of comets. Alice will provide unprecedented improvements in sensitivity and spatial resolution over previous cometary UV observations. For example, Alice will move the sensitivity threshold from the ~1 Rayleigh level achievable with the Hubble Space Telescope to the milliRayleigh level in deep integrations. In addition, Alice will (by virtue of its location at the comet) move the spatial exploration of nucleus UV surface properties from the present-day state-of-the-art (i.e., no data available on any comet) to complete nuclear maps at Nyquist-sampled resolutions of a few hundred meters. Stars occulted by the absorbing coma will also be observed and used to map the water molecule spatial distribution, giving us hints at how the production regions are located on the nuclear surface.

Through its remote-sensing nature, Alice will be able to:

- Obtain compositional and morphological information on the comet prior to the rendezvous, thereby providing planning observations for in situ instruments prior to entering orbit about the comet.

- Map the spatial distribution of key species in the coma, and small coma dust grains, as a function of time as the comet responds to the changing solar radiation field during its approach to the Sun.

- Obtain compositional and production rate measurements of nuclear jets and other inner coma features even when the Orbiter is not in the vicinity of these structures.

- Obtain certain ion abundance measurements around perihelion in order to connect nucleus activity to changes in tail morphology and structure, and coupling to the solar wind.

The primary scientific themes of the Alice investigation are the following (not prioritized):

- Determine the rare gas content of the nucleus to provide information on the temperature of formation and the thermal history of the comet since its formation. Ar and Ne will be prime targets of the Alice investigations.

- Determine the production rates and spatial distributions of the key parent molecule species, H2O, CO and CO2, thereby allowing the nucleus/coma coupling to be directly observed and measured on many time-scales in order to study the chemical heterogeneity of the nucleus and its coupling to the coma.

- Obtain an unambiguous budget of the cosmogonically most important atoms C, H, O, N, and S through the detection of their emissions far from the nucleus. This is required to understand their production processes and to derive the elemental composition of the volatile fraction of the nucleus. Coupled to the measure of the major molecule abundances of the nucleus, this will give us the total contribution of the secondary parent species to the compositional makeup of the nucleus.

- Study the onset of nuclear activity and nucleus output variations related to changing solar aspect and nuclear rotation with unprecedented sensitivity.

Additional scientific themes Alice will address include the following:

- Spectral mapping of the complete nucleus at far-UV wavelengths to characterize the distribution of UV absorbers on the surface, in particular icy patches and organics.

- Photometric properties and ice/rock ratio of small grains in the coma as an aid to understanding the size distribution of cometary grains and how they vary in time. Also, studying the grain coma to establish the relative contributions of the nucleus and coma grains to the observed gases.

- Mapping the time variability of O+, N+, and possibly S+ and C+ emissions in the coma and ion tail in order to connect nuclear activity to changes in tail morphology and structure, and tail interaction/coupling to the solar wind.

Alice will take data during cruise (typically checkout data during periods defined by ESA) and at every encounter with a planetary and asteroid body (within activity limitations imposed by ESA and the Rosetta project).  Extensive observations will be made during the various phases at the comet. At each asteroid flyby, for example, Alice may spatially scan the region of the asteroid by moving the slit across the field thought to contain the asteroid. If the S/N is high, this could result in a complete image cube. However, we may also, for example, carry out a staring observation at the limb which would ultimately result in a single, one-dimensional spectrum. Observations at the encounters with Earth and Mars will be aimed at measuring the airglow from those planets.  Observations during the first, closest Earth-Moon flyby will use the Moon as a calibration source.

At the comet, a variety of observing scenarios will be used. These include spatial scans with spatially resolved spectra at each point, 1-D spectra, and time series as stars are viewed through the coma and approaching occultation by the nucleus. This last can involve recording complete 2-D spectra as a function of time, but then collapsing the data in transmission to provide 1-D spectra, possibly integrating over significant wavelength regions, before transmission to Earth.

## Data Handling Process

### Responsible Individuals

The following individuals are responsible for the data preparation, monitoring and maintenance of the processing and calibration pipeline, and submission of the product delivery to the PSA:

- Andrew Steffl, Southwest Research Institute, steffl@boulder.swri.edu, +1 303-546-9670

- Joel Parker, Southwest Research Institute, joel@boulder.swri.edu, +1 303-546-9670

- Maarten Versteeg, Southwest Research Institute, mversteeg@swri.org, +1 210-522-5029

### Data Flow

The Alice instrument typically sends HK data on a regular time interval.  When exposures are taken, the HK data available at the end of the exposure are included as a header to the science data to provide instrument context for the exposure.  Events (command acknowledgements and error reports) are sent in response to TC.  Dumps of memory and context/parameters are sent when requested by specific TCs. These different types of data from the Alice instrument are associated with the following APIDs:

- 1473 (acknowledgements)

- 1476 (HK packets)

- 1479 (events)

- 1481 (memory dump)

- 1483 (context dump)

- 1484 (science {includes diagnostic and parameter dump} and telecommands)

- -4 (the “super APID”, that includes all Alice APIDs, so all data can be obtained from DDS in a single request)

Additional spacecraft APIDs are available, and used by Alice software, but will not be included in the archived Alice data products.

The Alice data are sent to the Rosetta spacecraft, relayed to ground, and stored in the DDS.  These data are sent (“pushed”) to computers at the Alice team institution in Boulder, Colorado at regular, pre-defined intervals (e.g., daily), and also by direct request to the DDS via FTP or web interface (see AD6).  The data are passed through instrument team software for calibration and analysis.  These pipeline programs can be run interactively, but typically will be called by a UNIX shell script whenever the DDS regularly pushes Alice data.

Auxiliary programs will be run on the data to create the appropriate PDS labels and format for archiving.

### Processing Software

- The Rosetta-Alice data processing software is used internally by the instrument team, and is included in the DOCUMENT/CODE/MIKE directory only for reference (i.e., it is not included in the archive as supported software). The software consists of programs to process and calibrate the science data.  Steps include flat fielding, deadtime correction, and applying wavelength and flux calibration.  This is performed by a set of IDL programs called “Mike”.

### Archive Data Levels

There are two types of notation for “processing levels” in use in the Rosetta mission: those defined by the ESA PSA, and those defined by the National Research Council (NRC) Committee on Data Management and Computation (CODMAC).  The PDS uses the CODMAC data processing levels as defined in Section 6.3.2 in AD2.  For consistency, we use the CODMAC levels (unless explicitly stated otherwise) in the text of this EAICD and in the descriptions of the PDS data labels in Section 4.  The relationship of the levels in question are given in Appendix B of AD3, but are repeated and expanded here:

| Table 2-2: Data Processing Levels |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CODMAC level | ESA/PSA level | NASA level | PRODUCT_TYPE PDS keyword value | Data Record (DR) Type | Alice File Type |
| 1 | 1a |  | UDR | Unprocessed (Raw) |  |
| 2 | 1b | 0 | EDR | Experiment (Edited) | engineering (eng) |
| 3 | 2 | 1A | RDR or REDR | Reduced (Calibrated) | science (sci) |
| 4 |  | 1B | REFDR | Reformatted (Resampled) |  |
| 5 | 3 | 2-5 | DDR | Derived |  |
| 6 |  |  | ANCDR | Ancillary |  |

The Alice data archive will contain:

- Level 2 data (ENG files), which are derived from the (Level 1) data sent by the DDS to the PI’s institution in Boulder, Colorado, USA (to the computer rosetta.boulder.swri.edu), and decommutated by the Rosetta-Alice data collection program Lima.

- Level 3 data (SCI files).  These calibrated data are created by the data processing pipeline Mike from the Level 2 data (as output by Lima).

- Although not formally required to do so, the Alice team often produces Level 4 data (LIN files). These resampled, calibrated data are produced by the Mike data processing pipeline from the Level 3 data. Level 4 data will be supplied to the PSA/PDS on a best effort basis.

## Overview of Data Products

### Pre-Flight Data Products

N/A

### Instrument Calibrations

Instrument calibration files are included in the CALIB/ directory. In addition, each SCI (level 3) and LIN (level 4) data file contains the 1024x32 element calibration array used to convert that specific file from instrumental units to flux units.

### In-Flight Data Products

As described in Section 2.1.2, Alice has three science modes: histogram, pixel list, and count rate.  Each mode corresponds to one type of data product that will be archived as FITS files, and the archive will include CODMAC Level 2 (“engineering”) and Level 3 (“science”) data for each mode.

The data from each observation also include a full set of HK data, providing a snapshot of the instrument status at the end of the given observation (though some parameters, such as the MCP voltage and strip current also include the value at the beginning of the observation).  In the FITS files output by the processing software, those HK parameters are included as keyword=value header fields as shown in the example in Section 4.10 (but only a few are propagated into the PDS label files, since many of them are very instrument-specific and not of general use to PDS users).  These header fields include raw and calibrated parameter values where appropriate, distinguished by the last character of the FITS header keyword: “R” for raw and “C” for calibrated. For example, T_MIRR1R is the “raw” (ADC counts) temperature value for mirror sensor #1, and T_MIRR1C is the corresponding “calibrated” (degrees Celsius) temperature; MCPVR is the “raw” (ADC counts) MCP voltage, and MCPVC is the “calibrated” (volts) voltage.

We also include all HK data sent in the TM stream as an in-flight data product.  These data are archived as flat ASCII files (tables), and provide a larger context of the instrument health and behavior over the course of the mission, and are also useful for calibration of the instrument as a function of any parameters that may affect instrument performance.

Below we describe all these data products in considerable detail.  Examples of the PDS labels are given in Section 4.  Detailed descriptions of the FITS headers are given in Section 4.10.

#### Level 2 (“Engineering”) data

##### Histogram Mode

Each dump of histogram data from the Alice instrument also contains pulse-height data (16 integers) and count rate data (up to 100 integers).  These arrays will be stored as additional blocks (“extensions”) in the FITS file.  So, the FITS file for histogram data will contain:

- Extension = 0 : The 1024x32 histogram image (or a subsection if windowing was selected in the dump)

- Extension = 1 : The 16-value pulse-height distribution table.  The pulse height distribution is a histogram of the amplitude of the photon events that were detected during the histogram acquisition.  The histogram values (counts) saturate at 65535 counts.  For Rosetta-Alice the resolution is rather coarse: the quantization is four bits, but the upper and lower 3 bins are hardly used.

- Extension = 2 : Up to 100-element count rate series.  These count rate data give the count rate measurements as reconstructed from the observation, providing a crude time-resolved “photometer” measurement of the observation.  This is possible because even in Histogram mode because the photons are time-tagged.

##### Pixel List Mode

Pixel list data will be stored as a 1-dimensional array of size equal to the number of values in the pixel list (number of entries may be less than the full 32678 limit depending on the particular observation) in extension 1.  For ease of quick-look, the extension 0 in the FITS file will contain a “ground-software-reconstructed” histogram (image) representation of the data, so any FITS viewer can display the image.  In addition, a count rate representation (using the pixel list time hack period) will be created and stored in extension 2.  So, the FITS file for pixel list data will contain:

- Extension = 0 : The 1024x32 reconstructed histogram-formatted image

- Extension = 1 : The list of all the pixel list values

- Extension = 2 : reconstructed count rate data with a time interval equal to the selected time hack period

The value of each pixel list event is the encoded X,Y positions of the detected event.  In addition to the detector events, a time-mark value is also inserted into the list on a periodic basis.  The pixel list data are encoded as follows:

Value (for events) = X * 1024 + Y

Value (for timehacks) = 65535

Another way to describe this is by representing the bits in the pixel list events (16-bit words) as:

h a a a a a e e e e e e e e e e

where each letter represents a single bit with the following meaning:

h - (most significant bit), set to 1 for a timehack, and 0 for a normal photon event. For timehacks, the other 15 bits have no meaning and are set to 1.

a - the five bits of spatial information of a photon event.

e - the ten bits of spectral information of a photon event.

##### Count Rate Mode

Data will be stored as a 1-dimensional FITS image with the number of elements equal to the number of intervals for which a count rate was obtained (the number of elements may be less than the full 32678 limit depending on the particular observation).

##### Housekeeping Telemetry

HK data are sent from Alice at regular, user-defined time intervals, and provide values for all instrument parameters of interest, e.g., temperatures, currents, voltages, limit values, software flags, door status, etc.  When these data are received from the DDS and processed through the data pipeline (by the “Lima” program), they are added to a running ASCII file.  Note that since one HK TM file can contain data covering a wide range of time, the time field used in the filename identification of the files will be the UTC-format SCET of the first data packet that appears in the file.

The structure of the ASCII files will be a header section followed by a data section (the PDS label will point past this header to the data field).  For use by other file-reading programs, the beginning of the data section is identified by the explicit text “START DATA” alone on a line.

- Description of Header block:

  - Each line in the header block defines one column of data in the data block.  The columns in the header are:

    - Column Number: column number of value in data block,

    - Key: the FITS-compatible keyword name,

    - Unit: the physical unit (if any) of the values in the column, a dash when no unit is applicable,

    - Width:  width of column, including padding spaces,

    - Format: format for data (“I”=integer, “F”=floating, “A”=string),

    - Info:  additional explanatory info about the entry (this field may be empty and starts with the pound (#) character), can contain the name of the parameter in the Alice code

  - All fields except the “Info” field must contain at least one character/value with no whitespace separating values.  So, e.g., if there are no Units or Name for a given entry, the field may have “-“ or “n/a” or some other indicator of no value.

  - The header section also may contain comment lines, identified as having the pound character (#) as the first character on the line.

  - The names of the calibration files used to translate from “raw” TM values to “engineering” values (e.g., from ADC counts to temperature) will be included as comments in the header.

- Description of Data block:

  - A single line (record) is added to the file for each received TM packet (i.e., from a single dump)

  - The order of the values/columns are as described in the header block.

  - No removal of doubles or sorting is to be performed.  Such sorting and trimming can be done easily by external programs (e.g., the UNIX or IDL commands “sort” and “uniq”)

  - The data will include both the raw TM value and a calibrated value.

#### Level 3 (“Science”) data

##### Histogram Mode

After Histogram-mode data are processed through the processing and calibration pipeline (the “Mike” program), the result is a 2-D array of the same size as the original: 1024x32 pixels.  The axes still correspond to wavelength vs. spatial, and value of each pixel is the calculated flux. However, since the wavelength calibration is not be linear and varies with Y position, it cannot be easily parameterized and stored in the FITS header; therefore, the wavelength calibration is stored as a second array in the FITS file.   The pulse-height and count rate data that are stored as additional extensions in the “engineering” files (Level 2 data), will be retained and are not processed:

- Extension = 0 : The 1024x32 histogram image (processed and flux calibrated); [photons-cm-2-s-1] implicitly per pixel in wavelength

- Extension = 1 : 1024x32 element image containing the statistical uncertainty associated with extension 0.

- Extension = 2 : The 1024x32 histogram wavelength image (calibrated wavelength at each pixel); [Å]

- Extension = 3 : The 16-value pulse-height array (not processed; same as in Level 2 data)

- Extension = 4 : The 100-value count rate array (not processed; same as in Level 2 data)

- Extension = 5 : The 1024x32 histogram calibration image (calibration constant used at each pixel); [cm2]

##### Pixel List Mode

The pixel list data will be a 2-dimensional array (effectively a table) with the following columns: [X, Y, Wavelength, Time Step]; this will be in the third extension of the FITS file.  For quick-look purposes, the primary extension in the FITS file is a histogram-formatted version of the data as is done for the Level 2 data.

- Extension = 0 : The 1024x32 constructed histogram-formatted image (processed and flux calibrated); [photons-cm-2-s-1] implicitly per pixel in wavelength

- Extension = 1 : 1024x32 element image containing the statistical uncertainty associated with extension 0.

- Extension = 2 : The 1024x32 histogram wavelength image (calibrated wavelength at each pixel); [Å]

- Extension = 3 : Table of pixel list data containing the x-position of the event, y-position, wavelength, time step number, and ephemeris time (ET).

- Extension = 4 : Reconstructed count rate data with interval equal to selected hack period (not processed, i.e. identical to Level 2 data)

- Extension = 5 : The 1024x32 histogram calibration image (calibration constant used at each pixel); [cm2]

##### Count Rate Mode

As with the Level 2 data, this is a 1-D array stored in extension 1 as a FITS binary table.  Values are still in counts (since there is no wavelength information, the counts cannot be converted to flux), but some calibrations have been applied, e.g., deadtime and dark current corrections (since there is no wavelength or position information, a flat field sensitivity correction cannot be applied to these data).

#### Level 4 (“Linearized”) data

##### Histogram Mode

After data are processed to Level 3, they are divided by the width, in wavelength, of each pixel in the 1024 x 32 image and interpolated so that the wavelength for the data is linear, equal for all rows and identical for all LIN files. The axes still correspond to wavelength vs. spatial, and value of each pixel is the calculated flux. The pulse-height and count rate data that are stored as additional extensions in the “engineering” files (Level 2 data), will be retained and are not processed:

- Extension = 0 : The 1024x32 histogram image (processed, flux calibrated, and resampled); [photons-cm-2-s-1 Å-1]

- Extension = 1 : 1024x32 element image containing the statistical uncertainty associated with extension 0.

- Extension = 2 : The 1024-element wavelength vector (calibrated wavelength at each pixel; identical for all rows); [Å]

- Extension = 3 : The 16-element pulse-height distribution array (not processed; same as in Level 2 data)

- Extension = 4 : The 100-element count rate array (not processed; same as in Level 2 data)

- Extension = 5 : The 1024x32 histogram calibration image (calibration constant used at each pixel); [cm2]

##### Pixel List Mode

The pixel list data will be a 2-dimensional array (effectively a table) with the following columns: [X, Y, Wavelength, Time Step]; this will be in the third extension of the FITS file.  For quick-look purposes, the primary extension in the FITS file is a histogram-formatted version of the data as is done for the Levels 2 & 3 data.

- Extension = 0 : The 1024x32 constructed histogram-formatted image (processed, flux calibrated, and resampled); [photons-cm-2-s-1 Å-1]

- Extension = 1 : 1024x32 element image containing the statistical uncertainty associated with extension 0.

- Extension = 2 : The 1024-element wavelength vector (calibrated wavelength at each pixel; identical for all rows); [Å]

- Extension = 3 : Table of pixel list data containing the x-position of the event, y-position, wavelength, time step number, and ephemeris time (ET).

- Extension = 4 : Reconstructed count rate data with interval equal to selected hack period (not processed, i.e. identical to Level 2 data)

- Extension = 5 : The 1024x32 histogram calibration image (calibration constant used at each pixel); [cm2]

##### Count Rate Mode

There is no Level 4 version of count rate data

### Software

As listed in Section 2.3.3, the Alice data calibration software “Mike” is used internally by the instrument team and is included as IDL procedures (source code) in the /document/code/ directory for reference only (i.e., it is not included in the archive as supported software). This software:

- Reads science-mode (pixel list, histogram, count rate) data files.

- Apply corrections as appropriate: deadtime, bias, dark and flat.

- Apply wavelength and flux calibrations.

- Write calibrated data to multi-extension FITS-format file, where additional extensions may include wavelength values, effective count rate array, etc.

### Documentation

- In addition to the required ASCII formats for documents, the Alice team will provide formatted documents in MS Word, and /or PDF. Documents to be included will describe the instrument and its calibration, data processing, and data archiving and may also include scientific results.  Documents will be located in the /DOCUMENT/ directory.

### Derived and other Data Products

No derived/other data products are expected from the Rosetta-Alice project.

### Ancillary Data Usage

The only ancillary data used by the Rosetta-Alice pipeline software are SPICE kernels, which are used to calculate observation geometry information (e.g., target distance, attitude, etc.). These can be obtained from the NAIF at JPL.

# Archive Format and Content

## Format and Conventions

### Deliveries and Archive Volume Format

Deliveries of Rosetta-Alice data will occur according to the schedule.

### Data Set ID Formation

All the Rosetta-Alice datasets have the naming convention generally following the definition in Section 6 in the PDS Standards Reference [AD2], and limited to 40 characters in length.  This will be the same value that will appear in the DATA_SET_ID element in the PDS label.  The format is:

DATA_SET_ID = RO-<target>-ALI-<L>-<phase>-<text>-V<#.#>

where:

- <target> is the abbreviated name of the target: “E” (Earth), “M” (Mars), “A” (asteroid), “C” (comet), “CAL” (calibration), “X” (other, e.g., checkout), “Y” (sky), or other targets as described in Section 6.3.2 in [AD2].

- <L> is the CODMAC processing level: “2”,“3”, or “4” (corresponding to PSA levels “1b” and “2”, respectively).  The CODMAC  level “6” may also be used to describe ancilliary data.

- <phase> is the abbreviated name of the phase (e.g., CVP1, EAR1, GMP, COP etc. as defined in Appendix F, Table 2 in AD3)

- <text> is additional descriptive text if needed.  Since the DATA_SET_ID is limited to 40 characters, this text field is then limited to typically no more than about 20 characters.

- V<#.#> is a version number

The expanded DATA_SET_NAME element will have the format (limited to 60 characters):

"ROSETTA-ORBITER <full_target> ALICE <L> <full_phase> <text> V<#.#>"

where:

- <full_target> is the full target name (Earth, Mars, Asteroid <name>, Comet 67P/Churyumov-Gerasimenko, Calibration).

- <full_phase>:  is the full name of the phase (e.g., Commissioning, Earth Swing-by 1, Global Mapping Phase, Close Observation Phase, etc.)

- <L>, <text>, V<#.#>: are as described above in the DATA_SET_ID.

Note that if the DATA_SET_NAME would exceed 60 characters, then the target and/or phase names will be abbreviated using the abbreviations in DATA_SET_ID.  Also note that if multiple targets or phases are included in a given dataset, their identifiers are concatenated with a forward slash, “/”, such as:

DATA_SET_ID = "RO-E/CAL-ALI-2-EAR1-V1.0"

to describe a dataset that includes Earth and calibration observations during the first Earth flyby.

### Data Directory Naming Convention

Note that since a submitted archive data set cannot contain multiple data processing levels (see the last paragraph in section 6.1 in the PDS Standards Reference [AD2]), we expect to deliver three data sets per phase.  In the first data set, the data directories will contain Level 2 data files (also referred to as “engineering files”), which are identified with “ENG” in the filename as shown in Section 3.1.4.1.  In the second data set, the data directories will contain Level 3 data files (also referred to as “science files”), which are identified with “SCI” in the filename as shown in Section 3.1.4.2.  In the third data set, the data directories will contain Level 4 data files (also referred to as “linearized files”), which are identified with “LIN” in the filename as shown in Section 3.1.4.3.

The Alice archive data directory structure has the following naming convention:

/DATA/<yyyy>/<mm>/<filename>

where

- <yyyy> is the year of the observation

- <mm> is the month of the observation e.g., 01 for January

### Filenaming Convention

All the Rosetta-Alice files are prefixed by “RA_” and are ISO 9660/level 2 compliant.

#### Engineering Files

These Level 2, “ENG” type files are output by the Alice data collection program Lima.  They have the filename convention:

RA_<YYMMDDhhmmss>_<TYPE>_ENG.<EXT>

where:

- <YYMMDDhhmmss> is the 12-character UTC date and time of the start of the observation (the start of collecting photons).

- <type> is one of the following 4-character values:

- PIX# 	(pixel list mode data; “#”is a running 1-digit version number to allow for different processing runs of the data (e.g., with different, improved calibrations). For Level 2 engineering files, which are not calibrated, this version number is envisioned to remain “0”

- HIS# 	(histogram mode data)

- CNT# 	(count rate mode data)

- HKTM 	(housekeeping telemetry data; these files contain a header describing the contents and columns, but such a header is not PDS-compliant, so these files are used only internally by the Alice team, and should not appear in the archive)

- HKTMX 	(housekeeping telemetry data; the “x” indicates that the header has been stripped off as required for archive format, and so these are the files that will appear in the archive)

- <EXT> is one of the following 3-character extensions:

- FIT 	(for FITS files: PIX, HIS, CNT)

- TAB 	(for ASCII table files: HKTM)

#### Science Files

Level 3, “SCI” files (also sometimes referred to as “processed” or “calibrated” files) are output by the processing and calibration program Mike.  Only the science data (types: PIX, HIST, CNT) are processed and calibrated. They have the filename convention:

RA_<YYMMDDhhmmss>_<type>_SCI.FIT

where:

- <YYMMDDhhmmss> is the 12-character UTC date and time of the start of the observation (the start of collecting photons). This will be the same value as in the filename of the input engineering file.

- <type> is one of the following 4-character values (the same value as in the filename of the input engineering file):

- PIX# 	(pixel list mode data; “#”is a running 1-digit version number to allow for different processing runs of the data (e.g., with different, improved calibrations)

- HIS# 	(histogram mode data)

- CNT# 	(count rate mode data)

- The extension is always “fit “ since these are all FITS files.

#### Linearized Files

Level 4, “LIN” files (also sometimes referred to as “resampled” files) are output by the processing and calibration program Mike.  Only the image data (types: PIX & HIST) are resampled. They have the filename convention:

RA_<YYMMDDhhmmss>_<type>_LIN.FIT

where:

- <YYMMDDhhmmss> is the 12-character UTC date and time of the start of the observation (the start of collecting photons). This will be the same value as in the filename of the input engineering file.

- <type> is one of the following 4-character values (the same value as in the filename of the input engineering file):

- PIX# 	(pixel list mode data; “#”is a running 1-digit version number to allow for different processing runs of the data (e.g., with different, improved calibrations)

- HIS# 	(histogram mode data)

- The extension is always “fit “ since these are all FITS files.

#### Science Calibration Files

These are files used by the processing and calibration program Mike (e.g., darks, flats, wavelength, parameter files). They have the filename convention:

ra_<type>_<vvv>.<ext>

where:

- <type> is one of the following 4-character values:

- AEFF (effective area calibration vectors)

- dark 	(dark image)

- flat 	(flatfield image)

- WAVE (wavelength calibration file)

- <vvv> is a running 3-digit version number.

- <ext> is one of the following extensions:

- fit 	(for FITS files: dark, flat, WAVE)

- tAB 	(for ASCII table files: AEFF)

## Standards Used in Data Product Generation

### PDS Standards

The PDS Standards Reference version 3.7 was used for the design of the Rosetta-Alice archive

### Time Standards

The timing information used in all Rosetta-Alice data is derived from the Rosetta onboard clock.  The Alice clock resolution is 1/256 seconds (~3.9 millisecond), although Rosetta provides sub-millisecond time resolution.  The OBT is a counter related to the UTC by the Time Correlation Packets (TCP) distributed via the DDS (APID 1966, type 190, subtype 40); note that the SCET can be written in either OBT or UTC formats. The various time definitions, formats, and relationships are described in AD6 and AD7.

In the Alice data (e.g., the headers of the FITS files), “OBT” is the time that Alice includes in its TM packets once the instrument has become synchronized to the spacecraft (which is done after a time update TC is sent to Alice); before receiving this synchronization, the time provided by Alice is the time since instrument startup. The OBT is the number of seconds since the spacecraft clock was last reset (though it is expected that the clock will not ever need to be reset during the mission). The format of OBT in the PDS labels for Rosetta data is: “<r>/<f.f>”, where <r> is the reset number, and <f.f> is the floating-point OBT value.  A reset value of <r>=1 indicates a zero point of January 1st, 2003 (2003-01-01T00:00:00), so the associated OBT is the number of seconds (not corrected for drift, etc.) since that zero point.

We use the term “SCET” for the ground-corrected OBT, modified by known deviations of the onboard clock as described in AD6 and AD7; the offset is given as the value for the Alice-specific keyword TOFFSET.  As defined those documents, the time format used is the Sun MJT, as standard on Sun Solaris UNIX platforms. The format is two 32-bit integers. The first contains the number of seconds since the beginning of January 1st, 1970 (1970-01-01T00:00:00) and the second integer contains the number of microseconds.

In the Alice data pipeline, the OBT and UTC/SCET used to calculate TOFFSET are the two values that appear in the Alice data TM header (see Figure 1 in AD7).  The OBT is the time of “measurement”, placed in the TM packet header by Alice.  The UTC/SCET is the corresponding time of the same event, calculated using the TCP and placed in the TM header by the DDS.  The calculated TOFFSET is then applied in the Alice data pipeline to calculate the UTC for all other OBT values that appear in the Alice data (e.g., exposure start/stop times, dust alert times, etc.  Note that although this is not precisely valid since using a single offset does not take into account any time drift (or other effects) that are more fully corrected by the functional form of the TCP, it should be sufficiently accurate based on the assumption that any drift is very small over the range of time covered by a given observation (represented by a single Alice data file).

The format used to represent “raw” time values (OBT and SCET) is a real number representing the number of seconds since time zero.  The format used to represent “calibrated” time values (SCET only) is the correlated UTC represented in the ISO 8601 compliant standard time string:

<YYYY>-<MM>-<DD>T<hh>:<mm>:<ss.sss>

Examples of the time format are given by the Alice-specific keywords parameters: SCETR, SCETC, INSTTIME, PRESSCET, and the PDS label keywords: SPACECRAFT_CLOCK_START_COUNT and SPACECRAFT_CLOCK_STOP_COUNT (which are OBT, “raw” times) and START_TIME and STOP_TIME (which are SCET “calibrated” UTC times).

N.B. During the original commissioning period (March—May 2004), a drift of the Alice internal timekeeping was discovered.  This amounted to –1.8 seconds/hour relative to the spacecraft clock (i.e., the Alice timekeeping ran slow).  This was not due to the actual clock drifting, but a hardware miscalculation (dividing the Alice clock ticks by 4000 rather than 4002).  The drift was corrected each time Alice use a time synchronization signal which are nominally sent from the spacecraft every 30 minutes; however, by design, Alice does not accept time synchronization signals when an acquisition is active, so if there are many observations in a row, the time error due to the drift can be significant (several seconds).   This problem was corrected by a software patch fix that was uploaded to the flight instrument in September 2004.  In all data files that were created before that date, the times listed in the data files will have uncertainties up to several seconds.

### Reference Systems

- ISO 9660, Level 2: file naming

- ISO 8601: time format

### Other Applicable Standards

Observation geometry parameters provided in the data product labels and the FITS headers are computed for the start time of a given observation. The Earth Mean Equator reference frame of J2000 is used as the inertial reference frame for geometry items in the data headers.

Latitude and longitude (IAU2000 model) are given in the PLANETOGRAPHIC ("geodetic") coordinate system. Light travel-time from target to observer is taken into account. Latitude is the angle between the equatorial plane and the line perpendicular to the reference ellipsoid of the body (e.g., reflects body oblateness). Positive longitude is to the west.  Units: DEGREES

## Data Validation

The Alice team will perform scientific validation of the data initially with a “quick-look” of the data soon after receipt, and more thoroughly by in-depth analysis and publication of the data.  If an error in the pipeline is discovered, the software will be corrected, and all affected data will be reprocessed through the pipeline.  If the affected data includes data already previously submitted to the archive, the subsequent archive submission will include a full incremental dataset containing the corrected data.

Validation of archive format compliance will be made by use of PDS validation tools:

- readpds – Software tool supplied by the PDS Small Bodies Node used in the verification of PDS Labels and data products.

- pvv - The PSA Validation and Verification Tool from ESA.

The readpds tool is available  from the PDS Small Bodies Node at:

- http://pdssbn.astro.umd.edu/nodehtml/software.shtml

## Content

### Volume Set

Each submission of Alice data to ESA will be separate volumes, where each volume is a single data set.  Data sets are described below.

### Data Set

The Rosetta-Alice data set delivery phases and approximate sizes are given in in Appendix E in AD3, and given in more detail here.  We only list here the mission phases that are likely to generate data (e.g., Alice will not obtain data during the Deep Space Maneuver phases).  “All” data types means both processing levels (2 and 3) and possibly all three Alice data types (histogram, pixel list, count rate, housekeeping).  Also note that at least two, possibly three datasets will be delivered for each phase, corresponding to the different data processing levels, so that “<L>” in the table below will be CODMAC data processing level numbers “2” or “3”.

| Table 3-3: Planned Alice Data Sets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Phase | DATA_SET_ID | Delivery Date | Size | Data Types |
| Commissioning 1 | RO-CAL-ALICE-<L>-CVP1-V1.0 | April 2007 | 76 MB | All |
| Cruise 1 | RO-CAL-ALICE-<L>-CR1-V1.0 | April 2007 | 19 MB | All |
| Commissioning 2 | RO-CAL-ALICE-<L>-CVP2-V1.0 | April 2007 | 395 MB | All |
| Earth Swing-by 1 | RO-CAL-ALICE-<L>-EAR1-V1.0 | April 2007 | 34MB | All |
| Cruise 2 | RO-CAL-ALICE-<L>-CR2-V1.0 | April 2007 | 536 MB | All |
| Mars | RO-CAL-ALICE-<L>-MARS-V1.0 | November 2007 | 100 MB? | All |
| Cruise 3 | RO-CAL-ALICE-<L>-CR3-V1.0 | March 2008 | TBD | All |
| Earth Swing-by 2 | RO-CAL-ALICE-<L>-EAR2-V1.0 | June 2008 | TBD | All |
| Earth Swing-by 3 | RO-CAL-ALICE-<L>-EAR3-V1.0 | April 2010 | TBD | All |
| Asteroid Fly-by 1 | RO-CAL-ALICE-<L>-AST1-V1.0 | April 2010 | TBD | All |
| Asteroid Fly-by 2 | RO-CAL-ALICE-<L>-AST2-V1.0 | March 2011 | TBD | All |
| Comet phases (each phase) | RO-CAL-ALICE-<L>-<ph>-V1.0 (<ph> = NCD, FAT, CAT,…) | 6 months after the end of each phase | 500 MB | All |

### Directories

The sections below illustrate the structure of the final archive of data from Alice to be delivered to the ESA-PDS team for inclusion in the Rosetta archive. Intermediate (incremental) deliveries during the mission will likely not include all types of data products. The details of what components are included in the archive and how they are grouped into data sets, subdirectories, and files comprises the design of the archive.

The Rosetta archive will be an online archive, where the data are delivered electronically. Thus, there is no need to bundle several data sets into one volume, and one data set will correspond to one volume. A data set will include the data products, secondary data, software and documentation that completely document and support the use of the data products from one or more mission phases.

#### Root Directory

The top-level structure of the root (/) directory of a typical data archive volume (= data set) corresponds to chapter 19 of the PDS Standards Reference (AD2) and is summarised below.  It will contain the following files:

- AAREADME.TXT : This file describes the volume (= data set) as a whole. It gives an overview of the contents and organisation of the data set, general instructions for its use and contact information.

- VOLDESC.CAT : This file contains the VOLUME object, which gives a high-level description of the contents of the volume (= data set).

The following directory structure is found under the root (/) directory.

#### Calib Directory

The calibration data will be archive in the /calib/ directory.  All calibration files have a version number in their filename (Section 3.1.4.3 and 3.1.4.4), which is also time-ordered.  Files in this directory include:

- CALINFO.TXT : Description of the contents of the CALIB directory.

- Flat field, wavelength, and dark calibration files (PDS-labelled FITS format) sorted by version number.

- Effective area calibration table (2-column ASCII table) giving the wavelength and the instrumental effective area at this wavelength. Note, this file will generally have a different wavelength scale than the Alice data files. It should be interpolated to the wavelength scale used by each individual Alice data file, before being applied to the data.

#### Catalog Directory

This directory contains the catalog object files for the entire volume (= data set).

- CATINFO.TXT : Description of the contents of the CATALOG directory.

- MISSION.CAT : PDS mission catalog information about the Rosetta mission. This file will be provided by ESA.

- INSTHOST.CAT : PDS instrument host catalog information about the Rosetta spacecraft and the mounting relationship of the instruments within the spacecraft.  This file will be provided by ESA.

- INST.CAT : PDS instrument catalog information about the Alice instrument (likely to be the same in all deliveries, unless updates are needed).

- DATASET.CAT : PDS data set catalog information about the data set submitted.

- <TARGET_NAME>.CAT : PDS target catalog information about the observation target, i.e. comet, asteroid, Earth or Mars, will be provided by ESA.

- PERSON.CAT : PDS personnel catalog information about the members of the Alice instrument team responsible for generating the data products.

- REF.CAT : PDS reference catalog information about every journal article, book, or other published reference mentioned in the above catalog objects or their components.

#### Index Directory

This directory contains the index files summarising all data products in the volume (= data set) by mode, key instrument parameters or mission phase, and organised to facilitate finding the data of interest for a particular scientific question. Information about the observation geometry of the data products is also included here, i.e. spacecraft position and attitude, illumination conditions etc. Information that is not accurately known at the time of delivery and thus will probably be updated later should be stored in the index files rather than in the data product labels.

- INDXINFO.TXT : Description of the contents of the INDEX directory.

- INDEX.LBL : Detached label for the index table INDEX.TAB. The INDEX_TABLE specific object should be used to identify and describe the columns of the index table.

- INDEX.TAB : Index of the data set in tabular format.

#### Browse Directory and Browse Files

This directory is only used for the Level 4 (“linearized”) data sets, and contains a set of quick-look images of the calibrated, linearized spectral data in JPEG format, together with their corresponding PDS label files.  The images are organized in a manner analogous to the data files themselves, in subdirectories, first by year (YYYY), then by month (MM).  The quick-look files are named in the same manner as their parent data files, except that “_TN” (for “thumbnail”) has been added to the base filename, and the suffix is “.JPG” instead of “.FIT”.  Lastly, in each BROWSE directory there is a BROWINFO.TXT file that describes the contents of the BROWSE directory.

#### Geometry Directory

The Rosetta-Alice team does not foresee providing any geometric information beyond that provided by ESOC. This directory, therefore, will be blank until the composite archive is produced by the EPT, at which time the geometry files from ESOC will be included.

#### Software Directory

The Rosetta-Alice team does not foresee providing any software that is intended for regular updates and multiple platform capability. The source code of software used internally by the Rosetta-Alice team will be provided as reference documents in the DOCUMENT directory. This software is for docunentation only, and is not supported.

#### Gazetteer Directory

The gazetteer directory will not be used for Rosetta-Alice.

#### Label Directory

The label directory will not be used for Rosetta-Alice.

#### Document Directory

This directory provides documentation and supplementary and ancillary information to assist in understanding and using the data products in the volume (= data set). All documents will be present in ASCII format to ensure long-term readability. Other formats (Word, PDF, TeX etc.) will be included as well as appropriate.

Files will include:

- DOCINFO.TXT : Description of the contents of the DOCUMENT directory.

- Alice Experiment User Manual (AD5)

- This EAICD.

- The Rosetta-Alice Data Processing Software Manual (AD4).  This provides details of the behavior, use, and methods of the Lima, Mike, and November programs.

- The IDL calibration programs (Mike) used for pipeline processing. Note that the Mike software includes the IDL programs for wavelength calibration and deadtime correction mentioned in Section 3.4.3.2.

- Calibration reports if available.

#### Extras Directory

Use of this directory is TBD. No specific items have yet been identified for this directory.

#### Data Directory

This directory contains the actual Alice data (images and tables). Data files with detached PDS label files are arranged in a logical subdirectory structure with subdirectories corresponding to the year and month the data were obtained, e.g. DATA/2004/04/.

- Level 2 data (also referred to in this document as “engineering” data).

- Level 3 data (also referred to in this document as “science”, “processed”, or “calibrated” data).

- Level 4 data (also referred to in this document as “linearized”, or “resampled” data).

# Detailed Interface Specifications: data product design

All our data products use detached labels.  In the subsections below, we provide explicit examples of the label files for all data products.

## HIS, Level 2 (engineering histogram data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 33

FILE_NAME                    = "RA_040419231832_HIS0_ENG.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040419231832_HIS0_ENG.FIT",1)

^IMAGE                       = ("RA_040419231832_HIS0_ENG.FIT",7)

^PULSE_HEIGHT_HEADER         = ("RA_040419231832_HIS0_ENG.FIT",30)

^PULSE_HEIGHT_TABLE          = ("RA_040419231832_HIS0_ENG.FIT",31)

^COUNT_RATE_HEADER           = ("RA_040419231832_HIS0_ENG.FIT",32)

^COUNT_RATE_SERIES           = ("RA_040419231832_HIS0_ENG.FIT",33)

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

INSTRUMENT_MODE_ID           = "HIS" /* HIS, PIX, CNT */

INSTRUMENT_MODE_DESC         = "HISTOGRAM"

START_TIME                   = 2004-04-19T23:18:31.633 /* UTC */

STOP_TIME                    = 2004-04-19T23:18:51.782 /* UTC */

SPACECRAFT_CLOCK_START_COUNT = "1/41037497.246"

SPACECRAFT_CLOCK_STOP_COUNT  = "1/41037517.395"

EXPOSURE_DURATION            = 20.1480 /* SECONDS */

DATA_QUALITY_ID              =        0

DATA_QUALITY_DESC            = "Data quality is nominal"

MISSION_PHASE_NAME           = "COMMISSIONING 1"

/* see Phase Definition table in RSOC Design Specification, */

/* RO-EST-PL-2010 and Archive GVT Plan, RO-EST-PL-5011 */

TARGET_NAME                  = CHECKOUT

TARGET_TYPE                  = "N/A"

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

INSTRUMENT_ID                = "ALICE"

INSTRUMENT_NAME              = "ALICE"

INSTRUMENT_TYPE              = "ULTRAVIOLET SPECTROMETER"

INSTRUMENT_HOST_ID           = "RO"

INSTRUMENT_HOST_NAME         = "ROSETTA-ORBITER"

MISSION_ID                   = "ROSETTA"

MISSION_NAME                 = "INTERNATIONAL ROSETTA MISSION"

/* PRODUCT, DATA SET, AND VOLUME DATA ELEMENTS */

PRODUCT_ID                   = "RA_040419231832_HIS0_ENG.FIT"

PRODUCT_TYPE                 = EDR /* CODMAC Data Level 2 */

PRODUCT_CREATION_TIME        = 2004-09-20T22:37:07

DATA_SET_ID                  = "RO-C/CAL/X-ALICE-2-CVP1-V1.0"

DATA_SET_NAME                = "

ROSETTA-ORBITER 2002T7/CAL/CHECK ALICE 2 CVP1 V1.0"

PRODUCER_ID                  = "ROSETTA_ALICE"

PRODUCER_FULL_NAME           = "ANDREW J. STEFFL"

PRODUCER_INSTITUTION_NAME    = "SOUTHWEST RESEARCH INSTITUTE"

/* DATA OBJECT DEFINITIONS */

OBJECT                       = HEADER

BYTES                      = 17280 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 6

DESCRIPTION                = "

FITS header for Rosetta-Alice uncalibrated (CODMAC Data Level 2)

histogram observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                =           16

SAMPLE_TYPE                = MSB_INTEGER

INTERCHANGE_FORMAT         = BINARY

OFFSET                     = 32768 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image for Rosetta-Alice uncalibrated (CODMAC Data Level 2)

histogram observation. Units are raw data numbers."

END_OBJECT                   = IMAGE

OBJECT                       = PULSE_HEIGHT_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PULSE_HEIGHT_HEADER

OBJECT                       = PULSE_HEIGHT_TABLE

NAME                       = "PULSE_HEIGHT_DISTRIBUTION"

COLUMNS                    = 1

ROWS                       = 16 /* FITS NAXIS2 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

DESCRIPTION                = "

The pulse height distribution (PHD) is a one-dimensional

histogram, that bins each of the detected events (pulses) based on

their 'amplitude'. Detected events are those that are larger than

the discriminator setting, which is an analog (voltage) value. Of

the 16 PHD bins, both the top and bottom 3 bins don't get any

events (this is caused by the digitization of this amplitude

information), so only 10 bins contain non-zero values. The

sampling parameter interval is effectively unknown and somewhat

arbitrary anyway since only the relative shape and stability of

the distribution is of interest."

OBJECT                     = COLUMN

NAME                     = "PHD     "

DATA_TYPE                = MSB_INTEGER /* 16-bit signed integers */

BYTES                    = 2

START_BYTE               = 1

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = PULSE_HEIGHT_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 100 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 1.90000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the measured count

rate from the observation. Units are counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* 16-bit signed integer */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

END

## HIS, Level 3 (science histogram data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 109

FILE_NAME                    = "RA_040419231832_HIS0_SCI.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040419231832_HIS0_SCI.FIT",1)

^IMAGE                       = ("RA_040419231832_HIS0_SCI.FIT",13)

^WAVELENGTH_HEADER           = ("RA_040419231832_HIS0_SCI.FIT",59)

^WAVELENGTH_IMAGE            = ("RA_040419231832_HIS0_SCI.FIT",60)

^PULSE_HEIGHT_HEADER         = ("RA_040419231832_HIS0_SCI.FIT",106)

^PULSE_HEIGHT_TABLE          = ("RA_040419231832_HIS0_SCI.FIT",107)

^COUNT_RATE_HEADER           = ("RA_040419231832_HIS0_SCI.FIT",108)

^COUNT_RATE_SERIES           = ("RA_040419231832_HIS0_SCI.FIT",109)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

/* OBSERVATION GEOMETRY ELEMENTS */

/* S/C TO SUN ELEMENTS */

SC_SUN_POSITION_VECTOR       =

(1.1170497e+08, 71725860., 30237734.)

SC_SUN_VELOCITY_VECTOR       =

(-20.288503, 23.269792, 9.9525278)

SUB_SOLAR_LONGITUDE          = 160.16964

SUB_SOLAR_LATITUDE           = -4.4102611

/* S/C TO TARGET ELEMENTS */

SC_TARGET_POSITION_VECTOR    =

(-1.0000000e+32, -1.0000000e+32, -1.0000000e+32)

SC_TARGET_VELOCITY_VECTOR    =

(-1.0000000e+32, -1.0000000e+32, -1.0000000e+32)

SUB_SPACECRAFT_LONGITUDE     = -1.0000000e+32

SUB_SPACECRAFT_LATITUDE      = -1.0000000e+32

SPACECRAFT_ALTITUDE          = -1.0000000e+32

CENTRAL_BODY_DISTANCE        = -1.0000000e+32

/* INSTRUMENT BORESIGHT LINE OF SIGHT ELEMENTS */

RIGHT_ASCENSION              = 46.574495

DECLINATION                  = -80.222574

PHASE_ANGLE                  = 0

INCIDENCE_ANGLE              = -1.0000000e+32

EMISSION_ANGLE               = -1.0000000e+32

SPICE_FILE_NAME              = {"naif0008.tls"

"ROS_060511_STEP.TSC",

"ROS_V06.TF",

"EARTHFIXEDIAU.TF",

"DE403-MASSES.TPC",

"PCK00008.TPC",

"DE405S.BSP",

"C2002T7.bsp",

"TEMPEL1-9P-DI-P.BSP",

"mar033_2000-2025.bsp",

"sat164.bsp",

"2867_Steins_2004_2016.bsp",

"21_Lutetia_2004_2016.bsp",

"67P_CHURY_GERAS_2004_2016.BSP",

"ORER_______________00031.BSP",

"ORFR_______________00047.BSP",

"ORGR_______________00047.BSP",

"ORHR_______________00047.BSP",

"ORMR_______________00047.BSP",

"ATNR_P040302093352_00046.BC"}

/* DATA OBJECT DEFINITIONS */

OBJECT                       = HEADER

BYTES                      = 34560 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 12

DESCRIPTION                = "

FITS header for Rosetta-Alice calibrated (CODMAC Data Level 3)

histogram observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

OFFSET                     = 0.00000 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image for Rosetta-Alice calibrated (CODMAC Data Level 3)

histogram observation. Units are photons cm**-2 s**-1 Data are in

units of photons per square centimeter per second"

END_OBJECT                   = IMAGE

OBJECT                       = WAVELENGTH_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = WAVELENGTH_HEADER

OBJECT                       = WAVELENGTH_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

INTERCHANGE_FORMAT         = BINARY

OFFSET                     = 0.00000 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS extension containing calibrated wavelengths for each detector

pixel. Units are Angstroms. Wavelengths are in Angstroms."

END_OBJECT                   = WAVELENGTH_IMAGE

OBJECT                       = PULSE_HEIGHT_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PULSE_HEIGHT_HEADER

OBJECT                       = PULSE_HEIGHT_TABLE

NAME                       = "PULSE_HEIGHT_DISTRIBUTION"

COLUMNS                    = 1

ROWS                       = 16 /* FITS NAXIS2 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

DESCRIPTION                = "

The pulse height distribution (PHD) is a one-dimensional

histogram, that bins each of the detected events (pulses) based on

their 'amplitude'. Detected events are those that are larger than

the discriminator setting, which is an analog (voltage) value. Of

the 16 PHD bins, both the top and bottom 3 bins don't get any

events (this is caused by the digitization of this amplitude

information), so only 10 bins contain non-zero values. The

sampling parameter interval is effectively unknown and somewhat

arbitrary anyway since only the relative shape and stability of

the distribution is of interest."

OBJECT                     = COLUMN

NAME                     = "PHD     "

DATA_TYPE                = MSB_INTEGER /* 16-bit signed integers */

BYTES                    = 2

START_BYTE               = 1

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = PULSE_HEIGHT_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 100 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 1.90000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the measured count

rate from the observation. Units are counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* 16-bit signed integer */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

END

## HIS, Level4  (linearized histogram data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 113

FILE_NAME                    = "RA_080902110002_HIS3_LIN.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_080902110002_HIS3_LIN.FIT",1)

^IMAGE                       = ("RA_080902110002_HIS3_LIN.FIT",14)

^ERROR_IMAGE_HEADER          = ("RA_080902110002_HIS3_LIN.FIT",60)

^ERROR_IMAGE                 = ("RA_080902110002_HIS3_LIN.FIT",61)

^WAVELENGTH_HEADER           = ("RA_080902110002_HIS3_LIN.FIT",107)

^WAVELENGTH_TABLE            = ("RA_080902110002_HIS3_LIN.FIT",108)

^PULSE_HEIGHT_HEADER         = ("RA_080902110002_HIS3_LIN.FIT",110)

^PULSE_HEIGHT_TABLE          = ("RA_080902110002_HIS3_LIN.FIT",111)

^COUNT_RATE_HEADER           = ("RA_080902110002_HIS3_LIN.FIT",112)

^COUNT_RATE_SERIES           = ("RA_080902110002_HIS3_LIN.FIT",113)

^CALIBRATION_HEADER          = (“RA_080902110002_HIS3_LIN.FIT”,114)

^CALIBRATION_IMAGE           = (“RA_080902110002_HIS3_LIN.FIT”,115)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.2, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

/* OBSERVATION GEOMETRY ELEMENTS */

/* S/C TO TARGET ELEMENTS */

/* INSTRUMENT BORESIGHT LINE OF SIGHT ELEMENTS */

OBJECT                       = HEADER

BYTES                      = 37440 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 13

DESCRIPTION                = "

FITS header for Rosetta-Alice calibrated (CODMAC Data Level 4)

histogram observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image for Rosetta-Alice calibrated (CODMAC Data Level 4)

histogram observation. Data are in units of photons cm**-2 s**-1

Angstrom**-1."

END_OBJECT                   = IMAGE

OBJECT                       = ERROR_IMAGE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = ERROR_IMAGE_HEADER

OBJECT                       = ERROR_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS extension containing the statistical uncertainty associated

with each pixel in the primary extension. This is derived by

assuming Gaussian statistics, such that for a given number of

detected counts, N, the corresponding uncertainty is sqrt(N). Data

are in units of photons cm**-2 s**-1 Angstrom**-1"

END_OBJECT                   = ERROR_IMAGE

OBJECT                       = WAVELENGTH_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = WAVELENGTH_HEADER

OBJECT                       = WAVELENGTH_TABLE

INTERCHANGE_FORMAT         = BINARY

ROWS                       = 1024

COLUMNS                    = 1

ROW_BYTES                  = 4

DESCRIPTION                = "

FITS extension containing the wavelength vector in units of

Angstroms."

OBJECT                     = COLUMN

NAME                     = "WAVELENGTH"

DATA_TYPE                = IEEE_REAL /* 32-bit floating point */

BYTES                    = 4

START_BYTE               = 1

UNIT                     = "ANGSTROMS"

END_OBJECT                 = COLUMN

END_OBJECT                   = WAVELENGTH_TABLE

OBJECT                       = PULSE_HEIGHT_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PULSE_HEIGHT_HEADER

OBJECT                       = PULSE_HEIGHT_TABLE

INTERCHANGE_FORMAT         = BINARY

ROWS                       = 16

COLUMNS                    = 1

ROW_BYTES                  = 2

DESCRIPTION                = "

The pulse height distribution (PHD) is a one-dimensional

histogram, that bins each of the detected events (pulses) based on

their 'amplitude'. Detected events are those that are larger than

the discriminator setting, which is an analog (voltage) value. Of

the 16 PHD bins, both the top and bottom 3 bins don't get any

events (this is caused by the digitization of this amplitude

information), so only 10 bins contain non-zero values. The

sampling parameter interval is effectively unknown and somewhat

arbitrary anyway since only the relative shape and stability of

the distribution is of interest."

OBJECT                     = COLUMN

NAME                     = "PULSE_HEIGHT_DISTRIBUTION"

DATA_TYPE                = MSB_INTEGER /* 16-bit signed integers */

BYTES                    = 2

START_BYTE               = 1

OFFSET                   = 32768

END_OBJECT                 = COLUMN

END_OBJECT                   = PULSE_HEIGHT_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

INTERCHANGE_FORMAT         = BINARY

ROWS                       = 100 /* FITS NAXIS1 keyword */

COLUMNS                    = 1

ROW_BYTES                  = 2

SAMPLING_PARAMETER_INTERVAL= 244.800

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a vector that gives the measured count

rate from the observation. Units are counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNTS"

DATA_TYPE                = MSB_INTEGER

START_BYTE               = 1

BYTES                    = 2

OFFSET                   = 32768 /* FITS BZERO keyword */

END_OBJECT                = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

OBJECT                       = CALIBRATION_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = CALIBRATION_HEADER

OBJECT                       = CALIBRATION_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image containing the calibration constants for each pixel in

the data necessary to convert from units of counts to physical

units (photons cm^-2). The CODMAC level 2 EDR data (ENG files) has

been divided by this image during the calibration pipeline

processing. These calibration constants are a function of time and

thus are unique to each data file."

END_OBJECT                   = CALIBRATION_IMAGE

END

## PIX, Level 2 (engineering pixel list data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 63

FILE_NAME                    = "RA_040323225136_PIX0_ENG.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040323225136_PIX0_ENG.FIT",1)

^IMAGE                       = ("RA_040323225136_PIX0_ENG.FIT",7)

^PIXEL_LIST_HEADER           = ("RA_040323225136_PIX0_ENG.FIT",30)

^PIXEL_LIST_TABLE            = ("RA_040323225136_PIX0_ENG.FIT",31)

^COUNT_RATE_HEADER           = ("RA_040323225136_PIX0_ENG.FIT",49)

^COUNT_RATE_SERIES           = ("RA_040323225136_PIX0_ENG.FIT",50)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

/* DATA OBJECT DEFINITIONS */

OBJECT                       = HEADER

BYTES                      = 17280 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 6

DESCRIPTION                = "

FITS header for Rosetta-Alice uncalibrated (CODMAC Data Level

2)pixel list observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                =           16

SAMPLE_TYPE                = MSB_INTEGER

INTERCHANGE_FORMAT         = BINARY

OFFSET                     = 32768 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

Reconstructed histogram image for Rosetta-Alice uncalibrated

(CODMAC Data Level 2) pixel list observation. Units are raw data

numbers."

END_OBJECT                   = IMAGE

OBJECT                       = PIXEL_LIST_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PIXEL_LIST_HEADER

OBJECT                       = PIXEL_LIST_TABLE

COLUMNS                    = 1

ROWS                       = 24491 /* FITS extension NAXIS1 */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

DESCRIPTION                = "

These pixel list data are an array up to 32767 elements that gives

the X,Y positions of every detected event. In pixel list mode, the

event addresses that are generated by the detector are stored

sequentially in the memory array, as in a list. In addition to the

detector events, a time-mark value is also inserted into the list

on a periodic basis. The data are encoded as follows:

Value (for events) = X * 1024 + Y

Value (for timehacks) > 32768

Another way to describe this is by representing the bits in the

pixel list events (16-bit words) as:

h a a a a a e e e e e e e e e e

where each letter represents a single bit with the following meaning:

h - (MSB) set to 1 for a timehack, and 0 for a normal photon event.

For timehacks, the other 15 bits have no meaning.

a - the five bits of spatial information of a photon event.

e - the ten bits of spectral information of a photon event."

OBJECT                     = COLUMN

NAME                     = "PIXEL_LIST"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* Each item is signed 16 bit integer */

OFFSET                   = 32768 /* BZERO keyword in FITS extension header

END_OBJECT                 = COLUMN

END_OBJECT                   = PIXEL_LIST_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 19221 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 0.016000000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the count rate

measurements as reconstructed from the pixel list observation.

Units are raw counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* 16-bit signed integer */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

END

## PIX, Level 3 (science pixel list data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 138

FILE_NAME                    = "RA_040323225136_PIX0_SCI.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040323225136_PIX0_SCI.FIT",1)

^IMAGE                       = ("RA_040323225136_PIX0_SCI.FIT",13)

^WAVELENGTH_HEADER           = ("RA_040323225136_PIX0_SCI.FIT",59)

^WAVELENGTH_IMAGE            = ("RA_040323225136_PIX0_SCI.FIT",60)

^PIXEL_LIST_HEADER           = ("RA_040323225136_PIX0_SCI.FIT",106)

^PIXEL_LIST_TABLE            = ("RA_040323225136_PIX0_SCI.FIT",107)

^COUNT_RATE_HEADER           = ("RA_040323225136_PIX0_SCI.FIT",124)

^COUNT_RATE_SERIES           = ("RA_040323225136_PIX0_SCI.FIT",125)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

N.B.: The formats of the following sets of data elements are identical to those in Section 4.2, so their descriptions are omitted here:

/* OBSERVATION GEOMETRY ELEMENTS */

/* S/C TO TARGET ELEMENTS */

/* INSTRUMENT BORESIGHT LINE OF SIGHT ELEMENTS */

/* DATA OBJECT DEFINITIONS */

OBJECT                       = HEADER

BYTES                      = 34560 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 12

DESCRIPTION                = "

FITS header for Rosetta-Alice calibrated (CODMAC Data Level 3)

pixel list observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

OFFSET                     = 0.00000 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image for reconstructed Rosetta-Alice calibrated (CODMAC Data

Level 3 histogram observation. Units are photons cm**-2 s**-1 Data

are in units of photons per square centimeter per second"

END_OBJECT                   = IMAGE

OBJECT                       = WAVELENGTH_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = WAVELENGTH_HEADER

OBJECT                       = WAVELENGTH_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

INTERCHANGE_FORMAT         = BINARY

OFFSET                     = 0.00000 /* FITS BZERO keyword */

SCALING_FACTOR             = 1.00000 /* FITS BSCALE keyword */

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS extension containing calibrated wavelengths for each detector

pixel. Units are Angstroms. Wavelengths are in Angstroms."

END_OBJECT                   = WAVELENGTH_IMAGE

OBJECT                       = PIXEL_LIST_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PIXEL_LIST_HEADER

OBJECT                       = PIXEL_LIST_TABLE

COLUMNS                    = 4

ROWS                       = 5270 /* NAXIS2 in FITS extension header

ROW_BYTES                  = 9

INTERCHANGE_FORMAT         = BINARY

DESCRIPTION                = "

These pixel list data are a 4-column table that lists: X, Y,

Wavelength, Time Step. Any necessary calibrations (flatfield,

dark) are applied to that pixel location. The deadtime correction

is applied to the total counts in a timestep. Photons appearing at

different X,Y positions during a particular timestep are listed,

in the order detected, as records with the same Time Step value."

OBJECT                     = COLUMN

NAME                     = "X_POSITION"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* Each item is Integer*2 (short integer) */

OFFSET                   = 0

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "Y_POSITION"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 1

START_BYTE               = 3

ITEM_BYTES               = 1 /* Each item is Integer*1 (byte) */

OFFSET                   = 0

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "PIXEL LIST WAVELENGTH"

DESCRIPTION              = "Calibrated wavelength of pixel list event"

DATA_TYPE                = IEEE_REAL

BYTES                    = 4

START_BYTE               = 4

ITEM_BYTES               = 4 /* Each item is an IEEE floating point */

OFFSET                   = 0

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "PIXEL LIST TIME STEP"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 8

ITEM_BYTES               = 2 /* Each item is Integer*2 (short integer) */

OFFSET                   = 0

END_OBJECT                 = COLUMN

END_OBJECT                   = PIXEL_LIST_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 19221 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 0.016000000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the measured count

rate from the observation. Units are counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* 16-bit signed integer */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

END

## PIX, Level 4 (Linearized pixel list data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 276

FILE_NAME                    = "RA_080726111411_PIX2_LIN.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_080726111411_PIX2_LIN.FIT",1)

^IMAGE                       = ("RA_080726111411_PIX2_LIN.FIT",14)

^ERROR_IMAGE_HEADER          = ("RA_080726111411_PIX2_LIN.FIT",60)

^ERROR_IMAGE                 = ("RA_080726111411_PIX2_LIN.FIT",61)

^WAVELENGTH_HEADER           = ("RA_080726111411_PIX2_LIN.FIT",107)

^WAVELENGTH_TABLE            = ("RA_080726111411_PIX2_LIN.FIT",108)

^PIXEL_LIST_HEADER           = ("RA_080726111411_PIX2_LIN.FIT",110)

^PIXEL_LIST_TABLE            = ("RA_080726111411_PIX2_LIN.FIT",111)

^COUNT_RATE_HEADER           = ("RA_080726111411_PIX2_LIN.FIT",275)

^COUNT_RATE_SERIES           = ("RA_080726111411_PIX2_LIN.FIT",276)

^CALIBRATION_HEADER          = ("RA_080726111411_PIX2_LIN.FIT",290)

^CALIBRATION_IMAGE           = ("RA_080726111411_PIX2_LIN.FIT",291)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

N.B.: The formats of the following sets of data elements are identical to those in Section 4.2, so their descriptions are omitted here:

/* OBSERVATION GEOMETRY ELEMENTS */

/* S/C TO TARGET ELEMENTS */

/* INSTRUMENT BORESIGHT LINE OF SIGHT ELEMENTS */

/* DATA OBJECT DEFINITIONS */

OBJECT                       = HEADER

BYTES                      = 37440 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 13

DESCRIPTION                = "

FITS header for Rosetta-Alice calibrated (CODMAC Data Level 4)

pixel list observation."

END_OBJECT                   = HEADER

OBJECT                       = IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image for reconstructed Rosetta-Alice calibrated (CODMAC Data

Level 4) histogram observation. Data are in units of photons

cm**-2 s**-1 Angstrom**-1"

END_OBJECT                   = IMAGE

OBJECT                       = ERROR_IMAGE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = ERROR_IMAGE_HEADER

OBJECT                       = ERROR_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS extension containing the statistical uncertainty associated

with each pixel in the primary extension. This is derived by

assuming Poisson statistics, such that for a given number of

detected counts, N, the corresponding uncertainty is sqrt(N). Data

are in units of photons cm**-2 s**-1"

END_OBJECT                   = ERROR_IMAGE

OBJECT                       = WAVELENGTH_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = WAVELENGTH_HEADER

OBJECT                       = WAVELENGTH_TABLE

INTERCHANGE_FORMAT         = BINARY

ROWS                       = 1024

COLUMNS                    = 1

ROW_BYTES                  = 4

DESCRIPTION                = "

FITS extension containing the wavelength vector in units of

Angstroms."

OBJECT                     = COLUMN

NAME                     = "WAVELENGTH"

DATA_TYPE                = IEEE_REAL /* 32-bit floating point */

BYTES                    = 4

START_BYTE               = 1

UNIT                     = "ANGSTROMS"

END_OBJECT                 = COLUMN

END_OBJECT                   = WAVELENGTH_TABLE

OBJECT                       = PIXEL_LIST_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = PIXEL_LIST_HEADER

OBJECT                       = PIXEL_LIST_TABLE

COLUMNS                    = 5

ROWS                       = 11791 /* FITS NAXIS2 keyword */

ROW_BYTES                  = 40

INTERCHANGE_FORMAT         = ASCII

DESCRIPTION                = "

These pixel list data are a 4-column table that lists: X, Y,

Wavelength, Time Step. Any necessary calibrations (flatfield,

dark) are applied to that pixel location. The deadtime correction

is applied to the total counts in a timestep. Photons appearing at

different X,Y positions during a particular timestep are listed,

in the order detected, as records with the same Time Step value."

OBJECT                     = COLUMN

NAME                     = "X_POSITION"

DATA_TYPE                = ASCII_INTEGER

BYTES                    = 4

START_BYTE               = 1

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "Y_POSITION"

DATA_TYPE                = ASCII_INTEGER

BYTES                    = 3

START_BYTE               = 5

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "WAVELENGTH"

DESCRIPTION              = "Calibrated wavelength of pixel list event"

DATA_TYPE                = ASCII_REAL /* IEEE floating point */

BYTES                    = 8

START_BYTE               = 8

UNIT                     = "ANGSTROMS"

END_OBJECT                 = COLUMN

OBJECT                     = COLUMN

NAME                     = "TIME_STEP"

DATA_TYPE                = ASCII_INTEGER

BYTES                    = 6 /* Each item is Integer*2 (short integer) */

START_BYTE               = 16

END_OBJECT                 = COLUMN

END_OBJECT                   = PIXEL_LIST_TABLE

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

INTERCHANGE_FORMAT         = BINARY

ROWS                       = 221 /* FITS NAXIS1 keyword */

COLUMNS                    = 1

ROW_BYTES                  = 2

SAMPLING_PARAMETER_INTERVAL= 0.00800000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a vector that gives the count rate

measurements as reconstructed from the pixel list observation.

Units are raw counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNTS"

DATA_TYPE                = MSB_INTEGER

START_BYTE               = 1

BYTES                    = 2

OFFSET                   = 32768 /* FITS BZERO keyword */

END_OBJECT                = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

OBJECT                       = CALIBRATION_HEADER

BYTES                      = 2880 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 1

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = CALIBRATION_HEADER

OBJECT                       = CALIBRATION_IMAGE

LINE_SAMPLES               = 1024 /* FITS NAXIS1 keyword */

LINES                      = 32 /* FITS NAXIS2 keyword */

SAMPLE_BITS                = 32

SAMPLE_TYPE                = IEEE_REAL

AXIS_ORDER_TYPE            = FIRST_INDEX_FASTEST

LINE_DISPLAY_DIRECTION     = UP

SAMPLE_DISPLAY_DIRECTION   = RIGHT

DESCRIPTION                = "

FITS image containing the calibration constants for each pixel in

the data necessary to convert from units of counts to physical

units (photons cm^-2). The CODMAC level 2 EDR data (ENG files) has

been divided by this image during the calibration pipeline

processing. These calibration constants are a function of time and

thus are unique to each data file."

END_OBJECT                   = CALIBRATION_IMAGE

END

## CNT, Level 2 (engineering count rate data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 8

FILE_NAME                    = "RA_040419231322_CNT0_ENG.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040419231322_CNT0_ENG.FIT",1)

^COUNT_RATE_HEADER           = ("RA_040419231322_CNT0_ENG.FIT",7)

^COUNT_RATE_SERIES           = ("RA_040419231322_CNT0_ENG.FIT",8)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

/* DATA OBJECT DEFINITIONS */

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 17280 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 6

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 1000 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 2

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 0.090000000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the count rate

measurements as reconstructed from the pixel list observation.

Units are raw counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 2

START_BYTE               = 1

ITEM_BYTES               = 2 /* 16-bit signed integer */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN

END_OBJECT                   = COUNT_RATE_SERIES

END

## CNT, Level 3 (science count rate data)

PDS_VERSION_ID               = PDS3

/* FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 2880 /* FITS standard record length */

FILE_RECORDS                 = 15

FILE_NAME                    = "RA_040419231322_CNT0_SCI.LBL"

DATA_FORMAT                  = FITS

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HEADER                      = ("RA_040419231322_CNT0_SCI.FIT",1)             ^COUNT_RATE_HEADER           = ("RA_040419231322_CNT0_SCI.FIT",13)

^COUNT_RATE_SERIES           = ("RA_040419231322_CNT0_SCI.FIT",14)

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

N.B.: The formats of the following sets of data elements are identical to those in Section 4.2, so their descriptions are omitted here:

/* OBSERVATION GEOMETRY ELEMENTS */

/* S/C TO TARGET ELEMENTS */

/* INSTRUMENT BORESIGHT LINE OF SIGHT ELEMENTS */

/* DATA OBJECT DEFINITIONS */

OBJECT                       = COUNT_RATE_HEADER

BYTES                      = 34560 /* RECORD_BYTES x RECORDS */

HEADER_TYPE                = FITS

INTERCHANGE_FORMAT         = ASCII

RECORDS                    = 12

DESCRIPTION                = "FITS extension header"

END_OBJECT                   = COUNT_RATE_HEADER

OBJECT                       = COUNT_RATE_SERIES

COLUMNS                    = 1

ROWS                       = 1000 /* FITS NAXIS1 keyword */

ROW_BYTES                  = 4

INTERCHANGE_FORMAT         = BINARY

SAMPLING_PARAMETER_INTERVAL= 0.090000000

SAMPLING_PARAMETER_NAME    = TIME

SAMPLING_PARAMETER_UNIT    = SECONDS

DESCRIPTION                = "

These count rate data are a table that gives the count rate

measurements as reconstructed from the pixel list observation.

Units are raw counts per time hack interval"

OBJECT                     = COLUMN

NAME                     = "COUNT_RATE"

DATA_TYPE                = MSB_INTEGER

BYTES                    = 4

START_BYTE               = 1

ITEM_BYTES               = 4 /* 32-bit floating point */

OFFSET                   = 32768 /* FITS TZERO1 keyword */

END_OBJECT                 = COLUMN                                         END_OBJECT                   = COUNT_RATE_SERIES

END

## HKTM (housekeeping telemetry data)

PDS_VERSION_ID               = PDS3

/*  FILE CHARACTERISTICS DATA ELEMENTS */

RECORD_TYPE                  = FIXED_LENGTH

RECORD_BYTES                 = 449

FILE_RECORDS                 = 3210

FILE_NAME                    = "RA_040322224947_HKTM_ENG.LBL"  /* this label

/* DATA OBJECT POINTERS IDENTIFICATION DATA ELEMENTS */

^HOUSEKEEPING_TABLE          = ("RA_040322224947_HKTM_ENG.TAB",1)

/* TIME RANGE DATA ELEMENTS */

START_TIME                   = 2004-03-22T22:49:47.995  /* UTC */

STOP_TIME                    = 2004-03-23T06:50:49.512 /* UTC */

SPACECRAFT_CLOCK_START_COUNT = "1/41.511" /* OBT */

SPACECRAFT_CLOCK_STOP_COUNT  = "1/38645435.812" /* OBT */

/* OBSERVATION AND EXPOSURE DATA ELEMENTS */

INSTRUMENT_MODE_ID           = "N/A"

INSTRUMENT_MODE_DESC         = "N/A"

EXPOSURE_DURATION            = "N/A"

DATA_QUALITY_ID              = 0

DATA_QUALITY_DESC            = "Data quality is nominal"

MISSION_PHASE_NAME           = "COMMISSIONING 1"

/* see Phase Definition table in RSOC Design Specification, */

/* RO-EST-PL-2010 and Archive GVT Plan, RO-EST-PL-5011 */

TARGET_NAME                  = "N/A"

TARGET_TYPE                  = HOUSEKEEPING

N.B.: The formats of the following sets of data elements are identical to those in Section 4.1, so their descriptions are omitted here:

/* INSTRUMENT AND MISSION DESCRIPTIVE DATA ELEMENTS */

/* PRODUCT AND DATA SET DATA ELEMENTS */

/* STRUCTURE DEFINITION OF INSTRUMENT PARAMETER OBJECTS */

/* DESCRIPTION OF INSTRUMENT */

/* PARAMETERS INDEX FILE DEFINITION */

/* MISSION SPECIFIC KEYWORDS */

/* DATA OBJECT DEFINITION */

OBJECT = HOUSEKEEPING_HEADER

BYTES       = 3903

HEADER_TYPE = PLAIN_TEXT

END_OBJECT = HOUSEKEEPING_HEADER

OBJECT = HOUSEKEEPING_TABLE

INTERCHANGE_FORMAT = ASCII

ROWS               = 865

COLUMNS            = 73

ROW_BYTES          = 442

DESCRIPTION        = "Parameter values from Alice housekeeping telemetry (HKTM) data."

OBJECT = COLUMN

NAME        = "ScetR"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 1

BYTES       = 15

DESCRIPTION = "SpaceCraft Event Time (secs since 1970)"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "ScetC"

DATA_TYPE   = CHARACTER

STARTBYTE   = 15

BYTES       = 24

DESCRIPTION = "SpaceCraft Event Time (UTC)"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "InstTime"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 39

BYTES       = 15

DESCRIPTION = "Alice instrument time (OBT, seconds)"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TOffset"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 54

BYTES       = 15

DESCRIPTION = "Alice OBT offset from SCET (seconds)"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "InstSync"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 69

BYTES       = 2

DESCRIPTION = "Alice instrument time synchronized"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "HvOn"

DATA_TYPE   = CHARACTER

STARTBYTE   = 71

BYTES       = 4

DESCRIPTION = "HkHvOn"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "DetStim"

DATA_TYPE   = CHARACTER

STARTBYTE   = 75

BYTES       = 5

DESCRIPTION = "HkDetStim"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AcqMode"

DATA_TYPE   = CHARACTER

STARTBYTE   = 80

BYTES       = 10

DESCRIPTION = "HkAcqCountRate/HkAcqMode"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AcqActiv"

DATA_TYPE   = CHARACTER

STARTBYTE   = 90

BYTES       = 4

DESCRIPTION = "HkAcqActive"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "DetDoor"

DATA_TYPE   = CHARACTER

STARTBYTE   = 94

BYTES       = 7

DESCRIPTION = "HkDetDoor"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "ApDoor"

DATA_TYPE   = CHARACTER

STARTBYTE   = 101

BYTES       = 8

DESCRIPTION = "HkApDoor"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "Code"

DATA_TYPE   = CHARACTER

STARTBYTE   = 109

BYTES       = 8

DESCRIPTION = "EEPromCode/EEPromPage"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "State"

DATA_TYPE   = CHARACTER

STARTBYTE   = 117

BYTES       = 9

DESCRIPTION = "HkState"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "MirrHtr"

DATA_TYPE   = CHARACTER

STARTBYTE   = 126

BYTES       = 4

DESCRIPTION = "HkMirrorHtrOn"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "GratHtr"

DATA_TYPE   = CHARACTER

STARTBYTE   = 130

BYTES       = 4

DESCRIPTION = "HkGratingHtrOn"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "CntRate"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 134

BYTES       = 6

DESCRIPTION = "HkCountRate"

UNIT        = Hz

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "CntRateL"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 140

BYTES       = 6

DESCRIPTION = "HkCountRateLo"

UNIT        = Hz

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "CntRateH"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 146

BYTES       = 6

DESCRIPTION = "HkCountRateHi"

UNIT        = Hz

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "HvSetR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 152

BYTES       = 4

DESCRIPTION = "HkHvSet"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "HvSetC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 156

BYTES       = 5

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 161

BYTES       = 4

DESCRIPTION = "HkMcpV"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 165

BYTES       = 5

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVLoR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 170

BYTES       = 4

DESCRIPTION = "HkMcpVlo"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVLoC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 174

BYTES       = 5

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVHiR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 179

BYTES       = 4

DESCRIPTION = "HkMcpVhi"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "McpVHiC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 183

BYTES       = 5

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 188

BYTES       = 4

DESCRIPTION = "HkAnodeV"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 192

BYTES       = 4

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVLoR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 196

BYTES       = 4

DESCRIPTION = "HkAnodeVlo"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVLoC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 200

BYTES       = 4

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVHiR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 204

BYTES       = 4

DESCRIPTION = "HkAnodeVhi"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "AnoVHiC"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 208

BYTES       = 4

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrIR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 212

BYTES       = 4

DESCRIPTION = "HkStripI"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrIC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 216

BYTES       = 6

DESCRIPTION = ""

UNIT        = microAmp

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrILoR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 222

BYTES       = 4

DESCRIPTION = "HkStripIlo"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrILoC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 226

BYTES       = 6

DESCRIPTION = ""

UNIT        = microAmp

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrIHiR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 232

BYTES       = 4

DESCRIPTION = "HkStripIhi"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "StrIHiC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 236

BYTES       = 6

DESCRIPTION = ""

UNIT        = microAmp

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Mirr1R"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 242

BYTES       = 4

DESCRIPTION = "HkMirrorTempPri"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Mirr1C"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 246

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Mirr2R"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 252

BYTES       = 4

DESCRIPTION = "HkMirrorTempSec"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Mirr2C"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 256

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TS_MirrR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 262

BYTES       = 4

DESCRIPTION = "HkMirrorHtrSet"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TS_MirrC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 266

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Grat1R"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 272

BYTES       = 4

DESCRIPTION = "HkGratingTempPri"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Grat1C"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 276

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Grat2R"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 282

BYTES       = 4

DESCRIPTION = "HkGratingTempSec"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_Grat2C"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 286

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TS_GratR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 292

BYTES       = 4

DESCRIPTION = "HkGratingHtrSet"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TS_GratC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 296

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "DetDiscR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 302

BYTES       = 4

DESCRIPTION = "HkDetDiscr"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "DetDiscC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 306

BYTES       = 5

DESCRIPTION = ""

UNIT        = Volt

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "CycRem"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 311

BYTES       = 4

DESCRIPTION = "HkCyclesRemaining"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "LastPrsR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 315

BYTES       = 4

DESCRIPTION = "HkLastPressure"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "LastPrsC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 319

BYTES       = 6

DESCRIPTION = ""

UNIT        = mbar

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "LastPRaR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 325

BYTES       = 4

DESCRIPTION = "HkLastPrsRate"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "LastPRaC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 329

BYTES       = 6

DESCRIPTION = ""

UNIT        = mbar/s

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "PresTime"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 335

BYTES       = 15

DESCRIPTION = "HkPressureTime (OBT, seconds)"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "PresScet"

DATA_TYPE   = CHARACTER

STARTBYTE   = 350

BYTES       = 24

DESCRIPTION = "HkPressureTime (SCET, UTC)"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "Unsafe"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 374

BYTES       = 6

DESCRIPTION = "HkUnsafeTimer"

UNIT        = sec

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "Safety"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 380

BYTES       = 4

DESCRIPTION = "HkSafety"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "OverRide"

DATA_TYPE   = CHARACTER

STARTBYTE   = 384

BYTES       = 4

DESCRIPTION = "HkSafetyOverride"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "SafeMask"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 388

BYTES       = 4

DESCRIPTION = "HkSafetyMask"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_DElecR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 392

BYTES       = 4

DESCRIPTION = "HkDetElecTemp"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_DElecC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 396

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_DHousR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 402

BYTES       = 4

DESCRIPTION = "HkDetHousingTemp"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_DHousC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 406

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_CNDHR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 412

BYTES       = 4

DESCRIPTION = "HkCdhElecTemp"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_CNDHC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 416

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_IHousR"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 422

BYTES       = 4

DESCRIPTION = "HkInstrHousingTemp"

UNIT        = counts

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "T_IHousC"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 426

BYTES       = 6

DESCRIPTION = ""

UNIT        = degC

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "TestStat"

DATA_TYPE   = MSB_INTEGER

STARTBYTE   = 432

BYTES       = 4

DESCRIPTION = "HkTestStatus"

UNIT        = "N/A"

END_OBJECT = COLUMN

OBJECT = COLUMN

NAME        = "SwVers"

DATA_TYPE   = IEEE_REAL

STARTBYTE   = 436

BYTES       = 6

DESCRIPTION = "HkAfsVersion"

UNIT        = "N/A"

END_OBJECT = COLUMN

END_OBJECT = HOUSEKEEPING_TABLE

## Sample FITS header

Below is a sample of a FITS header from an Alice histogram calibrated science data file.  Note this is the main header, and that the headers for extensions in the FITS files are “minimal” headers that do not contain much descriptive information about the contents of the data block.  Some of the comments (the text following the forward slash in a line in a FITS header record) are truncated because the line length is limited to 80 characters.

SIMPLE  =                    T / Written by IDL:  Fri Mar 26 13:01:12 2010

BITPIX  =                  -32 / Number of bits per data pixel

NAXIS   =                    2 / Number of data axes

NAXIS1  =                 1024 / histogram spectral axis size (1-1024)

NAXIS2  =                   32 / histrogram spatial axis size (1-32)

EXTEND  =                    T / Fits data may contain extensions

ORIGIN  =               'SwRI' / Southwest Research Institute (R)

DATE    = '2010-02-12T04:39:52' / file creation/processing date/time

LIMA_VER=               '0.16' / LIMA software version number

LIMA_DAT=        ' 2007-02-26' / LIMA software version date

MISSION =            'Rosetta' / ESA comet exploration mission

INSTRUME=            'R-Alice' / Ultra-Violet Spectrometer

/

COMMENT Alice Histogram data acquisition: Received Histogram data

BUNIT   = 'photons cm**-2 s**-1' / Units of data in the primary HDU

BZERO   =              0.00000 / Data are 32 bit floating point numbers

BSCALE  =              1.00000 / No scaling applied to data

/

COMMENT Science TM Packet re-assembly results

TOTLPKTS=                   19 / expected number of packets

RECVPKTS=                   19 / received number of packets

LOSTPKTS=                    0 / missing packets (bitmask)

/

COMMENT Acquisition common: used calibration files

L_CALDIR= '../../../data/cal/lima/' / directory of LIMA calibration data

CALHVSET=     'ra_hvs_000.cal' / high voltage set calibration file

CALMCPV =     'ra_mcp_000.cal' / microchannel plate voltage calibration file

CALANOV =     'ra_ano_000.cal' / anode voltage calibration file

CALSTPI =     'ra_stp_000.cal' / strip current calibration file

CALDISC =     'ra_dis_000.cal' / discriminator voltage calibration file

CALTEMP =     'ra_tmp_000.cal' / temperature calibration file (all channels)

/

COMMENT Acquisition common section 1

SCETR   =       1172389756.810 / SpaceCraft Event Time (seconds since 1970)

SCETC   = '2007-02-25T07:49:16.810' / SpaceCraft Event Time (UTC)

INSTTIME=        131010527.893 / Alice Instrument time (OBT)

TOFFSET =       1041379228.917 / Alice OBT offset from SCET (seconds)

MCPMAXVR=                  132 / Mcp high level during acquisition (ADC counts)

MCPMAXVC=                -3727 / McpMaxV (Volt)

MCPMINVR=                  132 / Mcp low level during acquisition (ADC counts)

MCPMINVC=                -3727 / McpMinV (Volt)

MCPSTRVR=                  132 / Mcp level at start of acquisition (ADC counts)

MCPSTRVC=                -3727 / McpStartV (Volt)

MCPSTPVR=                  132 / Mcp level at end of acquisition (ADC counts)

MCPSTPVC=                -3727 / McpStopV (Volt)

STRMAXIR=                   67 / strip current high level during acquisition (AD

STRMAXIC=                 6.04 / StripMaxI (microAmp)

STRMINIR=                   63 / strip current low level during acquisition (ADC

STRMINIC=                 5.84 / StripMinI (microAmp)

STRSTRIR=                   65 / strip current at start of acquisition (ADC coun

STRSTRIC=                 5.94 / StripStartI (microAmp)

STRSTPIR=                   64 / strip current at end of acquisition (ADC counts

STRSTPIC=                 5.89 / StripStopI (microAmp)

/

COMMENT Common Housekeeping section; Hk acquired at start of data dump

HVON    =                 'on' / HkHvOn (on/off)

DETSTIM =               'stim' / HkDetStim (stim/off)

ACQMODE =          'Histogram' / AcquisitionMode (PixelList/Histogram/CountRate)

ACQACTIV=                'off' / HkAcqActive (on/off)

DETDOOR =               'open' / HkDetDoor (open/closed)

APDOOR  =               'open' / HkApDoor (error/closed/open/between)

CODE    =            'EEPROM3' / HkEEPromCode/HkEEPromPage (PROM/EEPROM0-3)

STATE   =               'dump' / HkState (checkout/acquire/dump/safe/hold/safedu

MIRRHTR =                'off' / HkMirrorHtrOn (on/off)

GRATHTR =                'off' / HkGratingHtrOn (on/off)

CNTRATE =                 2417 / HkCountRate (Hz)

CNTRATEL=                 2417 / HkCountRateLo (Hz)

CNTRATEH=                 2557 / HkCountRateHi (Hz)

HVSETR  =                  136 / High Voltage Set Point (DAC counts)

HVSETC  =                -3821 / HkHvSet (Volt)

MCPVR   =                  132 / Mcp Voltage (ADC counts)

MCPVC   =                -3819 / HkMcpV (Volt)

MCPVLOR =                  132 / Mcp Voltage low level during Hk period (ADC cou

MCPVLOC =                -3819 / HkMcpVlo (Volt)

MCPVHIR =                  132 / Mcp Voltage high level during Hk period (ADC co

MCPVHIC =                -3819 / HkMcpVhi (Volt)

ANOVR   =                  228 / Anode Voltage (ADC counts)

ANOVC   =                 -604 / HkAnodeV (Volt)

ANOVLOR =                  228 / Anode Voltage low level during Hk period (ADC c

ANOVLOC =                 -604 / HkAnodeVlo (Volt)

ANOVHIR =                  229 / Anode Voltage high level during Hk period (ADC

ANOVHIC =                 -607 / HkAnodeVhi (Volt)

STRPIR  =                   64 / Strip Current (ADC counts)

STRPIC  =                 5.89 / HkStripI (microAmp)

STRPILOR=                   64 / Strip Current low level during Hk period (ADC c

STRPILOC=                 5.89 / HkStripIlo (microAmp)

STRPIHIR=                   66 / Strip Current high level during Hk period (ADC

STRPIHIC=                 5.99 / HkStripIhi (microAmp)

T_MIRR1R=                   75 / Mirror Temperature 1 (ADC counts)

T_MIRR1C=                 15.3 / HkMirrorTempPri (degC)

T_MIRR2R=                   74 / Mirror Temperature 2 (ADC counts)

T_MIRR2C=                 14.9 / HkMirrorTempSec (degC)

TS_MIRRR=                    0 / Mirror Heater Set Point (ADC counts)

TS_MIRRC=                -33.3 / HkMirrorHtrSet (degC)

T_GRAT1R=                   72 / Grating Temperature 1 (ADC counts)

T_GRAT1C=                 14.1 / HkGratingTempPri (degC)

T_GRAT2R=                   71 / Grating Temperature 2 (ADC counts)

T_GRAT2C=                 13.7 / HkGratingTempSec (degC)

TS_GRATR=                    0 / Grating Heater Set Point (ADC counts)

TS_GRATC=                -33.3 / HkGratingHtrSet (degC)

DETDISCR=                    9 / Detector Dicriminator readback (ADC counts)

DETDISCC=                 0.11 / HkDetDiscr (Volt)

CYCLSREM=                    1 / HkCyclesRemaining (in current MAD command)

LASTPRSR=                   31 / Last reported ROSINA pressure (RosinaReal)

LASTPRSC=                1E-15 / HkLastPressure (mbar)

LASTPRAR=                    0 / Last reported ROSINA pressure trend (RosinaReal

LASTPRAC=                0E-00 / HkLastPrsRate (mbar/s)

PRESTIME=       1845493760.000 / HkPressureTime (instrument time)

PRESSCET= '0000-01-01T00:00:00.000' / HkPressureTime (SCET)

SAFETIME=                    0 / HkUnsafeTimer (sec, timeout remaining)

SAFETY  =                    0 / HkSafety (bitmask)

SAFEOVER=                'off' / HkSafetyOverride (on/off)

SAFEMASK=                    0 / HkSafetyMask (bitmask)

T_DELECR=                   87 / Detector Electronics Temperature (ADC counts)

T_DELECC=                 19.8 / HkDetElecTemp (degC)

T_DHOUSR=                   74 / Detector Housing temperature (ADC counts)

T_DHOUSC=                 14.9 / HkDetHousingTemp (degC)

T_CDHR  =                   97 / C&DH Electronics Temperature (ADC counts)

T_CDHC  =                 23.6 / HkCdhElecTemp (degC)

T_IHOUSR=                   73 / Instrument Housing Temperature (ADC counts)

T_IHOUSC=                 14.5 / HkInstrHousingTemp (degC)

TESTSTAT=                    0 / HkTestStatus (bitmask)

SOFTVERS=               '2.04' / HkAfsVersion (version number of instrument sw)

/

COMMENT Common Instrument Parameter section

HSNSGRAT=                'pri' / PmHtrSenseGrating (pri/sec)

HSNSMIRR=                'pri' / PmHtrSenseMirror (pri/sec)

DOORENA =            'enabled' / PmDoorEnable (enabled/disabled)

DACADCFR=                  231 / PmDacAdcFactor (/240)

DACADCFC=                0.963 / conversion factor from HvSet to McpV

EVNTSAFT=             'ground' / PmEventSafety (ground/on-board)

DISCSETR=                   10 / Discriminator acquisition setpoint (DAC counts)

DISCSETC=                 0.12 / PmDiscriminator (Volt)

HOTSEG1 =             '0-0000' / PmHotSeg1: spatial-spectral

HOTSEG2 =             '0-0000' / PmHotSeg2: spatial-spectral

HOTSEG3 =             '0-0000' / PmHotSeg3: spatial-spectral

HOTSEG4 =             '0-0000' / PmHotSeg4: spatial-spectral

HOTSEG5 =             '0-0000' / PmHotSeg5: spatial-spectral

HOTSEG6 =             '0-0000' / PmHotSeg6: spatial-spectral

HOTSEG7 =             '0-0000' / PmHotSeg7: spatial-spectral

HOTSEG8 =             '0-0000' / PmHotSeg8: spatial-spectral

HVLEVELR=                  136 / HV acquisition setpoint (DAC counts)

HVLEVELC=                -3821 / PmHvLevel (Volt)

HVSTEPR =                    4 / HV step size (DAC counts)

HVSTEPC =                  -92 / PmHvStepSize (Volt)

HVRTIMR =                    2 / HV runtime (kernel ticks)

HVRTIMC =                  0.2 / PmHvRunTime (sec)

MAXSTRIR=                  168 / Maximum Strip Current (ADC counts)

MAXSTRIC=                10.96 / PmHvMaxStripI (microAmp)

MINANOVR=                  188 / Minimum Anode Voltage (ADC counts)

MINANOVC=                 -499 / PmHvMinAnodeV (Volt)

MAXANOVR=                  245 / Maximum Anode Voltage (ADC counts)

MAXANOVC=                 -649 / PmHvMaxAnodeV (Volt)

MCPTOLR =                    8 / HV-Mcp tolerance (delta ADC counts)

MCPTOLC =                  184 / PmHvMcpTol (Volt)

ANOFAIL =                    4 / PmHvFailAnode (count)

MCPFAIL =                    4 / PmHvFailMcp (count)

STPFAIL =                    4 / PmHvFailStrip (count)

DOORSENS=            'enabled' / Door sensor enabled (top bit PmDoorTimeout)

DOORTIME=                    2 / Aperture Door control time, PmDoorTimeout (sec)

CNTINTVR=                   10 / Detector event counter interval (kernel ticks)

CNTINTVC=                 0.96 / PmCounterInterval (sec)

CNTSLOTS=                  100 / PmCountSlots (fixed at afs compile time)

MAXPRESR=                   21 / Pressure Limit (Rosina Real)

MAXPRESC=                1E-05 / PmMaxPressure (mbar)

PTIMMARR=                   60 / Pressure trend time horizon (sec*10)

PTIMMARC=                  600 / PmPressTimeMargin (sec)

T_MAXR  =                  192 / Maximum temperature (ADC counts)

T_MAXC  =                 57.7 / PmMaxTemp (degC)

T_MASK  =                  255 / PmTempMask (bitmask)

TOLGRATR=                    8 / Grating heater temperature tolerance (ADC cnts)

TOLGRATC=                  3.0 / PmHtrTolGrating (degC)

TOLMIRRR=                    8 / Mirror heater temperature tolerance (ADC counts

TOLMIRRC=                  3.0 / PmHtrTolMirror (degC)

TSTMASK =                    0 / PmTestMask (bitmask)

HKRATE  =                   30 / PmHkRate (sec, Hk generation rate)

DIAGENA =                'off' / PmDiagEnable (diag/off)

MAXCOUNT=                40000 / PmMaxCountRate (Hz, safety limit)

SAFTIME =                  600 / PmSafetyTimeout (sec)

MAXDUST =                   30 / PmMaxDust (part/dm2/sec?, safety limit)

/

COMMENT Acquisition common section 2

PRSMAXR =                   31 / high ROSINA pressure reported during acquisitio

PRSMAXC =                1E-15 / PressMax (mbar)

PRSMINR =                  144 / low ROSINA pressure reported during acquisition

PRSMINC =                9E-00 / PressMin (mbar)

STRTTIME=        131008713.511 / exposure start time (instrument time)

STRTSCET= '2007-02-25T07:19:02.427' / exposure start time (SCET)

DATE-OBS= '2007-02-25T07:19:02.427' / exposure start time (SCET)

STOPTIME=        131010527.885 / exposure stop time (instrument time)

STOPSCET= '2007-02-25T07:49:16.802' / exposure stop time (SCET)

EXPTIME =             1814.375 / Calculated actual exposure time (seconds)

OPENTIME=              1814.38 / Time aperture door was open

CLOSTIME=              0.00000 / Time aperture door was closed

DUMPNO  =                    0 / DumpNo, dump specification number (0-7)

DETCNTS =              4929498 / DetectorCounts, total counts from detector even

/

COMMENT Histogram acquisition specific keywords

ACQTIME =                 1800 / AcqTime, commanded integration time (sec)

WILOSPEC=                    0 / WinLowSpectral, spectral window lower (inclusiv

WIHISPEC=                 1023 / WinHighSpectral, spectral window upper (inclusi

WICOSPEC=                    1 / CollapseSpectral, spectral collapse factor

WILOSPAT=                    0 / WinLowSpatial, spatial window lower (inclusive)

WIHISPAT=                   31 / WinHighSpatial, spatial window upper (inclusive

WICOSPAT=                    1 / CollapseSpatial, spatial collapse factor

/

COMMENT Calculated event statistics

EVENTS  =              2205881 / Events counted in Histogram

PHDEVNTS=               428967 / Events counted in PHD

=============================================

MIKE_BEG= 'Mar 26 13:01:12 2010' / START MIKE KEYWORD BLOCK

MIKE_VER= '19 [2010 Mar 26]'   / Version of Mike pipeline code

FILE_IN = 'RA_070225071902_HIS0_ENG.FIT' / Input file for processing

FILE_OUT= 'RA_070225071902_HIS3_SCI.FIT' / Output file after processing

DATAQUAL=                    0 / Data quality flag

APDRSRC = 'HK event file'      / Source of aperture door status

BADFILE = '        '           / File containing bad pixel mask

BADFLAG =                    F / Bad pixel mask applied to data?

BADVALUE= 'IEEE NaN'           / Value of bad pixel(s)

DEADFLAG=                    T / Dead-time correction applied?

DARKFLAG=                    T / Dark count correction applied?

DARKFILE= 'RA_DARK_3.8kV_002.FIT' / Detector dark count rate file

DARKSCAL=              1.27725 / Scale factor applied to dark file

FLATFLAG=                    F / Flatfield correction applied?

FLATFILE= '        '           / Detector flatfield file

AEFFFLAG=                    T / Effective area correction applied?

AEFFFILE= 'RA_AEFF_008.TAB'    / Detector effective area file

HVFLAG  =                    F / High voltage correction applied?

HVFILE  = '        '           / High voltage correction file

TVAEFLAG=                    T / Correct for time-variable A_eff?

CORRFILE= 'RA_MARS_PC10_CORRECTION.TAB' / Effective Area correction file

WAVEFLAG=                    T / Stim pixel correction applied?

LINFLAG =                    F / Data resampled onto common wavelength scale?

WAVEFILE= 'RA_WAVE_006.FIT'    / Detector wavelength solution used

MIKE_END= '        '           / END MIKE KEYWORD BLOCK

=============================================

COMMENT BEGIN GEOMETRY KEYWORD BLOCK

COMMENT All values calulated for start time of observation

RA      =        65.8142337287 / Right Ascension of ALICE Boresight [deg.]

DEC     =        21.8709124504 / Declination of ALICE Boresight [deg.]

COMMENT S/C -> Sun Geometry

SCSUNX  =       -18972143.5674 / J2000 EME S/C -> Sun X vector [km]

SCSUNY  =        195683305.439 / J2000 EME S/C -> Sun Y vector [km]

SCSUNZ  =        90265665.7906 / J2000 EME S/C -> Sun Z vector [km]

SCSUNVX =       -21.5042533559 / X vel. of Sun wrt S/C in [km/s] (J2000 EME)

SCSUNVY =        3.30735071195 / Y vel. of Sun wrt S/C in [km/s] (J2000 EME)

SCSUNVZ =        2.12469639072 / Z vel. of Sun wrt S/C in [km/s] (J2000 EME)

SCSUNLON=        209.504261708 / Sub solar longitude [deg.] (planetographic)

SCSUNLAT=        1.07779260836 / Sub solar latitude [deg.] (planetographic)

COMMENT S/C -> Target Geometry

OBS_TYPE= '"MARS"  '           / Epoch of observation, e.g., PC6 or Steins flyby

TARGNAME= 'MARS    '           / Name of observation target

TARGTYPE= 'PLANET  '           / Type of observation target

TARGDESC= 'Mars    '           / Description of observation target

TARG_RA =        65.8301577740 / Right ascension of target [deg]

TARG_DEC=        21.8655652420 / Declination of target [deg]

SCTARGX =        65210.9681397 / J2000 EME S/C -> target X vector [km]

SCTARGY =        145305.488553 / J2000 EME S/C -> target Y vector [km]

SCTARGZ =        63913.9867847 / J2000 EME S/C -> target Z vector [km]

TARGDIST=        171613.382567 / Distance to target center [km]

SCTARGVX=        3.54557389891 / X vel. of target wrt S/C in [km/s] (J2000 EME)

SCTARGVY=        7.38340417000 / Y vel. of target wrt S/C in [km/s] (J2000 EME)

SCTARGVZ=        3.31748543788 / Z vel. of target wrt S/C in [km/s] (J2000 EME)

SCLON   =        230.977553395 / Sub-Spacecraft planetographic longitude [deg.]

SCLAT   =       -7.11670811521 / Sub-Spacecraft planetographic latitude [deg.]

SCALT   =        168217.494966 / Spacecraft altitude [km]

TARGPHAS=        152.597482027 / Sub-S/C Phase angle (Sun-Target-S/C) [deg]

COMMENT Row-specific Geometry Information

COMMENT The ALICE slit is imaged onto detector rows 5-23 (inclusive)

COMMENT When the LOS intercepts the surface, LAT and LON are given for

COMMENT the intercept point. If the LOS fails to intercept the target,

COMMENT LAT and LON are given for the nearest point to the target surface

COMMENT along the LOS.

ROW5RA  =        62.4374914157 / Right Ascension [deg.]

ROW5DEC =        21.1350696809 / Declination [deg.]

ROW5LON = '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW5LAT = '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW5PHA = '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW5INC = '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW5EMI = '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW6RA  =        62.7638395564 / Right Ascension [deg.]

ROW6DEC =        21.2135914539 / Declination [deg.]

ROW6LON = '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW6LAT = '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW6PHA = '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW6INC = '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW6EMI = '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW7RA  =        63.0927065088 / Right Ascension [deg.]

ROW7DEC =        21.2910615275 / Declination [deg.]

ROW7LON = '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW7LAT = '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW7PHA = '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW7INC = '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW7EMI = '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW8RA  =        63.4240919960 / Right Ascension [deg.]

ROW8DEC =        21.3674628800 / Declination [deg.]

ROW8LON = '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW8LAT = '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW8PHA = '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW8INC = '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW8EMI = '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW9RA  =        63.7579954981 / Right Ascension [deg.]

ROW9DEC =        21.4427784547 / Declination [deg.]

ROW9LON = '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW9LAT = '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW9PHA = '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW9INC = '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW9EMI = '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW10RA =        64.0944162463 / Right Ascension [deg.]

ROW10DEC=        21.5169911656 / Declination [deg.]

ROW10LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW10LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW10PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW10INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW10EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW11RA =        64.4333532184 / Right Ascension [deg.]

ROW11DEC=        21.5900839035 / Declination [deg.]

ROW11LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW11LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW11PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW11INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW11EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW12RA =        64.7748051329 / Right Ascension [deg.]

ROW12DEC=        21.6620395412 / Declination [deg.]

ROW12LON=        174.174557891 / Planetographic Lon. of target along LOS [deg.]

ROW12LAT=        17.0745143602 / Planetographic Lat. of target along LOS [deg.]

ROW12PHA=        151.596104282 / Phase angle at LOS intercept [deg.]

ROW12INC=        146.317936222 / Incidence angle at LOS intercept [deg.]

ROW12EMI=        62.0850962213 / Emission angle at LOS intercept [deg.]

ROW13RA =        65.1187704446 / Right Ascension [deg.]

ROW13DEC=        21.7328409405 / Declination [deg.]

ROW13LON=        198.364690953 / Planetographic Lon. of target along LOS [deg.]

ROW13LAT=        7.98019659872 / Planetographic Lat. of target along LOS [deg.]

ROW13PHA=        151.923828312 / Phase angle at LOS intercept [deg.]

ROW13INC=        171.582134828 / Incidence angle at LOS intercept [deg.]

ROW13EMI=        36.4880092856 / Emission angle at LOS intercept [deg.]

ROW14RA =        65.4652473392 / Right Ascension [deg.]

ROW14DEC=        21.8024709577 / Declination [deg.]

ROW14LON=        215.278738407 / Planetographic Lon. of target along LOS [deg.]

ROW14LAT=       0.418564656459 / Planetographic Lat. of target along LOS [deg.]

ROW14PHA=        152.253346976 / Phase angle at LOS intercept [deg.]

ROW14INC=        169.929330316 / Incidence angle at LOS intercept [deg.]

ROW14EMI=        17.6864175204 / Emission angle at LOS intercept [deg.]

ROW15RA =        65.8142337287 / Right Ascension [deg.]

ROW15DEC=        21.8709124504 / Declination [deg.]

ROW15LON=        230.533361650 / Planetographic Lon. of target along LOS [deg.]

ROW15LAT=       -6.47280279742 / Planetographic Lat. of target along LOS [deg.]

ROW15PHA=        152.584702020 / Phase angle at LOS intercept [deg.]

ROW15INC=        153.222857087 / Incidence angle at LOS intercept [deg.]

ROW15EMI=       0.727935411944 / Emission angle at LOS intercept [deg.]

ROW16RA =        66.1657272467 / Right Ascension [deg.]

ROW16DEC=        21.9381482838 / Declination [deg.]

ROW16LON=        246.219193646 / Planetographic Lon. of target along LOS [deg.]

ROW16LAT=       -12.9184101543 / Planetographic Lat. of target along LOS [deg.]

ROW16PHA=        152.917900976 / Phase angle at LOS intercept [deg.]

ROW16INC=        136.482228596 / Incidence angle at LOS intercept [deg.]

ROW16EMI=        16.4373406027 / Emission angle at LOS intercept [deg.]

ROW17RA =        66.5197252438 / Right Ascension [deg.]

ROW17DEC=        22.0041613374 / Declination [deg.]

ROW17LON=        264.514851221 / Planetographic Lon. of target along LOS [deg.]

ROW17LAT=       -18.9056117618 / Planetographic Lat. of target along LOS [deg.]

ROW17PHA=        153.252938376 / Phase angle at LOS intercept [deg.]

ROW17INC=        117.912485380 / Incidence angle at LOS intercept [deg.]

ROW17EMI=        35.3407227315 / Emission angle at LOS intercept [deg.]

ROW18RA =        66.8762247829 / Right Ascension [deg.]

ROW18DEC=        22.0689345119 / Declination [deg.]

ROW18LON=        291.311656507 / Planetographic Lon. of target along LOS [deg.]

ROW18LAT=       -23.8737524681 / Planetographic Lat. of target along LOS [deg.]

ROW18PHA=        153.589777159 / Phase angle at LOS intercept [deg.]

ROW18INC=        92.5136169109 / Incidence angle at LOS intercept [deg.]

ROW18EMI=        61.0761734673 / Emission angle at LOS intercept [deg.]

ROW19RA =        67.2352226354 / Right Ascension [deg.]

ROW19DEC=        22.1324507360 / Declination [deg.]

ROW19LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW19LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW19PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW19INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW19EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW20RA =        67.5967152765 / Right Ascension [deg.]

ROW20DEC=        22.1946929735 / Declination [deg.]

ROW20LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW20LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW20PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW20INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW20EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW21RA =        67.9606988821 / Right Ascension [deg.]

ROW21DEC=        22.2556442305 / Declination [deg.]

ROW21LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW21LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW21PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW21INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW21EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW22RA =        68.3271693239 / Right Ascension [deg.]

ROW22DEC=        22.3152875628 / Declination [deg.]

ROW22LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW22LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW22PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW22INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW22EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

ROW23RA =        68.6961221668 / Right Ascension [deg.]

ROW23DEC=        22.3736060834 / Declination [deg.]

ROW23LON= '"N/A"   '           / Planetographic Lon. of target along LOS [deg.]

ROW23LAT= '"N/A"   '           / Planetographic Lat. of target along LOS [deg.]

ROW23PHA= '"N/A"   '           / Phase angle at LOS intercept [deg.]

ROW23INC= '"N/A"   '           / Incidence angle at LOS intercept [deg.]

ROW23EMI= '"N/A"   '           / Emission angle at LOS intercept [deg.]

COMMENT SPICE files used in geometry calculations

NUMBSPCK=                   19 / Number of loaded SPICE kernels

SPCK0001= 'NAIF0009.TLS'       / SPICE kernel 0001

SPCK0002= 'ROS_091210_STEP.TSC' / SPICE kernel 0002

SPCK0003= 'ROS_V16.TF'         / SPICE kernel 0003

SPCK0004= 'EARTHFIXEDIAU.TF'   / SPICE kernel 0004

SPCK0005= 'PCK00008.TPC'       / SPICE kernel 0005

SPCK0006= 'DE405.BSP'          / SPICE kernel 0006

SPCK0007= 'C2002T7.BSP'        / SPICE kernel 0007

SPCK0008= 'TEMPEL1-9P-DI-P.BSP' / SPICE kernel 0008

SPCK0009= 'OUTERPLANETS_V0002.BSP' / SPICE kernel 0009

SPCK0010= 'ORHR_______________00102.BSP' / SPICE kernel 0010

SPCK0011= 'ORER_______________00031.BSP' / SPICE kernel 0011

SPCK0012= 'ORMR_______________00052.BSP' / SPICE kernel 0012

SPCK0013= 'ORFR_______________00067.BSP' / SPICE kernel 0013

SPCK0014= 'ORGR_______________00096.BSP' / SPICE kernel 0014

SPCK0015= 'ORHW_______________00016.BSP' / SPICE kernel 0015

SPCK0016= 'ORHO_______________00077.BSP' / SPICE kernel 0016

SPCK0017= 'ORHS_______________00081.BSP' / SPICE kernel 0017

SPCK0018= 'ATNR_P040302093352_00101.BC' / SPICE kernel 0018

SPCK0019= 'ROS_ALICE_V15.TI'   / SPICE kernel 0019

END