From GHewitt@jftl.jpl.nasa.gov Thu Mar 30 14:09:26 2006

Return-Path: <GHewitt@jftl.jpl.nasa.gov>

Received: from jftl.jpl.nasa.gov (jftl.jpl.nasa.gov [128.149.207.22])

by magellan.stanford.edu (8.11.7p1+Sun/8.11.7) with SMTP id k2UM9Od28008

for <rsimpson@magellan.Stanford.EDU>; Thu, 30 Mar 2006 14:09:24 -0800 (PST)

Received: from jftl.jpl.nasa.gov (Not Verified[128.149.207.21]) by jftl.jpl.nasa.gov with MailMarshal (v5,0,3,100)

id <B0003a0e19>; Thu, 30 Mar 2006 14:11:03 -0800

Received: by jftl-mm.jpl.nasa.gov with Internet Mail Service (5.5.2657.72)

id <F1D0Y5HB>; Thu, 30 Mar 2006 14:09:15 -0800

Message-ID: <12373B687CF8324F843BE0CA103AA6F402EAFED3@jftl-mm.jpl.nasa.gov>

From: "Hewitt, Greg" <GHewitt@jftl.jpl.nasa.gov>

To: DSN-NOCC <dsn-nocc@jjpl.jpl.nasa.gov>,

"Asmar, Sami W" <asmar@mail.jpl.nasa.gov>,

"Goltz, Gene L" <ggoltz@mail.jpl.nasa.gov>,

DSN-PREDICTS <DSN-PREDICTS@jjpl.jpl.nasa.gov>,

DSN-RMDCT <DSN-RMDCT@jjpl.jpl.nasa.gov>,

"'John.Reynolds@esa.int'" <John.Reynolds@esa.int>,

"Kahan, Daniel S" <kahan@mail.jpl.nasa.gov>,

DSNO&M-DSOT <DSNO&M-DSOT@jftl.jpl.nasa.gov>,

"'Dick Simpson 650-723-3525'" <rsimpson@magellan.stanford.edu>,

MDSCC-Operations <MDSCC-Operations@mdscc.nasa.gov>

Cc: "Holmes, Dwight P" <dholmes@mail.jpl.nasa.gov>,

"Thompson, Thomas W" <twt@ampersand.jpl.nasa.gov>,

"Angold, Nigel G" <nangold@mail.jpl.nasa.gov>,

DSN-MPSETD <DSN-MPSETD@jjpl.jpl.nasa.gov>,

"'Gary.Whitehead-esa.int'" <Gary.Whitehead@esa.int>,

"'Alan.Moorhouse@esa.int'" <Alan.Moorhouse@esa.int>,

"Wallick, Jack" <JWallick@jftl.jpl.nasa.gov>,

"Landon, Arthur J" <ALandon@jftl.jpl.nasa.gov>,

"Berman, Allen L" <alberman@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Beyer, Patrick E" <pbeyer@mail.jpl.nasa.gov>,

"Williams, Raymond C" <rcw@mail.jpl.nasa.gov>,

"Medina, Margaret" <MMedina@jftl.jpl.nasa.gov>,

"Berner, Jeff B" <jbberner@mail.jpl.nasa.gov>,

"Guerrero, Anamaria P" <ana@mail.jpl.nasa.gov>,

"Buck, Robert D" <rbuck@mail.jpl.nasa.gov>,

"Klose, J C" <cklose@mail.jpl.nasa.gov>,

"Recce, David J" <drecce@mail.jpl.nasa.gov>,

"Marina, Miguel" <mmarina@mail.jpl.nasa.gov>,

"Hodgin, Wendy K" <whodgin@mail.jpl.nasa.gov>,

"Hodder, James A" <jahodder@mail.jpl.nasa.gov>,

"Eaton, Curtis A" <ceaton@mail.jpl.nasa.gov>,

"Davis, Esker B" <eskerb@mail.jpl.nasa.gov>,

"Kurtik, Susan C" <skurtik@mail.jpl.nasa.gov>

Subject: BRIEFING MESSAGE FOR MEX BI STATIC RADAR SUPPORT DOY 092 DSS-63

Date: Thu, 30 Mar 2006 14:08:59 -0800

MIME-Version: 1.0

X-Mailer: Internet Mail Service (5.5.2657.72)

Content-Type: multipart/mixed;

boundary="----_=_NextPart_000_01C65446.8B0741E0"

Content-Length: 332253

Status: O

Mars Express

Briefing Message

Level 3

DSS-63 Bi-Static Radar Experiment Support

Prepared by: G Hewitt / MEX  NOPE / MPSETD

Origination Date: DOY 089 / 22:10 UTC

Revision: Original

DSS-63 DOY 092 / 1405 UTC

ROUTING: JFTL / JJPL / LRID

FROM: MPSETD G Hewitt

SUBJECT: DSS-63 Mars Express Bi-Static Radar Support

NOP: 871-049-041, REV.C

SCHEDULE: Refer to DSN 7-Day Schedule for actual support times.

LEVEL / TYPE: Level 3 Bi-Static Radar

DOY   Start  BOT  EOT  End  Facility  User     Activity        Pass

092    1405 1605 1915 2015   DSS-63    MEX  T/P BI STATIC R/S  1037

1. OVERVIEW:

Bi Static Radar (BSR) experiment employs the MEX spacecraft's transmitter

and a DSN 70-m antenna to receive weak S-band and X-band dual polarized RF

signals and Radio Science Receivers (RSRs) to simultaneously receive and

record these four signals. At a predetermined time during the pass, the

spacecraft slews toward Mars so that its transmitted S-band and X-band RCP

signals illuminate the planet's surface. As these RCP signals bounce off the

surface, part of the signals change polarization to Left Circular

Polarization (LCP) effectively creating four signals; 1) S-band RCP, 2)

S-band LCP, 3) X-band RCP and 4) X-band LCP.

DSN Mars Express Bistatic Radar (BSR) calibration procedures are performed

in three parts: 1) Pre-calibration, 2) Mini- calibration and 3)

Post-calibration. Section-8 details the pre-calibration procedures,

section-9 details mini-calibration procedures, and section-10 details post

calibration procedures.

2. SUPPORT DOCUMENTATION:

a. Mars Express NOP 871-049-041, Rev. C

b. This briefing message

c. Bi-Static Radar SOE (see attachment) supersedes DSN SOE. Radio

Science shall provide start/stop times of BSR mini-pre-calibrations near

real-time.

3. SUPPORT REQUIREMENTS:

NOTE:  Do not use Time Stacked Directives.  We have experienced problems

with certain time stacked directives slow to execute causing the next

directive in the queue to be rejected.  Consequently, this leads to

configuration errors and the need to repeat steps.

a. DOWNLINK SUPPORT ONLY

b. DTTS (DOWNLINK TRACKING AND TELEMETRY) RECEIVERS:

*  One DTT shall be configured with table 7041HRX to support X-band,

and to control the 12.5k noise diode on/off at predetermined times during

pre-pass calibrations, real-time BSR activities and post calibrations

activities.

*  A second DTT shall be configured with table 7041HRXS to support

S-band carrier only and to control the 12.5 k noise diode on/off at

predetermined times during pre-pass calibrations, real-time BSR activities

and post calibrations activities.

c. UPLINK:

*  There are NO uplink requirements

d. COMMAND:

*  There are NO command requirements

e. RADIO SCIENCE RECEIVERS (RSR):

NOTE: For remote operations the FSPR MUST NOT be assigned to the DSS link.

The RSRs are controlled and configured remotely by the Radio Science group

located at JPL. The RSR operator will configure four radio science receivers

to simultaneously receive and record S-band and X-band RCP and LCP polarized

signals. The RSR equipment requires both S-band and X-band 1-way and 2-way

downlink predicts. The actual RSR resources configured for support will

depend on availability.

*  R1B IFS 63_S_RCP

*  R2A IFS 63_S_LCP

*  R1A IFS 63_X_RCP

*  R2B IFS 63_X_LCP

4. NOCT:

Transmit X-band up/down, and X-up/S-band down predicts to the station

5. GCF:

Voice: DSS /AMMOS / MEX MOC/ RS

6. DSOT

There is NO SLE telemetry return link services required

7. SPACECRAFT CONFIGURATION:

The spacecraft shall be reconfigured for X-band and S-band downlink carrier

only in accordance with the BSR SOE (see attachment).

8. PRE-CALIBRATION PROCEDURES AND CONFIGURATION

The times shown below in step-1 through step-18 are to illustrate event

chronology and event duration:

a. Pre-calibration requires configuring the 70m-antenna microwave

subsystem so that each receiving downlink channel (S-RCP, S-LCP, X-RCP,

X-LCP) can be cycled in sequence through the four configurations listed

below.  The antenna must be pointed to zenith for pre-calibration.

1. Receiving channel into ambient load with noise diode off

2. Receiving channel into ambient load with noise diode on

3. Receiving channel into sky (zenith) with noise diode on

4. Receiving channel into sky (zenith) with noise diode off

NOTE: DTT01 is used for X-band and DTT02 is used for S-band, but any

available DTT resource can be used.

b. Link controller perform the procedures in step 1 and advise radio

science when completed; wait at this point until directed by radio science

to proceed to step 2.

Step 01  00:00:00  US63 MOD 102 A         Confirm dichroic reflector in place

US63 CNF XRA1O1        X-RCP (LNA-1) to Sky

US63 CNF XLA2O2        X-LCP (LNA-2) to Sky

US63 CNF SLLA2C2O2     S-LCP (LNA-2) low noise to sky

US63 CNF SRDA1C1O1     S-RCP (LNA-1) via diplexer to sky

AP SCAN OFF

CONSCAN Off

AP SRC TRK             Sub-reflector in Tracking Mode

c. NOPE use the connection log display to verify DSS-63's microwave

switches are configured in accordance with step-1

d. Radio science operator set each RSR attenuator using the AUTO

feature. After checking for proper signal levels (about -10.0 dB ADC

amplitude on each RSR), direct the link controller to proceed to step 2.

Step 02  00:04:00  US63 CNF XA1A2O2       LNA-2 to ambient load-1 (X-LCP)

US63 CNF XA1A1O1       LNA-1 to ambient load-1 (X-RCP)

Step 03  00:06:00  US63 CNF SA2A2C2O2     LNA-2 to ambient load-2 (S-LCP)

Step 04  00:08:00  US63 CNF SA1A1C1O1     LNA-1 to ambient load-1 (S-RCP)

e. Link controller and radio science operator after completing steps 2

through 4, perform the following procedures, (1) through (5):

(1) Link Controller report ambient load temperatures (S1, S2, and X1)

from the PRFREA display and report current weather conditions,

(temperature, humidity, and sky conditions) to the Radio Science

operator.

(2) Radio Science operator set each RSR attenuator using AUTO feature

(3) RSR operator run radio science recorders

(4) RSR operator report to Link Controller when radio science recorders

are recording

(5) Link controller execute the following procedures at 5 min intervals

and report the completion of each step to the RSR operator:

Step 05  00:10:00  DC01 DISC

DC01 CONN DSS63XO1     Configure DTT 01 to X-RCP

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC

DC02 CONN DSS63SO1     Configure DTT 02 to S-RCP

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step 06  00:15:00  DC01 SNT E             Enable X-RCP noise diode

Step 07  00:20:00  US63 CNF XRA1O1        X-RCP to sky

US63 CNF XLA2O2        X-LCP to sky

Step 08  00:25:00  DC01 SNT D             Disable X-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS63XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 09  00:30:00  DC01 SNT E             Enable X-LCP noise diode

Confirm noise diode selection is 12.5

Step 10  00:35:00  US63 CNF XA1A1O1       X-RCP (LNA-1) to ambient load

US63 CNF XA1A2O2       X-LCP (LNA-2) to ambient load

00:36:00  Link controller report ambient load temp (S1, S2, X1)

from the PRFREA display to the RS operator.

Step 11  00:40:00  DC01 SNT D             Disable X-LCP noise diode

Step 12  00:45:00  US63 CNF XRA1O1        X-RCP (LNA-1) to sky

US63 CNF XLA2O2        X-LCP (LNA-2) to sky

Step 13  00:50:00  DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS63XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 14  00:55:00  US63 CNF SRDA1C1O1     S-RCP (LNA-1) via diplexer to sky

Step 15  01:00:00  DC02 SNT D             Disable S-RCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS63SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Confirm noise diode is 12.5

Step 16  01:05:00  DC02 SNT E             Enable S-LCP noise diode

Step 17  01:10:00  US63 CNF SLLA2C2O2     S-LCP (LNA-2) to sky

Step 18  01:15:00  DC02 SNT D             Disable S-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS63SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Link controller report ambient load physical temperatures

(S1,S2, and X1) from PRFREA display to the RS operator.

01:20:00  End of BSR noise measurement pre-cal procedure

9. MINI CALIBRATION PROCEDURES AND TIMELINE

a. Link controller, shortly after MEX begins its slew and before the

BSR surface observations begin, confirm DTT 01 and DTT 02 configurations.

b. Link controller confirm DTT 01 is configured for X-RCP; if not,

perform the following operator directives:

1. DC01 DISC

2. DC01 CONN DSS63XO1

3. DC01 SET SNT NDS=12.5

4. DC01 SET SNT MFQ=ON

c.  Link controller confirm DTT 02 is configured for S-RCP; if not,

perform the following operator directives:

1. DC02 DISC

2. DC02 CONN DSS63SO1

3. DC02 SET SNT NDS=12.5

4. DC02 SET SNT MFQ=ON

d. RSR operator ensure all radio science Recorders are recording.

e. Link controller, at direction from RSR operator, begin the BSR

mini-noise calibration procedures. The times shown are to illustrate event

chronology and event duration:

Step 01  00:00:00  DC01 SNT E             Enable X-RCP noise diode

Confirm noise diode value is 12.5

Step 02  00:03:00  DC01 SNT D             Disable X-RCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS63XO2

US63 D XMAP            Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 03  00:06:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS63SO2

US63 D SMAP            Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step 04  00:09:00  DC01 SNT D             Disable X-LCP noise diode

DC02 SNT E             Enable S-LCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS63XO1

US63 D XMAP            Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 05  00:12:00  DC02 SNT D             Disable S-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS63SO1

US63 D SMAP            Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

00:15:00  End of BSR Mini-Noise Cal.

Step 06            Link controller, repeat the BSR mini-noise calibration

procedures steps 1-5 after MEX has completed its surface

observations and before it returns to Earth point. The

start time will be provided by radio science.

10. POST CALIBRATION PROCEDURES AND TIMELINE:

Step 01  00:00:00  Link controller move antenna to zenith

US63 CNF SA1A1C1O1     S-RCP (LNA-1) to ambient load

US63 CNF SA2A2C2O2     S-LCP (LNA-2) to ambient load

US63 CNF XA1A1O1       X-RCP (LNA-1) to ambient load

US63 CNF XA1A2O2       X-LCP (LNA-2) to ambient load

DC01 DISC              Confirm DTT 01 for X-RCP

DC01 CONN DSS63XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC              Confirm DTT 02 for S-RCP

DC02 CONN DSS63SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

a. Link controller advise the RSR operator when the antenna is at

zenith and the configurations listed above have been completed.

b. Link controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display.

c. RSR operator run all radio science Recorders and record ambient

loads for 3 min. At completion, advise the link controller to proceed with

the following steps:

Step 02  00:03:00  DC01 SNT E             Enable X-RCP noise diode

Step 03  00:06:00  DC01 SNT D             Disable X-RCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS63XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 04  00:09:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS63SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step 05  00:12:00  DC01 SNT D             Disable X-LCP noise Diode

DC02 SNT E             Enable S-LCP noise diode

Step 06  00:15:00  US63 CNF SRDA1C1O1     S-RCP (LNA-1) to sky

US63 CNF SLLA2C2O2     S-LCP (LNA-2) to sky

US63 CNF XRA1O1        X-RCP (LNA-1) to sky

US63 CNF XLA2O2        X-LCP (LNA-2) to sky

Step 07  00:18:00  DC02 SNT D             Disable S-LCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS63SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step 08  00:21:00  DC01 SNT D             Disable X-LCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS63XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step 09  00:24:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-RCP noise diode

Step 10  00:27:00  DC01 SNT D             Disable X-RCP noise diode

00:30:00                         End

d. Link Controller report physical temperatures of each ambient load

(S1, S2, and X1) from the downlink PRFREA display and report the current

weather (temperature, humidity, and sky conditions) to radio science.

e. Radio Science operator stop RSRs.

f. End of BSR noise post-calibration procedures.

g. Link Controller proceed with normal tear-down procedures.

h. Re-initialization of the DTTs is recommended.

MPSETD POINTS OF CONTACT: DSN-MPSETD@jjpl.jpl.nasa.gov

<mailto:DSN-MPSETD@jjpl.jpl.nasa.gov>

MPSETD                OFFICE     OPS WORK STATION

Jose Valencia (LEAD)   (626) 305-6308   (626) 305-6370

(626) 305-6368

Jesse Velasco (NOPE)   (626) 305-6315

Romy Paredes (NOPE)    (626) 305-6309

Greg Hewitt (NOPE      (626) 305-6292

RADIO SCIENCE WORK STATION              (818) 393- 0666

Regards,

Gregory W. Hewitt

(626) 305-6292 (Office)

(626) 491-0017 (Pager)

(626) 305-6389 (Fax)

ghewitt@jftl.jpl.nasa.gov <mailto:ghewitt@jftl.jpl.nasa.gov>