Mars Express Bistatic Radar Experiment

Operations Plan and Report

02 April 2006

Summary

=======

DSN Antenna:                                          63

Pass:                                               1037

Orbit:                                              2857

Orbit Start Time:                    2006-04-02T12:54:13

HGA Pointing:                                   Inertial

Specular Condition (ERT):                          18:00

Target Latitude (deg N):                           -2.46

Longitude (deg E):                        -157.75

Rp (km):                                 3395.049

Incidence/Reflection Angle (deg):                  31.95

Slant Range (km):                                   7352

Slew Angle (deg):                                 116.13

Doppler (carrier, fd; Hz):                         18663

Doppler (echo, fr; Hz):                           -26784

Doppler Difference (fdd; Hz):                      45447

Earth-Mars Distance (m):                        2.485E11

Experiment Set Up

=================

This experiment was conducted using Madrid's DSS 63.  The support team at JPL included Danny Kahan, Gene Goltz, and Dick Simpson.  Jesse Velasco was in the NOPE area.

Danny Kahan monitored the LCP channels.  Gene Goltz monitored all four channels, a training exercise in preparation for the experiment on 6 April when Kahan will not be available.  this aspect of the experiment appeared to be successful.

The MEX MOC briefing message included instructions for both uplink commanding and downlink telemetry.  Goltz confirmed with the NOPE and DSS 63 that neither was required; MEX MOC concurred after review.

For the first time in approximately a year, the X-RCP FGAIN was set to 60 (reduced from 68, which had been needed at smaller Earth-Mars distances).

Performance Problems and Notes

==============================

During the pre-cal, X-LCP levels increased by about 1 dB after the noise diode was first turned on; X-LCP remained very stable thereafter.  Similarly, S-LCP levels appeared to be about 1 dB low after use of the noise diode in Mini-Cal #2, then about 1 dB higher when the antenna reached zenith.  These are puzzling, but probably minor, concerns.  The real-time observations were confirmed in the quick-look results; redundancy in the calibration procedure allowed the questionable results to be discounted.

A weak echo was observed in real-time only in X-LCP.  It lasted 4-5 minutes (centered on approximately 18:00), drifted from about -2500 Hz to about +2500 Hz in the 25 kHz window, and had a bandwidth of about 1500 Hz.  In the X-LCP 100 kHz window, the residual carrier was weak but visible throughout; briefly the carrier and echo were visible symmetrically about the center frequency at about +/-22 kHz.

If there was interference (from MGS or other sources), it was not apparent in real time.  During processing, an interfering signal was seen in X-RCP during 18:05-18:25 and in the central 5 kHz of the bandwidth.

Screen dumps of spectra, including sky frequency at both S- and X-Band, were taken during during the BCAL (Earth-point with no modulation) periods and also at about 18:00.  The sky frequency data will be used to develop an algorithm for predicting sky frequencies that can be used during the Arecibo support during the 8 April experiment.

Data Acquisition

================

RSRs were configured as in Table 1:

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    Kahan (Goltz shadow)

RSR1B  X-LCP   1-W auto   60    Kahan (Goltz shadow)

RSR2A  S-RCP   1-W auto   60    Goltz

RSR2B  X-RCP   1-W auto   60    Goltz

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Step/  RSR1A  RSR1B  RSR2A  RSR2B

Time  Action  S-LCP  X-LCP  S-RCP  X-RCP  Ancillary Information

----- -------- -----  -----  -----  -----  ----------------------------------

FRO      0      0      0      0   RSR frequency offset (Hz)

********************************** PRE-CAL **********************************

14:23     01    -9.9  -10.0                DSS 63 is ready (ADC amp, dB)

10.5   19.5    9.0   17.5  ATT settings after AUTO reset

14:25     02   -10.0   -0.3  -10.0   -0.1  X-Band to AMB (ADC amp, dB)

14:27     03    -0.6   -0.3  -10.0   -0.1  S-LCP to AMB (ADC amp, dB)

14:29     04    -0.6   -0.3   -0.5   -0.1  S-RCP to AMB (ADC amp, dB)

14:30     05    -9.8  -10.2   -9.6  -10.1  ATT AUTO reset (ADC amp, dB)

21.5   31.5   20.5   20.5  ATT settings after reset

Ambient load physical temps:

S1 = 18.00C

S2 = 20.62C

X1 = 17.19C

Weather conditions:

T = 17C

H = 45 percent

P = 934 mbar

sky = partly cloudy

14:37           -9.7  -10.1   -9.6  -10.1  ADC amp (dB)

14:40     06    -9.8  -10.1   -9.5   -9.1  X-RCP ND ON

14:45     07    -9.7  -22.4   -9.6  -15.1  X-Band to sky

14:50     08    -9.8  -22.5   -9.7  -22.3  X-RCP ND OFF

14:55     09    -9.7  -19.2   -9.7  -22.3  X-LCP ND ON; 12.5 ND confirmed

15:00     10    -9.8   -8.8   -9.9  -10.0  X-Band to AMB

Ambient load physical temps:

S1 = 18.19C

S2 = 21.06C

X1 = 17.25C

15:05     11    -9.9   -9.0   -9.7  -10.2  X-LCP ND OFF

15:10     12    -9.8  -21.2   -9.6  -22.3  X-Band to sky

15:15     13    -9.9  -21.4   -9.4  -22.3  S-RCP ND ON

15:20     14    -9.9  -21.2  -17.4  -22.3  S-RCP to sky

15:25     15    -9.8  -21.3  -21.4  -22.3  S-RCP ND OFF; 12.5 ND confirmed

15:30     16    -9.4  -21.3  -21.2  -22.3  S-LCP ND ON

15:35     17   -17.0  -21.1  -21.5  -22.3  S-LCP to sky

15:40     18   -21.1  -21.2  -21.5  -22.3  S-LCP ND OFF

Ambient load physical temps:

S1 = 18.56C

S2 = 21.62C

X1 = 17.44C

25 kHz recordings continue

*************************** BOT, BCAL #1, SLEW #1 ***************************

15:45                                      DSS 63 moving to point

15:50                                      DSS 63 on point

16:05          -21.0  -21.2  -21.6  -21.5  BOT (no signal)

16:33          -20.9  -21.2  -21.6  -21.4  AOS; DSS 63 in lock on carrier

45     67   carrier to noise ratio (dB/Hz)

Sky frequencies:

2296368507.49 at 16:33:05

8420017928.61 at 16:33:07

16:45          -7000         -7000         FRO_S (Hz)

-26000        -26000  FRO_X SCHAN 4 only (Hz)

5848  -2258   5844         Carrier offset in 25 kHz windows

23700  Carrier offset in 100 kHz windows

17:00          -21.0  -21.2  -21.5  -21.4  Begin SLEW #1 (ADC amp, dB)

17:04                                 31   carrier-to-noise ratio (dB/Hz)

-2500  Carrier offset in SCHAN 3 (Hz)

17:05                -46000        -46000  FRO_X SCHAN 3 only (Hz)

17:06                                      DSS 63 reports LOS

********************************_MINI-CAL #1 ********************************

17:10     01   -20.9  -21.3  -21.6  -14.9  XR ND ON

17:13     02   -21.0  -21.2  -17.5  -21.5  XR ND OFF, SR ND ON

17:16     03   -21.1  -17.8  -21.6  -21.4  SR ND OFF, XL ND ON

17:19     04   -16.8  -21.1  -21.6  -21.4  XL ND OFF, SL ND ON

17:22     05   -20.9  -21.2  -21.8  -21.6  SL ND OFF

************************************ BSR ************************************

17:29                                      End SLEW #1

17:35          -21.0  -21.3  -21.4

17:45                                      Begin 100 kHz recordings

17:57                 22871                Carrier offset in 100 kHz window

16                 CNR (dB/Hz)

18:00                 -2500                Echo offset in 25 kHz window (Hz)

15                 Peak echo SNR (dB/Hz)

1500                Approximate echo bandwidth (Hz)

18:02                                      Echo, carrier visible in 100 kHz

at +/- 22 kHz (X-LCP only)

18:03                     0                Echo offset in 25 kHz window (Hz)

18:04                                      Echo gone (carrier remains)

18:15                                      End 100 kHz recordings

*********************** SLEW #2, MINICAL #2, BCAL #2 ************************

18:30          -21.0  -21.1  -21.4  -21.1  Begin SLEW #2 back to Earth point

18:35     01   -21.6  -21.0  -21.4  -14.8  X-RCP ND ON

18:38     02   -21.0  -21.1  -17.3  -21.3  X-RCP ND OFF, S-RCP ND ON

18:41     03   -21.5  -17.8  -21.3  -21.1  S-RCP ND OFF, X-LCP ND ON

18:44     04   -17.9  -21.1  -21.4  -21.1  X-LCP ND OFF, S-LCP ND ON

18:47     05   -22.4  -21.1  -21.4  -20.9  S-LCP ND OFF

18:55             0      0      0      0   All FRO's reset to 0

19:00                        -1422  -3215  Carrier offset (Hz)

45     67   CNR (dB/Hz)

Sky Frequencies:

2296353728.17 at 18:59:53

8419963707.33 at 18:59:53

********************************* POST-CAL *********************************

19:10          -21.0  -21.2  -21.5  -22.4  DSS 63 at stow

19:21     01    -9.9   -9.0   -9.6  -10.2  All to AMB

Ambient load physical temps:

S1 = 18.56C

S2 = 21.12C

X1 = 17.38C

19:25     02    -9.8   -9.0   -9.5   -9.2  X-RCP ND ON

19:28     03    -9.9   -9.0   -9.3  -10.1  X-RCP ND OFF, S-RCP ND ON

19:31     04    -9.9   -8.6   -9.5  -10.1  S-RCP ND OFF, X-LCP ND ON

19:34     05    -9.4   -9.0   -9.7  -10.1  X-LCP ND OFF, S-LCP ND ON

19:37     06   -17.2  -21.4  -21.3  -22.5  All to sky

19:40     07   -21.1  -18.0  -21.3  -22.4  S-LCP ND OFF, X-LCP ND ON

19:43     08   -21.0  -21.4  -17.3  -22.4  X-LCP ND OFF, S-RCP ND ON

19:46     09   -21.0  -21.4  -21.3  -15.2  S-RCP ND OFF, X-RCP ND ON

19:49     10   -21.1  -21.3  -21.2  -22.4  X-RCP ND OFF

19:55                                      Stop 25 kHz recordings

Ambient load physical temps

S1 = 18.31C

S2 = 21.00C

X1 = 17.12C

Weather at DSS 63:

T = 15C

H = 38 percent

P = 934 mb

sky = dark (probably clear)

End of activity

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2006-04-02

Notes added from processing: 2006-04-06

Minor edits: 2006-04-12

Updated Summary: 2006-07-05