Mars Express Bistatic Radar Experiment

Operations Plan and Report

07 April 2006

Summary

=======

DSN Antenna:                                          63

Pass:                                               1042

Orbit:                                              2875

Orbit Start Time:                    2006-04-07T13:51:22

HGA Pointing:                                   Inertial

Specular Condition (ERT):                          19:06

Target Latitude (deg N):                           -1.26

Longitude (deg E):                        -127.04

Rp (km):                                 3400.028

Incidence/Reflection Angle (deg):                  33.38

Slant Range (km):                                   7865

Slew Angle (deg):                                 113.24

Doppler (carrier, fd; Hz):                         16572

Doppler (echo, fr; Hz):                           -24258

Doppler Difference (fdd; Hz):                      40829

Earth-Mars Distance (m):                       2.556E+11

Experiment Set Up

=================

This experiment was conducted using Madrid's DSS 63.  The RSSG support team was Gene Goltz (alone), who operated all four RSR's; Dick Simpson was available by phone at Stanford.  The following report is based largely on Gene's notes.

Performance Problems and Notes

==============================

There were two problems encountered during the pre-cal procedure:

1.  when Step #2 was performed, at ~15:48, all of the ADC values went up

to just below zero (e.g., -0.n) -- only the two X-band values should

have changed in that way -- as if Steps #2, #3, and #4 were all done

at the same time; Jesse checked the station's OCIs, and found that a

zero had been entered in place of the letter "O" in one of the input

directives; we restarted the pre-cal procedure from Step #1 at 15:58

2.  there was an unexpected increase in the X-band LCP ADC value at Step

#8, but another increase was seen (as expected) when Step #8 was ex-

cuted; Jesse found no errors in the station OCIs, and we did not try

to repeat any of the earlier steps. [Simpson, later - a check of the

NMC log showed that DC01 was configured to control the X-LCP noise

diode at 16:25 (Step-08) and it came on at a low level despite the

fact that no enabling command was issued.  The enable command was

issued at 16:30 (Step-09) at which time the noise power rose to its

expected level.]

Otherwise, everything went well -- but, as expected, any signal was very

difficult to discern in the FFTs.

Notes added by Simpson after processing:

S-RCP: The maser was unstable until about the beginning of MiniCal #1

(18:15); there were reductions in sample power of up to 0.3 dB lasting

a few seconds and occurring at intervals of 1-10 minutes.  By carefully

choosing the time intervals over which calibration data were taken, we

seem to have avoided any problems here.  There were no maser problems

during or after MiniCal #1, so the science should be fine as long as the

calibrations hold up.

S-LCP: The gain on this channel became unstable after MiniCal #1 (18:30),

remaining unstable until completion of the surface observations (19:35).

Sample power dropped 2 dB over 18:30-18:40. There was about a 50 percent

recovery by 18:44; the remaining recovery was in stages.  During the

BSR_CAL stage of processing we applied piecewise continuous gain

corrections (about 10 segments); the results appear satisfactory with

the possible exception of brief periods during 18:30-18:44 when the

sample levels were changing most quickly.  No echo has been detected in

S-LCP; if one is found (probably during 19:01-19:11), the gain

adjustments should be reviewed.  Also, the assumption that this was only

a gain change (and not also a Tsys change) should be reconsidered.

Data Acquisition

================

RSRs were configured as in Table 1:

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    Goltz

RSR1B  X-LCP   1-W auto   60    Goltz

RSR2A  S-RCP   1-W auto   60    Goltz

RSR2B  X-RCP   1-W auto   60    Goltz

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

-------- ----- --------- -----  -----  -----  -----  ------------------------

Set-Up          channel    1A     1B     2A     2B

fgain     60     60     60     60

Pre-Cal  15:45     1      -4.7   -1.7   -8.7   -9.8

15:46  att auto -10.3  -10.0   -9.6   -9.8

15:48     2      -0.3   -0.2   -0.1   -0.1  Configuration problem

15:58     1     -10.3  -10.0   -9.6   -9.8  Restart pre-cal

16:00     2     -10.2   -0.2   -9.6   -0.1

16:02     3      -1.0   -0.3   -9.7   -0.1

16:04     4      -1.0   -0.2   -0.3   -0.1  Ambient load temps:

S1=17.88

S2=20.25

X1=15.12"

16:05  att auto  21.5   31.5   20.5   24.5  ATT settings

Weather conditions:

T=14.89C

H=51.06 percent

P=926.82 mB"

sky=partly cloudy

16:07   rec 3 e  -9.9   -9.0   -9.9   -9.6  Begin 25 kHz recording

16:10     5     -10.0   -9.0   -9.7   -9.6

16:15     6      -9.9   -9.1  -10.0   -8.7

16:20     7      -9.9  -21.2   -9.9  -14.6

16:25     8      -9.9  -19.3   -9.8  -21.7  ? No error seen in

station OCIs ?

16:30     9     -10.0  -18.0  -10.0  -21.8  Confirmed 12.5K ND

16:35    10      -9.8   -8.7  -10.0   -9.6  Ambient load temps:

S1=18.12

S2=20.56

X1=15.12"

16:40    11      -9.8   -9.0  -10.0   -9.6

16:45    12      -9.8  -21.3  -10.1  -21.7

16:50    13      -9.8  -21.2   -9.5  -21.8

16:55    14      -9.8  -21.2  -17.6  -21.7

17:00    15      -9.8  -21.2  -21.7  -21.7  Confirmed 12.5K ND

17:05    16      -9.3  -21.2  -21.4  -21.8

17:10    17     -17.0  -21.3  -21.6  -21.8

17:15    18     -20.8  -21.3  -21.7  -21.8  Ambient load temps:

S1=18.10

S2=21.00

X1=15.10"

17:20   end                                 Moved antenna to

spacecraft point

17:31  acq. S                               Occultation egress

17:33  acq. X                               X TX turned on

17:35   BOT     -20.9  -21.1  -21.9  -20.4

17:50 sfro 4/SX  -7k   -23k    -7k"   -23k  Rcvr offsets (hz)

sfro 3/S   -7k     0     -7k      0   Rcvr offsets (hz)

18:10 sfro 3/X    0    -44k     0     -44k  Rcvr offsets (hz)

MiniCal1 18:15     1     -20.7  -21.3  -21.6  -14.2  Confirmed 12.5K ND

18:18     2     -20.8  -21.1  -17.7  -20.1

18:21     3     -20.9  -17.8  -21.5  -20.1

18:24     4     -16.9  -21.2  -21.5  -20.1

18:27     5     -20.8  -20.9  -21.5  -20.2

18:30   end

BSR      18:51  rec 4 e  -21.6  -20.9  -21.6  -20.0  Begin 100 kHz recording

19:06 max. sig.                             Peak Px/N0 (X-LCP=25.1)

19:21  rec 4 d  -21.0  -21.0  -21.3  -19.7  Stop 100 kHz recording

MiniCal2 19:40     1     -20.7  -20.8  -21.4  -14.1  Confirmed 12.5K ND

19:43     2     -20.6  -20.9  -17.4  -19.8

19:46     3     -20.7  -17.7  -21.3  -19.6

19:49     4     -16.8  -20.9  -21.4  -19.6

19:52     5     -20.8  -20.9  -21.3  -19.5

19:55  end

Post-Cal 20:00 EOT/stow                              Ambient load temps:

S1=17.62

S2=19.88

X1=14.94

20:06     1      -9.8   -9.0   -9.8   -9.5

20:10     2      -9.8   -9.0   -9.8   -8.6

20:13     3      -9.7   -9.0   -9.2   -9.6

20:16     4      -9.8   -8.8   -9.6   -9.6

20:19     5      -9.3   -9.0   -9.6   -9.6

20:22     6     -16.6  -21.1  -21.2  -21.8

20:25     7     -20.9  -17.9  -21.1  -21.6

20:28     8     -20.7  -21.2  -17.3  -21.7

20:31     9     -20.7  -21.2  -21.1  -14.5

20:34    10     -20.8  -21.2  -21.0  -21.6  Ambient load temps

S1=17.25

S2=19.56

X1=14.75

Weather conditions

T=11C

H=69 percent

P=928 mB

sky=partly cloudy

20:40  rec 3 d                              Stop 25 kHz recording

end

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LNA = low-noise amplifier

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2006-04-22

Updated Summary; added LNA to acronym list: 2006-07-05