Mars Express Bistatic Radar Experiment

Operations Plan and Report

23 April 2006

Summary

=======

DSN Antenna:                                          43

Pass:                                               1058

Orbit:                                              2930

Orbit Start Time:

HGA Pointing:                                   Inertial

Specular Condition (ERT):                          04:54

Target Latitude (deg N):                           -2.01

Longitude (deg E):                        -130.41

Rp (km):                                 3394.000

Incidence/Reflection Angle (deg):                  38.74

Slant Range (km):                                   8672

Slew Angle (deg):                                 102.40

Doppler (carrier, fd; Hz):                         13792

Doppler (echo, fr; Hz):                           -17583

Doppler Difference (fdd; Hz):                      31375

Earth-Mars Distance (m):                       2.765E+11

Experiment Set Up

=================

This experiment was conducted using Canberra's DSS 43.  Gene Goltz and Danny Kahan were in the Radio Science Support Area at JPL; Jesse Velasco was in the NOPE area, at least at the beginning.  Dick Simpson was in occasional contact via phone from Donner Summit.  The following report is based largely on notes provided by Goltz.

Performance Problems and Notes

==============================

There had been serious configuration problems during the BSR operation two days earlier (2006/111) at DSS 43.  By day 113, these had been mostly resolved except that S-RCP and S-LCP had been swapped as inputs to the RSR's.  It was not clear how this had happened; this had not been one of the issues on day 111.  But it was deemed unimportant so far as performance of the system was concerned and the operation went forward without any attempt to fix it.

Data Acquisition

================

RSRs were configured as in Table 1 (note that S-RCP and S-LCP are swapped compared with the usual assignments):

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-RCP   1-W auto   60    Goltz

RSR1B  X-LCP   1-W auto   60    Goltz

RSR2A  S-LCP   1-W auto   60    Goltz

RSR2B  X-RCP   1-W auto   60    Goltz

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          1 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-RCP  X-LCP  S-LCP  X-RCP  Notes / Comments

-------- ----- --------- -----  -----  -----  -----  ------------------------

Pre-Cal            1

01:54  att auto -10.0  -10.2  -10.0   -9.9  All to sky

01:46     2     -10.0   -0.3  -10.0   -0.0  X-Band to AMB

01:57     3     -10.0   -0.3   -0.5   -0.1  S-LCP to AMB

02:06     4      -0.3   -0.3   -0.5   -0.1  S-RCP to AMB

AMB physical temps:

S1=19.00,

S2=17.44,

X1=17.25

att auto  13.5   25.5   20.5   25.0  Reset ATT (prop to dB)

-9.8  -10.0   -9.9   -9.8  Weather confitions:

T=13C,

H=43 percent

sky=clear

02:10  rec 3 e                              Begin 25 kHz recording

02:10     5      -9.8  -10.0   -9.9   -9.8  Configure DTT's

02:15     6      -9.8  -10.0   -9.9   -9.0  XR ND ON

02:20     7      -9.7  -22.2   -9.9  -15.7  X-Band to sky

02:25     8      -9.7  -22.2  -10.0  -22.3  XR ND OFF

02:30     9      -9.8  -17.1   -9.9  -22.2  XL ND ON

Confirmed 12.5K ND

02:35    10      -9.9   -9.4  -10.0   -9.8  X-Band to AMB

AMB physical temps:

S1=18.94,

S2=17.88,

X1=17.50

02:40    11      -9.8  -10.1   -9.9   -9.9  XL ND OFF

02:45    12      -9.8  -22.2   -9.8  -22.2  X-Band to sky

02:50    13      -9.6  -22.1  -10.0  -22.3  SR ND ON

02:55    14     -19.2  -22.0   -9.9  -22.3  SR to sky

03:00    15     -21.7  -22.2  -10.0  -22.2  SR ND OFF

Confirmed 12.5K ND

03:05    16     -21.8  -22.0   -9.6  -22.1  SL ND ON

03:10    17     -21.7  -22.1  -18.6  -22.3  SL to sky

02:15    18     -21.7  -22.2  -21.3  -22.2  SL ND OFF

AMB physical temps:

S1=18.81,

S2=18.56,

X1=17.94

03:20   end                                 Move antenna to point

03:26   BOT     -20.2  -20.5  -19.8  -19.3

03:50 sfro 4/SX -5300 -19000  -5300 -19000  Rcvr offsets, SCHAN4(Hz)

sfro 3/S  -5300      0  -5300      0  Rcvr offsets, SCHAN3(Hz)

04:00 sfro 3/X      0 -34000      0 -34000  Rcvr offsets, SCHAn3(Hz)

MiniCal1 04:05     1     -20.6  -21.2  -20.4  -15.0  XR ND ON

04:08     2     -19.9  -21.2  -20.3  -19.9  XR ND OFF, SR ND ON

04:11     3     -20.8  -17.0  -20.4  -20.0  SR ND OFF, XL ND ON

04:14     4     -20.2  -21.2  -18.0  -20.0  XL ND OFF, SL ND ON

04:17     5     -20.8  -21.2  -20.3  -20.1  SL ND OFF

BSR      04:39  rec 4 e  -21.0  -21.4  -20.4  -20.1  Start 100 kHz recording

04:54                                       Maximum echoes

05:09  rec 4 d  -21.1  -21.6  -20.6  -20.2  Stop 100 kHz recording

MiniCal2 05:25     1     -21.0  -21.6  -20.5  -15.2  XR ND ON

05:28     2     -19.1  -21.5  -20.7  -20.3  XR ND OFF, SR ND ON

05:31     3     -21.2  -17.1  -20.6  -20.3  SR ND OFF, XL ND ON

05:34     4     -21.2  -21.6  -18.3  -20.4  XL ND OFF, SL ND ON

05:37     5     -21.2  -21.5  -20.6  -20.3  SL ND OFF

PostCal  06:05 EOT/stow  -21.7  -22.3  -21.3  -22.4

06:10     1      -9.8  -10.1   -9.9   -9.9  All to AMB

AMB phys temps:

S1=20.52

S2=21.00

X1=18.31

06:15     2      -9.8  -10.2   -9.8   -9.0  XR ND ON

06:18     3      -9.5  -10.1   -9.8   -9.9  XR ND OFF, SR ND ON

06:21     4      -9.8   -9.4   -9.8   -9.9  SR ND OFF, XL ND ON

06:24     5      -9.7  -10.0   -9.4   -9.9  XL ND OFF, SL ND ON

06:27     6     -21.7  -22.1  -18.5  -22.2  All to sky

06:30     7     -21.7  -17.1  -21.2  -22.3  SL ND OFF, XL ND ON

06:33     8     -19.2  -22.1  -21.2  -22.2  XL ND OFF, SR ND ON

06:36     9     -21.7  -22.1  -21.2  -15.8  SR ND OFF, XR ND ON

06:39    10     -21.8  -22.1  -21.3  -22.2  XR ND OFF

AMB physical temps:

S1=20.75,

S2=21.12,

X1=18.69

Weather conditions:

T=15C,

H=38 percent

sky=clear

06:45 rec 3 d                               Stop 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2006-05-02