Mars Express Bistatic Radar Experiment

Operations Plan and Report

29 February 2008

Summary

=======

DSN Antenna:                                                     43

Pass:                                                          1735

Orbit:                                                         5343

Orbit Start Time:                               2008-02-29T04:12:23

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):            10:43:00   11:06:00   11:30:00

Target Latitude (deg N):               -42.74     -38.02     -32.64

Longitude (deg E):              177.49     167.12     157.28

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       58.87      59.58      60.33

Slant Range (km):                      11407.     11628.     11585.

Slew Angle (deg):                       62.27      60.83      59.35

Doppler (carrier, fd; Hz):              -9267      -6782      -3991

Doppler (echo, fr; Hz):                 -7871      -2618       2802

Doppler Difference (fdd; Hz):           -1396      -4164      -6793

Earth-Mars Distance (m):            1.539E+11  1.539E+11  1.539E+11

Experiment Set Up

=================

This experiment was conducted using Canberra's DSS 43.  Gene Goltz, Danny Kahan, and Dick Simpson were in the Radio Science Support Area at JPL; Kahan operated the RSR's while Goltz handled communications.  Jesse Velasco was in the NOPE area.

Performance Problems and Notes

==============================

As DSS 43 was moving to point, the ADC amplitude on X-RCP rose about 3.5 dB.  As the spacecraft slew began, the ADC amplitude returned to its previous level.  The same behavior has been observed during other recent experiments at both DSS 43 and DSS 63.  Only the X-RCP channel seemed to be affected.  The cause is unknown.

We asked the NOPE at 10:35 whether it is difficult to change from spacecraft pointing to planetary pointing; he thought it could be done in 5 minutes.  He asked DSS 43 to make the change at 10:38; they reported they were using Mars-centered pointing at 10:45.  The echoes looked good, but there was no obvious change in the ADC amplitudes.  This is the more appropriate pointing technique for bistatic radar even if it didn't resolve the X-RCP level question.

X-Band echoes were visible at about 30 dB/Hz  and 2 kHz width in both polarizations throughout the observing period.  S-Band echoes could be seen with 40 seconds of integration in both polarizations, but at only 1-2 dB above the noise level.

No receiver tuning offsets (FROs) were used; the echo started close to the carrier and moved about 5 kHz toward higher frequencies during the observation.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    Kahan

RSR1B  X-LCP   1-W auto   60    Kahan

RSR2A  S-RCP   1-W auto   60    Kahan

RSR2B  X-RCP   1-W auto   65    Kahan

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   backup (not recorded)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-RCP  X-RCP  Notes / Comments

RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

2008/060

--------

Pre-Cal  07:40     1      -9.9   -9.9   -9.8   -9.7  All to sky (dB)

14.0   13.5   12.5   13.0  ATT settings

07:42     2      -9.7   -0.1   -9.8    0.1  X-Band to AMB

07:43     3      -0.7   -0.1   -9.7    0.0  S-LCP to AMB

07:44     4      -0.8    0.0   -0.7    0.1  S-RCP to AMB

AMB load physical temps:

S1=19.81

S2=21.81

X1=19.19

Weather conditions:

T=16.3C

H=36.5 percent

sky = clear

07:46            -9.6   -9.8   -9.6   -9.8  ADC amp after AUTO reset

24.5   28.0   23.0   25.5  ATT settings

07:46:01  5                                 Begin 25 kHz recording

07:50     6      -9.7   -9.9   -9.7   -9.0  X-RCP ND ON

07:55     7      -9.7  -22.3   -9.6  -15.9  X-Band to sky

08:00     8      -9.8  -22.3   -9.7  -22.4  X-RCP ND OFF

08:05     9      -9.5  -17.1   -9.6  -22.3  XL ND ON

Confirmed ND = 12.5K

08:10    10      -9.6   -9.2   -9.6   -9.9  X-Band to AMB

AMB physical temps:

S1=20.12

S2=21.81

X1=19.44

08:15    11      -9.6   -9.9   -9.6   -9.9  X-LCP ND OFF

08:20    12      -9.5  -22.3   -9.6  -22.3  X-Band to sky

08:25    13      -9.6  -22.2   -9.5  -22.3  S-RCP ND ON

08:30    14      -9.4  -22.3  -19.2  -22.4  S-RCP to sky

08:35    15      -9.6  -22.3  -21.8  -22.4  S-RCP ND OFF

Confirmed ND=12.5K

08:40    16      -9.4  -22.4  -21.6  -22.4  S-LCP ND ON

08:45    17     -18.1  -22.3  -21.6  -22.4  S-LCP to sky

08:50    18     -20.2  -22.2  -21.6  -22.3  S-LCP ND OFF

AMB physical temps:

S1=20.25

S2=21.69

X1=19.88

08:55                                       Stop recording

DSS 43 moving to point

09:03           -19.8  -21.2  -20.9  -18.8  All ADC levels rise

09:05                                       DSS 43 on point

09:29   AOS             25.2          65.1  X-Band visible (dB/Hz)

-2470         -2468  Carrier offset (Hz)

09:32   AOS      24.5          49.9         S-Band visible (dB/Hz)

-1218         -1218         Carrier offset (Hz)

09:33                                       DSS 43 in lock

09:48            23.6   33.2   49.9   72.1  X ranging off (dB/Hz)

-1281  -2694  Carrier offset (Hz)

-19.7  -21.2  -20.9  -18.5  ADC amplitude (dB)

09:50:00                                    Resume 25 kHz recording

Slew 1   10:14                                       Begin slew

10:17            23.3   38.5   46.5   45.8  CNR (dB/Hz)

-19.7  -21.2  -20.8  -21.4  ADC amplitude (dB)

MiniCal1 10:20     1     -19.7  -21.3  -20.9  -15.5  XR ND ON; conf ND=12.5K

10:23     2     -19.7  -21.4  -18.8  -21.2  XR ND OFF; SR ND ON

S-Band carrier gone

XR carrier 39 dB/Hz

10:26     3     -19.7  -16.8  -20.9  -18.8  SR ND OFF; XL ND ON

10:29     4     -17.8  -21.3  -20.8  -18.8  XL ND OFF; SL ND ON

10:32     5     -19.7  -21.2  -20.8  -20.6  SL ND OFF

BSR:     10:41                   30.0          30.0  Echo visible (dB/Hz)

10:42                                       Begin 100 kHz recording

End Slew 1

10:45                                       DSS43 on planetary

pointing predicts

11:02           -19.6  -21.1  -20.8  -18.8  ADC amplitude (dB)

11:30           -19.6  -21.0  -20.5  -18.9  ADC amplitude (dB)

End 100 kHz recording

Slew 2   11:30                                       Begin Slew 2

MiniCal2 11:35     1     -19.5  -20.9  -20.5  -14.8  XR ND ON

11:38     2     -19.5  -20.8  -18.6  -18.8  XR ND OFF; SR ND ON

11:41     3     -19.4  -16.6  -20.5  -18.9  SR ND OFF; XL ND ON

11:44     4     -17.6  -20.9  -20.4  -18.9  XL ND OFF; SL ND ON

11:47     5     -19.4  -20.8  -20.5  -18.6  SL ND OFF

11:51:25                                    DSS 43 returned to

spacecraft pointing

12:00           -19.2  -20.6  -20.3  -18.2  ADC amplitude (dB)

22.2   40.4   49.2   71.2  CNR (dB/Hz)

End slew; DSS 43 in lock

12:14           -19.2  -20.6  -20.3  -20.1  ADC amplitude (dB)

20.8   40.0   49.0   70.4  CNR (dB/Hz)

12:15                                       Early EOT

PostCal: 12:23           -20.2  -22.4  -21.6  -22.4  DSS 43 at zenith

12:25     1      -9.5   -9.8   -9.7   -9.9  All to AMB

AMB phys temps:

S1=19.31

S2=20.12

X1=19.31

12:28     2      -9.5   -9.8   -9.9   -9.0  XR ND ON

12:31     3      -9.5   -9.7   -9.7   -9.8  XR ND OFF; SR ND ON

12:34     4      -9.6   -9.4   -9.7   -9.8  SR ND OFF; XL ND ON

12:37     5      -9.4   -9.6   -9.7   -9.8  XL ND OFF; SL ND ON

12:40     6     -18.1  -22.2  -21.5  -22.4  All to sky

12:43     7     -20.2  -17.0  -21.6  -22.4  SL ND OFF; XL ND ON

12:46     8     -20.2  -22.3  -19.1  -22.5  XL ND OFF; SR ND ON

12:49     9     -20.3  -22.2  -21.6  -15.7  SR ND OFF; XR ND ON

12:52    10     -20.1  -22.4  -21.7  -22.4  XR ND OFF

AMB physical temps:

S1=19.44

S2=20.00

X1=19.31

Weather conditions:

T=10.1C

H=51.9 percent

sky=clear

EOA      12:55                                       Stop 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2008-02-29

Added orbit start time: 2008-03-05