Mars Express Bistatic Radar Experiment

Operations Plan and Report

25 April 2010

Summary

=======

DSN Antenna:                                                     43

Pass:                                                           N/A

Orbit:                                                         8085

Orbit Start Time:                              2010-04-25T07:40:38

HGA Pointing:                                              Specular

Start    Mid Point      End

--------  ----------  --------

Specular Condition (ERT):               09:15      09:45      10:12

Target Latitude (deg N):               -19.81     -27.55     -37.47

Longitude (deg E):              232.61     227.39     224.79

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       38.10      45.28      54.74

Slant Range (km):                       9397.      8118.      6600.

Slew Angle (deg):                      103.79     89.44      70.51

Doppler (carrier, fd; Hz):            -37906.    -42931.    -44808.

Doppler (echo, fr; Hz):                13077.     17757.     19153.

Doppler Difference (fdd; Hz):         -50983.    -60687.    -63960.

Earth-Mars Distance (m):            1.848E+11  1.848E+11  1.848E+11

NB: The DSN is no longer routinely using pass numbers; so this entry in the

summary will not be maintained in this and future reports.

Experiment Set Up

=================

This experiment was conducted using the Canberra DSS 43, the first experiment at that complex since 2008/060.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL.  This report is largely based on notes provided by Kahan and Goltz.

Performance Problems and Notes

==============================

There was a problem configuring the S-Band RCP and LCP during the mini-cals; there is no MINICAL #1 on S-LCP.  But this was fixed at step 4 of the second mini-cal, and everything went smoothly after that.

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR1A  S-LCP   1-W auto   60    UNK

RSR1B  X-LCP   1-W auto   60    UNK

RSR2A  S-RCP   1-W auto   60    UNK

RSR2B  X-RCP   1-W auto   60    UNK

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Occultation bandwidth (not recorded for BSR)

2          8 ksps   Occultation backup (not recorded for BSR)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-LCP  X-LCP  S-LCP  X-RCP  Notes / Comments

2010/115                 RSR1A  RSR1B  RSR2A  RSR2B

-------- ----- --------- -----  -----  -----  -----  ------------------------

FGAIN    06:10              60     60     60     65  Set FGAIN

Pre-Cal  06:51      1     -7.9  -14.2  -15.5  -13.0  ADC amplitude (dB)

06:52  att auto  -9.7   -9.9  -10.0   -9.7

19.5   15.5    9.5   12.5  Attenuator settings (dB)

06:53      2    -10.0   -0.1  -10.0   -0.0

06:55      3     -1.6   -0.1  -10.0   -0.1

06:56   "4,5"    -1.6   -0.1   -0.5   -0.1

06:59  att auto  -9.9   -9.8   -9.6   -9.8  AMB load phys temps:

S1=20.81

S2=21.25

X1=19.06

29.0   28.0   20.5   25.0  Attenuator settings (dB)

Local weather:

T=15.6C

H=46.7 percent

sky=cloudy

07:00  rec 3 e   -9.9   -9.9   -9.6   -9.8  Begin 25 kHz recording

07:01      6    -10.0   -9.8   -9.6   -9.8

07:08      7     -9.9   -9.9   -9.6   -9.1

07:13      8     -9.9  -22.2   -9.6  -15.9

07:18      9     -9.9  -22.2   -9.6  -22.3

07:23     10     -9.9  -22.2   -9.6  -22.3  Confirmed 12.5K ND

07:28  "11,12"   -9.9   -9.4   -9.5   -9.8  AMB load phys temps:

S1=20.75

S2=21.31

X1=19.00

07:33     13    -10.0   -9.8   -9.5  -10.0

07:38     14    -10.1  -22.2   -9.6  -22.3

07:43     15    -10.1  -22.2   -9.3  -22.4

07:48     16    -10.0  -22.2  -18.8  -22.4

07:53     17    -10.0  -22.2  -21.0  -22.5  Confirmed 12.5K ND

08:04     18     -9.7  -22.2  -21.0  -22.3

08:09     19    -17.9  -22.2  -21.0  -22.4

08:14     20    -19.2  -22.2  -21.0  -22.4  AMB load phys temps:

S1=20.56

S2=21.12

X1=18.94

08:19  21/end   -19.3  -22.2  -21.0  -22.4  Stop 25 kHz recording

BOT      08:30           -19.1  -21.5  -20.7  -19.4

08:45  rec 3 e  -19.2  -21.5  -20.9  -19.5  Resume 25 kHz recording

08:54           -19.2  -21.5  -20.9  -19.5  DSS 43 switch to Mars

pointing predicts

08:57 sfro 3/SX   +9K   +45K    +9K   +45K  Rcvr tuning offset (Hz)

sfro 4/X         +35K          +35K  Rcvr tuning offset (Hz)

MiniCal1 08:58      1    -19.2  -21.5  -20.7  -15.2  Confirmed 12.5K ND

09:01      2    -19.1  -21.5  -18.6  -19.6

09:04      3    -19.1  -16.7  -20.8  -19.6

09:11      4       *      *      *      *   Station experiencing

config. problems

09:15   5/end      *      *      *      *   Out of time / Mini-Cal

aborted

BSR      09:15 rec 4 e   -19.2  -21.5  -20.7  -21.8  Begin 100 kHz recording

09:30 sfro 3/X          +50K          +50K  Rcvr tuning offset (Hz)

09:46 sfro 3/X          +55K          +55K

end BSR  10:12 rec 4 d   -19.1  -21.4  -20.6  -19.7  End 100 kHz recording

MiniCal2 10:14      1    -19.1  -21.3  -20.7  -15.1  Confirm 12.5

10:17      2    -19.0  -21.5  -18.6  -19.7

10:20      3    -19.2  -16.8  -20.5  -19.7

10:31      4    -17.8  -21.5  -20.6  -19.6  S-band config. Problems

continue/fixed

10:34      5    -19.2  -21.2  -20.6  -19.2

10:37    end    -19.1  -21.2  -20.6  -19.2

10:37 sfro 3/SX     0      0      0      0  Rcvr tuning offsets (Hz)

DSS 43 to s/c pointing

Predicts

EOT      10:43           -19.2  -21.4  -20.6  -21.7

10:43   stow    -19.4  -22.2  -21.0  -22.4

Post-Cal 10:50      1    -10.0  -10.1   -9.5   -9.8  AMB load phys temps:

S1=19.40

S2=20.31

X1=17.94

10:54      2    -10.0   -9.8   -9.6   -9.0

10:57      3     -9.9  -10.0   -9.4   -9.7

11:00      4    -10.0   -9.3   -9.7   -9.8

11:03      5     -9.7   -9.9   -9.6   -9.8

11:06      6    -17.9  -22.1  -21.0  -22.3

11:09      7    -19.3  -17.1  -21.0  -22.3

11:12      8    -19.3  -22.2  -19.0  -22.3

11:15      9    -19.3  -22.1  -21.0  -15.9

11:18     10    -19.2  -22.3  -21.1  -22.4  AMB load phys temps:

S1=19.44

S2=20.12

X1=17.88

Local weather:

T=10.1C

H=74.0 percent

sky=clear"

11:22  11/end   -19.2  -22.3  -21.1  -22.3

AMB = ambient load

BOT = Beginning of Track

BW = bandwidth

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOA = End of activity

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

NOP = Network Operations Plan

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

s/c = spacecraft

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Post Analysis Summary

None

Dick Simpson

Original: 2010-04-29

Removed PDS unfriendly characters: 2011-05-16