From JVelasco@jftl.jpl.nasa.gov Wed Aug 24 12:50:43 2005

Return-Path: <JVelasco@jftl.jpl.nasa.gov>

Received: from jftl.jpl.nasa.gov (jftl.jpl.nasa.gov [128.149.207.22])

by magellan.stanford.edu (8.11.7p1+Sun/8.11.7) with SMTP id j7OJoer01116

for <rsimpson@magellan.stanford.edu>; Wed, 24 Aug 2005 12:50:40 -0700 (PDT)

Received: from jftl.jpl.nasa.gov (Not Verified[128.149.207.21]) by jftl.jpl.nasa.gov with MailMarshal (v5,0,3,100)

id <B0002f237c>; Wed, 24 Aug 2005 12:51:40 -0700

Received: by jftl-mm.jpl.nasa.gov with Internet Mail Service (5.5.2657.72)

id <QPB3LBH7>; Wed, 24 Aug 2005 12:50:37 -0700

Message-ID: <12373B687CF8324F843BE0CA103AA6F40394D6A8@jftl-mm.jpl.nasa.gov>

From: "Velasco, Jesse" <JVelasco@jftl.jpl.nasa.gov>

To: DSN-MPSETD <DSN-MPSETD@jjpl.jpl.nasa.gov>,

"GDSCC Operations," <GDSCCOperations@gdscc.nasa.gov>,

"'gene Goltz'" <gene.goltz@jpl.nasa.gov>,

"'Asmar, Sami W'" <Sami.W.Asmar@jpl.nasa.gov>,

DSN-OPS-CHIEFS <DSN-OPS-CHIEFS@jjpl.jpl.nasa.gov>,

DSN-RMDCT  <DSN-RMDCT@jjpl.jpl.nasa.gov>,

DSNO&M-DSOT  <DSNO&M-DSOT@jftl.jpl.nasa.gov>,

DSN-NOCC <dsn-nocc@jjpl.jpl.nasa.gov>,

"'Hodder, James A'" <James.A.Hodder@jpl.nasa.gov>,

"Bracamonte, Larry H"  <LBracamonte@gdscc.nasa.gov>

Cc: DSN-PREDICTS <DSN-PREDICTS@jjpl.jpl.nasa.gov>,

MDSCC-Operations  <MDSCC-Operations@mdscc.nasa.gov>,

"Beyer, Patrick E"  <pbeyer@mail.jpl.nasa.gov>,

"Berman, Allen L"  <alberman@mail.jpl.nasa.gov>,

"Thompson, Thomas W"  <twt@ampersand.jpl.nasa.gov>,

"Landon, Arthur J"  <ALandon@jftl.jpl.nasa.gov>,

"Wallick, Jack"  <JWallick@jftl.jpl.nasa.gov>,

"Holmes, Dwight P"  <dholmes@mail.jpl.nasa.gov>,

"'Dick Simpson 650-723-3525'"  <rsimpson@magellan.stanford.edu>,

"'John Reynolds (John.Reynolds@esa.int)'" <John.Reynolds@esa.int>,

"'Michel.Denis-esa.int'" <Michel.Denis@esa.int>,

"'Olivier.Reboud@esa.int'" <Olivier.Reboud@esa.int>,

CDSCC  <CDSCC@cdscc.nasa.gov>,

"Berner, Jeff B" <jbberner@mail.jpl.nasa.gov>,

"Guerrero, Anamaria P" <ana@mail.jpl.nasa.gov>,

"Kurtik, Susan C"  <skurtik@mail.jpl.nasa.gov>,

"Marina, Miguel"  <mmarina@mail.jpl.nasa.gov>,

"Hodgin, Wendy K"  <whodgin@mail.jpl.nasa.gov>

Subject: BRIEFING MESSAGE FOR MEX BISTATIC RADAR EXPERIMENT WEEK34 REVISION 1

Date: Wed, 24 Aug 2005 12:50:21 -0700

MIME-Version: 1.0

X-Mailer: Internet Mail Service (5.5.2657.72)

Content-Type: multipart/mixed;

boundary="----_=_NextPart_000_01C5A8E5.0DE94160"

Content-Length: 365679

Status: RO

This message is in MIME format. Since your mail reader does not understand

this format, some or all of this message may not be legible.

...

Mars Express

Briefing Message

Level 3

Bistatic Radar Experiment Support

Prepared by: JESSE VELASCO / MEX / NOPE / MPSETD

Origination Date: DOY 235 / 15:00 UTC

Revision: 1, DOY 236/ 1900 UTC

DSS-14   DOY 238/ 1230 UTC

DSS-14   DOY 239/ 0900 UTC

ROUTING: JFTL / JJPL / JGTS

FROM: MPSETD J. Velasco

SUBJECT: DSS-14 Mars Express Bistatic Radar Support

REVISION:  SOE for DOY 238 updated, at 15:20:59 AOS S-band un-modulated

1-way deleted

NOP: 871-049-041, REV.B

SCHEDULE: Refer to DSN 7-Day Schedule for actual support times.

DOY   SOA   BOT   EOT   EOA  FACILITY  USER   ACTIVITY  PASS

238  1230  1530  1850  1950   DSS-14    MEX   BISTATIC  0817

239  0900  1200  1505  1605   DSS-14    MEX   BISTATIC  0818

TYPE: Level 3 / Bistatic Radar Experiment Support

1.   OVERVIEW:

DSS-14 is scheduled to support a Mars Express bistatic radar (BSR)

experiment on DOY 205. BSR experiment employs the MEX spacecraft's

transmitter and a DSN 70-m antenna to simultaneous receive weak S-band and

X-band right hand circular and left hand circular polarized signals.  These

four RF signals are then routed to and recorded by Radio Science Receivers

(RSRs). At a predetermined time during the pass, the spacecraft slews toward

Mars so that its transmitted S-band and X-band RCP signals illuminate the

planet's surface. As the X and S-band RCP signals reflect off the surface, a

Left Circular Polarization (LCP) signal is produced effectively creating

four signals; 1) S-band RCP, 2) S-band LCP, 3) X-band RCP and 4) X-band LCP.

2.   SUPPORT DOCUMENTATION:

a. Mars Express NOP 871-049-041, Rev. A

b. This briefing message

c. The attached bistatic radar SOE supersedes the DSN SOE. Radio Science

shall provide start/stop times of BSR mini-pre-calibrations near real-time.

3.   SUPPORT REQUIREMENTS:

a. DOWNLINK SUPPORT ONLY

b. DTTS (DOWNLINK TRACKING AND TELEMETRY) RECEIVERS:

*        One DTT shall be configured with table 7041HRX to support X-band,

and to control the 12.5k noise diode on/off at predetermined times during

pre-pass calibrations, real-time BSR activities and post calibrations

activities.

*        A second DTT shall be configured with table 7041HRXS to support

S-band carrier lock only and to control the 12.5 k noise diode on/off at

predetermined times during pre-pass calibrations, real-time BSR activities

and post calibrations activities.

c.   UPLINK

*        There are NO uplink requirements

d.   COMMAND:

There are NO command requirements

e.   RADIO SCIENCE RECEIVERS (RSRS):

NOTE: For remote operations the FSPR must not be assigned to the DSS link.

The RSRs are controlled and configured remotely by the Radio Science group

located at JPL. The RSR operator will configure four radio science receivers

to simultaneously receive and record S-band and X-band RCP and LCP polarized

signals. The RSR equipment requires both S-band and X-band 1-way and 2-way

downlink predicts. The actual RSR resources configured for support will

depend on availability.

1.      R1B IFS 14_S_RCP

2.      R2A IFS 14_S_LCP

3.      R1A IFS 14_X_RCP

4.      R2B IFS 14_X_LCP

4.   NOCT:

Transmit X-band up/down, and X-up/S-band down predicts to the station

5.   GCF:

Voice: DSS /ROC MOPS/AMMOS/MEX MOC/ DSN OPS (RS)

6.   DSOT

There is NO SLE telemetry return link services required

7.   SPACECRAFT CONFIGURATION:

The spacecraft shall be reconfigured for X-band and S-band downlink carrier

only in accordance with the BSR SOE ( see attachment)

8.   PRE-CALIBRATION PROCEDURES AND CONFIGURATION

a.       Pre-calibration requires configuring the 70m-antenna microwave

subsystem so that each receiving downlink channel (S-RCP, S-LCP, X-RCP,

X-LCP) can be cycled in sequence through the four configurations listed

here:

1.      Receiving channel into ambient load with noise diode off

2.      Receiving channel into ambient load with noise diode on

3.      Receiving channel into sky (zenith) with noise diode on

4.      Receiving channel into sky (zenith) with noise diode off

b.      Link Controller perform the procedures in step-1 (below) and advise

Radio Science when completed; wait here until directed by Radio Science to

proceed to step-2.

Step-01  00:00:00  US14 MOD 102 A         Confirm dichroic reflector in place

US14 CNF XRA1O1        X-RCP (LNA-1) to Sky

US14 CNF XLA2O2        X-LCP (LNA-2) to Sky

US14 CNF SLLA2C2O2     S-LCP (LNA-2) low noise to Sky

US14 CNF SRDA1C1O1     S-RCP (LNA-1) via Diplexer to Sky

AP SCAN D              Conscan Off

AP TRK SR              Sub-reflector in Tracking Mode

NOPE: Utilizing the Connection Log Display, ensure DSS-14's micro wave

switch configurations are set in accordance with step-1

c.   Radio science operator after verifying correct signals levels, direct

link controller to proceed to step 2

Step-02  00:04:00  US14 CNF XA1A2O2       LNA2 to ambient load-1 (X-LCP)

US14 CNF XA1A1O1       LNA1 to ambient load-1 (X-RCP)

Step-03  00:06:00  US14 CNF SA2A2C2O2     LNA2 to ambient load-2 (S-LCP)

Step-04  00:08:00  US14 CNF SA1A1C1O1     LNA1 to ambient load-1 (S-RCP)

*        After steps 2, 3, and 4 (above) have been completed, perform

procedures given below in steps a through e:

a. Link Controller report ambient load temperatures (S1, S2, and X1) from

the PRFREA display and report current weather conditions, (temperature,

humidity, and sky conditions) to the Radio Science operator.

b. Radio Science operator set each RSR attenuator using AUTO feature

c. RSR operator run radio science recorders

d. RSR operator report to Link Controller when radio science recorders are

recording

e. Link controller execute the following procedures at 5 min intervals and

report the completion of each step to the RSR operator:

NOTE: The Downlink Channel (DC) configurations given below utilize DC01 for

X-band and DC02 for S-band, but any available DC0 resource can be used.

ATTENTION: Exercise care and do not confuse the number "0" with the letter

"O", for example "DC01" and "DSS14XO1"

Step-05  00:10:00  DC01 DISC

DC01 CONN DSS14XO1     Configure DTT 01 to X-RCP

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC

DC02 CONN DSS14SO1     Configure DTT 02 to S-RCP

DC02 SET SNT NDS=12.5      (possible problems with

DC02 SET SNT MFQ=ON         'SET': syntax error)

Step-06  00:15:00  DC01 SNT E             Enable X-RCP noise diode

Step-07  00:20:00  US14 CNF XRA1O1        X-RCP to sky

US14 CNF XLA2O2        X-LCP to sky

Step-08  00:25:00  DC01 SNT D             Disable X-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:30:00  DC01 SNT E             Enable X-LCP noise diode

CONFIRM NOISE DIODE SELECTION IS 12.5

Step-10  00:35:00  US14 CNF XA1A1O1       X-RCP (LNA-1) to ambient load

US14 CNF XA1A1O2       X-LCP (LNA-1) to ambient load

00:36:00  Link Controller report ambient load temp (S1, S2, X1)

from the PRFREA display to the RS operator.

Step-11  00:40:00  DC01 SNT D             Disable X-LCP noise diode

Step-12  00:45:00  US14 CNF XRA1O1        X-RCP (LNA-1) to sky

US14 CNF XLA2O2        X-LCP (LNA-2) to sky

Step-13  00:50:00  DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-14  00:55:00  US14 CNF SRDA1C1O1     S-RCP (LNA-1) via diplexer to sky

Step-15  01:00:00  DC02 SNT D             Disable S-RCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

CONFIRM NOISE DIODE IS 12.5

Step-16  01:05:00  DC02 SNT E             Enable S-LCP noise diode

Step-17  01:10:00  US14 CNF SLLA2C2O2     S-LCP (LNA-2) to sky

Step-18  01:15:00  DC02 SNT D             Disable S-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Link Controller report ambient load temp (S1, S2, X1)

from the PRFREA display to the RS operator.

01:20:00  End of BSR pre-calibration procedures, configure for

real-time and wait for acquisition of signal

9. MINI CALIBRATION PROCEDURES AND TIMELINE

Link Controller, shortly after MEX begins its slew and before the BSR

surface observations begin, confirm DTT 01 and DTT 02 configurations.

a.       Link Controller confirm DTT 01 is configured for X-RCP; if not,

perform the following operator directives:

*      DC01 DISC

*      DC01 CONN DSS14XO1

*      DC01 SET SNT NDS=12.5

*      DC01 SET SNT MFQ=ON

b.   Link Controller confirm DTT 02 is configured for S-RCP; if not, perform

the following operator directives:

*      DC02 DISC

*      DC02 CONN DSS14SO1

*      DC02 SET SNT NDS=12.5

*      DC02 SET SNT MFQ=ON

c.   RSR operator ensure all Radio Science Recorders are recording.

d.   Link Controller, at direction from RSR operator begin the BSR

mini-noise calibration procedures (step-1 through step-6. The times shown

are to illustrate event chronology and event duration:

Step-01  00:00:00  DC01 SNT E             Enable X-RCP noise diode

CONFIRM NOISE DIODE VALUE IS 12.5

Step-02  00:03:00  DC01 SNT D             Disable X-RCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

US14 D XMAP            Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-03  00:06:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

US14 D SMAP            Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-04  00:09:00  DC01 SNT D             Disable X-LCP noise diode

DC02 SNT E             Enable S-LCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

US14 D XMAP            Verify configuration

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-05  00:12:00  DC02 SNT D             Disable S-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

US14 D SMAP            Verify configuration

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

00:15:00  End of BSR Mini-Noise Cal

Step-06    Link Controller, repeat the BSR mini-noise

calibration procedures steps 1-5 after MEX has completed its surface

observations and before it returns to Earth point. The start time will be

provided by radio science.

10. POST CALIBRATION PROCEDURES AND TIMELINE:

Link Controller, at direction from RSR operator begin post-calibration

procedures (step-1 through step-10). The times shown are to illustrate event

chronology and event duration:

Step-01  00:00:00  Link controller move antenna to zenith (do not stow)

US14 CNF SA1A1C1O1     LNA1 to ambient load1 (S-RCP)

US14 CNF SA2A2C2O2     LNA2 to ambient load2 (S-LCP)

US14 CNF XA1A1O1       LNA1 to ambient load1 (X-RCP)

US14 CNF XA1A2O2       LNA2 to ambient load1 (X-LCP)

DC01 DISC              Configure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

DC02 DISC              Configure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

a. Link Controller advise the RSR operator when the antenna is at zenith and

the configurations listed above have been completed.

b. Link Controller report physical temperatures of each ambient load (S1,

S2, and X1) from the downlink PRFREA display.

c. RSR operator run all Radio Science Recorders and record ambient loads for

3 min. At completion, advise the Link Controller to proceed with the

following steps:

Step-02  00:03:00  DC01 SNT E             Enable X-RCP noise diode

Step-03  00:06:00  DC01 SNT D             Disable X-RCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-LCP

DC01 CONN DSS14XO2

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-04  00:09:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-LCP

DC02 CONN DSS14SO2

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-05  00:12:00  DC01 SNT D             Disable X-LCP noise Diode

DC02 SNT E             Enable S-LCP noise diode

Step-06  00:15:00  US14 CNF SRDA1C1O1     S-RCP (LNA-1) to sky

US14 CNF SLLA2C2O2     S-LCP (LNA-2) to sky

US14 CNF XRA1O1        X-RCP (LNA-1) to sky

US14 CNF XLA2O2        X-LCP (LNA-2) to sky

Step-07  00:18:00  DC02 SNT D             Disable S-LCP noise diode

DC01 SNT E             Enable X-LCP noise diode

DC02 DISC              Reconfigure DTT 02 for S-RCP

DC02 CONN DSS14SO1

DC02 SET SNT NDS=12.5

DC02 SET SNT MFQ=ON

Step-08  00:21:00  DC01 SNT D             Disable X-LCP noise diode

DC02 SNT E             Enable S-RCP noise diode

DC01 DISC              Reconfigure DTT 01 for X-RCP

DC01 CONN DSS14XO1

DC01 SET SNT NDS=12.5

DC01 SET SNT MFQ=ON

Step-09  00:24:00  DC02 SNT D             Disable S-RCP noise diode

DC01 SNT E             Enable X-RCP noise diode

Step-10  00:27:00  DC01 SNT D             Disable X-RCP noise diode

00:30:00  End

a. Link Controller report physical temperatures of each

b.ambient load (S1, S2, and X1) from the downlink PRFREA display and report

the current weather (temperature, humidity, and sky conditions) to radio

science

c. Radio Science operator stop RSRs

d. End of BSR noise post-calibration procedures

e. Link Controller proceed with normal tear-down procedures

f. Re-initialization of the DTTs is recommended

MPSETD POINTS OF CONTACT: DSN-MPSETD@jjpl.jpl.nasa.gov

<mailto:DSN-MPSETD@jjpl.jpl.nasa.gov>

MPSETD          OFFICE      OPS WORK STATION

Jose Valencia (LEAD)  (626) 305-6308  (626) 305-6370   (626) 305-6368

Jesse Velasco (NOPE)  (626) 305-6315

Romy Paredes (NOPE)   (626) 305-6309

Greg Hewitt (NOPE     (626) 305-6292

RADIO SCIENCE WORK STATION

(818) 393- 0666