Gene, Danny:

This is my recommendation for the MEX bistatic radar operation today.

I'll plan to check in by phone; I'll be at 650-723-0296 if you need me.

There is a MEX telemetry pass at DSS 63 just before the BSR and more telemetry at DSS 25 just after the BSR.

There will be FRO's for this experiment, contrary to what I told you Friday night (I'm glad I checked).  With these FRO's the X-band echo should be in the center of the passband; the residual carrier may be visible on subchannel 4 at about -19000 Hz.  The S-band residual carrier will be at about -2500 Hz and the echo will be at +2500 Hz on subchannel 3.  Both echoes should be narrow (less than 1 kHz) and not move much at all.

Dick

- - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2005/345

-----

21:10  Begin Pre-cal at DSS 63

X-RCP FGAIN = 68 dB; all others = 60 dB

FRO_x = 0; FRO_s = 0 (changes later)

Start recording after Step 4; stop after Step 18

2005/346

--------

00:10  Beginning of track (probably no spacecraft signals)

00:34  X TX ON (with telemetry, which we ignore)

00:38  S TX ON (C/No ~ 55 dB)

00:55  X-Band telemetry OFF (c/No ~76 dB)

00:58  X-RCP FGAIN = 68 dB; all others 60 dB

00:59  Begin 25 kHz RSR recordings

01:30  FRO_x = +16000 Hz (X carrier should be at about -19000 Hz)

FRO_s =  +1000 Hz (S carrier should be at about  -2500 Hz)

01:41  Begin Slew #1

01:45  Begin MiniCal #1

01:52  End Slew #1

01:53  Begin 100 kHz RSR recordings

02:00  End MiniCal #1 (end of Step 5)

02:23  Maximum echo signals

02:50  Begin MiniCal #2

02:53  S TX OFF

Begin Slew #2

End 100 kHz recordings

02:55  X telemetry modulation ON

03:04  End Slew #2

03:05  End MiniCal #2 (end of Step 5)

03:10  End of Track

03:15  Begin Post-Cal (time approximate)

04:10  End Post-Cal (time approximate)

Stop 25 kHz RSR recordings

Scheduled end of Activity