Mars Express Bistatic Radar Experiment

Operations Plan and Report

18 August 2008

Summary

=======

DSN Antenna:                                                     14

Pass:                                                          1907

Orbit:                                                         5945

Orbit Start Time:                               2008-08-18T19:36:34

HGA Pointing:                                              Specular

Start     Center       End

--------  ----------  --------

Specular Condition (ERT):            23:22:00   23:34:00   23:43:00

Target Latitude (deg N):                53.32      84.98      66.32

Longitude (deg E):               17.91     293.06     225.15

Rp (km):                         3394.      3394.      3394.

Incidence/Reflection Angle (deg):       89.29      60.74      42.08

Slant Range (km):                       1505.      1279.      1788.

Slew Angle (deg):                        1.42      58.51      95.84

Doppler (carrier, fd; Hz):             90179.     94851.     81915.

Doppler (echo, fr; Hz):                89168.     42463.     -2450.

Doppler Difference (fdd; Hz):           1011.     52388      84364.

Earth-Mars Distance (m):            3.540E+11  3.540E+11  3.540E+11

Experiment Set Up

=================

This experiment was conducted using Goldstone's DSS 14.  Danny Kahan and Gene Goltz were in the Radio Science Support Area (RSSA) at JPL; Dick Simpson monitored by telephone from Stanford.  Jesse Velasco was in the NOPE area.

Performance Problems and Notes

==============================

There was a delay in starting the pre-cal because one of the S-Band low noise amplifiers had failed (DR G108775).  The HEMP was used for S-RCP, and there was no reception of S-LCP.

To simplify the noise diode calibrations with no S-LCP in the system, some of the steps were omitted in real time.  However, at about Step 13 in the pre-cal, station personnel became confused and the procedure had to be taken back to Step 08 or Step 09 and then moved forward again.

Both Gene and Danny reported that the real-time readings of noise power and analog-to-digital converter (ADC) amplitude were very unstable on S-RCP.

The change from spacecraft pointing to planetary pointing (and back again) occurred without incident at DSS 14.

Real-time echoes were seen in all three channels (X-RCP, X-LCP, and S-RCP).

Data Acquisition

================

RSRs were configured as in Table 1.

Table 1

----------------------------------------------------

RSR  Channel Mode  ATT FGAIN        Operator

dB

----- ------- ---- ---- ----- ----------------------

RSR3A  S-LCP   1-W auto   60    unknown

RSR2B  X-LCP   1-W auto   60    unknown

RSR2A  S-RCP   1-W auto   60    unknown

RSR3B  X-RCP   1-W auto   65    unknown

RSR subchannels (SCHAN) were defined as follows:

Table 2

----------------------------------------------------------------------

Subchannel  Sample Rate  Comments

----------  -----------  ---------------------------------------------

1          2 ksps   Recording bandwidth for occultations (RCP only)

2          8 ksps   backup (not recorded)

3         25 ksps   Primary recording bandwidth

4        100 ksps   Backup recording

Table 3 lists ADC amplitude levels read from RSR displays during the experiment.  Times are in UTC and should be considered approximate.  "Steps" are as defined in the briefing message.  RSR ATT settings are in units proportional to dB.  Acronyms and abbreviations are explained after Table 3.

Table 3

-----------------------------------------------------------------------------

Activity  Time   Step #  S-RCP  X-LCP  S-LCP  X-RCP  Notes / Comments

2008/231                 RSR2A  RSR2B  RSR3A  RSR3B

-------- ----- --------- -----  -----  -----  -----  ------------------------

Pre-Cal 18:53       1

18:55            -10.0  -10.2   n/a    -9.6  Atten set using "auto"

19:02       2    -10.0   -0.3   n/a    +0.2

19:03       4     -3.1   -0.3   n/a    +0.2

19:06             18.0   22.5   n/a    24.0  Attenuators (dB)

Amb loads, phys temps

S1=n/a

S2=18.95

X1=18.95

-10.1  -10.1   n/a   -10.0  Weather condx:

T=33.1C

H=10.5 percent

sky=clear

19:08                                        Begin 25 kHz recording

19:15       5    -10.1  -10.0   n/a    -9.9

19:19       6     -9.9  -10.1   n/a    -9.7

19:25       7    -10.0  -22.3   n/a   -20.0

19:30       8    -10.1  -22.1   n/a   -22.6

19:35       9    -10.0  -20.0   n/a   -22.6  Confirm 12.5 noise diode

19:40      10    -10.0   -9.8   n/a   -10.0  Amb loads phys temps:

S1=n/a

S2=24.00

X1=16.88

19:45      11    -10.0  -10.0   n/a   -10.0

19:50      12    -10.0  -22.2   n/a   -22.5

20:11      13     -9.8  -22.1   n/a   -22.5  X-RCP problem; repeated

steps 8,9,11,13

20:15      14    -14.9  -22.5   n/a   -22.5

20:20      15    -16.7  -22.2   n/a   -22.5  Confirm 12.5 noise diode

Amb loads phys temps:

S1=n/a
                                                      S2=24.50

X1=17.25

20:25     end    -15.3  -22.2   n/a   -22.5  Stop 25 kHz recording

BOT     20:25            -15.8  -21.9   n/a   -21.6

22:25             -10K   -11K   n/a    -11K  FRO (SCHAN 3, Hz)

22:25             -40K   -40K   n/a    -40K  FRO (SCHAN 4, Hz)

22:30             -9.6  -22.0   n/a   -21.8  Resume 25 kHz recording

OCI     22:38                                        Begin 2 kHz recording

22:55             -7.4  -22.0   n/a   -21.8  Switch to Mars pointing

22:58                                        Stop 2 kHz recording

MinCal1 23:00       1     -8.7  -22.0   n/a   -19.7  Confirm 12.5 noise diode

23:03       2    -12.3  -22.0   n/a   -21.6

23:06       3    -10.7  -19.9   n/a   -21.7

23:09       4     -8.0  -22.0   n/a   -21.7

23:15     end

OCE     23:16                                        Resume 2 kHz recording

BSR     23:20            -12.7  -21.9   n/a   -22.2  Begin 100 kHz recording

23:26                    -22K          -22K  FRO (SCHAN 3, Hz)

23:28                    -32K          -32K  FRO (SCHAN 3, Hz)

23:30                    -42K          -42K  FRO (SCHAN 3, Hz)

23:31             -20K                       FRO (SCHAN 3, Hz)

23:31                    -80K          -80K  FRO (SCHAN 4, Hz)

23:32                    -52K          -52K  FRO (SCHAN 3, Hz)

23:34                    -62K          -62K  FRO (SCHAN 3, Hz)

23:36                    -72K          -72K  End 2 kHz recording

23:39             -30K   -82K          -82K  FRO (SCHAN 3, Hz)

23:42                    -92K          -92K

23:43            -16.5  -21.9   n/a   -22.4  End 100 kHz recording

MinCal2 23:45       1    -16.0  -21.9   n/a   -19.7  Confirm 12.5 noise diode

23:48       2    -17.1  -21.9   n/a   -21.6

23:51       3    -19.6  -19.8   n/a   -21.6

23:54       4    -17.7  -21.8   n/a   -21.6

2008/232

--------

00:00            -18.0  -21.8   n/a   -21.6

00:06             -10K   -11K   n/a    -11K  Switch to s/c pointing

EOT     00:25            -16.2  -22.0   n/a   -22.4

PostCal 00:30       1     -9.9  -10.0   n/a   -10.0  Amb load phys temps:

S1=n/a

S2=29.56

X1=19.56

00:35       2    -10.0  -10.1   n/a   -9.8

00:38       3    -10.0  -10.0   n/a  -10.0

00:41       4     -9.9   -9.9   n/a  -10.1

00:44       5     -9.9  -10.0   n/a  -10.0

00:47       6    -17.6  -22.2   n/a  -22.7

00:50       7    -17.7  -20.0   n/a  -22.8

00:53       8    -16.8  -22.2   n/a  -22.6

00:56       9    -17.5  -22.2   n/a  -20.1

00:59      10    -17.3  -22.2   n/a  -22.6   Amb load phys temps

S1=n/a

S2=29.12

X1=19.56

Weather conditions:

T=33.3C

H=11.8 percent

sky=clear

EOA     01:03                                        End 25 kHz recording

AMB = ambient load

BOT = Beginning of Track

CNR = Carrier to noise ratio

CONSCAN = conical scan tracking

CW = continuous wave (carrier only)

EOT = End of Track

FRO = frequency offset

HGA = high-gain antenna

LOS = loss of signal

ND = noise diode

No = noise power

Pc = carrier power

rcvr = receiver

S1 = ambient load for S-RCP

S2 = ambient load for S-LCP

SL = S-LCP

SNR = Signal to noise ratio

SR = S-RCP

TLM = telemetry

X1 = Ambient load for both X-band channels

XL = X-LCP

XR = X-RCP

Dick Simpson

Original: 2008-08-19