0    H_TYPE              Header type  (D=DISR, I=IMP)

1    H_DIRECTORY         First part of filename

2    H_FILENAME          Last part of filename

3    H_FILETIME          Time file was last written by D_WRITE

4    H_LENGTH            Length of header string array

5    H_DIMENSION         Number of dimensions in data

6    H_XSIZE             Number of columns

7    H_YSIZE             Number of rows

8    H_ZSIZE             Number of images

9    H_DATATYPE          Type of data

1=byte

2=16 bit integer

3=32 bit integer

4=floating point

5=double precision

6=complex

7=string

8=structure

10   H_EXTENSION         Position in header where extension area starts

11   H_EXTRA             Position in header where extra area starts

12   H_PROCESS           Position in header where processing area starts

13   H_PURPOSE           Purpose of observation

In Process, Verification, or Calibration

14   H_SUBJECT           Discipline taking data:

Optics, Electronics, SW, System, Environments

15   H_DATE              Date observation made

Format: Wed Nov 24 10:04:11 1997

16   H_ENGINEER          Engineer responsible

17   H_SITE              Site of observation

Martin Marietta

University of Arizona

MPAE

ESOC

MBB

18   H_SET_NAME          Text description of data set (see 86)

19

20

21

22   H_CCDTEMP           On chip CCD temperature in [K]

23   H_REF_TEMP          CCD electronics reference temperature (MPAE)

24

25

26

27

28

29

30   H_DETECTOR          detector type (CCD, IR, PHOTOMETER, Aux)

31   H_SENSOR_ID         sensor id number

32

33

34

35

36   H_EXPTIME           integration time [ms] for CCD measurement

37   H_IMAGE_TIME        start time of exposure

38   H_ABSCAL            absolute calibration factor used

39

40

41

42   H_COORD_XL_2 SA2, Dark col 2, and Vis Ext 2 lower left pixel

43   H_COORD_YL_2 SA2, Dark col 2, and Vis Ext 2 lower left pixel

44   H_COORD_XL_3 SA3 lower left pixel

45   H_COORD_YL_3 SA3 lower left pixel

46   H_COORD_XL_4 SA4 lower left pixel

47   H_COORD_YL_4 SA4 lower left pixel

48   H_AXIS_EL           elevation of DISR spin axis

49   H_AXIS_AZ           azimuth of DISR spin axis

50   H_COORD_XL          CCD coordinates (lower left pixel)

Images, SA1, Dark col 1, Vis Ext 1

51   H_COORD_YL          CCD coordinates (lower left pixel)

Images, SA1, Dark col 1, Vis Ext 1

52   H_COORD_XU          not used

53   H_COORD_YU          not used

54   H_FILTER            filter or wavelength in use

55   H_GAIN              gain factor (MPAE)

56   H_DC_OFFSET         DC offset (MPAE)

57

58

59

60   H_TIMING_GEN        timing generator code (MPAE)

61   H_CAMERA            BB/EM/FM model

62   H_VACUUM            Ambient or Vacuum

63   H_OPTICS            optical setup for data set

Collimated light

Int. Sphere

f/#

64   H_TESTLAMP          which external source used

Monochromator

High Intensity

Broadband

Laser

Dark

Other

65   H_SHUT_TIME         opening time of CCD test shutter (MPAE)

66   H_LED_TIME          LED on time (MPAE)

67

68

69

70

71

72

73

74

75

76

77

78

79

80   H_GSE_REV           GSE revision creating data set

81   H_CONFIG_FILE       configuration file associated with data set

82   H_TEST_LOG          test log filename generating data

83   H_REC_BEG           first record of this data set in test log

84   H_REC_END           last record of this data set in test log

85   H_PIXEL_UNITS       units of pixels (digital numbers)

All data will be stored from light in raw ADC units

86   H_SET_ID            data set id

1=Message

2=Time

3=Sun Sensor

6=Housekeeping

7=Lamp

8=Descent Cycle

9=Calibration Cycle

10=Visible

11=Image

12=Strip

13=Solar Aureole

14=Dark Current

15=Full CCD

16=IR

17=Violet

19=Visible Extra

20=Shutter Test

21=DCS Test

22=Heater Test

23=Calibration Lamp Test

24=Surface Lamp Test

25=Sun Lamp Test

26=Bad RAM

27=Bad EEPROM

28=Memory Dump

29=Image Graph (WGSE)

30=Raw

87   H_SET_NUMBER        sequential number of this data set

88   H_MISSION_TIME      mission time in seconds

89   H_CYCLE_NUMBER      sequential numbering of cycles

90   H_CYCLE_TYPE        cycle type

1=Standard non-image

2=Standard image

3=Flat Field

4=Cal Cycle A

5=Cal Cycle B

6=Cal Cycle C

8=Dark current only

9=Spectrophotometric

10=Drain cycle

11=High near surface

12=Medium near surface

13=Low near surface

14=Very low near surface

15=Surface A

16=Surface B

17=Surface C

18=Surface D

91   H_SCEN_STEP         cycle criteria table entry number

1=First cal cycle A

2=First cal cycle B

3=First cal cycle C

4=Second cal cycle A

5=Second cal cycle B

6=Second cal cycle C

7=Third cal cycle A

8=Third cal cycle B

9=Third cal cycle C

10=Fourth cal cycle A

11=Fourth cal cycle B

12=Fourth cal cycle C

13=Medium near surface when <0.3rpm and >3km

14=Standard image for first cycle

15=Flat field

16=Drain cycle

17=Dark current only when >1rpm and <10km

18=Spectrophotometric when >1rpm and <10km

19=Dark current only when >1rpm and <5km

20=Spectrophotometric only when >1rpm and <5km

21=High near surface when 3km < altitude < 20km

22=Medium near surface when 400m < altitude < 3km

23=Low near surface when 200m < altitude < 400m

24=Very low near surface when 0m < altitude < 200m

25=Surface cycle A for first cycle on the surface

26=Surface cycle B for second cycle on the surface

27=Surface cycle C for third cycle on the surface

28=Surface cycle D for fourth cycle on the surface

29=Standard image for cycles with < 4 min of telemetry

data left in the buffer

30=Standard non-image cycle (default)

92   H_SPM_FLAG          spectrophotometric flag (1=SPM)

93   H_CCD_MEAS          number of CCD set performed during cycle

94   H_IR_MEAS           number of IR set performed during cycle

95   H_VIOLET_MEAS       number of violet set performed during cycle

96   H_MEAS_TYPE         measurement type

0=Upper half DLI1

1=Upper half DLI2

2=Upper half SLI

3=Lower half DLI1

4=Lower half DLI2

5=Lower half SLI

6=DLV

7=ULV

8=DLIS

9=ULIS

10=IR Combined (normal operation)

11=IR Long (long integration for dark reading)

12=SLI strip

14=Solar Aureole

15=Near Surface DLVS

16=DLVS

17=ULVS

18=CCD Dark Current

19=Full CCD

21=Whole DLI2

22=Whole SLI

23=Whole DLI1

30=DLVS Extra

31=ULVS Extra

97   H_AZIMUTH           azimuth at start of cycle in degrees

98   H_ALTITUDE          altitude at start of cycle in meters

99   H_SPIN              spin rate at start of cycle in rpm

100  H_TARGET_AZ         target azimuth for measurement in degrees

101  H_ACTUAL_AZ         actual azimuth for measurement (Flight SW's idea)

102  H_LAMP_STATE        cal and surf lamp status, 1=on, four character string

Char 0 = Cal lamp 1

Char 1 = Cal lamp 2

Char 2 = Cal lamp 3

Char 3 = Surface Science Lamp

103  H_IR_STATUS         IR hardware status word

104  H_IR_FLAGS          IR processing, two character string

Char 0 = optimum sampling, 1=used

Char 1 = compression, 1=compressed

105  H_IR_CHP_TMPB       IR chip temperature (start) in Kelvin

106  H_IR_CHP_TMPE       IR chip temperature (end) in Kelvin

107  H_PRECHARGE         Average of ULIS and DLIS precharge voltage

at beginning of collection in ADU

108  H_IR_COL_TIME       IR collection time [in sec (multiple of 8.064 ms)]

109  H_NUM_ROT           number of region/rotations (IR)

110  H_NUM_REGIONS       number of regions (IR)

111  H_DC_OFFSET_U       DC Offset voltage for ULIS in ADU

112  H_DC_OFFSET_D       DC Offset voltage for DLIS in ADU

113  H_UIR_TGT_PRC       target %age for ULIS

114  H_DIR_TGT_PRC       target %age for DLIS

115  H_UIR_PRCTILE       %ile point used for ULIS

116  H_DIR_PRCTILE       %ile point used for DLIS

117  H_CCD_STATUS        CCD hardware status

118  H_DCS_STATUS        DCS hardware status

119  H_CCD_FLAGS         Four character string; 1=all ok, 0=error

Char 0 = Pixel error

Char 1 = New frame error

Char 2 = New line error

Char 3 = Not used; set to 0

120  H_PROC_FLAGS        Six character string for CCD processing; 0=off,1=on

Char 0 = Bad pixel replacement

Char 1 = Summing

Char 2 = Noiseless compression

Char 3 = Square root compression

Char 4 = Hardware compression

Char 5 = Optimum exposure

121  H_NUM_COL           number of columns transmitted

122  H_NULL_PIXEL2       sum of null pixels (col 2)

123  H_NULL_PIXEL3       sum of null pixels (col 3)

124  H_CCD_TGT_PRC       target %age for CCD

125  H_CCD_PRCTILE       %ile point for CCD

126  H_STP_CNT_COL       strip center column

127  H_STP_FST_COL       1st column of strip from readout side of CCD

128  H_CAL_TMP_SORS      Current through temperature sensors

129  H_CCDLUG_TEMP       CCD thermal lug temperature [K]

130  H_STRAP_TEMP        thermal strap temperature [K]

131  H_OPTICS_TEMP       conduit housing temperature [K]

132  H_VIOLET_TEMP       violet detector temperature [K]

133  H_SH_AUX_TEMP       sensor header aux board temperature [K]

134  H_SH_BOX_TEMP       sensor header box temperature [K]

135  H_EA_BOX_TEMP       EA box temperature [K]

136  H_AUX_BRD_VOLT      divided 12V in volts

137  H_CPU_BRD_VOLT      divided 5V in volts

138  H_ADC_OFFSET        Not used

139  H_ADC_GAIN          Not used

140  H_DISPQ_SIZE        max. dispatcher queue size since last HK data set

141  H_ALRMQ_SIZE        max. alarm queue size since last HK data set

142  H_TLMQ_SIZE         max. TM queue size since last HK data set

143  H_SCIPRQ_SIZE       max science processing size since last HK data set

144  H_STACK_SIZE        max stack size since last HK data set

145  H_CAL_SCENARIO      cal. scenario number, 1=health check, 2=inflight cal

146  H_CCD_REP           number of repetitions for CCD cal measurement

147  H_IR_REP            number of repetitions for IR cal measurement

148  H_VIOLET_REP        number of repetitions for violet cal measurement

149  H_SHUTTER_REP       number of repetitions for shutter test

150  H_SHUTTER_CYC       number of shutter cycles per shutter test

151  H_DCS_REP           number of repetitions for DCS test

152  H_COMP_RATIO        compression ratio for DCS

153  H_HEATER_REP        number of repetitions for heater test

154  H_HEATERS           which heaters tested, 1=strap heater, 2=Aux bd heater

155  H_CAL_LAMP_REP      number of repetitions for cal. lamp test

156  H_NUM_BINS          number of IR bins for data collection

157  H_SURF_LAMP_REP     number of repetitions for surface lamp test

158  H_SUN_LAMP_REP      number of repetitions for sun sensor LED test

159  H_CAL1_VOLT1        voltage on cal. lamp 1

160  H_CAL1_CURR         current on cal. lamp 1

161  H_CAL2_VOLT1        voltage on cal. lamp 2

162  H_CAL2_CURR         current on cal. lamp 2

163  H_CAL3_VOLT1        voltage on cal. lamp 3

164  H_CAL3_CURR         current on cal. lamp 3

165  H_SURF_VOLT1        voltage on surface lamp

166  H_SURF_CURR         current on surface lamp

167  H_DCS_TST_STAT      overall DCS test status

168  H_DCS_SLF_STAT      DCS self test status

169  H_DCS_SW_STAT       DCS s/w test status

170  H_SUN_LED_VLT1      voltage on Sun sensor LED

171  H_SUN_LED_CURR      current on Sun sensor LED

172  H_SUN_RESP          Sun sensor reading with LED on

173  H_NUM_HEAT          number of heaters tested 1 or 2

174  H_NUM_TRIPLET       number of sun triplets in data set, max is 25

175  H_NUM_TIME_PAIRS    number of time data pairs, set to 20

176  H_DUMP_START        address of first word in memory dump data set

177  H_DUMP_LEN          number of words in dump set

178  H_DUMP_FLAG         indicates if addresses containing only byte info

were packed together; 1=packed

179  H_MESSAGE           message type code, Appendix C in User's Manual

180  H_MESSAGE_ID        additional information code for 179

181  H_NUM_BAD           number of ranges or indices of bad RAM areas or

EEPROM patches

182  H_LOTS_BAD          flag - lots bad RAM or EEPROM areas=1

183  H_CCDTEMP_R         Raw CCD chip temperature

184  H_EXPTIME_R         Raw exposure time reading for CCD (0.5ms increments)

185  H_MISSION_TIME_R    Raw mission time reading (0.1ms increments)

186  H_AZIMUTH_R         Raw azimuth rdng at cycle start in 0.1 deg increments

187  H_ALTITUDE_R        Raw altitude reading at cycle start in 10m increments

188  H_SPIN_R            Raw spin rate at cycle start in 0.1 deg/sec increments

189  H_TARGET_AZ_R       Raw target azimuth in 0.1 deg increments

190  H_ACTUAL_AZ_R       Raw actual azimuth in 0.1 deg increments

191  H_IR_CHP_TMPB_R     Raw IR chip temperature start

192  H_IR_CHP_TMPE_R     Raw IR chip temperature  end

193  H_PRECHARGE_R       Raw average ULIS and DLIS precharge voltage

194  H_IR_COL_TIME_R     Raw IR collection time used (number of 8.064ms incrts)

195  H_CAL_TMP_SORS_R    Raw cal. source voltage

196  H_CCDLUG_TEMP_R     Raw CCD thermal lug temperature

197  H_STRAP_TEMP_R      Raw strap temperature

198  H_OPTICS_TEMP_R     Raw conduit temperature

199  H_VIOLET_TEMP_R     Raw violet temperature

200  H_SH_AUX_TEMP_R     Raw SH aux board temp

201  H_SH_BOX_TEMP_R     Raw SH box temperature

202  H_EA_BOX_TEMP_R     Raw EA box temperature

203  H_AUX_BRD_VOLT_R    Raw divided 12V

204  H_CPU_BRD_VOLT_R    Raw divided 5V

205  H_ADC_OFFSET_R      Not Used

206  H_ADC_GAIN_R        Not Used

207  H_CAL1_VOLT1_R      Raw cal. lamp 1 voltage

208  H_CAL1_VOLT2_R      Raw cal. lamp 1 voltage

209  H_CAL2_VOLT1_R      Raw cal. lamp 2 voltage

210  H_CAL2_VOLT2_R      Raw cal. lamp 2 voltage

211  H_CAL3_VOLT1_R      Raw cal. lamp 3 voltage

212  H_CAL3_VOLT2_R      Raw cal. lamp 3 voltage

213  H_SURF_VOLT1_R      Raw surf. lamp voltage

214  H_SURF_VOLT2_R      Raw surf. lamp voltage

215  H_SUN_LED_VLT1_R    Raw sun sensor LED volt

216  H_SUN_LED_VLT2_R    Raw sun sensor LED volt

217  H_OBJECTIVE         Objective of test, Depends on H_PURPOSE

PURPOSE=Optics:

Focus, Alignment, FOV, Performance

PURPOSE=Electronics:

Noise, Functionality, Performance

PURPOSE=System:

Noise, Crosstalk, Calibration, Alignment, FOV,

Optical Performance

PURPOSE=SW or Environments, OBJECTIVE blank

218  H_DUMMY             Flag for dummy detector

0000=no dummy

1100=dummy CCD

0011=dummy IR

1111=dummy CCD and IR

219  H_MODEL             DISR model

220  H_DCOFFSETU_R       Raw ADC value of DC offset voltage, ULIS

221  H_DCOFFSETD_R       Raw ADC value of DC offset voltage, DLIS

222  H_SQRT_MIN          Minimum value for sqrt table

223  H_SQRT_MAX          Maximum value for sqrt table

224

225  H_SUN_SOURCE        source of spin data, Probe, Sun, LED, OGSE

226  H_OGSE_ID           OGSE identifier in use

227  H_OGSE_MSG          last msg from EGSE displayed on OGSE

228  H_OGSE_LAMP         lamp current

229  H_OGSE_SW_VER       OGSE software version

230

231  H_OGSE_SAMP         sun amplitude current

232  H_OGSE_SUNTABLE     sun table

233  H_OGSE_RPM          sun pulse RPM

234  H_OGSE_SPAN         sun pulse span value

235  H_OGSE_SIDEC        Si detector reference

236  H_OGSE_INDEC        InGaAs detector reference

237  H_OGSE_FLSIDEC      filtered silicon detector reference

238  H_OGSE_5V           5V reference

239  H_OGSE_DETEMP       OGSE detector temperature

240  H_OGSE_SSS_TEMP     sun sensor stimulator temp.

241  H_OGSE_FILT         OGSE filter

242

243

244

245

246

247

248

249

250

251

252

253

254

255  H_MONO_WAVL         Monochromator wavelength [nm]

256  H_MONO_SI           Monochromator Si detector reading

257  H_MONO_GE           Monochromator Ge detector reading

258  H_MONO_FLTR         Monochromator filter

259  H_MONO_G            Monochromator gain

260  H_MONO_RT           Monochromator response time [s]

261  H_MONO_GRAT         Monochromator grating

262  H_MONO_TUR          Monochromator turret number

263  H_SPSI              Sphere Si detector reading

264  H_SPFLSI            Sphere filtered Si reading

265  H_SPINGAS           Sphere InGaAs detector reading

266  H_SP_DETTMP         Sphere detector temperature

267  H_COSI              Collimator silicon detector reading

268  H_COFLSI            Collimator filtered Si reading

269  H_COINGAS           Collimator InGaAs detector reading

270  H_COTMP             Collimator detector temperature

271  H_PLMINGAS          Palmer InGaAs detector reading

272  H_PLMTMP            Palmer detector temperature

273  H_CAL_D1            Spare calibration detector 1

274  H_CAL_D2            Spare calibration detector 2

275  H_CAL_D3            Spare calibration detector 3

276  H_CAL_D4            Spare calibration detector 4

277  H_CAL_D5            Spare calibration detector 5

278  H_CPGASTEMP         Temperature of SH internal purge gas

279  H_AMBTEMP           Ambient temperature of clean room

280  H_SPTEMP            Temperature of gas in sphere

281  H_BOXTEMP           Temperature of gas in dry box

282  H_CHMBTEMP          Temperature in environment chamber

283  H_CAL_T1            Spare calibration temp 1

284  H_CAL_T2            Spare calibration temp 2

285  H_CAL_T3            Spare calibration temp 3

286  H_CAL_EL            Sensor head elevation

287  H_CAL_AZ            Sensor head azimuth

288  H_CAL_POL           Collimated beam polarizing angle

289  H_CAL_SHUT          Calibrator high intensity shutter state

0=closed, 1=open

290  H_LAST_CALTIME      Time of last cal. log entry